3GPP TSG-RAN WG4 Meeting #112bis	R4-2417181

Hefei, Anhui, China, October 14th ‒ 18th, 2024

Title:  WF on MSD requirements with intra-band contiguous UL CA_n40A-n41C and CA_n41C-n79A

Agenda Item:	5.1

Source: 	Huawei, HiSilicon, ….

Document for:	Approval

Background

In the RAN4#112 meeting, companies provided some contributions [1][2][3][4][5][6][7] to discuss the intra-band UL CA interference and corresponding RF requirements.

In the contribution [1], companies provided the measurement results for intra-band UL CA and single carrier. The intra-band UL CA interference with some discrete RB allocations can be observed as an example below.

In the contribution [2], operators show the interests and preference and it’s expected that the typical scenario for intra-band UL CA can be considered when RAN4 specify the RF requirements instead of only considering the extreme cases.

In the contribution [3], companies proposed to reconsider the 1RB+1RB RB allocation for intra-band TDD ULCA within an inter-band DL CA and that fully allocated maximum aggregated BW is considered to define the cross-band MSD requirements resulting from intra-band contiguous UL CA.

In the contribution [6], companies observed the following points:

- The extreme discrete RB allocations for intra-band UL CA will lead to the poor RF performance for UE and decrease the output power.

- From network perspective, the extreme discrete RB allocations for intra-band UL CA not only can’t achieve higher UL throughput, but also network have to face the system performance loss due to the reduction of UE output power.

Thus, from RAN4 perspective, some reasonable UL RB allocations for intra-band UL CA can be recommended to benefit both UE and network. Some extreme cases with discrete RB allocations can be discussed from test perspective, but it’s still allowed for proponent to care about the typical scenario.

However, there is no consensus on how to specify the MSD requirements in the RAN4#112 meeting.

In the RAN4#112bis meeting, companies continue to discuss this issue with some contributions [8][9][10][11].

After ad-hoc discussion, a WF is proposed to capture the background and some consensus in order to address this issue.

Way forward on intra-band ULCA within an inter-band DL CA

Proposal 1: In Rel-19, to keep as-is in spec as most of cases are covered and some history is built since Rel-17.

Proposal 2: For the specific cases DL_n40A-n41C_UL_n41C and DL_n41C-n79A_UL_n41C, the following test configurations for MSD requirements can be introduced in next meeting.

- Test case 1: Discrete UL RB allocation, e.g. 1RB + 1RB UL RB allocation in UL intra-band CA.

- Test case 2: fully UL RB allocation with maximum aggregated BW.

NOTE: In the next meeting, RAN4 need to discuss how to capture these two test cases into the spec considering some consistencies.

Way forward on MSD for DL_n40A-n41C_UL_n41C

Proposal 3: For the MSD value DL_n40A-n41C_UL_n41C, the following inputs from companies can be further checked considering the assumption of filter performance and PA interference.

- 3.1) 1RB+1RB test case 1:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 42.5 | TDD | IMD3x |
|  | n41 | 2555 | 80 | 1 (RBSTART=0) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 1 (RBSTART=107) | 2635 | N/A | TDD | N/A |
| Note x: this DL band is also subject to IMD5 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

- 3.2) For fully UL RB allocation test case 2, RAN4 further discuss whether MSD due to IMD3 or ACLR1 cross band isolation is considered for DL_n40A-n41C_UL_n41C.

  - Option 1:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 34.9 | TDD | IMD3xy |
|  | n41 | 2555 | 80 | 216 (RBSTART=1) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 216 (RBSTART=0) | 2635 | N/A | TDD | N/A |
| Note x: this DL band is also subject to IMD5 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

  - Option 2:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| CA_n41C | n40 | 2595 | 190 | 30 | 270 (RBstart=0) + 243 ((RBstart=0) | 2397.5 | 5 | 34 | ACLR1 |

  - Option 3: The cross band isolation MSD caused by fully allocated maximum aggregated BW of UL CA_n41C falls into n40 is 31.5dB and 21.7dB for CA ACLR1 and CA ACLR2, respectively.

  - Other proposal and value are also welcome.

NOTE: In the next meeting, RAN4 can further evaluate the MSD values based on the following channel bandwidth configurations:

Band n41: one 100MHz BW CC at 2565MHz centre frequency and another one 60MHz BW CC at 2645MHz centre frequency with 30kHz SCS.

Band n40: one 5MHz BW CC at 2397.5MHz centre frequency.

Way forward on MSD for DL_n41C-n79A_UL_n41C

Proposal 4: For the MSD value DL_n41C-n79A_UL_n41C, the following inputs from companies can be further checked considering the assumption of filter performance and PA interference.

- 4.1) 1RB+1RB test case 1:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n41-n79 | n41 | 2555 | 80 | 1 (RBSTART=0) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 1 (RBSTART=0) | 2635 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4950 | 8.4 | TDD | IMD4x |
| Note x: this DL band is also subject to IMD6 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

- 4.2) For fully UL RB allocation test case 2, RAN4 further discuss whether MSD due to IMD4 or >ACLR2 cross band isolation is considered for DL_n41C-n79A_UL_n41C.

  - Option 1:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n41-n79 | n41 | 2555 | 80 | 216 (RBSTART=1) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 216 (RBSTART=0) | 2635 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4950 | 4.6 | TDD | IMD4xy |
| Note x: this DL band is also subject to IMD6 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

  - Option 2:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| CA_n41C | n79 | 2595 | 190 | 30 | 270 (RBstart=0) + 243 ((RBstart=0) | 4420 | 40 | 0.5 | >ACLR2 |

  - Option 3: There is no need to define additional cross band isolation MSD requirements caused by fully allocated maximum aggregated BW of UL CA_n41C falls into n79.

  - Other proposal and value are also welcome.

NOTE: In the next meeting, RAN4 can further evaluate the MSD values based on the following channel bandwidth configurations:

Band n41: one 100MHz BW CC at 2565MHz centre frequency and another one 60MHz BW CC at 2645MHz centre frequency with 30kHz SCS.

Band n79: one 10 or 40MHz BW CC at 4405 or 4420MHz centre frequency. Companies may provide input for both channel bandwidths.

Reference

RAN4#112

[1] R4-2411551, On intra-band ULCA interference versus single ULCC, Skyworks Solutions Inc.

[2] R4-2411738, Discussion on MSD requirements with intra-band contiguous UL CA, CMCC

[3] R4-2411887, On MSD requirements with intra-band contiguous UL CA, ZTE Corporation, Sanechips

[4] R4-2412934, Discussion on MSD for CA_n40A-n41C with intra-band UL CA_n41C, Huawei, HiSilicon

[5] R4-2412935, Discussion on MSD for CA_n41C-n79A with intra-band UL CA_n41C, Huawei, HiSilicon

[6] R4-2412940, Discussion on the reasonable UL RB configurations for intra-band UL CA, Huawei, HiSilicon

[7] R4-2412941, Draft CR for TS 38.101-1 to introduce the reasonable UL RB configurations for intra-band UL CA, Huawei, HiSilicon

RAN4#112bis

[8] R4-2416012, On MSD requirements with intra-band contiguous UL CA, ZTE Corporation, Sanechips

[9] R4-2416029, Discussion on MSD for CA_n40A-n41C with intra-band UL CA_n41C, Huawei, HiSilicon

[10] R4-2416030, Discussion on MSD for CA_n41C-n79A with intra-band UL CA_n41C, Huawei, HiSilicon

[11] R4-2417180, CA_n40A-n41C and CA_n41C-n79A MSD due to n41C UL, Skyworks Solutions Inc.