3GPP TSG-RAN WG4 Meeting#113                             R4-2419819

Orlando, USA, 18th – 22th Nov, 2024

Agenda item:	7.11.4.1

Source: 	ZTE Corporation, Sanechips

Title: 	TP to TR38.908: Clause 6.4 spatial sampling grid

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In this contribution, we would like to provide the TP to TR 38.908 on spatial sampling grid.

Discussion

6.4	Spatial sampling grid

For the maximum measurement/spatial sampling step size in the elevation angle, since the elevation angular range for some elevation bins bin is quite limited, especially for 1st, 2nd, 3rd and 4th elevation bin with only 5 degree angular range, in order to ensure the measurement accuracy of EEIRP measurement in the elevation angle to comply with WRC-23 resolution, RAN4 agree to use 1 degree as maximum measurement/spatial sampling step size in the elevation angle.

For the maximum measurement/spatial sampling step size in the azimuth angle, RAN4 agree to use the following approach similar as TRP measurement captured in Annex I in TS 38.141-2:

Δϕref=180∘πλDcyl

Dcyl are calculated as:

`	Dcyl=d2+w2

The definition of d, w and h is shown in figure 6.4-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure 6.4-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

Alternatively, when EUT radiating dimensions are not known,

∆ϕref=λλo BeWϕ

where λo is the wavelength of the wanted signal, and BeWφ are the minimum beamwidth of the wanted signal in the φ-axis,.

BeWφ may be set to minimum beamwidth declared for the OTA BS radiated transmit power requirement.