3GPP TSG-RAN WG4 Meeting #113	R4-2420096

Orlando, US, 18th ‒ 22nd November, 2024

Title: 	WF on RRM requirements for NR_MIMO_Ph5_Part1

Agenda Item:	7.19.4

Source: 	Samsung

Document for:	Approval

<Topic 1: RRM Requirement of Enhancement for UE-initiated/event-driven beam management >

Issue 1-1: How to define Event triggered measurement reporting delay?

< Agreement>

- Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air.

- Ending point is the point when the UE transmits a first PUCCH triggered by the event.

Issue 1-2: How to define measurement period (evaluation time / measurement time) in Event triggered measurement reporting delay?

< Agreement>

Agreement: For specifying the RAN4 delay requirement for event 2

- Measure both the current beam and new beam after the starting point, FFS the details.

- Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion.

- Start the RAN4 RRM discussion for FR2.

- Send LS to RAN1 to ask whether FR1 is covered as well. FFS the exact wording in the LS.

Issue 1-3: Whether RAN4 to specify requirements for case 1 or cover both case1 and case2?

Case 1: no time window based

Case 2: with time window based, M is configurable

< Agreement>

Agreement:

It is RAN4 common understanding that no additional filtering applies for case 2 compared to case 1, if RAN4 will define requirement for case 2.

Issue 1-4: Reporting delay requirements

<Way forward >

- Option 1: (Qualcomm)

  - Tgap + TL1-RSRP-measure; TL1-RSRP-measure in Issue 1-2; The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on; In test cases, set Tgap = 0;

- Option 2: (Nokia)

  - The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event.

- Option 3: (Xiaomi)

  - If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams.

- Option 4: (Samsung)

  - Need further RAN1 progress for RSs relationship between current beam and new beams.

- Option 5: (CMCC)

  - The event triggered measurement reporting delay shall be less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index. The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on

- Option 6: (Apple)

  - The total reporting delay is Tfirst-RS-NB + TL1-RSRP-NB + TPUCCH, Add TPucch in reporting delay

- Option 7: ( MediaTek)

  - The total reporting delay is TL1-RSRP_measure + Tfirst UL channel, TL1-RSRP_measure is base on SSB or CSI-RS

  - Event evaluation period for a SSB or CSI-RS is same as their respective measurement period

- Option 8: (Ericsson)

  - Event triggered measurement reporting delay for SSB is TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel and event triggered measurement delay for CSI-RS is TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel

  - RAN4 to discuss components and criteria for defining TTime to first UL channel.

Issue 1-5: Whether to update L1 report which is measured in L1 report?

<Way forward >

- Option 1: (Xiaomi)

  - It needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

- Option 2: (vivo)

  - RAN4 to specify the requirements to consider the condition which will trigger the event is remained fixed during the time UE performs L1 measurements.

  - Whether UE shall update the L1 report content based on latest L1 measurement results, or shall even cancel the report in some cases, is up to RAN1/2 discussion.

Issue 1-6: Conditions when UE performs L1 measurements for evaluating the event?

<Way forward >

- Option 1: (vivo)

  - No change on the indicated TCI state

  - No change on the to-be-measured RS of the new beam.

Issue 1-7: CSI-RS based L1-RSRP for inter-cell BM

<Way forward >

- Option 1: (Qualcomm, Nokia, Samsung, MediaTek)

  - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

- Option 2: (CMCC)

  - For reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused.

- Option 3: (Apple, Huawei)

  - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

- Option 4: (ZTE)

  - RAN4 needs to identify whether additional clarifications on the following points are necessary:

    - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

    - The prerequisite on L3 measurement on NSC.

    - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Option 5: (Ericsson)

  - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2

  - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

Issue 1-8: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM

<Way forward >

- Option 1: (Qualcomm, Nokia, Samsung, Apple, Huawei)

  - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

- Option 2: (vivo)

  - Regarding the impact to know conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions.

- Option 3: (ZTE)

  - Need to define the condition of known cell for UEIBM. The definition in ICBM can be starting point.

Issue 1-9: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- Option 1: (Qualcomm, Nokia, Samsung (For known conditions for TCI states), Apple, ZTE, Ericsson)

  - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

- Option 2: (Samsung)

  - No unknown case is applied if the TCI states switching is based on UEIBM.

- Option 3: (ZTE)

  - Need to decide whether to define L1-RSRP requirements for unknown cell

Issue 1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting?

<Way forward >

- Option 1: (Nokia)

  - Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting.

- Option 2: (Xiaomi, Samsung, Apple, Ericsson, MediaTek)

  - Reuse current beam sweeping factor for UE/event triggered beam reporting

Issue 1-11: Capabilities for Support of Event Triggering and Reporting Criteria?

<Way forward >

- Option 1: (Nokia)

  - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item.

- Option 2: (Apple)

  - Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

- Option 3: (Ericsson)

  - Define UE capability for total number of events supported for UE-initiated beam management across single and multiple TRP.

- Option 4: (MediaTek)

  - Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

Issue 1-12: Compatibility of capabilities with LTM

<Way forward >

- Option 1: (Nokia)

  - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item.

- Option 2: (Samsung, Apple)

  - Do not combine capabilities across different L1 measurements.

- Option 3: (Ericsson)

  - RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility.

- Option 4: (MediaTek)

  - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

  - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

Issue 1-13: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

<Way forward >

- Option 1: (Qualcomm, Nokia, Xiaomi, Apple, MediaTek)

  - The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements.

- Option 2: (Samsung)

  - Focus on core requirements firstly, FFS on accuracy requirements.

- Option 3: (CMCC)

  - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline.

  - For CSI-RS based beam report, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account.

- Option 4: (ZTE)

  - Discuss the accuracy for the two trigger-event solutions respectively.

    - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement;

    - For the optional trigger-event detection, resue the existing L1-RSRP accuracy requirement.