3GPP TSG-RAN WG4 Meeting #113	R4-2420099

Orlando, USA, 18/11/2024 to 22/11/2024

Agenda item:			7.22.3

Source:	Huawei

Title:	WF on RRM requirements for Netw_Energy_NR_enh_Part2

Document for:	Approval

- Agreements for Adaptation of common signal/channel transmission

Sub-topic 1-1 SSB adapation – Scenario

Issue 1-1-1: Scenario clarification based on RAN1 progress

Agreement:

    - No RRM impacts for SSB adaptation requirement for IDLE mode.

    - No RRM impacts for Cell DTX.

Issue 1-1-2: Scenario clarification- SSB periodicity

Agreement:

  - Postpone the discussion and wait for RAN1 progress.

Issue 1-1-3: Scenario clarification-  SSB adaptation triggering

Agreement:

  - RAN4 to focus on NW controlled SSB adaptation firstly.

Issue 1-1-4: Scenario clarification-  SSB adaptation mechanism

Agreement:

  - Postpone the discussion and wait for RAN1 progress.

Sub-topic 1-2 SSB adaptation – L1 Requirements impacts

Issue 1-2-1: L1 requirements at transition

Agreement:

  - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation.

    - FFS the applicable L1 requirements.

Issue 1-2-2: L1 requirements subsequent to “transition period”

Agreement:

  - Subsequent to transition period, requirements apply based on the adapted SSB periodicity after transition for SSB adaptation of periodicity.

  - Note: The transition period is discussed in other issues.

Sub-topic 1-3 SSB adaptation – L3 Requirements impacts

Issue 1-3-1: L3 requirements for SSB adaptation – general

Agreement:

  - Wait for LS reply on whether there is impact on SMTC.

Sub-topic 1-4 SSB adaptation – Other requirements impacts

Issue 1-4-1: SCell activation requirement for SSB adaptation

Agreement:

  - No requirements when SSB adaptation of SSB periodicity happens during SCell activation.

  - FFS the requirements impact when SSB adaptation happen before and/or at the same time as SCell activation.

- Open issues

Issues captured in summary R4-2418283

- Reference

R4-2418283	Topic summary for [113][225] Netw_Energy_NR_enh_Part2