3GPP TSG-RAN4 Meeting #113	R4-2420176

# Orlando, USA, 18th Nov. - 22nd Nov.

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4968 | rev | 1 | Current version: | 17.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_redcap-Perf) CR to CORESET RMC for RLM 1Rx RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Corporation, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Perf |  |  |  |  |  | Date: |  |  | 2024-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per Table 8.1B.2.1-1 and Table 8.1B.3.1-1, Aggregation Level = 16 and Bandwidth (PRBs) = 48 for 1Rx RedCap. As per Table 8.1B.2.1-2 and Table 8.1B.3.1-2, Aggregation Level = 8 and Bandwidth (PRBs) = 48 for 1Rx RedCap. So far, there is no CORESET for RMC scheduling (CCR.x.x) with these Aggregation Level and Bandwith (PRBs) combinations. Hence, current RLM 1Rx RedCap settings in A.16.5.1.1, A.16.5.1.3, A.16.5.1.5, A.16.5.1.7, A.16.5.1.9, A.16.5.1.11, A.16.5.13, A.16.5.1.15 of CORESET for RMC scheduling (CCR.x.x) do not match expected setting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new CCR.3.1 FDD in Table A.3.1.3.1-3, CCR.4.1 TDD in Table A.3.1.3.2-4 & CCR.5.1 TDD in Table A.3.1.3.2-5 to consider Aglevel=8 with CoresetRB=48. Added new CCR.3.2 FDD in Table A.3.1.3.1-3, CCR.4.2 TDD in Table A.3.1.3.2-4 & CCR.5.2 TDD in Table A.3.1.3.2-5 to consider Aglevel=16 with CoresetRB=48. Updated Table A.16.5.1.1.1-2, Table A.16.5.1.5.1-2, Table A.16.5.1.9.1-2, Table A.16.5.1.13.1-2's dedicated Coreset RMC parameter to CCR.3.2 FDD, CCR.4.2 TDD and CCR.5.2 TDD respectively. Updated Table A.16.5.1.3.1-2, Table A.16.5.1.7.1-2, Table A.16.5.1.11.1-2, Table A.16.5.1.15.1-2's dedicated Coreset RMC parameter to CCR.3.1 FDD, CCR.4.1 TDD and CCR.5.1 TDD respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect settings might be used |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.3, A.16.5.1.1, A.16.5.1.3, A.16.5.1.5, A.16.5.1.7, A.16.5.1.9, A.16.5.1.11, A.16.5.13, A.16.5.1.15 Isolated impact analysis: No change to UE requirements, changes test parameters only. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2417654 Separately created RMC tables - Table A.3.1.3.1-3, Table A.3.1.3.2-4 and Table A.3.1.3.2-5. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

<<Start of change>>

### A.3.1.3	CORESET for RMC scheduling

#### A.3.1.3.1	FDD

Table A.3.1.3.1-1: Control Channel RMC for FDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 FDD | CCR.1.2 FDD | CCR.1.3 FDD | CCR.1.4 FDD | CCR.1.5 FDD |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 18 | 24 | 18 | 24 |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 | 1 |  |  |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 | 2 |  |  |
| monitoringSymbolsWithinSlot |  | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 0010000 0000000 |  |  |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved |  |  |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 |  |  |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A |  |  |
| Aggregation level | CCE | 4 | 2 | 8 | 4 | 4 |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

Table A.3.1.3.1-2: Void

Table A.3.1.3.1-3: Control Channel RMC for FDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.3.1 FDD | CCR.3.2 FDD |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 48 | 48 |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |
| Duration of CORESET | symbols | 2 | 2 |  |  |  |
| monitoringSymbolsWithinSlot |  | 1000000 0000000 | 1000000 0000000 |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved |  |  |  |
| Interleave n_shift |  | 0 | 0 |  |  |  |
| Interleave size |  | 2 | 2 |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A |  |  |  |
| Aggregation level | CCE | 8 | 16 |  |  |  |
| DCI formats |  | Note 1 | Note 1 |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |

#### A.3.1.3.2	TDD

Table A.3.1.3.2-1: Control Channel RMC for TDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 TDD | CCR.1.2 TDD | CCR.1.3 TDD | CCR.1.4 TDD | CCR.1.5 TDD |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 18 | 24 | 18 | 18 |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 | 1 |  |  |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 | 2 |  |  |
| monitoringSymbolsWithinSlot |  | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 0010000 0000000 |  |  |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved |  |  |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 |  |  |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A |  |  |
| Aggregation level | CCE | 4 | 2 | 8 | 4 | 4 |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

Table A.3.1.3.2-2: Control Channel RMC for TDD with SCS=30KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.2.1 TDD | CCR.2.2 TDD | CCR.2.3 TDD | CCR.2.4 TDD |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 24 | 18 | 18 |  |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 |  |  |  |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 |  |  |  |
| REG bundle size |  | 6 | 6 | 6 | 6 |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved |  |  |  |
| Interleave n_shift |  | 0 | 0 | 0 | 0 |  |  |  |
| Interleave size |  | 2 | 2 | 2 | 2 |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A |  |  |  |
| Aggregation level | CCE | 4 | 8 | 4 | 2 |  |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

Table A.3.1.3.2-3: Void

Table A.3.1.3.2-4: Control Channel RMC for TDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.4.1 TDD | CCR.4.2 TDD |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 48 | 48 |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |
| Duration of CORESET | symbols | 2 | 2 |  |  |  |
| monitoringSymbolsWithinSlot |  | 1000000 0000000 | 1000000 0000000 |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved |  |  |  |
| Interleave n_shift |  | 0 | 0 |  |  |  |
| Interleave size |  | 2 | 2 |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A |  |  |  |
| Aggregation level | CCE | 8 | 16 |  |  |  |
| DCI formats |  | Note 1 | Note 1 |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |

Table A.3.1.3.2-5: Control Channel RMC for TDD with SCS=30KHz

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.5.1 TDD | CCR.5.2 TDD |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 48 | 48 |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |
| Duration of CORESET | symbols | 2 | 2 |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved |  |  |  |
| Interleave n_shift |  | 0 | 0 |  |  |  |
| Interleave size |  | 2 | 2 |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A |  |  |  |
| Aggregation level | CCE | 8 | 16 |  |  |  |
| DCI formats |  | Note 1 | Note 1 |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |

<<Unchanged sections skipped>>

#### A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE

##### A.16.5.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the 1Rx RedCap UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell for 1Rx RedCap UE. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1B.2.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0, and purpose set to ‘rlf’. Supported test configurations are shown in table A.16.5.1.1.1-1. The test parameters are given in Tables A.16.5.1.1.1-2, A.16.5.1.1.1-3, and A.16.5.1.1.1-4 below. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.16.5.1.1 .1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 1.

Table A.16.5.1.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz |
| 4 | HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.16.5.1.1.1-2: General test parameters for FR1 out-of-sync testing in non-DRX mode for 1 Rx UE

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| BWchannel |  | Config 1, 4 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 20: NRB,c = 51 |
| DL initial BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  | Config 1, 4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1, 4 |  | CCR.3.2 FDD |
|  |  | Config 2 |  | CCR.4.2 TDD |
|  |  | Config 3 |  | CCR.5.2 TDD |
| SSB Configuration |  | Config 1, 4 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  | Config 1, 2, 4 |  | SMTC.1 |
|  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1, 2, 4 |  | Table A.3.8.2.1-1 |
|  |  | Config 3 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1, 4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1, 4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.88 |
| T3 |  |  | s | 0.88 |
| D1 |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

<<Unchanged sections skipped>>

#### A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE

##### A.16.5.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1B.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0, and purpose set to ‘rlf’. Supported test configurations are shown in table A.16.5.1.3.1-1. The test parameters are given in Tables A.16.5.1.3.1-2, and A.16.5.1.3.1-3 below. There is one cell (Cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.16.5.1.3.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms.

Table A.16.5.1.3.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz |
| 4 | HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.16.5.1.3.1-2: General test parameters for FR1 in-sync testing in non-DRX mode

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active PCell |  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  |  | 1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2,3 |  | TDD |
|  |  |  | Config 4 |  | HD-FDD |
| BWchannel |  |  | Config 1,4 | MHz | 10: NRB,c = 52 |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  |  | Config 3 |  | 20: NRB,c = 51 |
| DL initial BWP configuration |  |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| TDD Configuration |  |  | Config 1,4 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  |  | Config 1,4 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 |  | CCR.3.1 FDD |
|  |  |  | Config 2 |  | CCR.4.1 TDD |
|  |  |  | Config 3 |  | CCR.5.1 TDD |
| SSB Configuration |  |  | Config 1,4 |  | SSB.1 FR1 |
|  |  |  | Config 2 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  | Config 1, 2, 3, 4 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2, 4 |  | 15 kHz |
|  |  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  |  | Config 1, 2, 3, 4 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  |  | 0 |
| OCNG parameters |  |  |  |  | OP.1 |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x1 Low |
| In sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| DRX |  |  |  |  | OFF |
| Gap pattern ID |  |  |  |  | N.A. |
| Layer 3 filtering |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms | 1000 |
| T311 timer |  |  |  | ms | 1000 |
| N310 |  |  |  |  | 1 |
| N311 |  |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1, 4 |  |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1, 4 |  |  | TRS.1.1 FDD |
|  |  | Config 2 |  |  | TRS.1.1 TDD |
|  |  | Config 3 |  |  | TRS.1.2 TDD |
| T1 |  |  |  | s | 0.2 |
| T2 |  |  |  | s | 0.2 |
| T3 |  |  |  | s | 0.44 |
| T4 |  |  |  | s | 0.2 |
| T5 |  |  |  | s | 0.88 |
| D1 |  |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

<<Unchanged sections skipped>>

#### A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE

##### A.16.5.1.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1B.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0, and purpose set to ‘rlf’. Supported test configurations are shown in table A.16.5.1.5.1-1. The test parameters are given in Tables A.16.5.1.5.1-2, and A.16.5.1.5.1-3. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.16.5.1.5.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.16.5.1.5.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz |
| 4 | HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.16.5.1.5.1-2: General test parameters for FR1 out-of-sync testing in DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
|  |  | Config 4 |  | HD-FDD |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 20: NRB,c = 51 |
|  |  | Config 4 |  | 10: NRB,c = 52 |
| DL initial BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
|  |  | Config 4 |  | Not Applicable |
| RMSI CORESET Reference Channel |  | Config 1, 4 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1, 4 |  | CCR.3.2 FDD |
|  |  | Config 2 |  | CCR.4.2 TDD |
|  |  | Config 3 |  | CCR.5.2 TDD |
| SSB Configuration |  | Config 1, 4 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  | Config 1, 2, 4 |  | SMTC.1 |
|  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2, 4 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1, 2, 4 |  | Table A.3.8.2.1-1 |
|  |  | Config 3 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX Configuration |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1, 4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1, 4 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 1.28 |
| T3 |  |  | s | 1.28 |
| D1 |  |  | s | 1.24 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

<<Unchanged sections skipped>>

#### A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE

##### A.16.5.1.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1B.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0, and purpose set to ‘rlf’. Supported test configurations are shown in table A.16.5.1.7.1-1. The test parameters are given in Tables A.16.5.1.7.1-2, and A.16.5.1.7.1-3. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.16.5.1.7.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.16.5.1.7.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz |
| 4 | HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.16.5.1.7.1-2: General test parameters for FR1 in-sync testing in DRX mode

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active PCell |  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  |  | 1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2, 3 |  | TDD |
|  |  |  | Config 4 |  | HD-FDD |
| BWchannel |  |  | Config 1, 4 | MHz | 10: NRB,c = 52 |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  |  | Config 3 |  | 20: NRB,c = 51 |
| DL initial BWP configuration |  |  | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| TDD Configuration |  |  | Config 1, 4 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  |  | Config 1, 4 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  |  | Config 1, 4 |  | CCR.3.1 FDD |
|  |  |  | Config 2 |  | CCR.4.1 TDD |
|  |  |  | Config 3 |  | CCR.5.1 TDD |
| SSB Configuration |  |  | Config 1, 4 |  | SSB.1 FR1 |
|  |  |  | Config 2 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration |  |  | Config 1, 2, 3, 4 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2, 4 |  | 15 kHz |
|  |  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  |  | Config 1, 2, 3, 4 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  |  | 0 |
| OCNG parameters |  |  |  |  | OP.1 |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x1 Low |
| In sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| DRX Configuration |  |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  |  | N.A. |
| Layer 3 filtering |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms | 2000 |
| T311 timer |  |  |  | ms | 1000 |
| N310 |  |  |  |  | 1 |
| N311 |  |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1,4 |  |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1,4 |  |  | TRS.1.1 FDD |
|  |  | Config 2 |  |  | TRS.1.1 TDD |
|  |  | Config 3 |  |  | TRS.1.2 TDD |
| T1 |  |  |  | s | 0.2 |
| T2 |  |  |  | s | 0.2 |
| T3 |  |  |  | s | 1.24 |
| T4 |  |  |  | s | 0.2 |
| T5 |  |  |  | s | 1.88 |
| D1 |  |  |  | s | 1.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

<<Unchanged sections skipped>>

#### A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE

##### A.16.5.1.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used for 1 Rx RedCap UE. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in clause 8.1B.3.

The test parameters are given in Tables A.16.5.1.9.1-1, A.16.5.1.9.1-2, A.16.5.1.9.1-3, and A.16.5.1.9.1-3A below. There is one cell, cell 1 which is the PCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.16.5.1.9.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting of 5ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test. In the test, SSB0 is configured as the BFD-RS.

Table A.16.5.1.9.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 20 MHz bandwidth |
| 4 | HD-FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.16.5.1.9.1-2: General test parameters for FR1 PCell for CSI-RS out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
|  | Config 4 |  | HD-FDD |
| TDD Configuration | Config 1,4 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1, 4 |  | CCR.3.2 FDD |
|  | Config 2 |  | CCR.4.2 TDD |
|  | Config 3 |  | CCR.5.2 TDD |
| SSB Configuration | Config 1, 4 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration | Config 1, 2, 4 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2, 4 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1, 4 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State. 2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1, 4 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.88 |
| T3 |  | s | 0.88 |
| D1 |  | s | 0.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

<<Unchanged sections skipped>>

#### A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE

##### A.16.5.1.11.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR1 PCell CSI-RS In-sync radio link monitoring requirements in clause 8.1B.3.

The test parameters are given in Tables A.16.5.1.11.1-1, A.16.5.1.11.1-2, and A.16.5.1.11.1-3 below. There is one cells, cell 1which is the PCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.16.5.1.11.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. In the test, DRX configuration is not enabled. In the test, SSB0 is configured as the BFD-RS.

Table A.16.5.1.11.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30kHz SSB SCS, 20 MHz bandwidth |
| 4 | HD-FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.16.5.1.11.1-2: General test parameters for FR1 PCell for CSI-RS in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
|  | Config 4 |  | HD-FDD |
| TDD Configuration | Config 1, 4 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1, 4 |  | CCR.3.1 FDD |
|  | Config 2 |  | CCR.4.1 TDD |
|  | Config 3 |  | CCR.5.1 TDD |
| SSB Configuration | Config 1, 4 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration | Config 1, 2, 4 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2, 4 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1, 4 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State. 2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 2000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1,4 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 0.84 |
| T4 |  | s | 0.2 |
| T5 |  | s | 1.88 |
| T6 |  | s | 1.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

<<Unchanged sections skipped>>

#### A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE

##### A.16.5.1.13.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in clause 8.1B.3.

The test parameters are given in Tables A.16.5.1.13.1-1, A.16.5.1.13.1-2, and A.16.5.1.13.1-3 below. There is one cell, cell 1 is the PCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.16.5.1.13.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. In the test, SSB0 is configured as the BFD-RS.

Table A.16.5.1.13.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 20 MHz bandwidth |
| 4 | HD-FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.16.5.1.13.1-2: General test parameters for FR1 PCell for CSI-RS out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
|  | Config 4 |  | HD-FDD |
| TDD Configuration | Config 1, 4 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1, 4 |  | CCR.3.2 FDD |
|  | Config 2 |  | CCR.4.2 TDD |
|  | Config 3 |  | CCR.5.2 TDD |
| SSB Configuration | Config 1, 4 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration | Config 1, 2, 4 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2, 4 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1, 4 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State. 2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1, 4 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 2.48 |
| T3 |  | s | 2.48 |
| D1 |  | s | 2.44 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

<<Unchanged sections skipped>>

#### A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE

##### A.16.5.1.15.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR1 PCell CSI-RS In-sync radio link monitoring requirements in clause 8.1B.3.

The test parameters are given in Tables A.16.5.1.15.1-1, A.16.5.1.151-2, A.16.5.1.15.1-3 and A.16.5.1.15.1-3A below. There is one cells, cell 1which is the PCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.16.5.1.15.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test. In the test, SSB0 is configured as the BFD-RS.

Table A.16.5.1.15.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30kHz SSB SCS, 20 MHz bandwidth |
| 4 | HD-FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.16.5.1.15.1-2: General test parameters for FR1 PCell for CSI-RS in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
|  | Config 4 |  | HD-FDD |
| TDD Configuration | Config 1, 4 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3, 4 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3, 4 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3, 4 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3, 4 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1, 4 |  | CCR.3.1 FDD |
|  | Config 2 |  | CCR.4.1 TDD |
|  | Config 3 |  | CCR.5.1 TDD |
| SSB Configuration | Config 1, 4 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.1 RedCap FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2, 4 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1, 4 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State. 2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x1 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 16 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 4000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1, 4 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 2.44 |
| T4 |  | s | 0.2 |
| T5 |  | s | 1.88 |
| T6 |  | s | 1.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

<<End of change >>