3GPP TSG-RAN WG4 Meeting # 113	R4-2420245

# Orlando , US, Nov 18 – 22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4995 | rev | 2 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.133 on core maintenance for R18 NR mobility enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Some numbering of clauses need to be revised. - The order of paragraphs needs to be adjusted. - Some typos need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following for core requirements of R18 NR mobility enhancements should be further revised: - Revise the numbering of clause from ‘8.9A.2.1’ to ‘8.9C.2.1’ in clause 8.9C.2. - Adjust the order of paragraphs in clause 8.9C.2 to align with clause 8.11E.2. - Correct some format issues. - Correct some typos in clause 8.11E.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The related requirements for further NR mobility enhancements would still be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9C, 8.11E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.9C	Subsequent Conditional PSCell Addition Delay

8.9C.1	Introduction

This clause defines requirements for the delay within which the UE shall be able to perform subsequent conditional PSCell addition in NR-DC. The requirements in this clause are applicable to NR-DC.

8.9C.2	Subsequent Conditional PSCell Addition Delay Requirement

The requirements in this clause shall apply for the UE configured with only PCell in FR1.

Upon receiving RRC Command containing MR-DCRelease configuration and sending an RRCReconfigurationComplete message confirming the release in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell no later than in subframe n + Tconfig_PSCell_Addition_Conditional:

Where:

-		Tconfig_PSCell_Addition_Conditional = TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

-	TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationComplete message confirming the release of SCG configuration until a condition exists at the measurement reference point which will trigger the conditional PSCell addition.

-	Tmeasure is the measurements time stated in clause 8.9C.2.1.

-	TUE_preparation is the UE preparation time for conditional PSCell addition, and starts after UE realizes the condition of PSCell addition is met and identity of the PSCell is determined. TUE_preparation is up to 10 ms.

-	Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20 ms when PSCell is in FR1, and Tprocessing = 40 ms when PSCell is in FR2.

-	T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms.

- Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

-	TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

The PCell interruption specified in clause 8.2 is allowed only after the UE starts to execute a conditional PSCell addition.

8.9C.2.1	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a PSCell addition and interruption time starts.

The measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index defined in clause 9.3.4. When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_inter_with_index for PSCell addition. If a cell, which has been detectable at least for the time period Tidentify_inter_with_index for PSCell addition, becomes undetectable for a period and then the cell becomes detectable again and triggers a PSCell addition, the measurement time delay shall be less than TSSB_measurement_period_inter provided the timing to that cell has not changed more than  3200/2µ Tc while the measurement gap has not been available and the L3 filter has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

8.11E	Subsequent Conditional PSCell Change

8.11E.1	Introduction

This clause defines requirements for the delay within which the UE shall be able to perform subsequent conditional PSCell change in NR-DC. The requirements in this clause are applicable to NR-DC.

8.11E.2	Subsequent Conditional PSCell Change delay

The requirements in this clause shall apply for the UE configured with only PCell in FR1.

The UE shall be capable to transmit PRACH preamble towards the new target PSCell no later than in slot n + Tconfig_PSCell_Subsequent_Change_Conditional:

Where:

-	Slot n is the time when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change.

-	Tconfig_PSCell_Subsequent_Change_Conditional = TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

-	TEvent_DU is the delay uncertainty which is the time from when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change until a condition exists at the measurement reference point which will trigger the subsequent conditional PSCell change.

-	Tmeasure is the measurements time stated in clause 8.11E.2.1.

-	TUE_preparation is the UE preparation time for subsequent conditional PSCell change, and starts after UE realizes the condition of PSCell change is met and identity of new PSCell is determined. TUE_preparation is up to 10ms.

-	Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20 ms when source and target cells are in the same FR, and Tprocessing = 40 ms when source and target cells are in different FRs.

-	T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms.

- Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

-	TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

The PCell interruption specified in clause 8.2 is allowed only after the UE starts to execute a subsequent conditional PSCell change.

8.11E.2.1	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a PSCell change to a target cell and interruption time starts.

For intra-frequency PSCell change, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_intra_with_index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

For inter-frequency PSCell change, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_without_index or Tidentify_inter_with_index defined in clause 9.3.4. When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency PSCell change or the time period Tidentify_inter_without_index or Tidentify_inter_with_index for inter-frequency PSCell change. If a cell, which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency PSCell change or the time period Tidentify_inter_without_index or Tidentify_inter_with_index for inter-frequency PSCell change, becomes undetectable for a period and then the cell becomes detectable again and triggers a PSCell change, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200/2µ Tc while the measurement gap has not been available and the L3 filter has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

<End of Change 1>