3GPP TSG-RAN WG4 Meeting # 113					R4-2420366

Orlando, US, 18th – 22nd November, 2024

Source:		LG Electronics, OPPO, Ericsson, Huawei, HiSilicon, vivo

Title:			Draft TP to TR 38.769 for A-IoT intermediate node RX requirements

Agenda item:		7.21.3.3

Document for:		Approval

Introduction

In this paper, we present our thoughts on Intermediate node(Rx) for A-IoT SI.

- References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA, RAN #103, Mar. ,2024

- R4-2417088, WF on impacts of A-IoT on RF requirements, Moderator (Huawei), RAN4#112bis, Oct.,2024

- TP for TR 38.769

<Start of Text Proposal>

- 6.7.3	Intermediate node (UE)

- 6.7.3.2 TX

- 6.7.3.3 RX

### 6.7.3.2.1 Reference sensitivity

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the intermediate UE antenna ports at certain performance metric. Distinguished from the legacy NR UE, Ambient IoT intermediate UE may suffer from residual noise from CW interference, So it should be considered when defining receiver sensitivity. Reference sensitivity requirement for intermediate UE could be derived based on the following equation:

PREFSENS(dBm)=-174dBm+10*log10BW+NF+IM+ICW+SNR

Where:

-	BW is the maximum transmission bandwidth

-	NF is noise figure of intermediate node

-	IM is the implementation margin, equal to 2dB.

-	For inside topology, Icw is IoT due the residual CW interference, For outside topology, Icw would be less and in this case, receiver to decode the backscattered signal without desensitization of receiver caused by CW leakage. value will be defined in the WI.

-	SNR is the SNR value for which we reach 10% BLER target. Each company provided simulation results, and average will be done for each BW.

-	The REFSENS requirements of an intermediate UE may be affected when it simultaneously transmits uu UL and A-IoT signals.

For inside topology, the receiver needs to tolerate the high leakage CW power from the transmitter when intermediate node can transmit CW signal and desensitization of receiver may be allowed with a CW cancellation function enabled in such case. For outside topology, the received CW power would be less and in this case, receiver to decode the backscattered signal without desensitization of receiver caused by CW leakage. Therefore, for REFSENS requirements, different requirements can be defined, corresponding to inside topology and outside topology respectively.

### 6.7.3.2.2 Maximum input power

Maximum input level for legacy NR UE is defined as the maximum mean power received at the UE antenna port. Considering the intermediate UE may need to implement the receiver at UL spectrum when CW/D2R are also located in UL, which is different from legacy NR UE, this requirement can be further discussed in WI phase.

### 6.7.3.2.3 Adjacent channel selectivity

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the center frequency of the assigned channel.

### ACS requirement and the interference level depend on co-existence study.

### 6.7.3.2.4 Blocking characteristics

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit.

In-band blocking requirement and the interference level depend on co-existence study.

The existing out-of-band blocking requirement for legacy NR UE is applicable for intermediate UE.

Narrow band blocking can be discussed in WI phase.

### 6.7.3.2.5 Spurious response

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained. The existing spurious emission requirement for legacy NR UE is applicable for intermediate UE.

### 6.7.3.2.6 Receiver intermodulation

Intermodulation response rejection is a measure of the receiver's capability to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals with a specific frequency relationship to the wanted signal. It depends on the RAN1 design. If multiple-tone carrier wave is supported, intermodulation requirements potentially needed.

### 6.7.3.2.7 Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector. The legacy NR UE receiver spurious emission requirement could be applicable.

<End of Text Proposal>