3GPP TSG-RAN WG4 Meeting # 113												R4-2420369

Orlando, US, 18th – 22nd November, 2024

Source: 	OPPO

Title: 	TP on the device 2b RF requirement

Agenda Item:	7.21.3.2

Document for:	Approval

- Introduction

In the previous meeting, the RF requirements for the device 2b were discussed, and this contribution summarize the conclusion so far.

- Text proposal

## <<Start of Change>>

6.7.4.3	Device 2b

6.7.4.3.1 TX Requirement

6.7.4.3.1.1 Maximum output power

In the transmission direction (D2R), for device 2b, as it can generate the signal by itself, the device function can be more similar to legacy IoT devices than backscattering-type devices. Therefore, its maximum output power and the corresponding power class can also be specified similarly to legacy IoT devices,

However, due to the nature of energy harvesting, its transmission and reception duty cycle may be limited by the availability of the harvested energy. Moreover, the power constraint on the AIoT devices may further impact the choice of components, e.g., oscillator, amplifier, etc.; thus, a significantly lower performance/capability can be expected.

6.7.4.3.1.2 Output power dynamics

-Transmit OFF power

As device type 2b, assuming the device is fully charged, it can be foreseen that it will work similarly to legacy IoT devices. Therefore, the transmit OFF power should be potentially specified for type 2b to ensure the power level when the RF part is turned off will not exceed a certain range.

- Transmit ON/OFF time mask

The time mask requirement is to test the time period between the RF part turned on and off. For A-IoT devices, this time period is not a crucial requirement and hence it is potentially not needed.

- Minimum output power

As device type 2b, assuming the device is fully charged, it can be foreseen that it will work similarly to legacy IoT devices. Therefore, the minimum output power should be potentially specified for type 2b to ensure the lower limit of the output power for coverage.

- Power control

As device type 2b, assuming the device is fully charged, it can be foreseen that it will work similarly to legacy IoT devices. Therefore, the power control should be potentially specified for type 2b to ensure the output power range.

6.7.4.3.1.3 Transmit signal quality

- Frequency error

As device type 2b, assuming the device is fully charged, it can be foreseen that it will work similarly to legacy IoT devices and the LO is assumed to be used. Hence the frequency error is potentially required.

6.7.4.3.1.4 Output RF spectrum emissions

- Occupied bandwidth

The transmission bandwidth configuration defines the bandwidth of the R2D/D2R signal in baseband. For the R2D channel, one or multiple PRBs are under consideration in RAN1. For D2R channel, the transmission bandwidth of 15kHz or more is being discussed.

6.7.4.3.2 RX Requirement

6.7.4.3.2.1 Reference sensitivity

The receiver sensitivity is to test the receiver capability to certain performance metrics in a given lowest RX power level. For device 2b, this requirement will be potentially left to be further defined in WI phase to consider the RF architecture and the exact RF component capability.

6.7.4.3.2.2 Maximum input power

In NR, the maximum input power is to guarantee the UE can still work normally when it is close to BS. Similarly, this requirement is still potentially needed for AIoT device to ensure the device have reasonable dynamic range.

6.7.4.3.2.3 ACS

The ACS requirements shall be derived based on co-existence simulation, and guard band/guard RB may also be defined. As the BB LPF is proposed, the ACS requirement is potentially supposed to be essential to verify its RF performance

It’s expected that the AIoT system will be deployed within an NR channel from the same operator. The AIoT device should be able to receive in the presence of a wideband NR interferer. Additionally, the device might also suffer from CW interference from a nearby reader/CW node, if it’s not synchronised with the serving reader.

6.7.4.3.2.4 ASCS

The ASCS requirement is supposed to define the selectivity of the in-channel receiver. However as the channel bandwidth of the device is limited to only 1RB, the ASCS is potentially not needed.

6.7.4.3.2.5 Spurious response

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained. In the A-IoT deployment, the CW signal is set at a certain frequency for energy harvesting and reflection. Hence the scenario for this test is potentially not needed.

## <<End of Change>>

- References