3GPP RAN WG4 Meeting #113		R4-2420371

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	TP for CW interference cancelation

Agenda Item:	7.21.3.4

Document for:	Approval

1 Introduction

The new SID on solutions for Ambient IoT was approved at the RAN#102 meeting [1] and later revised [2] at the RAN#103 meeting. The RAN4-led objectives are copied as below:

- RAN4-led:

  - Coexistence study of Ambient IoT and NR/LTE.

  - RF requirements study for Ambient IoT:

    - Ambient IoT BS transmission and reception

    - Ambient IoT Device, as per the General Scope, transmission and reception

    - Intermediate node (UE), as per the General Scope, transmission and reception

In this contribution, we provide TP for CW interference cancelation, on top of endorsed TP R4-2417195.

2	Conclusion

This contribution provides Text Proposals on Ambient CW cancellation.

Proposal 1: it is proposed to endorse the attached TP for TR 38.769 clause 6.7.5.

References

[1] RP-240829, New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, RAN4 Chair (Huawei)

[2] R4-2410597, WF on impacts of A-IoT on RF requirements, Huawei

[3] R4-2417195, TP for CW interference cancelation, Huawei, HiSilicon, Ericsson

TP for TR 38.769

6.7.5	CW interference cancelation study

6.7.5.1	General

The CW cancellation can be divided into 3 stages sequentially: (1) Spatial isolation for separate TX/RX antenna configuration; (2) Analog cancellation from antenna to ADC and (3) Digital cancellation at baseband.

Spatial isolation for separate TX/RX antenna configuration

For separate TX/RX antenna configuration, the spatial isolation can be achieved by antenna isolation, or coupling loss between CW transmitter and D2R receiver.

For example, if the CW transmission source is separately deployed from the D2R receiver, the spatial isolation relies on the distance and antenna sidelobe suppression between the CW node and A-IOT node.

Analog cancellation

Interference cancellation in the RF front-end can be applied to reduce the desensitization of the receiver. In principle, it can be implemented by reconstructing a local CW with the same amplitude but opposite phase of a CW interferer received from antenna at the input of the LNA, and combine this cancellation signal with the received signal. The cancellation performance depends on the phase and amplitude error between the reconstructed and actual received CW interference.

Additionally in the case of integrated carrier-wave transmitter and D2R receiver in sharing antenna case, the interference suppression could be provided by a circulator/directional coupler.

Digital cancellation

In the digital baseband, suppression can be achieved by filtering, as the received single-tone carrier-wave is shifted to direct current (DC) at baseband.  Digital interference cancellation can also be achieved by using a low complexity self-interference active cancellation technique.

6.7.5.2	Interference cancelation analysis

##### 6.7.5.2.1	Ericsson

The min received power could be -74 dBm or lower, the signal with such low level received power should be decoded in the presence of the CW transmission which is either 33 dBm or 23 dBm for monostatic antenna and reference at antenna. The CW transmission can leak to the receiver chain via either reflection from environment or the imperfection of the isolation between transmitter and receiver when CW is transmitted by A-IoT BS or UE as reader. As the receiver has limited dynamic range and linearity, this CW leakage needs to be attenuated/isolated to mitigate the desensitization of the receiver.  The CW signal frequency is used to modulate the backscatter signal and the frequency separation of the CW signal and modulated signal is around 100kHz for example as illustrated in Figure 6.7.5.2.1-1 according to the current D2R waveform design. Using an RF filter before the LNA to attenuate the CW signal is not possible. As such, a high-level CW leakage signal will be present together with a low wanted backscattered signal before LNA.

Figure 6.7.5.2.1-1: D2R waveform in frequency domain

To analysis the impact of the receiver desensitization, the backscattered signal is modelled as below [2]:

Sb (t)=Abcos⁡(ωb∙t+φb)∙cosωc∙t+φc

with ωc representing the CW frequency and ωb is the message frequency.

And the leakage CW signal is modelled as below:

SI (t)=AI∙cosωc∙t+φI

Then the receiver nonlinearity is modelled with a power series below assuming the second-order nonlinear product can be filtered:

yot=a1∙xt+a3∙xt3

Where a1 is the small signal gain of the receiver and a3<0 represents the nonlinear behaviour of the system.

The input signal before LNA is then:

xt=Sb t+SI (t)

Assuming the φb=φc=φI=0, at output of the LNA, only the terms associated with fundamental frequency coswbtcos⁡(ωct) are of interest and they are below:

yoIt=a1Ab+916a3Ab3+94a3AbAI2cosωb∙t∙cosωc∙t

As the CW signal is much larger than backscattered signal, the term 916∙a3∙Ab3 can be dropped and further simplified with:

yoIt=a1Ab+94a3AbAI2cosωb∙t∙cosωc∙t

The voltage of the fundamental signal at output of LNA can be reformulated as:

vt=a1∙Ab∙1+94∙a3a1∙AI2

Since a3<0, we can write:

vt=a1∙Ab∙1-94∙a3a1∙AI2

It can be observed from the above that the wanted signal level is degraded with scaling factor which directly reduces the SNR, therefore the desensitization factor is:

SNRD=20log101-94a3a1∙AI2

Further the IIP3 of LNA can be expressed with:

AIIP3_LNA2=|a1||a3|∙43

Therefore, the SNR degradation can be formulated below:

SNRD=20log101-3∙AI2AIIP3LNA2=20log101-3∙PcwleakgePIIP3LNA

Assuming CW signal is transmitted by A-IoT BS or UE as reader and monostatic antenna configuration, with the return loss of 15 dB, the CW leakage power will be 18 dBm. If the SNR degradation is allowed with 3 dB, the IIP3 of LNA should be 28 dBm. Such high IIP3 is demanding for design. For other combination value for SNR degradation and CW leakage power, it is illustrated in Figure 6.7.5.2.1-2 and the IIP3 value is listed in Table 6.7.5.2.1-1. Referring to TR 38.858, the reported IIP3 for WA BS is from the range of 10 dBm to -27.6 dBm, this in turn demands some CW cancellation capability before LNA.

Table 6.7.5.2.1-1:  IIP3 of LNA with different SNR degradation and CW leakage power

|  |  | SNR degradation (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | -1 | -3 | -10 |
| CW leakage power (dBm) | 18 | 32 | 28 | 24 |
|  | 8 | 22 | 18 | 14 |
|  | 0 | 14 | 10 | 6 |
|  | -10 | 4 | 0 | -4 |

Figure 6.7.5.2.1-2: IIP3 of LNA with different SNR degradation and CW leakage power

For CW cancellation, there are passive or active cancellation schemes existing for RFID reader front-end. In both schemes, a copy of the CW leakage signal is created with same amplitude but opposite phase. The cancellation scheme is illustrated in Figure 6.7.5.2.1-3. For the passive cancellation scheme, the cancellation path needs to be calibrated and for active cancellation scheme, the cancellation path can be tuned to adapt to the leakage CW signal. However, the cancellation is limited by either the hardware imperfection or the detecting signal amplitude/phase accuracy. In [6], it is mentioned that for 40 dB CW cancellation, the amplitude error should be 0.09 dB and phase error to be 0.5 degree. This is quite demanding in design considering the current RRM measurement accuracy for RSRP is +/-2 dB in normal condition.

When the CW and A-IoT signal are transmitted from different hardware, the active CW cancellation scheme should be used in this case. This applies to the case where the CW node is co-located with A-IoT BS for D1T1-A2 or D2T2-A2.

To enable the CW cancellation function, reader needs some time to train the circuit to generate the reverse CW leakage with the same amplitude and the opposite phase. The cancellation training time is implementation specific and also relates to the targeted cancellation performance. Considering this time and also the transient time to transmit the CW signal, the timing relation between the R2D, CW and D2R can be illustrated in Figure 6.7.5.1-4.

Figure 6.7.5.1-3: CW cancellation scheme

Figure 6.7.5.1-4: Transient time for CW and R2D transmission and CW cancellation training time before D2R reception

##### 6.7.5.2.2	Qualcomm

###### 6.7.5.2.2.1	Feasible values CW cancellation from implementation impact

In principle, the opposite phased CW can be injected to the receiver and then two signal will cancel each other out. The problem is how to know the phase of the CW in the receiver and how to adjust the cancelling signal perfectly. This is especially difficult in the intermediate node since it has smaller dimensions and shared antennas. The circuitry and relevant aspects are shown in Figure 6.7.5.2.2.1-1.

Figure 6.7.5.2.2.1-1. CW cancellation in an Intermediate node

From Figure 6.7.5.2.2.1-1 we can see that CW has many leakage paths and conclude that the cancellation circuitry has to cancel very high-level CW compared to the backscattered signal level.

RAN4 has not discussed techniques for active cancellation but it should be noted that the processing accuracy in a handset receiver typically results in the order of 1.5-4 % Rx EVM [R4-1706112]. This is the level Intermediate node baseband can provide information to the phase and amplitude cancellation circuits for the analogue cancellation. In the LS [R4-1706112], the typical UE Rx EVM is 2-4 % for a UE that supports 1024QAM. UE fulfilling minimum requirements and not support 1024QAM would have even smaller minimum implied assumption for Rx EVM. The analogue domain phase and amplitude controller also have finite resolution and 1% accuracy would mean 1.8 degrees phase step assuming signal can be inverted separately. This level of granularity maybe challenging in practice.  Intermediate node CW cancellation circuitry processing accuracy is in the order of 1.5-4 %.

Translating this to CW cancellation in dB, we need to split this in to phase accuracy and amplitude accuracy. With a simple mathematical analysis, if we assume that waveform with an amplitude of Y is cancelled with:

Cancellation waveform = Y*(1-amplitude_error)*sin(wt+phase_error)

And calculate the resulting CW as

Z= Y - Cancellation waveform,

We can get cancellation value of Z compared to Y as in Figure 3 below. The EVM % is calculated from the amplitude_error and phase_error.

Figure 6.7.5.2.2.1-2. Cancellation as a function of EVM in the cancellation circuitry.

The curve in Figure 6.7.5.2.2.1-2 is not explicitly defined since the contribution of phase error to the EVM % is not same as its contribution to the cancellation. In other words, cancellation is more sensitive to the amplitude error than it is for phase error. In Figure 6.7.5.2.2.1-3 we show the dependency as surface plot for reference.

Figure 6.7.5.2.2.1-3. Cancellation sensitivity as a function of amplitude and phase error

We can see that the ampltude error and phase have different impact to cancellation and Intermediate node realistic analogue domain CW cancellation capability is at most 50 dB and more likely typical value is closer to less than 40 dB.

Coming back to the incoming signal characteristics, this means that the UE can cope with the CW of the level:

CW_to_signal= Analogue CW cancellation + Digital domain cancellation.

The digital domain cancellation is same process as DC offset compensation. Normally, the DC offset to be compensated is small since it is a result of non-perfections in the circuitry. In this case, the CW is much larger than the receiver modulated signal so the DC offset cancellation circuitry needs some tuning to work in this new situation. We can assume here that the digital domain cancellation can cope at least with the CW to signal ratio where LNA remain linear enough not to distort the signal.

###### 6.7.5.2.2.2	Needed CW cancellation when CW from reader

In the first phase, to make the analysis a bit more concrete, we can assume that current handheld UE requirements are taken as baseline. Here we consider the maximum input level of -25 dBm and further assume that digital path dynamic range is defined by SNR for 256 QAM and ACS resulting in to 68 dB (12 bits) but in practice some more dynamic range is needed to accommodate some range for the digital AGC in practice. The output power is the one used in co-ex work, 23 dBm, and antenna coupling is 10 dB thus same what is used in earlier and in all other work such as MSD work in RAN4 for handhelds. Additionally, we can assume that the needed D2R signal SNR for successful decoding is 10 dB.

Table 6.7.5.2.2.2-1. Assumptions for the CW cancellation for intermediate node

| Parameter | Value | Note |
| --- | --- | --- |
| Max input level | -25 dBm |  |
| Output power of CW | 23 dBm |  |
| Antenna coupling | -10 dB | Also reflection loss |
| Digital path dynamic range | 12 bit (14 bit) | 68 dB (74 dB) |
| SN(I)R for D2R signal | 10 dB | At receiver |

With the above numbers, we can calculate that the maximum CW cancellation in digital domain is 68 – 10 = 58 dB. This will depend on number of bits used in the ACD, and for effective 14 bit ADC it would be 74 dB.

Reflected power from the antenna is 10 dB lower than the transmitted power, this means LNA without analogue domain CW cancellation will see 13 dBm CW but since the maximum input level is -25 dBm, the analogue domain cancellation has to be 38 dB for 23 dB output power and to ensure LNA max input level is not exceeded. Even if handheld device uses different antennas, the coupling can be 10 dB and the result is the same conclusion. Analogue domain CW cancellation must be 38 dB for 23 dBm output power and -25 dBm maximum input level for the receiver.

The analysis in previous section we discussed that this is feasible from the control point of view. With 38 dB in analogue domain, the total CW cancellation maximum is 112 dB.

Looking more closely, the range of the reader can be calculated from the above numbers following the logic:

Received D2R power = Max CW power – 2x pathloss – backscattering loss

Minimum received power at lowest SNR = max input level + SNRD2R – DNRADC

When me make both equations equal:

Max CW power – 2x pathloss – backscattering loss >= max input level + SNRD2R – DNRADC

Pathloss =< (-max input level - SNRD2R + DNRADC + Max CW power - backscattering loss)/2

In numbers, assuming 10 dB backscattering loss:

Pathloss =< (-(-25 dBm) – 10 dB + 84 dB + 23 dBm - 10 dB)/2 = 56 dB, which is about 14 meters at 1 GHz with 14 bit ADC.

And this is assuming the analogue domain CW cancellation is as above.

To understand the CW cancellation role to the range, we can first look at the analogue domain cancellation. If the cancellation is less than 38 dB but the maximum input level can not be exceeded, the output power has to be reduced  and this reduces the range accordingly. If the CW cancellation is better than the 38 dB, the output power can be increased and this increases the range.

Looking at the digital domain cancellation, firstly we state that the digital domain can cancellation can be very large if a long training period is allowed. When intermediate node is the reader, it can choose when to turn CW or and train its own receiver thus time can be long.

For a fixed output power, the digital domain dynamic range defines the range since more bits there are available, more bits can be used for the CW cancellation with 2 effective bits needed to cover 10 dB SNR needed for the successful decoding, only the training time may need to be increased. However, this analysis did not look in to the noise contribution. With the above values, the received D2R signal power is close to -100 dBm. Without knowing what is the bandwidth of the D2R signal, it is difficult to say exactly what is the noise contribution but at -100 dBm level and e.g. 200 kHz BW when kTB = -120 dBm, there will be some noise contribution to the SINR.

###### 6.7.5.2.2.3	Needed CW cancellation when “CW from outside of the topology”

Case is different when CW comes from outside the topology.  The level of the CW at the receiver is lower compared to the D2R signal level but the CW phase noise plays a role in the range since the down conversion will be done with a different signal source with non-correlating phase noise.

Figure 6.7.5.2.2.3-1. CW not in the reader “CW outside topology”

The way the CW nodes are distributed in the aged topologies result almost constant -39 dBm received CW in the reader. The assumption is to always use closest CW source. Looking at the statistical D2R received power distribution in Figure 2, for 90% of the connections see power higher than -101 dBm for D1T1 and -98 dBm for D2T2.

Figure 6.7.5.2.2.3-2. D2R received power distribution in the two topologies. CW level is almost constant at the receiver

Calculating the ratios yields that CW to D2R signal levels for 90 % probability are 62 and 62 dBc in D1T1 and D2T2 when CW comes outside the topology.

In this topology, it is difficult to relate the CW cancellation and range since it depends on how the CW source and reader are positioned. From statistical data in Figure we can see that the cancelation needs to be much lower level than when CW source is in the reader and up to the point that maybe analogue domain cancellation is not needed.

###### 6.7.5.2.2.4	Phase noise impact when CW is outside topology

If the reader will be using a different CW to down convert the D2R signal than the CW used in the backscattering, the phase noise of the incoming CW plays a critical role in system performance.

From the same conclusion, that the CW to D2R power ratio at the reader is approximately 62 dBc, we can conclude that the integrated phase noise over the D2R bandwidth can not be bigger than -72 dBc, to allow for 10 dB SNIR. However, this assumes phase noise is the only noise or interference source and more comfortable specification could be -80 dBc below the CW. Integrated phase noise for the CW coming outside the topology over the D2R signal BW (Btx,D2R  ) must be better than -80 dBc.

We ran system simulation and impact to SNR because of multiple phase noise values and in Figure 6.7.5.2.2.4-1 present the results. Here we chose 15 kHz D2R bandwidth and assumed it is offset from the CW sufficiently to avoid increase in integrated noise because of high flicker noise close to the CW. In that way, the simulated results show most optimised case. If the bandwidth I large, the integrated noise is also larger.

Figure 6.7.5.2.2.4-1 Impact of phase noise to SINR.

Typically the PLLs and oscillators are designed to meet wide BWs and in handheld device also settling time needs to be accounted for and they may not be optimised for close in noise where backscattering signals in A-IoT case are assumed to be positioned with relatively narrow bandwidth.

In practical design, the phase noise is a result of many components, such as VCO, PLL loop and 1/f noise. In Figure 6.7.5.2.2.4-2 these are summarize and the resulting phase noise is the darker red curve.

Figure 6.7.5.2.2.4-2. Example of a phase noise characterises and contributing sources (from https://www.analog.com/en/resources/analog-dialogue/articles/phase-locked-loop-pll-fundamentals.html)

From Figure 6.7.5.2.2.4-2 we can see that -80 dBc integrated phase noise over 100 kHz specification may require dedicated high-performance design.

RAN4 then should specify this for the component which provides the phase noise to the system but this issue should be firstly understood and documented in the study item and then what kind of specification will be written then discussed in work item.

Understanding phase noise of the CW source may also be important for the radio system design. RFID uses an offset from CW in the D2R link and this helps with the phase noise since as we can observe from Figure 3, offsetting the placement of Btx,D2R  further away might be useful in relaxing the CW source phase noise requirements.

Placing Btx,D2R  further away from CW in frequency is beneficial for the system throughput and range.

6.7.5.2.3	Huawei

2 aspects are evaluated: (1) Blocking of D2R receiver; (2) Carrier-wave interference cancellation

Blocking of D2R receiver

If the carrier-wave source is separately deployed from the D2R receiver (e.g., D1T1-A1 and D1T1-B scenarios), the spatial isolation relies on the distance between the CW node and A-IOT node. For example, the path loss is 57 dB for an isolation distance of 20 meters, assuming free-space propagation. Normally the CW node and A-IOT node deploy at the same height, the antenna gain is about -5 dB at 90°offset from the boresight for both TX antenna and RX antenna. For D1T1-A1 and D1T1-B scenarios the received power at the antenna connector is 23-57-10= -44 dBm and 23-51-10= -38 dBm for 20 meters and 10 meters distance respectively, which is close to the narrowband blocking requirements for small cell NR base station. Hence there is no blocking issue for these scenarios.

In the case of integrated carrier-wave transmitter and D2R receiver in D1T1-A2, analog interference cancellation in the RF front-end can be applied when the interference power exceeds the blocking threshold of receiver. The performance heavily depends on the similarity between the reconstructed and actual received interference. A suppression of 10-30 dB can usually be expected for RF interference cancellation. Assuming the same reference clock between the carrier-wave transmitter and D2R receiver (e.g., in D1T1-A1/A2 scenario, or D1T1-B scenario with wired connection between the base station and the carrier-wave node), higher interference suppression can be reached, such as 20 dB or higher.

Overall carrier-wave interference

In the case of separated carrier-wave transmitter and D2R receiver in D1T1-A1 or D1T1-B, through spatial isolation, RF interference cancellation and high-pass filtering, the overall suppression to the carrier-wave interference can reach about (61+10+80) = 151 dB at 10 meters distance. For larger spatial isolation corresponding to longer distance between the carrier-wave transmitter and D2R receiver (e.g., ~60 dB @ 25 m), the carrier-wave interference can be even higher (e.g., 70+10+80 = 160 dB).

In the case of integrated carrier-wave transmitter and D2R receiver in D1T1-A2, with ~40 dB interference suppression provided by a circulator/directional coupler and/or the antenna with high isolation between Tx and Rx, the carrier-wave interference suppression can reach about (40+20+80) = 140 dB.

6.7.5.2.4	vivo

Spatial domain

For A1/B scenario, the receiver and transmitter are separated, which means spatial domain isolation for the CW interference can be obtained through this deployment. The amount of spatial isolation can be achieved by pathloss isolation.

For the D1T1-A1/B scenario, the pathloss isolation can reach 61 dB, which assumes the distance between CW emitter and receiver is 20 meters for InH-DH-NLOS path loss model. Meanwhile, for the D2T2-A1/B scenario, the pathloss isolation is 54 dB when the CW emitter and receiver distance is 10 meters and the pathloss model is InH-Office-NLOS.

For A2 scenario, the D2R receiver and CW transmitter are located at same node, so the pathloss can’t provide suppression in this case, and the spatial domain suppression capability mainly from Tx/Rx antenna isolation. For D1T1 we assume the antenna isolation is 15 dB, and for D2T2 we assume 10 dB which is a typical value for NR UE.

The suppression capability in spatial domain is summarized below:

| Scenario | Part1 Spatial domain suppression |
| --- | --- |
| D1T1-A1/B | 61 dB |
| D1T1-A2 | 15 dB |
| D2T2-A1/B | 54 dB |
| D2T2-A2 | 10 dB |

Analog domain

This part is the suppression before the LNA. A typical self-interference cancellation architecture is show in Figure 6.7.5.x-1

Figure 6.7.5.x-1 A typical self-interference cancellation architecture [1]

The basic rationale is to generate a CW that have -180° phase difference compared to the incoming interference and combine them together. According to [1][2], the RF-IC suppression capability between 30dB~ 60dB can be achieved generally, and we assume 45dB here.

For A1/B scenario, since the CW transmitter is separated from D2R receiver, the method above actually hard to be used at receiver side, so we assume 0 dB suppression capability as worst case.

The suppression capability in analog domain is summarized below:

| Scenario | Part2 Analog domain suppression |
| --- | --- |
| D1T1-A1/B | 0 dB |
| D1T1-A2 | 45 dB |
| D2T2-A1/B | 0 dB |
| D2T2-A2 | 45 dB |

IM3 product

After LNA, the CW and D2R will generate intermodulation product, and the IM3 product will overlap with D2R signal in frequency domain, and the BB filter actually cannot help on this. The traditional IM3 calculation procedure is described in the table below

Table 6.7.5.x-1 The model of CW total cancellation capability

| System configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0A | Scenarios | D2T2-A2 | D2T2-B | D2T2-A2 | D2T2-B | D1T1-A2 | D1T1-B | D1T1-A2 | D1T1-B |
|  |  | InH-LOS | InH-LOS | InH-LOS | InH-LOS | InF-DH-NLOS | InF-DH-NLOS | InF-DH-NLOS | InF-DH-NLOS |
| 0A1 | CW case |  |  |  |  |  |  |  |  |
| 0B | Device type | Device1 |  | Device2a |  | Device1 |  | Device2a |  |
| 0C | Center frequency (GHz) | 0.9 |  |  |  | 0.9 |  |  |  |
| Transmitter |  |  |  |  |  |  |  |  |  |
| 1E | CW Tx power(dBm) | 23 |  |  |  | 33 |  |  |  |
| 1F | D2R bandwidth (Hz) | 160000 |  |  |  | 160000 |  |  |  |
| 1G | Tx antenna gain(dBi) | 0 |  |  |  | 2 |  |  |  |
| 1N | Cable/connector loss | 1 |  |  |  | 3 |  |  |  |
| Receiver |  |  |  |  |  |  |  |  |  |
| 2B | bandwidth used for the evaluated channel (Hz) | 160000 |  |  |  | 160000 |  |  |  |
| 2C | Receiver antenna gain(dBi) | 0 |  |  |  | 2 |  |  |  |
| 2D | Receiver Nosie Figure(dB) | 7 |  |  |  | 5 |  |  |  |
| 2E | Thermal Noise(dBm/Hz) | -174 |  |  |  | -174 |  |  |  |
| 2F | Noise Power(dBm) | -114.9588002 |  |  |  | -116.9588002 |  |  |  |
| 2G | Tx/Rx spatial isolation capability | 10 | 0 | 10 | 0 | 15 | 0 | 15 | 0 |
| 2F | RX cable, connector loss | 1 |  |  |  | 3 |  |  |  |
| 2J | Rx RF-IC suppression capability; For outside topology, this item is pathloss suppression interference | 45 | 54.46 | 45 | 54.46 | 45 | 61.18 | 45 | 61.18 |
|  | CW cancellation (dB) | 55 | 54.46 | 55 | 54.46 | 60 | 61.18 | 60 | 61.18 |  |
| 2K1 | Remaining CW interference(dB) | -34 | -33.46 | -34 | -33.46 | -29 | -30.18 | -29 | -30.18 |  |
| 2K2 | D2R power (dBm), for D1T1 scenario, assuming 10m distance between reader and tag; for D2T2 scenario, assuming 5m distance between reader and tag | -72.15 | -72.15 | -62.15 | -62.15 | -88.17 | -88.17 | -78.17 | -78.17 |  |
| 2K3 | Rx IIP3 capability(dBm) | -6 | -6 | -6 | -6 | -10 | -10 | -10 | -10 |  |
| 2K4_1 | Rx IM3#1 = 2K1+2*2K2-2*2K3 | -166.31 | -165.77 | -146.31 | -145.77 | -185.34 | -186.52 | -165.34 | -166.52 |  |
| 2K4_2 | Rx IM3#2 = 2K2+2*2K1-2*2K3 | -128.15 | -127.08 | -118.15 | -117.08 | -126.17 | -128.52 | -116.17 | -118.52 |  |
| 2K4 | Rx IM3 = 2K4_1+2K4_2 | -128.15 | -127.07 | -118.15 | -117.07 | -126.17 | -128.52 | -116.17 | -118.52 |  |
| 2K5 | Total CW cancellation capability=1E-2K4 | 151.15 | 150.07 | 141.15 | 140.07 | 159.17 | 161.52 | 149.17 | 151.52 |  |

Digital domain

From our side, it is not clear how digital domain process can provide benefit on the interference suppression. Since if there is no blocking or non-linear distortion, the single tone CW interference is converted to DC in baseband, which has no impact to detection performance as long as the ratio between desired signal and residual interference is within receiver dynamic range. If blocking happens, it is also hard to overcome it in digital baseband.

6.7.5.3	Summary

CW cancellation performance is implementation specific. Spatial isolation, Analog cancellation and Digital cancellation are studied in sub-clause 6.7.5.2. For RF CW cancellation performance, it is trade-off among the allowed receiver desensitization, receiver linearity and the CW leakage power received at antenna. To enable the CW cancellation, CW cancellation training time should be considered in the implementation. During the CW cancellation training time, D2R reception should be avoided.