3GPP TSG RAN WG4 Meeting #113	R4-2420480

Orlando, USA, 18th – 22nd November, 2024

Agenda item:			7.17.6

Source:	vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

Title:	Updated simulation assumptions for beam prediction

Document for:	Approval

- Introduction

In this contribution, the link-level simulation assumption for AI/ML based BM case-1 is updated based on [1]. Companies are encouraged to provide simulation results to evaluate the impact of the measurement error based on the performance metric in section 5.

- Simulation assumptions for baseband error

Link level simulation (LLS) assumptions for evaluation of measurement error on AI/ML based BM case-1 performance are defined in Table 1-5.

Table 1 Baseline Link Level Simulation assumptions for AI/ML in beam management evaluations

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz. |
| Subcarrier spacing | 120kHz |
| Channel model | NLOS channel: option 1: CDL-C , option 2: TDL-C, DS = 100ns Other channel models are not precluded. |
| BS antenna configurations | Option 1: One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline. Number of Tx beams is 32 is baseline Other number of Tx beams is not precluded Option 2: 1 Tx antenna Number of TX beams: N/A Companies are encouraged to provide the codebook used in SLS |
| BS antenna element radiation pattern | Option1: Table 2 Option 2: N/A |
| BS antenna height and antenna array down-tilt angle, if needed (Option 1 above) | Option1: 25m, 110° Option 2: N/A |
| Beamforming characteristic of the BS pattern | Option 1a [corresponding to BS antenna configurations option 1]: 32 beams with grid of 4 elevation angles from [-25 to 25] and 8 azimuth angles from [-60 to 60] Option 1b [corresponding to BS antenna configurations option 1]: The codebook construction can be clarified by companies. Option 2: N/A |
| UE antenna configurations | Option 1: Panel structure: (M, N, P) = (1, 4, 2), - 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline - 1 panel as optional\ Number of Rx beams is up to UE Option 2: 2 Rx |
| UE antenna element radiation pattern | Option 1: Table 3 Option 2: N/A |
| UE moving speed | 3km/h |
| Reference signal | SSB or CSI-RS |
| DRX | No |
| Number of samples in L1 averaging | 1 as baseline, other number of samples are not precluded |
| Note 1: TXRU weights mapping for BS and UE is up to company. |  |

Table 2 BS antenna radiation pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |

Table 3 UE antenna radiation pattern model

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in dim (dB) |  |
| Antenna element radiation pattern in dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
| Note: are in local coordinate system. |  |

Table 4 Cell-specific parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 |
| SSB periodicity | ms | 20 |
| SSB bandwidth | RB | 20 |
| SSB SCS | kHz | 120 |
| RB Utilization | % | 100 |
| Data Modulation | - | QPSK |
| CP Length | - | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| Relative Delay of 1st Path (synchronous) | µs | 0 |
| SNR | dB | -3 Note: Simulation with other SNR levels are not precluded, e.g., for determining the SNR for ideal measurement. |
| Note: When analog beamforming is used in LLS, SNR value of above row represents the post beamforming SNR of the strongest Tx-Rx beam pair. For other Tx-Rx beam pairs, there are two options. Option 1: Apply error distribution from the post beamforming SNR of the strongest Tx-Rx beam pair to all Tx-Rx beam pair’s measurements in the SLS Option 2: Generate error distribution separately for each individual Tx-Rx beam pair’s SNRs and apply to the corresponding Tx-Rx beam pair measurement in the SLS. When analog beamforming is not used in LLS, Apply error distribution based on the SNR of above row to the strongest Tx-Rx beam pair’s measurements in the SLS Option 1: Apply error distribution from the SNR of above row to all Tx-Rx beam pairs’ measurements in the SLS Option 2: Generate error distribution separately for each individual Tx-Rx beam pairs’ SNR values and apply to the corresponding Tx-Rx beam pair measurements in the SLS. SNR of the strongest Tx-Rx beam pair can be fixed or not fixed. |  |  |

Table 5 CSI-RS configuration parameters

| Parameter | Value |
| --- | --- |
| CSI-RS bandwidth | 48 PRBs; |
| CSI-RS SCS | 120 kHz |
| Periodicity | 5 ms (optional); 40 ms |
| CSI-RS configuration: <X,D,N,CDM>, where: X=number of CSI-RS ports, D=density [RE/RB/port], N=number of OFDM symbols in the same slot | <32,1,4,FD-CDM2> |

- Simulation assumptions for RF error

The impact of RF errors should also be considered. ±4.5dB RF error is considered. Following options can be used to generate RF errors.

- Option 1: Independent for each TX beam

- Option 2: Same for each Tx beam

- Option 3: Independent for each module/panel

- Option 4: Same for each module/panel

- Other options are not precluded

  - Companies are requested to provide details of other options.

Following options about distributions can be used:

- Option 1: Uniform distribution

- Option 2: Gaussian distribution

- Option 3: A fixed value

- Other options are not precluded

  - Companies are requested to provide details of other options.

- Companies are encouraged to provide simulation results with and without considering RF errors.

- Reference model

For better alignment of the performance between companies, it would be better to perform simulations based on a reference model. The model description in Fig. 1 is an example of reference model for BM-case 1 performance evaluation, while it is not the intention to preclude other reference models.

Fig. 1 Reference model description

- Performance metrics

At least the following performance characteristics are to be provided

- Top-K/1 and Top-1/K prediction accuracy, where K = 1, 2, 3, 4, 5

  - with considering measurement errors

  - without considering measurement errors

- 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/Top-5 predicted beam(s) and the RSRP of the genie aided strongest beam

  - with considering measurement errors

  - without considering measurement errors

- 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/Top-5 predicted beam(s) and the ideal L1-RSRP of the same beams

  - with considering measurement errors

  - without considering measurement errors

Note:

90%-tile L1-RSRP difference = max(abs(95%-tile L1-RSRP), abs(5%-tile L1-RSRP))

- Simulation procedures (For information)

We provide the following procedures for companies to perform simulations to evaluate the impact of measurement error in BM prediction Sceanario-1: spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement as a baseline.

- Companies to generate ideal L1-RSRP dataset from the SLS assumption defined in TR 38.843 Table 6.3.1-1.

- Use subset samples of the ideal dataset generated from the 1st step for training models.

Approach a:

- Filter UEs in the other subset samples of the ideal dataset (none overlap samples with the subset dataset in 2nd step) according to SNR defined in Table 4 with a range to derive the ideal dataset for inference. and to evaluate the performance metric without considering measurement errors. Companies are encouraged to report the range for UEs filtering.

- Use the LLS simulation assumptions defined in Table 1-5 to generate L1-RSRP difference as the baseband errors.

- Add the baseband errors generated from LLS into the ideal SLS dataset to derive the dataset with measurement errors.

- If RF errors are modelled, then they should be added into the ideal SLS dataset in addition to baseband errors.

- Use the dataset with measurement errors for inference and to evaluate the performance metric with considering measurement errors.

Approach b:

4.	Use the L1-RSRP dataset generated from LLS for inference and to evaluate the performance metric with considering measurement errors.

- Power of Tx beams: companies to report the Tx power used in LLS.

- If RF errors are modelled, then they should be added into the ideal SLS dataset in addition to baseband errors

Companies are encouraged to report the mapping of error distributions from LLS to SLS.

- References

- R4-2417211, Simulation assumptions for beam prediction, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)