3GPP TSG-RAN5 Meeting #105	R5-246246

# Orlando, US, 18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1839 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 MU - General Updates in 38.521-3 Tx test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Progress has been made during this meeting in the MU and TT definition for multiple FR2 SA Tx test cases in TS 38.521-2 for multiple power classes. Editor’s notes in corresponding NSA test cases need to be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Updated editor’s notes for the FR2 NSA test cases in the clauses listed below for the power classes indicated in brackets. In addition: -Removed note on extreme conditions in 6.3B.1.4. -Removed note on aggregrated BW in 6.5B.2.4.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.1.4.1 (PC7), 6.2B.1.4.2 (PC7), 6.2B.2.4 (PC6, PC7), 6.2B.2.4_1.1 PC5), 6.3B.1.4 (PC6, PC7), 6.3B.1.4_1.1 (PC5), 6.3B.2.4 (PC6), 6.4B.2.4.2 (PC7), 6.5B.2.4.1 (PC6), 6.5B.2.4.3 (PC6) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

##### 6.2B.1.4.1	UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP and TRP

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2.1.1 in TS 38.521-2 is incomplete for power class other than 1, 3, 5,6 and 7.

## <<< Skip unchanged sections >>>

##### 6.2B.1.4.2	UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - Spherical Coverage

Editor's note:	The following aspects are either missing or not yet determined:

-	Measurement Uncertainties and Test Tolerances are FFS for power class other than 1, 3, 5 and 7.

## <<< Skip unchanged sections >>>

#### 6.2B.2.4	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (1 NR CC)

Editor's note:	Following aspects are missing or under discussion

-	The referred test case 6.2.2 in TS 38.521-2 [9] is incomplete for other than 1, 3, 5,6 and 7.

## <<< Skip unchanged sections >>>

##### 6.2B.2.4_1.1	UE Maximum Output Power reduction for Inter-Band EN-DC including FR2 (2 NR CCs)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.2A.2.1 in TS 38.521-2 [9] is incomplete for aggregated BW > 400MHz and intra-band non-contiguous CA.

-	The referred test case 6.2A.2.1 in TS 38.521-2 [9] is incomplete for power class other than 3 and 5.

## <<< Skip unchanged sections >>>

#### 6.3B.1.4	Minimum Output Power for EN-DC Interband including FR2 (1 NR CC)

Editor's note:	The following aspects of the clause are for future consideration:

-

-	Measurement Uncertainties and Test Tolerances are FFS for power class other than 1, 3, 5, 6 and 7.

## <<< Skip unchanged sections >>>

##### 6.3B.1.4_1.1	Minimum output power for inter-band EN-DC including FR2 (2 NR CCs)

Editor's note: The following aspects are either missing or not yet determined:

-	The referred test case 6.3A.1.1 in TS 38.521-2 [9] is incomplete for NR aggregated channel bandwidth wider than 400MHz and power class other than 3 and 5.

## <<< Skip unchanged sections >>>

#### 6.3B.2.4	Transmit OFF Power for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	Following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class other than 1, 3, 5 and 6.

## <<< Skip unchanged sections >>>

##### 6.4B.2.4.2	Carrier Leakage for inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	Measurement Uncertainty and Test Tolerance are FFS for power class other than 1, 3, 5 and 7.

-	The test case is incomplete for band n259.

## <<< Skip unchanged sections >>>

##### 6.5B.2.4.1	Spectrum emissions mask for Inter-band EN-DC including FR2 (1 NR CC)

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class other than 1 FR2a, 3, 5 and 6.

## <<< Skip unchanged sections >>>

##### 6.5B.2.4.3	Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The referred test case 6.5.2.3 in TS 38.521-2 [9] is incomplete for power class other than 1, 3, 5 and 6.

-

## <<< END OF CHANGES >>>