3GPP TSG-RAN5 Meeting #105	R5-246304

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0058 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of A-MPR TC 6.2A.3 for NS_24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 updated A-MPR for NS_24 of NB-IoT NTN Band 256 power class 5 in TS 36.102 V18.6.0. therefore,the related minimum requirement and test requirement need to be updated |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the A-MPR for NS_24 of NB-IoT NTN band 256 Editorial update of the minimum requirement A-MPR for NS_24 of PC3 Update of minimum requirement and test requirement A-MPR for NS_24 of PC5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test requirements of TC are incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.905 CR 0277 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP analyzes in CR R5-246307 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change1>

6.2A.3	UE additional maximum output power reduction for category M1 UE

6.2A.3.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2A.3.2	Test applicability

The requirements of this test apply in test case 6.5A.3.3 Additional Spectrum Emission Mask for network signalled values NS_02N and NS-24 to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

The requirements of this test apply in test case 6.5A.4.4 Additional Spurious Emissions for network signalled values NS_02N and NS_24 to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.2A.3.3	Minimum conformance requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2A.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2A.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |  |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | Table 5.3.1-1 | N/A |  |
| NS_02N | 6.5A.4.4.2 | 255 | Table 5.3.1-1 | N/A |  |
| NS_24 | 6.5A.4.4.3 | 256 | Table 5.3.1-1 | PC3 | PC5 |
|  |  |  |  | ≤3.5 | ≤0.5 |

For subPRB allocation, the allowed A-MPR values specified below in Table 6.2A.3-2 for category M1 UE are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-2: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | N/A |  |
| NS_02N | 6.5A.4.4.2 | 255 | N/A |  |
| NS_24 | 6.5A.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤3.5 | ≤0.5 |

The normative reference for this requirement is TS 36.102 [11] subclause 6.2A.3

< End of Change1 >

< Skipped for changed>

<Start of Change2>

6.2A.3.5	Test requirements

The maximum output power derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4EA.5-1 to Table 6.2.4EA.5-22. The allowed A-MPR values specified in Table 6.2.4EA-1 are in addition to the allowed MPR requirements specified in clause 6.2.3 EA. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5 EA.3-1 and 6.2.5EA.3-2 apply.

Table 6.2A.3.5-1: UE Power Class test requirements (network signalled value “NS_02N”)

| Config ID | Power class 3 |  |  |  |  |  | Power class 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | A_MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | A_MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | N/A | 23 | 2 | 25.7 | 20.3 | 0 | N/A | 20 | 2 | 22.7 | 17.3 |
| 2 | 1 | N/A | 22 | 2 | 25.7 | 19.3 | 1 | N/A | 19 | 3.5 | 22.7 | 14.8 |
| 3 | 2 | N/A | 21 | 2 | 25.7 | 18.3 | 2 | N/A | 18 | 4 | 22.7 | 13.3 |
| 4 | 1 | N/A | 22 | 2 | 25.7 | 19.3 | 1 | N/A | 19 | 3.5 | 22.7 | 14.8 |
| 5 | 2 | N/A | 21 | 2 | 25.7 | 18.3 | 2 | N/A | 18 | 4 | 22.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.5-1a: UE Power Class test requirements, subPRB allocation (network signalled value “NS_02N”)

| Config ID | Power class 3 |  |  |  |  |  | Power class 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | A_MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | A_MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | N/A | 23 | 2 | 25.7 | 20.3 | 0 | N/A | 20 | 2 | 22.7 | 17.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.5-2: UE Power Class test requirements (network signalled value "NS_24")

| Config ID | Power class 3 |  |  |  |  |  | Power class 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | A-MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | A-MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 0 | 0.5 | 19.5 | 3.5 | 22.7 | 15.3 |
| 2 | 1 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 1 | 0.5 | 19.0 | 3.5 | 22.7 | 14.8 |
| 3 | 2 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 0.5 | 18.0 | 4.0 | 22.7 | 13.3 |
| 4 | 1 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 1 | 0.5 | 19.0 | 3.5 | 22.7 | 14.8 |
| 5 | 2 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 0.5 | 18.0 | 4.0 | 22.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.5-1a: UE Power Class test requirements, subPRB allocation (network signalled value "NS_24")

| Config ID | Power class 3 |  |  |  |  |  | Power class 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | A-MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | A-MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 0 | 0.5 | 19.5 | 3.5 | 22.7 | 15.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |  |  |

< End of Change2 >