3GPP TSG-RAN5 Meeting #105	R5-246364

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3345 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RRC IE RadioBearerConfig |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As UPIP TC and MR-DC TC 8.2.2.6.1 are tested in pure E-UTRA environment, the conditions MCG_NR_PDCP and SRB_NR_PDCP used in these cases need to be updated to remove the restriction of EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Explanations of MCG_NR_PDCP & SRB_NR_PDCP were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The condition of MCG_NR_PDCP and SRB_NR_PDCP will mismatch with the test configurations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

4.6.3	Radio resource control information elements

–	AdditionalSpectrumEmission

[Skip unchanged text]

–	RadioBearerConfig

Table 4.6.3-132: RadioBearerConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 1 entry |  | SRB1 |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| srb-Identity | SRB-Identity with condition SRB1 |  |  |
| reestablishPDCP | Not present |  |  |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 1 entry |  | SRB2, RESUME |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| srb-Identity | SRB-Identity with condition SRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | RESUME |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 1 entry |  | SRB3 |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| srb-Identity | SRB-Identity with condition SRB3 |  |  |
| reestablishPDCP | Not present |  |  |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 2 entries |  | SRB_NR_PDCP, REEST |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| srb-Identity | SRB-Identity with condition SRB1 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | SRB_NR_PDCP AND Re-establish_PDCP |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| SRB-ToAddMod[2] SEQUENCE { |  | entry 2 |  |
| srb-Identity | SRB-Identity with condition SRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | SRB_NR_PDCP AND Re-establish_PDCP, REEST |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| srb3-ToRelease | Not present |  |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | EN-DC_DRB |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 6 |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | EN-DC_DRB AND Re-establish_PDCP |
| recoverPDCP | Not present |  |  |
|  | true |  | EN-DC_DRB AND Recover_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | MCG_NR_PDCP |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 12 | EPS Bearer Id of default MCG DRB |  |
| } |  |  |  |
| drb-Identity | 8 | DRB Id of default MCG DRB |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | DRB1 |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB1 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | DRB1 AND Re-establish_PDCP |
| recoverPDCP | Not present |  |  |
|  | true |  | DRB1 AND Recover_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| daps-Config-r16 | Not present |  |  |
|  | true |  | DRB1 AND DAPS_PDCP |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | DRB2 |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB2 |  |  |
| reestablishPDCP | Not present |  |  |
|  | true |  | DRB2 AND Re-establish_PDCP |
| recoverPDCP | Not present |  |  |
|  | true |  | DRB2 AND Recover_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| daps-Config-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | n entries | n is the number of DRBs established before RRC resume or RRC re-establishment | RESUME, REEST |
| DRB-ToAddMod[k, k=1..n] SEQUENCE { |  | entry [k, k=1..n] |  |
| cnAssociation | Not present |  |  |
| drb-Identity | DRB-Identity with condition DRBk |  |  |
| reestablishPDCP | true |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | Not present |  |  |
| daps-Config-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  | DRBn |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
|  | SDAP-Config with condition No-defaultDRB |  | SCG_DRB |
| } |  |  |  |
| drb-Identity | DRB-Identity with condition DRBn |  |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
|  | PDCP-Config with condition Split |  | Split |
| } |  |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| securityConfig | Not present |  | SRB1 |
| securityConfig SEQUENCE { |  |  |  |
| securityAlgorithmConfig | SecurityAlgorithmConfig |  |  |
| keyToUse | master |  |  |
|  | secondary |  | SRB3, EN-DC_DRB, SecondaryKeys |
| } |  |  |  |
| mrb-ToAddModList-r17 | Not present |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 1 entry |  | MRBm |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm |  |  |
| mrb-IdentityNew-r17 | Not present |  |  |
| reestablishPDCP-r17 | Not present |  |  |
| recoverPDCP-r17 | Not present |  |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm |  | UM_bi_PTP, UM_PTP, UM_PTM, UMPTP_UMPTM |
|  | PDCP-Config with condition MRB_Initialization and MRBm |  |  |
| } |  |  |  |
| } |  |  |  |
| mrb-ToReleaseList-r17 | Not present |  |  |
| srb4-ToAddMod-r17 | Not present |  |  |
| srb4-ToAddMod-r17 SEQUENCE { |  |  | SRB4 |
| srb-Identity | SRB-Identity with condition SRB1 |  |  |
| reestablishPDCP | Not present |  |  |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | Not present |  |  |
| srb-Identity-v1700 | SRB-Identity-v1700 |  |  |
| } |  |  |  |
| srb4-ToRelease-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCG_NR_PDCP | MCG DRB configured or reconfigured with NR PDCP |
| SRB_NR_PDCP | NR SRB1 and SRB2 configured with NR PDCP |
| SRB1 | Establishment of SRB1 |
| SRB2 | Establishment of SRB2 |
| SRB3 | Establishment of SRB3 |
| SRB4 | Establishment of SRB4 |
| DRB1 | Establishment of DRB1 |
| DRB2 | Establishment of DRB2 |
| DRBn | Establishment of DRBn |
| EN-DC_DRB | EN-DC DRB configured on SCG |
| Re-establish_PDCP | Re-establishment of PDCP |
| Recover_PDCP | Recovery of PDCP |
| RESUME | Used in RRCResume Message |
| REEST | The first RRCReconfiguration message after successful completion of the RRC re-establishment procedure. |
| SecondaryKeys | NR-DC SCG or MCG DRB configured or reconfigured with secondary security keys |
| Split | Split PDCP: more than one RLC |
| DAPS_PDCP | Used when the bearer is configured as DAPS bearer |
| SCG_DRB | Used when the bearer is configured as an SCG bearer in either NR-DC or NE-DC |
| MRBm | Establishment of MRBm |
| UM_bi_PTP | Multicast MRB with bidirectional RLC-UM configuration for receiving PTP transmission |
| UM_PTP | Multicast MRB with DL only RLC-UM configuration for receiving PTP transmission |
| UM_PTM | Multicast MRB with DL only RLC-UM entity for receiving PTM transmission |
| UMPTP_UMPTM | Multicast MRB with two RLC-UM entities, one DL only RLC-UM entity for receiving PTP transmission and the other DL only RLC-UM entity for receiving PTM transmission |

[End of change]