3GPP TSG-RAN5 Meeting #105	R5-246380

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1491 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RRC IE SchedulingRequestConfig |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To align the RRC IE SchedulingRequestConfig with TS 36.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IE SchedulingRequestConfig was updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This IE will mismatch with core specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 and 4.8.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

4.6.3	Radio resource control information elements

Table 4.6.3-0A: Void

[Skip unchanged text]

–	SchedulingRequestConfig-DEFAULT

Table 4.6.3-20: SchedulingRequestConfig-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestConfig-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-PUCCH-ResourceIndex | See subclause 4.6.8 | Channel-bandwidth-dependent parameter |  |
| sr-ConfigIndex | 30 |  | FDD |
|  | 32 |  | FDD and (CEmodeA or CEmodeB) and 3 MHz BW |
|  | 27 |  | TDD |
| dsr-TransMax | n4 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |
| 3 MHz BW | Used for cells with 3 MHz bandwidth |

[Skip unchanged text]

4.8.2.1.6	Physical Layer configurations

Table 4.8.2.1.6-1: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| pdsch-ConfigDedicated | PDSCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |
|  | Not present |  | RBC or RBC-HO or SCell_AddMod |
| pucch-ConfigDedicated | PUCCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | (SRB1 and not (CEmodeA or CEmodeB)) or RBC-HO or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |
|  | Not present |  | RBC or SCell_AddMod or CEmodeA or CEmodeB |
| pusch-ConfigDedicated | PUSCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |
|  | Not present |  | RBC or RBC-HO or SCell_AddMod |
| uplinkPowerControlDedicated | UplinkPowerControlDedicated-DEFAULT | See subclause 4.6.3 | SRB1 or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |
|  | Not present |  | RBC or RBC-HO or SCell_AddMod |
| tpc-PDCCH-ConfigPUCCH | Not present |  | SRB1 or SCell_AddMod |
|  | TPC-PDCCH-Config-DEFAULT using condition PUCCH | See subclause 4.6.3 | RBC or RBC-HO or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |
| tpc-PDCCH-ConfigPUSCH | Not present |  | SRB1 SCell_AddMod |
|  | TPC-PDCCH-Config-DEFAULT using condition PUSCH | See subclause 4.6.3 | RBC or RBC-HO or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |
| cqi-ReportConfig | CQI-ReportConfig-DEFAULT | See subclause 4.6.3 | SRB1 or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |
|  | CQI-ReportConfig- DEFAULT using condition CQI_PERIODIC | See subclause 4.6.3 | (RBC or RBC-HO) and not RBC-HO-CA |
|  | Not present |  | CA HalfDuplex |
| soundingRS-UL-ConfigDedicated | Not present |  | SRB1 or HO-TO-EUTRA SCell_AddMod PSCell_AddMod HalfDuplex |
|  | SoundingRS-Ul-ConfigDedicated-DEFAULT | See subclause 4.6.3 | RBC or RBC-HO |
| antennaInfo CHOICE { |  |  |  |
| explicitValue SEQUENCE { |  |  | 2TX or 4TX |
| transmissionMode | tm3 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n2TxAntenna-tm3 | '11' | BIT STRING (SIZE (2)) | 2TX |
| n4TxAntenna-tm3 | '1111' | BIT STRING (SIZE (4)) | 4TX |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| defaultValue | NULL |  |  |
| } |  |  |  |
| schedulingRequestConfig | SchedulingRequestConfig-DEFAULT | See subclause 4.6.3 | SRB1 or RBC-HO or HO-TO-EUTRA |
|  | Not present |  | RBC SCell_AddMod PSCell_AddMod |
| cqi-ReportConfig-v920 | Not present |  |  |
| antennaInfo-v920 | Not present |  |  |
| antennaInfo-r10 | Not present |  |  |
| antennaInfoUL-r10 | Not present |  |  |
| cif-Presence-r10 | FALSE |  | SCell_AddMod |
|  | Not present |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  | SCell_AddMod or RBC-HO-CA |
|  | Not present |  |  |
| csi-RS-Config-r10 | Not present |  |  |
| pucch-ConfigDedicated-v1020 | PUCCH-ConfigDedicated-v1020-DEFAULT |  | SCell_AddMod |
|  | Not present |  |  |
| pusch-ConfigDedicated-v1020 | Not present |  |  |
| schedulingRequestConfig-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |
| uplinkPowerControlDedicated-v1020 | UplinkPowerControlDedicated-v1020-DEFAULT |  | SCell_AddMod and (TxD or SRSAp) |
|  | Not present |  |  |
| additionalSpectrumEmissionCA-r10 | Not present |  |  |
| csi-RS-ConfigNZPToReleaseList-r11 | Not present |  |  |
| csi-RS-ConfigNZPToAddModList-r11 | Not present |  |  |
| csi-RS-ConfigNZPToAddModList-r11 SEQUENCE (SIZE (1..maxCSI-RS-NZP-r11)) OF CSI-RS-ConfigNZP-r11 { | 1 entry |  | DL_CoMP |
| CSI-RS-ConfigNZP-r11[1] | CSI-RS-ConfigNZP-r11-DEFAULT |  |  |
| } |  |  |  |
| csi-RS-ConfigZPToReleaseList-r11 | Not present |  |  |
| csi-RS-ConfigZPToAddModList-r11 | Not present |  |  |
| csi-RS-ConfigZPToAddModList-r11 SEQUENCE (SIZE (1..maxCSI-RS-ZP-r11)) OF CSI-RS-ConfigZP-r11 { | 1 entry |  | DL_CoMP |
| CSI-RS-ConfigZP-r11[1] | CSI-RS-ConfigZP-r11-DEFAULT |  |  |
| } |  |  |  |
| epdcch-Config-r11 | EPDCCH-Config-r11-DEFAULT |  | ePDCCH or (Not(RBC) and (CEmodeA or CEmodeB)) |
|  | Not present |  |  |
| pdsch-ConfigDedicated-v1130 | PDSCH-ConfigDedicated-v1130-DEFAULT |  | DL_CoMP |
|  | Not present |  |  |
| cqi-ReportConfig-v1130 | CQI-ReportConfig-v1130-DEFAULT |  | DL_CoMP |
|  | Not present |  |  |
| pucch-ConfigDedicated-v1130 | PUCCH-ConfigDedicated-v1130-DEFAULT |  | UL_CoMP |
|  | Not present |  |  |
| pusch-ConfigDedicated-v1130 | PUSCH-ConfigDedicated-v1130-DEFAULT |  | UL_CoMP |
|  | Not present |  |  |
| uplinkPowerControlDedicated-v1130 | UplinkPowerControlDedicated-v1130-DEFAULT |  | UL_CoMP |
|  | Not present |  |  |
| cqi-ReportConfigPCell-v1250 | CQI-ReportConfig-v1250-DEFAULT |  | DL 256QAM |
|  | Not present |  |  |
| uplinkPowerControlDedicated-v1250 | Not present |  |  |
| pusch-ConfigDedicated-v1250 | Not present |  |  |
| csi-RS-Config-v1250 | Not present |  |  |
| pdsch-ConfigDedicated-v1280 | Not present |  |  |
| pdsch-ConfigDedicated-v1310 | Not present |  |  |
| pucch-ConfigDedicated-r13 | Not present |  |  |
|  | PUCCH-ConfigDedicated- r13-DEFAULT |  | Not (RBC) and (CEmodeA or CEmodeB) |
| pusch-ConfigDedicated-r13 | Not present |  |  |
| pdcch-CandidateReductions-r13 | Not present |  |  |
| cqi-ReportConfig-v1310 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1310 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedUpPTsExt-r13 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-v1310 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13 | Not present |  |  |
| csi-RS-Config-v1310 | Not present |  |  |
| ce-Mode-r13 | Not present |  |  |
| ce-Mode-r13 CHOICE { |  |  | Not (RBC) and (CEmodeA OR CEmodeB) |
| setup | ce-ModeA |  | CEmodeA |
| setup | ce-ModeB |  | CEmodeB |
| } |  |  |  |
| pusch-EnhancementsConfig-r14 | PUSCH-EnhancementsConfig-r14-DEFAULT |  | eVoLTE |
|  | Not present |  |  |
| physicalConfigDedicatedSTTI-r15 | Not present |  |  |
| physicalConfigDedicatedSTTI-r15 CHOICE { |  |  | Short-TTI |
| setup SEQUENCE{ |  |  |  |
| antennaInfoDedicatedSTTI-r15 CHOICE { |  |  |  |
| setup SEQUENCE{ |  |  |  |
| transmissionModeDL-MBSFN-r15 | tm9 |  |  |
| transmissionModeDL-nonMBSFN-r15 | tm3 |  |  |
| maxLayersMIMO-STTI-r15 | fourLayers |  |  |
| slotSubslotPDSCH-TxDiv-2Layer-r15 | Not present |  |  |
| slotSubslotPDSCH-TxDiv-4Layer-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| antennaInfoUL-STTI-r15 SEQUENCE { |  |  |  |
| transmissionModeUL-STTI-r15 | tm1 |  |  |
| } |  |  |  |
| pucch-ConfigDedicated-v1530 | Not present |  |  |
| schedulingRequestConfig-v1530 | Not present |  |  |
| uplinkPowerControlDedicatedSTTI-r15 SEQUENCE { |  |  |  |
| accumulationEnabledSTTI-r15 | TRUE |  |  |
| uplinkPower-CSIPayload | FALSE |  |  |
| } |  |  |  |
| semiOpenLoopSTTI-r15 | Not present |  |  |
| slotOrSubslotPDSCH-Config-r15 | SlotOrSubslotPDSCH-Config-r15-DEFAULT |  |  |
| slotOrSubslotPUSCH-Config-r15 | SlotOrSubslotPUSCH-Config-r15-DEFAULT |  |  |
| spdcch-Config-r15 | SPDCCH-Config-r15-DEFAULT |  |  |
| spucch-Config-r15 | SPUCCH-Config-r15-DEFAULT |  |  |
| srs-DCI7-TriggeringConfig-r15 | Not present |  |  |
| shortProcessingTime-r15 | Not present |  |  |
| shortTTI-r15 SEQUENCE{ |  |  |  |
| dl-STTI-Length-r15 | slot |  |  |
| ul-STTI-Length-r15 | slot |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| uplinkPowerControlDedicated-v1530 | Not present |  |  |
| uplinkPowerControlDedicated-v1530 | UplinkPowerControlDedicated-v1530-DEFAULT |  | Aerial UE |
| } |  |  |  |
| Note: For signalling test, table 6.6B.1.1.1-1 is applied. |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB1 | Used at configuration of SRB1 during RRC connection (re-)establishment |
| RBC | Used at configuration of a radio bearer combination during SRB2+DRB establishment |
| 2TX | Used for cells with two antenna ports |
| 4TX | Used for cells with four antenna ports |
| RBC-HO | Used during Handover |
| RBC-HO-CA | Used during Handover when Carrier Aggregation is configured for the source cells |
| SCell_AddMod | Addition or modification of SCell |
| HO-TO-EUTRA | Inter-RAT handover to E-UTRA |
| ePDCCH | Used at configuration of ePDCCH |
| DL_CoMP | Used for DL CoMP |
| UL_CoMP | Used for UL CoMP |
| CA | For Carrier Aggregation Test cases |
| TxD | When PUCCH Transmission Diversity is performed. |
| SRSAp | Aperiodic SRS configured |
| Intra-BC CA | Intra-band contiguous Carrier Aggregation |
| Inter-B CA | Inter-band Carrier Aggregation |
| UL CA | When UL Carrier Aggregation is used. |
| PSCell_Mod | Modification of PSCell |
| PSCell_Add_Split_DRB | Add PSCell(s) and setup of Split DRB |
| PSCell_Add_SCG_DRB | Add PSCell(s) and setup of SCG DRB |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |
| HalfDuplex | Used during Type B HalfDuplexOperation |
| DL 256QAM | When DL 256QAM is used |
| eVoLTE | When VoLTE enhancement is used. |
| Aerial UE | For Aerial vehicles |
| Short-TTI | When short TTI is used |

Table 4.8.2.1.6-1A: PhysicalConfigDedicated-eIMTA

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-eIMTA ::= SEQUENCE { |  |  |  |
| pdsch-ConfigDedicated | PDSCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 or HO-TO-EUTRA |
|  | Not present |  | RBC or RBC-HO SCell_AddMod |
| pucch-ConfigDedicated | PUCCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 or RBC-HO or HO-TO-EUTRA |
|  | Not present |  | RBC SCell_AddMod |
| pusch-ConfigDedicated | PUSCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 or HO-TO-EUTRA |
|  | Not present |  | RBC or RBC-HO SCell_AddMod |
| uplinkPowerControlDedicated | UplinkPowerControlDedicated-DEFAULT | See subclause 4.6.3 | SRB1 or HO-TO-EUTRA |
|  | Not present |  | RBC or RBC-HO SCell_AddMod |
| tpc-PDCCH-ConfigPUCCH | Not present |  | SRB1 SCell_AddMod |
|  | TPC-PDCCH-Config-DEFAULT using condition PUCCH | See subclause 4.6.3 | RBC or RBC-HO or HO-TO-EUTRA |
| tpc-PDCCH-ConfigPUSCH | Not present |  | SRB1 SCell_AddMod |
|  | TPC-PDCCH-Config-DEFAULT using condition PUSCH | See subclause 4.6.3 | RBC or RBC-HO or HO-TO-EUTRA |
| cqi-ReportConfig | CQI-ReportConfig-DEFAULT | See subclause 4.6.3 | SRB1 or HO-TO-EUTRA |
|  | CQI-ReportConfig- DEFAULT using condition CQI_PERIODIC | See subclause 4.6.3 | RBC or RBC-HO |
|  | Not present |  | CA |
| soundingRS-UL-ConfigDedicated | Not present |  | SRB1 or HO-TO-EUTRA SCell_AddMod |
|  | SoundingRS-Ul-ConfigDedicated-DEFAULT | See subclause 4.6.3 | RBC or RBC-HO |
| antennaInfo CHOICE { |  |  |  |
| explicitValue SEQUENCE { |  |  | 2TX |
| transmissionMode | tm3 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n2TxAntenna-tm3 | '11' | BIT STRING (SIZE (2)) |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| defaultValue | NULL |  |  |
| } |  |  |  |
| schedulingRequestConfig | SchedulingRequestConfig-DEFAULT | See subclause 4.6.3 | SRB1 or RBC-HO or HO-TO-EUTRA |
|  | Not present |  | RBC SCell_AddMod |
| cqi-ReportConfig-v920 | Not present |  |  |
| antennaInfo-v920 | Not present |  |  |
| antennaInfo-r10 | Not present |  |  |
| antennaInfoUL-r10 | Not present |  |  |
| cif-Presence-r10 | FALSE |  | SCell_AddMod |
|  | Not present |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  | SCell_AddMod |
|  | Not present |  |  |
| csi-RS-Config-r10 | Not present |  |  |
| pucch-ConfigDedicated-v1020 | PUCCH-ConfigDedicated-v1020-DEFAULT |  | SCell_AddMod |
| pusch-ConfigDedicated-v1020 | Not present |  |  |
| schedulingRequestConfig-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |
| uplinkPowerControlDedicated-v1020 | UplinkPowerControlDedicated-v1020-DEFAULT |  | SCell_AddMod and (TxD or SRSAp) |
|  | Not present |  |  |
| additionalSpectrumEmissionCA-r10 | Not present |  |  |
| csi-RS-ConfigNZPToReleaseList-r11 | Not present |  |  |
| csi-RS-ConfigNZPToAddModList-r11 | Not present |  |  |
| csi-RS-ConfigNZPToAddModList-r11 SEQUENCE (SIZE (1..maxCSI-RS-NZP-r11)) OF CSI-RS-ConfigNZP-r11 { | 1 entry |  | DL_CoMP |
| CSI-RS-ConfigNZP-r11[1] | CSI-RS-ConfigNZP-r11-DEFAULT |  |  |
| } |  |  |  |
| csi-RS-ConfigZPToReleaseList-r11 | Not present |  |  |
| csi-RS-ConfigZPToAddModList-r11 | Not present |  |  |
| csi-RS-ConfigZPToAddModList-r11 SEQUENCE (SIZE (1..maxCSI-RS-ZP-r11)) OF CSI-RS-ConfigZP-r11 { | 1 entry |  | DL_CoMP |
| CSI-RS-ConfigZP-r11[1] | CSI-RS-ConfigZP-r11-DEFAULT |  |  |
| } |  |  |  |
| epdcch-Config-r11 | EPDCCH-Config-r11- eIMTA |  | ePDCCH |
|  | Not present |  |  |
| pdsch-ConfigDedicated-v1130 | PDSCH-ConfigDedicated-v1130-DEFAULT |  | DL_CoMP |
|  | Not present |  |  |
| cqi-ReportConfig-v1130 | CQI-ReportConfig-v1130-eIMTA |  | DL_CoMP |
|  | Not present |  |  |
| pucch-ConfigDedicated-v1130 | PUCCH-ConfigDedicated-v1130-DEFAULT |  | UL_CoMP |
|  | Not present |  |  |
| pusch-ConfigDedicated-v1130 | PUSCH-ConfigDedicated-v1130-DEFAULT |  | UL_CoMP |
|  | Not present |  |  |
| uplinkPowerControlDedicated-v1130 | UplinkPowerControlDedicated-v1130-DEFAULT |  | UL_CoMP |
|  | Not present |  |  |
| antennaInfo-v1250 | Not present |  |  |
| eimta-MainConfig-r12 | EIMTA-MainConfig-r12-DEFAULT |  | eIMTA |
|  | Not present |  |  |
| eimta-MainConfigPCell-r12 | EIMTA-MainConfigServCell-r12-DEFAULT |  | eIMTA |
|  | Not present |  |  |
| pucch-ConfigDedicated-v1250 | PUCCH-ConfigDedicated-v1250-DEFAULT |  | eIMTA |
|  | Not present |  |  |
| cqi-ReportConfigPCell-v1250 | CQI-ReportConfig-v1250-DEFAULT |  | eIMTA |
|  | Not present |  |  |
| uplinkPowerControlDedicated-v1250 | UplinkPowerControlDedicated-v1250-DEFAULT |  | eIMTA |
|  | Not present |  |  |
| pusch-ConfigDedicated-v1250 | PUSCH-ConfigDedicated-v1250-DEFAULT |  | eIMTA |
|  | Not present |  |  |
| csi-RS-Config-v1250 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB1 | Used at configuration of SRB1 during RRC connection (re-)establishment |
| RBC | Used at configuration of a radio bearer combination during SRB2+DRB establishment |
| 2TX | Used for cells with two antenna ports |
| RBC-HO | Used during Handover |
| RBC-HO-CA | Used during Handover when Carrier Aggregation is configured for the source cells |
| SCell_AddMod | Addition or modification of SCell |
| HO-TO-EUTRA | Inter-RAT handover to E-UTRA |
| ePDCCH | Used at configuration of ePDCCH |
| DL_CoMP | Used for DL CoMP |
| UL_CoMP | Used for UL CoMP |
| CA | For Carrier Aggregation Test cases |
| TxD | When PUCCH Transmission Diversity is performed. |
| SRSAp | Aperiodic SRS configured |
| Intra-BC CA | Intra-band contiguous Carrier Aggregation |
| Inter-B CA | Inter-band Carrier Aggregation |
| UL CA | When UL Carrier Aggregation is used. |
| eIMTA | eIMTA cell environment |

[End of change]