3GPP TSG-RAN5 Meeting #105	R5-246385

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5359 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT TC 22.3.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The table references, RRC message and IEs need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table references and RRC message contents were updated with editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error will exist in this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.3.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

22.3.3.5	NB-IoT / PDCP re-establishment / stored UE AS context is used and drb-ContinueROHC is configured

22.3.3.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and upper layers indicate stored UE AS context }

ensure that {

when { UE is requested to resume a suspended RRC connection }

then { UE transmits next PDCP Data PDU with SN value 0 }

}

22.3.3.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clause 5.3.3.4a and 3GPP TS 36.523 clause 5.2.1.2

[TS 36.331, clause 5.3.3.4a]

Reception of the RRCConnectionResume by the UE

1>	stop timer T300;

1>	restore the PDCP state and re-establish PDCP entities for SRB2 and all DRBs;

1>	if drb-ContinueROHC is included:

2>	indicate to lower layers that stored UE AS context is used and that drb-ContinueROHC is configured;

2>	continue the header compression protocol context for the DRBs configured with the header compression protocol;

1>	else:

2>	indicate to lower layers that stored UE AS context is used;

2>	reset the header compression protocol context for the DRBs configured with the header compression protocol;

1>	 discard the stored UE AS context and resumeIdentity;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	resume SRB2 and all DRBs;

…

[TS 36.323, clause 5.2.1.2]

When upper layers request a PDCP re-establishment, the UE shall:

-	reset the header compression protocol for uplink and start with an IR state in U-mode [9] [11] if the DRB is configured with the header compression protocol and drb-ContinueROHC is not configured [3];

-	set Next_PDCP_TX_SN, and TX_HFN to 0;

-	apply the ciphering algorithm and key provided by upper layers during the re-establishment procedure;

-	if connected as an RN, apply the integrity protection algorithm and key provided by upper layers (if configured) during the re-establishment procedure;

-	for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment, as specified in the subclause 5.1.1 without restarting the discardTimer.

22.3.3.5.3	Test description

22.3.3.5.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 according to TS 36.508 [18] Table 8.1.4.2-1A.

UE:

None.

Preamble:

-	The UE is in State 2B-NB, Attach Connected Mode, UE Test Loopback Activated for UE test loop mode B on Ncell 1 (serving cell) according to TS 36.508 [18] clause 8.1.5.2B.

22.3.3.5.3.2	Test procedure sequence

Table 22.3.3.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS creates 3 IP Packets. | - | - | - | - |
|  | EXCEPTION: Step 2 and 3 shall be repeated for k=0 to 1 (increment=1). | - | - | - | - |
| 2 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context. | <- | IP Packet #k |  | - |
| 3 | The UE loops back the IP packet received in Step 2 on the DRB associated with the default EPS bearer context. | -> | IP Packet #k |  | - |
| 4 | SS transmits an RRCConnectionRelease-NB Message. (IE ReleaseCause-r13 with Value rrc-Suspend). | <- | RRCConnectionRelease-NB |  | - |
| 5 | Wait for 10 s for the UE to enter E-UTRA RRC_IDLE state on Ncell 1. | - |  |  | - |
| 6 | The SS transmits a Paging-NB message with a matching UE identity. | <- | Paging-NB |  | - |
| 7 | The UE transmits RRCConnectionResumeRequest-NB message. | -> | RRCConnectionResumeRequest-NB |  |  |
| 8 | The SS transmits RRCConnectionResume-NB message with drb-ContinueROHC-r13 present. | <- | RRCConnectionResume-NB |  |  |
| 9 | The UE transmits RRCConnectionResumeComplete-NB message to confirm the successful completion of an RRC connection resumption. | -> | RRCConnectionResumeComplete-NB |  |  |
| 10-11 | Void |  |  |  |  |
| 12 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context. | <- | IP Packet |  |  |
| 13 | Check: Does the UE loop back the IP packet on the DRB associated with the default EPS bearer context with PDCP SN=0? | -> | IP Packet | 1 | P |

22.3.3.5.3.3	Specific message contents

Table 22.3.3.5.3.3-1: PDCP-Config-NB (RRCConnectionReconfiguration-NB message in Preamble)

| Derivation Path: TS 36.331 clause 6.7.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config-NB-r13 ::= SEQUENCE { |  |  |  |
| headerCompression-r13 CHOICE { |  |  |  |
| rohc SEQUENCE { |  |  |  |
| maxCID-r13 | The IE shall be set to the value of maxNumberROHC-ContextSessions parameter as indicated by the UE in UECapabilityInformation |  |  |
| profiles-r13 SEQUENCE { |  |  |  |
| profile0x0002 | One of the profile shall be set to TRUE based on the parameter supportedROHC-Profiles as indicated by the UE in UECapabilityInformation. If support of more than one ROHC profile are indicated, then the profile corresponding to the highest value shall be set to TRUE |  |  |
| profile0x0003 |  |  |  |
| profile0x0004 |  |  |  |
| profile0x0006 |  |  |  |
| profile0x0102 |  |  |  |
| profile0x0103 |  |  |  |
| profile0x0104 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.3.5.3.3-2: RRCConnectionRelease-NB message (step 4, Table 22.3.3.5.3.2-1)

| Derivation Path: TS 36.508 table 8.1.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r13 SEQUENCE { |  |  |  |
| releaseCause-r13 | rrc-Suspend |  |  |
| resumeIdentity-r13 | BIT STRING (SIZE(40)) | Any value |  |
| extendedWaitTime-r13 | Not present |  |  |
| redirectedCarrierInfo | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.3.5.3.3-3: RRCConnectionResume-NB message (step 8, Table 22.3.3.5.3.2-1)

| Derivation Path: TS 36.508 table 8.1.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionResume-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionResume-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 | RadioResourceConfigDedicated-NB-DRB-exPDCP |  |  |
| nextHopChainingCount-r13 | 0 |  |  |
| drb-ContinueROHC-r13 | true |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.3.5.3.3-4: RadioResourceConfigDedicated-NB-DRB-exPDCP

| Derivation Path: TS 36.508 Table 8.1.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-NB-DRB-exPDCP ::= SEQUENCE { |  |  |  |
| drb-ToAddModList-r13 SEQUENCE (SIZE (1.. maxDRB-NB-r13)) OF DRB-ToAddMod-NB-r13 { | one entry |  |  |
| DRB-ToAddMod-NB-r13[1] | DRB-ToAddMod-NB-exPDCP (1) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.3.5.3.3-5: DRB-ToAddMod-NB-exPDCP(bid)

| Derivation Path: TS 36.508 Table 8.1.8.2.1.7-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddMod-NB-exPDCP(bid) ::= SEQUENCE { |  |  |  |
| pdcp-Config-r13 | Not present |  |  |
| } |  |  |  |

[End of change]