3GPP TSG-RAN5 Meeting #105	R5-246400

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3347 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to PDCCH-Config |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The implementation of the field SearchSpaceExt with the conditions DCI_1_3, DCI_0_3 and DCI_2_9 in PDCCH-Config is not following RAN5 PRD13. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reworked the content of SearchSpaceExt. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specific message of PDCCH-Config content will not be as per RAN5 PRD13. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### –	PDCCH-Config

Table 4.6.3-95: PDCCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList | Not present |  | EN-DC, MSS |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet | entry 1 |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace with condition USS | entry 1 |  |
|  | SearchSpace with condition MSS | entry 1 | MSS |
|  | SearchSpace with condition SLSS | entry 1 | SLSS |
|  | SearchSpace with condition DCI_2_6 | entry 1 | DCI_2_6 |
|  | SearchSpace with condition DCI_2_9 | entry 1 | DCI_2_9 |
|  | SearchSpace with condition DCI_MC | entry 1 | DCI_1_3, DCI_0_3 |
| } |  |  |  |
| searchSpacesToReleaseList | Not present |  |  |
| downlinkPreemption | Not present |  |  |
| tpc-PUSCH | Not present |  |  |
| tpc-PUCCH | Not present |  |  |
| tpc-SRS | Not present |  |  |
| controlResourceSetToAddModListSizeExt-v1610 | Not present |  |  |
| controlResourceSetToReleaseListSizeExt-r16 | Not present |  |  |
| searchSpacesToAddModListExt-r16 | Not present |  |  |
| searchSpacesToAddModListExt-r16 SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-r16 { | 1 entry |  | DCI_2_6 |
| SearchSpaceExt-r16[1] SEQUENCE{ |  | entry 1 |  |
| controlResourceSetId-r16 | ControlResourceSetId |  |  |
| searchSpaceType-r16 SEQUENCE { |  |  |  |
| common-r16 SEQUENCE { |  |  |  |
| dci-Format2-4-r16 | Not present |  |  |
| dci-Format2-5-r16 | Not present |  |  |
| dci-Format2-6-r16 SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceGroupIdList-r16 | Not present |  |  |
| freqMonitorLocations-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| uplinkCancellation-r16 | Not present |  |  |
| monitoringCapabilityConfig-r16 | Not present |  |  |
| searchSpaceSwitchConfig-r16 | Not present |  |  |
| searchSpacesToAddModListExt-v1700 | Not present |  |  |
| searchSpacesToAddModListExt-v1700 SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1700 { | 1 entry |  | MSS |
| SearchSpaceExt-v1700[1] SEQUENCE { |  | entry 1 |  |
| monitoringSlotPeriodicityAndOffset-v1710 | Not present |  |  |
| monitoringSlotsWithinSlotGroup-r17 | Not present |  |  |
| duration-r17 | Not present |  |  |
| searchSpaceType-r17 SEQUENCE { |  |  |  |
| common-r17 SEQUENCE { |  |  |  |
| dci-Format4-0-r17 | Not present |  |  |
| dci-Format4-1-r17 | Not present |  |  |
| dci-Format4-2-r17 | Not present |  |  |
| dci-Format4-1-AndFormat4-2-r17 SEQUENCE { |  |  |  |
| } |  |  |  |
| dci-Format2-7-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceGroupIdList-r17 | Not present |  |  |
| searchSpaceLinkingId-r17 | Not present |  |  |
| } |  |  |  |
| monitoringCapabilityConfig-v1710 | Not present |  |  |
| searchSpaceSwitchConfig-r17 | Not present |  |  |
| pdcch-SkippingDurationList-r17 | Not present |  |  |
| pdcch-MonitoringResumptionAfterNack-r18 | Not present |  |  |
| searchSpacesToAddModListExt-v1800 | Not present |  |  |
| searchSpacesToAddModListExt-v1800 SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1800 { | 1 entry |  | DCI_1_3, DCI_0_3, DCI_2_9 |
| SearchSpaceExt-v1800 [1] SEQUENCE { |  | entry 1 |  |
| searchSpaceType-r18 CHOICE { |  |  |  |
| ue-Specific-r18 SEQUENCE { |  |  | DCI_1_3, DCI_0_3 |
| dci-FormatsMC-r18 | formats1-3 |  | DCI_1_3 |
|  | formats0-3 |  | DCI_0_3 |
|  | formats0-3-And-1-3 |  | DCI_1_3 AND DCI_0_3 |
| } |  |  |  |
| common-r18 SEQUENCE { |  |  | DCI_2_9 |
| dci-Format2-9-r18 SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MSS | SearchSpace for MBS Multicast receptioin |
| SLSS | SearchSpace for SL mode 1 transmission |
| DCI_2_6 | Transmission of DCI_2_6 is required |
| DCI_2_9 | Transmission of DCI_2_9 is required |
| DCI_1_3 | Transmission of DCI_1_3 is required |
| DCI_0_3 | Transmission of DCI_0_3 is required |