3GPP TSG-RAN5 Meeting #105	R5-246413

# Orlando, USA, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5364 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to EMM TC 9.2.3.1.23 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TP2 is not implemented correctly in the message sequence. T3430 expiry may not happen. TP4 is implemented in such a way that T3430 may not expire. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to achieve TP2, steps 20A and 20B were added to ensure T3430 expiry in any case. Step 25 was deleted, ensuring that T3430 will happen at the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TP2 may not be realized. TP4 may not be realized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.1.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related to R5s240519 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 9.2.3.1.23	Normal tracking area update / Abnormal case / Success after several attempts due to no network response / TA belongs to TAI list and status is UPDATED / TA does not belong to TAI list or status is not UPDATED

9.2.3.1.23.1	Test Purpose (TP)

(1)

with { UE has sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Periodic updating' and has the tracking area updating attempt counter set to the value less than four, the TAI of the current serving cell is included in the TAI list and the update status is equal to EU1 UPDATED }

ensure that {
  when { UE detects release of the NAS signalling connection }

then { UE keeps the update status to EU1 UPDATED, enters state EMM-REGISTERED.NORMAL-SERVICE and starts timer T3411 }

}

(2)

with { UE has sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'Periodic updating', has the tracking area updating attempt counter set to the value less than four, has detected T3430 expiry, the TAI of the current serving cell is included in the TAI list and the update status is equal to EU1 UPDATED }

ensure that {
  when { UE detects T3411 expiry }

then { UE initiates the tracking area updating procedure }

}

(3)

with { UE has sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'TA updating' or 'combined TA/LA updating' and has the tracking area updating attempt counter set to the value less than four and the TAI of the current serving cell is not included in the TAI list or the update status is different to EU1 UPDATED }

ensure that {
  when { UE detects release of the NAS signalling connection }

then { UE starts timer T3411, sets the update status to EU2 NOT UPDATED and changes to state EMM-REGISTERED.ATTEMPTING-TO-UPDATE }

}

(4)

with { UE has sent a TRACKING AREA UPDATE REQUEST message with EPS update type set to 'TA updating' or 'combined TA/LA updating', has the tracking area updating attempt counter set to the value less than four, has detected T3430 expiry and the TAI of the current serving cell is not included in the TAI list or the update status is different to EU1 UPDATED }

ensure that {
  when { UE detects T3411 expiry }

then { UE initiates the tracking area updating procedure }

}

9.2.3.1.23.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.5.3.1, 5.5.3.2.6 and 5.5.3.3.6.

[TS 24.301, clause 5.5.3.1]

A tracking area updating attempt counter is used to limit the number of subsequently rejected tracking area update attempts. The tracking area updating attempt counter shall be incremented as specified in subclause 5.5.3.2.6. Depending on the value of the tracking area updating attempt counter, specific actions shall be performed. The tracking area updating attempt counter shall be reset when:

-	an attach or combined attach procedure is successfully completed;

-	a normal or periodic tracking area updating or a combined tracking area updating procedure is successfully completed; or

-	a normal or periodic tracking area updating or a combined tracking area updating procedure is rejected with EMM cause #11, #12, #13, #14, #15 or #25.

Additionally the tracking area updating attempt counter shall be reset when the UE is in substate EMM-REGISTERED.ATTEMPTING-TO-UPDATE or EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM, and:

-	a new tracking area is entered; or

-	timer T3402 expires.

[TS 24.301, clause 5.5.3.2.6]

The following abnormal cases can be identified:

...

b)	Lower layer failure or release of the NAS signalling connection before the TRACKING AREA UPDATE ACCEPT or TRACKING AREA UPDATE REJECT message is received

The tracking area updating procedure shall be aborted, and the UE shall proceed as described below.

c)	T3430 timeout

The UE shall abort the procedure and proceed as described below. The NAS signalling connection shall be released locally.

...

For the cases b, c, d, e, and f, the UE shall stop any ongoing transmission of user data.

For the cases b, c and d the UE shall proceed as follows:

Timer T3430 shall be stopped if still running. The tracking area updating attempt counter shall be incremented, unless it was already set to 5.

If the tracking area updating attempt counter is less than 5, and the TAI of the current serving cell is included in the TAI list and the EPS update status is equal to EU1 UPDATED:

-	the UE shall keep the EPS update status to EU1 UPDATED and enter state EMM-REGISTERED.NORMAL-SERVICE. The UE shall start timer T3411.

If in addition the TRACKING AREA UPDATE REQUEST indicated "periodic updating", the timer T3411 may be stopped when the UE enters EMM-CONNECTED mode.

If  timer T3411 expires the tracking area updating procedure is triggered again.

If the tracking area updating attempt counter is less than 5, and the TAI of the current serving cell is not included in the TAI list or the EPS update status is different to EU1 UPDATED:

-	the UE shall start timer T3411, shall set the EPS update status to EU2 NOT UPDATED and change to state EMM-REGISTERED.ATTEMPTING-TO-UPDATE. When timer T3411 expires the tracking area updating procedure is triggered again.

If A/Gb mode or Iu mode is supported by the UE, the UE shall in addition handle the GPRS update status as specified in 3GPP TS 24.008 [13] for the abnormal case when a normal or periodic routing area updating procedure fails and the routing area updating attempt counter is less than 5 and the GPRS update status is different from GU1 UPDATED.

[TS 24.301, clause 5.5.3.3.6]

If the tracking area updating attempt counter is incremented according to subclause 5.5.3.2.6 the next actions depend on the value of the tracking area updating attempt counter.

-	if the update status is U1 UPDATED and the tracking area updating attempt counter is less than 5, then the UE shall keep the update status to U1 UPDATED, the new MM state is MM IDLE substate NORMAL SERVICE;

-	if the tracking area updating attempt counter is less than 5 and, additionally, the update status is different from U1 UPDATED UE shall delete any LAI, TMSI, ciphering key sequence number and list of equivalent PLMNs and set the update status to U2 NOT UPDATED. The MM state remains MM LOCATION UPDATING PENDING; or

-	if the tracking area updating attempt counter is equal to 5, the UE shall delete any LAI, TMSI, ciphering key sequence number and list of equivalent PLMNs and set the update status to U2 NOT UPDATED. A UE operating in CS/PS mode 1 of operation shall select GERAN or UTRAN radio access technology and proceed with appropriate MM or GMM specific procedures.

9.2.3.1.23.3	Test description

9.2.3.1.23.3.1	Pre-test conditions

System Simulator:

-	cell A and cell B

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

none.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.3.1.23.3.2	Test procedure sequence

Table 9.2.3.1.23.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of Cell A to the "Serving cell". Set the cell type of Cell B to the "Suitable neighbour intra-frequency cell". | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE is powered on or switched on. | - | - | - | - |
| 3-8a2 | Void | - | - | - | - |
| 8A1-8A10 | Steps 2-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed | - | - | - | - |
| 8A | The SS activates UE radio bearer test mode. | - | - | - | - |
| 9 | The SS responds with an ATTACH ACCEPT message with the T3412 value indicating 6 min. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message | <-- | ATTACH ACCEPT | - | - |
| 10 | Void | - | - | - | - |
| 11 | Void | - | - | - | - |
| 11A | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| 11B-11Db1 | Steps 16 to 18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 12 | Wait for 6 min to ensure that T3412 expires. | - | - | - | - |
| 13 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 14 | The SS releases the RRC connection. NOTE: The tracking area updating attempt counter is 1. | - | - | - | - |
| 15 | The SS pages the UE using S-TMSI with CN domain indicator set to "PS". | - | - | - | - |
| 16 | Check: Does the UE transmit a SERVICE REQUEST message? | --> | SERVICE REQUEST | 1 | P |
| 17 | The SS transmits a SERVICE REJECT message with EMM cause set to "Congestion". | <-- | SERVICE REJECT | - | - |
| 18 | The SS releases the RRC connection. | - | - | - | - |
| 19 | IF pc_ue_CategoryDL_M1 wait for up to 11,25s else wait for 10s after step 14 to ensure that T3411 expires. (Note 2) | - | - | - | - |
| - | EXCEPTION: Steps 19Aa1 to 19Aa4 describes behaviour depending on UE behaviour; the “lower case letter” identifies a step sequence that take place if the UE does not transmit any TRACKING AREA UPDATE REQUEST message | - | - | - | - |
| 19Aa1 | IF the UE does not transmit any TRACKING AREA UPDATE REQUEST message THEN wait for 6 min to ensure that T3412 expires. | - | - | - | - |
| 19Aa2 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to ‘Periodic updating’. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 19Aa3 | The SS releases the RRC connection. | - | - | - | - |
| 19Aa4 | Wait for 10s after step 19Aa3 to ensure that T3411 expires. | - | - | - | - |
| 20 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 20A | Wait for 25s after step 20 to ensure that T3430 and T3411 expire. | - | - | - | - |
| 20B | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'Periodic updating'? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 21 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. NOTE: The tracking area updating attempt counter is reset. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 22 | The SS releases the RRC connection. | - | - | - | - |
| 22A | Generic test procedure in TS 36.508 subclause 4.5.3A.3 is performed. NOTE: The UE performs the establishment of the new data radio bearer associated with the default EPS bearer context. | - | - | - | - |
| 22B | Generic test procedure in TS 36.508 subclause 4.5.4.3 is performed. NOTE: The UE enters the UE test loop mode. | - | - | - | - |
| 22C | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context. | - | - | - | - |
| 22D | Wait for 1 s after the IP packet has been transmitted in step 22C. (Note 1) | - | - | - | - |
| 22E | The SS releases the RRC connection. | - | - | - | - |
| 23 | Set the cell type of Cell A to the "non-Suitable cell". Set the cell type of Cell B to the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell B unless explicitly stated otherwise. | - | - | - | - |
| 24 | The UE transmits a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'TA updating' or ‘combined TA/LA updating’. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 24A | Wait for 15s after step 24 to ensure that T3430 expires. | - | - | - | - |
| 25 | Void | - | - | - | - |
| 26 | Void | - | - | - | - |
| 27 | Check: Does the UE transmit a SERVICE REQUEST message within 5s? | --> | SERVICE REQUEST | 3 | F |
| 28 | Wait for 25s after step 24 to ensure that T3430 and T3411 expire. | - | - | - | - |
| 29 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message with the EPS update type value set to 'TA updating' or ‘combined TA/LA updating’? | --> | TRACKING AREA UPDATE REQUEST | 4 | P |
| 30 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| - | EXCEPTION: Step 31 and Step 32a1 can occur in any order. | - | - | - | - |
| 31 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. NOTE: The tracking area updating attempt counter is reset. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | EXCEPTION: Step 32a1 describes behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE has user data pending. | - | - | - | - |
| 32a1 | IF the UE has user data pending THEN the UE loop backs the IP packet received in step 22C on the DRB associated with the default EPS bearer context on Cell B within 5s. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |
| Note 1: The 1 second delay is used to secure that the UE have received and forwarded the IP Packet transmitted by the SS in step 22C to the UE test loop function before the RRCConnectionRelease message is sent by the SS in step 22E. Note 2: According to TS 36.331 cl. 5.3.8.3 Cat M1 UEs may start T3411 only 1,25 sec after having received RRC Connection Release. T3411 will therefore elapse in the interval from 9 sec to 12.375 sec. |  |  |  |  |  |

9.2.3.1.23.3.3	Specific message contents

Table 9.2.3.1.23.3.3-0: ACTIVATE TEST MODE (step 8A, Table 9.2.3.1.23.3.2-1)

Derivation Path: 36.508, Table 4.7A-1, condition UE TEST LOOP MODE B

Table 9.2.3.1.23.3.3-1: Message ATTACH ACCEPT (step 9, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| T3412 value |  | 6 minutes |  |
| Timer value | '0 0001'B |  |  |
| Unit | '010'B | value is incremented in multiples of decihours |  |
| GUTI | GUTI-1 |  |  |

Table 9.2.3.1.23.3.3-2: Message TRACKING AREA UPDATE REQUEST (step 13, step 19Aa2 and step 20, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '011'B | Periodic updating |  |
| Old GUTI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |

Table 9.2.3.1.23.3.3-3: Message TRACKING AREA UPDATE ACCEPT (step 21, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| GUTI |  | Not present |  |  |  |  |  |
| MS identity |  | Not present |  |  |  |  |  |

Table 9.2.3.1.23.3.3-3A: CLOSE UE TEST LOOP (step 22B, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0001 0100'B | 20seconds |  |

Table 9.2.3.1.23.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 24, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Old GUTI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |

Table 9.2.3.1.23.3.3-5: Message TRACKING AREA UPDATE REQUEST (step 29, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| "Active" flag | Any allowed value | The UE may set this flag due to failing SERVICE REQUEST procedure. |  |
| Old GUTI | GUTI-1 |  |  |
| Last visited registered TAI | TAI-1 |  |  |

Table 9.2.3.1.23.3.3-6: Message TRACKING AREA UPDATE ACCEPT (step 30, Table 9.2.3.1.23.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| GUTI | GUTI-2 |  |  |