3GPP TSG-RAN5 Meeting #105	R5-246421

# Orlando, United States, 18th - 22nd November 2024

Agenda Item:	6.6.9.9

Source:	MCC TF160

Title:	MCData: Conformance Testing of IP Connectivity

Document for:	Endorsement

# 1	Introduction

At RAN5#104 MCData test cases 6.7.1 and 6.7.2 have been added introducing test coverage for IP connectivity in Rel-16 using GRE (Generic Routing Encapsulation) for the IP tunnel between two MCData clients.

The below figure illustrates the IP tunnel between two clients (see also 23.282 clause 5.11).

# 2	Discussion

### 2.1	Assumption and general understanding

- In Rel-16 IP connectivity uses GRE (Generic Routing Encapsulation) according to RFC 2784 for the IP tunnel whereas from Rel-17 onward GRE-in-UDP Encapsulation according to RFC 8086 is used.

- The client acts as relay for the IP packets from/to the application; it does encapsulation and decapsulation of the IP packets sent from/to the application.

- In general, the IP addresses used by the application are different from the client's IP addresses and in principle the application can use IPv4 or IPv6 through an IPv4 or IPv6 tunnel.

- In general, there are no restrictions regarding the kind of application (e.g. which protocols it uses) and it can even be hosted on an external device.

### 2.2	Issues with Rel-16

- Rel-16 is not compatible with Rel-17/18: 
From Rel-17 onward IP Connectivity uses GRE-in-UDP Encapsulation according to RFC 8086 and a note in clause 20.1 of 24.282 says "IP Connectivity specified in the current document is not compatible with release 16".
NOTE: The major difference of GRE and GRE-in-UDP is that GRE encapsulates the payload (IP packet) directly into an IP(v4) packet with protocol type 47 (GRE) whereas GRE-in-UDP encapsulates the payload into UDP packets.

- According to clause 9 in RFC 2784 "IPv6 as delivery and/or payload protocol is not included in the currently deployed versions of GRE". There is RFC 7676 "IPv6 Support for Generic Routing Encapsulation (GRE)" but there are no references to this RFC in 24.282 or 24.582.
 In Rel-16 IP connectivity seems to be restricted to IPv4.

- In Rel-16 there is only clause 20.1.1 in 24.282 specifying "MC Data client SDP offer/answer generation" by referring to 24.229 but in contrast to Rel-17/18 there is nothing in clause 13 of 24.582 specifying the media description (m= line) or any SDP "fmtp" attributes.
 For Rel-16 neither the media type nor the transport protocol of the m= line is specified.
Furthermore, it is not clarified in clause 20.1 of 24.282 how exactly the port number of the m= line shall be set: GRE itself does not have a port number and a port number of zero would be used according to RFC 3264 e.g. to indicate that no resources are needed or to release a stream.

### 2.3	General issue with testing of the IP tunnel

As stated in clause 2.1 in general there are no restrictions regarding the kind of application using the IP tunnel.
 The IP traffic (IP packets) exchanged between the application endpoints is not specified.

To test the UL (MCData test case 6.7.1) the application hosted on the MCData client may be triggered to send "something" so that the test case gets one (or several) payloads of IP packets (Rel-16) or of UDP packets (Rel-17 and later). These payloads basically consist of a GRE header and the IP packet sent by the application. The GRE header can be checked by the test case, the IP packet should have a valid IP header, its payload does not need to be checked.  In general, it should be possible to test the IP tunnel in UL.

But in the first place there is no way to test the DL: To check that the tunnel works the DL the test case would need to send one or several IP packets triggering a specific behaviour at the application. This could be that the application informs the user or that the application sends a response in UL.

Nevertheless an easy way to test the IP tunnel in DL would be to use "ping" - if supported by the application: The test case sends an ICMP ECHO request through the tunnel and waits for the ICMP ECHO reply.

If there is nothing like "ping" then checking the IP tunnel in DL may not be possible.

### 2.4	MCData conformance test cases

As test cases 6.7.1 and 6.7.2 are for Rel-16 there are the issues as described in clause 2.2 and in addition for test case 6.7.2 there is the issue with check of the IP tunnel in DL as described in clause 2.3. The implementation of these test cases would require extension of the test model for support of GRE. In addition, especially for test case 6.7.2 it is not clear which media description to be used for SDP signalling. In contrast to Rel-16 it seems that the GRE-in-UDP signalling of Rel-17/18 has less or even no impact on the test model.

# 3	Conclusions

### 3.1	Rel-16 vs. Rel-17/18

Assuming that Rel-16 IP connectivity test cases cannot be verified due to the issue described in clause 2 the test cases will not be implemented and there will be no extension of the test model to support GRE as transport protocol. It is proposed either to remove MCData test cases 6.7.1 and 6.7.2 and to introduce new test cases for Rel17/18 or to update test cases 6.7.1 and 6.7.2 to Rel17/18.

### 3.2	Test of IP tunnel in DL

Further investigations are needed regarding the testability of the IP tunnel in DL.