3GPP TSG-RAN5 Meeting #105	R5-246492

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3503 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Core spec alignment update to LTM PDCCH order TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek India Technology Pvt. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Core spec alignment. Update TCs according to RAN4 TS 38.133 i70 clause A.6.3.2.4 - Add message content based on 38.331 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Core spec alignment to several changes including [ ] deletion, message content update and typo fix. - Corresponding editors notes are removed. - Typo fixed - Add includeBeamMeasurements=True, maxNrofRS-IndexesToReport=1 and measurement gap pattern ID for TC 6.3.2.4.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be misalignment between RAN4 and RAN5 spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | To spec implementation staff, Pls be awared of format change in the beginning of clause 6.3.2.4.2.5 and 6.3.2.4.3.5. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

6.3.2.4.1	PDCCH-order RACH on neighbour cell in FR1 when RACH BW is within active UL BWP

Editor's Note: This test case is incomplete in following aspects:

LTM specific configurations are missing.

Test applicability is missing in TS 38.522.

TT analysis is missing.

[ ] still exist in this case

6.3.2.4.1.1	Test Purpose

This test is to verify the requirement for PDCCH-order RACH on neighbour cell in FR1 when RACH BW is within active UL BWP specified in TS 38.213 clause 8.1 [8] and UE transmit timing in TS 38.133 [6] clause 7.1 for UE supporting rach-EarlyTA-Measurement-r18.

6.3.2.4.1.2	Test applicability

This test applies to all types of NR UE from Release 18 and forward.

6.3.2.4.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.3.2.4.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.3.2.4.1.

6.3.2.4.1.4	Test description

6.3.2.4.1.4.1	Initial conditions

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. Test configurations are given in table 6.3.2.4.1.4-1. No gap patterns are configured in the test case.

Table 6.3.2.4.1.4-1: PDCCH order RACH on Neighbour cell in FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Candidate cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Candidate cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Candidate: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.3.2.4.1.4-2.

Table 6.3.2.4.1.4-2: Initial conditions for PDCCH-order RACH on Neighbour cell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.4.1.4-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | No fader. |  |  |

1.	Message contents are defined in clause 6.3.2.4.1.4.3.

2.	Cell 1 is the NR FR1 serving cell (PCell). Cell 2 is the NR FR1 neighbour cell. The UE has to report L3 measurement results and performs SSB based L1-RSRP measurement on Cell 2.

6.3.2.4.1.4.2	Test procedure

This test case contains 3 tests (Test 1, 2, and 3) and UE may have to pass one of the tests based on the defined conditions.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.3.2.4.1.5-1 and 6.3.2.4.1.5-2.

3.	The UE shall establish a connection setup with SS Cell 1, the random access procedure within the connection setup is used in the test.

4.	The SS shall transmit an RRCReconfiguration message to configure event A3 triggered measurement reporting.

5.	The UE shall transmit a MeasurementReport message triggered by Event A3.

6.	The SS sends LTM-Candidate-r18 for Cell 2.

7.	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2, and UE shall report a valid L1-RSRP result of Cell 2.

If a UE supports ltm-MAC-CE-JointTCI-r18, it is only required to pass test 1. If a UE supports ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18, it is only required to pass test 2. If a UE supports neither ltm-MAC-CE-SeparateTCI-r18 nor ltm-MAC-CE-JointTCI-r18, it is only required to pass test 3.

8.	Test 1: Joint TCI state configuration

8.1.	T1 starts from UE transmitting a valid L1 report of Cell 2.

8.2.	The SS sends TCI state activation MAC CE to active CandidateTCI-State#1 of Cell 2 no later than T1 + 100 ms.

8.3.	The start of T2 is the instant when DCI 1_0 PDCCH order to trigger PRACH transmission on Cell 2 is sent to the UE. The SS change the parameters according to T2 in Table 6.3.2.4.1.5-1 and 6.3.2.4.1.5-2. The SS shall schedule on cell 1 continuously for both UL and DL at available slots.

8.4.	The UE shall transmit the PRACH preamble to Cell 2. If the UE transmits the PRACH preamble in the first available PRACH occasion after  NT,2 + 0.5ms + TSSB (specified in clause 6.3.2.4.1.5) from the beginning of T2, then count a success for the event "PRACH occasion". Otherwise count a fail for the event "PRACH occasion".

8.5.	If the timing of PRACH transmission on Cell 2 is within (NTA + NTA_offset) ×Tc ± Te (specified in clause 6.3.2.4.1.5) of the first detected path of DL SSB of Cell 2, then count a success for the event "PRACH timing”. Otherwise, count a fail for the event "PRACH timing".

8.6.	During T2, if UL DTX is only observed by the SS within [the overlapped slot to transmit PRACH] and 2 symbols before and after the PRACH occasion, and DL DTX is only observed by the SS within the overlapped slot to transmit PRACH, then count a success for the event "Pcell DTX”. Otherwise, count a fail for the event "Pcell DTX".

8.7.	Repeat step 1-8 until a test verdict has been achieved.

9.	Test 2: Separate TCI state configuration

Test steps are the same as Step 8 with following exceptions:

9.2.	The SS sends TCI state activation MAC CE to active CandidateTCI-State#1 and CandidateTCI-UL-State #1 of Cell 2 no later than T1 + 100 ms.

Repeat step 1-7 and 9 until a test verdict has been achieved.

10.	Test 3: No candidate TCI state configurations

Test steps are the same as Step 8 with following exceptions:

The SS shall not send any TCI state activation MAC CE to active TCI state of Cell 2.

Repeat step 1-7 and 10 until a test verdict has been achieved.

The rate of correct events observed during repeated tests shall be at least 90%.

Each of the events "PRACH occasion", "Pcell DTX" as well as "PRACH timing" are evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. Different events may require different times for a verdict. If all events pass, the test passes. If one event fails, the test fails.

6.3.2.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 6.3.2.4.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 Table H.3.1-7 with conditions INTRA-FREQ, SSB Index Table A.7.1-2 |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with condition INTRA-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.3.2.4.1.4.3-2: ReportConfigNR

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | -12 | The actual value is field value * 0.5 dB |  |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | True |  |  |
| } |  |  |  |
| } |  |  |  |

6.3.2.4.1.5	Test requirement

Both PDCCH order RACH delay, transmit timing requirement and the interruption requirements are tested by using the parameters in table 6.3.2.4.1.5-1, and 6.3.2.4.1.5-2.Table 6.3.2.4.1.5-1: General test parameters for PDCCH order RACH in FR1

| Parameter |  | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |  |
|  | Neighbouring cell |  | Cell 2 |  |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 1 |  |  | After transmitting PRACH on Cell 2, UE shall be back to Cell 1. |
| A3-Offset |  | dB | -6 |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |
| Time To Trigger |  | ms | 0 |  |  |  |
| Filter coefficient |  |  | 0 |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  | DRX is not used |
| Time offset between cells |  |  | 2 s |  |  | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |
| EarlyUL-SyncConfig | frequencyInfoUL |  | NR RF Channel Number 1 |  |  | Same as Cell 1 |
|  | PRACH configuration |  | FR1 PRACH configuration 5 |  |  | RACH bandwidth is within active UL BWP of Cell 1 |
|  | bwp-GenericParameters |  | ULBWP.0.1 |  |  |  |
|  | n-TimingAdvanceOffset | Tc | 25600 |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DlorJoint TCI.State.0 | DlorJoint TCI.State.2 | N/A | As specified in TS 38.133 [6] clause A.3.16B. Configured for early TCI state activation for test 1 and test 2. |
| Ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | As specified in TS 38.133 [6] clause A.3.16B. Configured for early TCI state activation for test 2. |
| Ltm-ConfigComplete |  |  | True |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |  |  |
| T2 |  | s | 0.5 |  |  |  |

Table 6.3.2.4.1.5-2: Cell specific test parameters for PDCCH order RACH test case

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  | N/A |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | N/A |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | N/A |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | N/A |  |  |
| CP length |  |  |  | Normal |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |
|  |  |  | dB | -0.64 | -0.64 |  | -0.64 | -0.64 |
|  |  |  | dB | 8 | 8 |  | 8 | 8 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -90 | -90 |
|  | Config 3 |  | dBm/SCS | -87 | -87 |  | -87 | -87 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -58.7 | -58.7 |  | -58.7 | -58.7 |
|  | Config 3 |  | dBm/ 38.16MHz | -52.6 | -52.6 |  | -52.6 | -52.6 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The UE shall transmit the PRACH preamble to Cell 2 in the first available PRACH occasion after  NT,2 + 0.5ms + TSSB from the beginning of time period T2. After transmitting PRACH on Cell 2, UE shall retune back to Cell 1.

NOTE:	The PDCCH order RACH delay can be expressed as: NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation, where:

-	NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission and is specified in Table 6.4-1 in 38.214 [9].

-	TBWPswitchDelay= 0, Tswitch= 0, ∆RF/BB preparation= 0

-	∆Delay= 0.5ms

-	TSSB= 0 for UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18, otherwise TSSB= Tfirst-SSB_RACH + TSSB-proc , where Tfirst-SSB_RACH is [the time to first SSB after 1 slot from the end of the slot that UE receives PDCCH-order] , and TSSB-proc = 2 ms, which is the time for SSB processing.

During T2, interruption on Cell 1 UL shall not happen outside [the overlapped slot to transmit PRACH] and N symbols before and after the PRACH occasion as defined in clause 8.1 in 38.213 [8], where N=2. During T2, interruption on Cell 1 DL shall not occur outside the overlapped slot to transmit PRACH.

The test equipment will verify that the timing of PRACH transmission on Cell 2 is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB of Cell 2.

a.	The NTA_offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in TS 38.133 [6] Table 7.1.2-1.

The rate of correct events observed during repeated tests shall be at least 90%.

<<< END OF CHANGES 1>>>

<<< START OF CHANGES 2>>>

6.3.2.4.2	PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM

Editor's Note: This test case is incomplete in following aspects:

LTM specific configurations are missing.

Test applicability is missing in TS 38.522.

TT analysis is missing.

[ ] still exist in this case

Interruption requirement needs to be clarified by RAN4

6.3.2.4.2.1	Test Purpose

This test is to verify the requirement for the NR FR1-NR FR1 PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM. This test is for UE supporting PDCCH-ordered RACH to an inter-frequency candidate cell, whose SSB is outside active BWPs of the UE.

6.3.2.4.2.2	Test applicability

This test applies to all types of NR UE from Release 18 and forward.

6.3.2.4.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.3.2.4.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.3.2.4.2.

6.3.2.4.2.4	Test description

6.3.2.4.2.4.1	Initial conditions

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on a different frequency from the PCell. Test configurations are given in Table 6.3.2.4.2.4-1. Measurement gap patterns are configured in the test case and the SSB of Cell2 is outside the active BWP of Cell 1. The PCell is continuously scheduled during the whole test.

Table 6.3.2.4.2.4-1: Inter-frequency PDCCH-ordered RACH from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.3.2.4.2.4-2.

Table 6.3.2.4.2.4-2: Initial conditions for PDCCH-order RACH on Neighbour cell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.4.2.4-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | No fader. |  |  |

1.	Message contents are defined in clause 6.3.2.4.2.4.3.

2.	Cell 1 is the NR FR1 serving cell (PCell). Cell 2 is the NR FR1 neighbour cell.

6.3.2.4.2.4.2	Test procedure

This test case contains 2 tests and UE is required to pass one among Test 1, Test 2.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.3.2.4.2.5-1 and 6.3.2.4.2.5-2.

3.	The UE shall establish a connection setup with SS Cell1, the random access procedure within the connection setup is used in the test.

4.	The SS shall transmit an RRCReconfiguration message to configure event A3 triggered measurement reporting.

5.	The UE shall transmit a MeasurementReport message triggered by Event A3.

6.	The SS sends LTM-Candidate-r18 for Cell 2.

7.	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2, and UE shall report a valid L1-RSRP result of Cell 2.

If a UE supports ltm-MAC-CE-JointTCI-r18, it is only required to pass test 1. If a UE supports neither ltm-MAC-CE-SeparateTCI-r18 nor ltm-MAC-CE-JointTCI-r18, it is only required to pass test 2.

8.	Test 1: Joint TCI state configuration

8.1.	T1 starts when UE receiving candidate cell TCI state activation MAC CE to active CandidateTCI-State#1 for Cell 2.

8.2.	The start of T2 is the instant when DCI 1_0 PDCCH order to trigger PRACH transmission on Cell 2 is sent to the UE. The SS change the parameters according to T2 in Table 6.3.2.4.2.5-1 and 6.3.2.4.2.5-2. The SS shall schedule on cell 1 continuously for both UL and DL at available slots.

8.3.	The UE shall transmit the PRACH preamble to Cell 2. If the UE transmits the PRACH preamble in the first available PRACH occasion after  NT,2 + 0.5ms + TSSB (specified in clause 6.3.2.4.2.5) from the beginning of T2, then count a success for the event "PRACH occasion". Otherwise count a fail for the event "PRACH occasion".

8.4.	If the timing of PRACH transmission on Cell 2 is within (NTA + NTA_offset) ×Tc ± Te (specified in clause 6.3.2.4.2.5) of the first detected path of DL SSB of Cell 2, then count a success for the event "PRACH timing”. Otherwise, count a fail for the event "PRACH timing".

8.5.	During T2, if UL DTX is only observed by the SS within [the overlapped slot to transmit PRACH] and 2 symbols before and after the PRACH occasion, and DL DTX is only observed by the SS within the overlapped slot to transmit PRACH, then count a success for the event "Pcell DTX”. Otherwise, count a fail for the event "Pcell DTX".

8.6.	Repeat step 1-8 until a test verdict has been achieved.

9.	Test 2: No candidate TCI state configurations

Test steps are the same as Step 8 with following exceptions:

T1 is skipped in Test 2. The SS shall not send any TCI state activation MAC CE to active TCI state of Cell 2.

Repeat step 1-7 and 9 until a test verdict has been achieved.

The rate of correct events observed during repeated tests shall be at least 90%.

Each of the events "PRACH occasion", "Pcell DTX" as well as "PRACH timing" are evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. Different events may require different times for a verdict. If all events pass, the test passes. If one event fails, the test fails.

6.3.2.4.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 6.3.2.4.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ and GAP NEEDED Table H.3.1-5 Table H.3.1-7 with conditions INTER-FREQ, SSB Index Table A.7.1-1 with PRACH.6 FR1 Table 9.1.2-1 in 38.133 pattern ID 1 |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with condition INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.3.2.4.2.4.3-2: ReportConfigNR

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | -12 | The actual value is field value * 0.5 dB |  |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | True |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.4.2.4.3-3: MeasGapConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-70 with condition GAP_FR1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapFR1 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mgrp | ms80 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.3.2.4.2.5	Test requirement

Both PDCCH order RACH delay, transmit timing requirement and the interruption requirements are tested by using the parameters in table 6.3.2.4.2.5-1 and 6.3.2.4.2.5-2.

Table 6.3.2.4.2.5-1: General test parameters for Inter-frequency PDCCH-ordered RACH test from FR1 to FR1 from FR1 to FR1

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |
|  | Neighbouring cell |  | Cell 2 |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 1 |  |  |
| A3-Offset |  | dB | -6 |  |  |
| Hysteresis |  | dB | 0 |  |  |
| Time To Trigger |  | s | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| Measurement gap periodicity |  | ms | 80 |  |  |
| DRX |  |  | OFF |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  | No additional delays in random access procedure. |
| Time offset between cells |  |  | 2 s |  | RTD between cells is less than CP |
| reportQuantityRS-Indexes |  |  | Rsrp |  |  |
| maxNrofRS-IndexesToReport |  |  | 1 |  |  |
| includeBeamMeasurements |  |  | True |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |
|  | spCellInclusion |  | N/A |  |  |
| EarlyUL-SyncConfig | frequencyInfoUL |  | NR RF Channel Number 2 |  | Cell 2 |
|  | PRACH configuration |  | FR1 PRACH configuration 6 |  |  |
|  | bwp-GenericParameters |  | ULBWP.0.1 |  |  |
|  | n-TimingAdvanceOffset | Tc | 25600 |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DLorJoint TCI.State.0 | N/A | As specified in TS 38.133 [6] clause A.3.16B. In test 1, CandidateTCI-State#1 is configured for early TCI state activation. In test 2, CandidateTCI-State#1 is configured for TCI state indication in cell switch command. |
| ltm-ConfigComplete |  |  | True |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.1 |  |  |
| T2 |  | s | 0.2 |  |  |

Table 6.3.2.4.2.5-2: Cell specific test parameters for NR FR1-FR1 Inter-frequency PDCCH-ordered RACH test

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1-T2 | T1-T2 |
| NR RF Channel Number |  |  |  | 1 | 2 |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DRx Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  |
| TRS configuration for serving cell |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |
|  |  | Config 3 |  | 30 |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |
|  |  | Config 3 |  | 30 |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 6 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |
|  | Config 3 |  |  | -95 |  |
|  |  |  | dB | 8 | 4 |
|  |  |  | dB | 8 | 4 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -87 | -90 |
|  | Config 3 |  | dBm/SCS | -84 | -87 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -57.06 | -61.41 |
|  | Config 3 |  | dBm/ 9.36MHz | -50.96 | -55.31 |
| Propagation condition |  |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

The UE shall transmit the PRACH preamble to Cell 2 in the first available PRACH occasion after  NT,2 + 0.5ms + TSSB from the beginning of time period T2. After transmitting PRACH on Cell 2, UE shall retune back to Cell 1.

NOTE:	The PDCCH order RACH delay can be expressed as: NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation, where:

-	NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission and is specified in Table 6.4-1 in 38.214 [9].

-	TBWPswitchDelay= 0, Tswitch= 0∆RF/BB preparation

-	∆Delay= 0.5ms

-	∆RF/BB preparation is defined in 6.2.2C

-	TSSB= 0 for UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18, otherwise TSSB= Tfirst-SSB_RACH + TSSB-proc, where Tfirst-SSB_RACHis [the time to first SSB occasion overlapped with MGL after [2]ms and 1 slot from the end of the slot that UE receives PDCCH-order], and TSSB-proc = 2 ms, which is the time for SSB processing.

TSSBTSSB

During T2, interruption on Cell 1 UL shall not happen outside [the overlapped slot to transmit PRACH] and N symbols before and after the PRACH occasion as defined in clause 8.1 in 38.213 [8], where N=2. During T2, interruption on Cell 1 DL shall not occur outside the overlapped slot to transmit PRACH.

The test equipment will verify that the timing of PRACH transmission on Cell 2 is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB of Cell 2.

a.	The NTA_offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in TS 38.133 [6] Table 7.1.2-1.

The rate of correct events observed during repeated tests shall be at least 90%.

<<< END OF CHANGES 2>>>

<<< START OF CHANGES 3>>>

6.3.2.4.3	PDCCH-order RACH on neighbour cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP

Editor's Note: This test case is incomplete in following aspects:

LTM specific configurations are missing.

Test applicability is missing in TS 38.522.

TT analysis is missing.

6.3.2.4.3.1	Test Purpose

This test is to verify the requirement for PDCCH-order RACH on neighbour cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP specified in TS 38.213 clause 8.1 [8] and UE transmit timing in TS 38.133 [6] clause 7.1 for UE supporting rach-EarlyTA-Measurement-r18..

6.3.2.4.3.2	Test applicability

This test applies to all types of NR UE from Release 18 and forward.

6.3.2.4.3.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.3.2.4.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.3.2.4.3.

6.3.2.4.3.4	Test description

6.3.2.4.3.4.1	Initial conditions

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. Test configurations are given in table 6.3.2.4.3.4-1. No gap patterns are configured in the test case.

Table 6.3.2.4.3.4-1: PDCCH order RACH on Neighbour cell in FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Candidate cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Candidate cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Candidate: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.3.2.4.3.4-2.

Table 6.3.2.4.3.4-2: Initial conditions for PDCCH-order RACH on Neighbour cell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.4.3.4-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | No fader. |  |  |

1.	Message contents are defined in clause 6.3.2.4.3.4.3.

2.	Cell 1 is the NR FR1 serving cell (PCell). Cell 2 is the NR FR1 neighbour cell. The UE has to report L3 measurement results on Cell 2.

6.3.2.4.3.4.2	Test procedure

This test case contains 3 tests (Test 1, 2, and 3) and UE may have to pass one of the tests based on the defined conditions.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.3.2.4.3.5-1 and 6.3.2.4.3.5-2.

3.	The UE shall establish a connection setup with SS Cell 1, the random access procedure within the connection setup is used in the test.

4.	The SS shall transmit an RRCReconfiguration message to configure event A3 triggered measurement reporting.

5.	The UE shall transmit a MeasurementReport message triggered by Event A3.

6.	The SS sends LTM-Candidate-r18 for Cell 2.

If a UE supports ltm-MAC-CE-JointTCI-r18, it is only required to pass test 1. If a UE supports ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18, it is only required to pass test 2. If a UE supports neither ltm-MAC-CE-SeparateTCI-r18 nor ltm-MAC-CE-JointTCI-r18, it is only required to pass test 3.

7.	Test 1: Joint TCI state configuration

7.1.	T1 starts from UE transmitting a L3 report of Cell 2.

7.2.	The SS sends TCI state activation MAC CE to active CandidateTCI-State#1 of Cell 2 no later than T1 + 100 ms.

7.3.	The start of T2 is the instant when DCI 1_0 PDCCH order to trigger PRACH transmission on Cell 2 is sent to the UE. The SS change the parameters according to T2 in Table 6.3.2.4.3.5-1 and 6.3.2.4.3.5-2. The SS shall schedule on cell 1 continuously for both UL and DL at available slots.

7.4.	The UE shall transmit the PRACH preamble to Cell 2. If the UE transmits the PRACH preamble in the first available PRACH occasion after  NT,2 + 0.5ms + TSSB (specified in clause 6.3.2.x.3.5) from the beginning of T2, then count a success for the event "PRACH occasion". Otherwise count a fail for the event "PRACH occasion".

7.5.	If the timing of PRACH transmission on Cell 2 is within (NTA + NTA_offset) ×Tc ± Te (specified in clause 6.3.2.4.3.5) of the first detected path of DL SSB of Cell 2, then count a success for the event "PRACH timing”. Otherwise, count a fail for the event "PRACH timing".

7.6.	During T2, if UL DTX is only observed by the SS within the overlapped slot to transmit PRACH and 2 symbols before and after the PRACH occasion, and DL DTX is only observed by the SS within the overlapped slot to transmit PRACH, then count a success for the event "Pcell DTX”. Otherwise, count a fail for the event "Pcell DTX".

7.7.	Repeat step 1-7 until a test verdict has been achieved.

8.	Test 2: Separate TCI state configuration

Test steps are the same as step 7 with following exceptions:

8.2.	The SS sends TCI state activation MAC CE to active CandidateTCI-State#1 and CandidateTCI-UL-State #1 of Cell 2 no later than T1 + 100 ms.

Repeat step 1-6 and 8 until a test verdict has been achieved.

9.	Test 3: No candidate TCI state configurations

Test steps are the same as Step 7 with following exceptions:

The SS shall not send any TCI state activation MAC CE to active TCI state of Cell 2.

Repeat step 1-6 and 9 until a test verdict has been achieved.

The rate of correct events observed during repeated tests shall be at least 90%.

Each of the events "PRACH occasion", "Pcell DTX" as well as "PRACH timing" are evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. Different events may require different times for a verdict. If all events pass, the test passes. If one event fails, the test fails.

6.3.2.4.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 6.3.2.4.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 Table H.3.1-7 with conditions INTRA-FREQ Table A.7.1-2 |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with condition INTRA-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.3.2.4.3.4.3-2: ReportConfigNR

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | -12 | The actual value is field value * 0.5 dB |  |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | True |  |  |
| } |  |  |  |
| } |  |  |  |

6.3.2.4.3.5	Test requirement

Both PDCCH order RACH delay, transmit timing requirement and the interruption requirements are tested by using the parameters in table 6.3.2.4.3.5-1 and 6.3.2.4.3.5-2.

Table 6.3.2.4.3.5-1: General test parameters for PDCCH order RACH in FR1

| Parameter |  | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |  |
|  | Neighbouring cell |  | Cell 2 |  |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 1 |  |  | After transmitting PRACH on Cell 2, UE shall be back to Cell 1. |
| A3-Offset |  | dB | -6 |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |
| Time To Trigger |  | ms | 0 |  |  |  |
| Filter coefficient |  |  | 0 |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  | DRX is not used |
| Time offset between cells |  |  | 2 s |  |  | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |
| EarlyUL-SyncConfig | frequencyInfoUL |  | NR RF Channel Number 1 |  |  | Same as Cell 1 |
|  | PRACH configuration |  | FR1 PRACH configuration 5 |  |  | RACH bandwidth is within active UL BWP of Cell 1 |
|  | bwp-GenericParameters |  | ULBWP.0.1 |  |  |  |
|  | n-TimingAdvanceOffset | Tc | 25600 |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DlorJoint TCI.State.0 | DlorJoint TCI.State.2 | N/A | As specified in TS 38.133 [6] clause A.3.16B. Configured for early TCI state activation for test 1 and test 2. |
| Ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | As specified in TS 38.133 [6] clause A.3.16B. Configured for early TCI state activation for test 2. |
| Ltm-ConfigComplete |  |  | True |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |  |  |
| T2 |  | s | 0.5 |  |  |  |

Table 6.3.2.4.3.5-2: Cell specific test parameters for PDCCH order RACH test case

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  | N/A |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | N/A |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | N/A |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | N/A |  |  |
| CP length |  |  |  | Normal |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |
|  |  |  | dB | -0.64 | -0.64 |  | -0.64 | -0.64 |
|  |  |  | dB | -1 | -1 |  | -1 | -1 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -90 | -90 |
|  | Config 3 |  | dBm/SCS | -87 | -87 |  | -87 | -87 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -58.7 | -58.7 |  | -58.7 | -58.7 |
|  | Config 3 |  | dBm/ 38.16MHz | -52.6 | -52.6 |  | -52.6 | -52.6 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The UE shall transmit the PRACH preamble to Cell 2 in the first available PRACH occasion after  NT,2 + 0.5ms + TSSB from the beginning of time period T2. After transmitting PRACH on Cell 2, UE shall retune back to Cell 1.

NOTE:	The PDCCH order RACH delay can be expressed as: NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation, where:

-	NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission and is specified in Table 6.4-1 in 38.214 [9].

-	TBWPswitchDelay= 0, Tswitch= 0, ∆RF/BB preparation= 0

-	∆Delay= 0.5ms

-	TSSB is the time to first SSB occasion, after 1slot from the end of the slot of the PDCCH plus 2ms (SSB processing time).

During T2, interruption on Cell 1 UL shall not happen outside the overlapped slot to transmit PRACH and N symbols before and after the PRACH occasion as defined in clause 8.1 in 38.213 [8], where N=2. During T2, interruption on Cell 1 DL shall not occur outside the overlapped slot to transmit PRACH.

The test equipment will verify that the timing of PRACH transmission on Cell 2 is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB of Cell 2.

a.	The NTA_offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in TS 38.133 [6] Table 7.1.2-1.

The rate of correct events observed during repeated tests shall be at least 90%.

<<< END OF CHANGES 3>>>