Title:		Discussion on Test Point Analysis for NR V2X test case “7.8E.2.1 Wide band Intermodulation for V2X / non-concurrent operation”

Source:			CAICT

Introduction

The purpose of this paper is to provide test point selection proposal for NR V2X test case “7.8E.2.1 Wide band Intermodulation for V2X / non-concurrent operation”.

Discussion

As reference, the test configuration for LTE V2X test case “7.8.1G.1	Wide band Intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions” is shown below:

Table 7.8.1G.1.4.1-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 | NC |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 | Mid range |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 | Lowest, Highest |

The test configuration for NR FR1 test case “7.8.2 Wide band Intermodulation” is shown below:

Table 7.8.2.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range (NOTE 4) |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest Lowest UL / Lowest DL, Lowest UL / Highest DL (NOTE 3) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | Mod'n | RB allocation |  | Mod'n | RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3.2.4.1-1. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: Additional test points selected according to asymmetric channel bandwidths specified in clause 5.3.6. DL channel bandwidth shall be selected first. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |  |

  - Test Environment

Test only at Normal test environment for Wide band Intermodulation for NR V2X non-concurrent operation, which is same as that for LTE V2X non-concurrent operation.

Proposal 1: Propose Test Environment at Normal condition only for Wide band Intermodulation for NR V2X non-concurrent operation.

  - Test Frequencies

The test frequencies for Wide band Intermodulation for NR V2X non-concurrent operation shall deviate from that for LTE V2X non-concurrent operation.

Proposal 2: Mid frequency range shall be tested for Wide band Intermodulation for NR V2X non-concurrent operation.

Test Channel Bandwidth

Test Lowest and Highest Channel Bandwidths for Wide band Intermodulation for NR V2X non-concurrent operation, which is same as that for LTE V2X non-concurrent operation.

Proposal 3: The Lowest and Highest Channel Bandwidths shall be tested for Wide band Intermodulation for NR V2X non-concurrent operation.

Test Subcarrier Spacing (SCS)

The Lowest SCS will be tested for Wide band Intermodulation for NR V2X non-concurrent operation, which is same as that for NR FR1.

Proposal 4: The Lowest SCS shall be tested for Wide band Intermodulation for NR V2X non-concurrent operation.

Modulations

The channel modulation and RB configuration choice to be used for Wide band Intermodulation for NR V2X non-concurrent operation test shall follow same as that for NR FR1.

Proposal 5: QPSK CP-OFDM modulation with full RB allocation shall be used for Wide band Intermodulation for NR V2X non-concurrent operation.

Number of Test Points

The total test points for Wide band Intermodulation for NR V2X non-concurrent operation are summarized in Table below.

Maximum Number of Test Steps for Wide band Intermodulation for NR V2X non-concurrent operation

| Environment conditions | Number SCS | Maximum Number of ChBW | Maximum Number of Frequencies | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 1 | 1 | 2 |

Conclusion

In conclusion, the following selection of test points are proposed for Wide band Intermodulation for NR V2X non-concurrent operation measurement:

Proposal 1: Propose Test Environment at Normal condition only for Wide band Intermodulation for NR V2X non-concurrent operation.

Proposal 2: Mid frequency range shall be tested for Wide band Intermodulation for NR V2X non-concurrent operation.

Proposal 3: The Lowest and Highest Channel Bandwidths shall be tested for Wide band Intermodulation for NR V2X non-concurrent operation.

Proposal 4: The Lowest SCS shall be tested for Wide band Intermodulation for NR V2X non-concurrent operation.

Proposal 5: QPSK CP-OFDM modulation with full RB allocation shall be used for Wide band Intermodulation for NR V2X non-concurrent operation.