3GPP TSG-RAN5 Meeting #105	R5-246594

# Orlando,USA,18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4724 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MBS broadcast test case 14.1.1.5-Multiple beams |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New MBS test case to check that UE could receive the MBS broadcast under multiple beams scenario. Two SSBs is configured and mtch-SSB-MappingWindowList-r17/ mtch-SSB-MappingWindowIndex-r17 are set to present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new MBS test case 14.1.1.5 MBS Broadcast/ MCCH Information Acquisition/ Multiple beams |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multiple beams test scenario for MBS broadcast is not covered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.1.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2. CR 0524 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.1.1.5	MBS Broadcast/ MCCH Information Acquisition/ Multiple beams

14.1.1.5.1	Test Purpose (TP)

(1)

with { UE in NR RRC IDLE state and camped on a cell providing SIB20 and interested to receive MBS broadcast service and multiple SSBs are configured in ssb-PositionsInBurst in SIB1 }

ensure that {

when { The [x×N+K]th PDCCH monitoring occasion for MCCH message in MCCH transmission window corresponds to the Kth transmitted SSB is used to transmit MCCH Information change notification due to the start of new MBS service.}

then { UE starts acquiring the MBSBroadcastConfiguration message on MCCH from the slot in which the change notification was received }

}

(2)

with { UE in NR RRC IDLE state and multiple SSBs are configured in ssb-PositionsInBurst in SIB1 and UE has received MBSBroadcastConfiguration message which configures MTCH and SSB mapping window in MTCH-SSB-MappingWindowList-r17 }

ensure that {

when { The [x×N+K]th PDCCH monitoring occasion for MTCH in this mapping window corresponds to the Kth transmitted SSB is used to transmit PDCCH for MTCH. }

then { UE receives MBS Packet on the MTCH }

}

14.1.1.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.9.1.2, 6.2.2 and 6.3.6. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.9.1.2]

The MCCH information (i.e. information transmitted in messages sent over MCCH) is transmitted periodically, using a configurable repetition period and within a configured transmission window. MCCH transmissions (and the associated radio resources and MCS) are indicated via the PDCCH addressed to MCCH-RNTI. PDCCH monitoring occasion(s) for MCCH transmission are determined according to the common search space indicated by searchspaceMCCH. If searchspaceMCCH is set to zero, PDCCH monitoring occasions for MCCH message reception in the MCCH transmission window are the same as PDCCH monitoring occasions for SIB1 where the mapping between PDCCH monitoring occasions and SSBs is specified in TS 38.213[13]. If searchspaceMCCH is not set to zero, PDCCH monitoring occasions for MCCH message are determined based on search space indicated by searchspaceMCCH. PDCCH monitoring occasions for MCCH message which are not overlapping with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from one in the MCCH transmission window. The [x×N+K]th PDCCH monitoring occasion for MCCH message in MCCH transmission window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in MCCH transmission window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes. The UE assumes that, in the MCCH transmission window, PDCCH for an MCCH message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception MCCH messages is up to UE implementation.

[TS 38.331, clause 6.3.6]

mtch-SSB-MappingWindowIndex Indicates the index of MTCH-SSB-MappingWindowCycleOffset configuration entry in MTCH-SSB-MappingWindowList. The value 0 corresponds to the first entry in MTCH-SSB-MappingWindowList, the value 1 corresponds to the second entry in MTCH-SSB-MappingWindowList and so on. This field is set to the same value for all MBS sessions mapped to the same G-RNTI.

…

MTCH-SSB-MappingWindowList field descriptions

MTCH-SSB-MappingWindowCycleOffset Indicates the cycle and offset for MTCH PDCCH ocassions to SSB mapping. Values in unit of ms. ms10 corresponds to cycle of 10 ms with corresponding offset between 0 and 9 ms, value ms20 corresponds to cycle of 20 ms with corresponding offset between 0 and 19 ms, and so on. The mapping window starts at a subframe in a SFN where [(SFN number × 10) + subframe number] modulo (cycle) = offset. PDCCH monitoring occasions for MTCH in a mapping window which are not overlapping with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered starting from 1 in the maping window. The [x×N+K]th PDCCH monitoring occasion for MTCH in this mapping window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in MTCH to SSB mapping transmission window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes.

14.1.1.5.3	Test description

14.1.1.5.3.1	Pre-test conditions

System Simulator:

-	The SS configures the NR Cell 1 as the "Serving cell".

-	System information combination NR-20 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1 (serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

-	UE is made interested in receiving MBS Broadcast service with MBS Service ID ‘000001’H.

14.1.1.5.3.2	Test procedure sequence

Table 14.1.1.5.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 14.1.1.5.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | E-UTRA Cell 1 | NR Cell 1 | NR Cell 1 Beam index #1 | NR Cell 1 Beam index #0 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | - | - | Beam#1 Switch ON and Beam#0 Switch OFF |
|  | Reference Power | dBm/SCS | - | -88 | - | - |  |
|  | CSI-RS EPRE SS/PBCH SSS EPRE, | dB | - | - | 0 | -57 |  |
| NOTE: "Beam index #1" refers to transmission of the SS/PBCH block with SSB index #1 (according to the ssb-PositionsInBurst); "Beam index #0" refers to transmission of the SS/PBCH block with SSB index #0 (according to the ssb-PositionsInBurst). |  |  |  |  |  |  |  |

Table 14.1.1.5.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | E-UTRA Cell 1 | NR Cell 1 | NR Cell 1 Beam index #1 | NR Cell 1 Beam index #0 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -96 | - | - | - | Beam#1 Switch ON and Beam#0 Switch OFF |
|  | Reference Power | dBm/SCS | - | -82 | - | - |  |
|  | CSI-RS EPRE SS/PBCH SSS EPRE, | dB | - | - | 0 | -63 |  |
| NOTE: "Beam index #1" refers to transmission of the SS/PBCH block with SSB index #1 (according to the ssb-PositionsInBurst); "Beam index #0" refers to transmission of the SS/PBCH block with SSB index #0 (according to the ssb-PositionsInBurst). |  |  |  |  |  |  |  |

Table 14.1.1.5.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS starts broadcasting MBSBroadcastConfiguration message to include the configuration for MBS Service ID ‘000001’H. | - | - | - | - |
| 2 | The SS transmits MCCH information change notification due to the start of new MBS service.(Note 1) | - | - | - | - |
| 3-11a2 | Steps 1 to 9a2 of the generic procedures described in TS 38.508-1 [4] subclause 4.5.4.2-3 are performed with condition UE TEST LOOP MODE C. | - | - | - | - |
| 12 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| - | Exception: Step 13 is repeated 5 times. | - | - | - | - |
| 13 | The SS transmits an MBS Packet on the MTCH.(Note 2) | <-- | MBS Packet. | - | - |
| 14-21 | Steps 1 to 8 of the generic procedures described in TS 38.508-1 [4] subclause 4.5.4.2-3 are performed. | - | - | - | - |
| 22 | The SS transmits a UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 23 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 24 | Check: Is the number of reported MBS Packets received on the MTCH in step 23 greater than zero? | - | - | 1.2 | P |
| Note 1: PDCCH monitoring occasion for MCCH message in MCCH transmission window corresponds to the SSB#1 is used.(e.g even SFN slot#5 for FDD OR (TDD AND SCS30),#6 for others) Note 2: PDCCH monitoring occasion for MTCH in a mapping window corresponds to the SSB#1 is used.(e.g odd SFN slot#7 for FDD OR (TDD AND SCS120),#6 for TDD AND SCS15, #14 for TDD AND SCS30) |  |  |  |  |  |

14.1.1.5.3.3	Specific message contents

Table 14.1.1.5.3.3-1: SIB1 of NR Cell 1 (preamble and all steps, Table 14.1.1.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommonSIB | TS 38.508-1[4],Table 4.6.3-169 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| si-SchedulingInfo-v1700 | SI-SchedulingInfo-v1700 | TS 38.508-1 [4], Table 4.6.3-173AA 1 entry for SIB20 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.5.3.3-2: ServingCellConfigCommonSIB (Table 14.1.1.5.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 with condition MBS_Broadcast |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’1100 0000’B | When carrier frequency is smaller than or equal to 3 GHz, only the 4 leftmost bits are valid |  |
| } |  |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon | Table 14.1.1.5.3.3-3 | SCS30 |
|  | TDD-UL-DL-ConfigCommon | TS 38.508-1[4],Table 4.6.3-192 |  |
| } |  |  |  |

Table 14.1.1.5.3.3-3: TDD-UL-DL-ConfigCommon

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-192 with condition SCS30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| pattern1 SEQUENCE { |  |  |  |
| nrofDownlinkSlots | 6 |  |  |
| nrofUplinkSlots | 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.5.3.3-4: ACTIVATE TEST MODE (preamble, Table 14.1.1.5.3.2-3)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.1.1.5.3.3-5: MBSBroadcastConfiguration (steps 1, Table 14.1.1.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-5ABA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBSBroadcastConfiguration-r17 := SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| mbsBroadcastConfiguration-r17 SEQUENCE { |  |  |  |
| mbs-SessionInfoList-r17 | MBS-SessionInfoList | Table 14.1.1.5.3.3-6 |  |
| mtch-SSB-MappingWindowList-r17 | mtch-SSB-MappingWindowList | Table 14.1.1.5.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.5.3.3-6: MBS-SessionInfoList (Table 14.1.1.5.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBS-SessionInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17 { | 1 entry |  |  |
| MBS-SessionInfo-r17[1] SEQUENCE { |  | entry 1 |  |
| mtch-SSB-MappingWindowIndex-r17 | 1 | The second entry in MTCH-SSB-MappingWindowList |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.5.3.3-7: MTCH-SSB-MappingWindowList (Table 14.1.1.5.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MTCH-SSB-MappingWindowList-r17 ::= SEQUENCE (SIZE (1..maxNrofMTCH-SSB-MappingWindow-r17)) OF MTCH-SSB-MappingWindowCycleOffset-r17 { | 2 entries |  |  |
| MTCH-SSB-MappingWindowCycleOffset-r17[1] CHOICE { |  | entry 1 |  |
| ms20 | 13 |  |  |
|  | 14 |  | SCS30 |
| } |  |  |  |
| MTCH-SSB-MappingWindowCycleOffset-r17[2] CHOICE { |  | entry 2 |  |
| ms20 | 12 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.5.3.3-8: Physical layer parameters for DCI format 4_0 (step 2, Table14.1.1.5.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.5.1-1, condition MCCH-RNTI |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| MCCH change notification | MSB indicates a new MBS service start. LSB indicates no modification of MCCH information other than the change caused by start of new MBS service(s). | “10” |  |

Table 14.1.1.5.3.3-9: CLOSE UE TEST LOOP (step 11a1, Table 14.1.1.5.3.2-3)

Derivation Path: TS 38.508-1 [4], clause 4.7A.2, condition UE TEST LOOP MODE C and Broadcast MRB

Table 14.1.1.5.3.3-10: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (steps 22, Table 14.1.1.5.3.2-3)

Derivation Path: TS 36.508 [6], Table 4.7A-9

<End of modified section 1>