3GPP TSG-RAN5 Meeting #105	R5-246596

# Orlando,USA,18th Nov 2024 – 22nd Nov 2024

Source:	Huawei, HiSilicon

Title:	Discussion paper on legacy test cases adjustment for eRedcap

Agenda item:	6.3.16.7

Document for:	Discussion

Background

The Rel-18 NR_redcap_enh_plus_CT1-UEConTest WI was approved at RAN#103 [1]. In this WI, several eRedCap dedicated test cases have been added and all the legacy test cases also need to be reviewed. This document discusses how to handle the legacy SIG test cases for the eRedCap UEs.

Discussion

Observation 1: eRedCap is a further UE complexity reduction UE type, it inherit many feature from R17 RedCap, shown as below:

1.1 The maximum bandwidth is 20 MHz for FR1. 1.2 Support of eRedCap early indication based on Msg1 for 4 step RACH 1.3 Separate initial UL/DL BWP is shared by RedCap UEs and eRedCap UEs. 1.4 one UE-specific RRC configured UL/DL BWP per carrier 1.5 UE-specific RRC-configured DL BWP with CD-SSB or NCD-SSB 1.6 NCD SSB based measurements in RRC configured DL BWP 1.7 The maximum mandatory supported DRB number is 8, while 16 DRB is optional(RedCap/eRedCap share the same UE capabiltiy). 1.8 The mandatory supported PDCP SN length is 12 bits while 18 bits being optional(RedCap/eRedCap share the same UE capabiltiy) 1.9 The mandatory supported RLC AM SN length is 12 bits while 18 bits being optional(RedCap/eRedCap share the same UE capabiltiy) 1.10 Half-duplex FDD operation is optional(RedCap/eRedCap share the same UE capabiltiy) 1.11 Not support CA, MR-DC, DAPS, CPAC .

Overall strategy and timeline are listed below:

Proposal 1-1: eRedCap can inherit the strategy from RedCap to handle the legacy SIG test cases. Not applicable test case can be filtered by PICS setting (e.g pc_EN_DC=false). R15 test cases are fully analysed, re-used and updated if needed.  R16 and later features can be analysed by interested company and added the test cases to a table in the Annex A in 38.523-2.

Proposal 1-2: Legacy SIG test cases adjustment for eRedCap UE in 38.523-1/2 will be started in the next meeting if the proposal in the discussion paper agreed by RAN5 in this meeting. The timeline for TTCN implementation and 38.523-3 adjustment is decided by MCC TF160.

For detailed changes, proposal and example for spec changes are listed below. In all, for the same feature between RedCap and eRedCap, the changes are also the same.

Proposal 2: All the test case for eRedCap UE also configure 12bits PDCP SN length and 12bits RLC AM SN length [2], so the following legacy test case need to be updated to consider the eRedCap. 7.1.1.3.4/7.1.1.4.1.1/7.1.1.4.2.1/7.1.1.4.2.3/7.1.1.4.2.4/7.1.1.4.2.5/7.1.1.9.1/7.1.2.3.3/7.1.2.3.5/7.1.2.3.6/7.1.2.3.7/7.1.2.3.8/7.1.2.3.9/7.1.2.3.10/7.1.2.3.11/7.1.3.4.1/7.1.3.5.4/8.1.5.2.2

Example: (7.1.1.3.4)

| 27a1 | IF NOT (pc_supportOfRedCap_r17 OR pc_supportOfERedCap_r18) THEN Check: Does the UE return a MAC PDU of length 256 bits including RLC SDUs, Padding and Short BSR or LongBSR with Buffer size(s) set to ‘0’? (Note 5) | --> | MAC PDU (13-Byte MAC Sub PDU from LC 4 and 12-Byte MAC Sub PDU from LCH6 and 5-Byte MAC Sub PDU containing Long BSR and 2-Byte MAC Sub PDU containing Padding) Or MAC PDU (13-Byte MAC Sub PDU from LCH4 and 12-Byte MAC Sub PDU from LCH6 and 2-Byte MAC Sub PDU containing short BSR and 5-Byte MAC Sub PDU containing Padding) | 8 | P |
| --- | --- | --- | --- | --- | --- |
| 27b1 | ELSE IF pc_supportOfRedCap_r17 OR pc_supportOfERedCap_r18 THEN Check: Does the UE return a MAC PDU of length 256 bits including RLC SDUs, Padding and Short BSR or LongBSR with Buffer size(s) set to ‘0’? (Note 5) | --> | MAC PDU (12-Byte MAC Sub PDU from LC 4 and 11-Byte MAC Sub PDU from LCH6 and 5-Byte MAC Sub PDU containing Long BSR and 4-Byte MAC Sub PDU containing Padding) Or MAC PDU (12-Byte MAC Sub PDU from LCH4 and 11-Byte MAC Sub PDU from LCH6 and 2-Byte MAC Sub PDU containing short BSR and 7-Byte MAC Sub PDU containing Padding) | 8 | P |
| 28 | SS transmits an RLC STATUS PDU to acknowledge correctly received data(LCID=LCH4) | <-- | RLC STATUS PDU (ACK_SN=3) | - | - |
| 29 | SS transmits an RLC STATUS PDU to acknowledge correctly received data(LCID=LCH6) | <-- | RLC STATUS PDU (ACK_SN=3) | - | - |
| Note 1: … Note 2: … Note 3: … Note 4: If pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false, the UE has 46 bytes of RLC SDU data (received in steps 2, 9 and 13) in the transmission buffer.608 bits enables UE to transmit user data in MAC PDU 2 RLC SDUs of 12 bytes on LCH4, each 3 Bytes RLC Header and 2 Bytes MAC Header resulting in 2 MAC Sub PDUs of 17 Bytes Each. Similarly one 17 Bytes MAC Sub PDU for 12 Bytes RLC SDU on LCH5. Two 5 Bytes RLC SDUs on LCH6 with 3 Bytes RLC header each and 2 Bytes MAC header each, will result in 2 MAC sub PDUs of 10 bytes each. Total comes to 17+17+17+10+10 +3 B LongBSR(2 Bytes LongBSR header + 1 Byte LongBSR) + 2 B padding =76 Bytes. If pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true, the UE has 46 bytes of RLC SDU data (received in steps 2, 9 and 13) in the transmission buffer.608 bits enables UE to transmit user data in MAC PDU 2 RLC SDUs of 12 bytes on LCH4, each 2Bytes RLC Header and 2 Bytes MAC Header resulting in 2 MAC Sub PDUs of 16 Bytes Each. Similarly one 16 Bytes MAC Sub PDU for 12 Bytes RLC SDU on LCH5. Two 5 Bytes RLC SDUs on LCH6 with 2 Bytes RLC header each and 2 Bytes MAC header each, will result in 2 MAC sub PDUs of 9 bytes each. Total comes to 16 + 16 + 16 + 9 + 9 + (3 B LongBSR + 7 B padding) or (2 B ShortBSR + 8 B padding) =76 Bytes. Note 5: The MAC SDUs for the different logical channels may be in any order in the MAC PDU. |  |  |  |  |  |

Proposal 3: Maximum bandwidth for eRedCap is also 20 MHz for FR1 [3], so the following legacy test case need to be updated to consider the eRedCap. 7.1.1.4.1.1/7.1.1.4.2.1/7.1.1.4.2.3/7.1.1.4.2.4/7.1.1.4.2.5/8.1.5.8.1

Example: (7.1.1.4.1.1)

7.1.1.4.1.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except set the NR Cell bandwidth and applicable BWP to maximum for the NR Band under test as specified in Table 5.3.5-1 in TS 38.101-1 [16] / TS 38.101-2 [17] (to enable testing of nPRB up to maximum value) and Short_DCI condition is applied in NR Serving cell configuration.

Test frequency NRf1 is as specified in TS 38.508-1 [4] clause 4.3.1 using the common highest mandatory UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

NOTE:	If pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true, the FR1 test channel bandwidth is MIN {20MHz, common highest mandatory UL and DL channel bandwidth in clause 4.3.1 of TS 38.508-1 [4]} and If pc_supportOfRedCap_r17=true, the FR2 test channel bandwidth is MIN {100MHz, common highest mandatory UL and DL channel bandwidth in clause 4.3.1 of TS 38.508-1 [4]}

Observation 2: eRedCap UE has some new features compared with R17 RedCap. For example if eRedCap UE does not support pc_eRedCapNotReducedBB_BW_r18, the PDSCH/PUSCH is further reduced to 5MHz, other new feature are list below:

| eRedCap UE with reduced BB BW (pc_eRedCapNotReducedBB_BW_r18=False) | eRedCap UE without reduced BB BW (pc_eRedCapNotReducedBB_BW_r18=True) |
| --- | --- |
| 2.1 DL/UL peak data rate of 10 Mbps 2.2 Not support FR2 and FR1 60kHz SCS. 2.3 Network configurable additional separate early indication in Msg1 for eRedCap UEs. 2.4 Support of eRedCap early indication based on Msg3 and MsgA PUSCH.(LCID is different from RedCap) 2.5 eRedCap UE ingores the capability filtering enquiry and conveys all the supported bands in the appliedFreqBandListFilter if it support eRedCapIgnoreCapabilityFiltering-r18 2.6 New IE introduced intraFreqReselection-eRedCap/cellBarred-eRedCap1Rx/cellBarred-eRedCap2Rx/eRedCapAccessAllowed... 2.7 Msg4 for TC-RNTI ≤ 25/12 PRBs for 15/30kHz SCS and Msg3 for TC-RNTI≤ 25/12 PRBs for 15/30kHz SCS |  |
| 2.8 unicast PDSCH/PUSCH ≤ 25/12 PRBs for 15/30kHz SCS | BB BW for unicast PDSCH/PUSCH in FR1 is 20MHz. |
| 2.9 If the Msg2 ≥25/12 PRBs for 15/30kHz SCS, the minimum time between Msg3 and Msg2 is 3.3/1.9 ms for 15/30kHz SCS. (Detailed calculation is in Annex A.1) | If the Msg2 ≥25/12 PRBs for 15/30kHz SCS, the minimum time between Msg3 and Msg2 is 3.3/1.9 ms for 15/30kHz SCS. |

Impact Analysis (whether or not need to update legacy test case in the test spec 38.523-1/2/3):

2.1 DL/UL peak data rate of 10 Mbps

-- Impact. TBsize is limited to 10000/5000 bits for 15/30kHz for eRedCap UE with or without reduced BB BW. (Detailed calculation is in Annex A.2)

2.2 Not support FR2 and FR1 60kHz SCS.

-- No impact. SIG does not test 60kHz SCS. If eRedCap UE does not support FR2, no FR2 part of the legacy test case will not be tested. The solution is same as other unsupported feature like CA,MR-DC [4].

2.3 Network configurable additional separate early indication in Msg1 for eRedCap UEs.

--No impact. Only new eRedCap test case involve this feature.

2.4 Support of eRedCap early indication based on Msg3 and MsgA PUSCH.(LCID is different from RedCap)

--Impact. LCID for CCCH/CCCH1 in 38.523-3 may need to be updated.

2.5 eRedCap UE ingores the capability filtering enquiry and conveys all the supported bands in the

appliedFreqBandListFilter if it support eRedCapIgnoreCapabilityFiltering-r18

--No impact. eRedCap UE may report more band in the bandcombination, and appliedFreqBandListFilter in 8.1.5.1.1 is not checked.

2.6 New IE introduced intraFreqReselection-eRedCap/cellBarred-eRedCap1Rx/cellBarred-eRedCap2Rx/eRedCapAccessAllowed.

--Impact. 6.1.2.3a need update message content to include intraFreqReselection-eRedCap= Not allowed.

2.7 Msg4 for TC-RNTI ≤ 25/12 PRBs for 15/30kHz SCS and Msg3 for TC-RNTI≤ 25/12 PRBs for 15/30kHz SCS

--Impact.

For 30kHz SCS, the LRB max for CCCH is 12 RB for eRedCap UE with or without reduced BB BW.

2.8 unicast PDSCH/PUSCH ≤ 25/12 PRBs for 15/30kHz SCS

--Impact.

For 15kHz and 30kHz SCS, TBsize test case has to add limitation for RB for eRedCap UE with reduced BB BW.

For 30kHz SCS, the LRB max for DCCH/DTCH is 12 RB for eRedCap UE with reduced BB BW.

2.9 If the Msg2 ≥25/12 PRBs for 15/30kHz SCS, the minimum time between Msg3 and Msg2 is 3.3/1.9 ms for 15/30kHz SCS.

--No Impact.

For 15kHz SCS, RAR will not exceed 24 RB, so extra time relax is not need.

For 30kHz, RAR may exceed 12RB, but the default time between first symbol of Msg3 and the last symbol of Msg2 in the SIG test case is 8 slot(4ms), which is longer than 1.9ms limitation.

According the impact analysis, the proposal and example for spec changes are listed below.

Proposal 4 (For analysis 2.7/2.8):Add extra limitation for LRB max for CCCH/DCCH/DTCH transmissions in 30kHz SCS in 38.523-3 Table 7.1.2.2.3.1-1. For example LRB max ≤ 12RB for CCCH/DCCH/DTCH transmissions in 30kHz SCS.

Proposal 5 (For analysis 2.1/2.8): The max TBsize under test is 10000/5000 bits for 15/30kHz for eRedCap UE with or without reduced BB BW. The max RB under test is 25/12 PRB for 15/30kHz for eRedCap UE with reduced BB BW. Therefore the following test case need to be updated. 7.1.1.4.1.1/7.1.1.4.2.1/7.1.1.4.2.3/7.1.1.4.2.4/7.1.1.4.2.5/8.1.5.8.1

Example: (7.1.1.4.1.1)

Table 7.1.1.4.1.1.3.2-2A: Number of downlink PDCP SDUs and PDCP SDU size used as test data for (e)RedCap UE

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 120 ≤ TBS ≤ 12112 note 2 | 1 | 8*FLOOR((TBS – 112)/8) |
| 12113 ≤ TBS ≤ 24168 | 2 | 8*FLOOR((TBS – 168)/16) |
| 24169 ≤ TBS ≤ 36224 | 3 | 8*FLOOR((TBS – 224)/24) |
| … | … | … |
| NOTE 1: … NOTE 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 120 bits. NOTE 3: If pc_supportOfERedCap_r18= True, the max TBS under test is 10000/5000 bits for 15/30kHz SCS. |  |  |

Table 7.1.1.4.1.1.3.2-4: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 to 5 are repeated for allowed values of 1 to Y in BWP, time domain resource as per table 7.1.1.4.1.0-1 and from 0 to 28. NOTE: Skip the execution of steps for which the TBS size results in coding rate exceeding 0.95. | - | - | - | - |
| 1 | The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. | - | - | - | - |
| - | EXCEPTION: Steps 2 to 5 are performed if TBS is less than or equal to UE capability "Maximum number of DL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.1.1.3.2-1 and larger than or equal to X bits as specified in Table 7.1.1.4.1.1.3.2-2/2A (Note 1). | - | - | - | - |
| 2 | The SS creates one or more PDCP SDUs, depending on TBS, in accordance with Table 7.1.1.4.1.1.3.2-2/2A (Note 2). | - | - | - | - |
| 3 | The SS transmits the PDCP SDUs concatenated into a MAC PDU and indicates on PDCCH DCI Format 1_0 and values of S, L,and nPRB. | <-- | MAC PDU (NxPDCP SDUs) DCI: (DCI Format 1_0, S, L,and nPRB.) | - | - |
| 4 | At the reception of scheduling request the SS transmits UL Grant for transmitting loop back PDCP SDUs. | <-- | (UL Grant) | - | - |
| 5 | Check: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 3? | --> | (NxPDCP SDUs) | 1 | P |
| NOTE 1: For pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false, Table 7.1.1.4.1.1.3.2-2 is used and X =136. For pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true, Table 7.1.1.4.1.1.3.2-2A is used and X=120. NOTE 2: For pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false, Table 7.1.1.4.1.1.3.2-2 is used. For pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true, Table 7.1.1.4.1.1.3.2-2A is used. NOTE 3: If pc_supportOfERedCap_r18= True AND pc_eRedCapNotReducedBB_BW_r18=False, Y = min {, 25} for 15kHz SCS and Y= min {, 12} for 30kHz SCS, otherwise Y= . |  |  |  |  |  |

Example: (8.1.5.8.1)

8.1.5.8.1 Step7A use LRB=7 and MCS=24, TBsize=4864bits < 5000bits

| 7 | The SS transmits a UECapabilityEnquiry message. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| --- | --- | --- | --- | --- | --- |
| 7A | The SS starts transmitting a periodic DCI with the maximum grant allowed by the carrier bandwidth with MCS=24 from (80∙2μ+1+TDL) slots after step 7 to schedule PUSCH. (Note 1, Note 2) Note : If pc_supportOfERedCap_r18=true, the SS configure the grant with the LRB = 7 and MCS= 24 to ensure the TBsize less than 5000 bits. | - | - | - | - |
| 8 | Check: Does the UE transmit a UECapabilityInformation message within (80∙2μ+1+TDL+K2) slots after successful completion of step 7? (Note 1, Note 2, Note 3) | --> | NR RRC: UECapabilityInformation | 4 | P |

Proposal 6 (For analysis 2.6): 6.1.2.3a need update message content to include intraFreqReselection-eRedCap= Not allowed.

Example:

Table 6.1.2.3a.3.3-2: SIB1 for NR Cell 1 (step 1, Table 6.1.2.3a.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| intraFreqReselectionRedCap-r17 | notAllowed |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| intraFreqReselection-eRedCap-r18 | notAllowed |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Conclusion

From detailed analysis above, if we start to handle eRedCap R15 legacy test cases, approximately 20 R15 legacy test cases need to be updated and all the modifications are not significant. Therefore, our understanding is that eRedCap can inherit the strategy from RedCap to handle the legacy SIG test cases. For the majority of legacy test cases that do not require any changes, we can re-use them and keep the prose as it is. For the minority of legacy test cases that need to be adjusted for eRedcap UE, we can also re-use them and directly modify the test cases.

Overall strategy and timeline:

Proposal 1-1:eRedCap can inherit the strategy from RedCap to handle the legacy SIG test cases. Not applicable test case can be filtered by PICS setting (e.g pc_EN_DC=false). R15 test cases are fully analysed, re-used and updated if needed. R16 and later features can be analysed by interested company and added the test cases to a table in the Annex A in 38.523-2.

Proposal 1-2:Legacy SIG test cases adjustment for eRedCap UE in 38.523-1/2 will be started in the next meeting if the proposal in the discussion paper agreed by RAN5 in this meeting. The timeline for TTCN implementation and 38.523-3 adjustment is decided by MCC TF160.

Details for spec updated:

Proposal 2: All the test case for eRedCap UE also configure 12bits PDCP SN length and 12bits RLC AM SN length [2], so the following legacy test case need to be updated to consider the eRedCap. 7.1.1.3.4/7.1.1.4.1.1/7.1.1.4.2.1/7.1.1.4.2.3/7.1.1.4.2.4/7.1.1.4.2.5/7.1.1.9.1/7.1.2.3.3/7.1.2.3.5/7.1.2.3.6/7.1.2.3.7/7.1.2.3.8/7.1.2.3.9/7.1.2.3.10/7.1.2.3.11/7.1.3.4.1/7.1.3.5.4/8.1.5.2.2.

Proposal 3:Maximum bandwidth for eRedCap is also 20 MHz for FR1 [3], so the following legacy test case need to be updated to consider the eRedCap. 7.1.1.4.1.1/7.1.1.4.2.1/7.1.1.4.2.3/7.1.1.4.2.4/7.1.1.4.2.5/8.1.5.8.1.

Proposal 4:Add extra limitation for LRB max for CCCH/DCCH/DTCH transmissions in 30kHz SCS in 38.523-3 Table 7.1.2.2.3.1-1. For example LRB max ≤ 12RB for CCCH/DCCH/DTCH transmissions in 30kHz SCS.

Proposal 5:The max TBsize under test is 10000/5000 bits for 15/30kHz for eRedCap UE with or without reduced BB BW. The max RB under test is 25/12 PRB for 15/30kHz for eRedCap UE with reduced BB BW. Therefore the following test case need to be updated. 7.1.1.4.1.1/7.1.1.4.2.1/7.1.1.4.2.3/7.1.1.4.2.4/7.1.1.4.2.5/8.1.5.8.1.

Proposal 6: 6.1.2.3a need update message content to include intraFreqReselection-eRedCap= Not allowed.

- References

- RP-240054, “New WID on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects”, China Unicom, Ericsson, Huawei, Hisilicon, Qualcomm, RAN #103, March 2024.

- R5-246598, “Update PDCP-Config and RLC-Config for eRedCap UE”, Huawei, Hisilicon, RAN5 #105

- R5-246597, “Update SIG default test frequencies for eRedCap UE”, Huawei, Hisilicon, RAN5 #105

- R5-246601, “Update PICS for eRedCap UE”, Huawei, Hisilicon, RAN5 #105

- Annex A.1

The minimum time between MSG3 and MSG2 required by core spec if MSG2 is larger than 25/12 PRBs for 15/30kHz:

For 15kHz SCS, min time = NT,1+NT,2+1.5=14 symbol+10symbol+1.5ms=3.3ms

For 30kHz SCS, min time = NT,1+NT,2+1.0=13 symbol+12symbol+1.0ms=1.9ms

The minimum time between MSG3 and MSG2 required by core spec if MSG2 is not larger than 25/12 PRBs for 15/30kHz:

For 15kHz SCS, min time = NT,1+NT,2+0.5=14 symbol+10symbol+1.5ms=2.3ms

For 30kHz SCS, min time = NT,1+NT,2+0.5=13 symbol+12symbol+1.0ms=1.4ms

The time between MSG3 and MSG2 in the test spec 38.508-1:

For 15kHz SCS, time = 4 slot – 1 slot =3 ms

For 30kHz SCS, time = 9 slot – 1 slot =4 ms

[TS 38.508-1, Table 4.6.3-122]

[TS 38.213, clause 17.1A] - eRedCap

When

-	a UE receives a PDSCH scheduled by a DCI format with CRC scrambled by a RA-RNTI or a MsgB-RNTI over a number of PRBs that is larger than 25 PRBs for 15 kHz SCS or larger than 12 PRBs for 30 kHz SCS, and

-	the PDSCH includes a RAR message with an RAR UL grant scheduling a Msg3 PUSCH transmission from the UE, as described in Clauses 8.2 and 8.2A

the UE transmits the Msg3 PUSCH if a time between the last symbol of a PDSCH reception conveying the RAR message and the first symbol of the Msg3 PUSCH transmission is not smaller than NT,1+NT,2+1.5 msec for 15 kHz SCS or NT,1+NT,2+1.0 msec for 30 kHz SCS where NT,1 and NT,2 are defined in clause 8.3; otherwise, the UE behaviour is based on UE implementation.

[TS 38.213, clause 8.3] – non-eRedCap

The UE may assume a minimum time between the last symbol of a PDSCH reception conveying a RAR message with a RAR UL grant and the first symbol of a corresponding PUSCH transmission scheduled by the RAR UL grant is equal to NT,1+NT,2+0.5 msec, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured, NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] and, for determining the minimum time, the UE considers that N1 and N2 correspond to the smaller of the SCS configurations for the PDSCH and the PUSCH. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

[TS 38.214, Table 5.3-1]

Table 5.3-1: PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB if either higher layer parameter is configured, and in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if either higher layer parameter is configured | dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in any of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2, or if none of the higher layer parameters is configured |
| 0 | 8 | N1,0 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |
| 5 | 80 | 96 |
| 6 | 160 | 192 |

[TS 38.214, Table 6.4-1]

Table 6.4-1: PUSCH preparation time for PUSCH timing capability 1

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |
| 5 | 144 |
| 6 | 288 |

- Annex A.2

If:

J=0 //one serving Cell

M=0 //one TB

T=10^-3 for 15kHz SCS, T=0.5*10^-3 for 30kHz SCS

C’=C

DataRate= 10Mbps = 10*10^6 bps

Then

A/T ≤ 10Mbps

TBS=A≤ 10000 /5000bits for 15/30kHz SCS

[TS 38.214, clause 5.1.3]

Within a cell group, a UE is not required to handle PDSCH(s) transmissions including unicast and/or multicast/broadcast in slot sj in serving cell-j, and for j = 0,1,2.. J-1, slot sj overlapping with any given point in time, if the following condition is not satisfied at that point in time:

j=0J-1m=0M-1Vj,mTslotμ(j)≤DataRate

where,

-	J is the number of configured serving cells belonging to a frequency range

-	for the j-th serving cell,

-	M is the number of TB(s) transmitted in slot sj. If there are two PDSCH transmission occasions of the same TB (in time domain or in frequency domain) in the slot sj, each transmission occasion is counted separately.

-	Tslot(j) =10-3/2(j), where (j) is the numerology for PDSCH(s) in slot sj of the j-th serving cell.

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Clause 7.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Clause 5.2.2 [5, TS 38.212].

-	C' is the number of scheduled code blocks for the transport block as defined in Clause 5.4.2.1 [5, TS 38.212]

-	DataRate [Mbps] is computed as the maximum data rate summed over all the carriers in the frequency range for any signaled band combination and feature set consistent with the configured servings cells, where the data rate value is given by the formula in Clause 4.1.2 in [13, TS 38.306], including the scaling factor f(i).

[TS 38.214, clause 6.1.4]

Within a cell group, a UE is not required to handle PUSCH(s) transmissions in slot sj in serving cell-j, and for j = 0,1,2.. J-1, slot sj overlapping with any given point in time, if the following condition is not satisfied at that point in time:

j=0J-1m=0M-1Vj,mTslotμ(j)≤DataRate,

where

-	J is the number of configured serving cells belong to a frequency range

-	for the j-th serving cell,

-	M is the number of TB(s) transmitted in slot-sj. For PUSCH repetition Type B, each actual repetition is counted separately.

-	Tslot(j) =10-3/2(j), where (j) is the numerology for PUSCH(s) in slot sj of the j-th serving cell.

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Clause 6.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Clause 5.2.2 [5, TS 38.212].

-	C'is the number of scheduled code blocks for the transport block as defined in Clause 5.4.2.1 [5,38.212]

-	DataRate [Mbps] is computed as the maximum data rate summed over all the carriers in the frequency range for any signaled band combination and feature set consistent with the configured servings cells, where the data rate value is given by the formula in Clause 4.1.2 in [13, TS 38.306], including the scaling factor f(i).