3GPP TSG-RAN5 Meeting #105	R5-246646

# Orlando, United States, 18th - 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0074 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of test case 6.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At step 3 the MCData client shall be triggered by MMI to "search for existing objects that were created in the message store with a maximum number of responses set to 3" but it is not clear how this maximum number can be set to 3. On the other hand it seems to be irrelevant for the test case to which value the maximum number of responses set and in Table 6.8.1.3.3-3 <maxEntries> is specified as "any value". |  |  |  |  |  |  |  |  |
| Summary of change: |  | "maximum number of responses set to 3" removed from steps 3 and 4 in Table 6.8.1.3.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.8.1	On-network / MCData Message Store / Objects / Object upload / Object search / Update Object / Object retrieval / Delete stored object

6.8.1.1	Test Purpose (TP)

(1)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to upload a file to the Message Store Function }

then { UE (MCData Client) uploads the file to the Message Store Function via an HTTP POST message }

}

(2)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to search for created objects in the Message Store Function }

then { UE (MCData Client) requests for created objects in the Message Store Function via an HTTP POST message }

}

(3)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to update a file in the Message Store Function by adding a flag }

then { UE (MCData Client) updates the file in the Message Store Function via an HTTP PUT message }

}

(4)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to retrieve information about an object from the Message Store Function }

then { UE (MCData Client) downloads the object from the Message Store Function via an HTTP GET message }

}

(5)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to delete a file from the Message Store Function }

then { UE (MCData Client) deletes the file from the Message Store Function via an HTTP DELETE message }

}

6.8.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 21.2.15.1, 21.2.2.1, 21.2.3.1, 21.2.1.1, 21.2.4.1. Unless otherwise stated, these are Rel-16 requirements.

[TS 24.282 clause 21.2.15.1]

To upload the object(s) to the message store, the message store client acting as an HTTP client, shall follow the procedure described in subclause 6.1 for single upload and subclause 6.10 for bulk upload as specified in the OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarification:

1)	shall generate an HTTP POST request as specified in subclause 6.1.5 and 6.10.5 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function;

b)	shall include a valid MCData access token in the HTTP Authorization header; and

c)	shall send the HTTP POST request towards the message store function.

Upon receipt of an HTTP response, the message store client shall follow the procedure as described in subclause 6.1.2 for single upload and 6.10.2 for bulk upload as specified in the OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

[TS 24.282 clause 21.2.2.1]

To search for information about a selected set of objects in the message store, the message store client, acting as an HTTP client shall follow the procedure described in subclause 6.8 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarification:

1)	shall generate an HTTP POST request as specified in subclause 6.8.5 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function;

b)	shall include a valid MCData access token in the HTTP Authorization header; and

c)	shall send the HTTP POST request, which may include a SelectionCriteria, towards the message store function.

Upon receipt of a HTTP response, the message store client shall follow the procedure as describe in subclause 6.8.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

[TS 24.282 clause 21.2.3.1]

To update object(s) in the message store, the message store client, acting as an HTTP client, shall either follow the procedure described in subclause 6.3 or 6.4, for individual object update, or 6.11 for bulk update of objects, of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarification:

1)	shall either generate an HTTP PUT request as specified in subclause 6.3.4, 6.4.4, for individual object update, or an HTTP POST request, as specified in subclause 6.11.5, for bulk update of objects, of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66], with following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function;

b)	shall include a valid MCData access token in the HTTP Authorization header; and

c)	shall send HTTP PUT request, for individual object update, or HTTP POST request, for bulk update of objects, towards the message store function.

Upon receipt of a HTTP response, the message store client shall either follow the procedure as described in subclause 6.3.2, 6.4.2 for individual object update response, or subclause 6.11.2 for bulk update of objects response, of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

[TS 24.282 clause 21.2.1.1]

To retrieve the object from message store function, the message store client, acting as an HTTP client shall follow the procedure described in subclause 6.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarification:

1)	shall generate an HTTP GET request as specified in subclause 6.2.3 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function

b)	shall include a valid MCData access token in the HTTP Authorization header; and

c)	shall send the HTTP GET request towards the message store function.

Upon receipt of a HTTP response, the message store client shall follow the procedure as described in subclause 6.2.2 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

[TS 24.282 clause 21.2.4.1]

To delete object(s) in the message store, the message store client, acting as an HTTP client, shall either follow the procedure described in subclause 6.2, for individual object delete, or subclause 6.12 for bulk delete of objects, of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] with following clarification:

1)	shall either generate an HTTP DELETE request as specified in subclause 6.2.6, for individual object delete, or an HTTP POST request as specified in subclause 6.12.6, for bulk delete of objects, of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66], with following clarifications:

a)	shall set the Host header field to a hostname identifying the message store function;

b)	shall include a valid MCData access token in the HTTP Authorization header; and

c)	shall send HTTP DELETE request, for individual object delete, or HTTP POST request, for bulk delete of objects, towards the message store function.

Upon receipt of a HTTP response, the message store client shall either follow the procedure as described in subclause 6.2.2, for individual object delete response, or subclause 6.12.2, for bulk delete of objects response, of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66].

6.8.1.3	Test description

6.8.1.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 36.879-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 36.879-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.8.1.3.2	Test procedure sequence

Table 6.8.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) upload an object, test file 1 (TS 36.579-7 A.2.1), to the message store. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Object Upload using HTTP' as described in TS 36.579-1 [2] Table 5.3C.14.2-1 to upload an object to the message store? | - | - | 1 | P |
| 3 | Make the UE (MCData client) search for existing objects that were created in the message store. (NOTE 1) | - | - | - | - |
| 4 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Post Request using HTTP' as described in TS 36.579-1 [2] Table 5.3C.17.2-1 to search for existing objects that were created in the message store? | - | - | 2 | P |
| 5 | Make the UE (MCData client) update an object by adding a flag. (NOTE 1) | - | - | - | - |
| 6 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Put Request using HTTP' as described in TS 36.579-1 [2] Table 5.3C.18.2-1 to update an object by adding a flag? | - | - | 3 | P |
| 7 | Make the UE (MCData client) retrieve an object. (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Retrieve using HTTP' as described in TS 36.579-1 [2] Table 5.3C.16.2-1 to retrieve an object in the message store? | - | - | 4 | P |
| 9 | Make the UE (MCData client) delete an object. (NOTE 1) | - | - | - | - |
| 10 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Delete using HTTP' as described in TS 36.579-1 [2] Table 5.3C.15.2-1 to delete an object in the message store? | - | - | 5 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.8.1.3.3	Specific message contents

Table 6.8.1.3.3-1: HTTP POST from the UE (Step 2, Table 6.8.1.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1, condition MSG_STORE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_BaseURL & "/objects" |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "multipart/form-data" |  | RFC 2388 [43] |  |
| Message-body |  |  |  |  |
| MIME body part |  | Object |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/xml" |  |  |  |
| MIME-Content-Disposition |  |  |  |  |
| type | form-data |  | RESTful Network API common [42] clause 5.7 |  |
| param | name=”root-fields” |  | RESTful Network API common [42] clause 5.7 |  |
| MIME-part-body |  |  |  |  |
| NMS Object |  |  | RESTful Network API for NMS [40], clause 5.3.2.1 |  |
| parentFolder | Not present (NOTE 1) |  |  |  |
| parentFolderPath | Not present (NOTE 1) |  |  |  |
| attributes | empty list |  |  |  |
| flags | empty list |  |  |  |
| imdns | Not present |  |  |  |
| resourceURL | Not present |  |  |  |
| path | Not present |  |  |  |
| payloadURL | Not present |  |  |  |
| payloadPart | Not present |  |  |  |
| lastModSeq | Not present |  |  |  |
| correlationId | Any value if present |  |  |  |
| correlationTag | Any value if present |  |  |  |
| MIME body part |  | File content |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/octet-stream" |  |  |  |
| MIME-Content-Disposition |  |  |  |  |
| type | attachments |  | RESTful Network API common [42] clause 5.7 |  |
| param | filename with any value |  | RESTful Network API common [42] clause 5.7 |  |
| MIME-part-body | binary data representing the file | The binary for test file 1 |  |  |
| NOTE 1: As the client does not know the resourceURL or path of any folder yet, it is assumed that the client does neither provide a <parentFolder> nor a <parentFolderPath> |  |  |  |  |

Table 6.8.1.3.3-2: HTTP 201 Created from the SS (step 2, Table 6.8.1.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Location |  |  |  |  |
| uri | tsc_MCData_MsgSF_AbsoluteBaseURL & "/objects/objectId0001" | "objectId0001" arbitrarily selected |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NMS Reference |  |  |  |  |
| resourceURL | tsc_MCData_MsgSF_AbsoluteBaseURL & "/objects/objectId0001" | same as in Location |  |  |
| path | "main/test-file-1" | arbitrarily selected |  |  |

Table 6.8.1.3.3-3: HTTP POST from the UE (Step 4, Table 6.8.1.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.17.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1, condition MSG_STORE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_BaseURL & "/objects/operations/search" | URI addressing the search operation |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NMS SelectionCriteria |  |  | RESTful Network API for NMS [40], clause 6.8 |  |
| fromCursor | Not present |  |  |  |
| maxEntries | any value | the maximum number of entries in the response |  |  |
| searchCriteria |  |  |  |  |
| Criterion [1] |  |  |  |  |
| type | CreatedObjects |  |  |  |
| name | Not present |  |  |  |
| value | "" | Searching for existing objects that were created in the store since a previous CreatedObjects search. Searching an empty string ("") denotes all existing objects in the store |  |  |
| operator | Not present |  |  |  |
| searchScope | Not present |  |  |  |
| nonRecursiveScope | Not present |  |  |  |
| sortCriteria | Not present |  |  |  |
| inlineImdn | Not present |  |  |  |

Table 6.8.1.3.3-4: HTTP 200 (OK) from the SS (step 4, Table 6.8.1.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.17.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NMS ObjectList |  |  |  |  |
| Object [1] |  |  |  |  |
| parentFolder | tsc_MCData_MsgSF_AbsoluteBaseURL & "/folders/folderId0001" | "folderId0001" arbitrarily selected as id of the root folder |  |  |
| attributes | empty list |  |  |  |
| flags | empty list |  |  |  |
| imdns | Not present |  |  |  |
| resourceURL | tsc_MCData_MsgSF_AbsoluteBaseURL & "/objects/objectId0001" |  |  |  |
| path | "main/test-file-1" |  |  |  |
| payloadURL | tsc_MCData_MsgSF_BaseURL & "/objects/objectId0001/payload" |  |  |  |
| payloadPart | Not present |  |  |  |
| lastModSeq | Not present |  |  |  |
| correlationId | As received in POST request at step 2 (if so) |  |  |  |
| correlationTag | As received in POST request at step 2 (if so) |  |  |  |

Table 6.8.1.3.3-5: HTTP PUT from the UE (Step 6, Table 6.8.1.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.18.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.4-1, condition MSG_STORE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_BaseURL & "/objects/objectId0001/flags" | URI addressing the object's flags |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NMS flagList |  |  | RESTful Network API for NMS [40], clause 6.3 |  |
| flag [1] | Any value | Flag provided by MMI at step 5 |  |  |
| resourceURL | tsc_MCData_MsgSF_AbsoluteBaseURL & "/objects/objectId0001/flags" | mandatory for PUT request |  |  |

Table 6.8.1.3.3-6: HTTP 200 (OK) from the SS (step 6, Table 6.8.1.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.18.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NMS flagList |  |  | RESTful Network API for NMS [40], clause 6.3 |  |
| flag [1] | same flag as in the PUT request |  |  |  |
| resourceURL | tsc_MCData_MsgSF_AbsoluteBaseURL & "/objects/objectId0001/flags" |  |  |  |

Table 6.8.1.3.3-7: HTTP GET from the UE (Step 8, Table 6.8.1.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.16.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.2-1, condition MSG_STORE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_BaseURL & "/objects/objectId0001" | URI addressing the object |  |  |

Table 6.8.1.3.3-8: HTTP 200 (OK) from the SS (step 8, Table 6.8.1.3.2-1; step 3, TS 36.579-1 [2] Table 5.3C.16.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NMS Object |  | same object as in the HTTP 200 (OK) at step 4 but with flag added at step 6 |  |  |
| parentFolder | tsc_MCData_MsgSF_AbsoluteBaseURL & "/folders/folderId0001" |  |  |  |
| attributes | empty list |  |  |  |
| flags |  |  |  |  |
| flag[1] | same flag as in the PUT request of step 6 |  |  |  |
| resourceURL | tsc_MCData_MsgSF_AbsoluteBaseURL & "/objects/objectId0001/flags" |  |  |  |
| imdns | Not present |  |  |  |
| resourceURL | tsc_MCData_MsgSF_AbsoluteBaseURL & "/objects/objectId0001" |  |  |  |
| path | "main/test-file-1" |  |  |  |
| payloadURL | tsc_MCData_MsgSF_BaseURL & "/objects/objectId0001/payload" |  |  |  |
| payloadPart | Not present |  |  |  |
| lastModSeq | Not present |  |  |  |
| correlationId | As received in POST request at step 2 (if so) |  |  |  |
| correlationTag | As received in POST request at step 2 (if so) |  |  |  |

Table 6.8.1.3.3-9: HTTP DELETE from the UE (Step 10, Table 6.8.1.3.2-1; step 2, TS 36.579-1 [2] Table 5.3C.15.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.5-1, condition MSG_STORE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_BaseURL & "/objects/objectId0001" | URI addressing the object |  |  |