3GPP TSG RAN WG5#105	R5-246764

# Orlando, USA, November 18 – November 22 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3062 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates in NR-U MPR test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test requirement in NR-U MPR test case is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated editor’s note - Updated message content - Added test requirement in NR-U MPR test case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MPR test case will be incomplete and the NR-U WI cannot be finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2F.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### << Start of changes >>

### 6.2F.2	UE maximum output power reduction for shared spectrum access

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- MU and TT for >6GHz (band n96) are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

6.2F.2.1	Test purpose

The number of RB identified in Table 6.2F.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2F.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR standalone shared spectrum channel access.

6.2F.2.3	Minimum conformance requirements

For UE maximum output power reduction, the general requirements of clause 6.2.2 do not apply but instead the UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations for power class 5 according to Table 6.2F.2.3-1 and Table 6.2F.2.3-2.

Table 6.2F.2.3-1: Maximum power reduction (MPR) for shared spectrum access UE power class 5

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-OFDM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 2.5 |
|  | QPSK | ≤ 1.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 2.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

Table 6.2F.2.3-2: MPR mapping for wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration |  |
| --- | --- | --- |
|  | A | B |
| 40 | 11 | 10, 01 |
| 60 | 111, 011, 110, 001, 010, 100 | None |
| 80 | 1111, 0111, 1110, 0110, 0001, 1000 | 1100, 0011, 0100, 0010 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. |  |  |

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2F.4 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2F.2.

6.2F.2.4	Test description

6.2F.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1 that are restricted to shared channel access. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2F.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2F.2.4.1-1: Test Configuration Table for power class 5

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | 20 MHz, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 12 | Default |  | DFT-s-OFDM Pi/2 BPSK | Full |
| 2 | Default |  | DFT-s-OFDM QPSK | Full |
| 3 | Default |  | DFT-s-OFDM 16 QAM | Full |
| 4 | Default |  | DFT-s-OFDM 64 QAM | Full |
| 5 | Default |  | DFT-s-OFDM 256 QAM | Full |
| 6 | Low |  | CP-OFDM QPSK | Partial_Left |
| 7 | High |  | CP-OFDM QPSK | Partial_Right |
| 8 | Default |  | CP-OFDM QPSK | Full |
| 9 | Low |  | CP-OFDM 16 QAM | Partial_Left |
| 10 | High |  | CP-OFDM 16 QAM | Partial_Right |
| 11 | Default |  | CP-OFDM 16 QAM | Full |
| 12 | Low |  | CP-OFDM 64 QAM | Partial_Left |
| 13 | High |  | CP-OFDM 64 QAM | Partial_Right |
| 14 | Default |  | CP-OFDM 64 QAM | Full |
| 15 | Low |  | CP-OFDM 256 QAM | Partial_Left |
| 16 | High |  | CP-OFDM 256 QAM | Partial_Right |
| 17 | Default |  | CP-OFDM 256 QAM | Full |
| 18 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS3 | Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1F-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1 and when IE powerBoostPi2BPSK is set to 0. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2F.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2F.1.4.3.

6.2F.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2F.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. Symbols with transient periods are not under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2F.2.4.1-1, Table 6.2F.2.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2F.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

Table 6.2F.2.4.3-1: DMRS-UplinkConfig (Test ID 18 in Table 6.2F.2.4.1-1)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.2F.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2F.2.5-1.

Table 6.2F.2.5-1: Test requirements for Power Class 5

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 20 | 0 | 1.5 | 0 | 18,5 | 4,0 | 3 | 22.0 + TT | 14,5 - TT |  |
| 2 | 20 | 0 | 1.5 | 0 | 18,5 | 4,0 | 3 | 22.0 + TT | 14,5 - TT |  |
| 3 | 20 | 0 | 2 | 0 | 18 | 4,0 | 3 | 22.0 + TT | 14 - TT |  |
| 4 | 20 | 0 | 3.5 | 0 | 16,5 | 5,0 | 3 | 22.0 + TT | 11,5 - TT |  |
| 5 | 20 | 0 | 5 | 0 | 15 | 5,0 | 3 | 22.0 + TT | 10 - TT |  |
| 6 | 20 | 0 | 3.5 | 0 | 16,5 | 5,0 | 3 | 22.0 + TT | 11,5 - TT |  |
| 7 | 20 | 0 | 3.5 | 0 | 16,5 | 5,0 | 3 | 22.0 + TT | 11,5 - TT |  |
| 8 | 20 | 0 | 3.5 | 0 | 16,5 | 5,0 | 3 | 22.0 + TT | 11,5 - TT |  |
| 9 | 20 | 0 | 4 | 0 | 16 | 5,0 | 3 | 22.0 + TT | 11 - TT |  |
| 10 | 20 | 0 | 4 | 0 | 16 | 5,0 | 3 | 22.0 + TT | 11 - TT |  |
| 11 | 20 | 0 | 4 | 0 | 16 | 5,0 | 3 | 22.0 + TT | 11 - TT |  |
| 12 | 20 | 0 | 5.5 | 0 | 14,5 | 5,0 | 3 | 22.0 + TT | 9,5 - TT |  |
| 13 | 20 | 0 | 5.5 | 0 | 14,5 | 5,0 | 3 | 22.0 + TT | 9,5 - TT |  |
| 14 | 20 | 0 | 5.5 | 0 | 14,5 | 5,0 | 3 | 22.0 + TT | 9,5 - TT |  |
| 15 | 20 | 0 | 7 | 0 | 13 | 5,0 | 3 | 22.0 + TT | 8 - TT |  |
| 16 | 20 | 0 | 7 | 0 | 13 | 5,0 | 3 | 22.0 + TT | 8 - TT |  |
| 17 | 20 | 0 | 7 | 0 | 13 | 5,0 | 3 | 22.0 + TT | 8 - TT |  |
| 18 | 20 | 0 | 1.5 | 0 | 18,5 | 4,0 | 3 | 22.0 + TT | 14,5 - TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2F.2.5-2. |  |  |  |  |  |  |  |  |  |

Table 6.2F.2.5-2: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 5.925GHz | 5.925GHz < f ≤ 7.125GHz |
| --- | --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |

#### << End of changes >>