3GPP TSG RAN WG5#105	R5-246778

# Orlando, USA, November 18 – November 22 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0502 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability for new NR-U test case 6.2A.3.1.1_1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 6.2A.3.1.1_1 has been added to 38.521-4 and applicability for the test case needs to be added to 38.522. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new NR-U test case 6.2A.3.1.1_1 to Table 4.1.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicability for test case 6.2A.3.1.1_1 will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### << Start of changes >>

### 4.1.4	Performance conformance test cases

Table 4.1.4-1: Applicability of performance test cases, ref. TS 38.521-4 [4]

| Clause | TC Title | Release |  | Applicability |  |  |  | Tested Bands Selection |  | Additional Information |  | Subtest Selection Criteria |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Condition |  | Comment |  |  |  |  |  |  |  |
| 5 | Demodulation performance requirements (Conducted requirements) |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2 | PDSCH demodulation requirements |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2.1.1.1 | 1Rx FDD FR1 PDSCH performance for RedCap | Rel-17 |  | C177a |  | RedCap UEs supporting 5GS FDD FR1 and 1Rx UE capability |  | D008 |  | NOTE 1 |  | Subtest 1-4: F023 |  |
| 5.2.1.1.2 | 1Rx FDD FR1 PDSCH performance for eRedCap | Rel-18 |  | TBD |  | eRedCap UEs supporting 5GS FDD FR1 and 1Rx UE capability |  | TBD |  | NOTE 1 |  |  |  |
| 5.2.1.2.1 | 1Rx TDD FR1 PDSCH performance for RedCap | Rel-17 |  | C177c |  | RedCap UEs supporting 5GS TDD FR1 and 1Rx UE capability |  | D009 |  |  |  | Subtest 1-4: F023 |  |
| 5.2.1.2.2 | 1Rx TDD FR1 PDSCH performance for eRedCap | Rel-18 |  | TBD |  | eRedCap UEs supporting 5GS TDD FR1 and 1Rx UE capability |  | TBD |  | NOTE 1 |  |  |  |
| 5.2.2.1.1_1 | 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  | Subtest 1-3: F023 |  |
| 5.2.2.1.1_2 | 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with enhanced receiver type 1 for both SA and NSA | Rel-15 |  | C015x |  | UEs supporting 5GS FDD FR1 and Enhanced Receiver Type 1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.1_3 | 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for DL1024QAM for both SA and NSA | Rel-17 |  | C200 |  | UEs supporting 5GS FDD FR1 and DL1024QAM and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.1_4 | 2Rx FDD FR1 PDSCH mapping Type A performance – 2x2 MIMO with enhanced DMRS for both SA and NSA | Rel-18 |  | C341 |  | UEs supporting 5GS FDD FR1 and Rel-18 enhanced DMRS and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.2_1 | 2Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.3_1 | 2Rx FDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C015b |  | UEs supporting 5GS FDD FR1 and PDSCH mapping Type B and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.4_1 | 2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C015y |  | UEs supporting 5GS FDD FR1 and receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  | Subtest 1-1 execution not necessary if subtest 1-2 is executed. |  | Subtest 1-2: F022 |  |
| 5.2.2.1.5_1 | 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C074 |  | UEs supporting 5GS FDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.6_1 | 2Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C120 |  | UEs supporting 5GS FDD FR1 and aggregationFactorDL > 1 for PDSCH repetition multislots and alternative 64QAM MCS table for PDSCH and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.7_1 | 2Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C116 |  | UEs supporting 5GS FDD FR1 and PDSCH processing capability 2 and PDSCH mapping type B and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.8_1 | 2Rx FDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C121 |  | UEs supporting 5GS FDD FR1 and PDSCH pre-emption indication and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.9_1 | 2Rx FDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C099 |  | UEs supporting 5GS FDD FR1 and enhanced demodulation processing for HST-SFN joint transmission scheme and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.10_1 | 2Rx FDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C152 |  | UEs supporting 5GS FDD FR1 and number of active TCI states per BWP per CC and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.11_1 | 2Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 |  | C070 |  | UEs supporting 5GS FDD FR1 and single DCI based spatial division multiplexing scheme and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.12_1 | 2Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 |  | C113 |  | UEs supporting 5GS FDD FR1 and multi-DCI based multi-TRP and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.13_1 | 2Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | Rel-16 |  | C114 |  | UEs supporting 5GS FDD FR1 and single DCI based FDMSchemeA and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.14_1 | 2Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 |  | C115 |  | UEs supporting 5GS FDD FR1 and single-DCI based inter-slot TDM and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  | Test execution not necessary if 5.2.2.1.6_1 is executed. |  |  |  |
| 5.2.2.1.15_1 | 2Rx FDD FR1 PDSCH with inter-cell interference - 2x2 MIMO for both SA and NSA | Rel-15, Rel-16 |  | C015d |  | UEs supporting 5GS FDD FR1 and MMSE-IRC receiver and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
|  |  | Rel-17 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.16_1 | 2Rx FDD FR1 for PDSCH with intra cell inter user interference performance – 2x2 MIMO for both NSA and SA | Rel-15, Rel-16 |  | C015d |  | UEs supporting 5GS FDD FR1 and MMSE-IRC receiver and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
|  |  | Rel-17 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.17 | 2Rx FDD FR1 PDSCH performance for RedCap | Rel-17 |  | C177b |  | RedCap UEs supporting 5GS FDD FR1 and 2Rx UE capability |  | D008 |  | NOTE 1 |  | Subtest 1-3: F023 |  |
| 5.2.2.1.18_1 | 2Rx FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 2x2 MIMO for both NSA and SA | Rel-17 |  | C015 |  | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability |  | D008 |  |  |  |  |  |
| 5.2.2.1.19_1 | 2Rx FDD FR1 for PDSCH with inter cell CRS interference scenario – 4x2 MIMO for both NSA and SA | Rel-17 |  | C015 |  | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability |  | D008 |  | Subtest 1-1 execution not necessary if subtest 2-1 is executed. |  |  |  |
| 5.2.2.1.20 | 2Rx FDD FR1 PDSCH HST-SFN Scheme A performance - 2x2 MIMO for both SA and NSA | Rel-17 |  | C245 |  | UEs supporting 5GS FDD FR1 and SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.2.2.1.21 | 2Rx FDD FR1 PDSCH HST-SFN Scheme B performance - 2x2 MIMO for both SA and NSA | Rel-17 |  | C246 |  | UEs supporting 5GS FDD FR1 and SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  | Test execution is not necessary if TC 5.2.2.1.20 is executed. |  |  |  |
| 5.2.2.2.1_1 | 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D009 D010 D029 |  |  |  | Subtest 1-3: F023 |  |
| 5.2.2.2.1_2 | 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with enhanced receiver type 1 for both SA and NSA | Rel-15 |  | C016x |  | UEs supporting 5GS TDD FR1 and Enhanced Receiver Type 1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.2.2.2.1_3 | 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for DL1024QAM for both SA and NSA | Rel-17 |  | C201 |  | UEs supporting 5GS TDD FR1 and DL 1024QAM and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.2.2.2.2_1 | 2Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.2.2.2.3_1 | 2Rx TDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C016b |  | UEs supporting 5GS TDD FR1 and PDSCH mapping Type B and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.2.2.2.4_1 | 2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C016y |  | UEs supporting 5GS TDD FR1 and receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D019 D029 |  | Subtest 1-1 execution not necessary if subtest 1-2 is executed. |  | Subtest 1-2: F022 |  |
| 5.2.2.2.5_1 | 2Rx TDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C075 |  | UEs supporting 5GS TDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D009 D029 |  |  |  |  |  |
| 5.2.2.2.6_1 | 2Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C122 |  | UEs supporting 5GS TDD FR1 and aggregationFactorDL > 1 for PDSCH repetition multislots and alternative 64QAM MCS table for PDSCH and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.2.2.2.7_1 | 2Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C117 |  | UEs supporting 5GS TDD FR1 and PDSCH processing capability 2 and PDSCH mapping type B and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.2.2.2.8_1 | 2Rx TDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C123 |  | UEs supporting 5GS TDD FR1 and PDSCH pre-emption indication and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.2.2.2.9_1 | 2Rx TDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C099a |  | UEs D029supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability |  | D010 |  |  |  |  |  |
| 5.2.2.2.10_1 | 2Rx TDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C153 |  | UEs supporting 5GS TDD FR1 and number of active TCI states per BWP per CC and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.2.2.2.11_1 | 2Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 |  | C071 |  | UEs supporting 5GS TDD FR1 and single DCI based spatial division multiplexing scheme and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D009 D029 |  |  |  |  |  |
| 5.2.2.2.12_1 | 2Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 |  | C113a |  | UEs supporting 5GS TDD FR1 and multi-DCI based multi-TRP and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D009 D029 |  |  |  |  |  |
| 5.2.2.2.13_1 | 2Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | Rel-16 |  | C114a |  | UEs supporting 5GS TDD FR1 and single DCI based FDMSchemeA and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D009 D029 |  |  |  |  |  |
| 5.2.2.2.14_1 | 2Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 |  | C115a |  | UEs supporting 5GS TDD FR1 and single-DCI based inter-slot TDM and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D009 D029 |  | Test execution not necessary if 5.2.2.2.6_1 is executed. |  |  |  |
| 5.2.2.2.15 | 2Rx TDD FR1 PDSCH mapping type A performance on band with shared spectrum access | Rel-16 |  | C204 |  | UEs supporting 5GS TDD FR1 and NR-U |  | D025 |  | NOTE 1 |  |  |  |
| 5.2.2.2.16_1 | 2Rx TDD FR1 for PDSCH with inter-cell interference performance – 2x2 MIMO for both NSA and SA | Rel-15, Rel-16 |  | C016d |  | UEs supporting 5GS TDD FR1 and MMSE-IRC receiver and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
|  |  | Rel-17 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.2.2.2.17_1 | 2Rx TDD FR1 for PDSCH with intra cell inter user interference performance – 2x2 MIMO for both NSA and SA | Rel-15, Rel-16 |  | C016d |  | UEs supporting 5GS TDD FR1 and MMSE-IRC receiver and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
|  |  | Rel-17 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.2.2.2.18 | 2Rx TDD FR1 PDSCH performance for RedCap | Rel-17 |  | C177b |  | RedCap UEs supporting 5GS TDD FR1 and 2Rx UE capability |  | D009 |  | NOTE 1 |  | Subtest 1-2: F023 |  |
| 5.2.2.2.19_1 | 2Rx TDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 4x2 MIMO for both NSA and SA | Rel-17 |  | C016 |  | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability |  | D009 |  |  |  |  |  |
| 5.2.2.2.20_1 | 2Rx TDD FR1 for PDSCH with inter cell CRS interference scenario – 4x2 MIMO for both NSA and SA | Rel-17 |  | C016 |  | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability |  | D009 |  | Subtest 1-1 execution not necessary if subtest 2-1 is executed. Subtest 1-2 execution not necessary if subtest 2-2 is executed. |  |  |  |
| 5.2.2.2.21 | 2Rx TDD FR1 PDSCH HST-SFN Scheme A performance - 2x2 MIMO for both SA and NSA | Rel-17 |  | C247 |  | UEs supporting 5GS TDD FR1 and SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.2.2.2.22 | 2Rx TDD FR1 PDSCH HST-SFN Scheme B performance - 2x2 MIMO for both SA and NSA | Rel-17 |  | C248 |  | UEs supporting 5GS TDD FR1 and SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  | Test execution is not necessary if TC 5.2.2.2.21 is executed. |  |  |  |
| 5.2.3.1.1_1 | 4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  |  |  | Subtest 1-3: F023 |  |
| 5.2.3.1.1_2 | 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.1_4 | 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA | Rel-15 |  | C017x |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and Enhanced Receiver Type 1 |  | D008 |  |  |  |  |  |
| 5.2.3.1.1_5 | 4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for DL1024QAM for both SA and NSA | Rel-17 |  | C202 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and DL1024QAM |  | D008 |  |  |  |  |  |
| 5.2.3.1.2_1 | 4Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.3_1 | 4Rx FDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C017b |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and PDSCH mapping Type B |  | D008 D011 |  |  |  |  |  |
| 5.2.3.1.4_1 | 4Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C017y |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS |  | D008 |  | Subtest 1-1 execution not necessary if subtest 1-2 is executed. |  | Subtest 1-2: F022 |  |
| 5.2.3.1.5_1 | 4Rx FDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C076 |  | UEs supporting 5GS FDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.6_1 | 4Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C124 |  | UEs supporting 5GS FDD FR1 and aggregationFactorDL > 1 for PDSCH repetition multislots and alternative 64QAM MCS table for PDSCH and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.7_1 | 4Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C118 |  | UEs supporting 5GS FDD FR1 and PDSCH processing capability 2 and PDSCH mapping type B and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.8_1 | 4Rx FDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C169 |  | UEs supporting 5GS FDD FR1 and PDSCH pre-emption indication and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.9_1 | 4Rx FDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C127 |  | UEs supporting 5GS FDD FR1 and enhanced demodulation processing for HST-SFN joint transmission scheme and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.10_1 | 4Rx FDD FR1 HST-DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C154 |  | UEs supporting 5GS FDD FR1 and number of active TCI states per BWP per CC and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.11_1 | 4Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x4 MIMO for both SA and NSA | Rel-16 |  | C072 |  | UEs supporting 5GS FDD FR1 and single DCI based spatial division multiplexing scheme and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.12_1 | 4Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x4 MIMO for both SA and NSA | Rel-16 |  | C113b |  | UEs supporting 5GS FDD FR1 and multi-DCI based multi-TRP and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.13_1 | 4Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x4 MIMO for both SA and NSA | Rel-16 |  | C114b |  | UEs supporting 5GS FDD FR1 and single DCI based FDMSchemeA and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.14_1 | 4Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x4 MIMO for both SA and NSA | Rel-16 |  | C115b |  | UEs supporting 5GS FDD FR1 and single-DCI based inter-slot TDM and 4Rx antenna ports |  | D008 |  | Test execution not necessary if 5.2.3.1.6_1 is executed. |  |  |  |
| 5.2.3.1.15_1 | 4Rx FDD FR1 PDSCH with inter-cell interference - 2x4 MIMO for both SA and NSA | Rel-15, Rel-16 |  | C017d |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and MMSE-IRC receiver |  | D008 |  |  |  |  |  |
|  |  | Rel-17 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.16_1 | 4Rx FDD FR1 for PDSCH with intra cell inter user interference performance – 2x4 MIMO for both NSA and SA | Rel-15, Rel-16 |  | C017d |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and MMSE-IRC receiver |  | D008 |  |  |  |  |  |
|  |  | Rel-17 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.16_2 | 4Rx FDD FR1 for PDSCH with intra cell inter user interference performance – 4x4 MIMO for both NSA and SA | Rel-15, Rel-16 |  | C017d |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and MMSE-IRC receiver |  | D008 |  |  |  |  |  |
|  |  | Rel-17 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.17_1 | 4Rx FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 2x4 MIMO for both NSA and SA | Rel-17 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.18_1 | 4Rx FDD FR1 for PDSCH with inter cell CRS interference scenario – 4x4 MIMO for both NSA and SA | Rel-17 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  | Subtest 1-1 execution not necessary if subtest 2-1 is executed. |  |  |  |
| 5.2.3.1.19 | 4Rx FDD FR1 PDSCH HST-SFN Scheme A performance - 2x4 MIMO for both SA and NSA | Rel-17 |  | C249 |  | UEs supporting 5GS FDD FR1 and SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH, and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.2.3.1.20 | 4Rx FDD FR1 PDSCH HST-SFN Scheme B performance - 2x4 MIMO for both SA and NSA | Rel-17 |  | C250 |  | UEs supporting 5GS FDD FR1 and SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH, and 4Rx antenna ports |  | D008 |  | Test execution is not necessary if TC 5.2.3.1.19 is executed. |  |  |  |
| 5.2.3.2.1_1 | 4Rx TDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D009 D010 |  |  |  | Subtest 1-3: F023 |  |
| 5.2.3.2.1_2 | 4Rx TDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 5.2.3.2.1_4 | 4Rx TDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA | Rel-15 |  | C019x |  | UEs supporting 5GS TDD FR1 and Enhanced Receiver Type 1 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 5.2.3.2.1_5 | 4Rx TDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for DL1024QAM for both SA and NSA | Rel-17 |  | C203 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and DL 1024QAM |  | D010 |  |  |  |  |  |
| 5.2.3.2.2_1 | 4Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D009 D010 |  |  |  |  |  |
| 5.2.3.2.3_1 | 4Rx TDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C019b |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and PDSCH mapping Type B |  | D009 D011 |  |  |  |  |  |
| 5.2.3.2.4_1 | 4Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C017z |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS |  | D009 |  | Subtest 1-1 execution not necessary if subtest 1-2 is executed. |  | Subtest 1-2: F022 |  |
| 5.2.3.2.5_1 | 4Rx TDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C077 |  | UEs supporting 5GS TDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 4Rx antenna ports |  | D009 |  |  |  |  |  |
| 5.2.3.2.6_1 | 4Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C125 |  | UEs supporting 5GS TDD FR1 and aggregationFactorDL > 1 for PDSCH repetition multislots and alternative 64QAM MCS table for PDSCH and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 5.2.3.2.7_1 | 4Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C119 |  | UEs supporting 5GS TDD FR1 and PDSCH processing capability 2 and PDSCH mapping type B and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 5.2.3.2.8_1 | 4Rx TDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-16 |  | C170 |  | UEs supporting 5GS TDD FR1 and PDSCH pre-emption indication and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 5.2.3.2.9_1 | 4Rx TDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C019y |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and enhanced demodulation processing for HST-SFN joint transmission scheme |  | D010 D011 |  |  |  |  |  |
| 5.2.3.2.10_1 | 4Rx TDD FR1 HST DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 D011 |  | NOTE 1 |  |  |  |
| 5.2.3.2.11_1 | 4Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x4 MIMO for both SA and NSA | Rel-16 |  | C073 |  | UEs supporting 5GS TDD FR1 and single DCI based spatial division multiplexing scheme and 4Rx antenna ports |  | D009 |  |  |  |  |  |
| 5.2.3.2.12_1 | 4Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x4 MIMO for both SA and NSA | Rel-16 |  | C113c |  | UEs supporting 5GS TDD FR1 and multi-DCI based multi-TRP and 4Rx antenna ports |  | D009 |  |  |  |  |  |
| 5.2.3.2.13_1 | 4Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x4 MIMO for both SA and NSA | Rel-16 |  | C114c |  | UEs supporting 5GS TDD FR1 and single DCI based FDMSchemeA and 4Rx antenna ports |  | D009 |  |  |  |  |  |
| 5.2.3.2.14_1 | 4Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x4 MIMO for both SA and NSA | Rel-16 |  | C115c |  | UEs supporting 5GS TDD FR1 and single-DCI based inter-slot TDM and 4Rx antenna ports |  | D009 |  | Test execution not necessary if 5.2.3.2.6_1 is executed. |  |  |  |
| 5.2.3.2.15 | 4Rx TDD FR1 PDSCH mapping type A performance on band with shared spectrum access | Rel-16 |  | C205 |  | UEs supporting 5GS FDD FR1 and NR-U |  | D025 |  | NOTE 1 |  |  |  |
| 5.2.3.2.16_1 | 4Rx TDD FR1 for PDSCH with inter-cell interference performance – 2x4 MIMO for both NSA and SA | Rel-15, Rel-16 |  | C019d |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and MMSE-IRC receiver |  | D010 |  |  |  |  |  |
|  |  | Rel-17 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 5.2.3.2.17_1 | 4Rx TDD FR1 for PDSCH with intra cell inter user interference performance – 2x4 MIMO for both NSA and SA | Rel-15, Rel-16 |  | C019d |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and MMSE-IRC receiver |  | D010 |  |  |  |  |  |
|  |  | Rel-17 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 5.2.3.2.17_2 | 4Rx TDD FR1 for PDSCH with intra cell inter user interference performance – 4x4 MIMO for both NSA and SA | Rel-15, Rel-16 |  | C019d |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and MMSE-IRC receiver |  | D010 |  |  |  |  |  |
|  |  | Rel-17 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 5.2.3.2.18_1 | 4Rx TDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 4x4 MIMO for both NSA and SA | Rel-17 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D009 |  |  |  |  |  |
| 5.2.3.2.19_1 | 4Rx TDD FR1 for PDSCH with inter cell CRS interference scenario – 4x4 MIMO for both NSA and SA | Rel-17 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D009 |  | Subtest 1-1 execution not necessary if subtest 2-1 is executed. Subtest 1-2 execution not necessary if subtest 2-2 is executed. |  |  |  |
| 5.2.3.2.20 | 4Rx TDD FR1 PDSCH HST-SFN Scheme A performance - 2x4 MIMO for both SA and NSA | Rel-17 |  | C251 |  | UEs supporting 5GS TDD FR1 and SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH, and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 5.2.3.2.21 | 4Rx TDD FR1 PDSCH HST-SFN Scheme B performance - 2x4 MIMO for both SA and NSA | Rel-17 |  | C252 |  | UEs supporting 5GS TDD FR1 and SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH, and 4Rx antenna ports |  | D010 |  | Test execution is not necessary if TC 5.2.3.2.20 is executed. |  |  |  |
| 5.2A.2.1.1 | 2Rx Normal PDSCH Demodulation Performance for CA (2DL CA) for both SA and NSA | Rel-15 |  | C261 |  | UEs supporting 5GS FR1 AND 2DL CA but not supporting 4Rx UE capability on any CCs |  | E016 |  | Test execution not necessary if 5.2A.2.1.2 is executed. |  |  |  |
| 5.2A.2.1.2 | 2Rx Normal PDSCH Demodulation Performance for CA (3DL CA) for both SA and NSA | Rel-15 |  | C262 |  | UEs supporting 5GS FR1 AND 3DL CA but not supporting 4Rx UE capability on any CCs |  | E017 |  | Test execution not necessary if 5.2A.2.1.3 is executed. |  |  |  |
| 5.2A.2.1.3 | 2Rx Normal PDSCH Demodulation Performance for CA (4DL CA) for both SA and NSA | Rel-15 |  | C263 |  | UEs supporting 5GS FR1 AND 4DL CA but not supporting 4Rx UE capability on any CCs |  | E018 |  |  |  |  |  |
| 5.2A.2.2.1 | 2Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) | Rel-15 |  | C261 |  | UEs supporting 5GS FR1 AND 2DL CA but not supporting 4Rx UE capability on any CCs |  | E003a |  | Test execution not necessary if 5.2A.2.2.2 is executed. |  |  |  |
| 5.2A.2.2.2 | 2Rx PDSCH Demodulation Performance for CA with power imbalance (3DL CA) | Rel-15 |  | FFS |  | UEs supporting 5GS FR1 AND 3DL CA but not supporting 4Rx UE capability on any CCs |  | E033 |  | NOTE 1 Test execution not necessary if 5.2A.2.2.3 is executed. |  |  |  |
| 5.2A.2.2.3 | 2Rx PDSCH Demodulation Performance for CA with power imbalance (4DL CA) | Rel-15 |  | FFS |  | UEs supporting 5GS FR1 and 4D LCA but not supporting 4Rx UE capability on any 4CCs |  | E034 |  | NOTE 1 |  |  |  |
| 5.2A.2.3 | 2Rx TDD FR1 PDSCH mapping type A performance of Scell on band with shared spectrum access | Rel-16 |  | C204 |  | UEs supporting 5GS TDD FR1 and NR-U |  | D025 |  | NOTE 1 |  |  |  |
| 5.2A.2.4.1 | 2RX PDSCH Demodulation Performance for HST-SFN CA | Rel-16 |  | C152b |  | UEs supporting 5GS FR1 AND enhanced demodulation processing for carrier aggregation for HST-SFN joint transmission and 2Rx antenna ports, but not supporting 4Rx UE capability on any CCs |  | E016 D029 |  |  |  |  |  |
| 5.2A.2.5.1 | 2RX PDSCH Demodulation Performance for HST-DPS CA | Rel-15 |  | C152a |  | UEs supporting 5GS FR1 AND 2DL CA AND number of active TCI and 2Rx antenna ports, but not supporting 4Rx UE capability on any CCs |  | E016 D029 |  |  |  |  |  |
| 5.2A.3.1.1 | 4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) | Rel-15 |  | C017g |  | UEs supporting 5GS FR1 AND 2DL CA AND supporting 4Rx antenna ports on all CCs |  | E016 |  | Test execution not necessary if 5.2A.3.1.2 is executed. |  |  |  |
| 5.2A.3.1.2 | 4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | Rel-15 |  | C017h |  | UEs supporting 5GS FR1 AND 3DL CA AND supporting 4Rx antenna ports on all CCs |  | E017 |  | Test execution not necessary if 5.2A.3.1.3 is executed. |  |  |  |
| 5.2A.3.1.3 | 4Rx Normal PDSCH Demodulation Performance for CA (4DL CA) | Rel-15 |  | C017i |  | UEs supporting 5GS FR1 AND 4DL CA AND supporting 4Rx antenna ports on all CCs |  | E018 |  |  |  |  |  |
| 5.2A.3.2.1 | 4Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) | Rel-15 |  | C017g |  | UEs supporting 5GS FR1 AND 2DL CA AND supporting 4Rx antenna ports on all CCs |  | E003a |  |  |  |  |  |
| 5.2A.3.3 | 4Rx TDD FR1 PDSCH mapping type A performance of Scell on band with shared spectrum access | Rel-16 |  | C205 |  | UEs supporting 5GS TDD FR1 and NR-U |  | D025 |  | NOTE 1 |  |  |  |
| 5.2A.3.4.1 | 4RX PDSCH Demodulation Performance for HST-SFN CA | Rel-16 |  | C127a |  | UEs supporting 5GS FR1 AND enhanced demodulation processing for carrier aggregation for HST-SFN joint transmission AND supporting 4Rx TDD and FDD UE capability on any CCs |  | E016 |  |  |  |  |  |
| 5.2A.3.5.1 | 4RX PDSCH Demodulation Performance for HST-DPS CA | Rel-15 |  | C154a |  | UEs supporting 5GS FR1 AND 2DL CA AND number of active TCI AND supporting 4Rx TDD and FDD UE capability on any CCs |  | E016 |  |  |  |  |  |
| 5.2A.3A.1.1 | 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) | Rel-15 |  | C017g |  | UEs supporting 5GS FR1 and 2DL CA AND supporting 4Rx UE capability on some of the CCs |  | E016 |  | Test execution not necessary if 5.2A.3A.1.2 is executed. |  |  |  |
| 5.2A.3A.1.2 | 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | Rel-15 |  | C017h |  | UEs supporting 5GS FR1 and 3DL CA AND supporting 4Rx UE capability on some of the CCs |  | E017 |  | Test execution not necessary if 5.2A.3A.1.3 is executed. |  |  |  |
| 5.2A.3A.1.3 | 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (4DL CA) | Rel-15 |  | C017i |  | UEs supporting 5GS FR1 and 4DL CA AND supporting 4Rx UE capability on some of the CCs |  | E018 |  |  |  |  |  |
| 5.3.1.1.1 | 1Rx FDD FR1 PDCCH performance for RedCap | Rel-17 |  | C177a |  | RedCap UEs supporting 5GS FDD FR1 and 1Rx UE capability |  | D008 |  |  |  |  |  |
| 5.3.2.1.1 | 2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.3.2.1.2 | 2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.3.2.1.3 | 2Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving | Rel-16 |  | C088 |  | UEs supporting 5GS FDD FR1 and Long DRX Cycle and DRX adaptation and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.3.2.1.4 | 2Rx FDD FR1 PDCCH performance for RedCap | Rel-17 |  | C177b |  | RedCap UEs supporting 5GS FDD FR1and 2Rx UE capability |  | D008 |  |  |  |  |  |
| 5.3.2.1.5 | 2RX FDD Minimum requirements for PDCCH with intra-slot repetition | Rel-17 |  | C015e |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 5.3.2.2.1 | 2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.3.2.2.2 | 2Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.3.2.2.3 | 2Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving | Rel-16 |  | C089 |  | UEs supporting 5GS TDD FR1 and Long DRX Cycle and DRX adaptation and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 5.3.2.2.4 | 2Rx TDD FR1 PDCCH performance for RedCap | Rel-17 |  | C177d |  | RedCap UEs supporting 5GS TDD FR1 and 2Rx UE capability |  | D009 |  |  |  |  |  |
| 5.3.2.2.5 | 2RX TDD Minimum requirements for PDCCH with intra-slot repetition | Rel-17 |  | C016e |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D009 D010 D029 |  |  |  |  |  |
| 5.3.3.1.1 | 4Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.3.3.1.2 | 4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.3.3.1.3 | 4Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving | Rel-16 |  | C090 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and Long DRX Cycle and DRX adaptation |  | D008 |  |  |  |  |  |
| 5.3.3.1.4 | 4RX FDD Minimum requirements for PDCCH with intra-slot repetition | Rel-17 |  | C017e |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 5.3.3.2.1 | 4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 5.3.3.2.2 | 4Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 5.3.3.2.3 | 4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving | Rel-16 |  | C091 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and Long DRX Cycle and DRX adaptation |  | D010 |  |  |  |  |  |
| 5.3.3.2.4 | 4RX TDD Minimum requirements for PDCCH with intra-slot repetition | Rel-17 |  | C019e |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 5.5.1 | FR1 Sustained downlink data rate performance for single carrier | Rel-15 |  | C001 |  | UEs supporting 5GS FDD FR1 or TDD FR1 (SA) |  | D001 |  |  |  |  |  |
| 5.5.2 | FR1 Sustained downlink data rate performance for single carrier with DL1024QAM |  | Rel-17 |  | C001r |  | UEs supporting 5GS FDD FR1 or TDD FR1 (SA) and supporting DL 1024QAM |  | D001 |  |  |  |  |  |
| 5.5A.1.1 | FR1 SDR performance for CA (2DL CA) | Rel-15 |  | C001e |  | UEs supporting 5GS FDD FR1 or TDD FR1 (SA) and supporting 2DL CA |  | E016 |  |  |  |  |  |
| 5.5A.1.2 | FR1 SDR performance for CA (3DL CA) | Rel-15 |  | C001i |  | UEs supporting 5GS FDD FR1 or TDD FR1 (SA) and supporting 3DL CA |  | E017 |  |  |  |  |  |
| 5.5A.1.3 | FR1 SDR performance for CA (4DL CA) | Rel-15 |  | C001j |  | UEs supporting 5GS FDD FR1 or TDD FR1 (SA) and supporting 4DL CA |  | E018 |  |  |  |  |  |
| 5.5A.1.4 | FR1 SDR performance for CA (5DL CA) | Rel-15 |  | C001k |  | UEs supporting 5GS FDD FR1 or TDD FR1 (SA) and supporting 5DL CA |  | E019 |  |  |  |  |  |
| 6 | CSI reporting requirements (Conducted requirements) |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2.1.1.1.1 | 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap | Rel-17 |  | C177a |  | RedCap UEs supporting 5GS FDD FR1 and 1Rx UE capability |  | D008 |  |  |  |  |  |
| 6.2.1.1.1.2 | 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for eRedCap | Rel-18 |  | TBD |  | eRedCap UEs supporting 5GS FDD FR1 and 1Rx UE capability |  | TBD |  | NOTE 1 |  |  |  |
| 6.2.1.1.2.1 | 1Rx FDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | Rel-17 |  | C177a |  | RedCap UEs supporting 5GS FDD FR1 and 1Rx UE capability |  | D008 |  |  |  |  |  |
| 6.2.1.2.1.1 | 1Rx TDD FR1 periodic CQI reporting under AWGN conditions for RedCap | Rel-17 |  | C177c |  | RedCap UEs supporting 5GS TDD FR1 and 1Rx UE capability |  | D009 |  |  |  |  |  |
| 6.2.1.2.2.1 | 1Rx TDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | Rel-17 |  | C177c |  | RedCap UEs supporting 5GS TDD FR1 and 1Rx UE capability |  | D009 |  |  |  |  |  |
| 6.2.2.1.1.1 | 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Rel-15 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 6.2.2.1.1.2 | 2Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | Rel-16 |  | C074 |  | UEs supporting 5GS FDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 6.2.2.1.1.4 | 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap for SA | Rel-17 |  | C177b |  | RedCap UEs supporting 5GS FDD FR1 and 2Rx UE capability |  | D008 |  |  |  |  |  |
| 6.2.2.1.2.1 | 2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Rel-15 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 6.2.2.1.2.2 | 2Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Rel-15 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 6.2.2.1.2.3 | 2Rx FDD FR1 Wideband CQI reporting with inter-cell intereference | Rel-15 |  | C015d |  | UEs supporting 5GS FDD FR1 and MMSE-IRC receiver and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 6.2.2.1.2.4 | 2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | Rel-17 |  | C177b |  | RedCap UEs supporting 5GS FDD FR1 and 2Rx UE capability |  | D008 |  |  |  |  |  |
| 6.2.2.2.1.1 | 2Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Rel-15 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 6.2.2.2.1.2 | 2Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | Rel-16 |  | C075 |  | UEs supporting 5GS TDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 6.2.2.2.1.5 | 2Rx TDD FR1 periodic CQI reporting under AWGN conditions for RedCap | Rel-17 |  | C177d |  | RedCap UEs supporting 5GS TDD FR1 and 2Rx UE capability |  | D009 |  |  |  |  |  |
| 6.2.2.2.2.1 | 2Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Rel-15 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 6.2.2.2.2.2 | 2Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Rel-15 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 6.2.2.2.2.3 | 2Rx TDD FR1 Wideband CQI reporting with inter-cell intereference | Rel-15 |  | C016d |  | UEs supporting 5GS TDD FR1 and MMSE-IRC receiver and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 6.2.3.1.1.1 | 4Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Rel-15 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 6.2.2.2.2.4 | 2Rx TDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | Rel-17 |  | C177d |  | RedCap UEs supporting 5GS TDD FR1 and 2Rx UE capability |  | D009 |  |  |  |  |  |
| 6.2.3.1.1.2 | 4Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | Rel-16 |  | C076 |  | UEs supporting 5GS FDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 6.2.3.1.2.1 | 4Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Rel-15 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 6.2.3.1.2.2 | 4Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Rel-15 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 |  |  |  |  |  |
| 6.2.3.1.2.3 | 4Rx FDD FR1 Wideband CQI reporting with inter-cell intereference | Rel-15 |  | C017d |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and MMSE-IRC receiver |  | D008 |  |  |  |  |  |
| 6.2.3.2.1.1 | 4Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 6.2.3.2.1.2 | 4Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | Rel-16 |  | C077 |  | UEs supporting 5GS TDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 6.2.3.2.2.1 | 4Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 6.2.3.2.2.2 | 4Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 6.2.3.2.2.3 | 4Rx TDD FR1 Wideband CQI reporting with inter-cell intereference | Rel-15 |  | C019d |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and MMSE-IRC receiver |  | D010 |  |  |  |  |  |
| 6.2A.3.1.1 | CQI reporting accuracy under AWGN conditions for CA (2DL CA) | Rel-15 |  | C031 |  | UEs supporting 5GS FR1 and CA (2DL CA) |  | E016 |  | Test execution not necessary if 6.2A.3.1.2 is executed. |  |  |  |
| 6.2A.3.1.1_1 | 2Rx FR1 CQI reporting under AWGN for Scell on band with shared spectrum access for CA (2DLCA) | Rel-16 |  | C204a |  | UEs supporting 5GS TDD FR1 and NR-U and 2DL CA |  | D025 |  |  |  |  |  |
| 6.2A.3.1.2 | CQI reporting accuracy under AWGN conditions for CA (3DL CA) | Rel-15 |  | C033 |  | UEs supporting 5GS FR1 and CA (3DL CA) |  | E017 |  | Test execution not necessary if 6.2A.3.1.3 is executed. |  |  |  |
| 6.2A.3.1.3 | CQI reporting accuracy under AWGN conditions for CA (4DL CA) | Rel-15 |  | C036 |  | UEs supporting 5GS FR1 and CA (4DL CA) |  | E018 |  |  |  |  |  |
| 6.2A.4.1.1 | 4Rx FR1 CQI reporting under AWGN for Scell on band with shared spectrum access for CA (2DLCA) | Rel-16 |  | C204a |  | UEs supporting 5GS TDD FR1 and NR-U and 2DL CA |  | D025 |  |  |  |  |  |
| 6.3.1.1.1 | 1Rx FDD Single PMI with 4TX TypeI-SinglePanel Codebook for RedCap | Rel-17 |  | C177a |  | RedCap UEs supporting 5GS FDD FR1 and 1Rx UE capability |  | D008 |  |  |  |  |  |
| 6.3.1.2.1 | 1Rx TDD Single PMI with 4TX TypeI-SinglePanel Codebook for RedCap | Rel-17 |  | C177c |  | RedCap UEs supporting 5GS TDD FR1 and 1Rx UE capability |  | D009 |  |  |  |  |  |
| 6.3.2.1.1 | 2Rx FDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 6.3.2.1.2 | 2Rx FDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 6.3.2.1.3 | 2Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | Rel-15 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 6.3.2.1.4 | 2Rx FDD FR1 Single PMI with 32Tx Type1 – SinglePanel Codebook for both SA and NSA | Rel-15 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 6.3.2.1.5 | 2Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | Rel-15 |  | C015c |  | UEs supporting 5GS FDD FR1 and supporting Type II codebook and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 6.3.2.1.6 | 2Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | Rel-16 |  | C128 |  | UEs supporting 5GS FDD FR1 and Enhanced Type II codebook with at least 16 ports per CSI-RS resource and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 6.3.2.1.7 | 2Rx FDD FR1 Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme for both SA and NSA | Rel-17 |  | C070 |  | UEs supporting 5GS FDD FR1 and single DCI based spatial division multiplexing scheme and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  | Note1 |  |  |  |
| 6.3.2.2.1 | 2Rx TDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 6.3.2.2.2 | 2Rx TDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 6.3.2.2.3 | 2Rx TDD FR1 Multiple PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA | Rel-15 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 6.3.2.2.4 | 2Rx TDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | Rel-15 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 6.3.2.2.5 | 2Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | Rel-15 |  | C016c |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, and supporting Type II codebook but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 6.3.2.2.6 | 2Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | Rel-16 |  | C129 |  | UEs supporting 5GS TDD FR1 and Enhanced Type II codebook with at least 16 ports per CSI-RS resource and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 6.3.2.2.7 | 2Rx TDD Single PMI with 4TX TypeI-SinglePanel Codebook for RedCap | Rel-17 |  | C177b |  | RedCap UEs supporting 5GS TDD FR1 and 2Rx UE capability |  | D009 |  |  |  |  |  |
| 6.3.2.2.8 | 2Rx TDD FR1 Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme for both SA and NSA | Rel-17 |  | C071 |  | UEs supporting 5GS TDD FR1 and single DCI based spatial division multiplexing scheme and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 6.3.3.1.1 | 4Rx FDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 D011 |  |  |  |  |  |
| 6.3.3.1.2 | 4Rx FDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 D011 |  |  |  |  |  |
| 6.3.3.1.3 | 4Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | Rel-15 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 D011 |  |  |  |  |  |
| 6.3.3.1.4 | 4Rx FDD FR1 Single PMI with 32Tx Type1 – SinglePanel Codebook for both SA and NSA | Rel-15 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 D011 |  |  |  |  |  |
| 6.3.3.1.5 | 4Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | Rel-15 |  | C017c |  | UEs supporting 5GS FDD FR1 and supporting Type II codebook and 4Rx antenna ports |  | D008 D011 |  |  |  |  |  |
| 6.3.3.1.6 | 4Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | Rel-16 |  | C130 |  | UEs supporting 5GS FDD FR1 and Enhanced Type II codebook with at least 16 ports per CSI-RS resource, and 4Rx antenna ports |  | D008 D011 |  |  |  |  |  |
| 6.3.3.1.7 | 4Rx FDD FR1 Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme for both SA and NSA | Rel-17 |  | C072 |  | UEs supporting 5GS FDD FR1 and single DCI based spatial division multiplexing scheme and 4Rx antenna ports |  | D008 D011 |  |  |  |  |  |
| 6.3.3.2.1 | 4Rx TDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D010 D011 |  |  |  |  |  |
| 6.3.3.2.2 | 4Rx TDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D010 D011 |  |  |  |  |  |
| 6.3.3.2.3 | 4Rx TDD FR1 Multiple PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 6.3.3.2.4 | 4Rx TDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 6.3.3.2.5 | 4Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | Rel-15 |  | C019c |  | UEs supporting 5GS TDD FR1 and supporting Type II codebook and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 6.3.3.2.6 | 4Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | Rel-16 |  | C131 |  | UEs supporting 5GS TDD FR1 and Enhanced Type II codebook with at least 16 ports per CSI-RS resource, and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 6.3.3.2.7 | 4Rx TDD FR1 Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme for both SA and NSA | Rel-17 |  | C073 |  | UEs supporting 5GS FDD FR1 and single DCI based spatial division multiplexing scheme and 4Rx antenna ports |  | D010 |  |  |  |  |  |
| 6.4.2.1_1 | 2Rx FDD FR1 RI reporting for both SA and NSA | Rel-15 |  | C015 |  | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability |  | D008 D029 |  |  |  |  |  |
| 6.4.2.1.1 | 2Rx FDD FR1 RI reporting for RedCap | Rel-17 |  | C177b |  | RedCap UEs supporting 5GS FDD FR1 and 2Rx UE capability |  | D008 |  |  |  |  |  |
| 6.4.2.2_1 | 2Rx TDD FR1 RI reporting for both SA and NSA | Rel-15 |  | C016 |  | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability |  | D010 D029 |  |  |  |  |  |
| 6.4.2.2.1 | 2Rx TDD FR1 RI reporting for RedCap | Rel-17 |  | C177d |  | RedCap UEs supporting 5GS TDD FR1 and 2Rx UE capability |  | D009 |  |  |  |  |  |
| 6.4.3.1_1 | 4Rx FDD FR1 RI reporting for both SA and NSA | Rel-15 |  | C017 |  | UEs supporting 5GS FDD FR1 and 4Rx antenna ports |  | D008 D011 |  |  |  |  |  |
| 6.4.3.2_1 | 4Rx TDD FR1 RI reporting for both SA and NSA | Rel-15 |  | C019 |  | UEs supporting 5GS TDD FR1 and 4Rx antenna ports |  | D010 D011 |  |  |  |  |  |
| 7 | Demodulation performance requirements (Radiated requirements) |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.2.2.2.1_1 | 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for SA and NSA | Rel-15 |  | C061 |  | UEs supporting 5GS TDD FR2 |  | D013 D014 D015 |  |  |  |  |  |
| 7.2.2.2.1_2 | 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with enhanced type 1 receiver for SA and NSA | Rel-15 |  | C062c |  | UEs supporting 5GS TDD FR2 and Enhanced Receiver Type 1 |  | D014 |  |  |  |  |  |
| 7.2.2.2.1_3 | 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with 256QAM for SA and NSA (Rel-16 and forward) | Rel-16 |  | C126 |  | UEs supporting 5GS TDD FR2 and PDSCH 256QAM for FR2 |  | D013 |  |  |  |  |  |
| 7.2.2.2.2_1 | 2Rx TDD FR2 PDSCH repetitions over multiple slots - 2x2 MIMO with baseline receiver for SA and NSA | Rel-16 |  | C171 |  | UEs supporting 5GS TDD FR2 and alternative 64QAM MCS table for PDSCH and aggregationFactorDL > 1 for PDSCH repetition multislots |  | D014 |  |  |  |  |  |
| 7.2.2.2.3_1 | 2Rx TDD FR2 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for SA and NSA | Rel-16 |  | C172 |  | UEs supporting 5GS TDD FR2 and aggregationFactorDL > 1 for PDSCH repetition multislots |  | D014 |  |  |  |  |  |
| 7.2.2.2.4_1 | 2Rx TDD FR2 PDSCH for HST-DPS - 2x2 MIMO with baseline receiver for SA and NSA | Rel-17 |  | C333 |  | 2Rx TDD FR2 PDSCH for HST-DPS - 2x2 MIMO with baseline receiver for SA and NSA |  | D015 |  |  |  |  |  |
| 7.2A.2.1 | 2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (2DLCA) | Rel-15 |  | C061a |  | UEs supporting 5GS TDD FR2 AND 2DL CA |  | E032 |  |  |  |  |  |
| 7.2A.2.2 | 2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (3DLCA) | Rel-15 |  | C061b |  | UEs supporting 5GS TDD FR2 AND 3DL CA |  | E033 |  |  |  |  |  |
| 7.3.2.2.1 | 2Rx TDD FR2 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 |  | C061 |  | UEs supporting 5GS TDD FR2 |  | D014 |  |  |  |  |  |
| 7.3.2.2.2 | 2Rx TDD FR2 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 |  | C061 |  | UEs supporting 5GS TDD FR2 |  | D014 |  |  |  |  |  |
| 7.3.2.2.3 | 2Rx TDD FR2 PDCCH 1 Tx antenna performance for power saving | Rel-16 |  | C092 |  | UEs supporting 5GS TDD FR2 and Long DRX Cycle and DRX adaptation |  | D014 |  |  |  |  |  |
| 7.5.1 | FR2 Sustained downlink data rate performance for single carrier | Rel-15 |  | C061 |  | UEs supporting 5GS TDD FR2 |  | D014 |  |  |  |  |  |
| 7.5.2 | FR2 Sustained downlink data rate performance for RedCap | Rel-17 |  | C197 |  | RedCap UEs supporting 5GS TDD FR2 and 2Rx UE capability |  | D014 |  |  |  |  |  |
| 7.5.1.1 | FR2 SDR performance for CA (2DL CA) | Rel-15 |  | C061a |  | UEs supporting 5GS TDD FR2 and CA (2DL CA) |  | E032 |  |  |  |  |  |
| 7.5.1.2 | FR2 SDR performance for CA (3DL CA) | Rel-15 |  | C061b |  | UEs supporting 5GS TDD FR2 and CA (3DL CA) |  | E033 |  | NOTE 1 |  |  |  |
| 7.5.1.3 | FR2 SDR performance for CA (4DL CA) | Rel-15 |  | TBD |  | UEs supporting 5GS TDD FR2 and CA (4DL CA) |  | E034 |  | NOTE 1 |  |  |  |
| 7.5.1.4 | FR2 SDR performance for CA (5DL CA) | Rel-15 |  | TBD |  | UEs supporting 5GS TDD FR2 and CA (5DL CA) |  | E035 |  | NOTE 1 |  |  |  |
| 7.5.1.5 | FR2 SDR performance for CA (6DL CA) | Rel-15 |  | TBD |  | UEs supporting 5GS TDD FR2 and CA (6DL CA) |  | E036 |  | NOTE 1 |  |  |  |
| 7.5.1.6 | FR2 SDR performance for CA (7DL CA) | Rel-15 |  | TBD |  | UEs supporting 5GS TDD FR2 and CA (7DL CA) |  | E037 |  | NOTE 1 |  |  |  |
| 7.5.1.7 | FR2 SDR performance for CA (8DL CA) | Rel-15 |  | TBD |  | UEs supporting 5GS TDD FR2 and CA (8DL CA) |  | E038 |  |  |  |  |  |
| 8 | CSI reporting requirements (Radiated requirements) |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2.2.2.1.1 | 2Rx TDD FR2 periodic wideband CQI reporting under AWGN performance for both SA and NSA | Rel-15 |  | C061 |  | UEs supporting 5GS TDD FR2 |  | D014 |  |  |  |  |  |
| 8.2.2.2.2.1 | 2Rx TDD FR2 aperiodic wideband CQI reporting under fading performance for both SA and NSA | Rel-15 |  | C061 |  | UEs supporting 5GS TDD FR2 |  | D014 |  | Skip TC 8.2.2.2.2.1 if TS 38.521-4 TC 8.2.2.2.2.1_1 has been executed and passed. |  |  |  |
| 8.2.2.2.2.1_1 | 2Rx TDD FR2 aperiodic CQI wideband reporting under fading performance for both SA and NSA – 256QAM (Rel-16 and forward) | Rel-16 |  | C126 |  | UEs supporting 5GS TDD FR2 and DL 256QAM |  | D013 |  | NOTE 1 |  |  |  |
| 8.2A.3.1.1 | 2Rx CQI reporting accuracy under AWGN conditions for CA (2DL CA) | Rel-15 |  | C006c |  | UEs supporting 5GS FR2 and CA (2DL CA) |  | E032 |  | Test execution not necessary if 8.2A.3.1.2 is executed. |  |  |  |
| 8.2A.3.1.2 | 2Rx CQI reporting accuracy under AWGN conditions for CA (3DL CA) | Rel-15 |  | C006d |  | UEs supporting 5GS FR2 and CA (3DL CA) |  | E033 |  | Test execution not necessary if 8.2A.3.1.3 is executed. |  |  |  |
| 8.2A.3.1.3 | 2Rx CQI reporting accuracy under AWGN conditions for CA (4DL CA) | Rel-15 |  | C006e |  | UEs supporting 5GS FR2 and CA (4DL CA) |  | E034 |  |  |  |  |  |
| 8.3.2.2.1 | 2Rx TDD FR2 Single PMI with 2TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 |  | C061 |  | UEs supporting 5GS TDD FR2 |  | D014 |  |  |  |  |  |
| 8.4.2.2.1 | 2Rx TDD FR2 RI reporting for both SA and NSA | Rel-15 |  | C061 |  | UEs supporting 5GS TDD FR2 |  | D014 |  |  |  |  |  |
| 9 | Demodulation performance requirements for interworking |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.4B.1.1 | Sustained downlink data rate performance for EN-DC within FR1 | Rel-15 |  | C020 |  | UEs supporting 5GS FDD FR1 or TDD FR1 (NSA) |  | D001 |  |  |  |  |  |
| 9.4B.1.2 | Sustained downlink data rate performance for EN-DC including FR2 NR carrier | Rel-15 |  | C006q |  | UEs supporting 5GS TDD FR2 and ENDC |  | D005 |  |  |  |  |  |
| 10 | CSI reporting requirements for interworking |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | V2X requirements |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.1.2.1.1_1 | 2Rx FR1 PSSCH performance - single active PSSCH link | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  |  |  |
| 11.1.3.1.1_1 | 2Rx FR1 PSCCH performance - single active PSSCH link | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  |  |  |
| 11.1.5.1.1_1 | 2Rx FR1 PSCCH performance - single active PSSCH link | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  |  |  |
| 11.1.6.1.1_1 | 2Rx FR1 Power imbalance performance - two active PSSCH link | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  |  |  |
| 11.1.7.1.1_1 | 2Rx FR1 HARQ buffer soft combining performance - maximum number of HARQ processes | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  |  |  |
| 11.1.8.1.1_1 | 2Rx FR1 PSCCH decoding capability - maximum number of received PSCCHs | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  |  |  |
| 11.1.9.1.1_1 | 2Rx FR1 PSFCH decoding capability - maximum number of received PSFCHs | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-4. NOTE 2: Void. NOTE 3: Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### << End of changes >>