# Title:	Test Tolerances Methodology for NR SA inter-frequency handover known target cell under CCA

# Source:	Qualcomm

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for inter-frequency handover under CCA to known target cell RRM test.

The test case in question, from TS 38.533, is:

- 11.2.1.4, NR SA FR1 Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. 	Test Tolerance Methodology for 11.2.1.4 in TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v16.21.0.

#### A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell

##### A.11.2.1.4.1	Test Purpose and Environment

This test is to verify the requirement for the NR with CCA FR1-NR FR1 handover requirements specified in clause 6.1.1.2.

##### A.11.2.1.4.2	Test Parameters

Supported test configurations are shown in table A.11.2.1.4.2-1. Both handover delay and interruption length are tested by using the parameters in table A.11.2.1.4.2-2, and A.11.2.1.4.2-3.

The test consists of three successive time periods, with time durations of T1 T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of Cell 2.

NR with CCA shall send a RRC message implying handover to Cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3. T3 is defined as the end of the last TTI containing the RRC message implying handover.

Table A.11.2.1.4.2-1: Handover from NR with CCA FR1 to NR FR1 test configuration

| Config | Description |
| --- | --- |
| 1 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.2.1.4.2-2: General test parameters handover from NR with CCA FR1 to NR FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | NR cell with CCA |
|  | Neighbouring cell |  | Cell 2 | NR cell |
| Final condition | Active cell |  | Cell 2 |  |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| T304 |  | ms | 500 |  |
| LCCA_DL |  |  | 5 |  |
| WCCA_DL |  | ms | T304 |  |
| LCCA_UL |  |  | 5 |  |
| WCCA_UL |  | ms | T304 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.11.2.1.4.2-3: Cell specific test parameters for NR with CCA FR1 – NR FR1 handover test case

| Parameter |  |  | Unit | Test | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | configuration | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  |  | 1,2,3 | 1 |  |  | 2 |  |  |
| Duplex mode |  |  |  | 1 | TDD |  |  | FDD |  |  |
|  |  |  |  | 2 | TDD |  |  | TDD |  |  |
|  |  |  |  | 3 | TDD |  |  | TDD |  |  |
| TDD configuration |  |  |  | 1 | TDDConf.1.1 CCA |  |  | Not Applicable |  |  |
|  |  |  |  | 2 | TDDConf.1.1 CCA |  |  | TDDConf.1.1 |  |  |
|  |  |  |  | 3 | TDDConf.1.1 CCA |  |  | TDDConf.2.1 |  |  |
| BWchannel |  |  | MHz | 1 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | 2 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | 3 | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| BWP BW |  |  | MHz | 1 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | 2 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | 3 | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| DRX Cycle |  |  | ms | 1,2,3 | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | 1 | SR.1.1 CCA |  |  | SR.1.1 FDD |  |  |
|  |  |  |  | 2 | SR.1.1 CCA |  |  | SR.1.1 TDD |  |  |
|  |  |  |  | 3 | SR.1.1 CCA |  |  | SR2.1 TDD |  |  |
| CORESET Reference Channel |  |  |  | 1 | CR1.1 CCA |  |  | CR.1.1 FDD |  |  |
|  |  |  |  | 2 | CR1.1 CCA |  |  | CR.1.1 TDD |  |  |
|  |  |  |  | 3 | CR1.1 CCA |  |  | CR2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  |  |  | 1 | CCR.1.1 CCA |  |  | CCR.1.1 FDD |  |  |
|  |  |  |  | 2 | CCR.1.1 CCA |  |  | CCR.1.1 TDD |  |  |
|  |  |  |  | 3 | CCR.1.1 CCA |  |  | CCR.2.1 TDD |  |  |
| DL CCA probability for semi-static channel access (PCCA_DL)DL CCA probability PCCA_DL |  |  |  | 1,2,3 | 0.9375 |  |  | N/A |  |  |
| DL CCA probability for dynamic channel access (PCCA_DL_1) |  |  |  | 1,2,3 | 0.75 |  |  | N/A |  |  |
| DL CCA probability for dynamic channel access (PCCA_DL_2) |  |  |  | 1,2,3 | 0.75 |  |  | N/A |  |  |
| UL CCA probability for semi-static channel access PCCA_UL |  |  |  | 1,2,3 | 0.75 |  |  | N/A |  |  |
| UL CCA probability for dynamic static channel access PCCA_UL |  |  |  | 1,2,3 | 0.87 |  |  |  |  |  |
| TRS configuration |  |  |  | 1 | TRS.1.2 TDD |  |  | TRS.1.1 FDD |  |  |
|  |  |  |  | 2 | TRS.1.2 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  |  |  | 3 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration |  | Semi-static channel access |  | 1,2 | SSB.1 CCA (As defined in A.3.10A ) |  |  | SSB.1 FR1 |  |  |
|  |  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3.10A ) |  |  |  |  |  |
|  |  | Semi-static channel access |  | 3 | SSB.1 CCA (As defined in A.3.10A ) |  |  | SSB.2 FR1 |  |  |
|  |  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3.10A ) |  |  |  |  |  |
| DBT window configuration |  |  |  |  | As defined in A.3.28.1 |  |  | Not applicable |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 1,2 | 30 kHz |  |  | 15 kHz |  |  |
|  |  |  |  | 3 | 30 kHz |  |  | 30 kHz |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 1,2 | 30 kHz |  |  | 15 kHz |  |  |
|  |  |  |  | 3 | 30 kHz |  |  | 30 kHz |  |  |
| PRACH configuration |  |  |  |  | FR1 PRACH configuration 1 under CCA |  |  | FR1 PRACH configuration 1 |  |  |
| BWP configuration |  | Initial DL BWP |  | 1,2,3 | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | 1,2,3 | DLBWP.1.1 |  |  |  |  |  |
|  |  | Initial UL BWP |  | 1,2,3 | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | 1,2,3 | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1,2,3 | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote1 |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote1 |  |  |  | 1,2,3 |  |  |  |  |  |  |
| Note2 |  |  | dBm/SCS | 1,2 | [-101] |  |  | -98 |  |  |
|  |  |  |  | 3 | [-101] |  |  | -95 |  |  |
|  |  |  | dB | 1,2,3 | 8 | -3.3 | -3.3 | -Infinity | 2.36 | 2.36 |
|  |  |  | dB | 1,2,3 | 8 | 8 | 8 | -Infinity | 11 | 11 |
| SSB_RP | Config 1 |  | dBm/SCS | 1,2,3 | -90 | -90 | -90 | -Infinity | -87 | -87 |
| IoNote3 | Config 1 |  | dBm/ 9.36MHz | 1,2,3 | -61.41 | -57.06 | -57.06 | -61.41 | -57.06 | -57.06 |
| Propagation condition |  |  | - | 1,2,3 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |  |

##### A.11.2.1.4.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 112 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 102 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 112 ms.

# 3. 	Discussion

The test case consists of 3 test configurations. Each test configuration has 3 time periods: T1, T2 and T3.

During T1 only NR Cell 1 is present and the nominal Ês/Iot value will be +8dB.

During T2 the SS-RSRP of NR Cell 2 is the 3dB larger than that of NR Cell 1, with a nominal Ês/Iot value of +5dB. The UE is expect to detect Cell 2 and perform handover to Cell 2. The effect of uncertainties on the test verdict needs to be checked. A decision can then be made on whether Test Tolerances need to be applied.

# 4. 	Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Table A.11.2.1.4.2-3 in TS 38.133 The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

b) Derived parameters

Several derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

At the start of T2, NR Cell 2 becomes detectable and the UE is expected to detect and handover to Cell 2

c) Uncertainties

The SS provides AWGN and 2 NR inter-frequency cells to the UE. We propose to control the following parameters:

- Frequency 1 AWGN Absolute power, Noc ±1.5 dB

- Frequency 2 AWGN Absolute power, Noc ±1.5 dB

- Ratio of NR cell 1 signal to noise, Ês1 / Noc ±0.3 dB

- Ratio of NR cell 2 signal to noise, Ês2 / Noc ±0.3 dB

This choice forms a minimum set, so the superposition principle can be applied if necessary.

##### 6.1B.1.2.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Where:

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = (1+L1) *Trs. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = (3+L1´) *Trs where L1 and L1´ are the number of SMTC occasions not available at the UE during the intra-frequency and inter-frequency detection period, respectively. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = (1+ L2) *Trs ms, where L2 is the number of SMTC occasions not available at the UE during the time tracking period.

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

TIU is the interruption uncertainty due to the random access procedure when sending PRACH to the new cell. TIU can be up to: (1+ L3)*TSSB,RO + 10 ms where TSSB,RO is the SSB to PRACH occasion association period as defined in Table 8.1-1 of TS 38.213 [3] and L3 is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure. L3 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [33]. When the UE is configured with both the UL BWP with PRACH occasion on the target cell and UL CCA failure detection/recovery, the interruption can be longer.

Trs is the SMTC periodicity of the target NR cell in a carrier frequency with CCA if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms.

NOTE 1:	The interruption time considering the potential extensions caused by L1, L1´,L2 , L3  and by the UL CCA failure detection/recovery mechanism is limited by the T304 timer. The UE behaviour at the T304 timer expiry is detailed in TS 38.331 [2].

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2A.5 for intra-frequency handover and Clause 9.3A.4 for inter-frequency handover to a carrier frequency with CCA.

In addition, the NR Cell 2 needs to be detectable. The condition that an inter-frequency cell is detectable is defined in TS 38.133 9.3.2.

### 9.3A.2	Requirements applicability

The requirements in clause 9.3A apply, provided:

-	The cell being identified or measured is detectable.

An inter-frequency CCA cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in clause 10.1.28,

-	SS-RSRQ related side conditions given in clause 10.1.30 and TBD for FR1 and FR2-2, respectively,

-	SS-SINR related side conditions given in clause 10.1.32 and TBD for FR1 and FR2-2, respectively,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.9.

##### 10.1.37.1.2	Relative Accuracy of SS-RSRP

The relative accuracy of SS-RSRP in inter frequency case is defined as the RSRP measured from one cell on a frequency compared to the RSRP measured from another cell on a different frequency, with at least one of the two frequencies being under CCA.

The accuracy requirements in Table 10.1.37.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3F of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.9 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1.37.1.2-1: SS-RSRP inter-frequency relative accuracy under CCA

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 6 | -6 | NR_CCA_FR1_I | -117 | -114 | N/A | -50 |
|  |  |  | NR_CCA_FR1_J | -116.5 | -113.5 |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

## B.2.10	Conditions for NR inter-frequency measurements under CCA

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.10-1 for NR cells under CCA.

Table B.2.10-1: Conditions for inter-frequency measurements under CCA

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_CCA_FR1_I | -121 | -118 | -4 |
|  | NR_CCA_FR1_J | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |

In summary the SSB Es/Iot of both NR Cell 1 and Cell 2 shall be above -3dB, the tightest requirement for Io of NR RF channel shall be between -113.5dBm/30kHz and -50dBm/BWchannel for band group NR_CCA_FR1_J, and the tightest requirement for SSB_RP shall be above -117.5dBm/30kHz for band group NR_CCA_FR1_J.

d) Controlled parameters critical to verdict

In many RRM test cases, and particularly inter-frequency multi-cell tests, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Source and Reference” column of section g) in the spreadsheet.

In general all the values are listed for each time interval. When a cell is not present during a time interval, the corresponding entries are greyed out.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x) and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute level Noc would cause 1dB error in the Io on RF Channel 1, so the sensitivity factor is 1.000 during each time interval. However, the same error of 1dB in the absolute level Noc would cause no change to NR Cell 1 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

The overall cell power Io is an exception, as the effect of Es/Noc uncertainty is diluted depending on the linear fraction of cell power making up the total. The sensitivity factor is made equal to the linear fraction of cell power on that frequency.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment and the UE RSRP relative measurement accuracy where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1 and T2.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

During T1 the UE is required to select NR Cell 1 as PCell. The uncertainties do not take NR Cell 1 Ês/Iot or SSB_RP power outside the allowed range, therefore no offsets are required.

During T2 the NR Cell 2 becomes detected. UE is expected to detect and handover to Cell 2. The uncertainties do not take NR Cell 2 Ês/Iot or SSB_RP or Io outside the allowed range, therefore no offsets are required.

The check that all other controlled parameters meet their required range is done in step g). In theory it is possible for steps e) to g) to be iterative, or possibly even steps c) to g) to be iterative, but this test case can be done without such iteration.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b). In general, all the values are listed for each time interval. Whenever a cell is not present during a time interval, the corresponding entries are greyed out (the formulae are left in place for use by future test cases).

g) Check controlled parameters Min/Max

Using a format like that in step d), the nominal value of each controlled parameter is recalculated.

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (RSRP difference, Es/Iot, SSB_RP and Io). It is not necessary to calculate all parameters during each time interval, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

With the uncertainty values and Test Tolerances proposed, all the requirements are met.