3GPP TSG-RAN5 Meeting #105	R5-246912

# Orlando, United States, Nov 18-22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3070 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test cases 6.3H.3.2 and 6.3H.3.3 for inter-band CA with UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case 6.3H.3.2 Transmit OFF time mask and 6.3H.3.3 Transmit ON/OFF time mask for inter-band CA with UL MIMO are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the new test cases 6.3H.3.2 and 6.3H.3.3 for inter-band CA with UL MIMO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mentioned test cases will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3H.3.2(new), 6.3H.3.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3H.3.1	Minimum output power for inter-band UL CA with UL MIMO

6.3H.3.1.1	Test purpose

To verify the UE’s ability to transmit with a broadband output power for inter-band UL CA with UL MIMO below the value specified in the test requirement when the power is set to a minimum value.

6.3H.3.1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with UL MIMO.

6.3H.3.1.3	Minimum conformance requirements

For inter-band UL CA with UL MIMO in one of the two frequency bands, the minimum output power is defined per carrier. The requirement is specified in clause 6.3.1 for the carrier without UL MIMO and specified in clause 6.3D.1 for the carrier configured with UL MIMO.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3H.3.1.

6.3H.3.1.4	Test description

This test is covered by clauses 6.3.1 Minimum output power and 6.3D.1 Minimum output power for UL MIMO.

6.3H.3.1.5	Test requirements

Same test requirement as 6.3D.1 for the band supporting UL MIMO and 6.3.1 for other band.

6.3H.3.2	Transmit OFF power for inter-band UL CA with UL MIMO

6.3H.3.2.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

An excess Transmit OFF power potentially increases the Rise Over Thermal (RoT) and therefore reduces the cell coverage area for other UEs.

6.3H.3.2.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with UL MIMO.

6.3H.3.2.3	Minimum conformance requirements

For inter-band UL CA with UL MIMO in one of the two frequency bands, the transmit OFF power specified in clause 6.3.2 is applicable for the carrier without UL MIMO and the transmit OFF power specified in clause 6.3D.2 is applicable for the carrier configured with UL MIMO when the transmitter is OFF on all component carriers. The transmitter is considered to be OFF when the UE is not allowed to transmit on any of its ports.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3H.3.2.

6.3H.3.2.4	Test description

This test is covered by clause 6.3H.3.3 Transmit ON/OFF time mask for inter-band UL CA with UL MIMO.

6.3H.3.2.5	Test requirements

Same test requirement as 6.3D.2 for the band supporting UL MIMO and 6.3.2 for the other band.

6.3H.3.3	Transmit ON/OFF time mask for inter-band UL CA with UL MIMO

6.3H.3.3.1	Test purpose

To verify that the general ON/OFF time mask meets the requirements given in 6.3H.3.3.5.

The transmit power time mask for transmit ON/OFF defines the transient period(s) allowed between transmit OFF power as defined in sub-clause 6.3.2 and transmit ON power symbols (transmit ON/OFF)

Transmission of the wrong power increases interference to other channels, or increases transmission errors in the uplink channel.

6.3H.3.3.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with UL MIMO.

6.3H.3.3.3	Minimum conformance requirements

For inter-band UL CA with UL MIMO in one of the two frequency bands, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for the component carrier without UL MIMO during the ON power period and the transient periods, the ON/OFF time mask specified in clause 6.3D.3 is applicable for the component carrier configured with UL MIMO. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3H.3.3.

6.3H.3.3.4	Test description

This test is covered by clauses 6.3.3.2 General ON/OFF time mask and 6.3D.3 General ON/OFF time mask for UL MIMO.

6.3H.3.3.5	Test requirements

Same test requirement as 6.3D.3 for the band supporting UL MIMO and 6.3.3.2 for the other band.

<End of Changes>