3GPP TSG-RAN5 Meeting #105	R5-247001

# Orlando, USA, 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3382 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PTRS configuration for FR2 PDSCH testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz, Keysight, Anritsu, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Table 7.2-1 in TS 38.521-4 mandates for the PTRS a frequency density of 2 and a time density of 1. With the current configuration in table 5.4.2.0-24 in TS 38.508-1 this is not always guaranteed, since with both IE not present at the moment, the presence of PTRS is depending on the used MCS according to 38.214. Thus frequency density and time density need to be set according to the values in table 7.2-1 in TS 38.521-4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the IE frequencyDensity and timeDensity to match the core requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect configuration remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged sections omitted >

PDSCH DMRS Configuration

DMRS-DownlinkConfig

Table 5.4.2.0-24: DMRS-DownlinkConfig

| Derivation Path: Table 4.6.3-50 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos1 |  |  |
| phaseTrackingRS | Not present |  | DEMOD_FR1 |
| phaseTrackingRS SEQUENCE { |  |  | DEMOD_FR2 |
| frequencyDensity | frequencyDensity |  |  |
| timeDensity | timeDensity |  |  |
| epre-Ratio | 0 |  |  |
| resourceElementOffset | Offset10 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.4.2.0-24a: frequencyDensity

| Derivation Path: Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| frequencyDensity ::= SEQUENCE { |  |  |  |
| NRB0 | 1 |  |  |
| NRB1 | 276 |  |  |
| } |  |  |  |

Table 5.4.2.0-24b: timeDensity

| Derivation Path: Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| timeDensity ::= SEQUENCE { |  |  |  |
| ptrs-MCS1 | 0 |  |  |
| ptrs-MCS2 | 0 |  |  |
| ptrs-MCS3 | 0 |  |  |
| } |  |  |  |

#### < End of changes >