3GPP TSG-RAN5 Meeting #105	R5-247069

# Orlando, USA, 18 Nov – 22 Nov, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3097 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 6.5E.3.2 NR V2X Spurious emissions for UE co-existence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NR V2X Spurious emissions for UE co-existence test cases are not completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Completing the test cases. The test configuration for non-concurrent reuses that of general spurious emission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5E.3.2.0 (new), 6.5E.3.2.1 (new), 6.5E.3.2.1D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0955 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5E.3.2	Spurious emissions for UE co-existence for V2X

6.5E.3.2.0	Minimum conformance requirements

6.5E.3.2.0.1	Spurious emissions for UE co-existence

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the requirements in clause 6.5.3.2.3 shall apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with the SL MIMO configurations described in clause 6.2D.1.

6.5E.3.2.0.2	Spurious emissions for UE co-existence for V2X con-current operation

This clause specifies the additional requirements for inter-band con-current V2X operation with the single CC uplink assigned to two NR bands for coexistence with protected bands for the specified simultaneous transmission of the inter-band con-current V2X configurations in Table 6.5E.3.3-1. The intersection of the requirements for the individual bands specified in clause 6.5.3.2 shall also apply for the specified simultaneous transmission of the inter-band con-current V2X. Intersection of a requirement means that both UL or sidelink transmission constituent bands have the same protected band requirement specified and if one or both protected bands have note(s) associated those note(s) also apply.

For the inter-band con-current NR V2X operation, the UE-coexistence requirements in Table 6.5E.3.3.1-1 apply for the corresponding inter-band con-current operation with transmission assigned to both uplink in licensed band and sidelink in Band n47.

NOTE:	For inter-band con-current V2X operation with uplink assigned to NR band and slidelink transmission assigned to NR V2X operating bands, the requirements in Table 6.5E.3.3-1 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5E.3.3-1 and in clause 6.5.3.2 would be considered to be verified by the measurements verifying the one uplink inter-band con-current UE to UE co-existence requirements.

Table 6.5E.3.3-1: Requirements for inter-band con-current V2X operation

| V2X | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| con-current operating band cofiguration | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_n71A-n47A | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Applicable when NS_33 is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |  |  |

6.5E.3.2.1	Spurious emissions for UE co-existence for V2X / non-concurrent operation

6.5E.3.2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

6.5E.3.2.1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication with non-concurrent operation.

6.5E.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.3.2.0.

6.5E.3.2.1.4	Test description

6.5E.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.1-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.5E.3.2.1.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annexes A.7.5 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.5E.3.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  |  | Modulation |  | PSCCH and PSSCH RB allocation (Note 1) |
| 1 | Default | QPSK |  | Outer_Full |
| 2 | Default | QPSK |  | Outer_10RB_Left |
| 3 | Default | QPSK |  | Outer_10RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.1 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.5E.3.2.1.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.5E.3.2.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Transmit Mode according to TS 38.508-1 [5] clause 4.5.

6.5E.3.2.1.4.2	Test procedure

1.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

2.	Measure the mean power of the UE in the channel bandwidth according to the test configuration from Table 6.5E.3.2.1.4.1-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5.3.2.3-2. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5E.3.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10.

6.5E.3.2.1.5	Test requirement

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Tables 6.5.3.1.3-1 from the edge of the channel bandwidth. The spurious emission limits in Tables 6.5.3.2.3-2 apply for all transmitter band configurations (NRB) and channel bandwidths.

The measured average power of spurious emission, derived in step 2, shall not exceed the described value in Table 6.5.3.2.3-2.

6.5E.3.2.1D	Spurious emissions for UE co-existence for V2X / non-concurrent operation / SL-MIMO

6.5E.3.2.1D.1	Test purpose

Same test purpose as in 6.5E.3.2.1.1.

6.5E.3.2.1D.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication and SL-MIMO.

6.5E.3.2.1D.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.3.1.0.

6.5E.3.2.1D.4	Test description

Same test description as in 6.5E.3.2.1.4  with following exceptions:

For initial conditions:

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.2 for TE diagram and section A.3.2.7 for UE diagram.

6.	Ensure the UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) and Transmit Mode with SL-MIMO. The UE is synchronized to GNSS,

For test procedures:

Subtest 1 are replaced by:

1.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR with 2-layer MIMO codebook TPMI 0. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

2.	Measure the mean power of the UE at each antenna connector in the channel bandwidth according to the test configuration from Table 6.5E.3.2.1.4.1-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5E.3.1.1D.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5E.3.2.1D.4.1	Void

6.5E.3.2.1D.4.2	Void

6.5E.3.2.1D.4.3	Void

6.5E.3.2.1D.5	Test requirement

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Tables 6.5.3.1.3-1 from the edge of the channel bandwidth. The spurious emission limits in Tables 6.5.3.2.3-2 apply for all transmitter band configurations (NRB) and channel bandwidths.

The measured average power of spurious emission at each antenna connector, derived in step 2, shall not exceed the described value in Table 6.5.3.2.3-2.

6.5E.3.2.2	Spurious emissions for UE co-existence for V2X / concurrent operation

6.5E.3.2.2.1	Test purpose

Same test purpose as in 6.5E.3.2.1.1 for concurrent operation.

6.5E.3.2.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

6.5E.3.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5E.3.2.0.

6.5E.3.2.2.4	Test description

6.5E.3.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.2-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.5E.3.2.2.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annexes A.7.5. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5E.3.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | For test frequencies refer to “Freq” columns for NR Uu and NR V2X respectively. |  |  |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest for NR Uu and V2X carrier |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest for NR Uu and V2X carrier |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  | NR Uu Configuration |  |  |  | NR V2X Configuration to Transmit |  |  |
|  | Freq | Downlink | Uplink Configuration |  | Freq | Modulation | PSCCH |
|  |  | Configuration | Modulation | RB allocation (NOTE 1) |  |  | and PSSCH RB allocation (Note 2) |
| Test Settings for V2X_n71A-n47A Configuration |  |  |  |  |  |  |  |
| No test points for V2X_n71A-n47A |  |  |  |  |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.3 for TE diagram and clause A.3.2.7 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The V2X Reference Measurement Channel and NR UL Reference Measurement Channel are set according to clause 6.2E.2.2.4.1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameter Connectivity NR, Connected without release on, Test Mode On and Sidelink On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5E.3.2.2.4.3.

6.5E.3.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to clause 6.5E.3.2.2.4.1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	SS sends sidelink scheduling information for each SL-HARQ process via PDCCH DCI format 3_0 for C_RNTI to schedule the Sidelink RMC according to clause 6.5E.3.2.2.4.1. UE is configured to transmit at Pcmax on the sidelink carrier. Since the UE has no payload and no loopback data to send the UE sends MAC padding bits on the Sidelink RMC.

4.	Measure the mean power of the UE in the channel bandwidth according to the test configuration from Table 6.5E.3.2.2.4.1-1. Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5E.3.2.2.5-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5E.3.2.2.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5E.3.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6.

6.5E.3.2.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence for V2X concurrent operation CA are release specific and can be found in Table 6.5E.3.2.2.5-1. If the UE support a band combination from a later release, the requirements for this band combination are taken from the table of earliest release where requirements for this band combination are defined.

Table 6.5E.3.2.2.5-1: Requirements for V2X con-current operation according to UE NR release

| V2X | Spurious emission |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| con-current operating band cofiguration | Protected band | Frequency range (MHz) | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_n71A-n47A | N/A |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Applicable when NS_33 is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |

< End of Changes >