3GPP TSG-RAN5 Meeting #105	R5-247073

# Orlando, USA, 18 Nov – 22 Nov, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3099 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing n38 from V2X Tx test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CAICT, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Band n38 for V2X operation is still pending in PRD21. The test related to this band needs to be removed from the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing band n38 related contents from V2X test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases still include the pending band. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2E.1, 5.3E.1, 6.2E.1.0, 6.2E.2.1, 6.2E.2.1D, 6.2E.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

5.2E.1	V2X operating bands

NR V2X is designed to operate in the operating bands in FR1 defined in Table 5.2E.1-1.

Table 5.2E.1-1: V2X operating bands in FR1

| V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n47 | 5855 MHz | - | 5925 MHz | 5855 MHz | - | 5925 MHz | HD | PC5 |
| NOTE 1: FFS. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped >

5.3E.1	General

NR V2X operation channel bandwidths for each operating band are specified in Table 5.3E.1-1 in subclause 5.3E.1. The same (symmetrical) channel bandwidth is specified for both the transmission and reception path. The maximum channel bandwidth for SL operation in licensed band is 40MHz.

Table 5.3E.1-1: NR V2X operation channel bandwidths for each operating band

| NR band / SCS / UE Channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS (kHz) | 5 | 10 | 20 | 30 | 40 |
| n47 | 15 |  | 10 | 20 | 30 | 40 |
|  | 30 |  | 10 | 20 | 30 | 40 |
|  | 60 |  | 10 | 20 | 30 | 40 |

<Unchanged Text Skipped >

6.2E.1.0	Minimum conformance requirements

6.2E.1.0.1	General

When NR V2X UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the allowed NR V2X UE maximum output power is specified in Table  6.2E.1.0.1-0.

Table 6.2E.1.0.1-0: NR V2X UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n47 |  |  | 26 | +2/-3 | 23 | ±2 |

When a UE is configured for NR V2X sidelink transmissions in NR Band n47, the V2X UE shall meet the following additional requirements for transmission within the frequency ranges 5855-5925 MHz:

-	The maximum mean power spectral density shall be restricted to 23 dBm/MHz EIRP when the network signalling value NS_33 is indicated.

where the network signalling values are specified in clause 6.2E.3.0.

NOTE:	The PSD limit in EIRP shall be converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11].

For NR V2X UE supporting SL MIMO or Tx diversity, the maximum output power requirements in Table 6.2E.1.0.1-1 is defined as the sum of the maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). For UE supporting SL MIMO, the requirements shall be met with the SL MIMO configurations specified in Table 6.2D.1.3-2.

Table 6.2E.1.0.1-1: NR V2X UE Power Class for SL-MIMO

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n47 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |

If the UE transmits on one antenna connector at a time, the requirements in Table 6.2E.1.0.1-0 shall apply to the active antenna connector.

<Unchanged Text Skipped >

6.2E.2.1	UE maximum output power reduction for V2X / non-concurrent operation

6.2E.2.1.1	Test purpose

Same test purpose as in 6.2.2.1.

6.2E.2.1.2	Test applicability

This test case applies to all types of UE release 16 and forward that support NR V2X sidelink communication.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5E.2.4.1 is executed.

6.2E.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.2.0.

6.2E.2.1.4	Test description

6.2E.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.1-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.2.1.4.1-1 to 6.2E.2.1.4.1-3. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.7.5 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.2E.2.1.4.1-1: Test Configuration Table for contiguous PSCCH and PSSCH allocation for PC3

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  |  | Modulation |  | PSCCH and PSSCH RB allocation (Note 1) |
| 1 | Default | 16QAM |  | Outer_Full |
| 2 | Default | 16QAM |  | Inner_Full |
| 3 | Default | 64QAM |  | Outer_Full |
| 4 | Default | 256QAM |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

Table 6.2E.2.1.4.1-1a: Test Configuration Table for contiguous PSCCH and PSSCH allocation for PC2

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  |  | Modulation |  | PSCCH and PSSCH RB allocation (Note 1) |
| 1 | Default | 16QAM |  | Outer_Full |
| 2 | Default | 16QAM |  | Inner_Full |
| 3 | Default | 64QAM |  | Outer_Full |
| 4 | Default | 64QAM |  | Inner_Full |
| 5 | Default | 256QAM |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

Table 6.2E.2.1.4.1-2: Test Configuration Table for PSFCH for PC2 and PC3

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |
|  |  | PSFCH RB allocation (Note 1) |  |
| 1 | Low range | PSFCH_1RB_Left |  |
| 2 | High range | PSFCH_1RB_Right |  |
| 3 | Low range | PSFCH_2RB_Left |  |
| 4 | High range | PSFCH_2RB_Right |  |
| 5 | Default | PSFCH_Max_Gap |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-2. |  |  |  |

Table 6.2E.2.1.4.1-3: Test Configuration Table for S-SSB for PC2 and PC3

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, High range |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |
|  |  | S-SSB RB allocation (Note 1) |  |
| 1 | Low range | S-SSB_Low |  |
| 2 | High range | S-SSB_High |  |
| 3 | Default | S-SSB_Mid |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-3. |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.1 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.2E.2.1.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.2E.2.1.4.1-1 to Table 6.2E.2.1.4.1-3.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.2E.2.1.4.2	Test procedure

Subtest 1: PSCCH/PSSCH

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Test Loop Function On with UE test loop mode E closed for Transmit Mode according to TS 38.508-1 [5] clause 4.5.

2.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

3.	Measure the mean power of the UE in the channel bandwidth according to the test configuration from Table 6.2E.2.1.4.1-1. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

Subtest 2: PSFCH

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On, Cast type Unicast, Test Loop Function On with UE test loop mode E closed for Receive Mode according to TS 38.508-1 [5] clause 4.5.

2.	The UE starts to perform the NR sidelink reception according to SL-PreconfigurationNR.

3.	The UE’s PSFCH transmission occasion is on slot n according to Table 6.2E.2.1.4.1-2. SS transmits PSSCH on combination of slot and subchannel as below:

a) Test ID 1: slot n-6, Lowest sub-channel

b) Test ID 2: slot n-3, Highest sub-channel

c) Test ID 3: slot n-6 and n-5, Lowest sub-channel

d) Test ID 4: slot n-4 and n-3, Highest sub-channel

e) Test ID 5: slot n-6, Highest sub-channel and slot n-3, Highest sub-channel

4.	Measure the mean power of the UE on slot n in the channel bandwidth according to the test configuration from Table 6.2E.2.1.4.1-2. The period of measurement is one active sub-frame (1ms) excluding guard symbols.

Subtest 3: S-SSB

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On according to TS 38.508-1 [5] clause 4.5. The UE is synchronized to GNSS,

2.	The UE transmits PSBCH according SL-PreconfigurationNR.

3.	Measure the mean power of the S-SSB in the channel bandwidth according to the test configuration from Table 6.2E.2.1.4.1-3. The period of measurement is one active sub-frame (1ms) excluding guard symbols.

6.2E.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10 with the following exceptions.

Table 6.2E.2.1.4.3-1: SL-ResourcePool for PSCCH/PSSCH Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| sl-FreqResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |

Table 6.2E.2.1.4.3-2: SL-ResourcePool for PSFCH Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSFCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-PSFCH-Period-r16 | sl4 |  |  |
| sl-PSFCH-RB-Set-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumMuxCS-Pair-r16 | n1 |  |  |
| sl-MinTimeGapPSFCH-r16 | sl3 |  |  |
| sl-PSFCH-HopID-r16 | Not present | Default frequency hopping ID 0 is used |  |
| sl-PSFCH-CandidateResourceType-r16 | startSubCH |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |

Table 6.2E.2.1.4.3-3: SL-FreqConfigCommon for S-SSB Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-FreqConfigCommon-r16 ::= SEQUENCE { |  |  |  |
| sl-AbsoluteFrequencySSB-r16 | According to section 4.3.1.8 of TS 38.508-1 [5] |  |  |
| } |  |  |  |

6.2E.2.1.5	Test requirement

The maximum output power, derived in step 3 of Subtest 1, step 4 of Subtest 2 and step 3 of Subtest 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2E.2.1.5-1 to Table 6.2E.2.1.5-6.

Table 6.2E.2.1.5-1: PC3 UE MPR test requirement for contiguous PSCCH/PSSCH (Bands n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 4.5 | 0 | 18.5 | 4 | 25.0 + TT | 14.5-TT |
| 2 | 23 | 2.5 | 0 | 20.5 | 2.5 | 25.0 + TT | 18-TT |
| 3 | 23 | 4.5 | 0 | 18.5 | 4 | 25.0 + TT | 14.5-TT |
| 4 | 23 | 7.0 | 0 | 16 | 5 | 25.0 + TT | 11-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1.5-4. |  |  |  |  |  |  |  |

Table 6.2E.2.1.5-2: PC3 UE MPR test requirement for PSFCH (Bands n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3.5 | 0 | 19.5 | 3.5 | 25.0 + TT | 16-TT |
| 2 | 23 | 7.5 | 0 | 15.5 | 5 | 25.0 + TT | 10.5-TT |
| 3 | 23 | 7.5 | 0 | 15.5 | 5 | 25.0 + TT | 10.5-TT |
| 4 | 23 | 12 | 0 | 11 | 6 | 25.0 + TT | 5-TT |
| 5 | 23 | 12 | 0 | 11 | 6 | 25.0 + TT | 5-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1.5-4. |  |  |  |  |  |  |  |

Table 6.2E.2.1.5-3: PC2 and PC3 UE MPR test requirement for S-SSB (Bands n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 6 | 0 | 17 | 5 | 25.0 + TT | 12-TT |
| 2 | 23 | 6 | 0 | 17 | 5 | 25.0 + TT | 12-TT |
| 3 | 23 | 2.5 | 0 | 20.5 | 2.5 | 25.0 + TT | 18-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1.5-4. |  |  |  |  |  |  |  |

Table 6.2E.2.1.5-4: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7dB | 1.0dB |

Table 6.2E.2.1.5-5: PC2 UE MPR test requirement for contiguous PSCCH/PSSCH (Bands n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 5.5 | 0 | 20.5 | 2.5 | 28.0 + TT | 18-TT |
| 2 | 26 | 2.5 | 0 | 23.5 | 2.0 | 28.0 + TT | 21.5-TT |
| 3 | 26 | 6 | 0 | 20 | 2.5 | 28.0 + TT | 17.5-TT |
| 4 | 26 | 4.5 | 0 | 21.5 | 2.0 | 28.0 + TT | 19.5-TT |
| 5 | 26 | 7 | 0 | 19 | 3.5 | 28.0 + TT | 15.5-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1.5-4. |  |  |  |  |  |  |  |

Table 6.2E.2.1.5-6: PC2 UE MPR test requirement for PSFCH (Bands n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 4.5 | 0 | 21.5 | 2.0 | 28.0 + TT | 19.5-TT |
| 2 | 26 | 8.5 | 0 | 17.5 | 5.0 | 28.0 + TT | 12.5-TT |
| 3 | 26 | 8.5 | 0 | 17.5 | 5.0 | 28.0 + TT | 12.5-TT |
| 4 | 26 | 14 | 0 | 12 | 6.0 | 28.0 + TT | 6.0-TT |
| 5 | 26 | 14 | 0 | 12 | 6.0 | 28.0 + TT | 6.0-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1.5-4. |  |  |  |  |  |  |  |

6.2E.2.1D	UE maximum output power reduction for V2X / non-concurrent operation / SL-MIMO

6.2E.2.1D.1	Test purpose

Same test purpose as in 6.2E.2.1.

6.2E.2.1D.2	Test applicability

This test case applies to all types of UE release 16 and forward that support NR V2X sidelink communication and SL-MIMO.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5E.2.4.1D is executed.

6.2E.2.1D.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.2.0.

6.2E.2.1D.4	Test description

Same test description as in 6.2E.2.1.4 for PSSCH/PSCCH subtest with following exceptions:

For initial conditions:

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.2 for TE diagram and section A.3.2.7 for UE diagram.

For test procedures:

Subtest 1 are replaced by:

1.	Ensure the UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) and Transmit Mode with SL-MIMO. The UE is synchronized to GNSS,

2.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR with 2-layer MIMO codebook TPMI 0. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

3.	Measure the sum of mean power of the UE at each transmit antenna connector in the channel bandwidth according to the test configuration from Table 6.2E.2.1D.4.1-1. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

6.2E.2.1D.4.1	Void

6.2E.2.1D.4.2	Void

6.2E.2.1D.4.3	Void

6.2E.2.1D.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2E.2.1D.5-1.

Table 6.2E.2.1D.5-1: PC3 UE MPR test requirement for contiguous PSCCH/PSSCH (Bands n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 4.5 | 0 | 18.5 | 5 | 3 | 25.0 + TT | 13.5 - TT |
| 2 | 23 | 2.5 | 0 | 20.5 | 5 | 3 | 25.0 + TT | 15.5 - TT |
| 3 | 23 | 4.5 | 0 | 18.5 | 5 | 3 | 25.0 + TT | 13.5 - TT |
| 4 | 23 | 7.0 | 0 | 16 | 5 | 3 | 25.0 + TT | 11.0 - TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1D.5-2. |  |  |  |  |  |  |  |  |

Table 6.2E.2.1D.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7dB | 1.0dB |

Table 6.2E.2.1D.5-3: PC2 UE MPR test requirement for contiguous PSCCH/PSSCH (Bands n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 6.0 | 0 | 20.0 | 5 | 3 | 28.0 + TT | 15.0 - TT |
| 2 | 26 | 3.0 | 0 | 23.0 | 3 | 3 | 28.0 + TT | 20.0 - TT |
| 3 | 26 | 7.0 | 0 | 19.0 | 5 | 3 | 28.0 + TT | 14.0 - TT |
| 4 | 26 | 5.5 | 0 | 20.5 | 5 | 3 | 28.0 + TT | 15.5 - TT |
| 5 | 26 | 9.0 | 0 | 17.0 | 5 | 3 | 28.0 + TT | 12.0 - TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1D.5-2. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped >

6.2E.3	UE additional maximum output power reduction for V2X

6.2E.3.0	Minimum conformance requirements

6.2E.3.0.1	General

For the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in clause 6.2E.2.0 and A-MPR requirements specified in current clause.

Additional emission requirements can be indicated by the network or pre-configured radio parameters. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field [additionalSpectrumEmission]. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR V2X frequency band number of the applicable operating band, the IE field [freqBandIndicatorNR] and an associated value of [additionalSpectrumEmission] in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2E.3.0.1-1: Additional Maximum Power Reduction (A-MPR) for PC3 NR V2X

| Network Signalling value | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2E.1-1 | 10, 20, 30, 40 | Table 5.3.2-1 | N/A |
| NS_33 | 6.5E.2.3.0.1 (A-SEM) 6.5E.3.3.0 (A-SE) | n47 | 10 | Clause 6.2E.3.0.2 |  |
| NS_52 | 6.5E.2.3.0.2 (A-SEM) | n47 | 40 | Clause 6.2E.3.0.3 |  |

Table 6.2E.3.0.1-2: Mapping of network signalling label

| NR V2X operating bands | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n47 | NS_01 | NS_33 | NS_52 |  |  |  |  |  |
| NOTE: [additionalSpectrumEmission] corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

For UE with two transmit antenna connectors, the A-MPR values specified in clause 6.2E.3.0.2 and 6.2E.3.0.3 shall apply to the maximum output power specified in Table 6.2E.1.0.1-1. The requirements shall be met with the SL MIMO configurations specified in Table 6.2D.1-2. For UE supporting SL MIMO, the maximum output power is defined as the sum of the maximum output power from each UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2E.4.0 apply.

< End of Changes >