3GPP TSG-RAN WG5 Meeting #105	R5-247083

Orlando, USA, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	On Testability and Interoperability Aspects for AI/ML Based Beam Management

Agenda item:	4.2.3

Document for:	Discussion

Introduction

In RAN5#104, a paper on how to handle RAN5 Artificial Intelligence (AI) work was discussed[1]. The document was noted with an action to RAN5 leadership to create a dedicated RAN5 AIML reflector.

Later, on 2024-10-10, RAN5 leadership sent the following guidance for RAN5#105 meeting:

AI/ML agenda item - 4.2.3 AI/ML Testability & Interoperability included in RAN5#105 agenda. - Contributions (open to all) under this sub-AI to be handled during the midweek joint session … RP-242399 AI/ML for NR Air Interface WID - Current RAN5 responsibility - RAN5 aspects related to testability and interoperability to be addressed on a request basis. - Test setup and methodology for the beam management to be studied under this WI (update to the WID in RP#105) - Possible area of RAN5 immediate contribution - Rapporteur proposed addition of RAN5 as secondary responsibility but accepted my recommendation to allow RAN5 make an assessment whether RAN5 is in a position now to make technical contributions to RAN4 specs covering any of the objectives in the WID before being added under secondary responsibility in the WID. Once RAN5 makes an affirmative determination on this, then a request can be made to the rapporteur about secondary responsibility. With secondary responsibility, dedicated agenda item will be included in RAN5 meeting agenda to handle technical contributions for endorsement and once endorsed to be submitted to RAN4 (like how we handled NR TRP/TRS contributions)

In this paper, we provide our views on issues/enhancements related to testability and interoperability aspects, as listed below:

- Testing setup and Channel models

- New UE Special Conformance Test function for AIML use cases

The discussions on AI/ML enabled beam management (BM) use case was continued in RAN4#112bis as defined in the scope of Rel-19 WI. Agreements achieved during this meeting are summarized in the WF [2] and the ad hoc minutes [3].

Discussion

Testing Setup and Channel Models

The discussion on the feasibility of testability aspects of AI/ML-enabled BM use case and the corresponding testing setup continued in the last RAN4#112bis meeting and the following agreements were made [2].

Agreement:

- For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes.

  - Set A and B should be generated in the chamber for ground-truth extraction.

- For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes.

  - Set A and B should be generated in the chamber for ground-truth extraction.

    - For single AoA, FSS how to derive Set A

Also, the WID for “NR_AIML_Air” work item was revised in RAN#105 which includes the following objective related to the testing of AI/ML-enabled BM use case[5].

Provide specification support for the following aspects: … - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. …

This revision of NR_AIML_Air WID justifies the need for defining the testing setup and procedure for the AI/ML-enabled BM use case. Furthermore, the agreements made in RAN4#112bis show that the channel model for the conformance testing is to be decided from the TDL and CDL/simplified CDL channel models. Moreover, there remain some open issues which need further investigation: (1) number of Tx beams in Set A and Set B during the conformance test, (2) details of the AoAs and AoDs to be supported during the conformance test (i.e., the number of different AoAs and their values), (3) whether to use the UE rotation or not during the conformance test of the AI/ML-enabled BM use-case.

In this section, we provide our further views on these open aspects.

Channel Models

Regarding the open issue of which channel model to be used during the conformance testing, companies in RAN4#112 had provided their views where some companies were advocating for CDL channel model whereas others were in the opinion of using simplified channel models like TDL or AWGN. There are pros and cons of each of the CDL and TDL channel models which are well known and need to be considered before deciding the channel model for the conformance testing of AI/ML-enabled BM use case.

If RAN4 decides to use CDL channel model during the conformance testing, it will bring a more realistic testing approach as compared to TDL or AWGN channel models as the signal reception from different and/or multiple AoAs is possible with CDL channel model. However, the use of CDL channel model will also increase the cost of the test systems due to the need of a large number of test probes to emulate each cluster of the CDL channel model.

CDL channel models present a more realistic way for testing AI/ML BM use cases due to the more realistic spatial behaviour.

- The AI/ML-enabled BM use case targets FR2 which indeed is a sparse channel resulting in a limited number of dominant clusters in FR2. Hence, a simplified CDL model with reduced number of clusters will be a good approach for the conformance testing of this use case. How various CDL channel model type spans in azimuth and zenith domains were described in the discussion paper [4].

As the FR2 channel is spatially sparse, a simplified CDL model with reduced number of clusters will be a good approach for the conformance testing of the AI/ML-enabled BM use case.

- Proposal 1: RAN5 to suggest RAN4 to decide the number of clusters and their angular location in the simplified CDL model considering the following aspects:

- A practical number of test probes to be made available in the OTA test chamber for AI/ML-enabled BM UE conformance testing,

- The angular location of the selected clusters should span the maximum part of the azimuth and zenith domains.

Testing Setup

The topic of testing setup for AI/ML-enabled BM use case was also discussed in the latest RAN4#112bis meeting where the companies had ambiguity regarding the criteria to select the Set A and Set B beams as well as the need to emulate and test the Set A and Set B beams at the same time. Ideally, all the beams in Set B should be generated without UE rotation as a similar scenario will be observed in the real deployments where the gNodeB will generate the Set B beams irrespective of the UE orientation. On the other hand, the emulation of Set A beams inside the OTA test chamber during/before/after the conformance testing may be needed to extract the ground truth.

Therefore, how many beams should be generated simultaneously in each test instance/interval/iteration in the TE chamber is a more relevant topic to the design of the test procedure and the ability of the TE.

It can be easily observed from [5] and [6] that the number of Set A/B beams to be emulated during the test is typically larger than the number of beams that can be generated by the TE. Hence, it is worth discussing whether all Set A/B beams can be emulated in the test chamber or some of those that are “most relevant” Tx beams can be emulated. Another point to discuss is how to define the “most relevant” Tx beams.

It is not very clear whether all Set A/B beams can be emulated inside the test chamber or only some of those beams that are “most relevant” Tx beams can be emulated.

- Proposal 2: RAN5 to discuss on below open issues regarding test setup:

- Does UE rotation required during the test as it shall impact on maximum number of set A TX beams that can be generated inside the chamber?

- Based on cost, chamber size, etc., what can be the “most relevant” Set A Tx beams that RAN WG5 can agree upon considering closer to field scenario?

New UE Conformance Test Function for AI/ML Use Cases

To support the conformance testing of 5G systems (5GS), UE special conformance test functions are required. They form a part of the core requirements and thus have a direct impact on the design of the UE. The UE special conformance test functions defined in TS 38.509 vary depending on the conformance testing functionality they are designed to support.

Observation 4: The existing test function isn’t sufficient to handle the Rel-19 AI/ML use cases and modes.

Proposal 3: RAN5 to further discuss a new UE special conformance test function to cater to AIML-enabled functionalities.

Conclusion

In this paper, we share our views on potential open RAN4 issues related to the testability and interoperability of AI/ML-enabled BM-Case1 and BM-Case2. Specifically, we cover the following aspects:

- Testing setup and Channel models

- New UE Conformance Test Function for AI/ML use cases

The following Observations and Proposals were made:

- Observation 1: CDL channel models present a more realistic way for testing AI/ML BM use cases due to the more realistic spatial behaviour.

Observation 2: As the FR2 channel is spatially sparse, a simplified CDL model with reduced number of clusters will be a good approach for the conformance testing of the AI/ML-enabled BM use case.

- Proposal 1: RAN5 to suggest RAN4 to decide the number of clusters and their angular location in the simplified CDL model considering the following aspects:

- A practical number of test probes to be made available in the OTA test chamber for AI/ML-enabled BM UE conformance testing,

- The angular location of the selected clusters should span the maximum part of the azimuth and zenith domains.

- Observation 3: It is not very clear whether all Set A/B beams can be emulated inside the test chamber or only some of those beams that are “most relevant” Tx beams can be emulated.

Proposal 2: RAN5 to discuss on below open issues regarding test setup:

- Is UE rotation during the test required or not as it shall impact on maximum number of set A TX beams that can be generated inside the chamber?

- Based on cost, chamber size, etc., what can be the “most relevant” Set A Tx beams that RAN WG5 can agree upon considering closer to field scenario?

Observation 4: The existing test function isn’t sufficient to handle the Rel-19 AI/ML use cases and modes.

Proposal 3: RAN5 to further discuss a new UE special conformance test function to cater to AIML-enabled functionalities.

- References

- R5-245606, Disc on handling of RAN5 AI, CMCC, RAN5#104, Maastricht, Netherlands, 19th – 23rd August 2024.

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm, RAN4#112bis, Hefei, Anhui, China, 14th – 18th October 2024.

- R4-2417188, AI/ML ad-hoc meeting minutes, Qualcomm Incorporated, RAN4#112bis, Hefei, Anhui, China, 14th – 18th October 2024.

- R4-2416440, On Testability and Interoperability aspects for AI/ML Based Beam Management, Nokia, Hefei, China, Oct 14 – Oct 18, 2024.

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#105, Melbourne, Australia, September 9-12, 2024.

- R4-2401570, WF on AI/ML, Qualcomm, RAN4#111, Fukuoka city, Japan, 20th – 24th May 2024.

- R4-2411254, OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases, Keysight, RAN4#112, Maastricht, Netherlands, 19th – 23rd August 2024.