3GPP TSG-RAN5 Meeting #105	R5-247116

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5383 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial updates on 36523 draft IETF reference - 15.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some ietf drafts existing in the Conformance Requirements clauses and the references to IETF drafts are already updated to the RFCs in TS 24.303-8a0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the core Spec references to align with TS 24.303-8a0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This may lead to misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

15.2	Discovery of the Home Agent via DHCP

15.2.1	Test Purpose (TP)

(1)

with { UE has acquired an IP address and UE is configured with the HA-APN Network Identifier }

ensure that {

when { UE is configured to discover IP address of Home Agent via DHCP }

then { UE transmits a DHCP Information-Request with Home Network Identifier Option containing the FQDN of the Home Agent}

}

15.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.303, clauses 5.1.2.1.1 and 5.1.2.1.5.

[TS 24.303, clause 5.1.2.1.1]

The first procedure the UE needs to perform for DSMIPv6 initial attach is the discovery of the node acting as the HA.

The UE can discover the IP addresses of the HA in one of the four following ways:

-	via DNS;

-	via attach procedure for 3GPP access or trusted non-3GPP access (if supported) based on protocol configuration options;

-	via IKEv2 during tunnel setup to ePDG for untrusted non-3GPP accesses;

-	via DHCPv6.

If the UE does not obtain the IP addresses of the HA via PCO during the 3GPP or trusted non-3GPP (if supported) attach or via IKEv2 signalling, it shall follow either the procedures described in subclause 5.1.2.1.5 or the procedures described in subclause 5.1.2.1.2. The UE may be configured to perform both procedures in parallel or one of the two procedures only in case the other failed.

[TS 24.303, clause 5.1.2.1.5]

The HA address discovery via DHCPv6 is possible in the following cases:

-	in 3GPP access, or

-	in trusted non-3GPP access, when a DHCPv6 relay exists in the trusted non-3GPP access and the PDN GW is the DHCPv6 server, or

-	in trusted non-3GPP access, when the DHCPv6 server is in the trusted non-3GPP access and it has the HA addresse information from static configuration, or received via STa reference point as specified in 3GPP TS 29.273 [20].

A UE performing HA discovery based on DHCPv6 shall support the implementation of stateless DHCPv6 as specified in IETF RFC 3736 [13] and the DHCPv6 options as specified in IETF RFC 6610 [12].

In order to discover the address of the HA the UE shall send an Information-Request message including the "MIP6 Home Network ID FQDN Option" as described in IETF RFC 6610 [12].

NOTE:	The new options described in IETF RFC 6610 [12] are applicable to DSMIPv6.

In order to connect to a HA for a specific target PDN, the UE shall include the desired HA-APN in the Home Network Identification FQDN field contained in the "MIP6 Home Network ID FQDN Option" as described in IETF RFC 6610 [12].

The HA information is provided to the UE as described in IETF RFC 6610 [12] in the sub-option contained in the "MIP6 Identified Home Network Information Option". The sub-option can be:

-	a "MIP6 Home Agent Address Network Information Option" (the IPv6 address and if available, the IPv4 address of the HA); or

-	a "MIP6 Home Agent FQDN Network Information Option" (the HA FQDN) as described in IETF RFC 6610 [12].

In the latter case the UE shall perform a DNS Lookup by Home Agent Name as specified in IETF RFC 5026 [10]. The QNAME shall be set to the received HA FQDN.

If a HA has both an IPv4 and an IPv6 address, the corresponding DNS record should be configured with both 'AAAA' and 'A' records. Accordingly the UE should perform one DNS lookup procedure to retrieve both 'AAAA' and 'A' records. The DNS server replies with one 'AAAA' and one 'A' record.

15.2.3	Test description

15.2.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

<End of Changes>