3GPP TSG-RAN5 Meeting #105 	R5-247167

# Orlando, USA, 18th Nov. 2024 - 22nd Nov. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0911 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of QoQZ for QZ size 40 cm and ETC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | QoQZ MU is not defined for QZ size = 40 cm and ETC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | QoQZ MU was defined for QZ size = 40 cm and ETC in Table B.2.2.3-1 and Table B.2.2.19-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will not be completed for QZ size = 40 cm and ETC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.2.3, B.2.2.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on the outcome of MU discussion R5-247166 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

B.2.2.3	Quality of Quiet Zone

See B.2.1.3.

The uncertainty value of quality of quiet zone is estimated as below table and used across clause B.

Table B.2.2.3-1: Uncertainty value for quality of quiet zone for IFF

| QZ size | Power class | Condition | Test case | Frequency range | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All | All | All | ACLR (relative measurement) | FR2a, FR2b, FR2c | 0.52 | Actual | 1.00 | 0.52 |
| ≤30cm | PC1, PC3, PC5, PC6 | NC | NOTE1 | FR2a, FR2b | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | FR2c, FR2d | 0.7 (NOTE 2) | Actual | 1.00 | 0.7 |
|  |  |  | SE | 6GHz to 12.75GHz | 0.7 | Actual | 1.00 | 0.7 |
|  |  |  |  | 12.75GHz to 23.45GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 23.45GHz to 40.8GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 40.8GHz to 66GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 66GHz to 80GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 80GHz to 87GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  | ETC | NOTE1 | FR2a, FR2b, FR2c, FR2d | 0.9 | Actual | 1.00 | 0.9 |
| 40cm | PC1, PC3, PC5, PC6 | NC | NOTE1 | FR2a, FR2b | 0.7 (NOTE 2) | Actual | 1.00 | 0.7 |
|  |  |  |  | FR2c, FR2d | 0.8 (NOTE 2) | Actual | 1.00 | 0.8 |
|  |  |  | SE | 6GHz to 12.75GHz | 1.0 (NOTE 4) | Actual | 1.00 | 1.0 |
|  |  |  |  | 12.75GHz to 23.45GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 23.45GHz to 40.8GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 40.8GHz to 66GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 66GHz to 80GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 80GHz to 87GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  | ETC | NOTE1 | FR2a, FR2b | 0.9 | Actual | 1.00 | 0.9 |
| NOTE 1: The uncertainty in current row applies to maximum output power with EIRP and TRP, EIRP spherical coverage, MPR, configured output power with power boost, minimum output power, transmit OFF power, spectrum emission mask, reference sensitivity, adjacent selectivity, in-band blocking. NOTE 2: For Uncertainty assessment in tables across clause B and MTSU calculation, QoQZ uncertainty value for QZ ≤ 30cm is used also for 40cm for FR2a through FR2c. For uncertainty assessment in tables across clause B and MTSU calculation for FR2d, QoQZ uncertainty value for QZ size of 40cm is used also for QZ size ≤ 30cm. NOTE 3: Void NOTE 4: For Uncertainty assessment in tables across clause B and MTSU calculation, QoQZ uncertainty value for QZ ≤ 30cm is used also for 40cm. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

B.2.2.19	Quality of the Quiet Zone for Calibration Process

See B.2.1.19.

The uncertainty value of quality of quiet zone for calibration process is estimated as below table and used across clause B.

Table B.2.2.19-1: Uncertainty value for quiet zone for calibration process for IFF

| QZ size | Power class | Condition | Test case | Frequency range | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All | All | All | ACLR (relative measurement) | FR2a, FR2b, FR2c | 0.32 | Actual | 1.00 | 0.32 |
| ≤ 30cm | PC1, PC3, PC5, PC6 | NC | NOTE1 | FR2a, FR2b | 0.4 | Actual | 1.00 | 0.4 |
|  |  |  |  | FR2c, FR2d | 0.5 | Actual | 1.00 | 0.5 |
|  |  |  | SE | 6GHz to 12.75GHz | 0.7 | Actual | 1.00 | 0.7 |
|  |  |  |  | 12.75GHz to 23.45GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 23.45GHz to 40.8GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 40.8GHz to 66GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 66GHz to 80GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 80GHz to 87GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  | ETC | NOTE1 | FR2a, FR2b, FR2c, FR2d | 0.6 | Actual | 1.00 | 0.6 |
| 40cm | PC1, PC3, PC5, PC6 | NC | NOTE1 | FR2a, FR2b | 0.4 | Actual | 1.00 | 0.4 |
|  |  |  |  | FR2c, FR2d | 0.5 | Actual | 1.00 | 0.5 |
|  |  |  | SE | 6GHz to 12.75GHz | 1.0 (Note 3) | Actual | 1.00 | 1.0 |
|  |  |  |  | 12.75GHz to 23.45GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 23.45GHz to 40.8GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 40.8GHz to 66GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 66GHz to 80GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  |  |  | 80GHz to 87GHz | 0.6 | Actual | 1.00 | 0.6 |
|  |  | ETC | NOTE 1 | FR2a, FR2b, FR2c, FR2d | 0.6 | Actual | 1.00 | 0.6 |
| NOTE 1: The uncertainty in current row applies to maximum output power with EIRP and TRP, EIRP spherical coverage, MPR, configured output power with power boost, minimum output power, transmit OFF power, spectrum emission mask, reference sensitivity, adjacent selectivity, in-band blocking. NOTE 2: Void NOTE 3: For Uncertainty assessment in tables across clause B and MTSU calculation, QoQZ uncertainty value for QZ ≤ 30cm is used also for 40cm. |  |  |  |  |  |  |  |  |

<<End of change>>