3GPP TSG-RAN5 Meeting #105 	R5-247172

# Orlando, USA, 18th Nov. 2024 - 22nd Nov. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3389 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TCI-State for performance SDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Table 5.4.2.3-26, IE cell = 0 for both TCI-State#0 (qcl-Type = typeC) and TCI-State#1 (qcl-Type = typeA). Based on TS 38.331, IE cell is the serving cell where the reference signal is configured, and it can be different from the cell where the TCI-state is applied only if qcl-Type is typeC or typeD. So, with current setting in Table 5.4.2.3-26, - TCI-State#0,#1 would be invalid for NSA (NR serving cell = 1), and - TCI-State#1 would be invalid in cases with more than 1 NR cell, e.g., NR CA cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 5.4.2.3-26: - IE cell = 0 -> 'Not present' |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance test cannot be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

5.4.2.3	Message contents for Sustained downlink data rate requirements

<<Unchanged sections skipped>>

PDSCH-Config

Table 5.4.2.3-26: PDSCH-Config

| Derivation Path: Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| dataScramblingIdentityPDSCH | 0 |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeA CHOICE { |  |  |  |
| setup | DMRS-DownlinkConfig |  |  |
| } |  |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 2 entries |  |  |
| TCI-State[1] SEQUENCE { |  | entry 1 |  |
| tci-StateId | 0 | TCI-StateId 0 |  |
| qcl-type1 SEQUENCE { |  |  |  |
| cell | Not present |  |  |
| bwp-id | Not present | BWP_ID |  |
| referenceSignal | ssb |  |  |
| ssb | 0 |  |  |
| qcl-Type | typeC |  |  |
| } |  |  |  |
| qcl-type2 SEQUENCE { |  |  | SDR_FR2 |
| cell | Not present |  |  |
| bwp-id | Not present | BWP_ID |  |
| referenceSignal | ssb |  |  |
| ssb | 0 |  |  |
| qcl-Type | typeD |  |  |
| } |  |  |  |
| } |  |  |  |
| TCI-State[2] SEQUENCE { |  | entry 2 |  |
| tci-StateId | 1 | TCI-StateId 1 |  |
| qcl-type1 { |  |  |  |
| cell | Not present |  |  |
| bwp-id | 0 | BWP ID |  |
| referenceSignal | csi-rs |  |  |
| csi-rs | 0 |  |  |
| qcl-Type | typeA |  |  |
| } |  |  |  |
| qcl-type2 SEQUENCE { |  |  | SDR_FR2 |
| cell | Not present |  |  |
| bwp-id | 0 | BWP ID |  |
| referenceSignal | csi-rs |  |  |
| csi-rs | 0 |  |  |
| qcl-Type | typeD |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| vrb-ToPRB-Interleaver | Not present |  |  |
| resourceAllocation | resourceAllocationType0 |  |  |
| pdsch-AggregationFactor | Not present |  |  |
| rbg-Size | config1 | The UE ignores this field if resourceAllocation is set to resourceAllocationType1 (see TS 38.214 [21], clause 5.1.2.2.1). |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | wideband |  | SDR_FR1, SDR_FR2 |
| } |  |  |  |
| } |  |  |  |
| ZP-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 1 entry |  |  |
| ZP-CSI-RS-Resource[1] | ZP-CSI-RS-Resource | entry 1 |  |
| } |  |  |  |
| p-ZP-CSI-RS-ResourceSet | p-ZP-CSI-RS-ResourceSet |  |  |
| } |  |  |  |

<<End of change>>