3GPP TSG-RAN5 Meeting #105 	R5-247198

# Orlando, USA, 18th Nov. 2024 - 22nd Nov. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3597 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Active BWP Dedicated CORESET RMC in 16.6.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Table 16.6.1.7.5-1: - Active DL/UL BWP configuration for Cell 1 is incorrect. - The current RMC CORESET for Configurations 1, 2, and 4 is 24 RBs (CCR.1.1 FDD/TDD), and its start position is 108. RMC CORESET is not contained in active BWP Config (DLBWP.1.2 RedCap) as shown below. CCR.1.2 FDD/TDD which have 18 RBs should be used instead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 16.6.1.7.5-1: - Updated Active DL/UL BWP configuration: DL/ULBWP.1.1 RedCap to DL/ULBWP.1.2 RedCap (Core spec alignment with TS 38.133 V17.15.0) - Updated Dedicated CORESET RMC configuration: CCR.1.1 FDD/TDD to CCR.1.2 FDD/TDD (RAN4 CR dependent) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance test cannot be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-2417656 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

16.6.1.7	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE

16.6.1.7.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell identification and measurement period requirements in 38.133 [6] clauses 9.2B.6.1 and 9.2B.6.2.

16.6.1.7.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from release 17 onwards and supporting CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle.

16.6.1.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.7.

16.6.1.7.4	Test description

16.6.1.7.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.1.4.4.1 with following exceptions:

-	Table 6.6.1.4.4.1-1 is replaced by Table 16.6.1.7.4.1-1.

-	Table 6.6.1.4.4.1-2 is replaced by Table 16.6.1.7.4.1-2.

-	Table 6.6.1.4.4.1-3 is replaced by Table 16.6.1.7.4.1-3.

Table 16.6.1.7.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.7-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 16.6.1.7-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 16.6.1.7-3 | 30 kHz SSB SCS, 20MHz bandwidth, TDD duplex mode |
| 16.6.1.7-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.6.1.7.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.1.7.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1,2,3,4 | Cell 1 |  |  |
| Neighbour cell |  | 1,2,3,4 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1,2,3,4 | 1: Cell 1 and Cell 2 |  |  |
| Measurement gap type |  | 1,2,3,4, | Per-UE gaps |  |  |
| Measurement gap repetition periodicity | ms | 1,2,3,4 | 40 |  |  |
| Measurement gap length | ms | 1,2,3,4 | 6 |  |  |
| Measurement gap offset | ms | 1,2,3,4 | 39 |  |  |
| SSB configuration |  | 1,2,4 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |  |
| SMTC configuration |  | 1,4 | SMTC.2 |  |  |
|  |  | 2,3 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1, 4 | CSI-RS.1.2 FDD resource #0 |  |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |  |
| A3-Offset | dB | 1,2,3,4 | -4.5 |  |  |
| CP length |  | 1,2,3,4 | Normal |  |  |
| Hysteresis | dB | 1,2,3,4 | 0 |  |  |
| Time To Trigger | s | 1,2,3,4 | 0 |  |  |
| Filter coefficient |  | 1,2,3,4 | 0 |  | L3 filtering is not used |
| DRX | ms | 1,2,3,4 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1,4 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1 |
|  |  | 2,3 | 3 us |  | Synchronous cells |
| T1 | s | 1,2,3,4 | 5 |  |  |
| T2 | s | 1,2,3,4 | 5 | 10 |  |

16.6.1.7.4.2	Test procedure

Same as the test procedure given in 6.6.1.4.4.2 with following exceptions:

-	Step 6 in 6.6.1.4.4.2 is replaced by following step.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms for Test 1 or less than 7682 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

16.6.1.7.4.3	Message contents

Same as the message contents given in 6.6.1.4.4.3.

16.6.1.7.5	Test requirement

Same as the test requirement given in 6.6.1.4.5 with following exceptions:

-	Table 6.6.1.4.5-1 is replaced by Table 16.6.1.7.5-1.

-	The overall delay measured is expressed as:

Overall delays measured = TPSS/SSS_sync_intra_RedCap + T SSB_measurement_period_intra_RedCap + TTI insertion uncertainty

-	TPSS/SSS_sync_intra_RedCap = 600 ms for Test 1, and TPSS/SSS_sync_intra_RedCap = 4480 ms for Test 2;

-	T SSB_measurement_period_intra_RedCap= 320 ms for Test 1, and TPSS/SSS_sync_intra_RedCap = 3200 ms for Test 2

-	TTI insertion uncertainty = 2 ms

As a result, the overall delay measured for Test 1 and Test 2 shall be less than a total of 922 ms and 7682ms respectively.

Table 16.6.1.7.5-1: NR Cell specific test parameters

| Parameter | Unit | Test config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1,4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1,4 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1,4 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2 | CCR.1.2 TDD |  |  |  |
|  |  | 3 | CCR.2.3 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3,4 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1,2,3,4 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1,2,3,4 | DLBWP.1.2 RedCap |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.2 RedCap |  | ULBWP.1.1 |  |
| RLM-RS |  | 1,2,3,4 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1,2,4 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1,2,3,4 | -98 |  |  |  |
|  | dB | 1,2,3,4 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1,2,3,4 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | 1,2,4 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/ 9.36 MHz | 1,2,4 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/ 18.36 MHz | 3 | -61.68 | -59.34 |  |  |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

<<End of change>>