3GPP TSG-RAN5 Meeting #105	R5-247207

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5391 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct test case 22.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Test case has editorial notes - Specific message contents for SystemInformationBlockType2-NB incorrect - UL data transport procedure in step 13 for user plane CIoT is not correct.. |
| Summary of change: |  | - Resolving Editorial notes by adding loop back data transfer to the test case - Correcting specific message contents for SystemInformationBlockType2-NB - Correcting UL data transport procedure for user plane CIoT.. |
| Consequences if not approved: |  | Tes case will be incomplete and will fail conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 22.1.4	NB-IoT / NTN / User Plane CIoT Optimization / EDT

#### 22.1.4.1	Test Purpose (TP)

(1)

with { NB-IoT UE using the User Plane CIoT Optimization and with UP-EDT supported, with RRC Connection suspended }

ensure that {

when { EDT data transfer is triggered }

then { the UE selects a resource associated with EDT to initiate resumption of a suspended RRC connection with EDT}

}

(2)

with { NB-IoT UE using the User Plane CIoT Optimization and with UP-EDT supported, with RRC Connection suspended }

ensure that {

when { EDT data transfer is triggered }

then { performs resumption of a suspended RRC connection by transmitting RRCConnectionResumeRequest-NB and transfers the EDT user data on DTCH multiplexed with the RRCConnectionResumeRequest-NB message on CCCH }

}

(3)

with { NB-IoT UE using the User Plane CIoT Optimization and with UP-EDT supported initiated by sending RRCConnectionResumeRequest-NB }

ensure that {

when { UE receives the RRCConnectionResume-NB message }

then { performs UP-EDT fallback by sending the RRCConnectionResumeComplete-NB message }

}

#### 22.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300 clause 7.3b.3, TS 36.321 clause 5.1.2, TS 36.331, clause 5.3.1.1, 5.3.3.1, 5.3.3.1b and 5.3.3.3a. Unless otherwise stated these are Rel-17 requirements.

[TS 36.300, clause 7.3b.3]

The MO-EDT procedure for User Plane CIoT EPS optimisation is illustrated in Figure 7.3b-2.

Figure 7.3b-2: MO-EDT for User Plane CIoT EPS Optimisation

0.	Upon connection resumption request for Mobile Originated data from the upper layers, the UE initiates the MO-EDT procedure and selects a random access preamble configured for EDT.

1.	The UE sends an RRCConnectionResumeRequest to the eNB, including its Resume ID, the establishment cause, and an authentication token. The UE resumes all SRBs and DRBs, derives new security keys using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous RRC connection and re-establishes the AS security. The user data are ciphered and transmitted on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH. If enabled in the cell, the UE may indicate AS Release Assistance Information.

2.	The eNB initiates the S1-AP Context Resume procedure to resume the S1 connection and re-activate the S1-U bearers.

3.	The MME requests the S-GW to re-activate the S1-U bearers for the UE.

4.	The MME confirms the UE context resumption to the eNB.

5.	The uplink data are delivered to the S-GW.

6.	If downlink data are available, the S-GW sends the downlink data to the eNB.

7.	If no further data are expected, the eNB can initiate the suspension of the S1 connection and the deactivation of the S1-U bearers.

8.	The eNB sends the RRCConnectionRelease message to keep the UE in RRC_IDLE. The message includes the releaseCause set to rrc-Suspend, the resumeID, the NextHopChainingCount and drb-ContinueROHC which are stored by the UE. If downlink data were received in step 6, they are sent ciphered on DTCH multiplexed with the RRCConnectionRelease message on DCCH. The procedure ends with the reception of the HARQ feedback (ARQ) acknowledging the successful DL transmission.

[TS 36.321, clause 5.1.2]

The Random Access Resource selection procedure shall be performed as follows:

-	for BL UEs or UEs in enhanced coverage or NB-IoT UEs, if EDT is initiated by the upper layers:

-	if the message size (UL data available for transmission plus MAC header and, where required, MAC control elements) is larger than the TB size signalled in edt-TBS for the selected enhanced coverage level for EDT; or

-	if the PRACH resource associated with EDT for the selected enhanced coverage level is not available:

-	indicate to upper layers that EDT is cancelled;

-	for BL UEs or UEs in enhanced coverage, select the PRACH resource set corresponding to the selected enhanced coverage level. For EDT, the PRACH resource set shall correspond to the set associated with EDT for the selected enhanced coverage level.

-	if, except for NB-IoT, ra-PreambleIndex (Random Access Preamble) and ra-PRACH-MaskIndex (PRACH Mask Index) have been explicitly signalled and ra-PreambleIndex is not 000000:

-	the Random Access Preamble and the PRACH Mask Index are those explicitly signalled;

-	else if, for NB-IoT, ra-PreambleIndex (Random Access Preamble) and PRACH resource have been explicitly signalled:

-	the PRACH resource is that explicitly signalled;

-	if the ra-PreambleIndex signalled is not 000000:

-	if ra-CFRA-Config is configured:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers + (ra-PreambleIndex modulo (nprach-NumSubcarriers - nprach-NumCBRA-StartSubcarriers)), where nprach-SubcarrierOffset, nprach-NumCBRA-StartSubcarriers and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + (ra-PreambleIndex modulo nprach-NumSubcarriers), where nprach-SubcarrierOffset and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	select the Random Access Preamble group according to the PRACH resource and the support for multi-tone Msg3 transmission. A UE supporting multi-tone Msg3 shall only select the single-tone Msg3 Random Access Preambles group if there is no multi-tone Msg3 Random Access Preambles group.

-	randomly select a Random Access Preamble within the selected group.

-	else the Random Access Preamble shall be selected by the MAC entity as follows:

-	if the UE is a BL UE or UE in enhanced coverage and EDT is initiated:

-	select the Random Access Preambles group corresponding to PRACH resource for EDT for the selected enhanced coverage level.

-	else if the UE is a BL UE or UE in enhanced coverage and Random Access Preamble group B does not exist:

-	select the Random Access Preambles group corresponding to the selected enhanced coverage level.

-	else if the UE is an NB-IoT UE:

-	if the UE supports carrier specific NRSRP thresholds for NPRACH resource selection and rsrp-ThresholdsPrachInfoList-r16 is signalled for a carrier in ul-ConfigList:

-	if the measured RSRP is lower than the RSRP threshold corresponding to the selected enhanced coverage level in rsrp-ThresholdsPrachInfoList-r16:

-	do not consider the PRACH resource on this non-anchor carrier for PRACH resource selection.

-	randomly select one of the PRACH resources corresponding to the selected enhanced coverage level according to the configured probability distribution, and select the Random Access Preambles group corresponding to the PRACH resource and the support for multi-tone Msg3 transmission. A UE supporting multi-tone Msg3 shall only select the single-tone Msg3 Random Access Preambles group if there is no multi-tone Msg3 Random Access Preambles group. For EDT, the PRACH resource shall correspond to resource associated with EDT for the selected enhanced coverage level.

[TS 36.331, clause 5.3.1.1]

In case of CP-EDT or CP transmission using PUR, the data are appended in the RRCEarlyDataRequest and RRCEarlyDataComplete messages, if available, and sent over SRB0. In case of UP-EDT or UP transmission using PUR, security is re-activated prior to transmission of RRC message using the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure and the radio bearers are re-established. The uplink data are transmitted ciphered on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH. In the downlink, the data, if available, are transmitted on DTCH multiplexed with the RRCConnectionRelease message on DCCH. In response to a request for EDT or transmission using PUR, E-UTRA/EPC or E-UTRA/5GC may also choose to establish or resume the RRC connection.

[TS 36.331, clause 5.3.3.1]

Figure 5.3.3.1-3: RRC connection resume (suspended RRC connection or RRC_INACTIVE), or UP-EDT fallback or fallback from UP transmission using PUR to RRC connection resume, successful

Figure 5.3.3.1-4: RRC connection resume (suspended RRC connection or RRC_INACTIVE) or UP-EDT fallback or fallback from UP transmission using PUR to RRC connection establishment, successful

Figure 5.3.3.1-5: RRC connection resume or UP-EDT or UP transmission using PUR, network reject (suspended RRC connection or RRC_INACTIVE) or release (suspended RRC connection)

Figure 5.3.3.1-6: RRC connection resume (RRC_INACTIVE), network release or suspend or UP-EDT or UP transmission using PUR, successful

[TS 36.331, clause 5.3.3.1b]

A BL UE, UE in CE or NB-IoT UE can initiate EDT when all of the following conditions are fulfilled:

1>	if the UE is connected to EPC:

2>	for CP-EDT, the upper layers request establishment of an RRC connection, the UE supports CP-EDT, and SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes cp-EDT; or

2>	for UP-EDT, the upper layers request resumption of an RRC connection, the UE supports UP-EDT, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes up-EDT, and the UE has a stored value of the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure;

1>	else if the UE is connected to 5GC:

2>	for CP-EDT, the upper layers request establishment of an RRC connection, the UE connected to 5GC supports CP-EDT, and SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes cp-EDT-5GC; or

2>	for UP-EDT, the upper layers request resumption of an RRC connection, the UE connected to 5GC supports UP-EDT, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes up-EDT-5GC, and the UE has a stored value of the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure;

1>	the establishment or resumption request is for mobile originating calls and the establishment cause is mo-Data or mo-ExceptionData or delayTolerantAccess; or

1>	the establishment or resumption request is for mobile terminating calls, the UE has a stored mt-EDT indication and the establishment cause is mt-Access;

1>	the establishment or resumption request is suitable for EDT as specified in TS 36.300 [9], clause 7.3b.1;

1>	SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes edt-Parameters;

1>	for mobile originating calls, the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS signalled in edt-TBS as specified in TS 36.321 [6], clause 5.1.1;

1>	EDT fallback indication has not been received from lower layers for this establishment or resumption procedure;

NOTE 1:	Upper layers request or resume an RRC connection. The interaction with NAS is up to UE implementation.

NOTE 2:	It is up to UE implementation how the UE determines whether the size of UL data is suitable for EDT.

[TS 36.331, clause 5.3.3.3a]

If the UE is resuming the RRC connection from a suspended RRC connection, the UE shall set the contents of RRCConnectionResumeRequest message as follows:

1>	if the UE is a NB-IoT UE; or

1>	if the UE is initiating UP-EDT for mobile originating calls in accordance with conditions in 5.3.3.1b; or

1>	if the UE is initiating UP transmission using PUR in accordance with conditions in 5.3.3.1c; or

1>	if field useFullResumeID is signalled in SystemInformationBlockType2:

2>	if the UE connected to 5GC is a BL UE or UE in CE:

3>	set the fullI-RNTI to the stored fullI-RNTI;

#### 22.1.4.3	Test description

##### 22.1.4.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 as specified in TS 36.508 clause 8.1.4.1.1 is configured.

-	System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used.

UE:

-	px_DoAttachWithoutPDN set to false

-	The pre-configured UE location is defined in TS 36.508 [18] Clause 4.13.

-		The UE is configured to utilise non-SMS services as transport mechanism for user data(px_nonSMSTransport_CP_CIoT=TRUE).

Preamble:

-	The UE shall be in state 2B-NB with 1 default EPS bearer context active according to TS 36.508 [18].

##### 22.1.4.3.2	Test procedure sequence

Table 22.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits 1 IP packet to the UE. | <-- | IP packet | - | - |
| 2 | The SS transmits an RRCConnectionRelease-NB message including resumeIdentity and rrc-Suspend as releaseCause. | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 3 | The UE transmits preamble on PRACH resources associated with EDT | --> | PRACH Preamble | 1 | P |
| 4 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 3, including TC-RNTI and not including Back off Indicator sub header. | <-- | Random Access Response | - | - |
| 5 | Check: Does the UE transmit a MAC PDU containing the RRCConnectionResumeRequest-NB message including the resumeIdentity and AS security context stored at Step 1 and the EDT data on DTCH? | --> | MAC PDU (RRC: RRCConnectionResumeRequest-NB, EDT data on DTCH) | 2 | P |
| 6 | The SS transmits 1 IP packet to the UE. | <-- | -IP packet | - | - |
| 7 | The SS transmits an RRCConnectionRelease-NB message including a new resumeIdentity and rrc-Suspend as releaseCause. | - | RRCConnectionRelease-NB | - | - |
| 8 | The UE transmits preamble on PRACH resources associated with EDT. | --> | PRACH Preamble | 1 | P |
| 9 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 8, including TC-RNTI and not including Back off Indicator sub header. | <-- | Random Access Response | - | - |
| 10 | Check: Does the UE transmit a MAC PDU containing the RRCConnectionResumeRequest-NB message including the resumeIdentity and AS security context stored at Step 6 and the EDT data on DTCH? | --> | MAC PDU (RRC: RRCConnectionResumeRequest-NB, EDT data on DTCH) | 2 | P |
| 11 | The SS transmits an RRCConnectionResume-NB message. | <-- | RRC: RRCConnectionResume-NB | - | - |
| 12 | Check: Does the UE transmit an RRCConnectionResumeComplete-NB message? | --> | RRC: RRCConnectionResumeComplete-NB NAS: CONTROL PLANE SERVICE REQUEST | 3 | P |
| 13 | Steps 5 to 8 of test procedure in Table 8.1.5A.9.3-1is TS 36.509. | - | - | - | - |
| 14 | The UE loops back the IP packet received in Step 6. | --> | IP packet | - | - |

22.1.4.3.3	Specific message contents

Table 22.1.4.3.3-0: CLOSE UE TEST LOOP (preamble)

| Derivation path: Table 8.1.5.2B.4-1 in 38.508 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode | ‘00000001’B | Loop back mode B |  |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | ‘00000011’B | T_delay_modeB timer = 3 seconds |  |

Table 22.1.4.3.3-1: SystemInformationBlockType2-NB (preamble and all steps of Table 22.1.4.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon-r13 ::= SEQUENCE { |  |  |  |
| nprach-Config-v1530 ::= SEQUENCE { |  |  |  |
| edt-Parameters-r15 ::= SEQUENCE { |  |  |  |
| edt-SmallTBS-Subset-r15 | Not Present |  |  |
| edt-TBS-InfoList-r15 SEQUENCE (SIZE (1..maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 1 entry |  |  |
| edt-SmallTBS-Enabled-r15[1] | False |  |  |
| edt-TBS-r15 | b504 |  |  |
| } |  |  |  |
| nprach-ParametersListEDT-r15 SEQUENCE (SIZE (1..maxNPRACH-Resources-NB-r13)) OF SEQUENCE { |  |  |  |
| nprach-Parameters-r14 ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms2460 |  |  |
| nprach-StartTime-r14 | ms8 |  |  |
| nprach-SubcarrierOffset-r14 | n12 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r14 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v4 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cp-EDT-r15 | Not present |  |  |
| up-EDT-r15 | True |  |  |
| } |  |  |  |

Table 22.1.4.3.3-2: RRCConnectionRelease-NB (steps 1 and 6, table 22.1.4.3.2-1)

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r13 SEQUENCE { |  |  |  |
| releaseCause-r13 | rrc-Suspend |  |  |
| resumeIdentity-r13 | BIT STRING (SIZE(40)) | Any value; Different value in steps 1 and 6 |  |
| extendedWaitTime-r13 | Not present |  |  |
| redirectedCarrierInfo-r13 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| redirectedCarrierInfo-v1430 | Not present |  |  |
| extendedWaitTime-CPdata-r14 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| drb-ContinueROHC-r15 | Not present |  |  |
| nextHopChainingCount-r15 | NCC | As defined in TS 33.401 |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.1.4.3.3-3: RRCConnectionResumeRequest-NB (step 5, Table 22.1.4.3.2-1)

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionResumeRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionResumeRequest-r13 SEQUENCE { |  |  |  |
| resumeID-r13 | BIT STRING (SIZE(40)) | Same value as Step 1 |  |
| shortResumeMAC-I-r13 | The same value as the 16 least significant bits of the MAC-I value calculated by SS. |  |  |
| resumeCause-r13 | mo-Data |  |  |
| earlyContentionResolution-r14 | Not checked |  |  |
| cqi-NPDCCH-r14 | Not checked |  |  |
| spare | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.1.4.3.3-4: RRCConnectionResumeRequest-NB (step 10, Table 22.1.4.3.2-1)

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionResumeRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionResumeRequest-r13 SEQUENCE { |  |  |  |
| resumeID-r13 | BIT STRING (SIZE(40)) | Same value as Step 6 |  |
| shortResumeMAC-I-r13 | The same value as the 16 least significant bits of the MAC-I value calculated by SS. |  |  |
| resumeCause-r13 | mo-Data |  |  |
| earlyContentionResolution-r14 | Not checked |  |  |
| cqi-NPDCCH-r14 | Not checked |  |  |
| spare | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |