3GPP TSG-RAN WG5 Meeting #105	R5-247252

# Orlando, USA,  18 – 22 Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3401 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test channel bandwidths and test frequencies for singalling test for band n254 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test CBWs and test frequencies need to be specified for NR band n254 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant clauses updated to add test frequencies for band n254 - 6.2.3.8 Test frequencies for NR NTN configurations for signalling testing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test CBWs and test frequencies for band n254 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.2.3.8	Test frequencies for NR NTN configurations for signalling testing

The default channel bandwidths for NR NTN signalling test are specified per NR NTN operation band.

For signalling test cases, the mapping of frequency ranges to NR NTN test frequencies are as follows:

-	for band with only one test frequency: Low Range (NRf1);

-	for band with up to two test frequencies: Low Range (NRf1) and High Range (NRf2);

-	for band with up to three test frequencies: Low Range (NRf1), Mid Range (NRf2) and High Range (NRf3);

The test frequencies, subcarrier spacing, default channel bandwidth and S-SSBparameters for signalling is specified in Table 6.2.3.8-1 for NR NTN operations.

Table 6.2.3.8-1: Test frequencies for NR NTN operating band using 5 MHz channel bandwidth

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n254 | 15 | 5 | 25 | Downlink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.9.1.3-1 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.9.1.3-1 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n255 | 15 | 5 | 25 | Downlink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.9.1.2-1 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.9.1.2-1 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n256 | 15 | 5 | 25 | Downlink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.9.1.1-1 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.9.1.1-1 for bandwidth=5 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |