3GPP TSG-RAN WG5 Meeting # 105	R5-247254

# Orlando, USA,  18 – 22 Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3115 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding transmitter requirements for band n39 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing transmitter requirements for band n39 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added transmitter requirements for band n39 in clauses - 6.2. Transmitter power - 6.2D Transmitter power for UL MIMO - 6.2G Transmitter power for Tx Diversity - 6.3G Output power dynamics for Tx Diversity - 6.5D Output RF spectrum emissions for UL MIMO - 6.5G Output RF spectrum emissions for Tx Diversity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transmitter requirements for band n39 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, 6.2D, 6.2G, 6.3G, 6.5D, 6.5G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

6.2.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and don't support Redcap.

6.2.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1.3-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n2 |  |  |  |  | 23 | ± 23 |
| n3 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n5 |  |  |  |  | 23 | ± 2 |
| n7 | 316 | +2/-3 |  |  | 23 | ± 23 |
| n8 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n12 |  |  |  |  | 23 | ± 23 |
| n13 |  |  |  |  | 23 | ±2 |
| n14 | 316 | +2/-3 |  |  | 23 | ± 2 |
| n20 |  |  |  |  | 23 | ± 23 |
| n24 |  |  |  |  | 23 | +2/-33 |
| n25 | 316 | +2/-3 |  |  | 23 | ± 23 |
| n26 |  |  |  |  | 23 | ± 23 |
| n28 |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  | 23 | ± 2 |
| n31 |  |  |  |  | 23 | ±2 |
| n34 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n38 |  |  |  |  | 23 | ± 2 |
| n39 |  |  | 26 | +2/-3 | 23 | ± 2 |
| n40 | 316 | +2/-3 | 26 | +2/-3 | 23 | ± 2 |
| n41 | 316 | +2/-3 | 26 | +2/-33 | 23 | ± 23 |
| n48 |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  | 23 | ± 2 |
| n51 |  |  |  |  | 23 | ± 2 |
| n53 |  |  |  |  | 23 | ±2 |
| n54 |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  | 23 | ±2 |
| n66 | 316 | +2/-3 |  |  | 23 | ± 2 |
| n70 |  |  |  |  | 23 | ± 2 |
| n71 | 316 | +2/-3 |  |  | 23 | +2/-2.5 |
| n72 |  |  |  |  | 23 | ±2 |
| n74 |  |  |  |  | 23 | ± 2 |
| n77 | 316 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n78 | 316 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 23 | ± 2 |
| n81 |  |  |  |  | 23 | ± 2 |
| n82 |  |  |  |  | 23 | ± 2 |
| n83 |  |  |  |  | 23 | + 2/-2.5 |
| n84 |  |  |  |  | 23 | ± 2 |
| n85 | 316 | +2/-3 |  |  | 23 | ±23 |
| n86 |  |  |  |  | 23 | ± 2 |
| n91 |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  | 23 | ±2 |
| n97 |  |  |  |  | 23 | ±2 |
| n99 |  |  |  |  | 23 | +2/-33 |
| n100 | 316 | +2/-3 |  |  | 23 | ±2 |
| n101 | 316 | +2/-3 |  |  | 23 | ±2 |
| n106 |  |  |  |  | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: FFS NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

- if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

- if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.331 [6] (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [6] is provided and set to the maximum output power of the default power class or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in sub-clause 6.2.4;

- else if the UE does not support a power class with higher maximum output power than PC2; or

-	if the IE P-Max as defined in TS 38.331 [6] is provided and set to the maximum output power of the power class 2 or lower;

-	shall apply all requirements for power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2.4;

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.1.

6.2.1.4	Test description

6.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest (NOTE 3) |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For band n28, the Highest test channel bandwidth is replaced by 20MHz due to MPR is always larger than 0dB for 30MHz bandwidth. NOTE 4: For P-Max test (Step 4 in Section 6.2.1.4.2) and maxUplinkDutyCycle test (Step 5 in Section 6.2.1.4.2), only “Normal” test environment, “Low range” test frequencies, “Lowest” Test Channel Bandwidth, “Lowest” test SCS and “Inner Full” RB allocation to be used. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.1.4.3.

6.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

4.	For UEs supporting Power Class 2 or Power Class 1, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2.1.4.3-2.

5.	For TDD UEs supporting Power Class 2 with UE capability maxUplinkDutyCycle-PC2-FR1, repeat steps 1~3 on the applicable bands with message exception of TDD UL-DL pattern defined in Table 6.2.1.4.3-4 and Table 6.2.1.4.3-5 which the UplinkDutyCycle is closest less than or equal to the maxUplinkDutyCycle-PC2-FR1 reported by UE. Before Step 3, wait for at least 10ms for the UE to complete the evaluation period.

6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 6.2.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2.1.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 | PC2 UE or PC1 UE will fallback to PC3 UE with P-Max=23 | PC2 UE or PC1 UE |

Table 6.2.1.4.3-3: Void

Table 6.2.1.4.3-4: TDD UL-DL pattern for SCS 15 KHz (UplinkDutyCycle=60%)

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | DSUUU |  |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U |  |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 15 |  |
|  | dl-UL-TransmissionPeriodicity | ms | 5 |  |
|  | nrofDownlinkSlots |  | 1 |  |
|  | nrofDownlinkSymbols |  | 10 |  |
|  | nrofUplinkSlot |  | 3 |  |
|  | nrofUplinkSymbols |  | 2 |  |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [4] if mod(i,5) = 0 |  |
| PUCCH resource indicator |  |  | PUCCH-ResourceId[8] = 7 in pucch-ResourceSetID[1] as defined in TS 38.508-1 [5] Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘111’B value in binary |
| SS/PBCH block index |  |  | 0 | SSB#0 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,9} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 1 and slot 6 to meet the specific UplinkDutyCycle. |  |  |  |  |

Table 6.2.1.4.3-5: TDD UL-DL pattern for SCS 30 KHz (UplinkDutyCycle=60%)

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | DSUUU |  |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U |  |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 30 |  |
|  | dl-UL-TransmissionPeriodicity | ms | 2.5 single |  |
|  | nrofDownlinkSlots |  | 1 |  |
|  | nrofDownlinkSymbols |  | 10 |  |
|  | nrofUplinkSlot |  | 3 |  |
|  | nrofUplinkSymbols |  | 2 |  |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [4] if mod(i,5) = 0 |  |
| PUCCH resource indicator |  |  | PUCCH-ResourceId[8] = 7 in pucch-ResourceSetID[1] as defined in TS 38.508-1 [5] Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘111’B value in binary |
| SS/PBCH block index |  |  | 0 | SSB#0 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,19} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 1, slot 6, slot 11 and slot 16 to meet the specific UplinkDutyCycle. |  |  |  |  |

6.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1 for Power Class 3, Table 6.2.1.5-2 for Power Class 2, Table 6.2.1.5-2a for Power Class 1.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1.

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-2 for Power Class 2 as defined in TS 38.306.

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±(2+TT) |
| n2 |  |  |  |  | 23 | ±(23+TT) |
| n3 |  |  |  |  | 23 | ±(23+TT) |
| n5 |  |  |  |  | 23 | ±(2+TT) |
| n7 |  |  |  |  | 23 | ±(23+TT) |
| n8 |  |  |  |  | 23 | ±(23+TT) |
| n12 |  |  |  |  | 23 | ±(23+TT) |
| n13 |  |  |  |  | 23 | ±(2+TT) |
| n14 |  |  |  |  | 23 | ±(2+TT) |
| n20 |  |  |  |  | 23 | ±(23+TT) |
| n24 |  |  |  |  | 23 | +2+TT/-3.03-TT |
| n25 |  |  |  |  | 23 | ±(23+TT) |
| n26 |  |  |  |  | 23 | ±(23+TT) |
| n28 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  | 23 | ±(2+TT) |
| n31 |  |  |  |  | 23 | ±(2+TT) |
| n34 |  |  |  |  | 23 | ±(2+TT) |
| n38 |  |  |  |  | 23 | ±(2+TT) |
| n39 |  |  |  |  | 23 | ±(2+TT) |
| n40 |  |  |  |  | 23 | ±(2+TT) |
| n41 |  |  |  |  | 23 | ±(23+TT) |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | ±(2+TT) |
| n51 |  |  |  |  | 23 | ±(2+TT) |
| n53 |  |  |  |  | 23 | ±(2+TT) |
| n54 |  |  |  |  | 23 | ±(2+TT) |
| n65 |  |  |  |  | 23 | ±(2+TT) |
| n66 |  |  |  |  | 23 | ±(2+TT) |
| n70 |  |  |  |  | 23 | ±(2+TT) |
| n71 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n72 |  |  |  |  | 23 | ±(2+TT) |
| n74 |  |  |  |  | 23 | ±(2+TT) |
| n77 |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  | 23 | +2+TT/-3-TT |
| n85 |  |  |  |  | 23 | ±(23+TT) |
| n91 |  |  |  |  | 23 | ±(23, 5+TT) |
| n92 |  |  |  |  | 23 | ±(23, 5+TT) |
| n93 |  |  |  |  | 23 | ±(23, 5+TT) |
| n94 |  |  |  |  | 23 | ±(23, 5+TT) |
| n100 |  |  |  |  | 23 | ±(2+TT) |
| n101 |  |  |  |  | 23 | ±(2+TT) |
| n106 |  |  |  |  | 23 | ±(2+TT) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 NOTE 5: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. |  |  |  |  |  |  |

Table 6.2.1.5-2: Maximum Output Power test requirement for Power Class 2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2+TT/-3-TT |  |  |
| n3 |  |  | 26 | +2+TT/-33-TT |  |  |
| n8 |  |  | 26 | +2+TT/-33-TT |  |  |
| n34 |  |  | 26 | +2+TT/-3-TT |  |  |
| n39 |  |  | 26 | +2+TT/-3-TT |  |  |
| n40 |  |  | 26 | +2+TT/-3-TT | n40 |  |
| n41 |  |  | 26 | +2+TT/-33-TT |  |  |
| n77 |  |  | 26 | +2+TT/-3-TT |  |  |
| n78 |  |  | 26 | +2+TT/-3-TT |  |  |
| n79 |  |  | 26 | +2+TT/-3-TT |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 6.2.1.5-2a: Maximum Output Power test requirement for Power Class 1

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n75 | 31 | +2+TT/-3-TT |  |  |  |  |
| n145 | 31 | +2+TT/-3-TT |  |  |  |  |
| n255 | 31 | +2+TT/-3-TT |  |  |  |  |
| n405 | 31 | +2+TT/-3-TT |  |  |  |  |
| n415 | 31 | +2+TT/-3-TT |  |  |  |  |
| n665 | 31 | +2+TT/-3-TT |  |  |  |  |
| n715 | 31 | +2+TT/-3-TT |  |  |  |  |
| n775 | 31 | +2+TT/-3-TT |  |  |  |  |
| n785 | 31 | +2+TT/-3-TT |  |  |  |  |
| n855 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1005 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1015 | 31 | +2+TT/-3-TT |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3. NOTE 5: Generally, PC1 UEis not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |

Table 6.2.1.5-2b: Void

Table 6.2.1.5-3: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2.2	UE maximum output power reduction

6.2.2.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2.2.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5.2.4.1 is executed.

6.2.2.3	Minimum conformance requirements

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 2 and 3 and UE power class 1, the allowed maximum power reduction (MPR) is defined in Table 6.2.2.3-2, Table 6.2.2.3-1, Table 6.2.2.3-4b and Table 6.2.2.3-5, respectively for channel bandwidths ≤ 100 MHz.

If the relative channel bandwidth ≤ 4% for TDD bands or ≤ 3% for FDD band, the ∆MPR is set to zero.

If the relative channel bandwidth is larger than 4% for TDD bands or 3% for FDD bands, the ∆MPR is defined in Table 6.2.2.3-3.

Where relative channel bandwidth = 2*BWChannel / (FUL_low + FUL_high).

The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

Table 6.2.2.3-1: Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52,3 | ≤ 0.52 | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 |  |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0dB MPR is 26dBm. NOTE 2: Applicable for conditions where Note 1 does not apply. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB. |  |  |  |  |

Table 6.2.2.3-2: Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 6.2.2.3-3: ∆MPR

| NR Band | Power class | Channel bandwidth | ∆MPR (dB) |
| --- | --- | --- | --- |
| n28 and n83 | Power class 3 | 30 MHz | 0.5 |
| n40 and n97 | Power class 3 and power class 2 | 100 MHz | 1 |

Table 6.2.2.3-4: Void

Table 6.2.2.3-4a: Void

Table 6.2.2.3-4b: Maximum power reduction (MPR) for power class 1 for bands other than Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | (NOTE 1) | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | (NOTE 1) | 0 | 0 |
|  | QPSK | (NOTE 1) | ≤ 1 | 0 |
|  | 16 QAM | (NOTE 1) | ≤ 2 | ≤ 1 |
|  | 64 QAM | (NOTE 1) | ≤ 2.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 4.5 |  |
| CP-OFDM | QPSK | (NOTE 1) | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | (NOTE 1) | ≤ 3 | ≤ 2 |
|  | 64 QAM | (NOTE 1) | ≤ 3.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 6.5 |  |
| NOTE 1: MPR for all modulations for Edge RB allocation is defined as following for two distinguished channel bandwidths groups as: Within the <50MHz channel bandwidth group: MPR= CEIL7.2 dB- 6 dB∙ CBW100 MHz, 0.5 dB Within the ≥50MHz channel bandwidth group: MPR= CEIL5.35 dB+3.15 dB∙ CBW100 MHz, 0.5 dB where CEIL(x,0.5 dB) means rounding x upwards to the closest multiple of 0.5 dB. |  |  |  |  |

Table 6.2.2.3-5: Maximum power reduction (MPR) for power class 1 for Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart ≤ RBStart,High, and

LCRB ≤ ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel with LCRB ≤ 2 RBs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

If CP-OFDM allocation satisfies following conditions, it is considered as almost contiguous allocation

NRB_gap / (NRB_alloc + NRB_gap) ≤ 0.25

and NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz SCS respectively where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs. The size and location of allocated and unallocated RBs are restricted by RBG parameters specified in sub-clause 6.1.2.2 of TS 38.214 [12]. For UE that indicates support for almostContiguousCP-OFDM-UL, the almost contiguous signals in power class 2 and 3, the allowed maximum power reduction defined in Table 6.2.2.3-2 and Table 6.2.2.3-1 are increased by

CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB,

where CEIL{x,0.5} means x rounding upwards to closest 0.5dB. The parameters of RBStart,Low and RBStart,High to specify valid RB allocation ranges for Outer and Inner RB allocations are defined as following:

RBStart,Low = max(1, floor((NRB_alloc + NRB_gap)/2))

RBStart,High = NRB – RBStart,Low – NRB_alloc –NRB_gap

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.4 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.2.

6.2.2.4	Test description

6.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a, Table 6.2.2.4.1-2c and Table 6.2.2.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.2.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 23 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 54 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 64 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 31 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 32 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 374,5 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 384,5 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 394,5 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

Table 6.2.2.4.1-1a: Void

Table 6.2.2.4.1-2: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

Table 6.2.2.4.1-2a: Test Configuration Table for power class 1 for Band n14 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 333 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 343 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 353 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for Ues which supports Pi/2 BPSK in FR1. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

Table 6.2.2.4.1-2b: Void

Table 6.2.2.4.1-2c: Test Configuration Table for power class 1 for bands other than Band n14 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest for all RB allocations Additionally, for edge RB allocations, the maximum channel bandwidth <50MHz and the minimum channel bandwidth ≥50MHz that are supported by the UE (NOTE 4) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 333 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 343 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 353 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for Ues which supports Pi/2 BPSK in FR1. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. NOTE 4: The maximum channel bandwidth <50MHz and the minimum channel bandwidth ≥50MHz are tested only with the highest order modulation supported by the UE. |  |  |  |  |

Table 6.2.2.4.1-3: Test Configuration Table for power class 2&3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Tables 6.2.2.4.1-4, 6.2.2.4.1-4a and 6.2.2.4.1-4b. NOTE 2: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

Table 6.2.2.4.1-4: Uplink configuration for almost contiguous allocation for low frequency range

| Channel Bandwidth(MHz) | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 48@0 | 53@80 | N/A | N/A |
|  | 30 | CP | 24@0 | 25@40 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 64@0 | 64@96 | N/A | N/A |
|  | 30 | CP | 32@0 | 30@48 | N/A | N/A |
|  | 60 | CP | 16@0 | 14@24 | N/A | N/A |
| 35 | 15 | CP | 80@0 | 76@112 | N/A | N/A |
|  | 30 | CP | 40@0 | 36@56 | N/A | N/A |
|  | 60 | CP | 20@0 | 16@28 | N/A | N/A |
| 40 | 15 | CP | 80@0 | 88@128 | N/A | N/A |
|  | 30 | CP | 40@0 | 42@64 | N/A | N/A |
|  | 60 | CP | 20@0 | 19@32 | 12@12 | 8@28 |
| 45 | 15 | CP | 80@0 | 114@128 | 48@64 | 48@128 |
|  | 30 | CP | 40@0 | 55@64 | 24@32 | 24@64 |
|  | 60 | CP | 20@0 | 26@32 | 12@16 | 12@32 |
| 50 | 15 | CP | 96@0 | 110@160 | 48@64 | 48@144 |
|  | 30 | CP | 48@0 | 53@80 | 24@32 | 24@72 |
|  | 60 | CP | 24@0 | 25@40 | 12@16 | 12@36 |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 64@0 | 66@96 | 32@32 | 16@80 |
|  | 60 | CP | 32@0 | 31@48 | 16@16 | 8@40 |
| 70 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 80@0 | 77@112 | 32@32 | 16@80 |
|  | 60 | CP | 40@0 | 37@56 | 16@16 | 8@40 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 80@0 | 89@128 | 32@32 | 16@80 |
|  | 60 | CP | 40@0 | 43@64 | 16@16 | 8@40 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 96@0 | 101@144 | 32@32 | 16@80 |
|  | 60 | CP | 48@0 | 49@72 | 16@16 | 8@40 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 112@0 | 97@176 | 48@64 | 48@144 |
|  | 60 | CP | 48@0 | 55@80 | 24@32 | 24@72 |

Table 6.2.2.4.1-4a: Uplink configuration for almost contiguous allocation for TDD high frequency range

| Channel Bandwidth(MHz) | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 48@0 | 53@80 | N/A | N/A |
|  | 30 | CP | 24@0 | 25@40 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 56@0 | 64@88 | N/A | N/A |
|  | 30 | CP | 32@0 | 30@48 | N/A | N/A |
|  | 60 | CP | 16@0 | 14@24 | N/A | N/A |
| 35 | 15 | CP | 72@0 | 84@104 | N/A | N/A |
|  | 30 | CP | 40@0 | 36@56 | N/A | N/A |
|  | 60 | CP | 20@0 | 16@28 | N/A | N/A |
| 40 | 15 | CP | 72@0 | 96@120 | N/A | N/A |
|  | 30 | CP | 40@0 | 42@64 | N/A | N/A |
|  | 60 | CP | 20@0 | 19@32 | 12@12 | 8@28 |
| 45 | 15 | CP | 72@0 | 122@120 | 48@56 | 48@120 |
|  | 30 | CP | 40@0 | 55@64 | 24@32 | 24@64 |
|  | 60 | CP | 20@0 | 26@32 | 12@16 | 12@32 |
| 50 | 15 | CP | 88@0 | 112@152 | 48@72 | 48@152 |
|  | 30 | CP | 48@0 | 53@80 | 24@32 | 24@72 |
|  | 60 | CP | 24@0 | 25@40 | 12@16 | 12@36 |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 56@0 | 74@88 | 32@40 | 16@88 |
|  | 60 | CP | 32@0 | 31@48 | 16@16 | 8@40 |
| 70 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 72@0 | 80@104 | 32@40 | 16@88 |
|  | 60 | CP | 40@0 | 37@56 | 16@16 | 8@40 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 72@0 | 96@120 | 32@40 | 16@88 |
|  | 60 | CP | 40@0 | 43@64 | 16@16 | 8@40 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 88@0 | 109@136 | 32@40 | 16@88 |
|  | 60 | CP | 48@0 | 49@72 | 16@16 | 8@40 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 104@0 | 105@168 | 48@72 | 48@152 |
|  | 60 | CP | 48@0 | 55@80 | 24@32 | 24@72 |

Table 6.2.2.4.1-4b: Uplink configuration for almost contiguous allocation for FDD high frequency range

| Channel Bandwidth(MHz) | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 42@0 | 56@74 | N/A | N/A |
|  | 30 | CP | 22@0 | 27@38 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 58@0 | 64@90 | N/A | N/A |
|  | 30 | CP | 26@0 | 32@42 | N/A | N/A |
|  | 60 | CP | 14@0 | 16@22 | N/A | N/A |
| 35 | 15 | CP | 74@0 | 82@106 | N/A | N/A |
|  | 30 | CP | 42@0 | 34@58 | N/A | N/A |
|  | 60 | CP | 22@0 | 18@26 | N/A | N/A |
| 40 | 15 | CP | 74@0 | 94@122 | N/A | N/A |
|  | 30 | CP | 34@0 | 48@58 | N/A | N/A |
|  | 60 | CP | 18@0 | 20@30 | 12@14 | 8@30 |
| 45 | 15 | CP | 74@0 | 120@122 | 48@58 | 48@122 |
|  | 30 | CP | 34@0 | 56@58 | 24@34 | 24@66 |
|  | 60 | CP | 18@0 | 28@30 | 12@14 | 12@30 |
| 50 | 15 | CP | 90@0 | 112@154 | 48@74 | 48@154 |
|  | 30 | CP | 42@0 | 56@74 | 24@34 | 24@74 |
|  | 60 | CP | 22@0 | 27@38 | N/A | N/A |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 58@0 | 72@90 | 32@42 | 16@90 |
|  | 60 | CP | 26@0 | 32@42 | 16@18 | 8@42 |
| 70 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 74@0 | 80@106 | 32@42 | 16@90 |
|  | 60 | CP | 34@0 | 40@50 | 16@18 | 8@42 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 74@0 | 95@122 | 32@42 | 16@90 |
|  | 60 | CP | 34@0 | 48@58 | 16@18 | 8@42 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 90@0 | 107@138 | 32@42 | 16@90 |
|  | 60 | CP | 42@0 | 55@66 | 16@18 | 8@42 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 106@0 | 103@170 | 48@74 | 48@154 |
|  | 60 | CP | 42@0 | 56@74 | 24@34 | 24@74 |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a, Table 6.2.2.4.1-2c and Table 6.2.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

6.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a, Table 6.2.2.4.1-2c and Table 6.2.2.4.1-3. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	For UEs supporting Power Class 1 and Power Class 2, repeat steps 1~3 for Test ID 22 and 36 in Table 6.2.2.4.1-1 on the applicable bands with message exception of P-Max defined in Table 6.2.2.4.3-2.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

Table 6.2.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.2.2.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE or PC1 UE |

Table 6.2.2.4.3-2a: Void

Table 6.2.2.4.3-3: DMRS-UplinkConfig (Test ID 37 - 39 in Table 6.2.2.4.1-1)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.4.3-4: ServingCellConfig

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 1 |  | Test IDs where NOTE 3 in Table 6.2.2.4.1-1 applies. |
|  | 0 |  | Test IDs where NOTE 4 in Table 6.2.2.4.1-1 applies. |
| } |  |  |  |
| } |  |  |  |

6.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 to Table 6.2.2.5-9a.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1,4 Table 6.2.2.5-r.

Table 6.2.2.5-1: UE Power Class test requirements(for Bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n30, n31, n34, n38, n39, n40, n41, n50, n51, n53, n54, n65, n66, n70, n72, n74, n85, n91, n92, n93, n94, n100, n101, n106) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | （1.52） | 25.8 | （24.32） | 2.0 |  | 2 | 28.0 + TT | 23.8 - TT | （22.3 - TT2） |
| 2 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 3 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 4 | 23 | -3 | 1.2 | 0 | （1.52） | 24.8 | （23.32） | 2.0 |  | 2 | 28.0 + TT | 22.8 - TT | （21.3 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| 6 | 23 | 0 | 0.55 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 7 | 23 | 0 | 0.55 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 2 | 25.0 + TT | 19.5 - TT | （17.5 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | (19.0 - TT2) |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25 n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: For bands n91, n92, n93 and n94 the maximum power reduction requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-1a: UE Power Class test requirements Band 24 for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 – TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （15.5 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 25.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 6 | 23 | 0 | 0.53 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 7 | 23 | 0 | 0.53 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2.5 | 25.0 + TT | 19.0 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: For 3 MHz channel bandwidth the Pi/2 BPSK edge allocation MPR is 1 dB |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2a: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 6 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 7 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 8 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 9 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 10 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 11 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 12 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 13 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 16 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 17 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 18 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 19 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 20 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 21 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 22 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 23 | 23 | 0 | 2 | 0 | 21 | 2 | 2.5 | 25.0 + TT | 18.5-TT |
| 24 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 25 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 26 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 27 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 28 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 29 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 30 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 32 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 33 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 34 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 35 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 36 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 37 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 38 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 39 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-3: UE Power Class test requirements (for Bands n48, n77, n78, n79) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | 25.8 | 2.0 | 3 | 28.0 + TT | 22.8 - TT |
| 2 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 3 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 4 | 23 | -3 | 1.2 | 0 | 24.8 | 2.0 | 3 | 28.0 + TT | 21.8 - TT |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| 6 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 7 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 3 | 25.0 + TT | 18.5 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4: UE Power Class test requirements (for Bands n1, n3, n34, n39, n40, n41, n77, n78, n79) for Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 2.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0- TT2） |
| 4 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 2.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 5 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 2.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 8 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 2.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 9 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 2.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 12 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 2.0 |  | 3 | 28.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0- TT2） |
| 15 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 16 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 17 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 18 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 19 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 2.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 20 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 21 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 22 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 28.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 23 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 2.0 |  | 3 | 28.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 24 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 26 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 28.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 27 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 28 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 29 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 30 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 31 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n3 and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4a: UE Power Class test requirements for Band n14 for Power Class 1 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 2 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 3 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 4 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 5 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 6 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 7 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 8 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 9 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 10 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 11 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 12 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 13 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 14 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 15 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 16 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 17 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 18 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 19 | 31 | 0 | 1.5 | 0 | 29.5 | 2.0 | 3 | 33.0 + TT | 26.5 — TT |
| 20 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 21 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 22 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 23 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 24 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 25 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 26 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 27 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 28 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 29 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 30 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 31 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 32 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 33 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 34 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 35 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4b: Void

Table 6.2.2.5-4c: UE Power Class test requirements for bands other than Band n14 (bands n7, n78) (bands n41, n100 and n101) for Power Class 1 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | <50 MHz CBW | ≥50 MHz CBW |  | <50 MHz CBW | ≥50 MHz CBW |  |  |  | <50 MHz CBW | ≥50 MHz CBW |
| 1 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| 2 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 3 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 4 | 31 | 0 | 0.5 | 0.5 | 0 | 30.5 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 - TT | 27.5 - TT |
| 5 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| 6 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 7 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 8 | 31 | 0 | 1 | 1 | 0 | 30.0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 - TT | 27.0 - TT |
| 9 | 31 | 0 | 1 | 1 | 0 | 30.0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 - TT | 27.0 - TT |
| 10 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 11 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 12 | 31 | 0 | 2 | 2 | 0 | 29.0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 - TT | 26.0 - TT |
| 13 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 14 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 15 | 31 | 0 | 2.5 | 2.5 | 0 | 28.5 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 - TT | 25.5 - TT |
| 16 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 17 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 18 | 31 | 0 | 4.5 | 4.5 | 0 | 26.5 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 - TT | 23.5 - TT |
| 19 | 31 | 0 | 1.5 | 1.5 | 0 | 29.5 | 29.5 | 2.0 | 3 | 33.0 + TT | 26.5 - TT | 26.5 - TT |
| 20 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 21 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 22 | 31 | 0 | 3 | 3 | 0 | 28.0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 - TT | 25.0 - TT |
| 23 | 31 | 0 | 2 | 2 | 0 | 29.0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 - TT | 26.0 - TT |
| 24 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 25 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 26 | 31 | 0 | 3 | 3 | 0 | 28.0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 - TT | 25.0 - TT |
| 27 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 28 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 29 | 31 | 0 | 3.5 | 3.5 | 0 | 27.5 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 - TT | 24.5 - TT |
| 30 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 31 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 32 | 31 | 0 | 6.5 | 6.5 | 0 | 24.5 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 - TT | 21.5 - TT |
| 33 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 34 | 31 | 0 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 24.0-26.5 | 24.0-22.5 | 2.0 | 3 | 33.0 + TT | (21.0 - TT)-(23.5 - TT) | (21.0 - TT)-(19.5 - TT) |
| 35 | 31 | 0 | 0 | 0 | 0 | 31.0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 - TT | 28.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-5: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.2.5-6: UE Power Class test requirements (for Bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n30, n31, n34, n38, n39, n40, n41, n50, n51, n53, n54, n65, n66, n70, n72, n74, n85, n91, n92, n93, n94, n100, n101, n106) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 23 | 0 | 1.5 | 1.5 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 15 | 23 | 0 | 1.5 | 1 | 0 | 1.52 | 20.5 | 192 | 2.5 | 3.52 | 2 | 25.0 + TT | 18 - TT | 15.5 - TT2 |
| 26 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 27 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 34 | 23 | 0 | 2 | 1.5 | 0 | 1.52 | 19.5 | 182 | 3.5 | 42 | 2 | 25.0 + TT | 16 - TT | 14 - TT2 |
| 35 | 23 | 0 | 2 | 1 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 46 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 47 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 56 | 23 | 0 | 3.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 2 | 25.0 + TT | 14 - TT | 11.5 - TT2 |
| 57 | 23 | 0 | 3.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 66 | 23 | 0 | 6.5 | 1.5 | 0 | 1.52 | 15 | 13.52 | 5 | 52 | 2 | 25.0 + TT | 10 - TT | 8.5- TT2 |
| 67 | 23 | 0 | 6.5 | 1 | 0 | 1.52 | 15.5 | 142 | 5 | 52 | 2 | 25.0 + TT | 10.5 - TT | 9 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25, n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-6a: Void

Table 6.2.2.5-7: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18 - TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5 - TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16 - TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5 - TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 5 | 2.5 | 25.0 + TT | 10 - TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 5 | 2.5 | 25.0 + TT | 10.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-7a: Void

Table 6.2.2.5-7b: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 24 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 43 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 44 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 53 | 23 | 0 | 4 | 1.5 | 0 | 17.5 | 5 | 2.5 | 25.0 + TT | 12.5 - TT |
| 54 | 23 | 0 | 4 | 1 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 63 | 23 | 0 | 7 | 1.5 | 0 | 14.5 | 5 | 2.5 | 25.0 + TT | 9.5 - TT |
| 64 | 23 | 0 | 7 | 1 | 0 | 15 | 5 | 2.5 | 25.0 + TT | 10 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW=30MHz when SCS=60kHz. NOTE 4: Applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz when SCS=30kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-8: UE Power Class test requirements (for Bands n48, n77, n78, n79) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17 - TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 3 | 25.0 + TT | 15.5 - TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16 - TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17 - TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 3 | 25.0 + TT | 15.5 - TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 3 | 25.0 + TT | 14 - TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 5 | 3 | 25.0 + TT | 10 - TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 5 | 3 | 25.0 + TT | 10.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-8a: Void

Table 6.2.2.5-9: UE Power Class test requirements (for Bands n1, n3, n34, n39, n40, n41, n77, n78, n79) for Power Class 2 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 26 | 0 | 1.5 | 1.5 | 0 | 1.52 | 23 | 21.52 | 2 | 22 | 3 | 28.0 + TT | 20 - TT | 18 - TT2 |
| 15 | 26 | 0 | 1.5 | 1 | 0 | 1.52 | 23.5 | 222 | 2 | 22 | 3 | 28.0 + TT | 20.5 - TT | 19.0 - TT2 |
| 26 | 26 | 0 | 3 | 1.5 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 28.0 + TT | 18.5 - TT | 17.0 - TT2 |
| 27 | 26 | 0 | 3 | 1 | 0 | 1.52 | 22 | 20.52 | 2 | 2.52 | 3 | 28.0 + TT | 19 - TT | 17.5 - TT2 |
| 34 | 26 | 0 | 2 | 1.5 | 0 | 1.52 | 22.5 | 212 | 2 | 22 | 3 | 28.0 + TT | 19.5 - TT | 18.0 - TT2 |
| 35 | 26 | 0 | 2 | 1 | 0 | 1.52 | 23 | 21.52 | 2 | 22 | 3 | 28.0 + TT | 20 - TT | 18.5 - TT2 |
| 46 | 26 | 0 | 3 | 1.5 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 28.0 + TT | 18.5 - TT | 17.0 - TT2 |
| 47 | 26 | 0 | 3 | 1 | 0 | 1.52 | 22 | 20.52 | 2 | 2.52 | 3 | 28.0 + TT | 19 - TT | 17.5 - TT2 |
| 56 | 26 | 0 | 3.5 | 1.5 | 0 | 1.52 | 21 | 19.52 | 2 | 3.52 | 3 | 28.0 + TT | 18 - TT | 16.0 - TT2 |
| 57 | 26 | 0 | 3.5 | 1 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 28.0 + TT | 18.5 - TT | 17.0 - TT2 |
| 66 | 26 | 0 | 6.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 3 | 28.0 + TT | 14 - TT | 11.5 - TT2 |
| 67 | 26 | 0 | 6.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 3 | 28.0 + TT | 14.5 - TT | 12.0 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n3 and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 8: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-9a: Void

Table 6.2.2.5-10: UE Power Class test requirements (for Band n40 with 100MHz channel bandwidth) for Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 1 | 0 | 25.0 | 2.0 | 3 | 28.0 + TT | 22.0 - TT |
| 2 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 3 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 4 | 26 | 0 | 1.5 | 0 | 24.5 | 2.0 | 3 | 28.0 + TT | 21.5 - TT |
| 5 | 26 | 0 | 1 | 0 | 25.0 | 2.0 | 3 | 28.0 + TT | 22.0 - TT |
| 6 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 7 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 8 | 26 | 0 | 2 | 0 | 24.0 | 2.0 | 3 | 28.0 + TT | 21.0 - TT |
| 9 | 26 | 0 | 2 | 0 | 24.0 | 2.0 | 3 | 28.0 + TT | 21.0 - TT |
| 10 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 11 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 12 | 26 | 0 | 3 | 0 | 23.0 | 2.0 | 3 | 28.0 + TT | 20.0 - TT |
| 13 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 14 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 15 | 26 | 0 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 16 | 26 | 0 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28.0 + TT | 17.5 - TT |
| 17 | 26 | 0 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28.0 + TT | 17.5 - TT |
| 18 | 26 | 0 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28.0 + TT | 17.5 - TT |
| 19 | 26 | 0 | 2.5 | 0 | 23.5 | 2.0 | 3 | 28.0 + TT | 20.5 - TT |
| 20 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 21 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 22 | 26 | 0 | 4 | 0 | 22.0 | 2.0 | 3 | 28.0 + TT | 19.0 - TT |
| 23 | 26 | 0 | 3 | 0 | 23.0 | 2.0 | 3 | 28.0 + TT | 20.0 - TT |
| 24 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 25 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 26 | 26 | 0 | 4 | 0 | 22.0 | 2.0 | 3 | 28.0 + TT | 19.0 - TT |
| 27 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 28 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 29 | 26 | 0 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28.0 + TT | 18.5 - TT |
| 30 | 26 | 0 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28.0 + TT | 14.5 - TT |
| 31 | 26 | 0 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28.0 + TT | 14.5 - TT |
| 32 | 26 | 0 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28.0 + TT | 14.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-11: UE Power Class test requirements (for Band n40 with 100MHz channel bandwidth) for Power Class 2 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 2.5 | 1.5 | 0 | 22 | 2 | 3 | 28.0 + TT | 19 - TT |
| 2 | 26 | 0 | 4 | 1.5 | 0 | 20.5 | 2.5 | 3 | 28.0 + TT | 17.5 - TT |
| 3 | 26 | 0 | 3 | 1.5 | 0 | 21.5 | 2 | 3 | 28.0 + TT | 18.5 - TT |
| 4 | 26 | 0 | 4 | 1.5 | 0 | 20.5 | 2.5 | 3 | 28.0 + TT | 17.5 - TT |
| 5 | 26 | 0 | 4.5 | 1.5 | 0 | 20 | 2.5 | 3 | 28.0 + TT | 17 - TT |
| 6 | 26 | 0 | 7.5 | 1.5 | 0 | 17 | 5 | 3 | 28.0 + TT | 12 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_07, NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same A-MPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, NS_07, NS_27 and NS_35 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and NR Power Class 1 UE release 15 and forward.

The requirements of this test apply in test case 6.5.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 3 UE release 15 and forward that don’t support Tx diversity.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_07, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_24, NS_27, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

NOTE:	Test execution is not necessary if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed.

6.2.3.3	Minimum conformance requirements

6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where CEIL{x, 0.5} means x rounding upwards to closest 0.5dB, NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 Table 6.2.2.3-1.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 8) | 3, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.3.7 |
| NS_03U | 6.5.2.3.3.3, 6.5.2.4.2.3 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.3.7 |
| NS_04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_05 | 6.5.3.3.3.4 | n1, n65, n84 (NOTE 1) | 5, 10, 15, 20 (Note 2) |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.3.4, 6.5.2.4.2.3 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_06 | 6.5.2.3.3.4 | n1213, n85 | 3, 5, 10, 15 |  | Clause 6.2.3.3.3212 |
|  |  | n14 | 5, 10 |  |  |
| NS_07 | 6.5.2.3.3.4, 6.5.3.3.5.26 | n13 | 5,10 | Table 6.2.3.3.29-1 | Table 6.2.3.3.29-2 |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3.3-1 | Table 6.2.3.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 | 3, 5, 10 | Table 6.2.3.3.21-1 | Table 6.2.3.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 3, 5 | Table 6.2.3.3.22-1 | Table 6.2.3.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10, 15, 20 | Table 6.2.3.3.23-1 | Table 6.2.3.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 3, 5, 10, 15, 20 | Table 6.2.3.3.24-1 | Table 6.2.3.3.24-2 |
| NS_17 | 6.5.3.3.3.2 | n28, n8313 | 3, 5, 10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3.3 | n28, n8313 | 3, 5 |  | Table 6.2.3.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.3.13-1, A2 |
|  |  |  | 30 |  | Table 6.2.3.3.13-1, A3, A4, A5 |
| NS_21 | 6.5.2.3.3.9, 6.5.3.3.3.12 | n30 | 5,10 |  | Clause 6.2.3.3.14 |
| NS_24 | 6.5.3.3.3.13 | n65 (Note 4) | 5, 10, 15, 20 | Table 6.2.3.3.15-1 | Subclause 6.2.3.3.15 |
| NS_27 | 6.5.2.3.3.8, 6.5.3.3.3.14 | n48 | 5, 10, 15, 20, 30, 40 | Table 6.2.3.3.16-1 | Table 6.2.3.3.16-2 |
| NS_35 | 6.5.2.3.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | Clause 6.2.3.3.3111 |
| NS_37 | 6.5.3.3.3.6 | n74 (Note 3) | 10, 15 | Table 6.2.3.3.8-1 | Table 6.2.3.3.8-1 |
| NS_38 | 6.5.3.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.3.9-1 | Table 6.2.3.3.9-1 |
| NS_39 | 6.5.3.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.3.10-1 | Table 6.2.3.3.10-1 |
| NS_40 | 6.5.3.3.3.9 | n51 | 5 |  | Table 6.2.3.3.5-1 |
| NS_41 | 6.5.3.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.11-1 |
| NS_42 | 6.5.3.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.12-1 |
| NS_43 | 6.5.3.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_43U | 6.5.3.3.3.5, 6.5.2.4.2.3 | n8, n81 (NOTE 1) | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25,30,40 | Table 6.2.3.3.20-1 | Table 6.2.3.3.20-1 |
| NS_45 | 6.5.3.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.3.25 |
| NS_46 | 6.5.3.3.3.25 | n7 | 25, 30, 40, 50 | Table 6.2.3.3.17-1 Table 6.2.3.3.17-311 | Table 6.2.3.3.17-2 Table 6.2.3.3.17-411 |
| NS_47 | 6.5.3.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.3.18-1 Table 6.2.3.3.18-3 | Table 6.2.3.3.18-2 Table 6.2.3.3.18-4 |
| NS_48 | 6.5.3.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.26-1, Table 6.2.3.3.26-3 | Table 6.2.3.3.26-2, Table 6.2.3.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.27-1 Table 6.2.3.3.27-3 | Table 6.2.3.3.27-2, Table 6.2.3.3.27-4 (NOTE 7) |
| NS_50 | 6.5.3.3.3.16 | n39 | 10, 15, 20, 25, 30, 35, 40 |  | Clause 6.2.3.3.19 |
| NS_55 | NOTE 6 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_56 | 6.5.3.3.3.27 | n24, n99 | 5, 10 | Table 6.2.3.3.30 | Clause 6.2.3.3.30 |
| NS_62 | 6.5.3.3.3.28 | n54 | 5 |  | N/A |
| NS_100 | 6.5.2.4.2.3 | n1, n2, n3, n5, n8, n25, n26, n66, n80, n81, n84, n86 (Note 1) |  |  | Table 6.2.3.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the upper channel edge is ≥ 1930 MHz, 10 MHz BWChannel where the upper channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the upper channel edge is ≥ 1955 MHz and 20 MHz BWChannel where the upper channel edge is ≥ 1970 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 [TS 38.306], the additional relaxation of [2] dB is applicable. NOTE 8: The NS_01 label with the field additionalPmax [6] absent is default for all NR bands. NOTE 9: Void. NOTE 10: FFS NOTE 11: Applicable only for power class 1 operation. NOTE 12: Applicable only for power class 1 operation on band n85. NOTE 13: 3 MHz channel bandwidth is not applicable. |  |  |  |  |  |

Table 6.2.3.3.1-1A: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n13 | NS_01 | NS_06 | NS_07 |  |  |  |  |  |
| n14 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n24 | NS_01 | NS_56 |  |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n31 | NS_01 |  |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n53 | NS_01 | NS_45 |  |  |  |  |  |  |
| n54 | NS_01 | NS_62 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U | NS_51 |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n72 | NS_01 |  |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 | NS_55 |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n85 | NS_01 | NS_06 |  |  |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n91 | NS_01 |  |  |  |  |  |  |  |
| n92 | NS_01 |  |  |  |  |  |  |  |
| n93 | NS_01 |  |  |  |  |  |  |  |
| n94 | NS_01 |  |  |  |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| n99 | NS_01 | NS_56 |  |  |  |  |  |  |
| n100 | NS_01 |  |  |  |  |  |  |  |
| n101 | NS_01 |  |  |  |  |  |  |  |
| n106 | NS_01 |  |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

Table 6.2.3.3.1-2: A-MPR for NS_100 (UTRA protection) (Power Class 3 and Power Class 2)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.1.

6.2.3.3.2	A-MPR for NS_04

For NS_04, A-MPR is not added to MPR. Also, when NS_04 is signalled, MPR shall be set to zero in the PCMAX equations to avoid double-counting MPR.

For power Class 1.5, 2 and 3, allowed maximum power reduction is defined as A-MPR=max(MPR, A-MPR'),

Note that A-MPR'=0 dB means only MPR is applied,

where A-MPR' is defined as

if RBstart ≤ fstart,max,IMD3 / (12SCS) and LCRB ≤ AWmax,IMD3 / (12SCS) and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the A-MPR' is defined according to Table 6.2.3.3.2-2 PC3_A2 relative to 23 dBm for power class 3, PC2 A4 relative to 26 dBm for power class 2, and PC1.5_A6 relative to 29 dBm for power class 1.5,

else,

if RBstart ≤ LCRB/2 + start / (12SCS) and LCRB ≤ AWmax,regrowth / (12SCS) and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the A-MPR' is defined according to Table 6.2.3.3.2-2 PC3_A1 relative to 23 dBm for power class 3, PC2 A3 relative to 26 dBm for power class 2, and PC1.5_A5 relative to 29 dBm for power class 1.5,

else

A-MPR' = 0 dB and apply MPR.

With the parameters defined in Table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Parameters for region edges and frequency offsets (Power Class 1.5, 2 and 3)

| Parameter | Symbol | Value |  | Related condition |
| --- | --- | --- | --- | --- |
|  |  | CP-OFDM | DFT-s-OFDM |  |
| Max allocation start in IMD3 region | fstart,max,IMD3 | 0.33 BWChannel |  | RBstart ≤ fstart,max,IMD3 / (12SCS) |
| Max allocation BW in IMD3 region | AWmax,IMD3 | 4 MHz |  | LCRB ≤ AWmax,IMD3 / (12SCS) |
| Freq. offset required to avoid A-MPR in IMD3 region | offsetIMD3 | BWChannel – 6 MHz |  | FC - BWChannel/2 ≥ FUL_low + offsetIMD3 |
| Right edge of regrowth region | start | 0.08 BWChannel |  | RBstart ≤ LCRB/2 + start / (12SCS) |
| Max allocation BW in regrowth region | AWmax,regrowth | 100 MHz |  | LCRB ≤ Min(LCRB,Max, AWmax,regrowth / (12SCS)) |
| Freq. offset required to avoid A-MPR in regrowth region | offsetregrowth | Max (10 MHz, 0.25* BWChannel MHz) | Max (10 MHz, 0.45* BWChannel MHz) | FC - BWChannel/2 ≥ FUL_low + offsetregrowth |

Table 6.2.3.3.2-2: A-MPR' values Access (Power Class 1.5, 2 and 3)

| Modulation/Waveform |  | A-MPR' (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PC3_A1 | PC3_A2 | PC2_A3 | PC2_A4 | PC1.5_A51 | PC1.5_A61 |
| DFT-s-OFDM | Pi/2-BPSK | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 5.5 | ≤ 5 | ≤ 7 |
|  | QPSK | ≤ 4 | ≤ 4 | ≤ 4.5 | ≤ 6 | ≤ 6 | ≤ 7.5 |
|  | 16-QAM | ≤ 4 | ≤ 4 | ≤ 5 | ≤ 6 | ≤ 6.5 | ≤ 7.5 |
|  | 64-QAM | ≤ 4 | ≤ 4.5 | ≤ 5 | ≤ 6.5 | ≤ 6.5 | ≤ 8 |
|  | 256-QAM | ≤ 4.5 | ≤ 6 | ≤ 6.5 | ≤ 8 | ≤ 8 | ≤ 9.5 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 16-QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 64-QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 256-QAM | ≤ 6.5 | ≤ 8 | ≤ 7.5 | ≤ 10 | ≤ 9 | ≤ 11.5 |
| NOTE 1: PC1.5 assumes dual Tx. |  |  |  |  |  |  |  |

For Power Class 1, NS_04 A-MPR is defined as 
A-MPR = max(MPR, A-MPRregrowth, A-MPRIMD3, A-MPRCIM3, A-MPRedge).

A-MPRregrowth is obtained from Table 6.2.3.3.2-3 in terms of total guard bandwidth (TGBW). The TGBW is defined as the frequency distance between the RB allocation and the additional spurious emission limit defined in Table 6.5.3.3.3.1-1, i.e.,

TGBW=falloc,low-2496 MHz,

where

falloc,low=FC-BWChannel2+BWGB+RBstart∙12 SCS

is the lower edge frequency of the RB allocation, FC is the channel centre frequency, BWChannel is the channel bandwidth, and BWGB is the minimum guard bandwidth defined in Table 5.3.3-1.

Table 6.2.3.3.2-3: A-MPRregrowth for NS_04 (Power Class 1)

| TGBW range | A-MPRregrowth *(dB) |
| --- | --- |
| G0dB(BWalloc) TGBW | 0 |
| G1dB(BWalloc) TGBW < G0dB(BWalloc) | 1 |
| G2dB(BWalloc) TGBW < G1dB(BWalloc) | 2 |
| G3dB(BWalloc) TGBW < G2dB(BWalloc) | 3 |
| G4dB(BWalloc) TGBW < G3dB(BWalloc) | 4 |
| G5dB(BWalloc) TGBW < G4dB(BWalloc) | 5 |
| TGBW < G5dB(BWalloc) | 6 |

Each function GA(BWalloc) defines the required minimum total guard bandwidth for A-MPR value A and is defined as

GA(BWalloc)=max⁡(0,  C2BWalloc100MHz2+C1BWalloc100MHz+C0),

where BWalloc=LCRB∙12 SCS is the allocation bandwidth, and C2, C1, and C0 are obtained from Table 6.2.3.3.2-4 for each combination of waveform, modulation, and back-off value A.

Table 6.2.3.3.2-4: Polynomial coefficients for determining the required 
total guard bandwidth for each value of A-MPRregrowth (Power Class 1)

| Waveform | Modulation | Back-off value A [dB] / Polynomial coefficients C2, C1, C0 [MHz] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| DFT-s-OFDM | Pi/2-BPSK | -3, 40, 10 | -91, 98, 0 | -248, 143, -9 | -2148, 604, -40 | 0,0,0 | N/A | N/A |
|  | QPSK | -27, 71, 1 | -49, 73, 0 | -76, 76, -3 | -108, 62, -3 | 0,0,0 | N/A | N/A |
|  | 16-QAM | -34, 81, -1 (NOTE 1) | -57, 92, -5 | -69, 82, -5 | -119, 79, -5 | 0,0,0 | N/A | N/A |
|  | 64-QAM | -23, 74, -1 (NOTE 1) | -61, 91, -4 (NOTE 1) | -84, 92, -5 | -116, 80, -5 | 0,0,0 | N/A | N/A |
|  | 256-QAM | 0, 0, 0 (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A |
| CP-OFDM | QPSK | -32, 81, -2 (NOTE 1) | -36, 77, -3 | -40, 70, -3 | -52, 67, -3 | -67, 60, -3 | -101, 56, -4 | 0,0,0 |
|  | 16-QAM | -31, 80, -2 (NOTE 1) | -32, 72, -2 (NOTE 1) | -36, 66, -2 | -48, 62, -2 | -66, 58, -2 | -108, 60, -4 | 0,0,0 |
|  | 64-QAM | -28, 78, -2 (NOTE 1) | -33, 73, -2 (NOTE 1) | -36, 66, -2 (NOTE 1) | -48, 62, -2 | -65, 58, -2 | -124, 67, -5 | 0,0,0 |
|  | 256-QAM | 0, 0, 0 (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: Back-off value A is less than floor(MPR), thus this polynomial is irrelevant. NOTE 2: For 256-QAM, always A-MPRregrowth = 0. |  |  |  |  |  |  |  |  |

For both OFDM and DFT-S-OFDM, A-MPRIMD3 = max0,  min3,  4-2.77 BWalloc1 MHz dB 
if  3falloc,low-2FC  2490.5 MHz; otherwise, A-MPRIMD3 = 0 dB.

For OFDM, A-MPRCIM3 = max0,  min11,  12-2 BWalloc1 MHz dB if  4 FC-3 falloc,high   2490.5 MHz; 
otherwise, A-MPRCIM3 = 0.

For DFT-S-OFDM, A-MPRCIM3 = 3 dB if BWalloc 1.08 MHz and 4 FC-3 falloc,high   2490.5 MHz; 
otherwise, A-MPRCIM3 = 0.

Here, falloc,high=falloc,low+BWalloc is the upper edge frequency of the RB allocation.

For both OFDM and DFT-S-OFDM, if RBstart=0, LCRB=1,  and FC-BWChannel2-2496 MHz<360 kHz, A-MPRedge is defined in Table 6.2.3.3.2-5. Otherwise, A-MPRedge = 0 dB.

Table 6.2.3.3.2-5: A-MPRedge for NS_04  (Power Class 1)

| Waveform | BWChannel [MHz] | BWalloc [MHz] | A-MPR [dB] |
| --- | --- | --- | --- |
| DFT-s-OFDM | < 40 | N/A | 0 |
|  | 40…90 | -9.8CBW100MHz2+24.1CBW100MHz-6.3 | 3.5 |
|  | 100 |  | 11 |
| OFDM | < 90 | N/A | 0 |
|  | 90 | 6.5 | 3.5 |
|  | 100 | 8 | 11 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.2.

6.2.3.3.3	A-MPR for NS_10

Table 6.2.3.3.3-1: A-MPR for NS_10

| Channel bandwidth [MHz] | Parameters | Region A |
| --- | --- | --- |
| 15 | RBstart | 0 – 10 |
|  | LCRB (RBs) | 1 –20 |
|  | A (dB) | ≤ 36 |
| 20 | RBstart | 0 – 15 |
|  | LCRB (RBs) | 1 – 20 |
|  | A (dB) | ≤ 66 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects Region A, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects Region A, notes 1 and 2 apply on a Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersect Region A, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the subslot. NOTE 5: The A-MPR for DFT-s-OFDM is the total backoff and is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3.3.1-1 and Table 6.2.4-1 in TS 36.521-1 [21] and A value specified in Table 6.2.3.3.3-1. NOTE 6: The A-MPR for CP-OFDM is the total backoff and is obtained by adding the A value in Table 6.2.3.3.3-1 to the corresponding MPR specified in Table 6.2.2.3-1. |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.3.

6.2.3.3.4	A-MPR for NS_05 and NS_05U

Table 6.2.3.3.4-1: A-MPR regions for NS_05 and NS_05U (Power Class 3)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 | 1922.5 ≤ FC < 1927.5 | <1.62MHz/12/SCS | >2.52MHz/12/SCS | A3 |  |  |  |  |  |  |
| 10 | 1925 ≤ FC < 1935 | <1.62MHz/12/SCS | >0 | A1 | >1.62MHz/12/SCS ≤3.60MHz/12/SCS | >5.4MHz/12/SCS | A7 | ≥7.2MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 10 | 1935 ≤ FC < 1945 |  | >4.5MHz/12/SCS | A4 |  |  |  |  |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | <3.24MHz/12/SCS | >0 | A1 | >3.24MHz/12/SCS ≤5.40MHz/12/SCS | >8.1MHz/12/SCS | A7 | ≥10.08MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 15 | 1932.5 ≤ FC < 1942.5 | <1.62MHz/12/SCS | >0 | A1 |  |  |  | ≥12.24 MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 15 | 1942.5 ≤ FC < 1947.5 |  | >7.2MHz/12/SCS | A5 |  |  |  |  |  |  |
| 20 | 1930 ≤ FC < 1950 | <4.86MHz/12/SCS | >0 | A1 | >4.86MHz/12/SCS ≤7.20MHz/12/SCS | >9.0MHz/12/SCS | A7 | ≥13.68 MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 20 | 1950 ≤ FC <1960 |  | >9.0MHz/12/SCS | A6 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.4-2, 6.2.3.3.4-3 and 6.2.3.3.4-4. NOTE 2: Void. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.3.4-2: A-MPR for NS_05 and NS_05U (Power Class 3)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 10 | ≤ 5 | ≤ 4 | N/A |
|  | QPSK | ≤ 10 | ≤ 5 | ≤ 4.5 |  |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 6 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 6 |  |
|  | 256 QAM | ≤ 13 | ≤ 5 | ≤ 7 |  |
| CP-OFDM | QPSK | ≤ 10 | ≤ 5 | ≤ 7.5 |  |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 8 |  |
|  | 256 QAM | ≤ 13 |  | ≤ 10 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Table 6.2.3.3.4-3: A-MPR for NS_05 (Power Class 3)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 1 | N/A | ≤ 1 | N/A | ≤ 6 |
|  | QPSK |  |  | ≤ 1.5 |  | ≤ 1.5 |  | ≤ 6 |
|  | 16 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 64 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 64 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

Table 6.2.3.3.4-4: A-MPR for modulation and waveform type for NS_05U (Power Class 3)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 2 | N/A | ≤ 2 | N/A | ≤ 6 |
|  | QPSK | ≤ 2 |  | ≤ 2 |  | ≤ 2 |  | ≤ 6 |
|  | 16 QAM | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 6 |
|  | 64 QAM | ≤ 3 |  | ≤ 3 |  | ≤ 3 |  | ≤ 6 |
|  | 256 QAM | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 6 |
| CP-OFDM | QPSK | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 16 QAM | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 64 QAM | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 256 QAM | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

Table 6.2.3.3.4-5: A-MPR regions for NS_05 and NS_05U (Power Class 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 | 1922.5 ≤ FC < 1927.5 | < 1.98 MHz/12/SCS | > 1.44 MHz/12/SCS | A3 | < 0.72 MHz/12/SCS | ≤ 1.44 MHz/12/SCS | A4 |  |  |  |
| 10 | 1925 ≤ FC < 1935 | < 1.98 MHz/12/SCS | > 0 | A1 | ≥ 1.98 MHz/12/SCS | > max(0, RBstart-1.08 MHz/12/SCS) | A7 | ≥ 7.2 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A1 |
|  |  |  |  |  | ≥ 1.98, ≤2.7 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A8 |  |  |  |
| 10 | 1935 ≤ FC < 1945 |  | > 3.96 MHz/12/SCS | A4 |  |  |  |  |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | < 3.6 MHz/12/SCS | > 0 | A1 | ≥ 3.6 MHz/12/SCS | > max(0, RBstart-1.8 MHz/12/SCS) | A7 | ≥ 10.08 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A1 |
|  |  |  |  |  | ≥ 3.6, ≤4.68 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A8 |  |  |  |
| 15 | 1932.5 ≤ FC < 1942.5 | < 1.98 MHz/12/SCS | > 0 | A1 | ≥ 1.98 MHz/12/SCS | > max(0, RBstart+1.08 MHz/12/SCS) | A7 | ≥ 12.24 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A1 |
| 15 | 1942.5 ≤ FC < 1947.5 |  | > 5.04 MHz/12/SCS | A5 |  |  |  |  |  |  |
| 20 | 1930 ≤ FC < 1950 | < 5.04 MHz/12/SCS | > 0 | A1 | ≥ 5.04 MHz/12/SCS | > max(0, RBstart-3.6 MHz/12/SCS) | A7 | ≥ 13.68 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A1 |
|  |  |  |  |  | ≥ 5.04, ≤6.66 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A8 |  |  |  |
| 20 | 1950 ≤ FC < 1960 |  | > 9.0 MHz/12/SCS | A6 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.4-6 and 6.2.3.3.4-7. NOTE 2: Void |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.3.4-6: A-MPR for NS_05 and NS_05U (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [13] | ≤ 6 | ≤ 6.5 | ≤ 2 |
|  | QPSK | ≤ [13] | ≤ 6 | ≤ 7 | ≤ 2 |
|  | 16 QAM | ≤ [13] | ≤ 6 | ≤ 8.5 | ≤ 2 |
|  | 64 QAM | ≤ [14] | ≤ 6 | ≤ 9 | ≤ 2 |
|  | 256 QAM | ≤ [15] | ≤ 6 | ≤ 9.5 |  |
| CP-OFDM | QPSK | ≤ [13] | ≤ 6 | ≤ 10 | ≤ 4 |
|  | 16 QAM | ≤ [13] | ≤ 6 | ≤ 10 | ≤ 4 |
|  | 64 QAM | ≤ [14] | ≤ 6 | ≤ 10 | ≤ 4 |
|  | 256 QAM | ≤ [16] |  | ≤ 10 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Table 6.2.3.3.4-7: A-MPR for NS_05 and NS_05U (Power Class 2)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) | A8 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3 | N/A | ≤ 2 |  | ≤ 2 | N/A | ≤ 8 | ≤ 3.5 |
|  | QPSK | ≤ 3 |  | ≤ 2 |  | ≤ 2 |  | ≤ 8 | ≤ 3.5 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 2.5 |  | ≤ 2 |  | ≤ 8 | ≤ 3.5 |
|  | 64 QAM | ≤ 3.5 |  | ≤ 2.5 |  |  |  | ≤ 8 | ≤ 3.5 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 8 |  |
| CP-OFDM | QPSK | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4 |  | ≤ 8.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4 |  | ≤ 8.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 5 |  | ≤ 5 | ≤ 5 | ≤ 4 |  | ≤ 8.5 |  |
|  | 256 QAM |  |  |  |  |  |  | ≤ 8.5 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.4.

6.2.3.3.5	A-MPR for NS_40

Table 6.2.3.3.5-1: A-MPR for NS_40

| Modulation/Waveform |  | A (dB) |  |
| --- | --- | --- | --- |
|  |  | Channel bandwidth: 5 MHz |  |
|  |  | Outer | Inner |
| DFT-s-OFDM | QPSK | ≤ 15.5 | ≤ 12 |
|  | 16 QAM | ≤ 14.5 | ≤ 11 |
|  | 64 QAM | ≤ 14.5 | ≤ 10 |
|  | 256 QAM | ≤ 12.5 | ≤ 7.5 |
| CP-OFDM | QPSK | ≤ 14.5 | ≤ 10 |
|  | 16 QAM | ≤ 14.5 | ≤ 10 |
|  | 64 QAM | ≤ 14 | ≤ 8 |
|  | 256 QAM | ≤ 11 | ≤ 5.5 |
| NOTE 1: The A-MPR for NS_40 is the total backoff and is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3.3.1-1 and Table 6.2.4-30a in TS 36.101 and MPR+A specified in Table 6.2.2.3-1 and Table 6.2.3.3.5-1. |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.5.

6.2.3.3.6	A-MPR for NS_43 and NS_43U

Table 6.2.3.3.6-1: A-MPR regions for NS_43 (Power class 3 and 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 2.7 MHz/12/SCS | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 7.2 MHz/12/SCS | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 8.1 MHz/12/SCS | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | < 1.8 MHz /12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
|  |  | > 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | < 7.2 MHz/12/SCS | A7 |
|  |  |  |  |  | ≥ 6.12 MHz/12/SCS ≤ 7.2 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.6-2. NOTE 2: 15 kHz SCS unless otherwise stated NOTE 3: Void |  |  |  |  |  |  |  |

Table 6.2.3.3.6-2: A-MPR for NS_43 (Power class 3)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | N/A | ≤ 1.5 | N/A |  | N/A |  | N/A |  | N/A | ≤ 9 |
|  | QPSK | ≤ 2 |  |  |  |  |  | ≤ 2.5 |  |  |  | ≤ 9 |
|  | 16 QAM |  |  |  |  |  |  |  |  | ≤ 2.5 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 2.5 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |
| CP-OFDM | QPSK | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 4 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |

Table 6.2.3.3.6-2a: A-MPR for NS_43 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) | A7 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A | ≤ 12 | ≤ 4 | 3 |
|  | QPSK | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  | ≤ 12 | ≤ 4 | 3 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  | ≤ 12 | ≤ 5 | 4 |
|  | 64 QAM | ≤ 3.5 |  | ≤ 4.5 |  | ≤ 5 |  |  |  |  |  | ≤ 12 | ≤ 5.5 | 5.5 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 12 | ≤ 6.5 | 5.5 |
| CP-OFDM | QPSK | ≤ 4 |  |  |  |  |  | ≤ 4 |  | ≤ 6 |  | ≤ 12 | ≤ 6 | 4.5 |
|  | 16 QAM | ≤ 4 |  |  |  |  |  | ≤ 4.5 |  | ≤ 6 |  | ≤ 12 | ≤ 6 | 5 |
|  | 64 QAM | ≤ 4 |  | ≤ 5 |  | ≤ 6.5 |  |  |  |  |  | ≤ 12 | ≤ 6.5 | 6 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 12 | ≤ 7 | 7 |

Table 6.2.3.3.6-3: Void

When NS_43U is signalled for 5 and 10 MHz channel bandwidths A-MPR is defined in Table 6.2.3.3.1-2 except for DFT-s-OFDM QPSK when LCRB > 5.4 MHz/12/SCS the A-MPR is 2.5 dB. For 15 MHz channel bandwidth Table 6.2.3.3.6-4 applies.

Table 6.2.3.3.6-4: A-MPR for NS_43U

| Modulation/Waveform |  | 15 MHz |
| --- | --- | --- |
|  |  | Outer / Inner (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9 |
|  | QPSK | ≤ 9 |
|  | 16 QAM | ≤ 9 |
|  | 64 QAM | ≤ 9 |
|  | 256 QAM | ≤ 9 |
| CP-OFDM | QPSK | ≤ 9 |
|  | 16 QAM | ≤ 9 |
|  | 64 QAM | ≤ 9 |
|  | 256 QAM | ≤ 9 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.6.

6.2.3.3.7	A-MPR for NS_03 and NS_03U

Table 6.2.3.3.7-1: A-MPR for NS_03

| Modulation/Waveform |  | Outer (dB) | Inner (dB) |
| --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | ≤ 1.5 | N/A |
|  | QPSK | ≤ 2 |  |
|  | 16 QAM | ≤ 3 |  |
|  | 64 QAM | ≤ 3.5 |  |
|  | 256 QAM | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 4 |  |
|  | 16 QAM | ≤ 4 |  |
|  | 64 QAM | ≤ 4.5 |  |
|  | 256 QAM | ≤ 7.5 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |

In case UE operates in a band where NS_03U applies and it receives additionalSpectrumEmission value of 3 then A-MPR values specified in Table 6.2.3.3.7-1 apply with an exception that DFT-s-OFDM Pi/2 BPSK A-MPR is 2 dB.

6.2.3.3.8	A-MPR for NS_37

Table 6.2.3.3.8-1: A-MPR regions for B11/B21 protection (NS_37) (1447.9 - 1462.9 MHz)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A (Outer/Inner) |  |  | Region B (Outer/Inner) |  |  | Region C (Outer/Inner) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 10 | 1452.9 < FC ≤1457.9 | ≥0 | > 7.2 MHz/12/SCS | ≤ A1 | N/A | N/A | N/A | N/A | N/A | N/A |
| 15 | FC =1455.4 | ≥0 | > 9.9 MHz/12/SCS | ≤ A1 | < 0.54 MHz/12/SCS | < 1.08 MHz/12/SCS | ≤ A2 | > 13.86 MHz/12/SCS | < 1.08 MHz/12/SCS | ≤ A2 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.8-2. NOTE 2: Void NOTE 3: Void NOTE 4: No A-MPR for SCS = 60 kHz for region B and C only. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.3.8-2: A-MPR for NS_37

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 3 |
|  | QPSK | ≤ 1.5 |  | ≤ 3 |
|  | 16 QAM | ≤ 2.5 |  | ≤ 3 |
|  | 64 QAM | ≤ 3 |  | ≤ 3 |
|  | 256 QAM |  |  |  |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3 |
|  | 64 QAM |  |  |  |
|  | 256 QAM |  |  |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.8.

6.2.3.3.9	A-MPR for NS_38

Table 6.2.3.3.9-1: A-MPR for EESS (NS_38) Protection (1430 – 1470 MHz)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A Outer/Inner |  |  | Region B Outer/Inner |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR (dB) | RBstart+LCRB | A-MPR (dB) |
| 5 | 1432.5 ≤ FC < 14347.5 | ≤ -1.8 MHz/12/SCS + LCRB /2 | > 3.6 MHz/12/SCS | ≤ 7 | ≤ 2.16 MHz/12/SCS | ≤ 5.5 |
| 10 | 1435 ≤ FC < 1442 | ≤ -1.8 MHz/12/SCS + LCRB/2 | > 3.6 MHz/12/SCS | ≤ 12 | ≤ 2.16 MHz/12/SCS | ≤ 9 |
| 15 | 1437.5 ≤ FC < 1447.5 | ≤ -1.8 MHz/12/SCS + LCRB/2 | > 3.6 MHz/12/SCS | ≤ 13 | ≤ 3.6 MHz/12/SCS | ≤ 10 |
| 20 | 1440 ≤ FC < 1450 | ≤ -1.8 MHz/12/SCS + LCRB/2 | > 3.6 MHz/12/SCS | ≤ 13 | ≤ 5.4 MHz/12/SCS | ≤ 10 |
| NOTE 1 - 4: Void |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.9.

6.2.3.3.10	A-MPR for NS_39

Table 6.2.3.3.10-1: A-MPR for own RX (NS_39) Protection (1440 – 1470 MHz)

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A (Outer/Inner) |  |
| --- | --- | --- | --- |
|  |  | RBstart+LCRB | A-MPR (dB) |
| 10 | 1460 < FC ≤1465 | > 7.9 MHz/12/SCS | ≤ 6 |
| 15 | 1452.5 < FC ≤ 1462.5 | > 11.2 MHz/12/SCS | ≤ 6 |
| 20 | 1450 < FC ≤ 1460 | > 14.4 MHz/12/SCS | ≤ 6 |
| NOTE 1 - 4: Void |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.10.

6.2.3.3.11	A-MPR for NS_41

Table 6.2.3.3.11-1: A-MPR for NS_41

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A (Outer/Inner) |  |  | Region B (Outer/Inner) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR (dB) | RBstart+LCRB | A-MPR (dB) |
| 5 | - | - | - | - | - | - |
| 10 | 1437 ≤ FC < 1442 | ≤ -4.5 MHz/12/SCS + LCRB | > 4.5 MHz/12/SCS | ≤ 9 | < 1.8MHz/12/SCS | ≤ 9 |
| 15 | 1439.5 ≤ FC < 1447.5 | ≤ -5.4 MHz/12/SCS + LCRB | > 5.4 MHz/12/SCS | ≤ 11 | < 3.42 MHz/12/SCS | ≤ 9 |
| 20 | 1442 ≤ FC < 1450 | ≤ -5.4 MHz/12/SCS + LCRB | > 5.4 MHz/12/SCS | ≤ 12 | < 5.04 MHz/12/SCS | ≤ 9 |
| 40 | 1452 ≤ FC < 1497 | ≤ -7.2 MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | < 11.7 MHz/12/SCS | ≤ 13.5 |
| 50 | 1457 ≤ FC < 1492 | ≤ -7.2 MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | < 15.12 MHz/12/SCS | ≤ 13.5 |
| 60 | 1462 ≤ FC < 1487 | ≤ -7.2 MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | < 18.72 MHz/12/SCS | ≤ 13.5 |
| NOTE 1 - 4: Void |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.11.

6.2.3.3.12	A-MPR for NS_42

Table 6.2.3.3.12-1: A-MPR for NS_42

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  | Region B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart+LCRB | A-MPR Outer/Inner (dB) | RBstart | RBstart+LCRB | A-MPR Inner (dB) | A-MPR Outer (dB) |
| 5 | 1512 ≤ FC ≤ 1514.5 | >3.1 MHz / 12 / SCS | ≤ 7 | <0.9 MHz / 12 / SCS | ≤ 3.1 MHz / 12 / SCS | ≤ 1.5 | ≤ 4 |
| 10 | 1497 ≤ FC ≤ 1512 | >6.2 MHz / 12 / SCS | ≤ 8 | <0.9 MHz / 12 / SCS | ≤ 6.2 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 15 | 1502 FC ≤ 1509.5 | >9.3 MHz / 12 / SCS | ≤ 8 | <3.06 MHz / 12 / SCS | ≤ 9.3 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 20 | 1497 ≤ FC ≤ 1507 | >12.4 MHz / 12 / SCS | ≤ 8 | <4.5 MHz / 12 / SCS | ≤ 12.4 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 40 | 1477 ≤ FC ≤ 1497 | >24.8 MHz / 12 / SCS | ≤ 8 | <5.4 MHz / 12 / SCS | ≤ 24.8 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 50 | 1467 ≤ FC ≤ 1492 | >31 MHz / 12 / SCS | ≤ 8 | <7.2 MHz / 12 / SCS | ≤ 31 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 60 | 1462 ≤ FC ≤ 1487 | >37.2 MHz / 12 / SCS | ≤ 8 | <7.2 MHz / 12 / SCS | ≤ 37.2 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| NOTE 1 - 5: Void |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.12.

6.2.3.3.13	A-MPR for NS_18

Table 6.2.3.3.13-0: Band n28 and n83 30MHz A-MPR regions for NS_18

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |

Table 6.2.3.3.13-1: A-MPR for NS_18

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 5 | 3 | 8 | 3 |
|  | QPSK | ≤ 2 |  | ≤ 5 | 3 | 8 | 3 |
|  | 16 QAM | ≤ 3 |  | ≤ 6 | 3 | 8 | 3 |
|  | 64 QAM | ≤ 4 |  | ≤ 7 | 3 | 8 | 4.5 |
|  | 256 QAM | ≤ 6 |  | ≤ 9 | 3 | 8 | 5.5 |
| CP-OFDM | QPSK | ≤ 5 |  | ≤ 6.5 | 4.5 | 9.5 | 5 |
|  | 16 QAM | ≤ 5 |  | ≤ 7 | 4.5 | 9.5 | 5 |
|  | 64 QAM | ≤ 5.5 |  | ≤ 8.5 | 4.5 | 9.5 | 5.5 |
|  | 256 QAM | ≤ 8.5 |  | ≤ 11.5 | 4.5 | 9.5 | 7.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.13.

6.2.3.3.14	A-MPR for NS_21

Table 6.2.3.3.14-1: A-MPR for "NS_21"

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 1.44MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS | Region A1b RBstart ≤ 1.44MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A2 LCRB > 5.4MHz/12/SCS | Region A3b RBend ≥ 7.74MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A3a RBend ≥ 7.74MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 10 | DFT-s-OFDM | PI/2 BPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | QPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | 16 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 64 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 256 QAM | 6 | 3 | 4 | 3 | 6 |
|  | CP-OFDM | QPSK | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 16 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 64 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 256 QAM | 6 | 4 | 5.5 | 4 | 6 |

Table 6.2.3.14-2: A-MPR for "NS_21" (applicable to UEs Release 17 and forward indicating modifiedMPR-Behavior)

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 0.36MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS | Region A1b RBstart ≤ 0.36MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A2 LCRB > 2.52MHz/12/SCS | Region A3b RBend ≥ 3.96MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A3a RBend ≥ 3.96MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 5 | DFT-s-OFDM | PI/2 BPSK | ≤ 4.0 | ≤ 2.0 | ≤ 1.5 | 2.0 | 4.0 |
|  |  | QPSK | ≤ 4.5 | ≤ 2.5 | ≤ 2.0 | ≤ 2.5 | ≤ 4.5 |
|  |  | 16 QAM | ≤ 4.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 4.5 |
|  |  | 64 QAM | ≤ 4.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 4.5 |
|  |  | 256 QAM | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 |
|  | CP-OFDM | QPSK | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 16 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 64 QAM | ≤ 4.5 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  |  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.14.

6.2.3.3.15	A-MPR for NS_24

Table 6.2.3.3.15-1: A-MPR for NS_24

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | Fc=1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | Fc=1997.5 |  | >3.24 | A4 |  |  |  |  |  |  |
| 5MHz | Fc=2002.5 |  | >2.16 | A1 | >3.78 | ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
| 10MHz | Fc=1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | Fc=1995 |  | >4.5 | A1 | >7.56 | ≤4.32 | A2 | ≤7.38 | ≤4.32 | A3 |
| 10MHz | Fc=2000 | >6.84 |  | A5 | <2.88 |  | A5 | ≥3.06 ≤6.66 | >1.44 | A6 |
| 15MHz | Fc=1987.5 |  | >7.02 | A1 | >11.52 | ≤6.84 | A2 | ≤11.34 | ≤6.84 | A3 |
| 15MHz | Fc=1997.5 | >9.36 |  | A5 | <3.6 |  | A5 | ≥3.78 ≤9.18 | >1.44 | A6 |
| 20MHz | Fc=1990 | >13.5 |  | A5 | <4.5 |  | A5 | ≥4.68 ≤13.32 | >2.16 | A6 |
| 20MHz | Fc=1995 | >12.6 |  | A5 | <5.4 |  | A5 | ≥5.58 ≤12.42 | >1.44 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.3.15-2: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 10 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 11 | ≤ 5 |  | ≤ 8.5 | ≤ 20 | ≤ 10 |  |
| CP-OFDM QPSK | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 13 | ≤ 6.5 |  | ≤ 8.5 | ≤ 20 | ≤ 12 |  |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2.3-1 NOTE 2: Outer and inner allocations are defined in clause 6.1 |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.15.

6.2.3.3.16	A-MPR for NS_27

Table 6.2.3.3.16-1: A-MPR for NS_27

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |  | Region B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | RBend*12*SCS | LCRB*12*SCS | A-MPR | LCRB*12*SCS | A-MPR |
| 15 MHz | 3557.5 ≤ FC < 3562.5 | <1.8 MHz |  |  | A3 | ≥10.8 MHz | A3 |
|  | 3687.5 < FC ≤ 3692.5 | >11.52 MHz |  |  |  |  |  |
| 15 MHz | 3562.5 ≤ FC < 3567.5 | ≤1.08 MHz |  | <1.44 MHz | A4 | ≥11.52 MHz | 2 |
|  | 3682.5 < FC ≤ 3687.5 |  | ≥13.22 MHz |  |  |  |  |
| 20 MHz | 3560 ≤ FC < 3570 | <3.6 MHz |  |  | A5 | ≥10.8 MHz | A5 |
|  | 3680 < FC ≤ 3690 | >12.96 MHz |  |  |  |  |  |
| 20 MHz | 3570 ≤ FC < 3580 | ≤2.16 MHz |  | <1.44 MHz | A6 | ≥14.4 MHz | 2 |
|  | 3670 < FC ≤ 3680 |  | ≥16.92 |  |  |  |  |
| 30 MHz | 3565 ≤ FC < 3585 | < 7.38MHz |  |  | A7 |  |  |
|  |  | ≥ 7.38MHz ≤24.48MHz |  | ≥ 15.3MHz | A2 |  |  |
|  |  |  |  | < 15.3 MHz | A1 |  |  |
|  |  | ≥24.48MHz |  | < 2.7 MHz | A7 |  |  |
|  | 3665 < FC ≤ 3685 |  | > 19.44 MHz |  | A7 |  |  |
|  |  |  | ≤19.44MHz ≥3.24 MHz | ≥ 15.3 MHz | A2 |  |  |
|  |  |  |  | < 15.3 MHz | A1 |  |  |
|  |  |  | <3.24 MHz | < 2.7MHz | A7 |  |  |
|  | 3585 ≤ FC ≤ 3665 | ≤[3.96] MHz |  | < 1.44MHz | A8 | ≥19.44 MHz | 4 |
|  |  |  | ≥24.48MHz |  | A8 |  |  |
| 40 MHz | 3570 ≤ FC < 3600 | <11.34 MHz |  |  | A7 |  |  |
|  |  | ≥11.34 MHz, ≤31.0 MHz |  | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  | >31.0 MHz |  | <3.6 MHz | A7 |  |  |
|  | 3650 < FC ≤ 3680 |  | >24.48 MHz |  | A7 |  |  |
|  |  |  | ≤24.48 MHz, ≥6.48 MHz | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  |  | <6.48 MHz | <3.6 MHz | A7 |  |  |
| 40 MHz | 3600 ≤ FC ≤ 3650 | ≤6.12 MHz |  | <1.44 MHz | A8 | >20 MHz | 4.5 |
|  |  |  | ≥ 32.76 |  |  |  |  |

Table 6.2.3.3.16-2: A-MPR for modulation and waveform type

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 6 | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | QPSK | 4.5 | 6 | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | 16 QAM | 4.5 | 6 | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 64 QAM | 4.5 | 6 | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 256 QAM |  | 6 |  |  |  |  | 11 |  |
| CP-OFDM | QPSK | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 16 QAM | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 64 QAM | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 256 QAM |  | 7 |  |  |  |  | 11.5 |  |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2.3-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2.3 |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.16.

6.2.3.3.17	A-MPR for NS_46

Table 6.2.3.3.17-1: A-MPR regions for NS_46 (Power class 3)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2534.5 ≤ FC ≤ 2557.5 |  | Note 1 | A3 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.14 | >0 | A4 |
|  |  | ≥4.14, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
| NOTE 1: > 9.72 MHz for DFT-s-OFDM, > 16.02 MHz for CP-OFDM. |  |  |  |  |

Table 6.2.3.3.17-2: A-MPR for NS_46 (Power class 3)

| Modulation/Waveform |  | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | QPSK | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 16 QAM | 4.5 | 5 | 2 | 3.5 | 6 | 10 |
|  | 64 QAM | 4.5 | 5 |  | 3.5 | 6 | 10 |
|  | 256 QAM |  |  |  |  | 6 | 10 |
| CP-OFDM | QPSK | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 16 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 64 QAM | 6 | 5 | 3.5 | 5.5 | 7 | 11 |
|  | 256 QAM | 6 |  |  |  | 7 | 11 |

Table 6.2.3.3.17-3: A-MPR regions for NS_46 (Power class 1)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 15 MHz | Fc > 2559.5 | ≥ 11.8 | ≥ -1.8*12*SCS*RBend + 32.5 | A1 |
|  |  | ≥ 13.3 | ≤ 1.62 | A2 |
| 20 MHz | Fc > 2547.5 | ≥ 15.3 | ≥ -1.5*12*SCS*RBend + 36.5 | A1 |
|  |  | ≥ 15 | ≤ 5.7 | A2 |
| 25 MHz | Fc > 2547.5 | ≥ 17 | ≥ -1.5*12*SCS*RBend + 43.5 | A3 |
|  |  | ≥ 18.5 | ≤ 6.7 | A2 |
|  | 2535.5 < Fc ≤ 2547.5 | ≥ 21.6 | ≥ 21.6 | A5 |
|  |  | ≥ 20.5 | ≤ 5.6 | A2 |
| 30 MHz | 2523 < Fc ≤ 2545 | ≥ 22.5 | ≥ -0.7*12*SCS*RBend + 41.0 | A5 |
|  |  | ≥ 23.4 | ≤ 7.2 | A2 |
|  | Fc > 2545 | ≥ 20.0 | ≥ -1.5*12*SCS*RBend + 50.5 | A8 |
|  |  | ≥ 21.6 | ≤ 7.2 | A9 |
|  |  | ≤ 9.6 | ≥ 1.1*12*SCS*RBend - 1.5 | A7 |
| 35 MHz | Fc ≤ 2532.5 | ≥ 28 | ≤ 7.2 | A2 |
|  | 2532.5 < Fc ≤ 2542.5 | ≥ 27.7 | ≥ -1.2*12*SCS*RBend + 59.4 | A4 |
|  |  | ≥ 24.5 | ≤ 7.2 | A2 |
|  | Fc > 2542.5 | ≤ 18.4 | ≥ 1.15*12*SCS*RBend – 3.4 | A7 |
|  |  | ≥ 21.6 | ≥ 9, ≥ -1.2*12*SCS*RBend + 47 | A8 |
|  |  | ≥ 23 | < 9 | A9 |
| 40 MHz | Fc > 2540 | ≤ 24 | ≥ 1.15*12*SCS*RBend - 6 | A7 |
|  |  | > 24, < 35 | ≥ -0.75*12*SCS*RBend + 40.5 | A9 |
|  |  | ≥ 34 | > 7.2 | A9 |
|  |  | ≥ 27 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 27 | ≤ 2.2 | A9 |
|  | 2530 < Fc ≤ 2540 | ≤ 10.8 | ≥ 1.2*12*SCS*RBend – 2 | A7 |
|  |  | ≥ 29 | ≤ 7.2 | A9 |
|  |  | ≥ 28 | ≥ -1*12*SCS*RBend + 56 | A10 |
|  | Fc ≤ 2530 | ≥ 30 | ≤ 7.2 | A2 |
| 50 MHz | Fc > 2535 | ≤ 19 | ≥ 1.17*12*SCS*RBend – 10 | A7 |
|  |  | > 19, ≤ 29.5 | ≥ 1.17*12*SCS*RBend – 10 | A11 |
|  |  | > 29.5 | > 12.8, ≥ -1.15*12*SCS*RBend + 60 | A9 |
|  |  | > 41 | > 7.2, ≤ 12.8 | A12 |
|  |  | ≥ 33 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 33 | ≤ 2.2 | A9 |
|  | 2525 < Fc ≤ 2535 | ≤ 14 | ≥ 1.17*12*SCS*RBend – 6 | A7 |
|  |  | > 14, ≤ 26 | ≥ 1.17*12*SCS*RBend – 6 | A11 |
|  |  | ≥ 34 | > 22.5, ≥ -1.25*12*SCS*RBend + 77.5 | A8 |
|  |  | ≥ 43 | > 7.2, ≤ 22.5 | A12 |
|  |  | ≥ 35 | > 2.2, ≤ 7.2 | A8 |
|  |  | ≥ 35 | ≤ 2.2 | A9 |
|  | Fc = 2525 | ≥ 47 | ≥ -1*12*SCS*RBend + 94 | A13 |
|  |  | ≤ 9 | ≥ 1.14*12*SCS*RBend – 2 | A7 |
|  |  | ≥ 37.4 | > 2.2, ≤ 7.6 | A8 |
|  |  | ≥ 37.4 | ≤ 2.2 | A9 |

Table 6.2.3.3.17-4: A-MPR regions for NS_46 (Power class 1)

| Modulation/ Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 | A11 | A12 | A13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 2 |  | 2 | 1 | 1 | 3 | 9.5 | 5 | 6 | 4 |  | 4 | 2.5 |
|  | QPSK | 3 | 4 | 3 | 2 | 2 | 6 | 11 | 6 | 6 | 5 |  | 4 | 4 |
|  | 16 QAM | 3 | 6.5 | 3 |  |  | 6 | 12 | 6 | 6.5 | 5 |  | 4 | 4 |
|  | 64 QAM | 3 | 7 | 3 |  |  | 6 | 12.5 | 6 | 7 | 5 |  | 4 | 4 |
|  | 256 QAM |  | 7 |  |  |  | 6 | 12.5 | 6 | 7 | 5 |  |  |  |
| CP-OFDM | QPSK | 4 | 12.5 | 6 | 4 |  | 6.5 | 15 | 9 | 12.5 | 5.5 | 10 | 5 | 4 |
|  | 16 QAM | 4 | 12.5 | 6 | 4 |  | 6.5 | 15 | 9 | 13 | 5.5 | 10 | 5 | 4 |
|  | 64 QAM | 4 | 13 | 6 |  |  | 6.5 | 15 | 9 | 13 | 5.5 | 10 | 5 | 4 |
|  | 256 QAM |  | 13 |  |  |  |  | 15 | 9 | 13 |  | 10 |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.17.

6.2.3.3.18	A-MPR for NS_47

Table 6.2.3.3.18-1: A-MPR regions for NS_47 (Power Class 2 and 3)

| Channel Bandwidth, (MHz) | Carrier Centre Frequency, Fc, (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 30MHz | Fc=2560-2560.020 | ≤5.04 | ≤1.44 | A1 |
|  |  | >5.04, ≤9.6 | ≤1.44 | A2 |
|  |  | >24.48 | ≤1.44 | A3 |
|  |  | ≤9.6 | >21 | A2 |
|  |  |  | >14.4, <21 | A4 |
|  |  | ≤6.12 | >10, ≤14.4 | A4 |
|  |  |  | >1.44, <10 | A2 |
| NOTE: The A-MPR values are listed in Table 6.2.3.3.18-2. |  |  |  |  |

Table 6.2.3.3.18-2: A-MPR for modulation and waveform type (Power Class 2 and 3)

| Modulation/Waveform | A1(dB) |  | A2(dB) |  | A3(dB) |  | A4(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 |
|  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 |  | ≤ 6 |
| CP-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 |  | ≤ 7 |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  |  |  | ≤ 7 |

Table 6.2.3.3.18-3: A-MPR regions and types for NS_47 (Power Class 1.5)

| Channel Bandwidth, (MHz) | Carrier Centre Frequency, Fc, (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 30MHz | Fc=2560-2560.020 | ≤5.04 | ≤1.44 | A1 |
|  |  | >5.04, ≤9.6 | ≤1.44 | A2 |
|  |  | >24.48 | ≤1.44 | A3 |
|  |  | ≤9.6 | >21 | A2 |
|  |  |  | >14.4, <21 | A4 |
|  |  | >6.12, ≤7.92 | >10, ≤14.4 | A5 |
|  |  | ≤6.12 | >10, ≤14.4 | A4 |
|  |  |  | >1.44, <10 | A2 |
| NOTE: The A-MPR values are listed in Table 6.2.3.3.18-4. |  |  |  |  |

Table 6.2.3.3.18-4: A-MPR for NS_47 (Power Class 1.5)

| Modulation/Waveform |  | A1(dB) | A2(dB) | A3(dB) | A4(dB) | A5(dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 | ≤ 3 |
|  | QPSK | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 | ≤ 3 |
|  | 16 QAM | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 | ≤ 3 |
|  | 64 QAM | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 |  |
|  | 256 QAM | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 |  |
| CP-OFDM | QPSK | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 | ≤ 4 |
|  | 16 QAM | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 | ≤ 4 |
|  | 64 QAM | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 |  |
|  | 256 QAM | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 |  |
| NOTE 1: PC1.5 assumes dual Tx. |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.18.

6.2.3.3.19	A-MPR for NS_50

Table 6.2.3.3.19-1: A-MPR regions for NS_50 (Power Class 3)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 25 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 6.48 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 30 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 8.64 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 35 MHz | ≤ 1.62 | > 0 | A1 |
|  | > 1.62, ≤ 9.36 | ≤ 9.0 | A3 |
|  | > 1.62, ≤ 13.68 | > 9.0 | A2 |
|  | > 13.68, ≤ 30.6 | > max (29.7 –RBstart *12*SCS, 0) | A6 |
|  | > 30.6 | > 0 | A5 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32, ≤ 10.44 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8 | A2 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A6 |
|  | > 31.68 | > 0 | A5 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.19-2. |  |  |  |

Table 6.2.3.3.19-2: A-MPR for NS_50 (Power Class 3)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 2 |  |
|  | QPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2 |  |
|  | 16 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 |  | ≤ 5 |  |  |
|  | 256 QAM | ≤ 11 | ≤ 7 |  | ≤ 5 |  | ≤ 5 |  |  |
| CP-OFDM | QPSK | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  | ≤ 3.0 |
|  | 16 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  | ≤ 3.0 |
|  | 64 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 |  | ≤ 6.5 |  |  |
|  | 256 QAM | ≤ 12 | ≤ 8 |  |  |  | ≤ 6.5 |  |  |

Table 6.2.3.3.19-3: A-MPR regions for NS_50 (Power Class 2)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10 MHz | ≤ 1.44 | < 1.44 | A5 |
|  | ≤ 1.8 | ≥ 2.7+2* RBstart*12*SCS | A4 |
|  | >1.8 | ≥ 8.1- RBstart*12*SCS | A4 |
| 15 MHz | ≤ 2.88 | < 2.7 | A5 |
|  | ≤ 3.24 | ≥ 2.7+2* RBstart*12*SCS | A3 |
|  | >3.24 | ≥ 12.42- RBstart*12*SCS | A4 |
| 20 MHz | ≤ 4.32 | < 3.6 | A5 |
|  | ≤ 4.5 | ≥ 3.6+2* RBstart*12*SCS | A3 |
|  | >4.5 | ≥ 17.1- RBstart*12*SCS | A4 |
| 25 MHz | ≤ LCRB*12*SCS – 5 | > 5 | A2 |
|  | ≤ 6.48 | ≤ 1.44 | A5 |
|  | > 8.28 | > max (21.6 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | >1.8, ≤6.48 | > 1.44, ≤ 3.6 | A6 |
|  | > LCRB *12*SCS – 5, ≤ 1.8 | > 1.44 | A4 |
| 30 MHz | ≤ LCRB*12*SCS – 5 | >5 | A2 |
|  | ≤ 7.56 | ≤ 1.44 | A5 |
|  | >1.8, ≤7.56 | > 1.44, ≤ 3.6 | A6 |
|  | ≤ 1.8 | >1.44, <RBstart*12*SCS+5 | A4 |
|  | > 10.8 | > max (26.64 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
| 35 MHz | ≤ 1.62 | > 0 | A1 |
|  | > 1.62, ≤ 9.36 | ≤ 9.0 | A3 |
|  | > 1.62, ≤ 13.68 | > 9.0 | A8 |
|  | > 13.68, ≤ 30.6 | > max (29.7 –RBstart *12*SCS, 0) | A4 |
|  | > 30.6 | > 0 | A9 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32 | > RBstart*12*SCS + 11.88 | A8 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8, <= RBstart*12*SCS + 11.88 | A7 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A4 |
|  | > 31.68 | > 0 | A9 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.19-4. |  |  |  |

Table 6.2.3.3.19-4: A-MPR for NS_50 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer /Inner | Outer /Inner | Outer /Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11.5 | ≤4.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | QPSK | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | 16 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1.5 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | 64 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 |  | ≤ 4 |  |  | ≤ 10 | ≤ 8 |
|  | 256 QAM | ≤ 11.5 | ≤ 5.5 |  |  |  |  |  | ≤ 10 | ≤ 8 |
| CP-OFDM | QPSK | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 | ≤ 2 | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 16 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 64 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 256 QAM | ≤ 12.5 | ≤ 7 |  |  |  |  |  | ≤ 11 | ≤ 8 |

6.2.3.3.20	A-MPR for NS_44

Table 6.2.3.3.20-1: A-MPR regions for NS_44

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 2582.5≤ FC ≤ 2602.5 | <18.0 | >max(0, 12*SCS* RBend - 3.6) | A3 |
|  |  | ≥18.0 | <7.2 | A3 |
|  |  | ≥18.0 | ≥7.2 | A6 |
| 30 MHz | 2585 ≤ FC ≤ 2600 | <21.6 | >max(0, 12*SCS* RBend - 3.6) | A3 |
|  |  | ≥21.6 | <12.6 | A3 |
|  |  | ≥21.6 | ≥12.6 | A6 |
| 40 MHz | 2590 ≤ FC ≤ 2595 | ≥0, <2.88 | >0 | A1 |
|  |  | ≥2.88, <14.4 | >max (0, 12*SCS*RBend - 3.6) | A2 |
|  |  | ≥14.4, <23.4 | >10.8 | A3 |
|  |  | ≥23.4, <32.4 | >16.2 | A4 |
|  |  | ≥32.4 | >0 | A5 |

Table 6.2.3.3.20-2: A-MPR for NS_44

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 5 | 2 | 3 | 7 | 12 | 4 |
|  | QPSK | 5 | 2 | 3 | 7 | 12 | 4 |
|  | 16 QAM | 5 | 2 | 3 | 7 | 12 | 4 |
|  | 64 QAM | 5 |  | 3 | 7 | 12 | 4 |
|  | 256 QAM | 5 |  |  | 7 | 12 |  |
| CP-OFDM | QPSK | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 16 QAM | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 64 QAM | 5 | 4 | 5 | 8 | 12 | 6 |
|  | 256 QAM |  |  |  | 8 | 12 |  |

6.2.3.3.21	A-MPR for NS_12

Table 6.2.3.3.21-1: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Table 6.2.3.3.21-2: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |

6.2.3.3.22	A-MPR for NS_13

Table 6.2.3.3.22-1: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A4 |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |

Table 6.2.3.3.22-2: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |

6.2.3.3.23	A-MPR for NS_14

Table 6.2.3.3.23-1: A-MPR regions for NS_14

| Channel BW | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10MHz | ≤0.18 | <1.08 | A1 |
|  | ≥0 | ≥9 | A2 |
| 15MHz | ≤1.8 | <1.8 | A1 |
|  | ≥0 | ≥9 | A2 |
| 20MHz | ≤3.42 | <1.8 | A3 |
|  | ≥0 | ≥9 | A2 |

Table 6.2.3.3.23-2: A-MPR for NS_14

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 3 |  | ≤ 3 |
| DFT-s-OFDM 256 QAM |  |  | ≤ 8 |
| CP-OFDM QPSK | ≤ 5 | ≤ 4 | ≤ 5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 4 | ≤ 5 |
| CP-OFDM 64 QAM | ≤ 6 |  | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 8 |  | ≤ 8 |

6.2.3.3.24	A-MPR for NS_15

Table 6.2.3.3.24-1: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.44 | >0 | A5 |
|  |  | ≤ 0.18 | ≤ 0.36 | A6 |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |
| 15MHz | 837.5 < Fc ≤ 841.5 | ≥9.36 | >1.08 | A1 |
|  |  | ≥9.36 | ≤1.08 | A4 |
|  |  | <9.36, ≥4.68 | ≥3.6 | A2 |
|  |  | <3.96 | ≤0.36 | A3 |
|  | 831.5 < Fc ≤ 837.5 | ≥10.8 | >1.08 | A1 |
|  |  | ≥10.8 | ≤1.08 | A4 |
|  |  | <10.8, ≥6.48 | ≥3.6 | A2 |
|  |  | <2.7 | ≤0.36 | A3 |
|  | Fc ≤ 831.5 | ≥13.14 | >0 | A1 |
|  |  | <13.14, ≥7.92 | ≥3.6 | A2 |
|  |  | <0.72 | ≤0.36 | A3 |
| 20MHz | 835 < Fc ≤ 839 | ≥12.24 | >1.08 | A1 |
|  |  | ≥12.24 | ≤1.08 | A4 |
|  |  | <12.24, ≥8.46 | ≥5.4 | A2 |
|  |  | <5.58 | ≤0.36 | A3 |
|  | Fc ≤ 835 | ≥13.68 | >1.08 | A1 |
|  |  | ≥13.68 | ≤1.08 | A4 |
|  |  | <13.68, ≥8.46 | ≥5.4 | A2 |
|  |  | <4.32 | ≤0.36 | A3 |

Table 6.2.3.3.24-2: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  |

6.2.3.3.25	A-MPR for NS_45

Table 6.2.3.3.25-1: A-MPR for NS_45

| Modulation/Waveform |  | Outer |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.5 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |

6.2.3.3.26	A-MPR for NS_48

Table 6.2.3.3.26-1: A-MPR regions for NS_48 (Power Class 3)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A3 |
|  |  | ≥18.72 | <1.<1.08 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0 | ≥13.5 | A3 |
|  |  | ≥21.6 | <1.08<1.08 | A5 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <2.88 | ≥0 | A2 |
|  |  | ≥2.88, <17.1 | ≥max (0, 12*SCS*RBend - 3.6) | A3 |
|  |  | ≥17.1, <27.36 | ≥13.5 | A4 |
|  |  | ≥27.36, <34.56 | ≥13.5 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A3 |
|  |  | ≥34.56 | ≥0 | A1 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥0, <4.86 | >0 | A2 |
|  |  | ≥4.86, <19 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥19, <37.44 | ≥15.4 | A2 |
|  |  | ≥30.96, <37.44 | <1.08 | A5 |
|  |  | ≥37.44 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <6.12 | >0 | A2 |
|  |  | ≥6.12, <20.7 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥20.7, <41.04 | ≥17.1 | A2 |
|  |  | ≥33.84, <41.04 | <1.08 | A5 |
|  |  | ≥41.04 | >0 | A1 |

Table 6.2.3.3.26-2: A-MPR for NS_48 (Power Class 3)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | QPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 16 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 64 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 256 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
| CP-OFDM | QPSK | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 16 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 64 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 256 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |

Table 6.2.3.3.26-3: A-MPR regions for NS_48 (Power Class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 1925≤ FC ≤ 1975 | ≥0 | ≥8.1 | A6 |
|  |  | <1.8 | ≥0 | A6 |
| 15 MHz | 1927.5≤ FC ≤ 1972.5 | ≥0 | ≥9 | A6 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 2.88) <9 | A6 |
| 20 MHz | 1930≤ FC ≤ 1970 | ≥0 | ≥9.72 | A4 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 3.6) <9.72 | A6 |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A4 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≥0 | ≥max (0, 12*SCS*RBend - 1.08) < 9.72 | A4 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0 | ≥12.96 | A4 |
|  |  | ≥21.6 | <1.08 | A5 |
|  |  | ≥0 | ≥max (0, 12*SCS*RBend - 3.6) < 12.96 | A4 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <2.88 | ≥0 | A1 |
|  |  | ≥2.88, <17.1 | ≥max (0, 12*SCS*RBend - 4.68) | A3 |
|  |  | ≥17.1, <27.36 | ≥12.96 | A4 |
|  |  | ≥27.36, <34.56 | ≥12.96 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A3 |
|  |  | ≥34.56 | ≥0 | A1 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥0, <5.22 | >0 | A1 |
|  |  | ≥5.22, <19 | ≥max (0, 12*SCS*RBend - 5.4) | A4 |
|  |  | ≥19, <37.44 | ≥14.04 | A2 |
|  |  | ≥30.96, <37.44 | <1.08 | A5 |
|  |  | ≥37.44 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <7.2 | >0 | A1 |
|  |  | ≥7.2, <20.7 | ≥max (0, 12*SCS*RBend – 5.4) | A4 |
|  |  | ≥20.7, <41.04 | ≥15.12 | A2 |
|  |  | ≥33.84, <41.04 | <1.08 | A5 |
|  |  | ≥41.04 | >0 | A1 |

Table 6.2.3.3.26-4: A-MPR for NS_48 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | QPSK | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | 16 QAM | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | 64 QAM | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | 256 QAM | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
| CP-OFDM | QPSK | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |
|  | 16 QAM | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |
|  | 64 QAM | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |
|  | 256 QAM | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |

6.2.3.3.27	A-MPR for NS_49

Table 6.2.3.3.27-1: A-MPR regions for NS_49

| Channel Bandwidth (MHz) | Carrier Center Frequency, Fc (MHz) | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) |  |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A3 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≤3.96 | <1.08 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0, <3.6 | ≥0 | A1 |
|  |  | ≥3.6, <6.48 | ≥0 | A5 |
|  |  | ≥6.48, <14.4 | ≥max (0,12*SCS* RBend - 3.6) | A3 |
|  |  | ≥14.4, <21.6 | ≥10.8 | A4 |
|  |  | ≥21.6 | ≥10.8 | A2 |
|  |  | ≥21.6 | <1.08 | A5 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <7.2 | ≥0 | A1 |
|  |  | ≥7.2, <10.44 | <1.08 | A5 |
|  |  | ≥7.2, <18 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥18, <34.56 | ≥14.4, <28.8 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A5 |
|  |  | <34.56 | ≥28.8 | A1 |
|  |  | ≥34.56 | ≥0 | A1 |
|  |  | ≥6.12, <12.42 | < min [1.08, max(0,12*SCS* RBend-6.12)] | A5 |
|  |  | ≥30.76, <36.72 | <1.08 | A5 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | <36.72 | ≥16.2, <max (0, 12*SCS*RBend – 6.12) | A2 |
|  |  | <36.72 | ≥max (0, 12*SCS*RBend – 6.12) | A1 |
|  |  | ≥36.72 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥7.74, <14.4 | < min (1.08, max(0,12*SCS* RBend-7.74)) | A5 |
|  |  | ≥36, <39.6 | <1.08 | A5 |
|  |  | <39.6 | ≥18, <max (0, 12*SCS*RBend – 7.74) | A2 |
|  |  | <39.6 | ≥max (0, 12*SCS*RBend – 7.74) | A1 |
|  |  | ≥39.6 | >0 | A1 |

Table 6.2.3.3.27-2: A-MPR for NS_49

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | QPSK | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 16 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 64 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
|  | 256 QAM | ≤10 | ≤6 | ≤3 | ≤4 | ≤5 |
| CP-OFDM | QPSK | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 16 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 64 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |
|  | 256 QAM | ≤11 | ≤7 | ≤4.5 | ≤5.5 | ≤5 |

Table 6.2.3.3.27-3: A-MPR regions for NS_49 (Power Class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 1925≤ FC ≤ 1975 | ≥0 | ≥8.1 | A3 |
|  |  | <1.8 | ≥0 | A3 |
| 15 MHz | 1927.5≤ FC ≤ 1972.5 | ≥0 | ≥9 | A3 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 2.88) <9 | A3 |
| 20 MHz | 1930≤ FC ≤ 1970 | ≥0 | ≥9.72 | A4 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 3.6) <9.72 | A3 |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥7.92 | A4 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≤6.48 | <3.6 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0, <3.6 | ≥0 | A1 |
|  |  | ≥3.6, <7.92 | ≥0 | A5 |
|  |  | ≥7.92, <14.4 | ≥max (0,12*SCS* RBend - 4.32) | A3 |
|  |  | ≥14.4, <21.6 | ≥10.44 | A4 |
|  |  | ≥21.6 | ≥10.44 | A2 |
|  |  | ≥21.6 | <1.8 | A1 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <9 | ≥0 | A1 |
|  |  | ≥9, <11.52 | <3.06 | A5 |
|  |  | ≥9, <18 | ≥max (3.06, 12*SCS*RBend – 6.48) | A4 |
|  |  | ≥18, <34.56 | ≥11.16, <27 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A5 |
|  |  | <34.56 | ≥27 | A1 |
|  |  | ≥34.56 | ≥0 | A1 |
|  |  | ≥7.92, <12.42 | < min [1.08, max(0,12*SCS* RBend-7.92)] | A5 |
|  |  | ≥30.76, <36.72 | <1.08 | A5 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | <36.72 | ≥12.24, <max (0, 12*SCS*RBend – 7.92) | A2 |
|  |  | <36.72 | ≥max (0, 12*SCS*RBend – 7.92) | A1 |
|  |  | ≥36.72 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥10.08, <14.4 | < min [1.08, max(0,12*SCS* RBend-10.08)] | A5 |
|  |  | ≥36, <39.6 | <1.08 | A5 |
|  |  | <39.6 | ≥13.68, <max (0, 12*SCS*RBend – 10.08) | A2 |
|  |  | <39.6 | ≥max (0, 12*SCS*RBend – 10.08) | A1 |
|  |  | ≥39.6 | >0 | A1 |

Table 6.2.3.3.27-4: A-MPR for NS_49 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | QPSK | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | 16 QAM | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | 64 QAM | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | 256 QAM | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
| CP-OFDM | QPSK | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |
|  | 16 QAM | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |
|  | 64 QAM | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |
|  | 256 QAM | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |

6.2.3.3.28	A-MPR for NS_51

Table 6.2.3.3.28: void

6.2.3.3.29	A-MPR for NS_07

Table 6.2.3.3.29-1: A-MPR regions for NS_07

| Channel Bandwidth, MHz | Carrier Frequency, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5 MHz | 782 ≤ Fc ≤ 784.5 | ≥0 | ≥ 1.8 | A3 |
| 5 MHz | 779.5 ≤ Fc < 782 | ≤ 0.9 | ≥ 0 | A1 |
|  |  | > 0.9, ≤ 1.26 | ≥1.26 | A2 |
|  |  | > 1.26, ≤ 3.42 | ≥ 1.8 | A3 |
|  |  | >3.42 | ≤ 0.36 | A4 |
| 10 MHz | Fc = 782 | ≤ 2.34 | ≥ 0 | A1 |
|  |  | >2.34, ≤ 3.24 | ≥ 1.44 | A2 |
|  |  | > 3.24, ≤ 6.48 | ≥ 3.24 | A3 |
|  |  | > 6.48 | ≤ 0.36 | A4 |

Table 6.2.3.3.29-2: A-MPR for NS_07

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | 12 | 9 | 6 | 3 |
| DFT-s-OFDM QPSK | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 16 QAM | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 64 QAM | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 256 QAM | 12 | 9 | 6 | 3 |
| CP-OFDM QPSK | 14 | 10 | 7 | 3 |
| CP-OFDM 16 QAM | 14 | 10 | 7 | 3 |
| CP-OFDM 64 QAM | 14 | 10 | 7 | 3 |
| CP-OFDM 256 QAM | 14 | 10 | 7 | 3 |

6.2.3.3.30	A-MPR for NS_56

For 5 MHz channel centred on frequencies (FC) = 1630.0, 1630.3 MHz, A-MPR is defined as

if RBstart <= ceil{3/SCS/15 kHz)}and LCRB <= ceil{17/SCS/15 kHz)},

then

the A-MPR = 14 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz,

else,

if RBstart <= ceil{3/(SCS/15 kHz)} and LCRB > ceil{17/(SCS/15 kHz)},

then

the A-MPR = 6 dB,

else,

if RBstart <= ceil{8/(SCS/15 kHz)},

then

the A-MPR = 4 dB.

For 5 MHz channel centred on frequencies (Fc) = 1635.0, 1649.0, 1654.0 MHz, no A-MPR is needed.

For Channel 10 MHz with centre frequency of 1632.5 MHz, A-MPR is defined as

if RBstart < ceil{3/(SCS/15 kHz)} and LCRB <= ceil{8/(SCS/15 kHz)},

then

the A-MPR = 12 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz,

else,

if RBstart < ceil{9/(SCS/15 kHz)}, and LCRB > ceil{8/(SCS/15 kHz)},

then

the A-MPR = 8 dB,

else,

if RBstart <= ceil{18/(SCS/15 kHz)},

then

the A-MPR = 6 dB,

else,

if RBstart >= floor{40/(SCS/15 kHz)}], and LCRB <= ceil{7/(SCS/15 kHz)},

then

the A-MPR = 5 dB,

else,

if RBstart >= floor{40/(SCS/15 kHz)} and LCRB > ceil{7/(SCS/15 kHz)},

then

the A-MPR = 3 dB,

else,

if RBstart >= floor{35/(SCS/15 kHz)} and LCRB <= ceil{7/(SCS/15 kHz)},

then

the A-MPR = 4 dB,

else,

if RBstart >= floor{35/(SCS/15 kHz)} and LCRB > ceil{7/(SCS/15 kHz)},

then

the A-MPR = 2 dB.

For 10 MHz channel centered on frequency of 1651.5 MHz, no A-MPR is needed.

6.2.3.3.31	A-MPR for NS_35

For power class 1 operation A-MPR = 8.5 dB if

( LCRB ≤ 0.20 ∙ NRB and ( RBstart = 0 or RBstart + LCRB = NRB ) )

or

( LCRB = 1 and 5 ∙ | RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz ).

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.31.

6.2.3.3.32	A-MPR for NS_06

For power class 1 operation on band n85 A-MPR = 8.5 dB if

( LCRB ≤ 0.20 ∙ NRB and ( RBstart = 0 or RBstart + LCRB = NRB ) )

or

( LCRB = 1 and 5 ∙ | RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz ).

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.32.

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2.3.4.1-1 to 6.2.3.4.1-30. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Network signalling value NS_10 is not tested. The relaxation specified under NS_10 is for the purpose to control / mitigate self-desensitization of the UE’s own receiver, but no corresponding REFSENS requirements are specified. There is no need to test A-MPR requirements for its own.

Table 6.2.3.4.1-1: Test Configuration table for NS_06 and NS_35

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_06 and NS_35 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq | ChBw | SCS | N/A for A-MPR testing. | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 | Low | Default | Default |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 3 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 14 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 15 | Default |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 16 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 17 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 18 | Default |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 21 | Default |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 22 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 23 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 24 | Default |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 25 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 26 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 27 | Default |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void NOTE 4: This configuration is only applicable for NS_06 power class 1 UE. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-2: Test Configuration table for NS_04 (Power Class 1.5, 2 and 3)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 (Note 3, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 3 (Note 3) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 4 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 5 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 6 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 7 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 8 (Note 4, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 9 (Note 4) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 10 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 11 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 12 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 14 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 16 | Low |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 17 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 18 | High |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 22 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 23 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 25 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 26 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 27 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 28 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 29 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 30 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 31 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 32 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 33 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 34 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 35 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 36 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 37 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 38 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 39 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 40 | Low |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 41 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 42 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 43 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 44 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 45 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 46 | Low |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 47 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 48 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 49 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |

| A-MPR test parameters for NS_04 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 50 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 51 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 52 | Low |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 53 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 54 | High |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 55 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 56 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 57 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 58 | Low |  |  |  | CP-OFDM 256 QAM | Outer Full |
| 59 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 60 | High |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Only applicable for 10 MHz and 15 MHz channel bandwidth |  |  |  |  |  |  |

Table 6.2.3.4.1-2a: Additional test frequencies for NS_04 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 |  |  | 2505 | 501000 | 2500.32 | 500064 | 0 | 15 | 6255 | 500430 | 2 | 0 | 0 (0) | 0 |
| 15 | 79 |  |  | 2512.5 | 502500 | 2505.39 | 501078 | 0 | 15 | 6270 | 501630 | 4 | 1 | 2 (4) | 5 |

zz

Table 6.2.3.4.1-2b: Additional test frequencies for NS_04 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 |  |  | 2505 | 501000 | 2500.68 | 500136 | 0 | 30 | 6261 | 500910 | 18 | 0 | 0 (0) | 0 |
| 15 | 38 |  |  | 2512.5 | 502500 | 2505.66 | 501132 | 0 | 30 | 6273 | 501870 | 6 | 0 | 0 (0) | 0 |

Table 6.2.3.4.1-2c: Additional test frequencies for NS_04 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 |  |  | 2505 | 501000 | 2501.04 | 500208 | 0 | 15 | - | 500568 |
| 15 | 18 |  |  | 2512.5 | 502500 | 2506.02 | 501204 | 0 | 15 | - | 501564 |

Table 6.2.3.4.1-2d: Additional test frequencies for NS_04 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 |  |  | 2511 | 502200 | 2506.32 | 501264 | 0 | 15 | 6270 | 501630 | 2 | 0 | 0 (0) | 0 |
| 15 | 79 |  |  | 2513.505 | 502701 | 2506.395 | 501279 | 0 | 15 | 6273 | 501870 | 5 | 2 | 2 (4) | 6 |
| 20 | 106 |  |  | 2520 | 504000 | 2510.46 | 502092 | 0 | 15 | 6282 | 502590 | 10 | 1 | 1 (2) | 3 |
| 30 | 160 |  |  | 2535 | 507000 | 2520.6 | 504120 | 0 | 15 | 6306 | 504510 | 10 | 0 | 0 (0) | 0 |
| 40 | 216 |  |  | 2550 | 510000 | 2530.56 | 506112 | 0 | 15 | 6333 | 506670 | 6 | 1 | 2 (4) | 5 |
| 50 | 270 |  |  | 2565 | 513000 | 2540.7 | 508140 | 0 | 15 | 6357 | 508590 | 6 | 0 | 1 (2) | 2 |

Table 6.2.3.4.1-2e: Additional test frequencies for NS_04 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 |  |  | 2511 | 502200 | 2506.68 | 501336 | 0 | 30 | 6276 | 502110 | 18 | 0 | 0 (0) | 0 |
| 15 | 38 |  |  | 2513.52 | 502704 | 2506.68 | 501336 | 0 | 30 | 6276 | 502110 | 18 | 0 | 0 (0) | 0 |
| 20 | 51 |  |  | 2520 | 504000 | 2510.82 | 502164 | 0 | 30 | 6288 | 503070 | 14 | 0 | 2 (2) | 4 |
| 30 | 78 |  |  | 2535 | 507000 | 2520.96 | 504192 | 0 | 30 | 6312 | 504990 | 2 | 0 | 1 (1) | 2 |
| 40 | 106 |  |  | 2550 | 510000 | 2530.92 | 506184 | 0 | 30 | 6336 | 506910 | 2 | 0 | 0 (0) | 0 |
| 50 | 133 |  |  | 2565 | 513000 | 2541.06 | 508212 | 0 | 30 | 6363 | 509070 | 22 | 0 | 1 (1) | 2 |
| 60 | 162 |  |  | 2580 | 516000 | 2550.84 | 510168 | 0 | 30 | 6387 | 510990 | 10 | 0 | 1 (1) | 2 |
| 70 | 189 |  |  | 2595 | 519000 | 2560.98 | 512196 | 0 | 30 | 6414 | 513150 | 6 | 0 | 3 (3) | 6 |
| 80 | 217 |  |  | 2610 | 522000 | 2570.94 | 514188 | 0 | 30 | 6438 | 515070 | 6 | 0 | 2 (2) | 4 |
| 90 | 245 |  |  | 2625 | 525000 | 2580.9 | 516180 | 0 | 30 | 6462 | 516990 | 6 | 0 | 1 (1) | 2 |
| 100 | 273 |  |  | 2640 | 528000 | 2590.86 | 518172 | 0 | 30 | 6486 | 518910 | 6 | 0 | 0 (0) | 0 |

Table 6.2.3.4.1-2f: Additional test frequencies for NS_04 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 |  |  | 2511 | 502200 | 2507.04 | 501408 | 0 | 15 | - | 501768 |
| 15 | 18 |  |  | 2513.52 | 502704 | 2507.04 | 501408 | 0 | 15 | - | 501768 |
| 20 | 24 |  |  | 2520 | 504000 | 2511.36 | 502272 | 0 | 15 | - | 502632 |
| 30 | 38 |  |  | 2535 | 507000 | 2521.32 | 504264 | 0 | 15 | - | 504624 |
| 40 | 51 |  |  | 2550 | 510000 | 2531.64 | 506328 | 0 | 15 | - | 506688 |
| 50 | 65 |  |  | 2550 | 510000 | 2526.6 | 505320 | 0 | 15 | - | 505680 |
| 60 | 79 |  |  | 2580 | 516000 | 2551.56 | 510312 | 0 | 15 | - | 510672 |
| 70 | 93 |  |  | 2595 | 519000 | 2561.52 | 512304 | 0 | 15 | - | 512664 |
| 80 | 107 |  |  | 2610 | 522000 | 2571.48 | 514296 | 0 | 15 | - | 514656 |
| 90 | 121 |  |  | 2625 | 525000 | 2581.44 | 516288 | 0 | 15 | - | 516648 |
| 100 | 135 |  |  | 2640 | 528000 | 2591.4 | 518280 | 0 | 15 | - | 518640 |

Table 6.2.3.4.1-2g: Test Configuration table for NS_04 (Power Class 1)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low, High |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_04 for PC1 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq | Channel BW |  | SCS | N/A for A-MPR testing | Modulation (NOTE 2) | RB allocation (NOTE 1) |  |
| 1 (Note 3) | Low | Default |  | Default |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |  |
| 2 (Note 3) | Low | Default |  | Default |  | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Left |  |
| 3 (Note 3) | High | Default |  | Default |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |  |
| 4 (Note 3) | High | Default |  | Default |  | DFT-s-OFDM PI/2 BPSK | Outer Full |  |
| 5 (Note 3, 4) | Low | 10 MHz |  | Default |  | DFT-s-OFDM PI/2 BPSK | Outer Full |  |
| 6 (Note 3, 4) | Low | 100 MHz |  | 30 kHz |  | DFT-s-OFDM PI/2 BPSK | 135@33 |  |
| 7 (Note 3, 4) | Low | 100 MHz |  | 60 kHz |  | DFT-s-OFDM PI/2 BPSK | 64@16 |  |
| 8 | Low | Default |  | Default |  | DFT-s-OFDM QPSK | Edge_1RB_Left |  |
| 9 | Low | Default |  | Default |  | DFT-s-OFDM QPSK | Inner_1RB_Left |  |
| 10 | High | Default |  | Default |  | DFT-s-OFDM QPSK | Edge_1RB_Right |  |
| 11 | High | Default |  | Default |  | DFT-s-OFDM QPSK | Outer Full |  |
| 12 (Note 4) | Low | 10 MHz |  | Default |  | DFT-s-OFDM QPSK | Outer Full |  |
| 13 (Note 4) | Low | 100 MHz |  | 30 kHz |  | DFT-s-OFDM QPSK | 135@33 |  |
| 14 (Note 4) | Low | 100 MHz |  | 60 kHz |  | DFT-s-OFDM QPSK | 64@16 |  |
| 15 | Low | Default |  | Default |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |  |
| 16 | Low | Default |  | Default |  | DFT-s-OFDM 16 QAM | Inner_1RB_Left |  |
| 17 | High | Default |  | Default |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |  |
| 18 | High | 100 MHz |  | 30 kHz |  | DFT-s-OFDM 16 QAM | 50@200 |  |
| 19 (Note 4) | Low | 10 MHz |  | Default |  | DFT-s-OFDM 16 QAM | Outer Full |  |
| 20 (Note 4) | Low | 100 MHz |  | 30 kHz |  | DFT-s-OFDM 16 QAM | 135@33 |  |
| 21 (Note 4) | Low | 100 MHz |  | 60 kHz |  | DFT-s-OFDM 16 QAM | 64@16 |  |
| 22 | Low | Default |  | Default |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |  |
| 23 | Low | Default |  | Default |  | DFT-s-OFDM 64 QAM | Inner_1RB_Left |  |
| 24 | High | Default |  | Default |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |  |
| 25 | High | 100 MHz |  | 30 kHz |  | DFT-s-OFDM 16 QAM | 50@200 |  |
| 26 (Note 4) | Low | 10 MHz |  | Default |  | DFT-s-OFDM 64 QAM | Outer Full |  |
| 27 (Note 4) | Low | 100 MHz |  | 30 kHz |  | DFT-s-OFDM 64 QAM | 135@33 |  |
| 28 (Note 4) | Low | 100 MHz |  | 60 kHz |  | DFT-s-OFDM 64 QAM | 64@16 |  |
| 29 | Low | Default |  | Default |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |  |
| 30 | Low | Default |  | Default |  | DFT-s-OFDM 256 QAM | Outer Full |  |
| 31 | High | Default |  | Default |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |  |
| 32 | High | Default |  | Default |  | DFT-s-OFDM 256 QAM | Outer Full |  |
| 33 (Note 4) | Low | 10 MHz |  | Default |  | DFT-s-OFDM 256 QAM | Outer Full |  |
| 34 (Note 4) | Low | 100 MHz |  | 30 kHz |  | DFT-s-OFDM 256 QAM | 135@33 |  |
| 35 (Note 4) | Low | 100 MHz |  | 60 kHz |  | DFT-s-OFDM 256 QAM | 64@16 |  |
| 36 | Low | Default |  | Default |  | CP-OFDM QPSK | Edge_1RB_Left |  |
| 37 | Low | Default |  | Default |  | CP-OFDM QPSK | Outer Full |  |
| 38 | High | 100 MHz |  | 30 kHz |  | CP-OFDM 16 QAM | 50@200 |  |
| 39 (Note 4) | Low | 10 MHz |  | Default |  | CP-OFDM QPSK | Outer Full |  |
| 40 (Note 4) | Low | 100 MHz |  | 30 kHz |  | CP-OFDM QPSK | 135@33 |  |
| 41 (Note 4) | Low | 100 MHz |  | 60 kHz |  | CP-OFDM QPSK | 64@16 |  |
| 42 | Low | Default |  | Default |  | CP-OFDM 16 QAM | Edge_1RB_Left |  |
| 43 | Low | Default |  | Default |  | CP-OFDM 16 QAM | Outer Full |  |
| 44 | High | 100 MHz |  | 30 kHz |  | CP-OFDM 16 QAM | 50@200 |  |
| 45 (Note 4) | Low | 10 MHz |  | Default |  | CP-OFDM 16 QAM | Outer Full |  |
| 46 (Note 4) | Low | 100 MHz |  | 30 kHz |  | CP-OFDM 16 QAM | 135@33 |  |
| 47 (Note 4) | Low | 100 MHz |  | 60 kHz |  | CP-OFDM 16 QAM | 64@16 |  |
| 48 | Low | Default |  | Default |  | CP-OFDM 64 QAM | Edge_1RB_Left |  |
| 49 | Low | Default |  | Default |  | CP-OFDM 64 QAM | Outer Full |  |
| 50 | High | 100 MHz |  | 30 kHz |  | CP-OFDM 16 QAM | 50@200 |  |
| 51 (Note 4) | Low | 10 MHz |  | Default |  | CP-OFDM 64 QAM | Outer Full |  |
| 52 (Note 4) | Low | 100 MHz |  | 30 kHz |  | CP-OFDM 64 QAM | 135@33 |  |
| 53 (Note 4) | Low | 100 MHz |  | 60 kHz |  | CP-OFDM 64 QAM | 64@16 |  |
| 54 | Low | Default |  | Default |  | CP-OFDM 256 QAM | Edge_1RB_Left |  |
| 55 | Low | Default |  | Default |  | CP-OFDM 256 QAM | Outer Full |  |
| 56 | High | Default |  | Default |  | CP-OFDM 256 QAM | 50@200 |  |
| 57 (Note 4) | Low | 10 MHz |  | Default |  | CP-OFDM 256 QAM | Outer Full |  |
| 58 (Note 4) | Low | 100 MHz |  | 30 kHz |  | CP-OFDM 256 QAM | 135@33 |  |
| 59 (Note 4) | Low | 100 MHz |  | 60 kHz |  | CP-OFDM 256 QAM | 64@16 |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: Applicable for requirement in regrowth region A-MPRregrowth |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-3: Test Configuration table for NS_03, NS_03U and NS_100

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_03, NS_03U and NS_100 |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
|  |  |  |  | N/A for A-MPR test cases | DFT-s-OFDM |  |  |
| 1 | Low | Default | Default |  |  | PI/2 BPSK | Edge_1RB_Left |
| 2 | High | Default | Default |  |  | PI/2 BPSK | Edge_1RB_Right |
| 3 | Default | Default | Default |  |  | PI/2 BPSK | Outer_Full |
| 4 | Low | Default | Default |  |  | QPSK | Edge_1RB_Left |
| 5 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 6 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 7 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 8 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 9 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 10 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 11 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 12 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 13 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 14 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |
| 15 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| 16 | Low | Default | Default |  | CP-OFDM | QPSK | Edge_1RB_Left |
| 17 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 18 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 19 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 20 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 21 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 22 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 23 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 24 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 25 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 26 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |
| 27 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void. NOTE 4: Void |  |  |  |  |  |  |  |

Table 6.2.3.4.1-4: Test Configuration table for NS_05

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |  |
| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 1 | 1922.5 | 5 | 15 | N/A for A-MPR testing | A3 | DFT-s-OFDM | PI/2 BPSK |  | Outer_Full |  |  |  |
| 2 | 1925 | 10 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 3 | 1925 | 10 | Default |  | A7 |  | PI/2 BPSK |  | 40@10 | 18@5 | 8@3 |  |
| 4 | 1925 | 10 | Default |  | A2 |  | PI/2 BPSK |  | 6@40 | 3@20 | 1@10 |  |
| 5 | 1935 | 10 | Default |  | A4 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 6 | 1927.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 7 | 1927.5 | 15 | Default |  | A7 |  | PI/2 BPSK |  | 60@19 | 27@10 | 12@5 |  |
| 8 | 1927.5 | 15 | Default |  | A2 |  | PI/2 BPSK |  | 6@56 | 3@28 | 1@14 |  |
| 9 | 1932.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 10 | 1932.5 | 15 | Default |  | A2 |  | PI/2 BPSK |  | 6@68 | 3@34 | 1@17 |  |
| 11 | 1942.5 | 15 | Default |  | A5 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 12 | 1930 | 20 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 13 | 1930 | 20 | Default |  | A7 |  | PI/2 BPSK |  | 72@28 | 36@14 | 16@7 |  |
| 14 | 1930 | 20 | Default |  | A2 |  | PI/2 BPSK |  | 6@76 | 3@38 | 1@19 |  |
| 15 | 1950 | 20 | Default |  | A6 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 16 | 1922.5 | 5 | 15 |  | A3 |  | QPSK |  | Outer_Full |  |  |  |
| 17 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 18 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 40@10 | 18@5 |  | 8@3 |
| 19 | 1925 | 10 | Default |  | A2 |  | QPSK |  | 6@40 | 3@20 |  | 1@10 |
| 20 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 21 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 60@19 | 27@10 |  | 12@5 |
| 22 | 1927.5 | 15 | Default |  | A2 |  | QPSK |  | 6@56 | 3@28 |  | 1@14 |
| 23 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 24 | 1932.5 | 15 | Default |  | A2 |  | QPSK |  | 6@68 | 3@34 |  | 1@17 |
| 25 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |  |
| 26 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 27 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 72@28 | 36@14 |  | 16@7 |
| 28 | 1930 | 20 | Default |  | A2 |  | QPSK |  | 6@76 | 3@38 |  | 1@19 |
| 29 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |  |
| 30 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |  |
| 31 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 32 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 40@10 | 18@5 |  | 8@3 |
| 33 | 1925 | 10 | Default |  | A2 |  | 16 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 34 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 35 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 60@19 | 27@10 |  | 12@5 |
| 36 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 37 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 38 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@68 | 3@34 |  | 1@17 |
| 39 | 1930 | 20 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 40 | 1930 | 20 | Default |  | A7 |  | 16 QAM |  | 72@28 | 36@14 |  | 16@7 |
| 41 | 1930 | 20 | Default |  | A2 |  | 16 QAM |  | 6@76 | 3@38 |  | 1@19 |
| 42 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM |  | Outer_Full |  |  |  |
| 43 | 1925 | 10 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 44 | 1925 | 10 | Default |  | A7 |  | 64 QAM |  | 40@10 | 18@5 |  | 8@3 |
| 45 | 1925 | 10 | Default |  | A2 |  | 64 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 46 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 47 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM |  | 60@19 | 27@10 |  | 12@5 |
| 48 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 49 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 50 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@68 | 3@34 |  | 1@17 |

| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 51 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 52 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 72@28 | 36@14 |  | 16@7 |
| 53 | 1930 | 20 | Default |  | A2 |  | 64 QAM | 6@76 | 3@38 |  | 1@19 |
| 54 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |  |
| 55 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 56 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 40@10 | 18@5 |  | 8@3 |
| 57 | 1925 | 10 | Default |  | A2 |  | 256 QAM | 6@40 | 3@20 |  | 1@10 |
| 58 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 59 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 60@19 | 27@10 |  | 12@5 |
| 60 | 1927.5 | 15 | Default |  | A2 |  | 256 QAM | 6@56 | 3@28 |  | 1@14 |
| 61 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 62 | 1932.5 | 15 | Default |  | A2 |  | 256 QAM | 6@68 | 3@34 |  | 1@17 |
| 63 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 64 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 72@28 | 36@14 |  | 16@7 |
| 65 | 1930 | 20 | Default |  | A2 |  | 256 QAM | 6@76 | 3@38 |  | 1@19 |
| 66 | 1922.5 | 5 | 15 |  | A3 | CP-OFDM | QPSK | Outer_Full |  |  |  |
| 67 | 1925 | 10 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 68 | 1925 | 10 | Default |  | A7 |  | QPSK | 42@10 | 18@5 |  | 8@3 |
| 69 | 1925 | 10 | Default |  | A2 |  | QPSK | 6@40 | 3@20 |  | 1@10 |
| 70 | 1935 | 10 | Default |  | A4 |  | QPSK | Outer_Full |  |  |  |
| 71 | 1927.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 72 | 1927.5 | 15 | Default |  | A7 |  | QPSK | 60@19 | 28@10 |  | 12@5 |
| 73 | 1927.5 | 15 | Default |  | A2 |  | QPSK | 6@56 | 3@28 |  | 1@14 |
| 74 | 1932.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 75 | 1932.5 | 15 | Default |  | A2 |  | QPSK | 6@68 | 3@34 |  | 1@17 |
| 76 | 1942.5 | 15 | Default |  | A5 |  | QPSK | Outer_Full |  |  |  |
| 77 | 1930 | 20 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 78 | 1930 | 20 | Default |  | A7 |  | QPSK | 78@28 | 37@14 |  | 17@7 |
| 79 | 1930 | 20 | Default |  | A2 |  | QPSK | 6@76 | 3@38 |  | 1@19 |
| 80 | 1950 | 20 | Default |  | A6 |  | QPSK | Outer_Full |  |  |  |
| 81 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM | Outer_Full |  |  |  |
| 82 | 1925 | 10 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 83 | 1925 | 10 | Default |  | A7 |  | 16 QAM | 42@10 | 18@5 |  | 8@3 |
| 84 | 1925 | 10 | Default |  | A2 |  | 16 QAM | 6@40 | 3@20 |  | 1@10 |
| 85 | 1935 | 10 | Default |  | A4 |  | 16 QAM | Outer_Full |  |  |  |
| 86 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 87 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM | 60@19 | 28@10 |  | 12@5 |
| 88 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM | 6@56 | 3@28 |  | 1@14 |
| 89 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 90 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM | 6@68 | 3@34 |  | 1@17 |
| 91 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM | Outer_Full |  |  |  |
| 92 | 1930 | 20 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 93 | 1930 | 20 | Default |  | A7 |  | 16 QAM | 78@28 | 37@14 |  | 17@7 |
| 94 | 1930 | 20 | Default |  | A2 |  | 16 QAM | 6@76 | 3@38 |  | 1@19 |
| 95 | 1950 | 20 | Default |  | A6 |  | 16 QAM | Outer_Full |  |  |  |
| 96 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM | Outer_Full |  |  |  |
| 97 | 1925 | 10 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 98 | 1925 | 10 | Default |  | A7 |  | 64 QAM | 42@10 | 18@5 |  | 8@3 |
| 99 | 1925 | 10 | Default |  | A2 |  | 64 QAM | 6@40 | 3@20 |  | 1@10 |
| 100 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 101 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM | 60@19 | 28@10 |  | 12@5 |
| 102 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM | 6@56 | 3@28 |  | 1@14 |
| 103 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 104 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM | 6@68 | 3@34 |  | 1@17 |
| 105 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 106 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 78@28 | 37@14 |  | 17@7 |
| 107 | 1930 | 20 | Default |  | A2 |  | 64 QAM | 6@76 | 3@38 |  | 1@19 |

| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 108 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |  |
| 109 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 110 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 42@10 | 18@5 |  | 8@3 |
| 111 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 112 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 60@19 | 28@10 |  | 12@5 |
| 113 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 114 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 115 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 78@28 | 37@14 |  | 17@7 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-4a: Test Configuration table for NS_05 and NS_05U power class 2

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |
| A-MPR test parameters for NS_05 and NS_05U |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 1922.5 | 5 | 15 | N/A for A-MPR testing | A3 | DFT-s-OFDM | PI/2 BPSK |  | Outer_Full |  |  |
| 2 | 1922.5 | 5 | Default |  | A4 |  | PI/2 BPSK |  | 8@1 | 4@1 |  |
| 3 | 1925 | 10 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 4 | 1925 | 10 | Default |  | A7 |  | PI/2 BPSK |  | 30@11 | 15@6 | 6@3 |
| 5 | 1925 | 10 | Default |  | A1 |  | PI/2 BPSK |  | 6@40 | 3@20 | 1@10 |
| 6 | 1925 | 10 | Default |  | A8 |  | PI/2 BPSK |  | 6@11 | 3@6 | 1@3 |
| 7 | 1935 | 10 | Default |  | A4 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 8 | 1927.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 9 | 1927.5 | 15 | Default |  | A7 |  | PI/2 BPSK |  | 54@20 | 27@10 | 12@5 |
| 10 | 1927.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | 6@56 | 3@28 | 1@14 |
| 11 | 1927.5 | 15 | Default |  | A8 |  | PI/2 BPSK |  | 6@20 | 3@10 | 1@5 |
| 12 | 1932.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 13 | 1932.5 | 15 | Default |  | A7 |  | PI/2 BPSK |  | 64@11 | 30@6 | 15@3 |
| 14 | 1932.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | 6@68 | 3@34 | 1@17 |
| 15 | 1942.5 | 15 | Default |  | A5 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 16 | 1930 | 20 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 17 | 1930 | 20 | Default |  | A7 |  | PI/2 BPSK |  | 72@28 | 36@14 | 16@7 |
| 18 | 1930 | 20 | Default |  | A1 |  | PI/2 BPSK |  | 6@76 | 3@38 | 1@19 |
| 19 | 1930 | 20 | Default |  | A8 |  | PI/2 BPSK |  | 6@28 | 3@14 | 1@7 |
| 20 | 1950 | 20 | Default |  | A6 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 21 | 1922.5 | 5 | 15 |  | A3 |  | QPSK |  | Outer_Full |  |  |
| 22 | 1922.5 | 5 | Default |  | A4 |  | QPSK |  | 8@1 | 4@1 |  |
| 23 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |
| 24 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 30@11 | 15@6 | 6@3 |
| 25 | 1925 | 10 | Default |  | A1 |  | QPSK |  | 6@40 | 3@20 | 1@10 |
| 26 | 1925 | 10 | Default |  | A8 |  | QPSK |  | 6@11 | 3@6 | 1@3 |
| 27 | 1935 | 10 | Default |  | A4 |  | QPSK |  | Outer_Full |  |  |
| 28 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |
| 29 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 54@20 | 27@10 | 12@5 |
| 30 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | 6@56 | 3@28 | 1@14 |
| 31 | 1927.5 | 15 | Default |  | A8 |  | QPSK |  | 6@20 | 3@10 | 1@5 |
| 32 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |
| 33 | 1932.5 | 15 | Default |  | A7 |  | QPSK |  | 64@11 | 30@6 | 15@3 |
| 34 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | 6@68 | 3@34 | 1@17 |
| 35 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |
| 36 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |
| 37 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 72@28 | 36@14 | 16@7 |
| 38 | 1930 | 20 | Default |  | A1 |  | QPSK |  | 6@76 | 3@38 | 1@19 |
| 39 | 1930 | 20 | Default |  | A8 |  | QPSK |  | 6@28 | 3@14 | 1@7 |
| 40 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |
| 41 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |
| 42 | 1922.5 | 5 | Default |  | A4 |  | 16 QAM |  | 8@1 | 4@1 |  |
| 43 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |
| 44 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 30@11 | 15@6 | 6@3 |
| 45 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | 6@40 | 3@20 | 1@10 |
| 46 | 1925 | 10 | Default |  | A8 |  | 16 QAM |  | 6@11 | 3@6 | 1@3 |
| 47 | 1935 | 10 | Default |  | A4 |  | 16 QAM |  | Outer_Full |  |  |
| 48 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |
| 49 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 54@20 | 27@10 | 12@5 |
| 50 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | 6@56 | 3@28 | 1@14 |

| A-MPR test parameters for NS_05 and NS_05U |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 51 | 1927.5 | 15 | Default |  | A8 |  | 16 QAM | 6@20 | 3@10 | 1@5 |
| 52 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |
| 53 | 1932.5 | 15 | Default |  | A7 |  | 16 QAM | 64@11 | 30@6 | 15@3 |
| 54 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | 6@68 | 3@34 | 1@17 |
| 55 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM | Outer_Full |  |  |
| 56 | 1930 | 20 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |
| 57 | 1930 | 20 | Default |  | A7 |  | 16 QAM | 72@28 | 36@14 | 16@7 |
| 58 | 1930 | 20 | Default |  | A1 |  | 16 QAM | 6@76 | 3@38 | 1@19 |
| 59 | 1930 | 20 | Default |  | A8 |  | 16 QAM | 6@28 | 3@14 | 1@7 |
| 60 | 1950 | 20 | Default |  | A6 |  | 16 QAM | Outer_Full |  |  |
| 61 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM | Outer_Full |  |  |
| 62 | 1922.5 | 5 | Default |  | A4 |  | 64 QAM | 8@1 | 4@1 |  |
| 63 | 1925 | 10 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |
| 64 | 1925 | 10 | Default |  | A7 |  | 64 QAM | 30@11 | 15@6 | 6@3 |
| 65 | 1925 | 10 | Default |  | A1 |  | 64 QAM | 6@40 | 3@20 | 1@10 |
| 66 | 1925 | 10 | Default |  | A8 |  | 64 QAM | 6@11 | 3@6 | 1@3 |
| 67 | 1935 | 10 | Default |  | A4 |  | 64 QAM | Outer_Full |  |  |
| 68 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |
| 69 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM | 54@20 | 27@10 | 12@5 |
| 70 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | 6@56 | 3@28 | 1@14 |
| 71 | 1927.5 | 15 | Default |  | A8 |  | 64 QAM | 6@20 | 3@10 | 1@5 |
| 72 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |
| 73 | 1932.5 | 15 | Default |  | A7 |  | 64 QAM | 64@11 | 30@6 | 15@3 |
| 74 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | 6@68 | 3@34 | 1@17 |
| 75 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM | Outer_Full |  |  |
| 76 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |
| 77 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 72@28 | 36@14 | 16@7 |
| 78 | 1930 | 20 | Default |  | A1 |  | 64 QAM | 6@76 | 3@38 | 1@19 |
| 79 | 1930 | 20 | Default |  | A8 |  | 64 QAM | 6@28 | 3@14 | 1@7 |
| 80 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |
| 81 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |
| 82 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 30@11 | 15@6 | 6@3 |
| 83 | 1925 | 10 | Default |  | A1 |  | 256 QAM | 6@40 | 3@20 | 1@10 |
| 84 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |
| 85 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 54@20 | 27@10 | 12@5 |
| 86 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | 6@56 | 3@28 | 1@14 |
| 87 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |
| 88 | 1932.5 | 15 | Default |  | A7 |  | 256 QAM | 64@11 | 30@6 | 15@3 |
| 89 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | 6@68 | 3@34 | 1@17 |
| 90 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |
| 91 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 72@28 | 36@14 | 16@7 |
| 92 | 1930 | 20 | Default |  | A1 |  | 256 QAM | 6@76 | 3@38 | 1@19 |
| 93 | 1922.5 | 5 | 15 |  | A3 |  | QPSK | Outer_Full |  |  |
| 94 | 1922.5 | 5 | Default |  | A4 |  | QPSK | 8@1 | 4@1 |  |
| 95 | 1925 | 10 | Default |  | A1 |  | QPSK | Outer_Full |  |  |
| 96 | 1925 | 10 | Default |  | A7 |  | QPSK | 30@11 | 15@6 | 6@3 |
| 97 | 1925 | 10 | Default |  | A1 |  | QPSK | 6@40 | 3@20 | 1@10 |
| 98 | 1925 | 10 | Default |  | A8 |  | QPSK | 6@11 | 3@6 | 1@3 |
| 99 | 1935 | 10 | Default |  | A4 |  | QPSK | Outer_Full |  |  |
| 100 | 1927.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |
| 101 | 1927.5 | 15 | Default |  | A7 |  | QPSK | 52@20 | 26@10 | 12@5 |
| 102 | 1927.5 | 15 | Default |  | A1 |  | QPSK | 6@56 | 3@28 | 1@14 |
| 103 | 1927.5 | 15 | Default |  | A8 |  | QPSK | 6@20 | 3@10 | 1@5 |
| 104 | 1932.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |
| 105 | 1932.5 | 15 | Default |  | A7 |  | QPSK | 62@11 | 30@6 | 14@3 |
| 106 | 1932.5 | 15 | Default |  | A1 |  | QPSK | 6@68 | 3@34 | 1@17 |
| 107 | 1942.5 | 15 | Default |  | A5 |  | QPSK | Outer_Full |  |  |

| A-MPR test parameters for NS_05 and NS_05U |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 108 | 1930 | 20 | Default |  | A1 | CP-OFDM | QPSK | Outer_Full |  |  |  |
| 109 | 1930 | 20 | Default |  | A7 |  | QPSK | 72@28 | 36@14 |  | 16@7 |
| 110 | 1930 | 20 | Default |  | A1 |  | QPSK | 6@76 | 3@38 |  | 1@19 |
| 111 | 1930 | 20 | Default |  | A8 |  | QPSK | 6@28 | 3@14 |  | 1@7 |
| 112 | 1950 | 20 | Default |  | A6 |  | QPSK | Outer_Full |  |  |  |
| 113 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM | Outer_Full |  |  |  |
| 114 | 1922.5 | 5 | Default |  | A4 |  | 16 QAM | 8@1 | 4@1 |  |  |
| 115 | 1925 | 10 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 116 | 1925 | 10 | Default |  | A7 |  | 16 QAM | 30@11 | 15@6 |  | 6@3 |
| 117 | 1925 | 10 | Default |  | A1 |  | 16 QAM | 6@40 | 3@20 |  | 1@10 |
| 118 | 1925 | 10 | Default |  | A8 |  | 16 QAM | 6@11 | 3@6 |  | 1@3 |
| 119 | 1935 | 10 | Default |  | A4 |  | 16 QAM | Outer_Full |  |  |  |
| 120 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 121 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM | 52@20 | 26@10 |  | 12@5 |
| 122 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM | 6@56 | 3@28 |  | 1@14 |
| 123 | 1927.5 | 15 | Default |  | A8 |  | 16 QAM | 6@20 | 3@10 |  | 1@5 |
| 124 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 125 | 1932.5 | 15 | Default |  | A7 |  | 16 QAM | 62@11 | 30@6 |  | 14@3 |
| 126 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | 6@68 | 3@34 |  | 1@17 |
| 127 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM | Outer_Full |  |  |  |
| 128 | 1930 | 20 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 129 | 1930 | 20 | Default |  | A7 |  | 16 QAM | 72@28 | 36@14 |  | 16@7 |
| 130 | 1930 | 20 | Default |  | A1 |  | 16 QAM | 6@76 | 3@38 |  | 1@19 |
| 131 | 1930 | 20 | Default |  | A8 |  | 16 QAM | 6@28 | 3@14 |  | 1@7 |
| 132 | 1950 | 20 | Default |  | A6 |  | 16 QAM | Outer_Full |  |  |  |
| 133 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM | Outer_Full |  |  |  |
| 134 | 1922.5 | 5 | Default |  | A4 |  | 64 QAM | 8@1 | 4@1 |  |  |
| 135 | 1925 | 10 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 136 | 1925 | 10 | Default |  | A7 |  | 64 QAM | 30@11 | 15@6 |  | 6@3 |
| 137 | 1925 | 10 | Default |  | A1 |  | 64 QAM | 6@40 | 3@20 |  | 1@10 |
| 138 | 1935 | 10 | Default |  | A4 |  | 64 QAM | Outer_Full |  |  |  |
| 139 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 140 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM | 52@20 | 26@10 |  | 12@5 |
| 141 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | 6@56 | 3@28 |  | 1@14 |
| 142 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 143 | 1932.5 | 15 | Default |  | A7 |  | 64 QAM | 62@11 | 30@6 |  | 14@3 |
| 144 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | 6@68 | 3@34 |  | 1@17 |
| 145 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM | Outer_Full |  |  |  |
| 146 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 147 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 72@28 | 36@14 |  | 16@7 |
| 148 | 1930 | 20 | Default |  | A1 |  | 64 QAM | 6@76 | 3@38 |  | 1@19 |
| 149 | 1950 | 20 | Default |  | A6 |  | 64 QAM | Outer_Full |  |  |  |
| 150 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |  |
| 151 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 152 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 30@11 | 15@6 |  | 6@3 |
| 153 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 154 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 52@20 | 26@10 |  | 12@5 |
| 155 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 156 | 1932.5 | 15 | Default |  | A7 |  | 256 QAM | 62@11 | 30@6 |  | 14@3 |
| 157 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 158 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 72@28 | 36@14 |  | 16@7 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-5: Test Configuration table for NS_05U power class 3

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |  |
| A-MPR test parameters for NS_05U |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 1 | 1922.5 | 5 | 15 | N/A for A-MPR testing. | A3 | DFT-s-OFDM | PI/2 BPSK |  | Outer_Full |  |  |  |
| 2 | 1925 | 10 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 3 | 1925 | 10 | Default |  | A7 |  | PI/2 BPSK |  | 40@10 | 18@5 | 8@3 |  |
| 4 | 1925 | 10 | Default |  | A2 |  | PI/2 BPSK |  | 6@40 | 3@20 | 1@10 |  |
| 5 | 1935 | 10 | Default |  | A4 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 6 | 1927.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 7 | 1927.5 | 15 | Default |  | A7 |  | PI/2 BPSK |  | 60@19 | 27@10 | 12@5 |  |
| 8 | 1927.5 | 15 | Default |  | A2 |  | PI/2 BPSK |  | 6@56 | 3@28 | 1@14 |  |
| 9 | 1932.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 10 | 1932.5 | 15 | Default |  | A2 |  | PI/2 BPSK |  | 6@68 | 3@34 | 1@17 |  |
| 11 | 1942.5 | 15 | Default |  | A5 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 12 | 1930 | 20 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 13 | 1930 | 20 | Default |  | A7 |  | PI/2 BPSK |  | 72@28 | 36@14 | 16@7 |  |
| 14 | 1930 | 20 | Default |  | A2 |  | PI/2 BPSK |  | 6@76 | 3@38 | 1@19 |  |
| 15 | 1950 | 20 | Default |  | A6 |  | PI/2 BPSK |  | Outer_Full |  |  |  |
| 16 | 1922.5 | 5 | 15 |  | A3 |  | QPSK |  | Outer_Full |  |  |  |
| 17 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 18 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 40@10 | 18@5 |  | 8@3 |
| 19 | 1925 | 10 | Default |  | A2 |  | QPSK |  | 6@40 | 3@20 |  | 1@10 |
| 20 | 1935 | 10 | Default |  | A4 |  | QPSK |  | Outer_Full |  |  |  |
| 21 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 22 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 60@19 | 27@10 |  | 12@5 |
| 23 | 1927.5 | 15 | Default |  | A2 |  | QPSK |  | 6@56 | 3@28 |  | 1@14 |
| 24 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 25 | 1932.5 | 15 | Default |  | A2 |  | QPSK |  | 6@68 | 3@34 |  | 1@17 |
| 26 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |  |
| 27 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 28 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 72@28 | 36@14 |  | 16@7 |
| 29 | 1930 | 20 | Default |  | A2 |  | QPSK |  | 6@76 | 3@38 |  | 1@19 |
| 30 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |  |
| 31 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |  |
| 32 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 33 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 40@10 | 18@5 |  | 8@3 |
| 34 | 1925 | 10 | Default |  | A2 |  | 16 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 35 | 1935 | 10 | Default |  | A4 |  | 16 QAM |  | Outer_Full |  |  |  |
| 36 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 37 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 60@19 | 27@10 |  | 12@5 |
| 38 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 39 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 40 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@68 | 3@34 |  | 1@17 |
| 41 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM |  | Outer_Full |  |  |  |
| 42 | 1930 | 20 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 43 | 1930 | 20 | Default |  | A7 |  | 16 QAM |  | 72@28 | 36@14 |  | 16@7 |
| 44 | 1930 | 20 | Default |  | A2 |  | 16 QAM |  | 6@76 | 3@38 |  | 1@19 |
| 45 | 1950 | 20 | Default |  | A6 |  | 16 QAM |  | Outer_Full |  |  |  |
| 46 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM |  | Outer_Full |  |  |  |
| 47 | 1925 | 10 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 48 | 1925 | 10 | Default |  | A7 |  | 64 QAM |  | 40@10 | 18@5 |  | 8@3 |
| 49 | 1925 | 10 | Default |  | A2 |  | 64 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 50 | 1935 | 10 | Default |  | A4 |  | 64 QAM |  | Outer_Full |  |  |  |

| A-MPR test parameters for NS_05U |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 51 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 52 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM | 60@19 | 27@10 |  | 12@5 |
| 53 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM | 6@56 | 3@28 |  | 1@14 |
| 54 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 55 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM | 6@68 | 3@34 |  | 1@17 |
| 56 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM | Outer_Full |  |  |  |
| 57 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 58 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 72@28 | 36@14 |  | 16@7 |
| 59 | 1930 | 20 | Default |  | A2 |  | 64 QAM | 6@76 | 3@38 |  | 1@19 |
| 60 | 1950 | 20 | Default |  | A6 |  | 64 QAM | Outer_Full |  |  |  |
| 61 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |  |
| 62 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 63 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 40@10 | 18@5 |  | 8@3 |
| 64 | 1925 | 10 | Default |  | A2 |  | 256 QAM | 6@40 | 3@20 |  | 1@10 |
| 65 | 1935 | 10 | Default |  | A4 |  | 256 QAM | Outer_Full |  |  |  |
| 66 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 67 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 60@19 | 27@10 |  | 12@5 |
| 68 | 1927.5 | 15 | Default |  | A2 |  | 256 QAM | 6@56 | 3@28 |  | 1@14 |
| 69 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 70 | 1932.5 | 15 | Default |  | A2 |  | 256 QAM | 6@68 | 3@34 |  | 1@17 |
| 71 | 1942.5 | 15 | Default |  | A5 |  | 256 QAM | Outer_Full |  |  |  |
| 72 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 73 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 72@28 | 36@14 |  | 16@7 |
| 74 | 1930 | 20 | Default |  | A2 |  | 256 QAM | 6@76 | 3@38 |  | 1@19 |
| 75 | 1950 | 20 | Default |  | A6 |  | 256 QAM | Outer_Full |  |  |  |
| 76 | 1922.5 | 5 | 15 |  | A3 | CP-OFDM | QPSK | Outer_Full |  |  |  |
| 77 | 1925 | 10 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 78 | 1925 | 10 | Default |  | A7 |  | QPSK | 42@10 | 18@5 |  | 8@3 |
| 79 | 1925 | 10 | Default |  | A2 |  | QPSK | 6@40 | 3@20 |  | 1@10 |
| 80 | 1935 | 10 | Default |  | A4 |  | QPSK | Outer_Full |  |  |  |
| 81 | 1927.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 82 | 1927.5 | 15 | Default |  | A7 |  | QPSK | 60@19 | 28@10 |  | 12@5 |
| 83 | 1927.5 | 15 | Default |  | A2 |  | QPSK | 6@56 | 3@28 |  | 1@14 |
| 84 | 1932.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 85 | 1932.5 | 15 | Default |  | A2 |  | QPSK | 6@68 | 3@34 |  | 1@17 |
| 86 | 1942.5 | 15 | Default |  | A5 |  | QPSK | Outer_Full |  |  |  |
| 87 | 1930 | 20 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 88 | 1930 | 20 | Default |  | A7 |  | QPSK | 78@28 | 37@14 |  | 17@7 |
| 89 | 1930 | 20 | Default |  | A2 |  | QPSK | 6@76 | 3@38 |  | 1@19 |
| 90 | 1950 | 20 | Default |  | A6 |  | QPSK | Outer_Full |  |  |  |
| 91 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM | Outer_Full |  |  |  |
| 92 | 1925 | 10 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 93 | 1925 | 10 | Default |  | A7 |  | 16 QAM | 42@10 | 18@5 |  | 8@3 |
| 94 | 1925 | 10 | Default |  | A2 |  | 16 QAM | 6@40 | 3@20 |  | 1@10 |
| 95 | 1935 | 10 | Default |  | A4 |  | 16 QAM | Outer_Full |  |  |  |
| 96 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 97 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM | 60@19 | 28@10 |  | 12@5 |
| 98 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM | 6@56 | 3@28 |  | 1@14 |
| 99 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 100 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM | 6@68 | 3@34 |  | 1@17 |
| 101 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM | Outer_Full |  |  |  |
| 102 | 1930 | 20 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 103 | 1930 | 20 | Default |  | A7 |  | 16 QAM | 78@28 | 37@14 |  | 17@7 |
| 104 | 1930 | 20 | Default |  | A2 |  | 16 QAM | 6@76 | 3@38 |  | 1@19 |
| 105 | 1950 | 20 | Default |  | A6 |  | 16 QAM | Outer_Full |  |  |  |
| 106 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM | Outer_Full |  |  |  |
| 107 | 1925 | 10 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 108 | 1925 | 10 | Default |  | A7 |  | 64 QAM | 42@10 | 18@5 |  | 8@3 |

| A-MPR test parameters for NS_05U |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 109 | 1925 | 10 | Default |  | A2 |  | 64 QAM | 6@40 | 3@20 |  | 1@10 |
| 110 | 1935 | 10 | Default |  | A4 |  | 64 QAM | Outer_Full |  |  |  |
| 111 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 112 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM | 60@19 | 28@10 |  | 12@5 |
| 113 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM | 6@56 | 3@28 |  | 1@14 |
| 114 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 115 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM | 6@68 | 3@34 |  | 1@17 |
| 116 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM | Outer_Full |  |  |  |
| 117 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 118 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 78@28 | 37@14 |  | 17@7 |
| 119 | 1930 | 20 | Default |  | A2 |  | 64 QAM | 6@76 | 3@38 |  | 1@19 |
| 120 | 1950 | 20 | Default |  | A6 |  | 64 QAM | Outer_Full |  |  |  |
| 121 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |  |
| 122 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 123 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 42@10 | 18@5 |  | 8@3 |
| 124 | 1935 | 10 | Default |  | A4 |  | 256 QAM | Outer_Full |  |  |  |
| 125 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 126 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 60@19 | 28@10 |  | 12@5 |
| 127 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 128 | 1942.5 | 15 | Default |  | A5 |  | 256 QAM | Outer_Full |  |  |  |
| 129 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 130 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 78@28 | 37@14 |  | 17@7 |
| 131 | 1950 | 20 | Default |  | A6 |  | 256 QAM | Outer_Full |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-6: Test Configuration table for NS_43 for PC3 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_43 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 910 | 10 | default | N/A for A-MPR | DFT-s--OFDM | PI/2 BPSK | Outer_Full (A2) |
| 2 | 907.5 | 15 |  |  |  | PI/2 BPSK | Edge_1RB_Left (A6) |
| 3 | 907.5 | 15 |  |  |  | PI/2 BPSK | Outer_Full (A6) |
| 43 | 902.5 | 5 |  |  |  | QPSK | Outer_Full (A1) |
| 5 | 910 | 10 |  |  |  | QPSK | Outer_Full (A4) |
| 6 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 7 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 8 | 910 | 10 |  |  |  | 16 QAM | Outer_Full (A5) |
| 9 | 907.5 | 15 |  |  |  | 16 QAM | Edge_1RB_Left (A6) |
| 10 | 907.5 | 15 |  |  |  | 16 QAM | Outer_Full (A6) |
| 113 | 910 | 10 |  |  |  | 64 QAM | Outer_Full (A3) |
| 12 | 907.5 | 15 |  |  |  | 64 QAM | Edge_1RB_Left (A6) |
| 13 | 907.5 | 15 |  |  |  | 64 QAM | Outer_Full (A6) |
| 14 | 907.5 | 15 |  |  |  | 256 QAM | Edge_1RB_Left (A6) |
| 15 | 907.5 | 15 |  |  |  | 256 QAM | Outer_Full (A6) |
| 16 | 902.5 | 5 |  |  | CP-OFDM | QPSK | Outer_Full (A1) |
| 17 | 910 | 10 |  |  |  | QPSK | Outer_Full (A5) |
| 18 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 19 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 20 | 902.5 | 5 |  |  |  | 16 QAM | Outer_Full (A1) |
| 21 | 910 | 10 |  |  |  | 16 QAM | Outer_Full (A5) |
| 22 | 907.5 | 15 |  |  |  | 16 QAM | Edge_1RB_Left (A6) |
| 23 | 907.5 | 15 |  |  |  | 16 QAM | Outer_Full (A6) |
| 243 | 910 | 10 |  |  |  | 64 QAM | Outer_Full (A3) |
| 25 | 907.5 | 15 |  |  |  | 64 QAM | Edge_1RB_Left (A6) |
| 26 | 907.5 | 15 |  |  |  | 64 QAM | Outer_Full (A6) |
| 27 | 907.5 | 15 |  |  |  | 256 QAM | Edge_1RB_Left (A6) |
| 28 | 907.5 | 15 |  |  |  | 256 QAM | Outer_Full (A6) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Not applicable when SCS=60kHz. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-6a: Test Configuration table for NS_43 for PC2 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_43 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 902.5 | 5 | Default | N/A for A-MPR | DFT-s--OFDM | Pi/2 BPSK | Outer_Full (A1) |
| 2 | 910 | 10 |  |  |  | Pi/2 BPSK | Outer_Full (A2) |
| 3 | 907.5 | 15 |  |  |  | Pi/2 BPSK | Outer_Full (A6) |
| 4 | 907.5 | 15 |  |  |  | Pi/2 BPSK | Edge_1RB_Left (A6) |
| 53 | 902.5 | 5 |  |  |  | QPSK | Outer_Full (A1) |
| 6 | 910 | 10 |  |  |  | QPSK | Outer_Full (A4) |
| 7 | 910 | 10 |  |  |  | QPSK | Outer_Full (A5) |
| 8 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 9 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 103 | 902.5 | 5 |  |  |  | 16QAM | Outer_Full (A1) |
| 11 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A4) |
| 12 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A5) |
| 13 | 907.5 | 15 |  |  |  | 16QAM | Outer_Full (A6) |
| 14 | 907.5 | 15 |  |  |  | 16QAM | Edge_1RB_Left (A6) |
| 153 | 902.5 | 5 |  |  |  | 64QAM | Outer_Full (A1) |
| 16 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A2) |
| 173 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A3) |
| 18 | 907.5 | 15 |  |  |  | 64QAM | Outer_Full (A6) |
| 19 | 907.5 | 15 |  |  |  | 64QAM | Edge_1RB_Left (A6) |
| 20 | 907.5 | 15 |  |  |  | 256QAM | Outer_Full (A6) |
| 21 | 907.5 | 15 |  |  |  | 256QAM | Edge_1RB_Left (A6) |
| 223 | 902.5 | 5 |  |  | CP-OFDM | QPSK | Outer_Full (A1) |
| 23 | 910 | 10 |  |  |  | QPSK | Outer_Full (A4) |
| 24 | 910 | 10 |  |  |  | QPSK | Outer_Full (A5) |
| 25 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 26 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 273 | 902.5 | 5 |  |  |  | 16QAM | Outer_Full (A1) |
| 28 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A4) |
| 29 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A5) |
| 30 | 907.5 | 15 |  |  |  | 16QAM | Outer_Full (A6) |
| 31 | 907.5 | 15 |  |  |  | 16QAM | Edge_1RB_Left (A6) |
| 323 | 902.5 | 5 |  |  |  | 64QAM | Outer_Full (A1) |
| 33 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A2) |
| 343 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A3) |
| 35 | 907.5 | 15 |  |  |  | 64QAM | Outer_Full (A6) |
| 36 | 907.5 | 15 |  |  |  | 64QAM | Edge_1RB_Left (A6) |
| 37 | 907.5 | 15 |  |  |  | 256QAM | Outer_Full (A6) |
| 38 | 907.5 | 15 |  |  |  | 256QAM | Edge_1RB_Left (A6) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Not applicable when SCS=60kHz |  |  |  |  |  |  |  |

Table 6.2.3.4.1-7: Test Configuration table for NS_43U

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15 kHz |  |
| A-MPR test parameters for NS_43U |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 902.5 | 5 | 15 | N/A for A-MPR | DFT-s--OFDM | PI/2 BPSK | Outer_Full |
| 2 | 910 | 10 | 15 |  |  | PI/2 BPSK | Outer_Full |
| 3 | 907.5 | 15 | 15 |  |  | PI/2 BPSK | Edge_1RB_Left |
| 4 | 907.5 | 15 | 15 |  |  | PI/2 BPSK | Outer_Full |
| 5 | 902.5 | 5 | 15 |  |  | QPSK | Outer_Full |
| 6 | 910 | 10 | 15 |  |  | QPSK | Outer_Full |
| 7 | 907.5 | 15 | 15 |  |  | QPSK | Edge_1RB_Left |
| 8 | 907.5 | 15 | 15 |  |  | QPSK | Outer_Full |
| 9 | 902.5 | 5 | 15 |  |  | 16 QAM | Outer_Full |
| 10 | 910 | 10 | 15 |  |  | 16 QAM | Outer_Full |
| 11 | 907.5 | 15 | 15 |  |  | 16 QAM | Edge_1RB_Left |
| 12 | 907.5 | 15 | 15 |  |  | 16 QAM | Outer_Full |
| 13 | 902.5 | 5 | 15 |  |  | 64 QAM | Outer_Full |
| 14 | 910 | 10 | 15 |  |  | 64 QAM | Outer_Full |
| 15 | 907.5 | 15 | 15 |  |  | 64 QAM | Edge_1RB_Left |
| 16 | 907.5 | 15 | 15 |  |  | 64 QAM | Outer_Full |
| 17 | 902.5 | 5 | 15 |  |  | 256 QAM | Outer_Full |
| 18 | 910 | 10 | 15 |  |  | 256 QAM | Outer_Full |
| 19 | 907.5 | 15 | 15 |  |  | 256 QAM | Edge_1RB_Left |
| 20 | 907.5 | 15 | 15 |  |  | 256 QAM | Outer_Full |
| 21 | 902.5 | 5 | 15 |  | CP-OFDM | QPSK | Outer_Full |
| 22 | 910 | 10 | 15 |  |  | QPSK | Outer_Full |
| 23 | 907.5 | 15 | 15 |  |  | QPSK | Edge_1RB_Left |
| 24 | 907.5 | 15 | 15 |  |  | QPSK | Outer_Full |
| 25 | 902.5 | 5 | 15 |  |  | 16 QAM | Outer_Full |
| 26 | 910 | 10 | 15 |  |  | 16 QAM | Outer_Full |
| 27 | 907.5 | 15 | 15 |  |  | 16 QAM | Edge_1RB_Left |
| 28 | 907.5 | 15 | 15 |  |  | 16 QAM | Outer_Full |
| 29 | 902.5 | 5 | 15 |  |  | 64 QAM | Outer_Full |
| 30 | 910 | 10 | 15 |  |  | 64 QAM | Outer_Full |
| 31 | 907.5 | 15 | 15 |  |  | 64 QAM | Edge_1RB_Left |
| 32 | 907.5 | 15 | 15 |  |  | 64 QAM | Outer_Full |
| 33 | 902.5 | 5 | 15 |  |  | 256 QAM | Outer_Full |
| 34 | 910 | 10 | 15 |  |  | 256 QAM | Outer_Full |
| 35 | 907.5 | 15 | 15 |  |  | 256 QAM | Edge_1RB_Left |
| 36 | 907.5 | 15 | 15 |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-8: Test Configuration table for NS_37

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 10 MHz, 15 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Unless otherwise specified in the SCS column select Lowest and Highest. |  |  |
| A-MPR test parameters for NS_37 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 1457.9 | 10 | Default | N/A for A-MPR testing | DFT-s-OFDM | PI/2 BPSK (A1) | Outer_Full |  |  |
| 2 | 1455.4 | 15 | Default |  |  | PI/2 BPSK (A1) | Outer_Full |  |  |
| 3 | 1455.4 | 15 | 15, 30 |  |  | PI/2 BPSK (A2) | 5@0 | 2@0 | N/A |
| 4 | 1455.4 | 15 | 15, 30 |  |  | PI/2 BPSK (A2) | Edge_1RB_Right | Edge_1RB_Right | N/A |
| 5 | 1457.9 | 10 | Default |  |  | QPSK (A1) | Outer_Full |  |  |
| 6 | 1455.4 | 15 | Default |  |  | QPSK (A2) | Outer_Full |  |  |
| 7 | 1455.4 | 15 | 15, 30 |  |  | QPSK (A2) | 5@0 | 2@0 | N/A |
| 8 | 1455.4 | 15 | 15, 30 |  |  | QPSK (A2) | Edge_1RB_Right | Edge_1RB_Right | N/A |
| 9 | 1457.9 | 10 | Default |  |  | 16 QAM (A1) | Outer_Full |  |  |
| 10 | 1455.4 | 15 | Default |  |  | 16 QAM (A2) | Outer_Full |  |  |
| 11 | 1455.4 | 15 | 15, 30 |  |  | 16 QAM (A2) | 5@0 | 2@0 | N/A |
| 12 | 1455.4 | 15 | 15, 30 |  |  | 16 QAM (A2) | Edge_1RB_Right | Edge_1RB_Right | N/A |
| 13 | 1457.9 | 10 | Default |  |  | 64 QAM (A1) | Outer_Full |  |  |
| 14 | 1455.4 | 15 | Default |  |  | 64 QAM (A2) | Outer_Full |  |  |
| 15 | 1455.4 | 15 | 15, 30 |  |  | 64 QAM (A2) | 5@0 | 2@0 | N/A |
| 16 | 1455.4 | 15 | 15, 30 |  |  | 64 QAM (A2) | Edge_1RB_Right | Edge_1RB_Right | N/A |
| 17 | 1457.9 | 10 | Default |  | CP-OFDM | QPSK (A1) | Outer_Full |  |  |
| 18 | 1455.4 | 15 | Default |  |  | QPSK (A2) | Outer_Full |  |  |
| 19 | 1455.4 | 15 | 15, 30 |  |  | QPSK (A2) | 5@0 | 2@0 | N/A |
| 20 | 1455.4 | 15 | 15, 30 |  |  | QPSK (A2) | Edge_1RB_Right | Edge_1RB_Right | N/A |
| 21 | 1457.9 | 10 | Default |  |  | 16 QAM (A1) | Outer_Full |  |  |
| 22 | 1455.4 | 15 | Default |  |  | 16 QAM (A2) | Outer_Full |  |  |
| 23 | 1455.4 | 15 | 15, 30 |  |  | 16 QAM (A2) | 5@0 | 2@0 | N/A |
| 24 | 1455.4 | 15 | 15, 30 |  |  | 16 QAM (A2) | Edge_1RB_Right | Edge_1RB_Right | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-9: Test Configuration table for NS_38

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 10 MHz, 15 MHz and 20 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest and Highest. |  |  |
| A-MPR test parameters for NS_38 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 1435 | 10 | Default | N/A for A-MPR testing | DFT-s-OFDM | PI/2 BPSK | Outer_Full |  |  |
| 2 | 1435 | 10 | Default |  |  | PI/2 BPSK | 12@0 | 6@0 | 3@0 |
| 3 | 1437.5 | 15 | Default |  |  | PI/2 BPSK | Outer_Full |  |  |
| 4 | 1437.5 | 15 | Default |  |  | PI/2 BPSK | 20@0 | 10@0 | 5@0 |
| 5 | 1440 | 20 | Default |  |  | PI/2 BPSK | Outer_Full |  |  |
| 6 | 1440 | 20 | Default |  |  | PI/2 BPSK | 30@0 | 12@0 | 6@0 |
| 7 | 1435 | 10 | Default |  |  | QPSK | Outer_Full |  |  |
| 8 | 1435 | 10 | Default |  |  | QPSK | 12@0 | 6@0 | 3@0 |
| 9 | 1437.5 | 15 | Default |  |  | QPSK | Outer_Full |  |  |
| 10 | 1437.5 | 15 | Default |  |  | QPSK | 20@0 | 10@0 | 5@0 |
| 11 | 1440 | 20 | Default |  |  | QPSK | Outer_Full |  |  |
| 12 | 1440 | 20 | Default |  |  | QPSK | 30@0 | 12@0 | 6@0 |
| 13 | 1435 | 10 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 14 | 1435 | 10 | Default |  |  | 16 QAM | 12@0 | 6@0 | 3@0 |
| 15 | 1437.5 | 15 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 16 | 1437.5 | 15 | Default |  |  | 16 QAM | 20@0 | 10@0 | 5@0 |
| 17 | 1440 | 20 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 18 | 1440 | 20 | Default |  |  | 16 QAM | 30@0 | 12@0 | 6@0 |
| 19 | 1435 | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 20 | 1435 | 10 | Default |  |  | 64 QAM | 12@0 | 6@0 | 3@0 |
| 21 | 1437.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 22 | 1437.5 | 15 | Default |  |  | 64 QAM | 20@0 | 10@0 | 5@0 |
| 23 | 1440 | 20 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 24 | 1440 | 20 | Default |  |  | 64 QAM | 30@0 | 12@0 | 6@0 |
| 25 | 1435 | 10 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 26 | 1435 | 10 | Default |  |  | 256 QAM | 12@0 | 6@0 | 3@0 |
| 27 | 1437.5 | 15 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 28 | 1437.5 | 15 | Default |  |  | 256 QAM | 20@0 | 10@0 | 5@0 |
| 29 | 1440 | 20 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 30 | 1440 | 20 | Default |  |  | 256 QAM | 30@0 | 12@0 | 6@0 |
| 31 | 1435 | 10 | Default |  | CP-OFDM | QPSK | Outer_Full |  |  |
| 32 | 1435 | 10 | Default |  |  | QPSK | 12@0 | 6@0 | 3@0 |
| 33 | 1437.5 | 15 | Default |  |  | QPSK | Outer_Full |  |  |
| 34 | 1437.5 | 15 | Default |  |  | QPSK | 20@0 | 10@0 | 5@0 |
| 35 | 1440 | 20 | Default |  |  | QPSK | Outer_Full |  |  |
| 36 | 1440 | 20 | Default |  |  | QPSK | 30@0 | 15@0 | 7@0 |
| 37 | 1435 | 10 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 38 | 1435 | 10 | Default |  |  | 16 QAM | 12@0 | 6@0 | 3@0 |
| 39 | 1437.5 | 15 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 40 | 1437.5 | 15 | Default |  |  | 16 QAM | 20@0 | 10@0 | 5@0 |
| 41 | 1440 | 20 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 42 | 1440 | 20 | Default |  |  | 16 QAM | 30@0 | 15@0 | 7@0 |
| 43 | 1435 | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 44 | 1435 | 10 | Default |  |  | 64 QAM | 12@0 | 6@0 | 3@0 |
| 45 | 1437.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 46 | 1437.5 | 15 | Default |  |  | 64 QAM | 20@0 | 10@0 | 5@0 |

| A-MPR test parameters for NS_38 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 47 | 1440 | 20 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 48 | 1440 | 20 | Default |  |  | 64 QAM | 30@0 | 15@0 | 7@0 |
| 49 | 1435 | 10 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 50 | 1435 | 10 | Default |  |  | 256 QAM | 12@0 | 6@0 | 3@0 |
| 51 | 1437.5 | 15 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 52 | 1437.5 | 15 | Default |  |  | 256 QAM | 20@0 | 10@0 | 5@0 |
| 53 | 1440 | 20 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 54 | 1440 | 20 | Default |  |  | 256 QAM | 30@0 | 15@0 | 7@0 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: There is no need to test this frequency if it has already been tested in the Low Range. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-10: Test Configuration table for NS_39

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 10 MHz, 15 MHz and 20 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest and Highest. |  |
| A-MPR test parameters for NS_39 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |
| 1 | 1465 | 10 | Default | N/A for A-MPR testing | DFT-s-OFDM | PI/2 BPSK |  | Outer_Full |
| 2 | 1462.5 | 15 | Default |  |  | PI/2 BPSK |  | Outer_Full |
| 3 | 1460 | 20 | Default |  |  | PI/2 BPSK |  | Outer_Full |
| 4 | 1465 | 10 | Default |  |  | QPSK |  | Outer_Full |
| 5 | 1462.5 | 15 | Default |  |  | QPSK |  | Outer_Full |
| 6 | 1460 | 20 | Default |  |  | QPSK |  | Outer_Full |
| 7 | 1465 | 10 | Default |  |  | 16 QAM |  | Outer_Full |
| 8 | 1462.5 | 15 | Default |  |  | 16 QAM |  | Outer_Full |
| 9 | 1460 | 20 | Default |  |  | 16 QAM |  | Outer_Full |
| 10 | 1465 | 10 | Default |  |  | 64 QAM |  | Outer_Full |
| 11 | 1462.5 | 15 | Default |  |  | 64 QAM |  | Outer_Full |
| 12 | 1460 | 20 | Default |  |  | 64 QAM |  | Outer_Full |
| 13 | 1465 | 10 | Default |  |  | 256 QAM |  | Outer_Full |
| 14 | 1462.5 | 15 | Default |  |  | 256 QAM |  | Outer_Full |
| 15 | 1460 | 20 | Default |  |  | 256 QAM |  | Outer_Full |
| 16 | 1465 | 10 | Default |  | CP-OFDM | QPSK |  | Outer_Full |
| 17 | 1462.5 | 15 | Default |  |  | QPSK |  | Outer_Full |
| 18 | 1460 | 20 | Default |  |  | QPSK |  | Outer_Full |
| 19 | 1465 | 10 | Default |  |  | 16 QAM |  | Outer_Full |
| 20 | 1462.5 | 15 | Default |  |  | 16 QAM |  | Outer_Full |
| 21 | 1460 | 20 | Default |  |  | 16 QAM |  | Outer_Full |
| 22 | 1465 | 10 | Default |  |  | 64 QAM |  | Outer_Full |
| 23 | 1462.5 | 15 | Default |  |  | 64 QAM |  | Outer_Full |
| 24 | 1460 | 20 | Default |  |  | 64 QAM |  | Outer_Full |
| 25 | 1465 | 10 | Default |  |  | 256 QAM |  | Outer_Full |
| 26 | 1462.5 | 15 | Default |  |  | 256 QAM |  | Outer_Full |
| 27 | 1460 | 20 | Default |  |  | 256 QAM |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: There is no need to test this frequency if it has already been tested in the Low Range. |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-11: Test Configuration table for NS_18

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use channel bandwidth as specified in test parameters |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_18 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) | Comment |
| 1 | Low | 3MHz, 5MHz, 10MHz, 20MHz | Default | N/A | DFT-s-OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 2 |  |  |  |  |  | QPSK | Outer_Full | A1, A2 |
| 3 |  |  |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 4 |  |  |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 5 |  |  |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 6 |  |  |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 7 |  |  |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 8 |  |  |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 9 |  |  |  |  | CP-OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 10 |  |  |  |  |  | QPSK | Outer_Full | A1, A2 |
| 11 |  |  |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 12 |  |  |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 13 |  |  |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 14 |  |  |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 15 |  |  |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 16 |  |  |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 17 |  | 30MHz | 15kHz |  | DFT-s-OFDM | 256 QAM | 81@70 | A3 |
| 18 |  |  | 30kHz |  |  | 256 QAM | 40@35 | A3 |
| 19 |  |  | Default |  |  | 256 QAM | Edge_1RB_Right | A3 |
| 20 |  |  | Default |  |  | 256 QAM | Outer_Full | A4 |
| 21 |  |  | Default |  |  | 16QAM | Edge_1RB_Left | A5 |
| 22 |  |  | Default |  |  | 64QAM | Edge_1RB_Left | A5 |
| 23 |  |  | Default |  |  | 256QAM | Edge_1RB_Left | A5 |
| 24 |  |  | 15kHz |  | CP-OFDM | 256 QAM | 87@73 | A3 |
| 25 |  |  | 30kHz |  |  | 256 QAM | 42@36 | A3 |
| 26 |  |  | Default |  |  | 256 QAM | Edge_1RB_Right | A3 |
| 27 |  |  | Default |  |  | 256 QAM | Outer_Full | A4 |
| 28 |  |  | Default |  |  | 16QAM | Edge_1RB_Left | A5 |
| 29 |  |  | Default |  |  | 64QAM | Edge_1RB_Left | A5 |
| 30 |  |  | Default |  |  | 256QAM | Edge_1RB_Left | A5 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-12: Test Configuration table for NS_24

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to uplink carrier centre frequency (Fc) in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to test parameters (5, 10, 15, 20 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_24 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | Region A | Region B | Region C |
| 1-3 | 1992.5 | 5 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 4-6 | 1997.5 | 5 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 7-15 | 2002.5 | 5 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 16-18 | 1985 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | N/A | N/A |
| 19-21 | 1985 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 22-30 | 1995 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 31-39 | 2000 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 40-42 | 2000 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 43-51 | 1987.5 | 15 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 52-60 | 1997.5 | 15 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 61-63 | 1997.5 | 15 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 64-72 | 1990 | 20 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 75@0 |
| 73-75 | 1990 | 20 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 76-84 | 1995 | 20 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 64@0 |
| 85-87 | 1995 | 20 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 88 | 1992.5 | 5 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 89 | 1997.5 | 5 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 90-92 | 2002.5 | 5 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 93 | 1985 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right | N/A | N/A |
| 94 | 1985 | 10 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |

| A-MPR test parameters for NS_24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | Region A | Region B | Region C |
| 95-97 | 1995 | 10 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 98-100 | 2000 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 101 | 2000 | 10 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 103-105 | 1987.5 | 15 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 106-108 | 1997.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 109 | 1997.5 | 15 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 110-112 | 1990 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 75@0 |
| 113 | 1990 | 20 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 114-116 | 1995 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 64@0 |
| 117 | 1995 | 20 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 118 | 1992.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 119 | 1997.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 120-122 | 2002.5 | 5 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 123 | 1985 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | N/A | N/A |
| 124 | 1985 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 125-127 | 1995 | 10 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 128-130 | 2000 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 131 | 2000 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 133-135 | 1987.5 | 15 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 136-138 | 1997.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 139 | 1997.5 | 15 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 140-142 | 1990 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 75@0 |
| 143 | 1990 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 144-146 | 1995 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 64@0 |
| 147 | 1995 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 148-150 | 1992.5 | 5 | Default |  | CP-OFDM | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 151-153 | 1997.5 | 5 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 154-162 | 2002.5 | 5 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 163-165 | 1985 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | N/A | N/A |
| 166-168 | 1985 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 169-177 | 1995 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |

| A-MPR test parameters for NS_24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | Region A | Region B | Region C |
| 178-186 | 2000 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 37@0 |
| 187-189 | 2000 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 190-198 | 1987.5 | 15 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 199-207 | 1997.5 | 15 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 51@0 |
| 208-210 | 1997.5 | 15 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 211-219 | 1990 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 220-222 | 1990 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 223-231 | 1995 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 232-234 | 1995 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 235 | 1992.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 236 | 1997.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 237-239 | 2002.5 | 5 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 240 | 1985 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | N/A | N/A |
| 241 | 1985 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 242-244 | 1995 | 10 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 245-247 | 2000 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 37@0 |
| 248 | 2000 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 250-252 | 1987.5 | 15 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 253-255 | 1997.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 51@0 |
| 256 | 1997.5 | 15 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 257-259 | 1990 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 260 | 1990 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 261-263 | 1995 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 264 | 1995 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 265 | 1992.5 | 5 | Default |  |  | QPSK | Edge_1RB_Right | N/A | N/A |
| 266 | 1997.5 | 5 | Default |  |  | 16 QAM | Edge_1RB_Right | N/A | N/A |
| 267 | 1985 | 10 | Default |  |  | QPSK | Edge_1RB_Left | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: In test IDs with multiple modulations, each UL Modulation shall be tested separately against Range A, B, and C. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-13: Test Configuration table for NS_27 (contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies |  |  |  |  |  |  | Refer to uplink carrier centre frequency (Fc) in test parameters. Associated parameters defined in Table 6.2.3.4.1-13a and 6.2.3.4.1-13b. |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Refer to test parameters (15, 20, 30, 40 MHz) |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |  |
| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 3557.52 | 15 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK |  | Edge_1RB_Left (A3) |  |  |
| 2 | 3557.52 | 15 | Default |  |  | PI/2 BPSK |  | Outer_Full (A3) |  |  |
| 3 | 3692.49 | 15 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A3) |  |  |
| 4 | 3692.49 | 15 | Default |  |  | PI/2 BPSK |  | Outer_Full (A3) |  |  |
| 5 | 3562.5 | 15 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A4) |  |  |
| 6 | 3562.5 | 15 | Default |  |  | PI/2 BPSK |  | Outer_Full (2) |  |  |
| 7 | 3687.48 | 15 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A4) |  |  |
| 8 | 3687.48 | 15 | Default |  |  | PI/2 BPSK |  | Outer_Full (2) |  |  |
| 9 | 3560.01 | 20 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A5) |  |  |
| 10 | 3560.01 | 20 | Default |  |  | PI/2 BPSK |  | Outer_Full (A5) |  |  |
| 11 | 3690 | 20 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A5) |  |  |
| 12 | 3690 | 20 | Default |  |  | PI/2 BPSK |  | Outer_Full (A5) |  |  |
| 13 | 3570 | 20 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A6) |  |  |
| 14 | 3570 | 20 | Default |  |  | PI/2 BPSK |  | Outer_Full (2) |  |  |
| 15 | 3679.98 | 20 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A6) |  |  |
| 16 | 3679.98 | 20 | Default |  |  | PI/2 BPSK |  | Outer_Full (2) |  |  |
| 17 | 3565.02 | 30 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A7) |  |  |
| 18 | 3565.02 | 30 | Default |  |  | PI/2 BPSK |  | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 19 | 3565.02 | 30 | Default |  |  | PI/2 BPSK |  | 75@41 (A1) | 40@21 (A1) | 18@11 (A1) |
| 20 | 3565.02 | 30 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A7) |  |  |
| 21 | 3684.99 | 30 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A7) |  |  |
| 22 | 3684.99 | 30 | Default |  |  | PI/2 BPSK |  | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 23 | 3684.99 | 30 | Default |  |  | PI/2 BPSK |  | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 24 | 3684.99 | 30 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A7) |  |  |
| 25 | 3585 | 30 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A8) |  |  |
| 26 | 3585 | 30 | Default |  |  | PI/2 BPSK |  | Outer_Full (4) |  |  |
| 27 | 3664.98 | 30 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A8) |  |  |
| 28 | 3664.98 | 30 | Default |  |  | PI/2 BPSK |  | Outer_Full (4) |  |  |
| 29 | 3570 | 40 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A7) |  |  |
| 30 | 3570 | 40 | Default |  |  | PI/2 BPSK |  | 150@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 31 | 3570 | 40 | Default |  |  | PI/2 BPSK |  | 96@73 (A1) | 48@35 (A1) | 24@16 (A1) |
| 32 | 3570 | 40 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A7) |  |  |
| 33 | 3679.98 | 40 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A7) |  |  |
| 34 | 3679.98 | 40 | Default |  |  | PI/2 BPSK |  | 135@0 (A2) | 64@0 (A2) | 32@0 (A2) |
| 35 | 3679.98 | 40 | Default |  |  | PI/2 BPSK |  | 96@38 (A1) | 48@18 (A1) | 24@9 (A1) |
| 36 | 3679.98 | 40 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A7) |  |  |
| 37 | 3600 | 40 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Left (A8) |  |  |
| 38 | 3600 | 40 | Default |  |  | PI/2 BPSK |  | Outer_Full (4.5) |  |  |
| 39 | 3649.98 | 40 | Default |  |  | PI/2 BPSK |  | Edge_1RB_Right (A8) |  |  |
| 40 | 3649.98 | 40 | Default |  |  | PI/2 BPSK |  | Outer_Full (4.5) |  |  |
| 41 | 3557.52 | 15 | Default |  |  | QPSK |  | Edge_1RB_Left (A3) |  |  |
| 42 | 3557.52 | 15 | Default |  |  | QPSK |  | Outer_Full (A3) |  |  |
| 43 | 3692.49 | 15 | Default |  |  | QPSK |  | Edge_1RB_Right (A3) |  |  |
| 44 | 3692.49 | 15 | Default |  |  | QPSK |  | Outer_Full (A3) |  |  |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 45 | 3562.5 | 15 | Default |  |  | QPSK | Edge_1RB_Left (A4) |  |  |
| 46 | 3562.5 | 15 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 47 | 3687.48 | 15 | Default |  |  | QPSK | Edge_1RB_Right (A4) |  |  |
| 48 | 3687.48 | 15 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 49 | 3560.01 | 20 | Default |  |  | QPSK | Edge_1RB_Left (A5) |  |  |
| 50 | 3560.01 | 20 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 51 | 3690 | 20 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 52 | 3690 | 20 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 53 | 3570 | 20 | Default |  |  | QPSK | Edge_1RB_Left (A6) |  |  |
| 54 | 3570 | 20 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 55 | 3679.98 | 20 | Default |  |  | QPSK | Edge_1RB_Right (A6) |  |  |
| 56 | 3679.98 | 20 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 57 | 3565.02 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 58 | 3565.02 | 30 | Default |  |  | QPSK | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 59 | 3565.02 | 30 | Default |  |  | QPSK | 75@41 (A1) | 40@21 (A1) | 18@11 (A1) |
| 60 | 3565.02 | 30 | Default |  |  | QPSK | Edge_1RB_Right (A7) |  |  |
| 61 | 3684.99 | 30 | Default |  |  | QPSK | Edge_1RB_Right (A7) |  |  |
| 62 | 3684.99 | 30 | Default |  |  | QPSK | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 63 | 3684.99 | 30 | Default |  |  | QPSK | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 64 | 3684.99 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 65 | 3585 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A8) |  |  |
| 66 | 3585 | 30 | Default |  |  | QPSK | Outer_Full (4) |  |  |
| 67 | 3664.98 | 30 | Default |  |  | QPSK | Edge_1RB_Right (A8) |  |  |
| 68 | 3664.98 | 30 | Default |  |  | QPSK | Outer_Full (4) |  |  |
| 69 | 3570 | 40 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 70 | 3570 | 40 | Default |  |  | QPSK | 150@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 71 | 3570 | 40 | Default |  |  | QPSK | 96@73 (A1) | 48@35 (A1) | 24@16 (A1) |
| 72 | 3570 | 40 | Default |  |  | QPSK | Edge_1RB_Right (A7) |  |  |
| 73 | 3679.98 | 40 | Default |  |  | QPSK | Edge_1RB_Right (A7) |  |  |
| 74 | 3679.98 | 40 | Default |  |  | QPSK | 135@0 (A2) | 64@0 (A2) | 32@0 (A2) |
| 75 | 3679.98 | 40 | Default |  |  | QPSK | 96@38 (A1) | 48@18 (A1) | 24@9 (A1) |
| 76 | 3679.98 | 40 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 77 | 3600 | 40 | Default |  |  | QPSK | Edge_1RB_Left (A8) |  |  |
| 78 | 3600 | 40 | Default |  |  | QPSK | Outer_Full (4.5) |  |  |
| 79 | 3649.98 | 40 | Default |  |  | QPSK | Edge_1RB_Right (A8) |  |  |
| 80 | 3649.98 | 40 | Default |  |  | QPSK | Outer_Full (4.5) |  |  |
| 81 | 3557.52 | 15 | Default |  |  | 16 QAM | Edge_1RB_Left (A3) |  |  |
| 82 | 3557.52 | 15 | Default |  |  | 16 QAM | Outer_Full (A3) |  |  |
| 83 | 3692.49 | 15 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |
| 84 | 3692.49 | 15 | Default |  |  | 16 QAM | Outer_Full (A3) |  |  |
| 85 | 3562.5 | 15 | Default |  |  | 16 QAM | Edge_1RB_Left (A4) |  |  |
| 86 | 3562.5 | 15 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |
| 87 | 3687.48 | 15 | Default |  |  | 16 QAM | Edge_1RB_Right (A4) |  |  |
| 88 | 3687.48 | 15 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |
| 89 | 3560.01 | 20 | Default |  |  | 16 QAM | Edge_1RB_Left (A5) |  |  |
| 90 | 3560.01 | 20 | Default |  |  | 16 QAM | Outer_Full (A5) |  |  |
| 91 | 3690 | 20 | Default |  |  | 16 QAM | Edge_1RB_Right (A5) |  |  |
| 92 | 3690 | 20 | Default |  |  | 16 QAM | Outer_Full (A5) |  |  |
| 93 | 3570 | 20 | Default |  |  | 16 QAM | Edge_1RB_Left (A6) |  |  |
| 94 | 3570 | 20 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 95 | 3679.98 | 20 | Default |  |  | 16 QAM | Edge_1RB_Right (A6) |  |  |
| 96 | 3679.98 | 20 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |
| 97 | 3565.02 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 98 | 3565.02 | 30 | Default |  |  | 16 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 99 | 3565.02 | 30 | Default |  |  | 16 QAM | 75@41 (A1) | 40@21 (A1) | 18@11 (A1) |
| 100 | 3565.02 | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 101 | 3684.99 | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 102 | 3684.99 | 30 | Default |  |  | 16 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 103 | 3684.99 | 30 | Default |  |  | 16 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 104 | 3684.99 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 105 | 3585 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A8) |  |  |
| 106 | 3585 | 30 | Default |  |  | 16 QAM | Outer_Full (4) |  |  |
| 107 | 3664.98 | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A8) |  |  |
| 108 | 3664.98 | 30 | Default |  |  | 16 QAM | Outer_Full (4) |  |  |
| 109 | 3570 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 110 | 3570 | 40 | Default |  |  | 16 QAM | 150@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 111 | 3570 | 40 | Default |  |  | 16 QAM | 96@73 (A1) | 48@35 (A1) | 24@16 (A1) |
| 112 | 3570 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 113 | 3679.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 114 | 3679.98 | 40 | Default |  |  | 16 QAM | 135@0 (A2) | 64@0 (A2) | 32@0 (A2) |
| 115 | 3679.98 | 40 | Default |  |  | 16 QAM | 96@38 (A1) | 48@18 (A1) | 24@9 (A1) |
| 116 | 3679.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 117 | 3600 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A8) |  |  |
| 118 | 3600 | 40 | Default |  |  | 16 QAM | Outer_Full (4.5) |  |  |
| 119 | 3649.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A8) |  |  |
| 120 | 3649.98 | 40 | Default |  |  | 16 QAM | Outer_Full (4.5) |  |  |
| 121 | 3557.52 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left (A3) |  |  |
| 122 | 3557.52 | 15 | Default |  |  | 64 QAM | Outer_Full (A3) |  |  |
| 123 | 3692.49 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |
| 124 | 3692.49 | 15 | Default |  |  | 64 QAM | Outer_Full (A3) |  |  |
| 125 | 3562.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left (A4) |  |  |
| 126 | 3562.5 | 15 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 127 | 3687.48 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right (A4) |  |  |
| 128 | 3687.48 | 15 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 129 | 3560.01 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left (A5) |  |  |
| 130 | 3560.01 | 20 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |
| 131 | 3690 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right (A5) |  |  |
| 132 | 3690 | 20 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |
| 133 | 3570 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left (A6) |  |  |
| 134 | 3570 | 20 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 135 | 3679.98 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right (A6) |  |  |
| 136 | 3679.98 | 20 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 137 | 3565.02 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 138 | 3565.02 | 30 | Default |  |  | 64 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 139 | 3565.02 | 30 | Default |  |  | 64 QAM | 75@41 (A1) | 40@21 (A1) | 18@11 (A1) |
| 140 | 3565.02 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 141 | 3684.99 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 142 | 3684.99 | 30 | Default |  |  | 64 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 143 | 3684.99 | 30 | Default |  |  | 64 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 144 | 3684.99 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 145 | 3585 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A8) |  |  |
| 146 | 3585 | 30 | Default |  |  | 64 QAM | Outer_Full (4) |  |  |
| 147 | 3664.98 | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A8) |  |  |
| 148 | 3664.98 | 30 | Default |  |  | 64 QAM | Outer_Full (4) |  |  |
| 149 | 3570 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 150 | 3570 | 40 | Default |  |  | 64 QAM | 150@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 151 | 3570 | 40 | Default |  |  | 64 QAM | 96@73 (A1) | 48@35 (A1) | 24@16 (A1) |
| 152 | 3570 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A7) |  |  |
| 153 | 3679.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A7) |  |  |
| 154 | 3679.98 | 40 | Default |  |  | 64 QAM | 135@0 (A2) | 64@0 (A2) | 32@0 (A2) |
| 155 | 3679.98 | 40 | Default |  |  | 64 QAM | 96@38 (A1) | 48@18 (A1) | 24@9 (A1) |
| 156 | 3679.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 157 | 3600 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A8) |  |  |
| 158 | 3600 | 40 | Default |  |  | 64 QAM | Outer_Full (4.5) |  |  |
| 159 | 3649.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A8) |  |  |
| 160 | 3649.98 | 40 | Default |  |  | 64 QAM | Outer_Full (4.5) |  |  |
| 161 | 3557.52 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |
| 162 | 3557.52 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 163 | 3692.49 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 164 | 3692.49 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 165 | 3562.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left (A4) |  |  |
| 166 | 3562.5 | 15 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 167 | 3687.48 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right (A4) |  |  |
| 168 | 3687.48 | 15 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 169 | 3560.01 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left (A5) |  |  |
| 170 | 3560.01 | 20 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |
| 171 | 3690 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |
| 172 | 3690 | 20 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |
| 173 | 3570 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left (A6) |  |  |
| 174 | 3570 | 20 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 175 | 3679.98 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right (A6) |  |  |
| 176 | 3679.98 | 20 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 177 | 3565.02 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 178 | 3565.02 | 30 | Default |  |  | 256 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 179 | 3565.02 | 30 | Default |  |  | 256 QAM | 75@41 (A1) | 40@21 (A1) | 18@11 (A1) |
| 180 | 3565.02 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 181 | 3684.99 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 182 | 3684.99 | 30 | Default |  |  | 256 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 183 | 3684.99 | 30 | Default |  |  | 256 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 184 | 3684.99 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 185 | 3585 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A8) |  |  |
| 186 | 3585 | 30 | Default |  |  | 256 QAM | Outer_Full (4) |  |  |
| 187 | 3664.98 | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A8) |  |  |
| 188 | 3664.98 | 30 | Default |  |  | 256 QAM | Outer_Full (4) |  |  |
| 189 | 3570 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 190 | 3570 | 40 | Default |  |  | 256 QAM | 150@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 191 | 3570 | 40 | Default |  |  | 256 QAM | 96@73 (A1) | 48@35 (A1) | 24@16 (A1) |
| 192 | 3570 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A7) |  |  |
| 193 | 3679.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A7) |  |  |
| 194 | 3679.98 | 40 | Default |  |  | 256 QAM | 135@0 (A2) | 64@0 (A2) | 32@0 (A2) |
| 195 | 3679.98 | 40 | Default |  |  | 256 QAM | 96@38 (A1) | 48@18 (A1) | 24@9 (A1) |
| 196 | 3679.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 197 | 3600 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A8) |  |  |
| 198 | 3600 | 40 | Default |  |  | 256 QAM | Outer_Full (4.5) |  |  |
| 199 | 3649.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A8) |  |  |
| 200 | 3649.98 | 40 | Default |  |  | 256 QAM | Outer_Full (4.5) |  |  |
| 201 | 3557.52 | 15 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A3) |  |  |
| 202 | 3557.52 | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| 203 | 3692.49 | 15 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 204 | 3692.49 | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| 205 | 3562.5 | 15 | Default |  |  | QPSK | Edge_1RB_Left (A4) |  |  |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 206 | 3562.5 | 15 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 207 | 3687.48 | 15 | Default |  |  | QPSK | Edge_1RB_Right (A4) |  |  |
| 208 | 3687.48 | 15 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 209 | 3560.01 | 20 | Default |  |  | QPSK | Edge_1RB_Left (A5) |  |  |
| 210 | 3560.01 | 20 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 211 | 3690 | 20 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 212 | 3690 | 20 | Default |  |  | QPSK | Outer_Full (A5) |  |  |
| 213 | 3570 | 20 | Default |  |  | QPSK | Edge_1RB_Left (A6) |  |  |
| 214 | 3570 | 20 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 215 | 3679.98 | 20 | Default |  |  | QPSK | Edge_1RB_Right (A6) |  |  |
| 216 | 3679.98 | 20 | Default |  |  | QPSK | Outer_Full (2) |  |  |
| 217 | 3565.02 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 218 | 3565.02 | 30 | Default |  |  | QPSK | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 219 | 3565.02 | 30 | Default |  |  | QPSK | 74@41 (A1) | 24@54 (A1) | 11@27 (A1) |
| 220 | 3565.02 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 221 | 3684.99 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 222 | 3684.99 | 30 | Default |  |  | QPSK | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 223 | 3684.99 | 30 | Default |  |  | QPSK | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 224 | 3684.99 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 225 | 3585 | 30 | Default |  |  | QPSK | Edge_1RB_Left (A8) |  |  |
| 226 | 3585 | 30 | Default |  |  | QPSK | Outer_Full (4) |  |  |
| 227 | 3664.98 | 30 | Default |  |  | QPSK | Edge_1RB_Right (A8) |  |  |
| 228 | 3664.98 | 30 | Default |  |  | QPSK | Outer_Full (4) |  |  |
| 229 | 3570 | 40 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 230 | 3570 | 40 | Default |  |  | QPSK | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 231 | 3570 | 40 | Default |  |  | QPSK | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 232 | 3570 | 40 | Default |  |  | QPSK | Edge_1RB_Right (A7) |  |  |
| 233 | 3679.98 | 40 | Default |  |  | QPSK | Edge_1RB_Right (A7) |  |  |
| 234 | 3679.98 | 40 | Default |  |  | QPSK | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 235 | 3679.98 | 40 | Default |  |  | QPSK | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 236 | 3679.98 | 40 | Default |  |  | QPSK | Edge_1RB_Left (A7) |  |  |
| 237 | 3600 | 40 | Default |  |  | QPSK | Edge_1RB_Left (A8) |  |  |
| 238 | 3600 | 40 | Default |  |  | QPSK | Outer_Full (4.5) |  |  |
| 239 | 3649.98 | 40 | Default |  |  | QPSK | Edge_1RB_Right (A8) |  |  |
| 240 | 3649.98 | 40 | Default |  |  | QPSK | Outer_Full (4.5) |  |  |
| 241 | 3557.52 | 15 | Default |  |  | 16 QAM | Edge_1RB_Left (A3) |  |  |
| 242 | 3557.52 | 15 | Default |  |  | 16 QAM | Outer_Full (A3) |  |  |
| 243 | 3692.49 | 15 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |
| 244 | 3692.49 | 15 | Default |  |  | 16 QAM | Outer_Full (A3) |  |  |
| 245 | 3562.5 | 15 | Default |  |  | 16 QAM | Edge_1RB_Left (A4) |  |  |
| 246 | 3562.5 | 15 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |
| 247 | 3687.48 | 15 | Default |  |  | 16 QAM | Edge_1RB_Right (A4) |  |  |
| 248 | 3687.48 | 15 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |
| 249 | 3560.01 | 20 | Default |  |  | 16 QAM | Edge_1RB_Left (A5) |  |  |
| 250 | 3560.01 | 20 | Default |  |  | 16 QAM | Outer_Full (A5) |  |  |
| 251 | 3690 | 20 | Default |  |  | 16 QAM | Edge_1RB_Right (A5) |  |  |
| 252 | 3690 | 20 | Default |  |  | 16 QAM | Outer_Full (A5) |  |  |
| 253 | 3570 | 20 | Default |  |  | 16 QAM | Edge_1RB_Left (A6) |  |  |
| 254 | 3570 | 20 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 255 | 3679.98 | 20 | Default |  |  | 16 QAM | Edge_1RB_Right (A6) |  |  |
| 256 | 3679.98 | 20 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |
| 257 | 3565.02 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 258 | 3565.02 | 30 | Default |  |  | 16 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 259 | 3565.02 | 30 | Default |  |  | 16 QAM | 74@41 (A1) | 24@54 (A1) | 11@27 (A1) |
| 260 | 3565.02 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 261 | 3684.99 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 262 | 3684.99 | 30 | Default |  |  | 16 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 263 | 3684.99 | 30 | Default |  |  | 16 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 264 | 3684.99 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 265 | 3585 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A8) |  |  |
| 266 | 3585 | 30 | Default |  |  | 16 QAM | Outer_Full (4) |  |  |
| 267 | 3664.98 | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A8) |  |  |
| 268 | 3664.98 | 30 | Default |  |  | 16 QAM | Outer_Full (4) |  |  |
| 269 | 3570 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 270 | 3570 | 40 | Default |  |  | 16 QAM | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 271 | 3570 | 40 | Default |  |  | 16 QAM | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 272 | 3570 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 273 | 3679.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 274 | 3679.98 | 40 | Default |  |  | 16 QAM | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 275 | 3679.98 | 40 | Default |  |  | 16 QAM | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 276 | 3679.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 277 | 3600 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A8) |  |  |
| 278 | 3600 | 40 | Default |  |  | 16 QAM | Outer_Full (4.5) |  |  |
| 279 | 3649.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A8) |  |  |
| 280 | 3649.98 | 40 | Default |  |  | 16 QAM | Outer_Full (4.5) |  |  |
| 281 | 3557.52 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left (A3) |  |  |
| 282 | 3557.52 | 15 | Default |  |  | 64 QAM | Outer_Full (A3) |  |  |
| 283 | 3692.49 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |
| 284 | 3692.49 | 15 | Default |  |  | 64 QAM | Outer_Full (A3) |  |  |
| 285 | 3562.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left (A4) |  |  |
| 286 | 3562.5 | 15 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 287 | 3687.48 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right (A4) |  |  |
| 288 | 3687.48 | 15 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 289 | 3560.01 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left (A5) |  |  |
| 290 | 3560.01 | 20 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |
| 291 | 3690 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right (A5) |  |  |
| 292 | 3690 | 20 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |
| 293 | 3570 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left (A6) |  |  |
| 294 | 3570 | 20 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 295 | 3679.98 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right (A6) |  |  |
| 296 | 3679.98 | 20 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 297 | 3565.02 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 298 | 3565.02 | 30 | Default |  |  | 64 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 299 | 3565.02 | 30 | Default |  |  | 64 QAM | 74@41 (A1) | 24@54 (A1) | 11@27 (A1) |
| 300 | 3565.02 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 301 | 3684.99 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 302 | 3684.99 | 30 | Default |  |  | 64 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 303 | 3684.99 | 30 | Default |  |  | 64 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 304 | 3684.99 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 305 | 3585 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A8) |  |  |
| 306 | 3585 | 30 | Default |  |  | 64 QAM | Outer_Full (4) |  |  |
| 307 | 3664.98 | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A8) |  |  |
| 308 | 3664.98 | 30 | Default |  |  | 64 QAM | Outer_Full (4) |  |  |
| 309 | 3570 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 310 | 3570 | 40 | Default |  |  | 64 QAM | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 311 | 3570 | 40 | Default |  |  | 64 QAM | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 312 | 3570 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A7) |  |  |
| 313 | 3679.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A7) |  |  |
| 314 | 3679.98 | 40 | Default |  |  | 64 QAM | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 315 | 3679.98 | 40 | Default |  |  | 64 QAM | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 316 | 3679.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 317 | 3600 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A8) |  |  |
| 318 | 3600 | 40 | Default |  |  | 64 QAM | Outer_Full (4.5) |  |  |
| 319 | 3649.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A8) |  |  |
| 320 | 3649.98 | 40 | Default |  |  | 64 QAM | Outer_Full (4.5) |  |  |
| 321 | 3557.52 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |
| 322 | 3557.52 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 323 | 3692.49 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 324 | 3692.49 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 325 | 3562.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left (A4) |  |  |
| 326 | 3562.5 | 15 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 327 | 3687.48 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right (A4) |  |  |
| 328 | 3687.48 | 15 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 329 | 3560.01 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left (A5) |  |  |
| 330 | 3560.01 | 20 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |
| 331 | 3690 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |
| 332 | 3690 | 20 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |
| 333 | 3570 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left (A6) |  |  |
| 334 | 3570 | 20 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 335 | 3679.98 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right (A6) |  |  |
| 336 | 3679.98 | 20 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 337 | 3565.02 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 338 | 3565.02 | 30 | Default |  |  | 256 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 339 | 3565.02 | 30 | Default |  |  | 256 QAM | 74@41 (A1) | 24@54 (A1) | 11@27 (A1) |
| 340 | 3565.02 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 341 | 3684.99 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 342 | 3684.99 | 30 | Default |  |  | 256 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 343 | 3684.99 | 30 | Default |  |  | 256 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 344 | 3684.99 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 345 | 3585 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A8) |  |  |
| 346 | 3585 | 30 | Default |  |  | 256 QAM | Outer_Full (4) |  |  |
| 347 | 3664.98 | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A8) |  |  |
| 348 | 3664.98 | 30 | Default |  |  | 256 QAM | Outer_Full (4) |  |  |
| 349 | 3570 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 350 | 3570 | 40 | Default |  |  | 256 QAM | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 351 | 3570 | 40 | Default |  |  | 256 QAM | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 352 | 3570 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A7) |  |  |
| 353 | 3679.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A7) |  |  |
| 354 | 3679.98 | 40 | Default |  |  | 256 QAM | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 355 | 3679.98 | 40 | Default |  |  | 256 QAM | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 356 | 3679.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 357 | 3600 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A8) |  |  |
| 358 | 3600 | 40 | Default |  |  | 256 QAM | Outer_Full (4.5) |  |  |
| 359 | 3649.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A8) |  |  |
| 360 | 3649.98 | 40 | Default |  |  | 256 QAM | Outer_Full (4.5) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-13a: Test frequencies for NR band n48 and NS_27 (SCS=15 kHz, ΔFRaster = 15kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 79 | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
|  |  | 3562.5 | 637500 | 3555.39 | 637026 |  |  | 7888 | 637344 | 6 | 0 | 1 (6) | 6 |
|  |  | 3687.48 | 645832 | 3680.37 | 645358 |  |  | 7975 | 645696 | 2 | 2 | 1 (6) | 8 |
|  |  | 3692.49 | 646166 | 3685.38 | 645692 |  |  | 7978 | 645984 | 4 | 2 | 0 (2) | 4 |
| 20 | 106 | 3560.01 | 637334 | 3550.47 | 636698 |  |  | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
|  |  | 3570 | 638000 | 3560.46 | 637364 |  |  | 7891 | 637632 | 4 | 0 | 0 (2) | 2 |
|  |  | 3679.98 | 645332 | 3670.44 | 644696 |  |  | 7968 | 645024 | 4 | 1 | 1 (6) | 7 |
|  |  | 3690 | 646000 | 3680.46 | 645364 |  |  | 7975 | 645696 | 8 | 1 | 1 (6) | 7 |
| 30 | 160 | 3565.02 | 637668 | 3550.62 | 636708 |  |  | 7885 | 637056 | 0 | 3 | 1 (6) | 9 |
|  |  | 3585 | 639000 | 3570.6 | 638040 |  |  | 7898 | 638304 | 0 | 0 | 0 (2) | 2 |
|  |  | 3664.98 | 644332 | 3650.58 | 643372 |  |  | 7954 | 643680 | 8 | 3 | 0 (2) | 5 |
|  |  | 3684.99 | 645666 | 3670.59 | 644706 |  |  | 7968 | 645024 | 6 | 0 | 1 (6) | 6 |
| 40 | 216 | 3570 | 638000 | 3550.56 | 636704 |  |  | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
|  |  | 3600 | 640000 | 3580.56 | 638704 |  |  | 7905 | 638976 | 8 | 0 | 0 (2) | 2 |
|  |  | 3649.98 | 643332 | 3630.54 | 642036 |  |  | 7940 | 642336 | 0 | 3 | 0 (2) | 5 |
|  |  | 3679.98 | 645332 | 3660.54 | 644036 |  |  | 7961 | 644352 | 4 | 0 | 1 (6) | 6 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-13b: Test frequencies for NR band n48 and NS_27 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 38 | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
|  |  | 3562.5 | 637500 | 3555.66 | 637044 |  |  | 7888 | 637344 | 12 | 0 | 2 (2) | 4 |
|  |  | 3687.48 | 645832 | 3680.64 | 645376 |  |  | 7975 | 645696 | 8 | 0 | 3 (3) | 6 |
|  |  | 3692.49 | 646166 | 3685.65 | 645710 |  |  | 7978 | 645984 | 10 | 0 | 1 (1) | 2 |
| 20 | 51 | 3560.01 | 637334 | 3550.83 | 636722 |  |  | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
|  |  | 3570 | 638000 | 3560.82 | 637388 |  |  | 7891 | 637632 | 4 | 0 | 0 (0) | 0 |
|  |  | 3679.98 | 645332 | 3670.8 | 644720 |  |  | 7968 | 645024 | 16 | 0 | 2 (2) | 4 |
|  |  | 3690 | 646000 | 3680.82 | 645388 |  |  | 7975 | 645696 | 20 | 0 | 2 (2) | 4 |
| 30 | 78 | 3565.02 | 637668 | 3550.98 | 636732 |  |  | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
|  |  | 3585 | 639000 | 3570.96 | 638064 |  |  | 7898 | 638304 | 0 | 0 | 0 (0) | 0 |
|  |  | 3664.98 | 644332 | 3650.94 | 643396 |  |  | 7954 | 643680 | 20 | 0 | 1 (1) | 2 |
|  |  | 3684.99 | 645666 | 3670.95 | 644730 |  |  | 7968 | 645024 | 6 | 0 | 2 (2) | 4 |
| 40 | 106 | 3570 | 638000 | 3550.92 | 636728 |  |  | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
|  |  | 3600 | 640000 | 3580.92 | 638728 |  |  | 7905 | 638976 | 8 | 0 | 0 (0) | 0 |
|  |  | 3649.98 | 643332 | 3630.9 | 642060 |  |  | 7940 | 642336 | 12 | 0 | 1 (1) | 2 |
|  |  | 3679.98 | 645332 | 3660.9 | 644060 |  |  | 7961 | 644352 | 4 | 0 | 2 (2) | 4 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-14: Test Configuration table for NS_40

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest (5 MHz) |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_40 |  |  |  |  |  |  |  |
| Test ID | Freq | Ch BW | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Low | Default | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Edge_1RB_Left |
| 2 | Default | Default | Default |  |  | QPSK | Inner_Full |
| 3 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 4 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 5 | Default | Default | Default |  |  | 16 QAM | Inner_Full |
| 6 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 7 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 8 | Default | Default | Default |  |  | 64 QAM | Inner_Full |
| 9 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 10 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 11 | Default | Default | Default |  |  | 256 QAM | Inner_Full |
| 12 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| 13 | Low | Default | Default |  | CP-OFDM | QPSK | Edge_1RB_Left |
| 14 | Default | Default | Default |  |  | QPSK | Inner_Full |
| 15 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 16 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 17 | Default | Default | Default |  |  | 16 QAM | Inner_Full |
| 18 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 19 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 20 | Default | Default | Default |  |  | 64 QAM | Inner_Full |
| 21 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 22 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 23 | Default | Default | Default |  |  | 256 QAM | Inner_Full |
| 24 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-15: Test Configuration table for NS_41

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | See Ch BW column |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_41 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 1437 | 10 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK | Outer_Full |
| 2 | 1437 | 10 | Default |  |  | PI/2 BPSK | Edge_1RB_Left |
| 3 | 1439.5 | 15 | Default |  |  | PI/2 BPSK | Outer_Full |
| 4 | 1439.5 | 15 | Default |  |  | PI/2 BPSK | Edge_1RB_Left |
| 5 | 1442 | 20 | Default |  |  | PI/2 BPSK | Outer_Full |
| 6 | 1442 | 20 | Default |  |  | PI/2 BPSK | Edge_1RB_Left |
| 7 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | PI/2 BPSK | Outer_Full |
| 8 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | PI/2 BPSK | Edge_1RB_Left |
| 9 | 1437 | 10 | Default |  |  | QPSK | Outer_Full |
| 10 | 1437 | 10 | Default |  |  | QPSK | Edge_1RB_Left |
| 11 | 1439.5 | 15 | Default |  |  | QPSK | Outer_Full |
| 12 | 1439.5 | 15 | Default |  |  | QPSK | Edge_1RB_Left |
| 13 | 1442 | 20 | Default |  |  | QPSK | Outer_Full |
| 14 | 1442 | 20 | Default |  |  | QPSK | Edge_1RB_Left |
| 15 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | QPSK | Outer_Full |
| 16 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | QPSK | Edge_1RB_Left |
| 17 | 1437 | 10 | Default |  |  | 16 QAM | Outer_Full |
| 18 | 1437 | 10 | Default |  |  | 16 QAM | Edge_1RB_Left |
| 19 | 1439.5 | 15 | Default |  |  | 16 QAM | Outer_Full |
| 20 | 1439.5 | 15 | Default |  |  | 16 QAM | Edge_1RB_Left |
| 21 | 1442 | 20 | Default |  |  | 16 QAM | Outer_Full |
| 22 | 1442 | 20 | Default |  |  | 16 QAM | Edge_1RB_Left |
| 23 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 16 QAM | Outer_Full |
| 24 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 16 QAM | Edge_1RB_Left |
| 25 | 1437 | 10 | Default |  |  | 64 QAM | Outer_Full |
| 26 | 1437 | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |
| 27 | 1439.5 | 15 | Default |  |  | 64 QAM | Outer_Full |
| 28 | 1439.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |
| 29 | 1442 | 20 | Default |  |  | 64 QAM | Outer_Full |
| 30 | 1442 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |
| 31 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 64 QAM | Outer_Full |
| 32 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 64 QAM | Edge_1RB_Left |
| 33 | 1437 | 10 | Default |  |  | 256 QAM | Outer_Full |
| 34 | 1437 | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |
| 35 | 1439.5 | 15 | Default |  |  | 256 QAM | Outer_Full |
| 36 | 1439.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |
| 37 | 1442 | 20 | Default |  |  | 256 QAM | Outer_Full |
| 38 | 1442 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left |
| 39 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 256 QAM | Outer_Full |
| 40 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 256 QAM | Edge_1RB_Left |
| 41 | 1437 | 10 | Default |  | CP-OFDM | QPSK | Outer_Full |
| 42 | 1437 | 10 | Default |  |  | QPSK | Edge_1RB_Left |
| 43 | 1439.5 | 15 | Default |  |  | QPSK | Outer_Full |

| A-MPR test parameters for NS_41 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 44 | 1439.5 | 15 | Default |  |  | QPSK | Edge_1RB_Left |
| 45 | 1442 | 20 | Default |  |  | QPSK | Outer_Full |
| 46 | 1442 | 20 | Default |  |  | QPSK | Edge_1RB_Left |
| 47 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | QPSK | Outer_Full |
| 48 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | QPSK | Edge_1RB_Left |
| 49 | 1437 | 10 | Default |  |  | 16 QAM | Outer_Full |
| 50 | 1437 | 10 | Default |  |  | 16 QAM | Edge_1RB_Left |
| 51 | 1439.5 | 15 | Default |  |  | 16 QAM | Outer_Full |
| 52 | 1439.5 | 15 | Default |  |  | 16 QAM | Edge_1RB_Left |
| 53 | 1442 | 20 | Default |  |  | 16 QAM | Outer_Full |
| 54 | 1442 | 20 | Default |  |  | 16 QAM | Edge_1RB_Left |
| 55 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 16 QAM | Outer_Full |
| 56 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 16 QAM | Edge_1RB_Left |
| 57 | 1437 | 10 | Default |  |  | 64 QAM | Outer_Full |
| 58 | 1437 | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |
| 59 | 1439.5 | 15 | Default |  |  | 64 QAM | Outer_Full |
| 60 | 1439.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |
| 61 | 1442 | 20 | Default |  |  | 64 QAM | Outer_Full |
| 62 | 1442 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |
| 63 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 64 QAM | Outer_Full |
| 64 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 64 QAM | Edge_1RB_Left |
| 65 | 1437 | 10 | Default |  |  | 256 QAM | Outer_Full |
| 66 | 1437 | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |
| 67 | 1439.5 | 15 | Default |  |  | 256 QAM | Outer_Full |
| 68 | 1439.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |
| 69 | 1442 | 20 | Default |  |  | 256 QAM | Outer_Full |
| 70 | 1442 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left |
| 71 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 256 QAM | Outer_Full |
| 72 | (Note 3) | Highest (≤ 60 MHz) | Default |  |  | 256 QAM | Edge_1RB_Left |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Select Fc = 1452 MHz for 40 MHz Ch BW, Fc = 1457 MHz for 50 MHz Ch BW and Fc = 1462 MHz for 60 MHz Ch BW. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-16: Test Configuration table for NS_42

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | See Ch BW column |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_42 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1, 3) |
| 1 | 1514.5 | 5 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK |  | Outer_Full |
| 2 | 1514.5 | 5 | Default |  |  | PI/2 BPSK |  | Outer (LCRB@RBstart) |
| 3 | 1514.5 | 5 | Default |  |  | PI/2 BPSK |  | Inner (LCRB@RBstart) |
| 4 | (Note 3) | Highest (≤ 60) | Default |  |  | PI/2 BPSK |  | Outer_Full |
| 5 | (Note 3) | Highest (≤ 60) | Default |  |  | PI/2 BPSK |  | Outer (LCRB@RBstart) |
| 6 | (Note 3) | Highest (≤ 60) | Default |  |  | PI/2 BPSK |  | Inner (LCRB@RBstart) |
| 7 | 1514.5 | 5 | Default |  |  | QPSK |  | Outer_Full |
| 8 | 1514.5 | 5 | Default |  |  | QPSK |  | Outer (LCRB@RBstart) |
| 9 | 1514.5 | 5 | Default |  |  | QPSK |  | Inner (LCRB@RBstart) |
| 10 | (Note 3) | Highest (≤ 60) | Default |  |  | QPSK |  | Outer_Full |
| 11 | (Note 3) | Highest (≤ 60) | Default |  |  | QPSK |  | Outer (LCRB@RBstart) |
| 12 | (Note 3) | Highest (≤ 60) | Default |  |  | QPSK |  | Inner LCRB@RBstart) |
| 13 | 1514.5 | 5 | Default |  |  | 16 QAM |  | Outer_Full |
| 14 | 1514.5 | 5 | Default |  |  | 16 QAM |  | Outer (LCRB@RBstart) |
| 15 | 1514.5 | 5 | Default |  |  | 16 QAM |  | Inner (LCRB@RBstart) |
| 16 | (Note 3) | Highest (≤ 60) | Default |  |  | 16 QAM |  | Outer_Full |
| 17 | (Note 3) | Highest (≤ 60) | Default |  |  | 16 QAM |  | Outer (LCRB@RBstart) |
| 18 | (Note 3) | Highest (≤ 60) | Default |  |  | 16 QAM |  | Inner (LCRB@RBstart) |
| 19 | 1514.5 | 5 | Default |  |  | 64 QAM |  | Outer_Full |
| 20 | 1514.5 | 5 | Default |  |  | 64 QAM |  | Outer (LCRB@RBstart) |
| 21 | 1514.5 | 5 | Default |  |  | 64 QAM |  | Inner (LCRB@RBstart) |
| 22 | (Note 3) | Highest (≤ 60) | Default |  |  | 64 QAM |  | Outer_Full |
| 23 | (Note 3) | Highest (≤ 60) | Default |  |  | 64 QAM |  | Outer (LCRB@RBstart) |
| 24 | (Note 3) | Highest (≤ 60) | Default |  |  | 64 QAM |  | Inner (LCRB@RBstart) |
| 25 | 1514.5 | 5 | Default |  |  | 256 QAM |  | Outer_Full |
| 26 | 1514.5 | 5 | Default |  |  | 256 QAM |  | Outer (LCRB@RBstart) |
| 27 | 1514.5 | 5 | Default |  |  | 256 QAM |  | Inner (LCRB@RBstart) |
| 28 | (Note 3) | Highest (≤ 60) | Default |  |  | 256 QAM |  | Outer_Full |
| 29 | (Note 3) | Highest (≤ 60) | Default |  |  | 256 QAM |  | Outer (LCRB@RBstart) |
| 30 | (Note 3) | Highest (≤ 60) | Default |  |  | 256 QAM |  | Inner (LCRB@RBstart) |
| 31 | 1514.5 | 5 | Default |  | CP-OFDM | QPSK |  | Outer_Full |
| 32 | 1514.5 | 5 | Default |  |  | QPSK |  | Outer (LCRB@RBstart) |
| 33 | 1514.5 | 5 | Default |  |  | QPSK |  | Inner (LCRB@RBstart) |
| 34 | (Note 3) | Highest (≤ 60) | Default |  |  | QPSK |  | Outer_Full |
| 35 | (Note 3) | Highest (≤ 60) | Default |  |  | QPSK |  | Outer (LCRB@RBstart) |
| 36 | (Note 3) | Highest (≤ 60) | Default |  |  | QPSK |  | Inner (LCRB@RBstart) |
| 37 | 1514.5 | 5 | Default |  |  | 16 QAM |  | Outer_Full |
| 38 | 1514.5 | 5 | Default |  |  | 16 QAM |  | Outer (LCRB@RBstart) |
| 39 | 1514.5 | 5 | Default |  |  | 16 QAM |  | Inner (LCRB@RBstart) |
| 40 | (Note 3) | Highest (≤ 60) | Default |  |  | 16 QAM |  | Outer_Full |
| 41 | (Note 3) | Highest (≤ 60) | Default |  |  | 16 QAM |  | Outer (LCRB@RBstart) |
| 42 | (Note 3) | Highest (≤ 60) | Default |  |  | 16 QAM |  | Inner (LCRB@RBstart) |
| 43 | 1514.5 | 5 | Default |  |  | 64 QAM |  | Outer_Full |
| 44 | 1514.5 | 5 | Default |  |  | 64 QAM |  | Outer (LCRB@RBstart) |
| 45 | 1514.5 | 5 | Default |  |  | 64 QAM |  | Inner (LCRB@RBstart) |
| 46 | (Note 3) | Highest (≤ 60) | Default |  |  | 64 QAM |  | Outer_Full |
| 47 | (Note 3) | Highest (≤ 60) | Default |  |  | 64 QAM |  | Outer (LCRB@RBstart) |
| 48 | (Note 3) | Highest (≤ 60) | Default |  |  | 64 QAM |  | Inner (LCRB@RBstart) |
| 49 | 1514.5 | 5 | Default |  |  | 256 QAM |  | Outer_Full |
| 50 | 1514.5 | 5 | Default |  |  | 256 QAM |  | Outer (LCRB@RBstart) |

| A-MPR test parameters for NS_42 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1, 3) |
| 51 | 1514.5 | 5 | Default |  |  | 256 QAM | Inner (LCRB@RBstart) |
| 52 | (Note 3) | Highest (≤ 60) | Default |  |  | 256 QAM | Outer_Full |
| 53 | (Note 3) | Highest (≤ 60) | Default |  |  | 256 QAM | Outer (LCRB@RBstart) |
| 54 | (Note 3) | Highest (≤ 60) | Default |  |  | 256 QAM | Inner (LCRB@RBstart) |
| NOTE 1: The Outer_Full RB configuration is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For 10 – 60 MHz channel bandwidth the RB allocation and test frequency are specified in Table 6.2.3.4.1-16a. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-16a: Uplink configuration for NS_42

| Ch BW (MHz) | Fc (MHz) | SCS (kHz) | OFDM | Outer LCRB@RBstart | Inner LCRB@RBstart |
| --- | --- | --- | --- | --- | --- |
| 5 | 1514.5 | 15 | DFT-s | 16@0 | 9@4 |
|  |  |  | CP | 17@0 | 9@4 |
|  |  | 30 | DFT-s | 8@0 | 5@2 |
|  |  |  | CP | 8@0 | 5@2 |
| 10 | 1512.0 | 15 | DFT-s | 32@0 | 9@4 |
|  |  |  | CP | 34@0 | 9@4 |
|  |  | 30 | DFT-s | 16@0 | 5@2 |
|  |  |  | CP | 17@0 | 5@2 |
|  |  | 60 | DFT-s | 8@0 | 3@1 |
|  |  |  | CP | 8@0 | 3@1 |
| 15 | 1509.5 | 15 | DFT-s | 50@0 | 32@16 |
|  |  |  | CP | 51@0 | 33@16 |
|  |  | 30 | DFT-s | 25@0 | 16@8 |
|  |  |  | CP | 25@0 | 17@8 |
|  |  | 60 | DFT-s | 12@0 | 9@4 |
|  |  |  | CP | 12@0 | 9@4 |
| 20 | 1507.0 | 15 | DFT-s | 64@0 | 48@24 |
|  |  |  | CP | 68@0 | 49@24 |
|  |  | 30 | DFT-s | 32@0 | 25@12 |
|  |  |  | CP | 34@0 | 25@12 |
|  |  | 60 | DFT-s | 16@0 | 12@6 |
|  |  |  | CP | 17@0 | 12@6 |
| 40 | 1497.0 | 15 | DFT-s | 135@0 | 54@29 |
|  |  |  | CP | 137@0 | 59@29 |
|  |  | 30 | DFT-s | 64@0 | 27@14 |
|  |  |  | CP | 68@0 | 29@14 |
|  |  | 60 | DFT-s | 32@0 | 15@7 |
|  |  |  | CP | 34@0 | 15@7 |
| 50 | 1492.0 | 15 | DFT-s | 162@0 | 75@39 |
|  |  |  | CP | 172@0 | 79@39 |
|  |  | 30 | DFT-s | 81@0 | 36@19 |
|  |  |  | CP | 86@0 | 39@19 |
|  |  | 60 | DFT-s | 40@0 | 18@9 |
|  |  |  | CP | 43@0 | 19@9 |
| 60 | 1487.0 | 30 | DFT-s | 100@0 | 36@19 |
|  |  |  | CP | 103@0 | 39@19 |
|  |  | 60 | DFT-s | 50@0 | 18@9 |
|  |  |  | CP | 51@0 | 19@9 |

Table 6.2.3.4.1-17: Test Configuration table for NS_47 power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 30 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest (Note 6) |  |  |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 (Note 3) | Default | 30 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK | Edge_1RB_Left (A1) |  |  |  |
| 2 (Note 3) | Default | 30 | Default |  |  | PI/2 BPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 3 (Note 3) | Default | 30 | Default |  |  | PI/2 BPSK | Edge_1RB_Right (A3) |  |  |  |
| 4 (Note 3) | Default | 30 | Default |  |  | PI/2 BPSK | Outer_Full (A2) |  |  |  |
| 5 (Note 3) | Default | 30 | Default |  |  | PI/2 BPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 6 (Note 3) | Default | 30 | Default |  |  | PI/2 BPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 7 (Note 3) | Default | 30 | Default |  |  | PI/2 BPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 8 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | Edge_1RB_Left (A1) |  |  |  |
| 9 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 10 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | Edge_1RB_Right (A3) |  |  |  |
| 11 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | Outer_Full (A2) |  |  |  |
| 12 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 13 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 14 (Note 4) | Default | 30 | Default |  |  | PI/2 BPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 15 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 16 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 17 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 18 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 19 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 20 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 21 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 22 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 23 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 24 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |  |
| 25 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 26 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 27 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 28 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 29 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |
| 30 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 31 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |  |
| 32 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |  |
| 33 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 34 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 35 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 36 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |  |
| 37 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 38 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 39 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 40 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 41 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 42 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 43 | Default | 30 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 44 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 45 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 46 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 47 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 48 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 49 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 50 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |

| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 51 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 52 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |
| 53 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |
| 54 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 55 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 56 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 57 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |
| 58 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 59 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |
| 60 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |
| 61 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 62 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 63 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 64 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |
| 65 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 66 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 67 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |
| 68 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 69 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 70 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Void NOTE 6: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-17a: Test Configuration table for NS_47 power class 3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 30 MHz |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest (Note 3) |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation |  | RB allocation (Note 1) |
| 1 | Default | 30 | Default |  | CP-OFDM | QPSK | Outer_Full (A2) |
| 2 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |
| 3 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |
| 4 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2.2.4.1-4, since offsetToCarrier = 0 in Tables 6.2.3.4.1-18a and 6.2.3.4.1-18b. NOTE 2: Void NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. NOTE 4: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-18: Test Configuration table for NS_47 power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 30 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 30 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK | Edge_1RB_Left (A1) |  |  |  |
| 2 | Default | 30 | Default |  |  | PI/2 BPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 3 | Default | 30 | Default |  |  | PI/2 BPSK | Edge_1RB_Right (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | PI/2 BPSK | Outer_Full (A2) |  |  |  |
| 5 | Default | 30 | Default |  |  | PI/2 BPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 6 | Default | 30 | Default |  |  | PI/2 BPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 7 | Default | 30 | Default |  |  | PI/2 BPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 8 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 9 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 10 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 11 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 12 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 13 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 14 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 15 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 16 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 17 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |  |
| 18 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 19 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 20 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 21 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 22 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |
| 23 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 24 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |  |
| 25 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |  |
| 26 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 27 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 28 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 29 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |  |
| 30 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 31 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 32 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 33 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 34 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 35 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 36 | Default | 30 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 37 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 38 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 39 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 40 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 41 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 42 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 43 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 44 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 45 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |  |
| 46 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 47 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 48 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 49 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 50 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |

| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
| 51 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 52 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |
| 53 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |
| 54 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 55 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 56 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 57 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |
| 58 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 59 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 60 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |
| 61 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 62 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 63 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-18-1A: Test Configuration table for NS_47 power class 2 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 30 MHz |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest (Note 3) |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation |  | RB allocation (Note 1) |
| 1 | Default | 30 | Default |  | CP-OFDM | QPSK | Outer_Full (A2) |
| 2 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |
| 3 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |
| 4 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2.2.4.1-4, since offsetToCarrier = 0 in Tables 6.2.3.4.1-18a and 6.2.3.4.1-18b. NOTE 2: Void NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. NOTE 4: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-18a: Test frequencies for NS_47 (SCS=15 kHz, ΔFRaster = 15kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | 160 | 2560.005 | 512001 | 2545.605 | 509121 | 0 | 15 | 6369 | 509550 | 11 | 1 | 0 (0) | 1 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-18b: Test frequencies for NS_47 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | 78 | 2560.02 | 512004 | 2545.98 | 509196 | 0 | 30 | 6375 | 510030 | 14 | 0 | 1 (1) | 2 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-19: Test Configuration table for NS_48 (Power class 3)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  | lowest and highest carrier centre frequency for each CBW in table 6.2.3.3.26-1 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 45MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Outer_Full (A3) |  |  |
| 2 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 3 | Default | 30 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 5 | Default | 40 | Default |  |  | QPSK | 16@0 (A2) | 8@0 (A2) | 4@0 (A2) |
| 6 | Default | 40 | Default |  |  | QPSK | 90@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 7 | Default | 40 | Default |  |  | QPSK | 150@0 (A4) | 75@0 (A4) | 36@0 (A4) |
| 8 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 9 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 10 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 11 | Default | 45 | Default |  |  | QPSK | 27@0 (A2) | 12@0 (A2) | 6@0 (A2) |
| 12 | Default | 45 | Default |  |  | QPSK | 100@0 (A4) | 50@0 (A4) | 27@0 (A4) |
| 13 | Default | 45 | Default |  |  | QPSK | 200@0 (A2) | 100@0 (A2) | 50@0 (A2) |
| 14 | Default | 45 | Default |  |  | QPSK | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 15 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 16 | Default | 50 | Default |  |  | QPSK | 32@0 (A2) | 16@0 (A2) | 8@0 (A2) |
| 17 | Default | 50 | Default |  |  | QPSK | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 18 | Default | 50 | Default |  |  | QPSK | 225@0 (A2) | 108@0 (A2) | 54@0 (A2) |
| 19 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 20 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 21 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 22 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 23 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 24 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |
| 25 | Default | 40 | Default |  |  | 256 QAM | 16@0 (A2) | 8@0 (A2) | 4@0 (A2) |
| 26 | Default | 40 | Default |  |  | 256 QAM | 90@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 27 | Default | 40 | Default |  |  | 256 QAM | 150@0 (A4) | 75@0 (A4) | 36@0 (A4) |
| 28 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 29 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 30 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 31 | Default | 45 | Default |  |  | 256 QAM | 27@0 (A2) | 12@0 (A2) | 6@0 (A2) |
| 32 | Default | 45 | Default |  |  | 256 QAM | 100@0 (A4) | 50@0 (A4) | 27@0 (A4) |
| 33 | Default | 45 | Default |  |  | 256 QAM | 200@0 (A2) | 100@0 (A2) | 50@0 (A2) |
| 34 | Default | 45 | Default |  |  | 256 QAM | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 35 | Default | 45 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 36 | Default | 50 | Default |  |  | 256 QAM | 32@0 (A2) | 16@0 (A2) | 8@0 (A2) |
| 37 | Default | 50 | Default |  |  | 256 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 38 | Default | 50 | Default |  |  | 256 QAM | 225@0 (A2) | 108@0 (A2) | 54@0 (A2) |
| 39 | Default | 50 | Default |  |  | 256 QAM | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 40 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 41 | Default | 25 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  |  |
| 42 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 43 | Default | 30 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| 44 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 45 | Default | 40 | Default |  |  | QPSK | 16@0 (A2) | 8@0 (A2) | 4@0 (A2) |
| 46 | Default | 40 | Default |  |  | QPSK | 95@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 47 | Default | 40 | Default |  |  | QPSK | 152@0 (A4) | 76@0 (A4) | 38@0 (A4) |
| 48 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 49 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 50 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 51 | Default | 45 | Default |  |  | QPSK | 27@0 (A2) | 13@0 (A2) | 6@0 (A2) |
| 52 | Default | 45 | Default |  |  | QPSK | 106@0 (A4) | 53@0 (A4) | 27@0 (A4) |
| 53 | Default | 45 | Default |  |  | QPSK | 208@0 (A2) | 104@0 (A2) | 52@0 (A2) |
| 54 | Default | 45 | Default |  |  | QPSK | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 55 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 56 | Default | 50 | Default |  |  | QPSK | 34@0 (A2) | 17@0 (A2) | 9@0 (A2) |
| 57 | Default | 50 | Default |  |  | QPSK | 115@0 (A4) | 58@0 (A4) | 29@0 (A4) |
| 58 | Default | 50 | Default |  |  | QPSK | 228@0 (A2) | 114@0 (A2) | 57@0 (A2) |
| 59 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 60 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 61 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 62 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 63 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 64 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |
| 65 | Default | 40 | Default |  |  | 256 QAM | 16@0 (A2) | 8@0 (A2) | 4@0 (A2) |
| 66 | Default | 40 | Default |  |  | 256 QAM | 95@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 67 | Default | 40 | Default |  |  | 256 QAM | 152@0 (A4) | 76@0 (A4) | 38@0 (A4) |
| 68 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 69 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 70 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 71 | Default | 45 | Default |  |  | 256 QAM | 27@0 (A2) | 13@0 (A2) | 6@0 (A2) |
| 72 | Default | 45 | Default |  |  | 256 QAM | 106@0 (A4) | 53@0 (A4) | 27@0 (A4) |
| 73 | Default | 45 | Default |  |  | 256 QAM | 208@0 (A2) | 104@0 (A2) | 52@0 (A2) |
| 74 | Default | 45 | Default |  |  | 256 QAM | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 75 | Default | 45 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 76 | Default | 50 | Default |  |  | 256 QAM | 34@0 (A2) | 17@0 (A2) | 9@0 (A2) |
| 77 | Default | 50 | Default |  |  | 256 QAM | 115@0 (A4) | 58@0 (A4) | 29@0 (A4) |
| 78 | Default | 50 | Default |  |  | 256 QAM | 228@0 (A2) | 114@0 (A2) | 57@0 (A2) |
| 79 | Default | 50 | Default |  |  | 256 QAM | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 80 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-20: Test Configuration table for NS_48 (Power class 2)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  | lowest and highest carrier centre frequency for each CBW in table 6.2.3.3.26-1 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 45MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 10 | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Outer_Full (A6) |  |  |
| 2 | Default | 10 | Default |  |  | QPSK | 8@0 (A6) | 4@0 (A6) | 2@0 (A6) |
| 3 | Default | 15 | Default |  |  | QPSK | Outer_Full (A6) |  |  |
| 4 | Default | 15 | Default |  |  | QPSK | 48@0 (A6) | 24@0 (A6) | 12@0 (A6) |
| 5 | Default | 20 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 6 | Default | 20 | Default |  |  | QPSK | 50@0 (A6) | 25@0 (A6) | 12@0 (A6) |
| 7 | Default | 25 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 8 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 9 | Default | 25 | Default |  |  | QPSK | 50@0 (A4) | 25@0 (A4) | 12@0 (A4) |
| 10 | Default | 30 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 11 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 12 | Default | 30 | Default |  |  | QPSK | 64@0 (A4) | 32@0 (A4) | 16@0 (A4) |
| 13 | Default | 40 | Default |  |  | QPSK | 16@0 (A1) | 8@0 (A1) | 4@0 (A1) |
| 14 | Default | 40 | Default |  |  | QPSK | 90@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 15 | Default | 40 | Default |  |  | QPSK | 150@0 (A4) | 75@0 (A4) | 36@0 (A4) |
| 16 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 17 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 18 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 19 | Default | 45 | Default |  |  | QPSK | 27@0 (A1) | 12@0 (A1) | 6@0 (A1) |
| 20 | Default | 45 | Default |  |  | QPSK | 100@0 (A4) | 50@0 (A4) | 27@0 (A4) |
| 21 | Default | 45 | Default |  |  | QPSK | 200@0 (A2) | 100@0 (A2) | 50@0 (A2) |
| 22 | Default | 45 | Default |  |  | QPSK | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 23 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 24 | Default | 50 | Default |  |  | QPSK | 36@0 (A1) | 18@0 (A1) | 9@0 (A1) |
| 25 | Default | 50 | Default |  |  | QPSK | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 26 | Default | 50 | Default |  |  | QPSK | 225@0 (A2) | 108@0 (A2) | 54@0 (A2) |
| 27 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 28 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 29 | Default | 10 | Default |  |  | 256QAM | Outer_Full (A6) |  |  |
| 30 | Default | 10 | Default |  |  | 256QAM | 8@0 (A6) | 4@0 (A6) | 2@0 (A6) |
| 31 | Default | 15 | Default |  |  | 256QAM | Outer_Full (A6) |  |  |
| 32 | Default | 15 | Default |  |  | 256QAM | 48@0 (A6) | 24@0 (A6) | 12@0 (A6) |
| 33 | Default | 20 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |
| 34 | Default | 20 | Default |  |  | 256QAM | 50@0 (A6) | 25@0 (A6) | 12@0 (A6) |
| 35 | Default | 25 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |
| 36 | Default | 25 | Default |  |  | 256QAM | Edge_1RB_Right (A3) |  |  |
| 37 | Default | 25 | Default |  |  | 256QAM | 50@0 (A4) | 25@0 (A4) | 12@0 (A4) |
| 38 | Default | 30 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |
| 39 | Default | 30 | Default |  |  | 256QAM | Edge_1RB_Right (A5) |  |  |
| 40 | Default | 30 | Default |  |  | 256QAM | 64@0 (A4) | 32@0 (A4) | 16@0 (A4) |
| 41 | Default | 40 | Default |  |  | 256QAM | 16@0 (A1) | 8@0 (A1) | 4@0 (A1) |
| 42 | Default | 40 | Default |  |  | 256QAM | 90@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 43 | Default | 40 | Default |  |  | 256QAM | 150@0 (A4) | 75@0 (A4) | 36@0 (A4) |
| 44 | Default | 40 | Default |  |  | 256QAM | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 45 | Default | 40 | Default |  |  | 256QAM | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 46 | Default | 40 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |
| 47 | Default | 45 | Default |  |  | 256QAM | 27@0 (A1) | 12@0 (A1) | 6@0 (A1) |
| 48 | Default | 45 | Default |  |  | 256QAM | 100@0 (A4) | 50@0 (A4) | 27@0 (A4) |
| 49 | Default | 45 | Default |  |  | 256QAM | 200@0 (A2) | 100@0 (A2) | 50@0 (A2) |
| 50 | Default | 45 | Default |  |  | 256QAM | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 51 | Default | 45 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |
| 52 | Default | 50 | Default |  |  | 256QAM | 36@0 (A1) | 18@0 (A1) | 9@0 (A1) |
| 53 | Default | 50 | Default |  |  | 256QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 54 | Default | 50 | Default |  |  | 256QAM | 225@0 (A2) | 108@0 (A2) | 54@0 (A2) |
| 55 | Default | 50 | Default |  |  | 256QAM | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 56 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |
| 57 | Default | 10 | Default |  | CP-OFDM | QPSK | Outer_Full (A6) |  |  |
| 58 | Default | 10 | Default |  |  | QPSK | 8@0 (A6) | 4@0 (A6) | 2@0 (A6) |
| 59 | Default | 15 | Default |  |  | QPSK | Outer_Full (A6) |  |  |
| 60 | Default | 15 | Default |  |  | QPSK | 48@0 (A6) | 24@0 (A6) | 12@0 (A6) |
| 61 | Default | 20 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 62 | Default | 20 | Default |  |  | QPSK | 52@0 (A6) | 26@0 (A6) | 13@0 (A6) |
| 63 | Default | 25 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 64 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 65 | Default | 25 | Default |  |  | QPSK | 52@0 (A4) | 26@0 (A4) | 13@0 (A4) |
| 66 | Default | 30 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 67 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 68 | Default | 30 | Default |  |  | QPSK | 70@0 (A4) | 35@0 (A4) | 17@0 (A4) |
| 69 | Default | 40 | Default |  |  | QPSK | 16@0 (A1) | 8@0 (A1) | 4@0 (A1) |
| 70 | Default | 40 | Default |  |  | QPSK | 90@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 71 | Default | 40 | Default |  |  | QPSK | 150@0 (A4) | 81@0 (A4) | 40@0 (A4) |
| 72 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 73 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 74 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 75 | Default | 45 | Default |  |  | QPSK | 28@0 (A1) | 14@0 (A1) | 7@0 (A1) |
| 76 | Default | 45 | Default |  |  | QPSK | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 77 | Default | 45 | Default |  |  | QPSK | 200@0 (A2) | 100@0 (A2) | 50@0 (A2) |
| 78 | Default | 45 | Default |  |  | QPSK | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 79 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 80 | Default | 50 | Default |  |  | QPSK | 36@0 (A1) | 18@0 (A1) | 9@0 (A1) |
| 81 | Default | 50 | Default |  |  | QPSK | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 82 | Default | 50 | Default |  |  | QPSK | 225@0 (A2) | 108@0 (A2) | 54@0 (A2) |
| 83 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 84 | Default | 50 | Default |  |  | QPSK | Outer_Full (A6) |  |  |
| 85 | Default | 10 | Default |  |  | 256QAM | Outer_Full (A6) |  |  |
| 86 | Default | 10 | Default |  |  | 256QAM | 8@0 (A6) | 4@0 (A6) | 2@0 (A6) |
| 87 | Default | 15 | Default |  |  | 256QAM | Outer_Full (A6) |  |  |
| 88 | Default | 15 | Default |  |  | 256QAM | 48@0 (A6) | 24@0 (A6) | 12@0 (A6) |
| 89 | Default | 20 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |
| 90 | Default | 20 | Default |  |  | 256QAM | 52@0 (A6) | 26@0 (A6) | 13@0 (A6) |
| 91 | Default | 25 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |
| 92 | Default | 25 | Default |  |  | 256QAM | Edge_1RB_Right (A3) |  |  |
| 93 | Default | 25 | Default |  |  | 256QAM | 52@0 (A4) | 26@0 (A4) | 13@0 (A4) |
| 94 | Default | 30 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |
| 95 | Default | 30 | Default |  |  | 256QAM | Edge_1RB_Right (A5) |  |  |
| 96 | Default | 30 | Default |  |  | 256QAM | 70@0 (A4) | 35@0 (A4) | 17@0 (A4) |
| 97 | Default | 40 | Default |  |  | 256QAM | 16@0 (A1) | 8@0 (A1) | 4@0 (A1) |
| 98 | Default | 40 | Default |  |  | 256QAM | 90@0 (A3) | 48@0 (A3) | 24@0 (A3) |
| 99 | Default | 40 | Default |  |  | 256QAM | 150@0 (A4) | 81@0 (A4) | 40@0 (A4) |
| 100 | Default | 40 | Default |  |  | 256QAM | 192@0 (A2) | 96@0 (A2) | 48@0 (A2) |
| 101 | Default | 40 | Default |  |  | 256QAM | 5@187 (A3) | 2@94 (A3) | 1@47 (A3) |
| 102 | Default | 40 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |
| 103 | Default | 45 | Default |  |  | 256QAM | 28@0 (A1) | 14@0 (A1) | 7@0 (A1) |
| 104 | Default | 45 | Default |  |  | 256QAM | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 105 | Default | 45 | Default |  |  | 256QAM | 200@0 (A2) | 100@0 (A2) | 50@0 (A2) |
| 106 | Default | 45 | Default |  |  | 256QAM | 5@203 (A5) | 2@102 (A5) | 1@51 (A5) |
| 107 | Default | 45 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |
| 108 | Default | 50 | Default |  |  | 256QAM | 36@0 (A1) | 18@0 (A1) | 9@0 (A1) |
| 109 | Default | 50 | Default |  |  | 256QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 110 | Default | 50 | Default |  |  | 256QAM | 225@0 (A2) | 108@0 (A2) | 54@0 (A2) |
| 111 | Default | 50 | Default |  |  | 256QAM | 5@223 (A5) | 2@112 (A5) | 1@56 (A5) |
| 112 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A6) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-21: Test Configuration table for NS_12

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to test parameters (3, 5, 10 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_12 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
| 1 | Default | 5 | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Edge_1RB_Left |  |  |
| 2 | Default | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 3 | Default | 5 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 4 | Default | 5 | Default |  |  | QPSK | Outer_Full |  |  |
| 5 | Default | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 6 | Default | 5 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 7 | Default | 10 | Default |  |  | QPSK | Edge_1RB_Left |  |  |
| 8 | Default | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 9 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 10 | Default | 10 | Default |  |  | QPSK | Outer_Full |  |  |
| 11 | Default | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 12 | Default | 10 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 13 | Default | 5 | Default |  |  | QPSK | 12@11 |  |  |
| 14 | Default | 10 | Default |  |  | QPSK | 30@21 |  |  |
| 15 | Default | 5 | Default |  | CP-OFDM | 64 QAM | Edge_1RB_Left |  |  |
| 16 | Default | 5 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 17 | Default | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 18 | Default | 5 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 19 | Default | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 20 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 21 | Default | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 22 | Default | 10 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 23 | Default | 5 | Default |  |  | QPSK | 12@11 |  |  |
| 24 | Default | 10 | Default |  |  | QPSK | 30@21 |  |  |
| 25 | Default | 3 | Default |  | DFT-s-OFDM | QPSK | Edge_1RB_Left |  |  |
| 26 | Default | 3 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 27 | Default | 3 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 28 | Default | 3 | Default |  |  | QPSK | Outer_Full |  |  |
| 29 | Default | 3 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 30 | Default | 3 | Default |  |  | 256 QAM | Outer_Full |  |  |
| 31 | Default | 3 | Default |  |  | QPSK | 8@6 |  |  |
| 32 | Default | 3 | Default |  | CP-OFDM | 64 QAM | Edge_1RB_Left |  |  |
| 33 | Default | 3 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 34 | Default | 3 | Default |  |  | QPSK | 8@6 |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: In test IDs with multiple modulations, each UL Modulation shall be tested separately against RB allocation. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-22: Test Configuration table for NS_13

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to uplink carrier center frequency (Fc) in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to test parameters (3, 5 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_13 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
| 1 | 819.5 | 5 | Default | N/A for A-MPR testing. | DFT-s-OFDM | 16 QAM | Edge_1RB_Left |  |  |
| 2 | 819.5 | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 3 | 819.5 | 5 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 4 | 819.5 | 5 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 5 | 819.5 | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 6 | 819.5 | 5 | Default |  |  | 256QAM | Outer_Full |  |  |
| 7 | 821.5 | 5 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 8 | 821.5 | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 9 | 821.5 | 5 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 10 | High | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 11 | Void |  |  |  |  |  |  |  |  |
| 12 | 819.5 | 5 | Default |  | CP-OFDM | 16 QAM | Edge_1RB_Left |  |  |
| 13 | 819.5 | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 14 | 819.5 | 5 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 15 | 819.5 | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 16 | 819.5 | 5 | Default |  |  | 256QAM | Outer_Full |  |  |
| 17 | 821.5 | 5 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 18 | 821.5 | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 19 | 821.5 | 5 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 20 | High | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 21 | Void |  |  |  |  |  |  |  |  |
| 22 | 818.5 | 3 | Default |  | DFT-s-OFDM | 16 QAM | Edge_1RB_Left |  |  |
| 23 | 818.5 | 3 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 24 | 818.5 | 3 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 25 | 818.5 | 3 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 26 | 820 | 3 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 27 | 820 | 3 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 28 | 818.5 | 3 | Default |  | CP-OFDM | 16 QAM | Edge_1RB_Left |  |  |
| 29 | 818.5 | 3 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 30 | 818.5 | 3 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 31 | 818.5 | 3 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 32 | 820 | 3 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: In test IDs with multiple modulations, each UL Modulation shall be tested separately against RB allocation. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-23: Test Configuration table for NS_14

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to test parameters (10, 15, 20 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_14 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
| 1 | Default | 10 | Default | N/A for A-MPR testing. | DFT-s-OFDM | 64 QAM | Edge_1RB_Left |  |  |
| 2 | Default | 10 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 3 | Default | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 4 | Default | 15 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 5 | Default | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 6 | Default | 20 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 7 | Default | 20 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 8 | Default | 10 | Default |  |  | QPSK | 8@2 |  |  |
| 9 | Default | 15 | Default |  |  | QPSK | 8@11 |  |  |
| 10 | Default | 20 | Default |  |  | QPSK | 8@20 |  |  |
| 11 | Default | 10 | Default |  | CP-OFDM | 16 QAM | Edge_1RB_Left |  |  |
| 12 | Default | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 13 | Default | 10 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 14 | Default | 10 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 15 | Default | 15 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 16 | Default | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 17 | Default | 15 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 18 | Default | 15 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 19 | Default | 20 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 20 | Default | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 21 | Default | 20 | Default |  |  | 256QAM | Edge_1RB_Left |  |  |
| 22 | Default | 20 | Default |  |  | 16 QAM | Outer_Full |  |  |
| 23 | Default | 10 | Default |  |  | QPSK | 8@2 |  |  |
| 24 | Default | 15 | Default |  |  | QPSK | 8@11 |  |  |
| 25 | Default | 20 | Default |  |  | QPSK | 8@20 |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: In test IDs with multiple modulations, each UL Modulation shall be tested separately against RB allocation. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-24: Test Configuration table for NS_15

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to uplink carrier center frequency (Fc) in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to test parameters (3, 5, 10, 15, 20 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_15 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
| 1 | 846.5 | 5 | Default | N/A for A-MPR testing. | DFT-s-OFDM | 64 QAM | Edge_1RB_Right |  |  |
| 2 | 846.5 | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 3 | 846.5 | 5 | Default |  |  | 64 QAM | 8@9 |  |  |
| 4 | 846.5 | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 5 | 846.5 | 5 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 6 | 844 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 7 | 844 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 8 | 844 | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 9 | 844 | 10 | Default |  |  | 64 QAM | 15@16 |  |  |
| 10 | 844 | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 11 | 844 | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 12 | 840 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 13 | 840 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 14 | 840 | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 15 | 840 | 10 | Default |  |  | 64 QAM | 24@15 |  |  |
| 16 | 840 | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 17 | 840 | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 18 | 841.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 19 | 841.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 20 | 841.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 21 | 841.5 | 15 | Default |  |  | 64 QAM | 20@31 |  |  |
| 22 | 841.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 23 | 841.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 24 | 837.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 25 | 837.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 26 | 837.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 27 | 837.5 | 15 | Default |  |  | 64 QAM | 20@39 |  |  |
| 28 | 837.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 29 | 837.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 30 | 831.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 31 | 831.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 32 | 831.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 33 | 831.5 | 15 | Default |  |  | 64 QAM | 20@52 |  |  |
| 34 | 831.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 35 | 831.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 36 | 839 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 37 | 839 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 38 | 839 | 20 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 39 | 839 | 20 | Default |  |  | 64 QAM | 30@37 |  |  |
| 40 | 839 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 41 | 839 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 42 | 835 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 43 | 835 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 44 | 835 | 20 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 45 | 835 | 20 | Default |  |  | 64 QAM | 30@45 |  |  |
| 46 | 835 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 47 | 835 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 48 | 840.5 | 5 | Default |  |  | QPSK | Outer_Full |  |  |
| 49 | 835 | 10 | Default |  |  | QPSK | Outer_Full |  |  |
| 50 | 831.5 | 15 | Default |  |  | QPSK | 20@23 |  |  |
| 51 | 835 | 20 | Default |  |  | QPSK | 30@16 |  |  |

| A-MPR test parameters for NS_15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
| 52 | 846.5 | 5 | Default |  | CP-OFDM | 64 QAM | Edge_1RB_Right |  |  |
| 53 | 846.5 | 5 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 54 | 846.5 | 5 | Default |  |  | 64 QAM | 8@9 |  |  |
| 55 | 846.5 | 5 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 56 | 846.5 | 5 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 57 | 844 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 58 | 844 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 59 | 844 | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 60 | 844 | 10 | Default |  |  | 64 QAM | 15@16 |  |  |
| 61 | 844 | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 62 | 844 | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 63 | 840 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 64 | 840 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 65 | 840 | 10 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 66 | 840 | 10 | Default |  |  | 64 QAM | 24@15 |  |  |
| 67 | 840 | 10 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 68 | 840 | 10 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 69 | 841.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 70 | 841.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 71 | 841.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 72 | 841.5 | 15 | Default |  |  | 64 QAM | 20@31 |  |  |
| 73 | 841.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 74 | 841.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 75 | 837.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 76 | 837.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 77 | 837.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 78 | 837.5 | 15 | Default |  |  | 64 QAM | 20@39 |  |  |
| 79 | 837.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 80 | 837.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 81 | 831.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 82 | 831.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 83 | 831.5 | 15 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 84 | 831.5 | 15 | Default |  |  | 64 QAM | 20@52 |  |  |
| 85 | 831.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 86 | 831.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 87 | 839 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 88 | 839 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 89 | 839 | 20 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 90 | 839 | 20 | Default |  |  | 64 QAM | 30@37 |  |  |
| 91 | 839 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 92 | 839 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 93 | 835 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right |  |  |
| 94 | 835 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right |  |  |
| 95 | 835 | 20 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 96 | 835 | 20 | Default |  |  | 64 QAM | 30@45 |  |  |
| 97 | 835 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left |  |  |
| 98 | 835 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left |  |  |
| 99 | 840.5 | 5 | Default |  |  | QPSK | Outer_Full |  |  |
| 100 | 835 | 10 | Default |  |  | QPSK | Outer_Full |  |  |
| 101 | 831.5 | 15 | Default |  |  | QPSK | 20@23 |  |  |
| 102 | 835 | 20 | Default |  |  | QPSK | 30@16 |  |  |
| 103 | 847.5 | 3 | Default |  | DFT-s-OFDM | 64 QAM | Edge_1RB_Right |  |  |
| 104 | 847.5 | 3 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 105 | 847.5 | 3 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 106 | 844.5 | 3 | Default |  |  | QPSK | Outer_Full |  |  |
| 107 | 847.5 | 3 | Default |  | CP-OFDM | 64 QAM | Edge_1RB_Right |  |  |
| 108 | 847.5 | 3 | Default |  |  | 64 QAM | Outer_Full |  |  |
| 109 | 847.5 | 3 | Default |  |  | 16 QAM | Edge_1RB_Left |  |  |
| 110 | 844.5 | 3 | Default |  |  | QPSK | Outer_Full |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: In test IDs with multiple modulations, each UL Modulation shall be tested separately against RB allocation. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-25: Test Configuration table for NS_46 (Power class 3)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_46 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 |  |  |  | N/A for A-MPR testing. | DFT-s-OFDM |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 | Default | 25 | Default |  |  | 64QAM | 60@73 (A3) |  | 30@35 (A3) | 15@16 (A3) |
| 4 | Default | 50 | Default |  |  | 64QAM | Edge_1RB_Left (A4) |  |  |  |
| 5 | Default | 50 | Default |  |  | 16QAM | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 6 | Default | 50 | Default |  |  | 64QAM | 162@0 (A6) |  | 81@0 (A6) | 40@0 (A6) |
| 7 | Default | 50 | Default |  |  | 256QAM | 216@0 (A7) |  | 108@0 (A7) | 54@0 (A7) |
| 8 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A8) |  |  |  |
| 9 |  |  |  |  | CP-OFDM |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |
| 11 | Default | 25 | Default |  |  | QPSK | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 12 | Default | 25 | Default |  |  | 256QAM | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 13 | Default | 50 | Default |  |  | QPSK | Edge_1RB_Left (A4) |  |  |  |
| 14 | Default | 50 | Default |  |  | 64QAM | Edge_1RB_Left (A4) |  |  |  |
| 15 | Default | 50 | Default |  |  | QPSK | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 16 | Default | 50 | Default |  |  | 64QAM | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 17 | Default | 50 | Default |  |  | QPSK | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 18 | Default | 50 | Default |  |  | 64QAM | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 19 | Default | 50 | Default |  |  | QPSK | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 20 | Default | 50 | Default |  |  | 256QAM | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 21 | Default | 50 | Default |  |  | QPSK | Outer_Full (A8) |  |  |  |
| 22 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A8) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-25a: Test Configuration table for NS_46 (Power class 1)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 15 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_46 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 15 | Default | N/A for A-MPR testing. | DFT-s-OFDM | 64QAM | Outer_Full (A1) |  |  |  |
| 2 | Default | 15 | Default |  |  | 256QAM | Edge_1RB_Right (A2) |  |  |  |
| 3 | Default | 50 | Default |  |  | 256QAM | 68@35 (A7) |  | 31@19 (A7) | 17@7(A7) |
| 4 | Default | 50 | Default |  |  | N/A | N/A (A11) |  |  |  |
| 5 | Default | 50 | Default |  |  | 256QAM | 146@20 (A9) |  | 74@10 (A9) | 38@5 (A9) |
| 6 | Default | 50 | Default |  |  | 64QAM | 10@260 (A12) |  | 10@123 (A12) | 10@55 (A12) |
| 7 | Default | 50 | Default |  |  | 256QAM | 28@160 (A8) |  | 14@80 (A8) | 7@40 (A8) |
| 8 | Default | 50 | Default |  |  | 256QAM | 12@40 (A9) |  | 6@20 (A9) | 3@10 (A9) |
| 9 | Default | 50 | Default |  |  | 256QAM | 57@20 (A7) |  | 28@10 (A7) | 14@5 (A7) |
| 10 | Default | 50 | Default |  |  | N/A | N/A (A11) |  |  |  |
| 11 | Default | 50 | Default |  |  | 256QAM | 195@0 (A8) |  | 97@0 (A8) | 48@0 (A8) |
| 12 | Default | 50 | Default |  |  | 64QAM | 122@117 (A12) |  | 61@59 (A12) | 30@30 (A12) |
| 13 | Default | 50 | Default |  |  | 256QAM | 16@179 (A8) |  | 8@90 (A8) | 4@45 (A8) |
| 14 | Default | 50 | Default |  |  | 256QAM | 11@195 (A9) |  | 5@98 (A9) | 2@49 (A9) |
| 15 | Default | 50 | Default |  |  | 64QAM | 229@34 (A13) |  | 115@16 (A13) | 57@9 (A13) |
| 16 | Default | 50 | Default |  |  | 256QAM | 46@0 (A7) |  | 23@0 (A7) | 11@0 (A7) |
| 17 | Default | 50 | Default |  |  | 256QAM | 17@195 (A8) |  | 9@97 (A8) | 5@48 (A8) |
| 18 | Default | 50 | Default |  |  | 256QAM | 11@201 (A9) |  | 5@101 (A9) | 2@51 (A9) |
| 19 | Default | 15 | Default |  | CP-OFDM | 64QAM | Outer_Full (A1) |  |  |  |
| 20 | Default | 15 | Default |  |  | 256QAM | Edge_1RB_Right (A2) |  |  |  |
| 21 | Default | 50 | Default |  |  | 256QAM | 68@35 (A7) |  | 31@19 (A7) | 17@7(A7) |
| 22 | Default | 50 | Default |  |  | 256QAM | 68@90(A11) |  | 31@45(A11) | 17@18(A11) |
| 23 | Default | 50 | Default |  |  | 256QAM | 146@20 (A9) |  | 74@10 (A9) | 38@5 (A9) |
| 24 | Default | 50 | Default |  |  | 64QAM | 10@260 (A12) |  | 10@123 (A12) | 10@55 (A12) |
| 25 | Default | 50 | Default |  |  | 256QAM | 28@160 (A8) |  | 14@80 (A8) | 7@40 (A8) |
| 26 | Default | 50 | Default |  |  | 256QAM | 12@40 (A9) |  | 6@20 (A9) | 3@10 (A9) |
| 27 | Default | 50 | Default |  |  | 256QAM | 57@20 (A7) |  | 28@10 (A7) | 14@5 (A7) |
| 28 | Default | 50 | Default |  |  | 256QAM | 57@87 (A11) |  | 28@54 (A7) | 14@22 (A7) |
| 29 | Default | 50 | Default |  |  | 256QAM | 195@0 (A8) |  | 97@0 (A8) | 48@0 (A8) |
| 30 | Default | 50 | Default |  |  | 64QAM | 122@117 (A12) |  | 61@59 (A12) | 30@30 (A12) |
| 31 | Default | 50 | Default |  |  | 256QAM | 16@179 (A8) |  | 8@90 (A8) | 4@45 (A8) |
| 32 | Default | 50 | Default |  |  | 256QAM | 11@195 (A9) |  | 5@98 (A9) | 2@49 (A9) |
| 33 | Default | 50 | Default |  |  | 64QAM | 229@34 (A13) |  | 115@16 (A13) | 57@9 (A13) |
| 34 | Default | 50 | Default |  |  | 256QAM | 46@0 (A7) |  | 23@0 (A7) | 11@0 (A7) |
| 35 | Default | 50 | Default |  |  | 256QAM | 17@195 (A8) |  | 9@97 (A8) | 5@48 (A8) |
| 36 | Default | 50 | Default |  |  | 256QAM | 11@201 (A9) |  | 5@101 (A9) | 2@51 (A9) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-26: Test Configuration table for NS_44

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Defined for each test ID |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Defined for each test ID |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_44 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | 2602.5 | 25 | Default | N/A | DFT-s-OFDM | 64 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 2 | 2602.5 | 25 | Default |  |  | 64 QAM | 36@97 (A3) |  | 18@47 (A3) | 9@22 (A3) |
| 3 | 2602.5 | 25 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 4 | 2602.5 | 25 | Default |  |  | 256 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 5 | 2602.5 | 25 | Default |  |  | 256 QAM | 36@97 (A3) |  | 18@47 (A3) | 9@22 (A3) |
| 6 | 2602.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 7 | 2600 | 30 | Default |  |  | 64 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 8 | 2600 | 30 | Default |  |  | 64 QAM | 64@96 (A3) |  | 32@46 (A3) | 16@22 (A3) |
| 9 | 2600 | 30 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 10 | 2600 | 30 | Default |  |  | 256 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 11 | 2600 | 30 | Default |  |  | 256 QAM | 64@96 (A3) |  | 32@46 (A3) | 16@22 (A3) |
| 12 | 2600 | 30 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 13 | 2595 | 40 | Default |  |  | 64 QAM | 16@0 (A1) |  | 8@0 (A1) | 4@0 (A1) |
| 14 | 2595 | 40 | Default |  |  | 64 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 15 | 2595 | 40 | Default |  |  | 64 QAM | 128@2 (A3) |  | 64@1 (A3) | 32@1 (A3) |
| 16 | 2595 | 40 | Default |  |  | 64 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 17 | 2595 | 40 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |  |
| 18 | 2595 | 40 | Default |  |  | 256 QAM | 16@0 (A1) |  | 8@0 (A1) | 4@0 (A1) |
| 19 | 2595 | 40 | Default |  |  | 256 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 20 | 2595 | 40 | Default |  |  | 256 QAM | 128@2 (A3) |  | 64@1 (A3) | 32@1 (A3) |
| 21 | 2595 | 40 | Default |  |  | 256 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 22 | 2595 | 40 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |  |
| 23 | 2602.5 | 25 | Default |  | CP-OFDM | 64 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 24 | 2602.5 | 25 | Default |  |  | 64 QAM | 39@94 (A3) |  | 19@46 (A3) | 9@22 (A3) |
| 25 | 2602.5 | 25 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 26 | 2602.5 | 25 | Default |  |  | 256 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 27 | 2602.5 | 25 | Default |  |  | 256 QAM | 39@94 (A3) |  | 19@46 (A3) | 9@22 (A3) |
| 28 | 2602.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 29 | 2600 | 30 | Default |  |  | 64 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 30 | 2600 | 30 | Default |  |  | 64 QAM | 69@91 (A3) |  | 34@44 (A3) | 17@21 (A3) |
| 31 | 2600 | 30 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 32 | 2600 | 30 | Default |  |  | 256 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 33 | 2600 | 30 | Default |  |  | 256 QAM | 69@91 (A3) |  | 34@44 (A3) | 17@21 (A3) |
| 34 | 2600 | 30 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 35 | 2595 | 40 | Default |  |  | 64 QAM | 16@0 (A1) |  | 8@0 (A1) | 4@0 (A1) |
| 36 | 2595 | 40 | Default |  |  | 64 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 37 | 2595 | 40 | Default |  |  | 64 QAM | 130@0 (A3) |  | 65@0 (A3) | 33@0 (A3) |
| 38 | 2595 | 40 | Default |  |  | 64 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 39 | 2595 | 40 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |  |

| A-MPR test parameters for NS_44 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 40 | 2595 | 40 | Default |  |  | 256 QAM | 16@0 (A1) | 8@0 (A1) | 4@0 (A1) |
| 41 | 2595 | 40 | Default |  |  | 256 QAM | 80@0 (A2) | 40@0 (A2) | 20@0 (A2) |
| 42 | 2595 | 40 | Default |  |  | 256 QAM | 130@0 (A3) | 65@0 (A3) | 33@0 (A3) |
| 43 | 2595 | 40 | Default |  |  | 256 QAM | 180@0 (A4) | 90@0 (A4) | 45@0 (A4) |
| 44 | 2595 | 40 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-27: Test Configuration table for NS_21

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_21 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |
| 1 | Low | Default | Default | N/A for A-MPR test cases | DFT-s-OFDM | PI/2 BPSK | Edge_1RB_Left |  |
| 2 | High | Default | Default |  |  | PI/2 BPSK | Edge_1RB_Right |  |
| 3 | Default | Default | Default |  |  | PI/2 BPSK | Outer_Full |  |
| 4 | Default | Default | Default |  |  | PI/2 BPSK | 4@0 | 2@0 |
| 5a | Default | 5 MHz | Default |  |  | PI/2 BPSK | 4@21 | 2@7 |
| 5b | Default | 10 MHz | Default |  |  | PI/2 BPSK | 4@48 | 2@22 |
| 6 | Low | Default | Default |  |  | QPSK | Edge_1RB_Left |  |
| 7 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |  |
| 8 | Default | Default | Default |  |  | QPSK | Outer_Full |  |
| 9 | Default | Default | Default |  |  | QPSK | 4@0 | 2@0 |
| 10a | Default | 5 MHz | Default |  |  | QPSK | 4@21 | 2@7 |
| 10b | Default | 10 MHz | Default |  |  | QPSK | 4@48 | 2@22 |
| 11 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |  |
| 12 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |  |
| 13 | Default | Default | Default |  |  | 16 QAM | Outer_Full |  |
| 14 | Default | Default | Default |  |  | 16 QAM | 4@0 | 2@0 |
| 15a | Default | 5 MHz | Default |  |  | 16 QAM | 4@21 | 2@7 |
| 15b | Default | 10 MHz | Default |  |  | 16 QAM | 4@48 | 2@22 |
| 16 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |  |
| 17 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |  |
| 18 | Default | Default | Default |  |  | 64 QAM | Outer_Full |  |
| 19 | Default | Default | Default |  |  | 64 QAM | 4@0 | 2@0 |
| 20a | Default | 5 MHz | Default |  |  | 64 QAM | 4@21 | 2@7 |
| 20b | Default | 10 MHz | Default |  |  | 64 QAM | 4@48 | 2@22 |
| 21 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |  |
| 22 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |  |
| 23 | Default | Default | Default |  |  | 256 QAM | Outer_Full |  |
| 24 | Default | Default | Default |  |  | 256 QAM | 4@0 | 2@0 |
| 25a | Default | 5 MHz | Default |  |  | 256 QAM | 4@21 | 2@7 |
| 25 | Default | 10 MHz | Default |  |  | 256 QAM | 4@48 | 2@22 |
| 26 | Low | Default | Default |  | CP-OFDM | QPSK | Edge_1RB_Left |  |
| 27 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |  |
| 28 | Default | Default | Default |  |  | QPSK | Outer_Full |  |
| 29 | Default | Default | Default |  |  | QPSK | 4@0 | 2@0 |
| 30a | Default | 5 MHz | Default |  |  | QPSK | 4@21 | 2@7 |
| 30b | Default | 10 MHz | Default |  |  | QPSK | 4@48 | 2@22 |
| 31 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |  |
| 32 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |  |
| 33 | Default | Default | Default |  |  | 16 QAM | Outer_Full |  |
| 34 | Default | Default | Default |  |  | 16 QAM | 4@0 | 2@0 |
| 35a | Default | 5 MHz | Default |  |  | 16 QAM | 4@21 | 2@7 |
| 35b | Default | 10 MHz | Default |  |  | 16 QAM | 4@48 | 2@22 |
| 36 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |  |
| 37 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |  |
| 38 | Default | Default | Default |  |  | 64 QAM | Outer_Full |  |
| 39 | Default | Default | Default |  |  | 64 QAM | 4@0 | 2@0 |
| 40a | Default | 5 MHz | Default |  |  | 64 QAM | 4@21 | 2@7 |
| 40b | Default | 10 MHz | Default |  |  | 64 QAM | 4@48 | 2@22 |
| 41 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |  |
| 42 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |  |
| 43 | Default | Default | Default |  |  | 256 QAM | Outer_Full |  |
| 44 | Default | Default | Default |  |  | 256 QAM | 4@0 | 2@0 |
| 45a | Default | 5 MHz | Default |  |  | 256 QAM | 4@21 | 2@7 |
| 45b | Default | 10 MHz | Default |  |  | 256 QAM | 4@48 | 2@22 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-28: Test Configuration table for NS_45

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_45 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq | ChBw | SCS | N/A for A-MPR testing. | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 | Low | Default | Default |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 3 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-29: Test Configuration table for NS_49 (power class 3)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | Lowest and highest carrier centre frequency for each A-MPR requirement.in table 6.2.3.3.27-1 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 45MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Outer_Full (A3) |  |  |  |
| 2 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 3 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 5 | Default | 30 | Default |  |  | QPSK | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 6 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 7 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 8 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 9 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 10 | Default | 40 | Default |  |  | QPSK | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 11 | Default | 40 | Default |  |  | QPSK | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |
| 12 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 13 | Default | 40 | Default |  |  | QPSK | 150@33 (A2) |  | 75@17 (A2) | 36@9 (A2) |
| 14 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 15 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 16 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 17 | Default | 45 | Default |  |  | QPSK | 5@64 (A5) |  | 2@32 (A5) | 1@16 (A5) |
| 18 | Default | 45 | Default |  |  | QPSK | 5@199 (A5) |  | 2@100 (A5) | 1@50 (A5) |
| 19 | Default | 45 | Default |  |  | QPSK | 162@36 (A2) |  | 81@19 (A2) | 40@10 (A2) |
| 20 | Default | 45 | Default |  |  | QPSK | 200@0 (A1) |  | 100@0 (A1) | 50@0 (A1) |
| 21 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 22 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 23 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 24 | Default | 50 | Default |  |  | QPSK | 150@45 (A2) |  | 81@23 (A2) | 40@12 (A2) |
| 25 | Default | 50 | Default |  |  | QPSK | 216@0 (A1) |  | 108@0 (A1) | 54@0 (A1) |
| 26 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 27 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 28 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 29 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |  |
| 30 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 31 | Default | 30 | Default |  |  | 256 QAM | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 32 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 33 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 34 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 35 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |  |
| 36 | Default | 40 | Default |  |  | 256 QAM | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 37 | Default | 40 | Default |  |  | 256 QAM | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |
| 38 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 39 | Default | 40 | Default |  |  | 256 QAM | 150@33 (A2) |  | 75@17 (A2) | 36@9 (A2) |
| 40 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 41 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 42 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 43 | Default | 45 | Default |  |  | 256 QAM | 5@64 (A5) |  | 2@32 (A5) | 1@16 (A5) |
| 44 | Default | 45 | Default |  |  | 256 QAM | 5@199 (A5) |  | 2@100 (A5) | 1@50 (A5) |
| 45 | Default | 45 | Default |  |  | 256 QAM | 162@36 (A2) |  | 81@19 (A2) | 40@10 (A2) |
| 46 | Default | 45 | Default |  |  | 256 QAM | 200@0 (A1) |  | 100@0 (A1) | 50@0 (A1) |
| 47 | Default | 45 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 48 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 49 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 50 | Default | 50 | Default |  |  | 256 QAM | 150@45 (A2) |  | 81@23 (A2) | 40@12 (A2) |
| 51 | Default | 50 | Default |  |  | 256 QAM | 216@0 (A1) |  | 108@0 (A1) | 54@0 (A1) |
| 52 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 53 | Default | 25 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  |  |  |
| 54 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 55 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |  |
| 56 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 57 | Default | 30 | Default |  |  | QPSK | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 58 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 59 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 60 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 61 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 62 | Default | 40 | Default |  |  | QPSK | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 63 | Default | 40 | Default |  |  | QPSK | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |

| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 64 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 65 | Default | 40 | Default |  |  | QPSK | 159@33 (A2) | 79@17 (A2) | 39@9 (A2) |
| 66 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) | 2@94 (A5) | 1@47 (A5) |
| 67 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) | 96@0 (A1) | 48@0 (A1) |
| 68 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 69 | Default | 45 | Default |  |  | QPSK | 5@64 (A5) | 2@32 (A5) | 1@16 (A5) |
| 70 | Default | 45 | Default |  |  | QPSK | 5@199 (A5) | 2@100 (A5) | 1@50 (A5) |
| 71 | Default | 45 | Default |  |  | QPSK | 168@36 (A2) | 83@19 (A2) | 40@10 (A2) |
| 72 | Default | 45 | Default |  |  | QPSK | 204@0 (A1) | 102@0 (A1) | 51@0 (A1) |
| 73 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 74 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) | 2@38 (A5) | 1@19 (A5) |
| 75 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) | 2@108 (A5) | 1@54 (A5) |
| 76 | Default | 50 | Default |  |  | QPSK | 175@45 (A2) | 87@23 (A2) | 43@12 (A2) |
| 77 | Default | 50 | Default |  |  | QPSK | 220@0 (A1) | 110@0 (A1) | 55@0 (A1) |
| 78 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 79 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 80 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 81 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |
| 82 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) | 10@0 (A1) | 5@0 (A1) |
| 83 | Default | 30 | Default |  |  | 256 QAM | 36@0 (A5) | 18@0 (A5) | 9@0 (A5) |
| 84 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) | 40@0 (A3) | 20@0 (A3) |
| 85 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) | 60@0 (A4) | 30@0 (A4) |
| 86 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |
| 87 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |
| 88 | Default | 40 | Default |  |  | 256 QAM | 40@0 (A1) | 20@0 (A1) | 10@0 (A1) |
| 89 | Default | 40 | Default |  |  | 256 QAM | 5@53 (A5) | 2@27 (A5) | 1@14 (A5) |
| 90 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 91 | Default | 40 | Default |  |  | 256 QAM | 159@33 (A2) | 79@17 (A2) | 39@9 (A2) |
| 92 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) | 2@94 (A5) | 1@47 (A5) |
| 93 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) | 96@0 (A1) | 48@0 (A1) |
| 94 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 95 | Default | 45 | Default |  |  | 256 QAM | 5@64 (A5) | 2@32 (A5) | 1@16 (A5) |
| 96 | Default | 45 | Default |  |  | 256 QAM | 5@199 (A5) | 2@100 (A5) | 1@50 (A5) |
| 97 | Default | 45 | Default |  |  | 256 QAM | 168@36 (A2) | 83@19 (A2) | 40@10 (A2) |
| 98 | Default | 45 | Default |  |  | 256 QAM | 204@0 (A1) | 102@0 (A1) | 51@0 (A1) |
| 99 | Default | 45 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 100 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) | 2@38 (A5) | 1@19 (A5) |
| 101 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) | 2@108 (A5) | 1@54 (A5) |
| 102 | Default | 50 | Default |  |  | 256 QAM | 175@45 (A2) | 87@23 (A2) | 43@12 (A2) |
| 103 | Default | 50 | Default |  |  | 256 QAM | 220@0 (A1) | 110@0 (A1) | 55@0 (A1) |
| 104 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-29a: Test Configuration table for NS_49(power class 2)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 10 MHz, 15 MHz, 20 MHz, 25 MHz, 30MHz, 40MHz, 45 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 10 | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Outer_Full (A3) |  |  | - |
| 2 | Default | 10 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |  |
| 3 | Default | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 4 | Default | 15 | Default |  |  | QPSK | 48@0 (A3) |  | 24@0 (A3) | 12@0 (A3) |
| 5 | Default | 20 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 6 | Default | 20 | Default |  |  | QPSK | 50@0 (A3) |  | 25@0 (A3) | 12@0 (A3) |
| 7 | Default | 25 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 8 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 9 | Default | 25 | Default |  |  | QPSK | 18@18 (A3) |  | 9@9 (A3) | 4@5 (A3) |
| 10 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 11 | Default | 30 | Default |  |  | QPSK | 40@0 (A5) |  | 20@0 (A5) | 10@0 (A5) |
| 12 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 13 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 14 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 15 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A1) |  |  |  |
| 16 | Default | 40 | Default |  |  | QPSK | 50@0 (A1) |  | 25@0 (A1) | 12@0 (A1) |
| 17 | Default | 40 | Default |  |  | QPSK | 16@48 (A5) |  | 8@24 (A5) | 4@12 (A5) |
| 18 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 19 | Default | 40 | Default |  |  | QPSK | 144@47 (A2) |  | 72@23 (A2) | 36@11 (A2) |
| 20 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 21 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 22 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 23 | Default | 45 | Default |  |  | QPSK | 5@46 (A5) |  | 2@24 (A5) | 1@13 (A5) |
| 24 | Default | 45 | Default |  |  | QPSK | 5@167 (A5) |  | 2@85 (A5) | 1@43 (A5) |
| 25 | Default | 45 | Default |  |  | QPSK | 125@46 (A2) |  | 60@26 (A2) | 30@13 (A2) |
| 26 | Default | 45 | Default |  |  | QPSK | 162@0 (A1) |  | 81@0 (A1) | 40@0 (A1) |
| 27 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 28 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 29 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 30 | Default | 50 | Default |  |  | QPSK | 150@58 (A2) |  | 80@30 (A2) | 36@16 (A2) |
| 31 | Default | 50 | Default |  |  | QPSK | 216@0 (A1) |  | 108@0 (A1) | 54@0 (A1) |
| 32 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 33 | Default | 10 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  | - |
| 34 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |  |
| 35 | Default | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 36 | Default | 15 | Default |  |  | 256 QAM | 48@0 (A3) |  | 24@0 (A3) | 12@0 (A3) |
| 37 | Default | 20 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |
| 38 | Default | 20 | Default |  |  | 256 QAM | 50@0 (A3) |  | 15@0 (A3) | 12@0 (A3) |
| 39 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |
| 40 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 41 | Default | 25 | Default |  |  | 256 QAM | 18@18 (A3) |  | 9@9 (A3) | 4@5 (A3) |
| 42 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 43 | Default | 30 | Default |  |  | 256 QAM | 40@0 (A5) |  | 20@0 (A5) | 10@0 (A5) |
| 44 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 45 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 46 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 47 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A1) |  |  |  |
| 48 | Default | 40 | Default |  |  | 256 QAM | 50@0 (A1) |  | 25@0 (A1) | 12@0 (A1) |
| 49 | Default | 40 | Default |  |  | 256 QAM | 16@48 (A5) |  | 8@24 (A5) | 4@12 (A5) |
| 50 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 51 | Default | 40 | Default |  |  | 256 QAM | 144@47 (A2) |  | 72@23(A2) | 36@11 (A2) |
| 52 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 53 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 54 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 55 | Default | 45 | Default |  |  | 256 QAM | 5@46 (A5) |  | 2@24 (A5) | 1@13 (A5) |
| 56 | Default | 45 | Default |  |  | 256 QAM | 5@167 (A5) |  | 2@85 (A5) | 1@43 (A5) |
| 57 | Default | 45 | Default |  |  | 256 QAM | 125@46 (A2) |  | 60@26 (A2) | 30@13 (A2) |
| 58 | Default | 45 | Default |  |  | 256 QAM | 162@0 (A1) |  | 81@0 (A1) | 40@0 (A1) |
| 59 | Default | 45 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 60 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 61 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 62 | Default | 50 | Default |  |  | 256 QAM | 150@58 (A2) |  | 80@30 (A2) | 36@16 (A2) |
| 63 | Default | 50 | Default |  |  | 256 QAM | 216@0 (A1) |  | 108@0 (A1) | 54@0 (A1) |

| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 64 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 65 | Default | 10 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  | - |
| 66 | Default | 10 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |
| 67 | Default | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| 68 | Default | 15 | Default |  |  | QPSK | 48@0 (A3) | 24@0 (A3) | 12@0 (A3) |
| 69 | Default | 20 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 70 | Default | 20 | Default |  |  | QPSK | 52@0 (A3) | 26@0 (A3) | 13@0 (A3) |
| 71 | Default | 25 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 72 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 73 | Default | 25 | Default |  |  | QPSK | 18@18 (A3) | 9@9 (A3) | 4@5 (A3) |
| 74 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) | 10@0 (A1) | 5@0 (A1) |
| 75 | Default | 30 | Default |  |  | QPSK | 44@0 (A5) | 22@0 (A5) | 11@0 (A5) |
| 76 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) | 40@0 (A3) | 20@0 (A3) |
| 77 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) | 60@0 (A4) | 30@0 (A4) |
| 78 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |
| 79 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A1) |  |  |
| 80 | Default | 40 | Default |  |  | QPSK | 50@0 (A1) | 25@0 (A1) | 12@0 (A1) |
| 81 | Default | 40 | Default |  |  | QPSK | 16@48 (A5) | 8@24 (A5) | 4@12 (A5) |
| 82 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 83 | Default | 40 | Default |  |  | QPSK | 144@47 (A2) | 72@23(A2) | 36@11 (A2) |
| 84 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) | 2@94 (A5) | 1@47 (A5) |
| 85 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) | 96@0 (A1) | 48@0 (A1) |
| 86 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 87 | Default | 45 | Default |  |  | QPSK | 5@46 (A5) | 2@24 (A5) | 1@13 (A5) |
| 88 | Default | 45 | Default |  |  | QPSK | 5@167 (A5) | 2@85 (A5) | 1@43 (A5) |
| 89 | Default | 45 | Default |  |  | QPSK | 125@46 (A2) | 62@24 (A2) | 30@13 (A2) |
| 90 | Default | 45 | Default |  |  | QPSK | 171@0 (A1) | 86@0 (A1) | 42@0 (A1) |
| 91 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 92 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) | 2@38 (A5) | 1@19 (A5) |
| 93 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) | 2@108 (A5) | 1@54 (A5) |
| 94 | Default | 50 | Default |  |  | QPSK | 150@58 (A2) | 80@30 (A2) | 36@16 (A2) |
| 95 | Default | 50 | Default |  |  | QPSK | 216@0 (A1) | 108@0 (A1) | 54@0 (A1) |
| 96 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 97 | Default | 10 | Default |  |  | 256 QAM | Outer_Full (A3) |  | - |
| 98 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |
| 99 | Default | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 100 | Default | 15 | Default |  |  | 256 QAM | 48@0 (A3) | 24@0 (A3) | 12@0 (A3) |
| 101 | Default | 20 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |
| 102 | Default | 20 | Default |  |  | 256 QAM | 52@0 (A3) | 26@0 (A3) | 13@0 (A3) |
| 103 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |
| 104 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 105 | Default | 25 | Default |  |  | 256 QAM | 18@18 (A3) | 9@9 (A3) | 4@5 (A3) |
| 106 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) | 10@0 (A1) | 5@0 (A1) |
| 107 | Default | 30 | Default |  |  | 256 QAM | 44@0 (A5) | 22@0 (A5) | 11@0 (A5) |
| 108 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) | 40@0 (A3) | 20@0 (A3) |
| 109 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) | 60@0 (A4) | 30@0 (A4) |
| 110 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |
| 111 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A1) |  |  |
| 112 | Default | 40 | Default |  |  | 256 QAM | 50@0 (A1) | 25@0 (A1) | 12@0 (A1) |
| 113 | Default | 40 | Default |  |  | 256 QAM | 16@48 (A5) | 8@24 (A5) | 4@12 (A5) |
| 114 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 115 | Default | 40 | Default |  |  | 256 QAM | 144@47 (A2) | 72@23 (A2) | 36@11 (A2) |
| 116 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) | 2@94 (A5) | 1@47 (A5) |
| 117 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) | 96@0 (A1) | 48@0 (A1) |
| 118 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 119 | Default | 45 | Default |  |  | 256 QAM | 5@46 (A5) | 2@24 (A5) | 1@13 (A5) |
| 120 | Default | 45 | Default |  |  | 256 QAM | 5@167 (A5) | 2@85 (A5) | 1@43 (A5) |
| 121 | Default | 45 | Default |  |  | 256 QAM | 125@46 (A2) | 62@24 (A2) | 30@13 (A2) |
| 122 | Default | 45 | Default |  |  | 256 QAM | 171@0 (A1) | 86@0 (A1) | 42@0 (A1) |
| 123 | Default | 45 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 124 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) | 2@38 (A5) | 1@19 (A5) |
| 125 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) | 2@108 (A5) | 1@54 (A5) |
| 126 | Default | 50 | Default |  |  | 256 QAM | 150@58 (A2) | 80@30 (A2) | 36@16 (A2) |
| 127 | Default | 50 | Default |  |  | 256 QAM | 216@0 (A1) | 108@0 (A1) | 54@0 (A1) |
| 128 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-30: Test Configuration table for NS_56

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Low | 5 | 15 | N/A for A-MPR testing | 14 | DFT-s-OFDM | PI/2 BPSK |  | Edge_1RB_Left | N/A | N/A |
| 2 | Low | 5 | 30 |  | 8 |  | PI/2 BPSK |  | N/A | Edge_1RB_Left | N/A |
| 3 | Low | 5 | Default |  | 6 |  | PI/2 BPSK |  | Outer_Full | Outer_Full | N/A |
| 4 | Low | 5 | Default |  | 4 |  | PI/2 BPSK |  | 20@4 | 8@3 | N/A |
| 5 | Low | 5 | Default |  | 4 |  | PI/2 BPSK |  | 1@4 | 1@3 | N/A |
| 6 | Low | 5 | 15 |  | 14 |  | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 7 | Low | 5 | 30 |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | N/A |
| 8 | Low | 5 | Default |  | 6 |  | QPSK |  | Outer_Full | Outer_Full | N/A |
| 9 | Low | 5 | Default |  | 4 |  | QPSK |  | 20@4 | 8@3 | N/A |
| 10 | Low | 5 | Default |  | 4 |  | QPSK |  | 1@4 | 1@3 | N/A |
| 11 | Low | 5 | 15 |  | 14 |  | 16 QAM |  | Edge_1RB_Left | N/A | N/A |
| 12 | Low | 5 | 30 |  | 8 |  | 16 QAM |  | N/A | Edge_1RB_Left | N/A |
| 13 | Low | 5 | Default |  | 6 |  | 16 QAM |  | Outer_Full | Outer_Full | N/A |
| 14 | Low | 5 | Default |  | 4 |  | 16 QAM |  | 20@4 | 8@3 | N/A |
| 15 | Low | 5 | Default |  | 4 |  | 16 QAM |  | 1@4 | 1@3 | N/A |
| 16 | Low | 5 | 15 |  | 14 |  | 64 QAM |  | Edge_1RB_Left | N/A | N/A |
| 17 | Low | 5 | 30 |  | 8 |  | 64 QAM |  | N/A | Edge_1RB_Left | N/A |
| 18 | Low | 5 | Default |  | 6 |  | 64 QAM |  | Outer_Full | Outer_Full | N/A |
| 19 | Low | 5 | Default |  | 4 |  | 64 QAM |  | 20@4 | 8@3 | N/A |
| 20 | Low | 5 | Default |  | 4 |  | 64 QAM |  | 1@4 | 1@3 | N/A |
| 21 | Low | 5 | 15 |  | 14 |  | 256 QAM |  | Edge_1RB_Left | N/A | N/A |
| 22 | Low | 5 | 30 |  | 8 |  | 256 QAM |  | N/A | Edge_1RB_Left | N/A |
| 23 | Low | 5 | Default |  | 6 |  | 256 QAM |  | Outer_Full | Outer_Full | N/A |
| 24 | Low | 5 | Default |  | 4 |  | 256 QAM |  | 20@4 | 8@3 | N/A |
| 25 | Low | 5 | Default |  | 4 |  | 256 QAM |  | 1@4 | 1@3 | N/A |
| 26 | Low | 10 | 15 |  | 12 |  | PI/2 BPSK |  | Edge_1RB_Left | N/A | N/A |
| 27 | Low | 10 | Highest |  | 8 |  | PI/2 BPSK |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 28 | Low | 10 | Default |  | 8 |  | PI/2 BPSK |  | Outer_Full | Outer_Full | Outer_Full |
| 29 | Low | 10 | Default |  | 6 |  | PI/2 BPSK |  | 1@3 | 1@2 | 1@1 |
| 30 | Low | 10 | Default |  | 6 |  | PI/2 BPSK |  | 40@9 | 18@5 | 8@2 |
| 31 | Low | 10 | Default |  | 4 |  | PI/2 BPSK |  | 1@35 | 1@17 | 1@8 |
| 32 | Low | 10 | Default |  | 2 |  | PI/2 BPSK |  | 16@35 | 5@19 | 3@8 |
| 33 | Low | 10 | Default |  | 5 |  | PI/2 BPSK |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 34 | Low | 10 | 15 |  | 5 |  | PI/2 BPSK |  | 1@40 | N/A | N/A |
| 35 | Low | 10 | 30 |  | 5 |  | PI/2 BPSK |  | N/A | 1@20 | N/A |
| 36 | Low | 10 | 15 |  | 3 |  | PI/2 BPSK |  | 8@44 | N/A | N/A |
| 37 | Low | 10 | 15 |  | 12 |  | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 38 | Low | 10 | Highest |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 39 | Low | 10 | Default |  | 8 |  | QPSK |  | Outer_Full | Outer_Full | Outer_Full |
| 40 | Low | 10 | Default |  | 6 |  | QPSK |  | 1@3 | 1@2 | 1@1 |
| 41 | Low | 10 | Default |  | 6 |  | QPSK |  | 40@9 | 18@5 | 8@2 |
| 42 | Low | 10 | Default |  | 4 |  | QPSK |  | 1@35 | 1@17 | 1@8 |
| 43 | Low | 10 | Default |  | 2 |  | QPSK |  | 16@35 | 5@19 | 3@8 |
| 44 | Low | 10 | Default |  | 5 |  | QPSK |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 45 | Low | 10 | 15 |  | 5 |  | QPSK |  | 1@40 | N/A | N/A |

| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 46 | Low | 10 | 30 |  | 5 |  | QPSK | N/A | 1@20 | N/A |
| 47 | Low | 10 | 15 |  | 3 |  | QPSK | 8@44 | N/A | N/A |
| 48 | Low | 10 | 15 |  | 12 |  | 16 QAM | Edge_1RB_Left | N/A | N/A |
| 49 | Low | 10 | Highest |  | 8 |  | 16 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 50 | Low | 10 | Default |  | 8 |  | 16 QAM | Outer_Full | Outer_Full | Outer_Full |
| 51 | Low | 10 | Default |  | 6 |  | 16 QAM | 1@3 | 1@2 | 1@1 |
| 52 | Low | 10 | Default |  | 6 |  | 16 QAM | 40@9 | 18@5 | 8@2 |
| 53 | Low | 10 | Default |  | 4 |  | 16 QAM | 1@35 | 1@17 | 1@8 |
| 54 | Low | 10 | Default |  | 2 |  | 16 QAM | 16@35 | 5@19 | 3@8 |
| 55 | Low | 10 | Default |  | 5 |  | 16 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 56 | Low | 10 | 15 |  | 5 |  | 16 QAM | 1@40 | N/A | N/A |
| 57 | Low | 10 | 30 |  | 5 |  | 16 QAM | N/A | 1@20 | N/A |
| 58 | Low | 10 | 15 |  | 3 |  | 16 QAM | 8@44 | N/A | N/A |
| 59 | Low | 10 | 15 |  | 12 |  | 64 QAM | Edge_1RB_Left | N/A | N/A |
| 60 | Low | 10 | Highest |  | 8 |  | 64 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 61 | Low | 10 | Default |  | 8 |  | 64 QAM | Outer_Full | Outer_Full | Outer_Full |
| 62 | Low | 10 | Default |  | 6 |  | 64 QAM | 1@3 | 1@2 | 1@1 |
| 63 | Low | 10 | Default |  | 6 |  | 64 QAM | 40@9 | 18@5 | 8@2 |
| 64 | Low | 10 | Default |  | 4 |  | 64 QAM | 1@35 | 1@17 | 1@8 |
| 65 | Low | 10 | Default |  | 2 |  | 64 QAM | 16@35 | 5@19 | 3@8 |
| 66 | Low | 10 | Default |  | 5 |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 67 | Low | 10 | 15 |  | 5 |  | 64 QAM | 1@40 | N/A | N/A |
| 68 | Low | 10 | 30 |  | 5 |  | 64 QAM | N/A | 1@20 | N/A |
| 69 | Low | 10 | 15 |  | 3 |  | 64 QAM | 8@44 | N/A | N/A |
| 70 | Low | 10 | 15 |  | 12 |  | 256 QAM | Edge_1RB_Left | N/A | N/A |
| 71 | Low | 10 | Highest |  | 8 |  | 256 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 72 | Low | 10 | Default |  | 8 |  | 256 QAM | Outer_Full | Outer_Full | Outer_Full |
| 73 | Low | 10 | Default |  | 6 |  | 256 QAM | 1@3 | 1@2 | 1@1 |
| 74 | Low | 10 | Default |  | 6 |  | 256 QAM | 40@9 | 18@5 | 8@2 |
| 75 | Low | 10 | Default |  | 4 |  | 256 QAM | 1@35 | 1@17 | 1@8 |
| 76 | Low | 10 | Default |  | 2 |  | 256 QAM | 16@35 | 5@19 | 3@8 |
| 77 | Low | 10 | Default |  | 5 |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 78 | Low | 10 | 15 |  | 5 |  | 256 QAM | 1@40 | N/A | N/A |
| 79 | Low | 10 | 30 |  | 5 |  | 256 QAM | N/A | 1@20 | N/A |
| 80 | Low | 10 | 15 |  | 3 |  | 256 QAM | 8@44 | N/A | N/A |
| 81 | Low | 5 | 15 | N/A for A-MPR testing | 14 | CP-OFDM | QPSK | Edge_1RB_Left | N/A | N/A |
| 82 | Low | 5 | 30 |  | 8 |  | QPSK | N/A | Edge_1RB_Left | N/A |
| 83 | Low | 5 | Default |  | 6 |  | QPSK | Outer_Full | Outer_Full | N/A |
| 84 | Low | 5 | Default |  | 4 |  | QPSK | 21@4 | 8@3 | N/A |
| 85 | Low | 5 | Default |  | 4 |  | QPSK | 1@4 | 1@3 | N/A |
| 86 | Low | 5 | 15 |  | 14 |  | 16 QAM | Edge_1RB_Left | N/A | N/A |
| 87 | Low | 5 | 30 |  | 8 |  | 16 QAM | N/A | Edge_1RB_Left | N/A |
| 88 | Low | 5 | Default |  | 6 |  | 16 QAM | Outer_Full | Outer_Full | N/A |
| 89 | Low | 5 | Default |  | 4 |  | 16 QAM | 21@4 | 8@3 | N/A |
| 90 | Low | 5 | Default |  | 4 |  | 16 QAM | 1@4 | 1@3 | N/A |
| 91 | Low | 5 | 15 |  | 14 |  | 64 QAM | Edge_1RB_Left | N/A | N/A |
| 92 | Low | 5 | 30 |  | 8 |  | 64 QAM | N/A | Edge_1RB_Left | N/A |
| 93 | Low | 5 | Default |  | 6 |  | 64 QAM | Outer_Full | Outer_Full | N/A |
| 94 | Low | 5 | Default |  | 4 |  | 64 QAM | 21@4 | 8@3 | N/A |
| 95 | Low | 5 | Default |  | 4 |  | 64 QAM | 1@4 | 1@3 | N/A |
| 96 | Low | 5 | 15 |  | 14 |  | 256 QAM | Edge_1RB_Left | N/A | N/A |
| 97 | Low | 5 | 30 |  | 8 |  | 256 QAM | N/A | Edge_1RB_Left | N/A |
| 98 | Low | 5 | Default |  | 6 |  | 256 QAM | Outer_Full | Outer_Full | N/A |
| 99 | Low | 5 | Default |  | 4 |  | 256 QAM | 21@4 | 8@3 | N/A |

| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 100 | Low | 5 | Default |  | 4 |  | 256 QAM | 1@4 | 1@3 | N/A |
| 101 | Low | 10 | 15 |  | 12 |  | QPSK | Edge_1RB_Left | N/A | N/A |
| 102 | Low | 10 | Highest |  | 8 |  | QPSK | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 103 | Low | 10 | Default |  | 8 |  | QPSK | Outer_Full | Outer_Full | Outer_Full |
| 104 | Low | 10 | Default |  | 6 |  | QPSK | 1@3 | 1@2 | 1@1 |
| 105 | Low | 10 | Default |  | 6 |  | QPSK | 43@9 | 19@5 | 8@2 |
| 106 | Low | 10 | Default |  | 4 |  | QPSK | 1@35 | 1@17 | 1@8 |
| 107 | Low | 10 | Default |  | 2 |  | QPSK | 17@35 | 5@19 | 3@8 |
| 108 | Low | 10 | Default |  | 5 |  | QPSK | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 109 | Low | 10 | 15 |  | 5 |  | QPSK | 1@40 | N/A | N/A |
| 110 | Low | 10 | 30 |  | 5 |  | QPSK | N/A | 1@20 | N/A |
| 111 | Low | 10 | 15 |  | 3 |  | QPSK | 8@44 | N/A | N/A |
| 112 | Low | 10 | 15 |  | 12 |  | 16 QAM | Edge_1RB_Left | N/A | N/A |
| 113 | Low | 10 | Highest |  | 8 |  | 16 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 114 | Low | 10 | Default |  | 8 |  | 16 QAM | Outer_Full | Outer_Full | Outer_Full |
| 115 | Low | 10 | Default |  | 6 |  | 16 QAM | 1@3 | 1@2 | 1@1 |
| 116 | Low | 10 | Default |  | 6 |  | 16 QAM | 43@9 | 19@5 | 8@2 |
| 117 | Low | 10 | Default |  | 4 |  | 16 QAM | 1@35 | 1@17 | 1@8 |
| 118 | Low | 10 | Default |  | 2 |  | 16 QAM | 17@35 | 5@19 | 3@8 |
| 119 | Low | 10 | Default |  | 5 |  | 16 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 120 | Low | 10 | 15 |  | 5 |  | 16 QAM | 1@40 | N/A | N/A |
| 121 | Low | 10 | 30 |  | 5 |  | 16 QAM | N/A | 1@20 | N/A |
| 122 | Low | 10 | 15 |  | 3 |  | 16 QAM | 8@44 | N/A | N/A |
| 123 | Low | 10 | 15 |  | 12 |  | 64 QAM | Edge_1RB_Left | N/A | N/A |
| 124 | Low | 10 | Highest |  | 8 |  | 64 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 125 | Low | 10 | Default |  | 8 |  | 64 QAM | Outer_Full | Outer_Full | Outer_Full |
| 126 | Low | 10 | Default |  | 6 |  | 64 QAM | 1@3 | 1@2 | 1@1 |
| 127 | Low | 10 | Default |  | 6 |  | 64 QAM | 43@9 | 19@5 | 8@2 |
| 128 | Low | 10 | Default |  | 4 |  | 64 QAM | 1@35 | 1@17 | 1@8 |
| 129 | Low | 10 | Default |  | 2 |  | 64 QAM | 17@35 | 5@19 | 3@8 |
| 130 | Low | 10 | Default |  | 5 |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 131 | Low | 10 | 15 |  | 5 |  | 64 QAM | 1@40 | N/A | N/A |
| 132 | Low | 10 | 30 |  | 5 |  | 64 QAM | N/A | 1@20 | N/A |
| 133 | Low | 10 | 15 |  | 3 |  | 64 QAM | 8@44 | N/A | N/A |
| 134 | Low | 10 | 15 |  | 12 |  | 256 QAM | Edge_1RB_Left | N/A | N/A |
| 135 | Low | 10 | Highest |  | 8 |  | 256 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 136 | Low | 10 | Default |  | 8 |  | 256 QAM | Outer_Full | Outer_Full | Outer_Full |
| 137 | Low | 10 | Default |  | 6 |  | 256 QAM | 1@3 | 1@2 | 1@1 |
| 138 | Low | 10 | Default |  | 6 |  | 256 QAM | 43@9 | 19@5 | 8@2 |
| 139 | Low | 10 | Default |  | 4 |  | 256 QAM | 1@35 | 1@17 | 1@8 |
| 140 | Low | 10 | Default |  | 2 |  | 256 QAM | 17@35 | 5@19 | 3@8 |
| 141 | Low | 10 | Default |  | 5 |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 142 | Low | 10 | 15 |  | 5 |  | 256 QAM | 1@40 | N/A | N/A |
| 143 | Low | 10 | 30 |  | 5 |  | 256 QAM | N/A | 1@20 | N/A |
| 144 | Low | 10 | 15 |  | 3 |  | 256 QAM | 8@44 | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-31: Test Configuration table for NS_07 and NS_50

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_07 and NS_50 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |
| 1 | Low | Default | Default | N/A for A-MPR test cases | DFT-s-OFDM | PI/2 BPSK | Edge_1RB_Left |  |
| 2 | Low | 5 MHz | Default |  |  | PI/2 BPSK | 8@6 | 4@3 |
| 3 | Low | 10 MHz | Default |  |  | PI/2 BPSK | 8@14 | 4@7 |
| 4 | Low | 5 MHz | Default |  |  | PI/2 BPSK | 10@8 | 5@4 |
| 5 | Low | 10 MHz | Default |  |  | PI/2 BPSK | 18@19 | 10@10 |
| 6 | High | 5 MHz | Default |  |  | PI/2 BPSK | 10@0 | 5@0 |
| 7 | Low | 5 MHz | Default |  |  | PI/2 BPSK | 2@20 | 1@10 |
| 8 | Low | 10 MHz | Default |  |  | PI/2 BPSK | 2@37 | 1@19 |
| 9 | Low | Default | Default |  |  | QPSK | Edge_1RB_Left |  |
| 10 | Low | 5 MHz | Default |  |  | QPSK | 8@6 | 4@3 |
| 11 | Low | 10 MHz | Default |  |  | QPSK | 8@14 | 4@7 |
| 12 | Low | 5 MHz | Default |  |  | QPSK | 10@8 | 5@4 |
| 13 | Low | 10 MHz | Default |  |  | QPSK | 18@19 | 10@10 |
| 14 | High | 5 MHz | Default |  |  | QPSK | 10@0 | 5@0 |
| 15 | Low | 5 MHz | Default |  |  | QPSK | 2@20 | 1@10 |
| 16 | Low | 10 MHz | Default |  |  | QPSK | 2@37 | 1@19 |
| 17 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |  |
| 18 | Low | 5 MHz | Default |  |  | 16 QAM | 8@6 | 4@3 |
| 19 | Low | 10 MHz | Default |  |  | 16 QAM | 8@14 | 4@7 |
| 20 | Low | 5 MHz | Default |  |  | 16 QAM | 10@8 | 5@4 |
| 21 | Low | 10 MHz | Default |  |  | 16 QAM | 18@19 | 10@10 |
| 22 | High | 5 MHz | Default |  |  | 16 QAM | 10@0 | 5@0 |
| 23 | Low | 5 MHz | Default |  |  | 16 QAM | 2@20 | 1@10 |
| 24 | Low | 10 MHz | Default |  |  | 16 QAM | 2@37 | 1@19 |
| 25 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |  |
| 26 | Low | 5 MHz | Default |  |  | 64 QAM | 8@6 | 4@3 |
| 27 | Low | 10 MHz | Default |  |  | 64 QAM | 8@14 | 4@7 |
| 28 | Low | 5 MHz | Default |  |  | 64 QAM | 10@8 | 5@4 |
| 29 | Low | 10 MHz | Default |  |  | 64 QAM | 18@19 | 10@10 |
| 30 | High | 5 MHz | Default |  |  | 64 QAM | 10@0 | 5@0 |
| 31 | Low | 5 MHz | Default |  |  | 64 QAM | 2@20 | 1@10 |
| 32 | Low | 10 MHz | Default |  |  | 64 QAM | 2@37 | 1@19 |
| 33 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |  |
| 34 | Low | 5 MHz | Default |  |  | 256 QAM | 8@6 | 4@3 |
| 35 | Low | 10 MHz | Default |  |  | 256 QAM | 8@14 | 4@7 |
| 36 | Low | 5 MHz | Default |  |  | 256 QAM | 10@8 | 5@4 |
| 37 | Low | 10 MHz | Default |  |  | 256 QAM | 18@19 | 10@10 |
| 38 | High | 5 MHz | Default |  |  | 256 QAM | 10@0 | 5@0 |
| 39 | Low | 5 MHz | Default |  |  | 256 QAM | 2@20 | 1@10 |
| 40 | Low | 10 MHz | Default |  |  | 256 QAM | 2@37 | 1@19 |
| 41 | Low | Default | Default |  | CP-OFDM | QPSK | Edge_1RB_Left |  |
| 42 | Low | 5 MHz | Default |  |  | QPSK | 7@6 | 4@3 |
| 43 | Low | 10 MHz | Default |  |  | QPSK | 8@14 | 4@7 |
| 44 | Low | 5 MHz | Default |  |  | QPSK | 10@8 | 5@4 |
| 45 | Low | 10 MHz | Default |  |  | QPSK | 18@19 | 10@10 |
| 46 | High | 5 MHz | Default |  |  | QPSK | 10@0 | 5@0 |
| 47 | Low | 5 MHz | Default |  |  | QPSK | 2@20 | 1@10 |
| 48 | Low | 10 MHz | Default |  |  | QPSK | 2@37 | 1@19 |
| 49 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |  |
| 50 | Low | 5 MHz | Default |  |  | 16 QAM | 7@6 | 4@3 |
| 51 | Low | 10 MHz | Default |  |  | 16 QAM | 8@14 | 4@7 |
| 52 | Low | 5 MHz | Default |  |  | 16 QAM | 10@8 | 5@4 |
| 53 | Low | 10 MHz | Default |  |  | 16 QAM | 18@19 | 10@10 |
| 54 | High | 5 MHz | Default |  |  | 16 QAM | 10@0 | 5@0 |
| 55 | Low | 5 MHz | Default |  |  | 16 QAM | 2@20 | 1@10 |
| 56 | Low | 10 MHz | Default |  |  | 16 QAM | 2@37 | 1@19 |
| 57 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |  |
| 58 | Low | 5 MHz | Default |  |  | 64 QAM | 7@6 | 4@3 |
| 59 | Low | 10 MHz | Default |  |  | 64 QAM | 8@14 | 4@7 |
| 60 | Low | 5 MHz | Default |  |  | 64 QAM | 10@8 | 5@4 |
| 61 | Low | 10 MHz | Default |  |  | 64 QAM | 18@19 | 10@10 |
| 62 | High | 5 MHz | Default |  |  | 64 QAM | 10@0 | 5@0 |
| 63 | Low | 5 MHz | Default |  |  | 64 QAM | 2@20 | 1@10 |
| 64 | Low | 10 MHz | Default |  |  | 64 QAM | 2@37 | 1@19 |
| 65 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |  |
| 66 | Low | 5 MHz | Default |  |  | 256 QAM | 7@6 | 4@3 |
| 67 | Low | 10 MHz | Default |  |  | 256 QAM | 8@14 | 4@7 |
| 68 | Low | 5 MHz | Default |  |  | 256 QAM | 10@8 | 5@4 |
| 69 | Low | 10 MHz | Default |  |  | 256 QAM | 18@19 | 10@10 |
| 70 | High | 5 MHz | Default |  |  | 256 QAM | 10@0 | 5@0 |
| 71 | Low | 5 MHz | Default |  |  | 256 QAM | 2@20 | 1@10 |
| 72 | Low | 10 MHz | Default |  |  | 256 QAM | 2@37 | 1@19 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-32: Test Configuration table for NS_50 (Power Class 2)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | (See Freq column for test frequency, and configuration detail specified in Table 6.2.3.4.1-32a, Table 6.2.3.4.1-32b and Table 6.2.3.4.1-32c) |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 10MHz, 15MHz, 20MHz, 25 MHz, 30MHz, 35MHz, 40MHz |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| A-MPR test parameters for NS_50 (Power Class 2) |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |
| 1 | 1890 | 10 | Default | N/A | DFT-s-OFDM | QPSK | Edge_1RB_left (A5) |  |  |  |
| 2 | 1890 | 10 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 3 | 1890 | 10 | Default |  |  | QPSK | 40@11 (A4) | 18@6 (A4) |  | / |
| 4 | 1892.5 | 15 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 5 | 1892.5 | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 6 | 1892.5 | 15 | Default |  |  | QPSK | 60@19 (A4) | 27@10 (A4) |  | 15@5 (A4) |
| 7 | 1895 | 20 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 8 | 1895 | 20 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 9 | 1895 | 20 | Default |  |  | QPSK | 80@26 (A4) | 36@13 (A4) |  | 18@7 (A4) |
| 10 | 1897.5 | 25 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 11 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 12 | 1897.5 | 25 | Default |  |  | QPSK | 75@48 (A4) | 36@25 (A4) |  | 18@13 (A4) |
| 13 | 1897.5 | 25 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |
| 14 | 1897.5 | 25 | Default |  |  | QPSK | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |
| 15 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 16 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 17 | 1900 | 30 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |
| 18 | 1900 | 30 | Default |  |  | QPSK | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |
| 19 | 1900 | 30 | Default |  |  | QPSK | 90@63 (A4) | 45@32 (A4) |  | 24@18 (A4) |
| 20 | 1900 | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 21 | 1900 | 35 | Default |  |  | QPSK | 50@10 (A3) | 25@5 (A3) |  | 12@3 (A3) |
| 22 | 1900 | 35 | Default |  |  | QPSK | 178@10 (A8) | 87@5 (A8) |  | 40@3 (A8) |
| 23 | 1900 | 35 | Default |  |  | QPSK | 111@77 (A4) | 52@39 (A4) |  | 24@20 (A4) |
| 24 | 1900 | 35 | Default |  |  | QPSK | Edge 1RB right (A9) |  |  |  |
| 25 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 26 | 1900 | 40 | Default |  |  | QPSK | 180@25 (A8) | 90@13 (A8) |  | 40@7 (A8) |
| 27 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |
| 28 | 1900 | 40 | Default |  |  | QPSK | 90@25 (A7) | 45@13 (A7) |  | 20@7 (A7) |
| 29 | 1900 | 40 | Default |  |  | QPSK | 108@101 (A4) | 54@51 (A4) |  | 25@26 (A4) |
| 30 | 1900 | 40 | Default |  |  | QPSK | 36@177 (A9) | 16@89 (A9) |  | 6@45 (A9) |
| 31 | 1890 | 10 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 32 | 1890 | 10 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |
| 33 | 1890 | 10 | Default |  |  | 256 QAM | 40@11 (A4) | 18@6 (A4) |  | / |
| 34 | 1892.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 35 | 1892.5 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 36 | 1892.5 | 15 | Default |  |  | 256 QAM | 60@19 (A4) | 27@10 (A4) |  | 15@5 (A4) |
| 37 | 1895 | 20 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 38 | 1895 | 20 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 39 | 1895 | 20 | Default |  |  | 256 QAM | 80@26 (A4) | 36@13 (A4) |  | 18@7 (A4) |
| 40 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 41 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 42 | 1897.5 | 25 | Default |  |  | 256 QAM | 75@48 (A4) | 36@25 (A4) |  | 18@13 (A4) |
| 43 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |
| 44 | 1897.5 | 25 | Default |  |  | 256 QAM | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |
| 45 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 46 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 47 | 1900 | 30 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |
| 48 | 1900 | 30 | Default |  |  | 256 QAM | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |
| 49 | 1900 | 30 | Default |  |  | 256 QAM | 90@63 (A4) | 45@32 (A4) |  | 24@18 (A4) |
| 50 | 1900 | 35 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 51 | 1900 | 35 | Default |  |  | 256 QAM | 50@10 (A3) | 25@5 (A3) |  | 12@3 (A3) |
| 52 | 1900 | 35 | Default |  |  | 256 QAM | 178@10 (A8) | 87@5 (A8) |  | 40@3 (A8) |
| 53 | 1900 | 35 | Default |  |  | 256 QAM | 111@77 (A4) | 52@39 (A4) |  | 24@20 (A4) |
| 54 | 1900 | 35 | Default |  |  | 256 QAM | Edge 1RB right (A9) |  |  |  |
| 55 | Low | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 56 | Low | 40 | Default |  |  | 256 QAM | 180@25 (A8) | 90@13 (A8) |  | 40@7 (A8) |
| 57 | Low | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |
| 58 | Low | 40 | Default |  |  | 256 QAM | 90@25 (A7) | 45@13 (A7) |  | 20@7 (A7) |
| 59 | Low | 40 | Default |  |  | 256 QAM | 108@101 (A4) | 54@51 (A4) |  | 25@26 (A4) |
| 60 | Low | 40 | Default |  |  | 256 QAM | 36@177 (A9) | 16@89 (A9) |  | 6@45 (A9) |
| 61 | 1890 | 10 | Default | N/A | CP-OFDM | QPSK | Edge_1RB_left (A5) |  |  |  |
| 62 | 1890 | 10 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 63 | 1890 | 10 | Default |  |  | QPSK | 41@11 (A4) | 18@6 (A4) | / |  |
| 64 | 1892.5 | 15 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 65 | 1892.5 | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 66 | 1892.5 | 15 | Default |  |  | QPSK | 60@19 (A4) | 28@10 (A4) | 13@5 (A4) |  |
| 67 | 1895 | 20 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 68 | 1895 | 20 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 69 | 1895 | 20 | Default |  |  | QPSK | 80@26 (A4) | 38@13 (A4) | 17@7 (A4) |  |
| 70 | 1897.5 | 25 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 71 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 72 | 1897.5 | 25 | Default |  |  | QPSK | 74@47 (A4) | 37@24 (A4) | 19@13 (A4) |  |
| 73 | 1897.5 | 25 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) | 5@3 (A6) |  |
| 74 | 1897.5 | 25 | Default |  |  | QPSK | 27@0 (A4) | 13@0 (A4) | 6@0 (A4) |  |
| 75 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 76 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |
| 77 | 1900 | 30 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) | 5@3 (A6) |  |
| 78 | 1900 | 30 | Default |  |  | QPSK | 27@0 (A4) | 13@0 (A4) | 6@0 (A4) |  |
| 79 | 1900 | 30 | Default |  |  | QPSK | 88@61 (A4) | 44@31 (A4) | 22@16 (A4) |  |
| 80 | 1900 | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 81 | 1900 | 35 | Default |  |  | QPSK | 50@10 (A3) | 25@5 | 12@3 |  |
| 82 | 1900 | 35 | Default |  |  | QPSK | 178@10 (A8) | 87@5 (A8) | 41@3 (A8) |  |
| 83 | 1900 | 35 | Default |  |  | QPSK | 111@77 (A4) | 53@39 (A4) | 24@20 (A4) |  |
| 84 | 1900 | 35 | Default |  |  | QPSK | Edge 1RB right (A9) |  |  |  |
| 85 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 86 | 1900 | 40 | Default |  |  | QPSK | 191@25 (A8) | 93@13 (A8) | 44@7 (A8) |  |
| 87 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |  |
| 88 | 1900 | 40 | Default |  |  | QPSK | 91@25 (A7) | 46@13 (A7) | 23@7 (A7) |  |
| 89 | 1900 | 40 | Default |  |  | QPSK | 115@101 (A4) | 55@51 (A4) | 25@26 (A4) |  |
| 90 | 1900 | 40 | Default |  |  | QPSK | 39@177 (A9) | 17@89 (A9) | 6@45 (A9) |  |
| 91 | 1890 | 10 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 92 | 1890 | 10 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |
| 93 | 1890 | 10 | Default |  |  | 256 QAM | 41@11 (A4) | 18@6 (A4) | / |  |
| 94 | 1892.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 95 | 1892.5 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 96 | 1892.5 | 15 | Default |  |  | 256 QAM | 60@19 (A4) | 28@10 (A4) | 13@5 (A4) |  |
| 97 | 1895 | 20 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 98 | 1895 | 20 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 99 | 1895 | 20 | Default |  |  | 256 QAM | 80@26 (A4) | 38@13 (A4) | 17@7 (A4) |  |
| 100 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 101 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 102 | 1897.5 | 25 | Default |  |  | 256 QAM | 74@47 (A4) | 37@24 (A4) | 19@13 (A4) |  |
| 103 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) | 5@3 (A6) |  |
| 104 | 1897.5 | 25 | Default |  |  | 256 QAM | 27@0 (A4) | 13@0 (A4) | 6@0 (A4) |  |
| 105 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 106 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |
| 107 | 1900 | 30 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) | 5@3 (A6) |  |
| 108 | 1900 | 30 | Default |  |  | 256 QAM | 27@0 (A4) | 13@0 (A4) | 6@0 (A4) |  |
| 109 | 1900 | 30 | Default |  |  | 256 QAM | 88@61 (A4) | 44@31 (A4) | 22@16 (A4) |  |
| 110 | 1900 | 35 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 111 | 1900 | 35 | Default |  |  | 256 QAM | 50@10 (A3) | 25@5 (A3) | 12@3 (A3) |  |
| 112 | 1900 | 35 | Default |  |  | 256 QAM | 178@10 (A8) | 87@5 (A8) | 41@3 (A8) |  |
| 113 | 1900 | 35 | Default |  |  | 256 QAM | 111@77 (A4) | 53@39 (A4) | 24@20 (A4) |  |
| 114 | 1900 | 35 | Default |  |  | 256 QAM | Edge 1RB right (A9) |  |  |  |
| 115 | 1900 | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 116 | 1900 | 40 | Default |  |  | 256 QAM | 191@25 (A8) | 93@13 (A8) | 44@7 (A8) |  |
| 117 | 1900 | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |  |
| 118 | 1900 | 40 | Default |  |  | 256 QAM | 91@25 (A7) | 46@13 (A7) | 23@7 (A7) |  |
| 119 | 1900 | 40 | Default |  |  | 256 QAM | 115@101 (A4) | 55@51 (A4) | 25@26 (A4) |  |
| 120 | 1900 | 40 | Default |  |  | 256 QAM | 39@177 (A9) | 17@89 (A9) | 6@45 (A9) |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-32a: Additional test frequencies for NS_50 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | DL & UL | - | 1890 | 378000 | 1885.32 | 377064 | 0 | 30 | 4725 | 378030 | 10 | 0 | 1(6) | 6 |
| 15 | 79 | DL & UL | - | 1892.5 | 378500 | 1885.39 | 377078 | 0 | 15 | 4729 | 378290 | 8 | 7 | 7(16) | 23 |
| 20 | 106 | DL & UL | - | 1895 | 379000 | 1885.46 | 377092 | 0 | 15 | 4718 | 377530 | 2 | 0 | 1(2) | 2 |
| 25 | 133 | DL & UL | - | 1897.5 | 379500 | 1885.53 | 377106 | 0 | 15 | 4743 | 379470 | 8 | 17 | 12(38) | 55 |
| 35 | 188 | DL & UL | - | 1900 | 380000 | 1883.08 | 376616 | 0 | 15 | 4750 | 379970 | 2 | 81 | 1(2) | 83 |
| 30 | 160 | DL & UL | - | 1900 | 380000 | 1885.6 | 377120 | 0 | 15 | 4729 | 378290 | 6 | 6 | 7(16) | 22 |
| 40 | 216 | DL & UL | - | 1900 | 380000 | 1880.56 | 376112 | 0 | 15 | 4729 | 378290 | 6 | 12 | 12(38) | 50 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-32b: Additional test frequencies for NS_50 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | DL & UL | - | 1890 | 378000 | 1885.68 | 377136 | 0 | 30 | 4725 | 378030 | 10 | 0 | 2(2) | 4 |
| 15 | 38 | DL & UL | - | 1892.5 | 378500 | 1885.66 | 377132 | 0 | 15 | 4729 | 378290 | 2 | 3 | 3(8) | 22 |
| 20 | 51 | DL & UL | - | 1895 | 379000 | 1885.82 | 377164 | 0 | 15 | 4754 | 380410 | 2 | 20 | 9(20) | 80 |
| 25 | 65 | DL & UL | - | 1897.5 | 379500 | 1885.8 | 377160 | 0 | 15 | 4743 | 379470 | 2 | 7 | 9(20) | 54 |
| 30 | 78 | DL & UL | - | 1900 | 380000 | 1885.96 | 377192 | 0 | 15 | 4729 | 378290 | 6 | 2 | 3(8) | 20 |
| 35 | 92 | DL & UL | - | 1900 | 380000 | 1883.44 | 376688 | 0 | 15 | 4750 | 379970 | 14 | 32 | 3(8) | 81 |
| 40 | 106 | DL & UL | - | 1900 | 380000 | 1880.92 | 376184 | 0 | 15 | 4729 | 378290 | 6 | 4 | 9(20) | 48 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-32c: Additional test frequencies for NS_50 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | DL & UL | - | 1890 | 378000 | 1886.04 | 377208 | 0 | 15 | - | 377568 |
| 15 | 18 | DL & UL | - | 1892.5 | 378500 | 1886.02 | 377204 | 0 | 15 | - | 377564 |
| 20 | 24 | DL & UL | - | 1895 | 379000 | 1886.36 | 377272 | 0 | 15 | - | 377632 |
| 25 | 31 | DL & UL | - | 1897.5 | 379500 | 1886.34 | 377268 | 0 | 15 | - | 377628 |
| 30 | 38 | DL & UL | - | 1900 | 380000 | 1886.32 | 377264 | 0 | 15 | - | 377624 |
| 35 | 44 | DL & UL | - | 1900 | 380000 | 1884.16 | 376832 | 0 | 15 |  | 377192 |
| 40 | 51 | DL & UL | - | 1900 | 380000 | 1881.64 | 376328 | 0 | 15 | - | 376688 |

Table 6.2.3.4.1-33: Test Configuration table for NS_50 (Power Class 3)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | (See Freq column) |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 25 MHz, 30MHz, 35MHz, 40MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_50 (Power Class 3) |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 1897.5 | 25 | Default | N/A | DFT-s-OFDM | QPSK | Outer_Full (A7) |  |  |
| 2 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |
| 3 | 1897.5 | 25 | Default |  |  | QPSK | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 4 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A7) |  |  |
| 5 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |
| 6 | 1900 | 30 | Default |  |  | QPSK | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 7 | 1900 | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 8 | 1900 | 35 | Default |  |  | QPSK | 50@10 (A3) | 25@5 (A3) | 12@3 (A3) |
| 9 | 1900 | 35 | Default |  |  | QPSK | 178@10 (A2) | 87@5 (A2) | 40@3 (A2) |
| 10 | 1900 | 35 | Default |  |  | QPSK | 111@77 (A6) | 52@39 (A6) | 24@20 (A6) |
| 11 | 1900 | 35 | Default |  |  | QPSK | Edge 1RB right (A9) |  |  |
| 12 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 13 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |
| 14 | 1900 | 40 | Default |  |  | QPSK | 180@25 (A2) | 90@13 (A2) | 40@7 (A2) |
| 15 | 1900 | 40 | Default |  |  | QPSK | 108@101 (A6) | 54@51 (A6) | 25@26 (A6) |
| 16 | 1900 | 40 | Default |  |  | QPSK | 36@177 (A5) | 16@89 (A5) | 6@45 (A5) |
| 17 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |
| 18 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |
| 19 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 20 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |
| 21 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |
| 22 | 1900 | 30 | Default |  |  | 256 QAM | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 23 | 1900 | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 24 | 1900 | 35 | Default |  |  | QPSK | 50@10 (A3) | 25@5 (A3) | 12@3 (A3) |
| 25 | 1900 | 35 | Default |  |  | QPSK | 178@10 (A2) | 87@5 (A2) | 40@3 (A2) |
| 26 | 1900 | 35 | Default |  |  | QPSK | 111@77 (A6) | 52@39 (A6) | 24@20 (A6) |
| 27 | 1900 | 35 | Default |  |  | QPSK | Edge 1RB right (A9) |  |  |
| 28 | 1900 | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 29 | 1900 | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |
| 30 | 1900 | 40 | Default |  |  | 256 QAM | 180@25 (A2) | 90@13 (A2) | 40@7 (A2) |
| 31 | 1900 | 40 | Default |  |  | 256 QAM | 108@101 (A6) | 54@51 (A6) | 25@26 (A6) |
| 32 | 1900 | 40 | Default |  |  | 256 QAM | 36@177 (A5) | 16@89 (A5) | 6@45 (A5) |
| 33 | 1897.5 | 25 | Default | N/A | CP-OFDM | QPSK | Outer_Full (A7) |  |  |
| 34 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |
| 35 | 1897.5 | 25 | Default |  |  | QPSK | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 36 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A7) |  |  |
| 37 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |
| 38 | 1900 | 30 | Default |  |  | QPSK | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 39 | 1900 | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 40 | 1900 | 35 | Default |  |  | QPSK | 50@10 (A3) | 25@5 (A3) | 12@3 (A3) |
| 41 | 1900 | 35 | Default |  |  | QPSK | 178@10 (A2) | 87@5 (A2) | 41@3 (A2) |
| 42 | 1900 | 35 | Default |  |  | QPSK | 111@77 (A6) | 53@39 (A6) | 24@20 (A6) |
| 43 | 1900 | 35 | Default |  |  | QPSK | Edge 1RB right (A9) |  |  |
| 44 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 45 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |
| 46 | 1900 | 40 | Default |  |  | QPSK | 191@25 (A2) | 93@13 (A2) | 44@7 (A2) |
| 47 | 1900 | 40 | Default |  |  | QPSK | 115@101 (A6) | 55@51 (A6) | 25@26 (A6) |
| 48 | 1900 | 40 | Default |  |  | QPSK | 39@177 (A5) | 17@89 (A5) | 6@45 (A5) |
| 49 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |
| 50 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |
| 51 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 52 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |
| 53 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |
| 54 | 1900 | 30 | Default |  |  | 256 QAM | 20@0 (A9) | 10@0 (A9) | 5@0 (A9) |
| 55 | 1900 | 35 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 56 | 1900 | 35 | Default |  |  | 256 QAM | 50@10 (A3) | 25@5 (A3) | 12@3 (A3) |
| 57 | 1900 | 35 | Default |  |  | 256 QAM | 178@10 (A2) | 87@5 (A2) | 41@3 (A2) |
| 58 | 1900 | 35 | Default |  |  | 256 QAM | 111@77 (A6) | 53@39 (A6) | 24@20 (A6) |
| 59 | 1900 | 35 | Default |  |  | 256 QAM | Edge 1RB right (A9) |  |  |
| 60 | Low | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 61 | Low | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) | 15@7 (A3) |
| 62 | Low | 40 | Default |  |  | 256 QAM | 191@25 (A2) | 93@13 (A2) | 44@7 (A2) |
| 63 | Low | 40 | Default |  |  | 256 QAM | 115@101 (A6) | 55@51 (A6) | 25@26 (A6) |
| 64 | Low | 40 | Default |  |  | 256 QAM | 39@177 (A5) | 17@89 (A5) | 6@45 (A5) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-30.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.3.4.3.

6.2.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-31. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD, only slots consisting of only UL symbols are under test.

4.	For network signalling value “NS_04” and UEs supporting Power Class 2, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2.3.4.3.4-2.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2.3.4.1-1 to Table 6.2.3.4.1-20, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2.3.4.3	Message contents

6.2.3.4.3.0	General

Message contents are according to TS 38.508-1 [5] subclause 4.6.1, with the following exceptions for each network signalling value.

For almost contiguous allocation testing, message contents are according to TS 38.508-1 [5] subclause 4.6.1 with the following exceptions:

Table 6.2.3.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  |  |
| } |  |  |  |

6.2.3.4.3.1	Message contents exceptions for network signalling value “NS_03”

1.	Information element additionalSpectrumEmission is set to NS_03. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n2, n25, n66 and n86

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_03) |  |  |

Table 6.2.3.4.3.1-2: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n70

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_03) |  |  |

6.2.3.4.3.2	Message contents exceptions for network signalling value "NS_35"

1.	Information element additionalSpectrumEmission is set to NS_35. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_35" and NR band n71

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_35) |  |  |

6.2.3.4.3.3	Message contents exceptions for network signalling value "NS_03U"

1.	Information element additionalSpectrumEmission is set to NS_03U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03U) |  |  |

6.2.3.4.3.4	Message contents exceptions for network signalling value "NS_04"

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.4-1: AdditionalSpectrumEmission Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

Table 6.2.3.4.3.4-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE |

Table 6.2.3.4.3.4-3: Void

6.2.3.4.3.5	Message contents exceptions for network signalling value "NS_05"

1.	Information element additionalSpectrumEmission is set to NS_05. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_05) |  |  |

6.2.3.4.3.6	Message contents exceptions for network signalling value "NS_05U"

1.	Information element additionalSpectrumEmission is set to NS_05U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.6-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_05U) |  |  |

6.2.3.4.3.7	Message contents exceptions for network signalling value "NS_06"

1.	Information element additionalSpectrumEmission is set to NS_06. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.7-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_06"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_06) |  |  |

6.2.3.4.3.8	Message contents exceptions for network signalling value "NS_08"

1.	Information element additionalSpectrumEmission is set to NS_08. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.8-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_08"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_08) |  |  |

6.2.3.4.3.9	Message contents exceptions for network signalling value "NS_08U"

1.	Information element additionalSpectrumEmission is set to NS_08U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.9-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_08U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_08U) |  |  |

6.2.3.4.3.10	Message contents exceptions for network signalling value "NS_10"

1.	Information element additionalSpectrumEmission is set to NS_10. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.10-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_10"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_10) |  |  |

6.2.3.4.3.11	Message contents exceptions for network signalling value "NS_17"

1.	Information element additionalSpectrumEmission is set to NS_17. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.11-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_17"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_17) |  |  |

6.2.3.4.3.12	Message contents exceptions for network signalling value "NS_18"

1.	Information element additionalSpectrumEmission is set to NS_18. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.12-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_18"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_18) |  |  |

6.2.3.4.3.13	Message contents exceptions for network signalling value "NS_37"

1.	Information element additionalSpectrumEmission is set to NS_37. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.13-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_37"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_37) |  |  |

6.2.3.4.3.14	Message contents exceptions for network signalling value "NS_38"

1.	Information element additionalSpectrumEmission is set to NS_38. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.14-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_38"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_38) |  |  |

6.2.3.4.3.15	Message contents exceptions for network signalling value "NS_39"

1.	Information element additionalSpectrumEmission is set to NS_39. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.15-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_39"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_39) |  |  |

6.2.3.4.3.16	Message contents exceptions for network signalling value "NS_40"

1.	Information element additionalSpectrumEmission is set to NS_40. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.16-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_40"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_40) |  |  |

6.2.3.4.3.17	Message contents exceptions for network signalling value "NS_41"

1.	Information element additionalSpectrumEmission is set to NS_41. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.17-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_41"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_41) |  |  |

6.2.3.4.3.18	Message contents exceptions for network signalling value "NS_42"

1.	Information element additionalSpectrumEmission is set to NS_42. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.18-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_42"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_42) |  |  |

6.2.3.4.3.19	Message contents exceptions for network signalling value "NS_100"

1.	Information element additionalSpectrumEmission is set to NS_100. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.19-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_100) |  | not for band n65 |
|  | 2 (NS_100) |  | for band n65 |

6.2.3.4.3.20	Message contents exceptions for network signalling value "NS_21"

1.	Information element additionalSpectrumEmission is set to NS_21. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.20-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_21"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_21) |  |  |

6.2.3.4.3.21	Message contents exceptions for network signalling value "NS_24"

1.	Information element additionalSpectrumEmission is set to NS_24. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.21-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_24"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_24) |  |  |

6.2.3.4.3.22	Message contents exceptions for network signalling value "NS_27"

1.	Information element additionalSpectrumEmission is set to NS_27. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.22-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_27"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_27) |  |  |

6.2.3.4.3.23	Message contents exceptions for network signalling value "NS_43"

1.	Information element additionalSpectrumEmission is set to NS_43. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.23-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_43) |  |  |

6.2.3.4.3.24	Message contents exceptions for network signalling value "NS_47"

1.	Information element additionalSpectrumEmission is set to NS_47. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.24-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_47"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_47) |  |  |

6.2.3.4.3.25	Message contents exceptions for network signalling value "NS_48"

1.	Information element additionalSpectrumEmission is set to NS_48. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.25-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_48"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_48) |  |  |

6.2.3.4.3.26	Message contents exceptions for network signalling value "NS_49"

1.	Information element additionalSpectrumEmission is set to NS_49. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.26-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_49"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_49) |  |  |

6.2.3.4.3.27	Message contents exceptions for network signalling value "NS_12"

1.	Information element additionalSpectrumEmission is set to NS_12. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.27-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_12"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_12) |  |  |

6.2.3.4.3.28	Message contents exceptions for network signalling value "NS_13"

1.	Information element additionalSpectrumEmission is set to NS_13. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.28-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_13"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_13) |  |  |

6.2.3.4.3.29	Message contents exceptions for network signalling value "NS_14"

1.	Information element additionalSpectrumEmission is set to NS_14. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.29-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_14"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_14) |  |  |

6.2.3.4.3.30	Message contents exceptions for network signalling value "NS_15"

1.	Information element additionalSpectrumEmission is set to NS_15. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.30-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_15"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_15) |  |  |

6.2.3.4.3.31	Message contents exceptions for network signalling value "NS_45"

1.	Information element additionalSpectrumEmission is set to NS_45. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

6.2.3.4.3.32	Message contents exceptions for network signalling value "NS_46"

1.	Information element additionalSpectrumEmission is set to NS_46. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.32-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_46"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_46) |  |  |

6.2.3.4.3.33	Message contents exceptions for network signalling value "NS_44"

1.	Information element additionalSpectrumEmission is set to NS_44. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.33-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_44"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_44) |  |  |

6.2.3.4.3.34	Message contents exceptions for network signalled value "NS_56"

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.34-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_56) |  |  |

6.2.3.4.3.35	Message contents exceptions for network signalled value "NS_07"

1.	Information element additionalSpectrumEmission is set to NS_07. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.35-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_07"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_07) |  |  |

6.2.3.4.3.36	Message contents exceptions for network signalled value "NS_50"

1.	Information element additionalSpectrumEmission is set to NS_50. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.36-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_50"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_50) |  |  |

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable Table from table 6.2.3.5-1 to Table 6.2.3.5-38. The allowed A-MPR values specified in Table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.1.3-1 apply.

Table 6.2.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.3.5-1: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 2 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 3 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 4 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 5 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 6 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 7 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 8 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 9 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 10 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 11 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 13 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 14 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 16 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 17 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 18 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 19 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 20 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 21 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 23 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 24 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 25 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 26 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

r

Table 6.2.3.5-2: UE Power Class 2 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 26 | 3.5 | 5.5 | 1.5 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 8 | 26 | 3.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 9 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 10 | 26 | 0.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 11 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 12 | 26 | 0.5 | 0 | 0 | 25.5 | 2.0 | 3 | 28+TT | 22.5-TT |
| 13 | 26 | 3.5 | 6 | 1.5 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 14 | 26 | 3.5 | 4.5 | 0 | 21 | 2.0 | 3 | 28+TT | 18.5-TT |
| 15 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 16 | 26 | 1 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 17 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 18 | 26 | 1 | 0 | 0 | 25 | 2.0 | 3 | 28+TT | 22-TT |
| 19 | 26 | 3.5 | 6 | 1.5 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 20 | 26 | 3.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 21 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 22 | 26 | 2 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 23 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 24 | 26 | 2 | 0 | 0 | 24 | 2.0 | 3 | 28+TT | 21-TT |
| 25 | 26 | 3.5 | 6.5 | 1.5 | 18 | 4.0 | 3 | 28+TT | 14-TT |
| 26 | 26 | 3.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 27 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 28 | 26 | 2.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 29 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 30 | 26 | 2.5 | 0 | 0 | 23.5 | 2.0 | 3 | 28+TT | 20.5-TT |
| 31 | 26 | 4.5 | 8 | 1.5 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 32 | 26 | 4.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 33 | 26 | 4.5 | 0 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 34 | 26 | 4.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 35 | 26 | 4.5 | 0 | 1.5 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 36 | 26 | 4.5 | 0 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 37 | 26 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 38 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 39 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 40 | 26 | 3 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 41 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 42 | 26 | 3 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 43 | 26 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 44 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 45 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 46 | 26 | 3 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 47 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 48 | 26 | 3 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 49 | 26 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 50 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 51 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 52 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 53 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 54 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 55 | 26 | 6.5 | 10 | 1.5 | 14.5 | 5.0 | 3 | 28+TT | 9.5-TT |
| 56 | 26 | 6.5 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 57 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 58 | 26 | 6.5 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 59 | 26 | 6.5 | 0 | 1.5 | 18 | 4.0 | 3 | 28+TT | 14-TT |
| 60 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Void NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3: UE Power Class 3 test requirements (NS_04) for band n41 (Step 3) and UE Power Class 2 test requirements (NS_04) for band n41 (Step 4)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 5.5 | 1.5 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 2 | 23 | -3 | 3.5 | 3.5 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 3 | 23 | -3 | 3.5 | 0 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 4 | 23 | -3 | 1.2 | 3.5 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 5 | 23 | -3 | 3.5 | 0 | 1.5 | 21 | 2.0 | 2 | 28+TT | 19-TT |
| 6 | 23 | -3 | 1.2 | 0 | 0 | 24.8 | 2.0 | 2 | 28+TT | 22.8-TT |
| 7 | 23 | 0 | 0.5 | 3.5 | 1.5 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 8 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 9 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2.0 | 2 | 25+TT | 20.5-TT |
| 10 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 11 | 23 | 0 | 0.5 | 0 | 1.5 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 12 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 2.0 | 2 | 25+TT | 20.5-TT |
| 13 | 23 | 0 | 1 | 4 | 1.5 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 14 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | 0 | 1 | 0 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 16 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 17 | 23 | 0 | 1 | 0 | 1.5 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 18 | 23 | 0 | 1 | 0 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 19 | 23 | 0 | 2 | 4 | 1.5 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 0 | 2 | 0 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 22 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 0 | 2 | 0 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 24 | 23 | 0 | 2 | 0 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 25 | 23 | 0 | 2.5 | 4.5 | 1.5 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 26 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 28 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 29 | 23 | 0 | 2.5 | 0 | 1.5 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 31 | 23 | 0 | 4.5 | 6 | 1.5 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 32 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 33 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 35 | 23 | 0 | 4.5 | 0 | 1.5 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 36 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 3 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 38 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 40 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 41 | 23 | 0 | 3 | 0 | 1.5 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 42 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 43 | 23 | 0 | 3 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 44 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 45 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 46 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 47 | 23 | 0 | 3 | 0 | 1.5 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 48 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 49 | 23 | 0 | 3.5 | 5.5 | 1.5 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 50 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 51 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 52 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 53 | 23 | 0 | 3.5 | 0 | 1.5 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 55 | 23 | 0 | 6.5 | 8 | 1.5 | 13.5 | 5.0 | 2 | 25+TT | 8.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 57 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 58 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 59 | 23 | 0 | 6.5 | 0 | 1.5 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 60 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Void NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3a: UE Power Class 1 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) |  | A-MPR (dB) |  |  |  | ΔTC,c (dB) | PCMAX,c (dBm) |  | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | <50 MHz CBW | ≥50 MHz CBW | regrowth | IMD3 | CIM3 | edge |  | <50 MHz CBW | ≥50 MHz CBW |  |  |  | <50 MHz CBW | ≥50 MHz CBW |
| 1 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 11 | 0 | 20 | 20 | 2.5 | 3 | 33+TT | 17-TT | 17-TT |
| 2 | 31 | 0.5 | 0.5 | 0 | 2 | 0 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 3 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 4 | 31 | 0.5 | 0.5 | 0 | 0 | 0 | 0 | 0 | 32.5 | 32.5 | 2.5 | 3 | 33+TT | 29.5-TT | 29.5-TT |
| 5 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2 | 3 | 33+TT | 24-TT | 24-TT |
| 6 | 31 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 7 | 31 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 8 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 2 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 9 | 31 | 1 | 1 | 0 | 2 | 0 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 10 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21-4-TT | 19.5-TT |
| 11 | 31 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 30 | 30 | 2.5 | 3 | 33+TT | 27-TT | 27-TT |
| 12 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2 | 3 | 33+TT | 24-TT | 24-TT |
| 13 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 14 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 15 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 2.5 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 16 | 31 | 2 | 2 | 0 | 2.5 | 0 | 0 | 0 | 28.5 | 28.5 | 2.5 | 3 | 33+TT | 25.5-TT | 25.5-TT |
| 17 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 2 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 18 | 31 | 2 | 2 | 0 | 0 | 2 | 0 | 0 | 29 | 29 | 2.5 | 3 | 33+TT | 26-TT | 26-TT |
| 19 | 31 | 1 | 1 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2 | 3 | 33+TT | 24-TT | 24-TT |
| 20 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 21 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 22 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 3 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 23 | 31 | 2.5 | 2.5 | 0 | 3 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 24 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 3 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 25 | 31 | 2.5 | 2.5 | 0 | 0 | 3 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 26 | 31 | 2.5 | 2.5 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2 | 3 | 33+TT | 24-TT | 24-TT |
| 27 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 28 | 31 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 29 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 30 | 31 | 4.5 | 4.5 | 0 | 0 | 0 | 0 | 0 | 26.5 | 26.5 | 2.5 | 3 | 33+TT | 23.5-TT | 23.5-TT |
| 31 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 32 | 31 | 4.5 | 4.5 | 0 | 0 | 0 | 0 | 0 | 26.5 | 26.5 | 2.5 | 3 | 33+TT | 23.5-TT | 23.5-TT |
| 33 | 31 | 4.5 | 4.5 | 0 | 0 | 0 | 0 | 0 | 26.5 | 26.5 | 2 | 3 | 33+TT | 23.5-TT | 23.5-TT |
| 34 | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 31 | 31 | 2 | 3 | 33+TT | 28-TT | 28-TT |
| 35 | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 31 | 31 | 2 | 3 | 33+TT | 28-TT | 28-TT |
| 36 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 11 | 0 | 20 | 20 | 2.5 | 3 | 33+TT | 17-TT | 17-TT |
| 37 | 31 | 3 | 3 | 0 | 2.5 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 38 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 10.5 | 0 | 0 | 20.5 | 20.5 | 2.5 | 3 | 33+TT | 17.5-TT | 17.5-TT |
| 39 | 31 | 3 | 3 | 6 | 0 | 0 | 0 | 0 | 25 | 25 | 2 | 3 | 33+TT | 22-TT | 22-TT |
| 40 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 41 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 42 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 3 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 43 | 31 | 3 | 3 | 0 | 3 | 0 | 0 | 0 | 28 | 28 | 2.5 | 3 | 33+TT | 25-TT | 25-TT |
| 44 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 10.5 | 0 | 0 | 20.5 | 20.5 | 2.5 | 3 | 33+TT | 17.5-TT | 17.5-TT |
| 45 | 31 | 3 | 3 | 6 | 0 | 0 | 0 | 0 | 25 | 25 | 2 | 3 | 33+TT | 22-TT | 22-TT |
| 46 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 47 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 48 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 49 | 31 | 3.5 | 3.5 | 0 | 0 | 0 | 0 | 0 | 27.5 | 27.5 | 2.5 | 3 | 33+TT | 24.5-TT | 24.5-TT |
| 50 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 10.5 | 0 | 0 | 20.5 | 20.5 | 2.5 | 3 | 33+TT | 17.5-TT | 17.5-TT |
| 51 | 31 | 3.5 | 3.5 | 6 | 0 | 0 | 0 | 0 | 25 | 25 | 2 | 3 | 33+TT | 22-TT | 22-TT |
| 52 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 53 | 31 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 27 | 27 | 2.5 | 3 | 33+TT | 24-TT | 24-TT |
| 54 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 0 | 0 | 0 | 24.4 | 22.5 | 2.5 | 3 | 33+TT | 21.4-TT | 19.5-TT |
| 55 | 31 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 0 | 24.5 | 24.5 | 2.5 | 3 | 33+TT | 21.5-TT | 21.5-TT |
| 56 | 31 | 7.2-6xCBW/100 | 5.35+3.15xCBW/100 | 0 | 0 | 11 | 0 | 0 | 20 | 20 | 2.5 | 3 | 33+TT | 17-TT | 17-TT |
| 57 | 31 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 0 | 24.5 | 24.5 | 2 | 3 | 33+TT | 21.5-TT | 21.5-TT |
| 58 | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 31 | 31 | 2 | 3 | 33+TT | 28-TT | 28-TT |
| 59 | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 31 | 31 | 2 | 3 | 33+TT | 28-TT | 28-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-4: UE Power Class 3 test requirements (NS_03 and NS_03U) for band n66, n70

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | NS_03, NS_03U | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | NS_03, NS_03U | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | NS_03, NS_03U | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | NS_03, NS_03U | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10, 11 | NS_03, NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 12 | NS_03, NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 13, 14 | NS_03, NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 15 | NS_03, NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 16, 17 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25, 26 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 27 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-5: UE Power Class 3 test requirements (NS_03 and NS_03U) for band n2, n25

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 1.5 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | NS_03; NS_03U | 23 | 1 | 2 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 6 | NS_03; NS_03U | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | NS_03; NS_03U | 23 | 2 | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 9 | NS_03; NS_03U | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10, 11 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 1.5 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 13, 14 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 1.5 | 16 | 5 | 2 | 25+TT | 11-TT |
| 15 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 16, 17 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 18 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 21 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 1.5 | 17 | 5 | 2 | 25+TT | 12-TT |
| 24 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25, 26 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 1.5 | 14 | 5 | 2 | 25+TT | 9-TT |
| 27 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-6: UE Power Class 3 test requirements (NS_05) for bands n1, n65

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 2 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 6 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 12 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 0.5 | 1 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 16 | 23 | 0 | 1.0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 17 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 22 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 23 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 24 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 25 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 26 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 27 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 28 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 29 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 30 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 31 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 32 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 33 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 34 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 35 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 36 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 37 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 38 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 40 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 41 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 42 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 43 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 44 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 45 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 46 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 47 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 48 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 49 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 50 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 51 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 52 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 53 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 55 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 56 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 57 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 58 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 59 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 61 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 62 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 63 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 64 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 65 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 66 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 67 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 68 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 69 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 70 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 71 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 72 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 73 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 74 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 75 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 76 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 77 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 78 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 79 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 80 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 81 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 82 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 83 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 84 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 85 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 86 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 87 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 88 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 89 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 90 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 91 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 92 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 93 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 94 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 95 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 96 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 97 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 98 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 99 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 100 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 101 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 102 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 103 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 104 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 105 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 106 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 107 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 108 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 109 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 110 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 111 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 112 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 113 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 114 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 115 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-6a: UE Power Class 2 test requirements (NS_05 and NS_05U) for bands n1, n65

| Test ID | PPowerClass (dBm) | DPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 2 | 26 | 0 | 0 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 3 | 26 | 0 | 0.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 4 | 26 | 0 | 0.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 5 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 6 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 7 | 26 | 0 | 0.5 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 8 | 26 | 0 | 0.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 9 | 26 | 0 | 0.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 10 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 11 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 12 | 26 | 0 | 0.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 13 | 26 | 0 | 0.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 14 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 15 | 26 | 0 | 0.5 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 16 | 26 | 0 | 0.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 17 | 26 | 0 | 0.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 18 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 19 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 20 | 26 | 0 | 0.5 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 21 | 26 | 0 | 1 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 22 | 26 | 0 | 0 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 23 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 24 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 25 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 26 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 27 | 26 | 0 | 1 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 28 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 29 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 30 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 31 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 32 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 33 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 34 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 35 | 26 | 0 | 1 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 36 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 37 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 38 | 26 | 0 | 0 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 39 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 40 | 26 | 0 | 1 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 41 | 26 | 0 | 2 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 42 | 26 | 0 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 43 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 44 | 26 | 0 | 2 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 45 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 46 | 26 | 0 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 47 | 26 | 0 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 48 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 49 | 26 | 0 | 2 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 50 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 51 | 26 | 0 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 52 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 53 | 26 | 0 | 2 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 54 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 55 | 26 | 0 | 2 | 2.5 | 0 | 23.5 | 2 | 3 | 28+TT | 20.5-TT |
| 56 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 57 | 26 | 0 | 2 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 58 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 59 | 26 | 0 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |

| Test ID | PPowerClass (dBm) | DPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 26 | 0 | 2 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 61 | 26 | 0 | 2.5 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 62 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 63 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 64 | 26 | 0 | 2.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 65 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 66 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 67 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 68 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 69 | 26 | 0 | 2.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 70 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 71 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 72 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 73 | 26 | 0 | 2.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 74 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 75 | 26 | 0 | 2.5 | 2.5 | 0 | 23.5 | 2 | 3 | 28+TT | 20.5-TT |
| 76 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 77 | 26 | 0 | 2.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 78 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 79 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 80 | 26 | 0 | 4.5 | 9.5 | 0 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 81 | 26 | 0 | 4.5 | 15 | 0 | 11 | 6 | 3 | 28+TT | 5-TT |
| 82 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 83 | 26 | 0 | 4.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 84 | 26 | 0 | 4.5 | 15 | 0 | 11 | 6 | 3 | 28+TT | 5-TT |
| 85 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 86 | 26 | 0 | 4.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 87 | 26 | 0 | 4.5 | 15 | 0 | 11 | 6 | 3 | 28+TT | 5-TT |
| 88 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 89 | 26 | 0 | 4.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 90 | 26 | 0 | 4.5 | 15 | 0 | 11 | 6 | 3 | 28+TT | 5-TT |
| 91 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 92 | 26 | 0 | 4.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 93 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 94 | 26 | 0 | 1.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 95 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 96 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 97 | 26 | 0 | 1.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 98 | 26 | 0 | 1.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 99 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 100 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 101 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 102 | 26 | 0 | 1.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 103 | 26 | 0 | 1.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 104 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 105 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 106 | 26 | 0 | 1.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 107 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 108 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 109 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 110 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 111 | 26 | 0 | 3 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 112 | 26 | 0 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 113 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 114 | 26 | 0 | 2 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 115 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 116 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 117 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 118 | 26 | 0 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 119 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 120 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 121 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 122 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| Test ID | PPowerClass (dBm) | DPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 123 | 26 | 0 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 124 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 125 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 126 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 127 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 128 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 129 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 130 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 131 | 26 | 0 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 132 | 26 | 0 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 133 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 134 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 135 | 26 | 0 | 3.5 | 14 | 0 | 12 | 6 | 3 | 28+TT | 6-TT |
| 136 | 26 | 0 | 3.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 137 | 26 | 0 | 3.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 138 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 139 | 26 | 0 | 3.5 | 14 | 0 | 12 | 6 | 3 | 28+TT | 6-TT |
| 140 | 26 | 0 | 3.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 141 | 26 | 0 | 3.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 142 | 26 | 0 | 3.5 | 14 | 0 | 12 | 6 | 3 | 28+TT | 6-TT |
| 143 | 26 | 0 | 3.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 144 | 26 | 0 | 3.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 145 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 146 | 26 | 0 | 3.5 | 14 | 0 | 12 | 6 | 3 | 28+TT | 6-TT |
| 147 | 26 | 0 | 3.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 148 | 26 | 0 | 3.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 149 | 26 | 0 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 150 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 151 | 26 | 0 | 6.5 | 16 | 0 | 10 | 6 | 3 | 28+TT | 4-TT |
| 152 | 26 | 0 | 6.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 153 | 26 | 0 | 6.5 | 16 | 0 | 10 | 6 | 3 | 28+TT | 4-TT |
| 154 | 26 | 0 | 6.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 155 | 26 | 0 | 6.5 | 16 | 0 | 10 | 6 | 3 | 28+TT | 4-TT |
| 156 | 26 | 0 | 6.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 157 | 26 | 0 | 6.5 | 16 | 0 | 10 | 6 | 3 | 28+TT | 4-TT |
| 158 | 26 | 0 | 6.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-7: UE Power Class 3 test requirements (NS_05U) for bands n1, n65

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 2 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 3 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 6 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 9 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 10 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 11 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 12 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 16 | 23 | 0 | 1.0 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 17 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 21 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 26 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 27 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 28 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 1.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 31 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 32 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 33 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 36 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 40 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 41 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 42 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 44 | 23 | 0 | 2.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 46 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 47 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 48 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 49 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 50 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 51 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 52 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 53 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 55 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 56 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 57 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 58 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 59 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 61 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 62 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 63 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 64 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 65 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 66 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 67 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 68 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 69 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 70 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 71 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 72 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 73 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 74 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 75 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 76 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 77 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 78 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 79 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 80 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 81 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 82 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 83 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 84 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 85 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 86 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 87 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 88 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 89 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 90 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 91 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 92 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 93 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 94 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 95 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 96 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 97 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 98 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 99 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 100 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 101 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 102 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 103 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 104 | 23 | 0 | 3.0 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 105 | 23 | 0 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 106 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 107 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 108 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 109 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 110 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 111 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 112 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 113 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 114 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 115 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 116 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 117 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 118 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 119 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 121 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 122 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 123 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 124 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 125 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 126 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 127 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 128 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 129 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 130 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 131 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-8: UE Power Class 3 test requirements (NS_18)

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 3, 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
|  | 10, 15, 20 | 23 | 1 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 3, 4 | 3, 5 | 23 | 2 | 3 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
|  | 10, 15, 20 | 23 | 2 | 6 | 0 | 17 | 5 | 2.5 | 25+TT | 12-TT |
| 5, 6 | 3, 5 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 2.5 | 25+TT | 15.5-TT |
|  | 10, 15, 20 | 23 | 2.5 | 7 | 0 | 16 | 5 | 2.5 | 25+TT | 11-TT |
| 7, 8 | 3, 5 | 23 | 4.5 | 6 | 0 | 17 | 5 | 2.5 | 25+TT | 12-TT |
|  | 10, 15, 20 | 23 | 4.5 | 9 | 0 | 14 | 5 | 2.5 | 25+TT | 9-TT |
| 9, 10 | 3, 5 | 23 | 3 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
|  | 10, 15, 20 | 23 | 3 | 6.5 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 11, 12 | 3, 5 | 23 | 3 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
|  | 10, 15, 20 | 23 | 3 | 7 | 0 | 16 | 5 | 2.5 | 25+TT | 11-TT |
| 13, 14 | 3, 5 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
|  | 10, 15, 20 | 23 | 3.5 | 8.5 | 0 | 14.5 | 5 | 2.5 | 25+TT | 9.5-TT |
| 15, 16 | 3, 5 | 23 | 6.5 | 8.5 | 0 | 14.5 | 5 | 2.5 | 25+TT | 9.5-TT |
|  | 10, 15, 20 | 23 | 6.5 | 11.5 | 0 | 11.5 | 6 | 2.5 | 25+TT | 5.5-TT |
| 17, 18, 19 | 30 | 23 | 5 | 3 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 20 | 30 | 23 | 5 | 8 | 0 | 15 | 5 | 2.5 | 25+TT | 10–TT |
| 21 | 30 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 30 | 23 | 3 | 4.5 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 23 | 30 | 23 | 5 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
| 24, 25, 26 | 30 | 23 | 7 | 4.5 | 0 | 16 | 5 | 2.5 | 25+TT | 11–TT |
| 27 | 30 | 23 | 7 | 9.5 | 0 | 13.5 | 5 | 2.5 | 25+TT | 8.5–TT |
| 28 | 30 | 23 | 3.5 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 29 | 30 | 23 | 4 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
| 30 | 30 | 23 | 7 | 7.5 | 0 | 15.5 | 5 | 2.5 | 25+TT | 10.5-TT |

Table 6.2.3.5-10: UE Power Class 3 test requirements (NS_43)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 2 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 3 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 4 | 23 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 5 | 23 | 1.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 6 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 7 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 8 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 10 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 11 | 23 | 2.5 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 13 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 14 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 15 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 16 | 23 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 17 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 19 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 20 | 23 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 21 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 23 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 26 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 27 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 28 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-10a: UE Power Class 2 test requirements (NS_43)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0.5 | 1 | 0 | 25 | 2 | 3 | 28+TT | 22-TT |
| 2 | 26 | 0.5 | 1.5 | 0 | 24.5 | 2 | 3 | 28+TT | 21.5-TT |
| 3 | 26 | 0.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 4 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 5 | 26 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 6 | 26 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 7 | 26 | 1 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 8 | 26 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 9 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 10 | 26 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 11 | 26 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 12 | 26 | 2 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 13 | 26 | 2 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 14 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 15 | 26 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 16 | 26 | 2.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 17 | 26 | 2.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 18 | 26 | 2.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 19 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 20 | 26 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 21 | 26 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 22 | 26 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 23 | 26 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 24 | 26 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 25 | 26 | 3 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 26 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 27 | 26 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 28 | 26 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 29 | 26 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 30 | 26 | 3 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 31 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 32 | 26 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 33 | 26 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 34 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 35 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 36 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 37 | 26 | 6.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 38 | 26 | 6.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-11: UE Power Class 3 test requirements (NS_43U)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 2 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 4 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 5 | 23 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 7 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 8 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 9 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 11 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 12 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 13 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 14 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 15 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 16 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 17 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 18 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 19 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 20 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 21 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 24 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 25 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 26 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 28 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 29 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 32 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 33 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 34 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 35 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 36 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-12: UE Power Class 3 test requirements (NS_100) for band n1, n5, n18, n65, n66

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13, 14 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 16, 17 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25, 26 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-13: UE Power Class 3 test requirements (NS_100) for band n2, n3, n8, n25 and n26

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 2 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 3 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | 23 | 1 | 2 | 1.5 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 6 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 1.5 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13, 14 | 23 | 4.5 | 4.5 | 1.5 | 17 | 5 | 2 | 25+TT | 12-TT |
| 15 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 16, 17 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 18 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 21 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 24 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 25, 26 | 23 | 6.5 | 6.5 | 1.5 | 15 | 5 | 2 | 25+TT | 10-TT |
| 27 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-13a: UE Power Class 2 test requirements (NS_100) for band n8

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 26 | 3.5 | 2 | 1.5 | 21 | 2 | 3 | 28+TT | 18 |
| 3 | 26 | 0.5 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21 |
| 4, 5 | 26 | 3.5 | 2 | 1.5 | 21 | 2 | 3 | 28+TT | 18 |
| 6 | 26 | 1 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21 |
| 7, 8 | 26 | 3.5 | 2.5 | 1.5 | 21 | 2 | 3 | 28+TT | 18 |
| 9 | 26 | 2 | 2.5 | 0 | 23.5 | 2 | 3 | 28+TT | 20.5 |
| 10, 11 | 26 | 3.5 | 3 | 1.5 | 21 | 2 | 3 | 28+TT | 18 |
| 12 | 26 | 2.5 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20 |
| 13, 14 | 26 | 4.5 | 4.5 | 1.5 | 20 | 2.5 | 3 | 28+TT | 17 |
| 15 | 26 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5 |
| 16, 17 | 26 | 3.5 | 4 | 1.5 | 20.5 | 2.5 | 3 | 28+TT | 17.5 |
| 18 | 26 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19 |
| 19, 20 | 26 | 3.5 | 4 | 1.5 | 20.5 | 2.5 | 3 | 28+TT | 17.5 |
| 21 | 26 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19 |
| 22, 23 | 26 | 3.5 | 4 | 1.5 | 20.5 | 2.5 | 3 | 28+TT | 17.5 |
| 24 | 26 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19 |
| 25, 26 | 26 | 6.5 | 6.5 | 1.5 | 18 | 4 | 3 | 28+TT | 14 |
| 27 | 26 | 6.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-14: UE Power Class 3 test requirements (NS_37)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 1.0 | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 2 | 23 | 0 | 0.5 | 1.0 | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 3 | 23 | 0 | 0.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 0.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 5 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 6 | 23 | 0 | 1.0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 7 | 23 | 0 | 1.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 8 | 23 | 0 | 1.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10 | 23 | 0 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 11 | 23 | 0 | 2.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 0 | 2.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13 | 23 | 0 | 2.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 14 | 23 | 0 | 2.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 15 | 23 | 0 | 2.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 16 | 23 | 0 | 2.5 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 17 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 18 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 19 | 23 | 0 | 3.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 20 | 23 | 0 | 3.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 21 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 22 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 23 | 23 | 0 | 3.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 24 | 23 | 0 | 3.0 | 3.0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-15: UE Power Class 3 test requirements (NS_38)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 2 | 23 | 0 | 0.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 3 | 23 | 0 | 0.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 4 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 5 | 23 | 0 | 0.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 6 | 23 | 0 | 0.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 7 | 23 | 0 | 1.0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 8 | 23 | 0 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 9 | 23 | 0 | 1.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 10 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 1.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 12 | 23 | 0 | 1.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 13 | 23 | 0 | 2.0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 14 | 23 | 0 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 15 | 23 | 0 | 2.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 16 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 17 | 23 | 0 | 2.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 18 | 23 | 0 | 2.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 19 | 23 | 0 | 2.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 20 | 23 | 0 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 21 | 23 | 0 | 2.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 22 | 23 | 0 | 2.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 23 | 23 | 0 | 2.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 24 | 23 | 0 | 2.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 25 | 23 | 0 | 4.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 26 | 23 | 0 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 27 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 28 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 29 | 23 | 0 | 4.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 30 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 31 | 23 | 0 | 3.0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 32 | 23 | 0 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 33 | 23 | 0 | 3.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 34 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 35 | 23 | 0 | 3.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 36 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 3.0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 38 | 23 | 0 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 39 | 23 | 0 | 3.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 40 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 41 | 23 | 0 | 3.0 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 42 | 23 | 0 | 3.0 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 44 | 23 | 0 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 45 | 23 | 0 | 3.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46 | 23 | 0 | 3.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 47 | 23 | 0 | 3.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 48 | 23 | 0 | 3.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 49 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 50 | 23 | 0 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 51 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 52 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 53 | 23 | 0 | 6.5 | 13 | 0 | 10 | 6.0 | 2 | 25+TT | 4-TT |
| 54 | 23 | 0 | 6.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-16: UE Power Class 3 test requirements (NS_39)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 2 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 3 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 5 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 6 | 23 | 0 | 1.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 7 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 2.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 10 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 11 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 12 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 13 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 15 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 16 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 17 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 20 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 21 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 22 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 24 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 25 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 26 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-17: UE Power Class 3 test requirements (NS_24) for n65

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-3 | 23 | N/A | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 4-6 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 7-15 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 16-TT |
| 16-18 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 19-21 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 22-30 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31-39 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 40-42 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
| 43-51 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 52-60 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 61-63 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
| 64-72 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 73-75 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
| 76-84 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 85-87 | 23 | N/A | 18 | 0 | 5 | 7 | 2 | 25+TT | -2-TT |
| 88 | 23 | N/A | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 89 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 90-92 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 16-TT |
| 93 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 94 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 95-97 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 98-100 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 101 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 103-105 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 106-108 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 109 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 110-112 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 113 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 114-116 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 117 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 118 | 23 | 4.5 |  | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 119 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 120-122 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | 4.5 |  | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 123 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 124 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 125-127 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | 4.5 |  | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 128-130 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 131 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 133-135 | 23 | N/A | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
|  | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
|  | 23 | 4.5 |  | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 136-138 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 139 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 140-142 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 143 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 144-146 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 147 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 148-150 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 151-153 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 154-162 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 16-TT |
| 163-165 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 166-168 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 169-177 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 178-186 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 187-189 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 190-198 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 199-207 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 208-210 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 211-219 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 220-222 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 223-231 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 232-234 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 235 | 23 | 6.5 |  | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 236 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 237-239 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | 6.5 |  | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 240 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 241 | 23 | N/A | 8.5 | 0 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 242-244 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | 6.5 |  | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 245-247 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 248 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 250-252 | 23 | N/A | 13 | 0 | 10 | 6 | 2 | 25+TT | 4-TT |
|  | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 23 | 6.5 |  | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 253-255 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 256 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 257-259 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 260 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 261-263 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
|  | 23 | N/A | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 264 | 23 | N/A | 19 | 0 | 4 | 7 | 2 | 25+TT | -3-TT |
| 265 | 23 | 1 |  | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 266 | 23 | 2 |  | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 267 | 23 | 1 |  | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-18: UE Power Class 3 test requirements (NS_27)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 2 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 3 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 4 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 5 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 6 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 7 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 9 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 10 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 11 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 12 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 14 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 15 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 16 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 17 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 18 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 19 | 23 | 0 | 0.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 20 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 21 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 22 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 23 | 23 | 0 | 0.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 25 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 26 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 27 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 28 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 29 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 30 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 31 | 23 | 0 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 32 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 33 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 34 | 23 | 0 | 0.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 35 | 23 | 0 | 0.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 36 | 23 | 0 | 0.5 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 37 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 38 | 23 | 0 | 0.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 39 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 40 | 23 | 0 | 0.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 41 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 42 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 43 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 44 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 45 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 46 | 23 | 0 | 1 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 47 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 48 | 23 | 0 | 1 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 49 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 50 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 51 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 52 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 53 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 54 | 23 | 0 | 1 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 55 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 56 | 23 | 0 | 1 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 57 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 58 | 23 | 0 | 1 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 59 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 60 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 61 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 62 | 23 | 0 | 1 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 63 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 64 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 65 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 66 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 67 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 68 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 69 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 70 | 23 | 0 | 1 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 71 | 23 | 0 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 72 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 73 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 74 | 23 | 0 | 1 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 75 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 76 | 23 | 0 | 1 | 10.5 | 0 | 12.5 | 6.0 | 3 | 25+TT | 6.5-TT |
| 77 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 78 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 79 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 80 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 81 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 82 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 83 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 84 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 85 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 86 | 23 | 0 | 2 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 87 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 88 | 23 | 0 | 2 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 89 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 90 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 91 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 92 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 93 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 94 | 23 | 0 | 2 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 95 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 96 | 23 | 0 | 2 | 2 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 97 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 98 | 23 | 0 | 2 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 99 | 23 | 0 | 2 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 100 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 101 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 102 | 23 | 0 | 2 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 103 | 23 | 0 | 2 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 104 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 105 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 106 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 107 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 108 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 109 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 110 | 23 | 0 | 2 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 111 | 23 | 0 | 1 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 112 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 113 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 114 | 23 | 0 | 2 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 115 | 23 | 0 | 2 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 116 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 117 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 118 | 23 | 0 | 2 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 119 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 120 | 23 | 0 | 2 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 121 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 122 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 123 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 124 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 125 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 126 | 23 | 0 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 127 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 128 | 23 | 0 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 129 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 130 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 131 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 132 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 133 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 134 | 23 | 0 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 135 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 136 | 23 | 0 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 137 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 138 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 139 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 140 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 141 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 142 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 143 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 144 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 145 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 146 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 147 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 148 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 149 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 150 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 151 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 152 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 153 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 154 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 155 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 156 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 157 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 158 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 159 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 160 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 161 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 162 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 163 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 164 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 165 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 166 | 23 | 0 | 4.5 | 2 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 167 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 168 | 23 | 0 | 4.5 | 2 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 169 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 170 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 171 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 172 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 173 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 174 | 23 | 0 | 4.5 | 2 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 175 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 176 | 23 | 0 | 4.5 | 2 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 177 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 178 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 179 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 180 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 181 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 182 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 183 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 184 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 185 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 186 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 187 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 188 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 189 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 190 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 191 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 192 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 193 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 194 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 195 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 196 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 197 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 198 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 199 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 200 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 201 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 202 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 203 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 204 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 205 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 206 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 207 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 208 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 209 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 210 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 211 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 212 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 213 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 214 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 215 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 216 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 217 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 218 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 219 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 220 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 221 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 222 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 223 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 224 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 225 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 226 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 227 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 228 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 229 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 230 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 231 | 23 | 0 | 1.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 232 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 233 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 234 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 235 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 236 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 237 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 238 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 239 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 240 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 241 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 242 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 243 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 244 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 245 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 246 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 247 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 248 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 249 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 250 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 251 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 252 | 23 | 0 | 3 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 253 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 254 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 255 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 256 | 23 | 0 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 257 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 258 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 259 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 260 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 261 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 262 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 263 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 264 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 265 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 266 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 267 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 268 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 269 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 270 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 271 | 23 | 0 | 2 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 272 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 273 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 274 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 275 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 276 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 277 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 278 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 279 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 280 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 281 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 282 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 283 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 284 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 285 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 286 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 287 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 288 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 289 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 290 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 291 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 292 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 293 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 294 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 295 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 296 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 297 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 298 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 299 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 300 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 301 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 302 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 303 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 304 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 305 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 306 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 307 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 308 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 309 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 310 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 311 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 312 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 313 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 314 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 315 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 316 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 317 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 318 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 319 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 320 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 321 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 322 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 323 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 324 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 325 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 326 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 327 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 328 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 329 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 330 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 331 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 332 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 333 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 334 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 335 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 336 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 337 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 338 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 339 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 340 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 341 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 342 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 343 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 344 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 345 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 346 | 23 | 0 | 6.5 | 4 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 347 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 348 | 23 | 0 | 6.5 | 4 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 349 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 350 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 351 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 352 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 353 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 354 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 355 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 356 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6.0 | 3 | 25+TT | 5.5-TT |
| 357 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 358 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 359 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 360 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-19: UE Power Class 3 test requirements (NS_40)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 15.5 | 0 | 7.5 | 7.0 | 2 | 25+TT | 0.5-TT |
| 2 | 23 | 0 | 0 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 3 | 23 | 0 | 1 | 15.5 | 0 | 7.5 | 7.0 | 2 | 25+TT | 0.5-TT |
| 4 | 23 | 0 | 2 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 5 | 23 | 0 | 1 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 6 | 23 | 0 | 2 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 7 | 23 | 0 | 2.5 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 8 | 23 | 0 | 2.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 9 | 23 | 0 | 2.5 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 10 | 23 | 0 | 4.5 | 12.5 | 0 | 10.5 | 6.0 | 2 | 25+TT | 4.5-TT |
| 11 | 23 | 0 | 4.5 | 7.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 12 | 23 | 0 | 4.5 | 12.5 | 0 | 10.5 | 6.0 | 2 | 25+TT | 4.5-TT |
| 13 | 23 | 0 | 3 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 14 | 23 | 0 | 1.5 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 15 | 23 | 0 | 3 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 16 | 23 | 0 | 3 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 17 | 23 | 0 | 2 | 10 | 0 | 13 | 5.0 | 2 | 25+TT | 8-TT |
| 18 | 23 | 0 | 3 | 14.5 | 0 | 8.5 | 6.0 | 2 | 25+TT | 2.5-TT |
| 19 | 23 | 0 | 3.5 | 14 | 0 | 9 | 6.0 | 2 | 25+TT | 3-TT |
| 20 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 21 | 23 | 0 | 3.5 | 14 | 0 | 9 | 6.0 | 2 | 25+TT | 3-TT |
| 22 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 23 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 24 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-20: UE Power Class 3 test requirements (NS_41)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 2 | 23 | 0 | 0.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 3 | 23 | 0 | 0.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 4 | 23 | 0 | 0.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 5 | 23 | 0 | 0.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 6 | 23 | 0 | 0.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 7 | 23 | 0 | 0.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 8 | 23 | 0 | 0.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 9 | 23 | 0 | 1 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 10 | 23 | 0 | 1 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 11 | 23 | 0 | 1 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 12 | 23 | 0 | 1 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 13 | 23 | 0 | 1 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 14 | 23 | 0 | 1 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 15 | 23 | 0 | 1 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 16 | 23 | 0 | 1 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 17 | 23 | 0 | 2 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 18 | 23 | 0 | 2 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 19 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 20 | 23 | 0 | 2 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 21 | 23 | 0 | 2 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 22 | 23 | 0 | 2 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 23 | 23 | 0 | 2 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 24 | 23 | 0 | 2 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 25 | 23 | 0 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 26 | 23 | 0 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 27 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 28 | 23 | 0 | 2.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 29 | 23 | 0 | 2.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 30 | 23 | 0 | 2.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 31 | 23 | 0 | 2.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 32 | 23 | 0 | 2.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 33 | 23 | 0 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 34 | 23 | 0 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 35 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 36 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 37 | 23 | 0 | 4.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 38 | 23 | 0 | 4.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 39 | 23 | 0 | 4.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 40 | 23 | 0 | 4.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 41 | 23 | 0 | 3 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 42 | 23 | 0 | 3 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 43 | 23 | 0 | 3 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 44 | 23 | 0 | 3 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 45 | 23 | 0 | 3 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 46 | 23 | 0 | 3 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 47 | 23 | 0 | 3 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 48 | 23 | 0 | 3 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 49 | 23 | 0 | 3 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 50 | 23 | 0 | 3 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 51 | 23 | 0 | 3 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 52 | 23 | 0 | 3 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 53 | 23 | 0 | 3 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 54 | 23 | 0 | 3 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 55 | 23 | 0 | 3 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 56 | 23 | 0 | 3 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 57 | 23 | 0 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 58 | 23 | 0 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 59 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 61 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 62 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 63 | 23 | 0 | 3.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 64 | 23 | 0 | 3.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 65 | 23 | 0 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 66 | 23 | 0 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 67 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 68 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6.0 | 2 | 25+TT | 6-TT |
| 69 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 70 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6.0 | 2 | 25+TT | 5-TT |
| 71 | 23 | 0 | 6.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| 72 | 23 | 0 | 6.5 | 13.5 | 0 | 9.5 | 6.0 | 2 | 25+TT | 3.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-21: UE Power Class 3 test requirements (NS_42)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 2 | 23 | 0 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 3 | 23 | 0 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 4 | 23 | 0 | 0.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 5 | 23 | 0 | 0.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 7 | 23 | 0 | 1 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 8 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 9 | 23 | 0 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 10 | 23 | 0 | 1 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 11 | 23 | 0 | 1 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 13 | 23 | 0 | 2 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 14 | 23 | 0 | 2 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | 0 | 1 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 16 | 23 | 0 | 2 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 17 | 23 | 0 | 2 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 18 | 23 | 0 | 1 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 19 | 23 | 0 | 2.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 20 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 0 | 2.5 | 1.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 22 | 23 | 0 | 2.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 23 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 2.5 | 1.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 25 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 26 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 4.5 | 1.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 29 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 4.5 | 1.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 31 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 32 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 33 | 23 | 0 | 1.5 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 34 | 23 | 0 | 3 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 35 | 23 | 0 | 3 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 1.5 | 1.5 | 0 | 21.5 | 2.0 | 2 | 25+TT | 19.5-TT |
| 37 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 38 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 39 | 23 | 0 | 2 | 1.5 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 40 | 23 | 0 | 3 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 41 | 23 | 0 | 3 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 42 | 23 | 0 | 2 | 1.5 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 43 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 44 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 45 | 23 | 0 | 3.5 | 1.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 46 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 47 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 48 | 23 | 0 | 3.5 | 1.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 49 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 50 | 23 | 0 | 6.5 | 4 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 51 | 23 | 0 | 6.5 | 1.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 8 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 53 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 54 | 23 | 0 | 6.5 | 1.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-22: UE Power Class 3 test requirements for NS_47 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 10 | 0 | 16 | 5 | 2 | 28+TT | 11-TT |
| 2 | 23 | -3 | 0.2 | 8.5 | 0 | 17.5 | 5 | 2 | 28+TT | 12.5-TT |
| 3 | 23 | -3 | 3.5 | 5 | 0 | 21 | 2 | 2 | 28+TT | 19-TT |
| 4 | 23 | -3 | 1.2 | 8.5 | 0 | 17.5 | 5 | 2 | 28+TT | 12.5-TT |
| 5 | 23 | -3 | 1.2 | 6 | 0 | 20 | 2.5 | 2 | 28+TT | 17.5-TT |
| 6 | 23 | -3 | 1.2 | 6 | 0 | 20 | 2.5 | 2 | 28+TT | 17.5-TT |
| 7 | 23 | -3 | 1.2 | 8.5 | 0 | 17.5 | 5 | 2 | 28+TT | 12.5-TT |
| 8 | 23 | 0 | 0.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 9 | 23 | 0 | 0.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 10 | 23 | 0 | 0.5 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 11 | 23 | 0 | 0.5 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 12 | 23 | 0 | 0.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 13 | 23 | 0 | 0.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 14 | 23 | 0 | 0.5 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 15 | 23 | 0 | 1.0 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 16 | 23 | 0 | 0.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 17 | 23 | 0 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 18 | 23 | 0 | 1.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 19 | 23 | 0 | 1.0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 20 | 23 | 0 | 1.0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 21 | 23 | 0 | 1.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 22 | 23 | 0 | 2.0 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 23 | 23 | 0 | 1.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 24 | 23 | 0 | 2.0 | 0 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 25 | 23 | 0 | 2.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 26 | 23 | 0 | 2.0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 27 | 23 | 0 | 2.0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 28 | 23 | 0 | 2.0 | 5.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 29 | 23 | 0 | 2.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 30 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 31 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 32 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 33 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 34 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 35 | 23 | 0 | 2.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 36 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 37 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 39 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 40 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 41 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 42 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 43 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 44 | 23 | 0 | 1.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 45 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 46 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 47 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 48 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 49 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 50 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 51 | 23 | 0 | 2.0 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 52 | 23 | 0 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 53 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 54 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 55 | 23 | 0 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 56 | 23 | 0 | 3 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 57 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 58 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 59 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 61 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 62 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 63 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 64 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 65 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 66 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 67 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 68 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 69 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 70 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-22a: UE Power Class 3 test requirements for NS_47 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | MPR & A-MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7 | 1.0 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 2 | 23 | 0 | 3.0 | 7 | 1.0 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 3 | 23 | 0 | 3.5 | 7 | 1.0 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| 4 | 23 | 0 | 6.5 | 7 | 1.0 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-23: UE Power Class 2 test requirements for NS_47 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 2 | 26 | 0 | 0.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 3 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 4 | 26 | 0 | 0.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 5 | 26 | 0 | 0.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 6 | 26 | 0 | 0.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 7 | 26 | 0 | 0.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 8 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 9 | 26 | 0 | 0.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 10 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 11 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 12 | 26 | 0 | 1.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 13 | 26 | 0 | 1.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 14 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 15 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 16 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 17 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 18 | 26 | 0 | 2.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 19 | 26 | 0 | 2.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 20 | 26 | 0 | 2.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 21 | 26 | 0 | 2.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 22 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 23 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 24 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 25 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 26 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 27 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 28 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 29 | 26 | 0 | 4.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 30 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 31 | 26 | 0 | 4.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 32 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 33 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 34 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 35 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 36 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 37 | 26 | 0 | 1.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 38 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 39 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 40 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 41 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 42 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 43 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 44 | 26 | 0 | 2.0 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 45 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 46 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 47 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 48 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 49 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 50 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 51 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 52 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 53 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 54 | 26 | 0 | 3.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 55 | 26 | 0 | 3.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 56 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 57 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 58 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 59 | 26 | 0 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 61 | 26 | 0 | 6.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 62 | 26 | 0 | 6.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 63 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-23a: UE Power Class 2 test requirements for NS_47 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 3.0 | 10 | 1.0 | 0 | 16 | 5.0 | 3 | 28+TT | 11-TT |
| 2 | 26 | 0 | 3.0 | 10 | 1.0 | 0 | 16 | 5.0 | 3 | 28+TT | 11-TT |
| 3 | 26 | 0 | 3.5 | 10 | 1.0 | 0 | 16 | 5.0 | 3 | 28+TT | 11-TT |
| 4 | 26 | 0 | 6.5 | 10 | 1.0 | 0 | 16 | 5.0 | 3 | 28+TT | 11-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-24: UE Power Class 3 test requirements for NS_48 (PC3 contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 6 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 7 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 12 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 16 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 17 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 22 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 23 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 26 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 30 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 31 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 32 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 33 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 36 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 37 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 38 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 39 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 40 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 41 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 42 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 43 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 44 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 46 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 47 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 48 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 49 | 23 | 0 | 1.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 50 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 51 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 52 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 53 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 54 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 55 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 56 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 57 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 58 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 59 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 60 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 61 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 62 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 63 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 64 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 65 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 66 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 67 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 68 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 69 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 70 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 71 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 72 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 73 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 74 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 75 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 76 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 77 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 78 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 79 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 80 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-24a: UE Power Class 2 test requirements for NS_48 (PC2 contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 1 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 2 | 26 | 0 | 1 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 3 | 26 | 0 | 1 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 4 | 26 | 0 | 1 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 5 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 6 | 26 | 0 | 1 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 7 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 8 | 26 | 0 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 9 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 10 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 11 | 26 | 0 | 3.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 12 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 13 | 26 | 0 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 14 | 26 | 0 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 15 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 16 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 17 | 26 | 0 | 0 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 18 | 26 | 0 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 19 | 26 | 0 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 20 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 21 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 22 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 23 | 26 | 0 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 24 | 26 | 0 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 25 | 26 | 0 | 1 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 26 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 27 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 28 | 26 | 0 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 29 | 26 | 0 | 4.5 | 3 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 30 | 26 | 0 | 4.5 | 3 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 31 | 26 | 0 | 4.5 | 3 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 32 | 26 | 0 | 4.5 | 3 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 33 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 34 | 26 | 0 | 4.5 | 3 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 35 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 36 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 37 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 38 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 39 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 40 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 41 | 26 | 0 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 42 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 43 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 44 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 45 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 46 | 26 | 0 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 47 | 26 | 0 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 48 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 49 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 50 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 51 | 26 | 0 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 52 | 26 | 0 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 53 | 26 | 0 | 4.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 54 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 55 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 56 | 26 | 0 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 57 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 58 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 59 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 60 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 61 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 63 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 64 | 26 | 0 | 3.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 65 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 66 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 67 | 26 | 0 | 3.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 68 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 69 | 26 | 0 | 3 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 70 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 71 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 72 | 26 | 0 | 3 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 73 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 74 | 26 | 0 | 3 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 75 | 26 | 0 | 3 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 76 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 77 | 26 | 0 | 3 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 78 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 79 | 26 | 0 | 3 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 80 | 26 | 0 | 3 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 81 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 82 | 26 | 0 | 3 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 83 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 84 | 26 | 0 | 3 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 85 | 26 | 0 | 6.5 | 4.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 86 | 26 | 0 | 6.5 | 4.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 87 | 26 | 0 | 6.5 | 4.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 88 | 26 | 0 | 6.5 | 4.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 89 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 90 | 26 | 0 | 6.5 | 4.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 91 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 92 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 93 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 94 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 95 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 96 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 97 | 26 | 0 | 6.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 98 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 99 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 100 | 26 | 0 | 6.5 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 101 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 102 | 26 | 0 | 6.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 103 | 26 | 0 | 6.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 104 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 105 | 26 | 0 | 6.5 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 106 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 107 | 26 | 0 | 6.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 108 | 26 | 0 | 6.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 109 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 110 | 26 | 0 | 6.5 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 111 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 112 | 26 | 0 | 6.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-25: UE Power Class 3 test requirements (NS_12) for n26 low range

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 2 | 23 | N/A | 5.5 | 1.5 | 16 | 5 | 2 | 25+TT | 11-TT |
| 3 | 23 | N/A | 9.5 | 1.5 | 12 | 6 | 2 | 25+TT | 6-TT |
| 4 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 6 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 7 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 8 | 23 | N/A | 5.5 | 1.5 | 16 | 5 | 2 | 25+TT | 11-TT |
| 9 | 23 | N/A | 9.5 | 1.5 | 12 | 6 | 2 | 25+TT | 6-TT |
| 10 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 11 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 12 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 13 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 14 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 15 | 23 | N/A | 7 | 1.5 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 16 | 23 | N/A | 9.5 | 1.5 | 12 | 6 | 2 | 25+TT | 6-TT |
| 17 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 18 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 19 | 23 | N/A | 7 | 1.5 | 14.5 | 5 | 2 | 25+TT | 9.5-TT |
| 20 | 23 | N/A | 9.5 | 1.5 | 12 | 6 | 2 | 25+TT | 6-TT |
| 21 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 22 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 23 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 24 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 25 | 23 | N/A | 4.5 | 1.5 | 17 | 5 | 2 | 25+TT | 12-TT |
| 26 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | N/A | 5 | 1.5 | 16.5 | 6 | 2 | 25+TT | 10.5-TT |
| 28 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 29 | 23 | N/A | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 30 | 23 | N/A | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 31 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 32 | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 33 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-25b: UE Power Class 3 test requirements (NS_12) for n26 high range

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 2 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 3 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 4 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 6 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 7 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 8 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 9 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 10 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 11 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 12 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 13 | 23 | 1 | N/A | 1.5 | 20.5 | 2 | 2 | 25+TT | 18.5-TT |
| 14 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 15 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 16 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 17 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 18 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 19 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 20 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 21 | 23 | N/A | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 22 | 23 | N/A | 9.5 | 0 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 23 | 23 | 3 | 0 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 25 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 26 | 23 | N/A | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 27 | 23 | N/A | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 28 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 29 | 23 | N/A | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 30 | 23 | N/A | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 31 | 23 | 1 | N/A | 1.5 | 20.5 | 2 | 2 | 25+TT | 18.5-TT |
| 32 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 33 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 3 | 0 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-26: UE Power Class 3 test requirements (NS_13) for n26

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 3.5 | 1.5 | 18 | 4 | 2 | 25+TT | 14-TT |
| 2 | 23 | N/A | 4.5 | 1.5 | 17 | 5 | 2 | 25+TT | 12-TT |
| 3 | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 4 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 6 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 7 | 23 | N/A | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 8 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 9 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 10 | 23 | N/A | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 11 | Void |  |  |  |  |  |  |  |  |
| 12 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 13 | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 14 | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 15 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 16 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 17 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 18 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 19 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 20 | 23 | N/A | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 21 | Void |  |  |  |  |  |  | r |  |
| 22 | 23 | N/A | 4 | 1.5 | 17.5 | 4 | 2 | 25+TT | 13.5-TT |
| 23 | 23 | N/A | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 24 | 23 | N/A | 4 | 0 | 19 | 4 | 2 | 25+TT | 15-TT |
| 25 | 23 | N/A | 4 | 0 | 19 | 4 | 2 | 25+TT | 15-TT |
| 26 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | N/A | 4 | 0 | 19 | 4 | 2 | 25+TT | 15-TT |
| 28 | 23 | N/A | 5.5 | 1.5 | 16 | 5 | 2 | 25+TT | 11-TT |
| 29 | 23 | N/A | 5.5 | 1.5 | 16 | 5 | 2 | 25+TT | 11-TT |
| 30 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 31 | 23 | N/A | 5.5 | 0 | 17.5 | 4 | 2 | 25+TT | 13.5-TT |
| 32 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-27a: UE Power Class 3 test requirements (NS_14) for n26 low range

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 2 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | N/A | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 5 | 23 | N/A | 3 | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 6 | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 7 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 8 | 23 | 1 | N/A | 1.5 | 20 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 0 | N/A | 1.5 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 10 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 11 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 12 | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 13 | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 14 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 16 | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 17 | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 18 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 20 | 23 | N/A | 6 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 21 | 23 | N/A | 8 | 1.5 | 13.5 | 5 | 2 | 25+TT | 8.5-TT |
| 22 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 3 | N/A | 1.5 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | 23 | 1.5 | N/A | 1.5 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 25 | 23 | 1.5 | N/A | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-27b: UE Power Class 3 test requirements (NS_14) for n26 high range

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | N/A | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 5 | 23 | N/A | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 6 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 7 | 23 | N/A | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 8 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 9 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 10 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 11 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 13 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 14 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 16 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 17 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 18 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 20 | 23 | N/A | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 21 | 23 | N/A | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 22 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 3 | N/A | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 24 | 23 | 1.5 | N/A | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 25 | 23 | 1.5 | N/A | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-28: UE Power Class 3 test requirements (NS_15) for n26

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | N/A | 9 | 1.5 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 2 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 3 | 23 | N/A | 5 | 1.5 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 4 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 5 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 6 | 23 | N/A | 9 | 1.5 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 7 | 23 | N/A | 13.5 | 1.5 | 8 | 6 | 2 | 25+TT | 2-TT |
| 8 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 9 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 10 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 11 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 12 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 13 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 14 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 15 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 16 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 17 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 18 | 23 | N/A | 9 | 1.5 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 19 | 23 | N/A | 13.5 | 1.5 | 8 | 6 | 2 | 25+TT | 2-TT |
| 20 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 21 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 22 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 24 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 25 | 23 | N/A | 13.5 | 0 | 9.5 | 6 | 2 | 25+TT | 3.5-TT |
| 26 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 27 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 28 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 29 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 30 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 31 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 32 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 33 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 34 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 35 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 36 | 23 | N/A | 9 | 1.5 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 37 | 23 | N/A | 13.5 | 1.5 | 8 | 6 | 2 | 25+TT | 2-TT |
| 38 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 39 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 40 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 41 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 42 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 43 | 23 | N/A | 13.5 | 0 | 9.5 | 6 | 2 | 25+TT | 3.5-TT |
| 44 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 45 | 23 | N/A | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 47 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 48 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 49 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 50 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 51 | 23 | 0 | N/A | 0 | 23 | 2 | 2 | 25+TT | 21-TT |
| 52 | 23 | N/A | 10.5 | 1.5 | 11 | 6 | 2 | 25+TT | 5-TT |
| 53 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 54 | 23 | N/A | 6.5 | 1.5 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 55 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 56 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 57 | 23 | N/A | 10.5 | 1.5 | 11 | 6 | 2 | 25+TT | 5-TT |
| 58 | 23 | N/A | 13.5 | 1.5 | 8 | 6 | 2 | 25+TT | 2-TT |
| 59 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 60 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 61 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 62 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 63 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 64 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 65 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 66 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 67 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 68 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 69 | 23 | N/A | 10.5 | 1.5 | 11 | 6 | 2 | 25+TT | 5-TT |
| 70 | 23 | N/A | 13.5 | 1.5 | 8 | 6 | 2 | 25+TT | 2-TT |
| 71 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 72 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 73 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 74 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 75 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 76 | 23 | N/A | 13.5 | 0 | 9.5 | 6 | 2 | 25+TT | 3.5-TT |
| 77 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 78 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 79 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 80 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 81 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 82 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 83 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 84 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 85 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 86 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 87 | 23 | N/A | 10.5 | 1.5 | 11 | 6 | 2 | 25+TT | 5-TT |
| 88 | 23 | N/A | 13.5 | 1.5 | 8 | 6 | 2 | 25+TT | 2-TT |
| 89 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 90 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 91 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 92 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 93 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 94 | 23 | N/A | 13.5 | 0 | 9.5 | 6 | 2 | 25+TT | 3.5-TT |
| 95 | 23 | N/A | 10.5 | 0 | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| 96 | 23 | N/A | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 97 | 23 | N/A | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 98 | 23 | N/A | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 99 | 23 | 3 | N/A | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 100 | 23 | 3 | N/A | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 101 | 23 | 1.5 | N/A | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 102 | 23 | 1.5 | N/A | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 103 | 23 | N/A | 5.5 | 1.5 | 16 | 6 | 2 | 25+TT | 10-TT |
| 104 | 23 | N/A | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 105 | 23 | N/A | 2 | 0 | 21 | 3.5 | 2 | 25+TT | 17.5-TT |
| 106 | 23 | 1 | N/A | 0 | 22 | 2 | 2 | 25+TT | 20-TT |
| 107 | 23 | N/A | 7 | 1.5 | 14.5 | 6 | 2 | 25+TT | 8.5-TT |
| 108 | 23 | N/A | 7 | 0 | 16 | 6 | 2 | 25+TT | 10-TT |
| 109 | 23 | N/A | 3 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 110 | 23 | 3 | N/A | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-29: UE Power Class 3 test requirements for NS_45 (contiguous allocation)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 3 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 4, 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 9 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10, 11 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 12 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-30: UE Power Class 3 test requirements for NS_46 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 2 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 3 | 23 | 0 | 2.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 2 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | 23 | 0 | 2.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 7 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 9 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 10 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 11 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 12 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 13 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 14 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 16 | 23 | 0 | 3.5 | 3.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 17 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 18 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 19 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 20 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 21 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 22 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-30a: UE Power Class 1 test requirements for NS_46 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0 | 2.5 | 3 | 0 | 28 | 2 | 3 | 33+TT | 26-TT |
| 2 | 31 | 0 | 6.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 3 | 31 | 0 | 4.5 | 12.5 | 0 | 18.5 | 4 | 3 | 33+TT | 14.5-TT |
| 4 | 31 | 0 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 24.5-TT |
| 5 | 31 | 0 | 4.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 6 | 31 | 0 | 2.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 7 | 31 | 0 | 4.5 | 6 | 0 | 25 | 2 | 3 | 33+TT | 23-TT |
| 8 | 31 | 0 | 4.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 9 | 31 | 0 | 4.5 | 12.5 | 0 | 18.5 | 4 | 3 | 33+TT | 14.5-TT |
| 10 | 31 | 0 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 24.5-TT |
| 11 | 31 | 0 | 4.5 | 6 | 0 | 25 | 2 | 3 | 33+TT | 23-TT |
| 12 | 31 | 0 | 2.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 13 | 31 | 0 | 4.5 | 6 | 0 | 25 | 2 | 3 | 33+TT | 23-TT |
| 14 | 31 | 0 | 4.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 15 | 31 | 0 | 2.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 16 | 31 | 0 | 4.5 | 12.5 | 0 | 18.5 | 4 | 3 | 33+TT | 14.5-TT |
| 17 | 31 | 0 | 4.5 | 6 | 0 | 25 | 2 | 3 | 33+TT | 23-TT |
| 18 | 31 | 0 | 4.5 | 7 | 0 | 24 | 2 | 3 | 33+TT | 22-TT |
| 19 | 31 | 0 | 3.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 20 | 31 | 0 | 6.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| 21 | 31 | 0 | 6.5 | 15 | 0 | 16 | 5 | 3 | 33+TT | 11-TT |
| 22 | 31 | 0 | 6.5 | 10 | 0 | 21 | 2 | 3 | 33+TT | 19-TT |
| 23 | 31 | 0 | 4.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| 24 | 31 | 0 | 3.5 | 5 | 0 | 26 | 2 | 3 | 33+TT | 24-TT |
| 25 | 31 | 0 | 6.5 | 9 | 0 | 22 | 2 | 3 | 33+TT | 20-TT |
| 26 | 31 | 0 | 6.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| 27 | 31 | 0 | 6.5 | 15 | 0 | 16 | 5 | 3 | 33+TT | 11-TT |
| 28 | 31 | 0 | 6.5 | 10 | 0 | 21 | 2 | 3 | 33+TT | 19-TT |
| 29 | 31 | 0 | 6.5 | 9 | 0 | 22 | 2 | 3 | 33+TT | 20-TT |
| 30 | 31 | 0 | 3.5 | 5 | 0 | 26 | 2 | 3 | 33+TT | 24-TT |
| 31 | 31 | 0 | 6.5 | 9 | 0 | 22 | 2 | 3 | 33+TT | 20-TT |
| 32 | 31 | 0 | 6.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| 33 | 31 | 0 | 3.5 | 4 | 0 | 27 | 2 | 3 | 33+TT | 25-TT |
| 34 | 31 | 0 | 6.5 | 15 | 0 | 16 | 5 | 3 | 33+TT | 11-TT |
| 35 | 31 | 0 | 6.5 | 9 | 0 | 22 | 2 | 3 | 33+TT | 20-TT |
| 36 | 31 | 0 | 6.5 | 13 | 0 | 18 | 4 | 3 | 33+TT | 14-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-31: UE Power Class 3 test requirements (NS_44) for band n38

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 4 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 5 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 6 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 7 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 8 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | 23 | 0 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 10 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 11 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 12 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 13 | 23 | 0 | 2.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 14 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 15 | 23 | 0 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 16 | 23 | 0 | 2.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 17 | 23 | 0 | 2.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 18 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 19 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 20 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 21 | 23 | 0 | 4.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 22 | 23 | 0 | 4.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 23 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 25 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 26 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 27 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 29 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 31 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 32 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 33 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 34 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 35 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 37 | 23 | 0 | 3.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 38 | 23 | 0 | 3.5 | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 39 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 40 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 41 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 42 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 43 | 23 | 0 | 6.5 | 8 | 0 | 15 | 5 | 2 | 25+TT | 10-TT |
| 44 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-32: UE Power Class 3 test requirements (NS_21)

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 5 | 23 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 10 | 23 | 0.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 3 | 5 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | 10 | 23 | 0.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 5 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19 -TT |
| 4 | 10 | 23 | 0.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 5a | 5 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19 -TT |
| 5b | 10 | 23 | 0.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 6, 7 | 5 | 23 | 1 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 5 | 23 | 1 | 2.0 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
|  | 10 | 23 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 5 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 9 | 10 | 23 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10a | 5 | 23 | 1 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 10b | 10 | 23 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 11, 12 | 5 | 23 | 2 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5 -TT |
|  | 10 | 23 | 2 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 13 | 5 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
|  | 10 | 23 | 2 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 5 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 14 | 10 | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 15a | 5 | 23 | 2 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 15b | 10 | 23 | 2 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 16, 17 | 5 | 23 | 2.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5 -TT |
|  | 10 | 23 | 2.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 18 | 5 | 23 | 2.5 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
|  | 10 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
|  | 5 | 23 | 2.5 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 19 | 10 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 20a | 5 | 23 | 2.5 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18 -TT |
| 20b | 10 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 21, 22 | 5 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 4.5 | 6 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 23 | 5 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 4.5 | 4 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 5 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 24 | 10 | 23 | 4.5 | 3 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 25a | 5 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 25b | 10 | 23 | 4.5 | 3 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 26, 27 | 5 | 23 | 3 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5 -TT |
|  | 10 | 23 | 3 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 28 | 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5 -TT |
|  | 10 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
|  | 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 29 | 10 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30a | 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30b | 10 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31, 32 | 5 | 23 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5 -TT |
|  | 10 | 23 | 3 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 33 | 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5 -TT |
|  | 10 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
|  | 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 34 | 10 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 35a | 5 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 35b | 10 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 36, 37 | 5 | 23 | 3.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 38 | 5 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5 -TT |
|  | 10 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
|  | 5 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 39 | 10 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 40a | 5 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 40b | 10 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 41, 42 | 5 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 43 | 5 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 5 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 44 | 10 | 23 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 45a | 5 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 45b | 10 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-33: UE Power Class 3 test requirements (NS_49)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 5 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 7 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 10 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 16 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 17 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 18 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 19 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 20 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 23 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 24 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 25 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 26 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 27 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 29 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 30 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 31 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 32 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 33 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 35 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 38 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 39 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 40 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 41 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 42 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 44 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 46 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 47 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 48 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 49 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 50 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 51 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 52 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 53 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 54 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 55 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 56 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 57 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 58 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 59 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 60 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 61 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 62 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 63 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 64 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 65 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 66 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 67 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 68 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 69 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 70 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 71 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 72 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 73 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 74 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 75 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 76 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 77 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 78 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 79 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 80 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 81 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 82 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 83 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 84 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 85 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 86 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 87 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 88 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 89 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 90 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 91 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 92 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 93 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 94 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 95 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 96 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 97 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 98 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 99 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 100 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 101 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 102 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 103 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 104 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-33a: UE Power Class 2 test requirements (NS_49)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 2 | 26 | 0 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 3 | 26 | 0 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 4 | 26 | 0 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 5 | 26 | 0 | 1 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 6 | 26 | 0 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 7 | 26 | 0 | 1 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 8 | 26 | 0 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 9 | 26 | 0 | 0 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 10 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 11 | 26 | 0 | 1 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 12 | 26 | 0 | 1 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 13 | 26 | 0 | 1 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 14 | 26 | 0 | 1 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 15 | 26 | 0 | 3.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 16 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 17 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 18 | 26 | 0 | 1 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 19 | 26 | 0 | 1 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 20 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 21 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 22 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 23 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 24 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 25 | 26 | 0 | 1 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 26 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 27 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 28 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 29 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 30 | 26 | 0 | 1 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 31 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 32 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 33 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 34 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 35 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 36 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 37 | 26 | 0 | 4.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 38 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 39 | 26 | 0 | 4.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 40 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 41 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 42 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 43 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 44 | 26 | 0 | 4.5 | 4 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 45 | 26 | 0 | 4.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 46 | 26 | 0 | 4.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 47 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 48 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 49 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 50 | 26 | 0 | 4.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 51 | 26 | 0 | 4.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 52 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 53 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 54 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 55 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 56 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 57 | 26 | 0 | 4.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 58 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 59 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 60 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 61 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 62 | 26 | 0 | 4.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 63 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 64 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 65 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 66 | 26 | 0 | 3.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 67 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 68 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 69 | 26 | 0 | 3 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 71 | 26 | 0 | 3 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 72 | 26 | 0 | 3.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 73 | 26 | 0 | 1.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 74 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 75 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 76 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 77 | 26 | 0 | 3 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 78 | 26 | 0 | 3 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 79 | 26 | 0 | 3.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 80 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 81 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 82 | 26 | 0 | 3 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 83 | 26 | 0 | 3 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 84 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 85 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 86 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 87 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 88 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 89 | 26 | 0 | 3 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 90 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 91 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 92 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 93 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 94 | 26 | 0 | 3 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 95 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 96 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 97 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 98 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 99 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 100 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 101 | 26 | 0 | 6.5 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 102 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 103 | 26 | 0 | 6.5 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 104 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 105 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 106 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 107 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 108 | 26 | 0 | 6.5 | 5.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 109 | 26 | 0 | 6.5 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 110 | 26 | 0 | 6.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 111 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 112 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 113 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 114 | 26 | 0 | 6.5 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 115 | 26 | 0 | 6.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 116 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 117 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 118 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 119 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 120 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 121 | 26 | 0 | 6.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 122 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 123 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 124 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 125 | 26 | 0 | 6.5 | 6 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 126 | 26 | 0 | 6.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 127 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 128 | 26 | 0 | 6.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-34: UE Power Class 3 test requirements (NS_06) for band n12 and n14

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20.5-TT |
| 2 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20.5-TT |
| 3 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20.5-TT |
| 4 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 20-TT |
| 5 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 20-TT |
| 6 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 20-TT |
| 7 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 19-TT |
| 8 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 19-TT |
| 9 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 19-TT |
| 10 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 11 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 13 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 14 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 16 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 17 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 18 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 19 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 20 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 21 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 23 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 24 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 25 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 26 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-35: UE Power Class 1 test requirements (NS_06) for band n14

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 2 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 3 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 4 | 31 | 1 | 0 | 0 | 30 | 2 | 3 | 33+TT | 27-TT |
| 5 | 31 | 1 | 0 | 0 | 30 | 2 | 3 | 33+TT | 27-TT |
| 6 | 31 | 1 | 0 | 0 | 30 | 2 | 3 | 33+TT | 27-TT |
| 7 | 31 | 2 | 0 | 0 | 29 | 2 | 3 | 33+TT | 26-TT |
| 8 | 31 | 2 | 0 | 0 | 29 | 2 | 3 | 33+TT | 26-TT |
| 9 | 31 | 2 | 0 | 0 | 29 | 2 | 3 | 33+TT | 26-TT |
| 10 | 31 | 2.5 | 0 | 0 | 28.5 | 2 | 3 | 33+TT | 25.5-TT |
| 11 | 31 | 2.5 | 0 | 0 | 28.5 | 2 | 3 | 33+TT | 25.5-TT |
| 12 | 31 | 2.5 | 0 | 0 | 28.5 | 2 | 3 | 33+TT | 25.5-TT |
| 13 | 31 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 23.5-TT |
| 14 | 31 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 23.5-TT |
| 15 | 31 | 4.5 | 0 | 0 | 26.5 | 2 | 3 | 33+TT | 23.5-TT |
| 16 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 17 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 18 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 19 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 20 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 21 | 31 | 3 | 0 | 0 | 28 | 2 | 3 | 33+TT | 25-TT |
| 22 | 31 | 3.5 | 0 | 0 | 27.5 | 2 | 3 | 33+TT | 24.5-TT |
| 23 | 31 | 3.5 | 0 | 0 | 27.5 | 2 | 3 | 33+TT | 24.5-TT |
| 24 | 31 | 3.5 | 0 | 0 | 27.5 | 2 | 3 | 33+TT | 24.5-TT |
| 25 | 31 | 6.5 | 0 | 0 | 24.5 | 2 | 3 | 33+TT | 21.5-TT |
| 26 | 31 | 6.5 | 0 | 0 | 24.5 | 2 | 3 | 33+TT | 21.5-TT |
| 27 | 31 | 6.5 | 0 | 0 | 24.5 | 2 | 3 | 33+TT | 21.5-TT |
| 28 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 29 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 3 | 33+TT | 27.5-TT |
| 30 | 31 | 0 | 0 | 0 | 31 | 2 | 3 | 33+TT | 28-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-35a: UE Power Class 1 test requirements (NS_06) for band n85

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 2 | 31 | 0.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 3 | 31 | 0.5 | 0 | 0 | 30.5 | 2 | 2.5 | 31+TT | 28-TT |
| 4 | 31 | 1 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 5 | 31 | 1 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 6 | 31 | 1 | 0 | 0 | 30 | 2 | 2.5 | 31+TT | 27.5-TT |
| 7 | 31 | 2 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 8 | 31 | 2 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 9 | 31 | 2 | 0 | 0 | 29 | 2 | 2.5 | 31+TT | 26.5-TT |
| 10 | 31 | 2.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 11 | 31 | 2.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 12 | 31 | 2.5 | 0 | 0 | 28.5 | 2 | 2.5 | 31+TT | 26-TT |
| 13 | 31 | 4.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 14 | 31 | 4.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 15 | 31 | 4.5 | 0 | 0 | 26.5 | 2 | 2.5 | 31+TT | 24-TT |
| 16 | 31 | 3 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 17 | 31 | 3 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 18 | 31 | 3 | 0 | 0 | 28 | 2 | 2.5 | 31+TT | 25.5-TT |
| 19 | 31 | 3 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 20 | 31 | 3 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 21 | 31 | 3 | 0 | 0 | 28 | 2 | 2.5 | 31+TT | 25.5-TT |
| 22 | 31 | 3.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 23 | 31 | 3.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 24 | 31 | 3.5 | 0 | 0 | 27.5 | 2 | 2.5 | 31+TT | 25-TT |
| 25 | 31 | 6.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 26 | 31 | 6.5 | 8.5 | 0 | 22.5 | 2 | 2.5 | 31+TT | 20-TT |
| 27 | 31 | 6.5 | 0 | 0 | 24.5 | 2 | 2.5 | 31+TT | 22-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-36: UE Power Class 3 test requirements (NS_56) for band n24

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 2 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 3 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 5 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 6 | 23 | 1 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 7 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 8 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 9 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 10 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 11 | 23 | 2 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 12 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 13 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 15 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 16 | 23 | 2.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 17 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 18 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 20 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 21 | 23 | 4.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 22 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 23 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 24 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 25 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 26 | 23 | 0.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 27 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 28 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 29 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 30 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 32 | 23 | 0.5 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 33 | 23 | 0.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 34 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 35 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 36 | 23 | 0.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 37 | 23 | 1 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 38 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 39 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 40 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 41 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 42 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 43 | 23 | 1 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 44 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 45 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 46 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 47 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 48 | 23 | 2 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 49 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 50 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 51 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 52 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 53 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 54 | 23 | 2 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 55 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 56 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 57 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 58 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 59 | 23 | 2.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 61 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 62 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 63 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 64 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 65 | 23 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 66 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 67 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 68 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 69 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 70 | 23 | 4.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 71 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 72 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 73 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 74 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 75 | 23 | 4.5 | 4 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 76 | 23 | 4.5 | 2 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 77 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 78 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 79 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 80 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 81 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 82 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 83 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 84 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 85 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 86 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 87 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 88 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 89 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 90 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 91 | 23 | 3.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 92 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 93 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 94 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 95 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 96 | 23 | 6.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 97 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 98 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 99 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 100 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 101 | 23 | 3 | 12 | 0 | 9 | 6 | 3 | 25+TT | 5-TT |
| 102 | 23 | 3 | 8 | 0 | 11 | 5 | 3 | 25+TT | 10-TT |
| 103 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 104 | 23 | 1.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 105 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 106 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 107 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 108 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 109 | 23 | 1.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 110 | 23 | 1.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 111 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 112 | 23 | 3 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 113 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 114 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 115 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 116 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 117 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 118 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 119 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 121 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 122 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 123 | 23 | 3.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 124 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 125 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 126 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 127 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 128 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 129 | 23 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 130 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 131 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 132 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 133 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 134 | 23 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 135 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 136 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 137 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 138 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 139 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 140 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 141 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 142 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 143 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 144 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-37: UE Power Class 3 test requirements (NS_07) for band n13

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 2 | 23 | 0 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 3 | 23 | 0 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 4 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 5 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 6 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 7 | 23 | 0 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 8 | 23 | 0 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 9 | 23 | 1 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 10 | 23 | 0 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 11 | 23 | 0 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 12 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 13 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 15 | 23 | 0 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 16 | 23 | 0 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 17 | 23 | 2 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 18 | 23 | 1 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 19 | 23 | 1 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 20 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 21 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 22 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 23 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 24 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 25 | 23 | 2.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 26 | 23 | 2.5 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 27 | 23 | 2.5 | 9 | 0 | 14 | 5 | 3 | 25+TT | 19-TT |
| 28 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 29 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 30 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 31 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 32 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 33 | 23 | 4.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 34 | 23 | 4.5 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 35 | 23 | 4.5 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 36 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 37 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 38 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 39 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 40 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 41 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 42 | 23 | 1.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 43 | 23 | 1.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 44 | 23 | 1.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 45 | 23 | 1.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 46 | 23 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 47 | 23 | 1.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 48 | 23 | 1.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 49 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 50 | 23 | 2 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 51 | 23 | 2 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 52 | 23 | 2 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 53 | 23 | 2 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 54 | 23 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 55 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 56 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 57 | 23 | 3.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 58 | 23 | 3.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 59 | 23 | 3.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 60 | 23 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 61 | 23 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 62 | 23 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 63 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 64 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 65 | 23 | 6.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 66 | 23 | 6.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 67 | 23 | 6.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 68 | 23 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 69 | 23 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 70 | 23 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 71 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 72 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-38: UE Power Class 2 test requirements (NS_50) for band n39 (Step 4)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19.0 - TT |  |
| 2 | 26 | 0 | 1 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 3 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 4 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 5 | 26 | 0 | 1 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 6 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 7 | 26 | 0 | 3.5 | 5 | 0 |  | 21 |  | 2.0 |  | 3 | 28.0 + TT | 18 - TT |  |
| 8 | 26 | 0 | 1 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19.0 - TT |  |
| 9 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 10 | 26 | 0 | 1 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 11 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19.0 - TT |  |
| 12 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 – TT |  |
| 13 | 26 | 0 | 0 | 1 | 0 |  | 25 |  | 2.0 |  | 3 | 28.0 + TT | 22 - TT |  |
| 14 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 – TT |  |
| 15 | 26 | 0 | 1 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 16 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 17 | 26 | 0 | 0 | 1 | 0 |  | 25 |  | 2.0 |  | 3 | 28.0 + TT | 22 - TT |  |
| 18 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 19 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 20 | 26 | 0 | 1 | 11.5 | 0 |  | 14.5 |  | 5.0 |  | 3 | 28.0 + TT | 9.5 - TT |  |
| 21 | 26 | 0 | 1 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19.0 - TT |  |
| 22 | 26 | 0 | 1 | 10 | 0 |  | 16 |  | 5.0 |  | 3 | 28.0 + TT | 11 - TT |  |
| 23 | 26 | 0 | 1 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 24 | 26 | 0 | 3.5 | 8 | 0 |  | 18 |  | 4.0 |  | 3 | 28.0 + TT | 14.0 - TT |  |
| 25 | 26 | 0 | 1 | 11.5 | 0 |  | 14.5 |  | 5.0 |  | 3 | 28.0 + TT | 9.5 - TT |  |
| 26 | 26 | 0 | 0 | 10 | 0 |  | 16 |  | 2.0 |  | 3 | 28.0 + TT | 13 - TT |  |
| 27 | 26 | 0 | 0 | 4 | 0 |  | 22 |  | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT |  |
| 28 | 26 | 0 | 0 | 3.5 | 0 |  | 22.5 |  | 2.0 |  | 3 | 28.0 + TT | 19.5 – TT |  |
| 29 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 30 | 26 | 0 | 0 | 8 | 0 |  | 18 |  | 4.0 |  | 3 | 28.0 + TT | 14.0 - TT |  |
| 31 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 32 | 26 | 0 | 4.5 | 2.5 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 33 | 26 | 0 | 4.5 | 2.5 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 34 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 35 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 36 | 26 | 0 | 4.5 | 2.5 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 37 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 38 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 39 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 40 | 26 | 0 | 4.5 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 41 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 42 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 43 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 44 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 45 | 26 | 0 | 4.5 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 46 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 47 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 48 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 49 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 50 | 26 | 0 | 4.5 | 11.5 | 0 |  | 14.5 |  | 5.0 |  | 3 | 28.0 + TT | 9.5 - TT |  |
| 51 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 52 | 26 | 0 | 4.5 | 10 | 0 |  | 16 |  | 5.0 |  | 3 | 28.0 + TT | 11 - TT |  |
| 53 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 54 | 26 | 0 | 4.5 | 8 | 0 |  | 18.0 |  | 4.0 |  | 3 | 28.0 + TT | 14 - TT |  |
| 55 | 26 | 0 | 4.5 | 11.5 | 0 |  | 14.5 |  | 5.0 |  | 3 | 28.0 + TT | 9.5 - TT |  |
| 56 | 26 | 0 | 4.5 | 10 | 0 |  | 16 |  | 5.0 |  | 3 | 28.0 + TT | 11 - TT |  |
| 57 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 58 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 59 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 60 | 26 | 0 | 4.5 | 8 | 0 |  | 18 |  | 5.0 |  | 3 | 28.0 + TT | 13 - TT |  |
| 61 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 62 | 26 | 0 | 3 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 63 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 64 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 65 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 66 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 67 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 68 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 69 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 70 | 26 | 0 | 3 | 7 | 0 |  | 19 |  | 3.5 |  | 3 | 28.0 + TT | 15.5 - TT |  |
| 71 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 72 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 73 | 26 | 0 | 1.5 | 2 | 0 |  | 24 |  | 2.0 |  | 3 | 28.0 + TT | 21 - TT |  |
| 74 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 75 | 26 | 0 | 3 | 7 | 0 |  | 19 |  | 3.5 |  | 3 | 28.0 + TT | 15.5 - TT |  |
| 76 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 77 | 26 | 0 | 1.5 | 2 | 0 |  | 24 |  | 2.0 |  | 3 | 28.0 + TT | 21 - TT |  |
| 78 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 79 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 80 | 26 | 0 | 3 | 12.5 | 0 |  | 13.5 |  | 5.0 |  | 3 | 28.0 + TT | 8.5 - TT |  |
| 81 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 82 | 26 | 0 | 3 | 11 | 0 |  | 15.0 |  | 5.0 |  | 3 | 28.0 + TT | 10 - TT |  |
| 83 | 26 | 0 | 3 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 84 | 26 | 0 | 3.5 | 8 | 0 |  | 18 |  | 4.0 |  | 3 | 28.0 + TT | 14 - TT |  |
| 85 | 26 | 0 | 3 | 12.5 | 0 |  | 13.5 |  | 5.0 |  | 3 | 28.0 + TT | 8.5 - TT |  |
| 86 | 26 | 0 | 1.5 | 11 | 0 |  | 15 |  | 5.0 |  | 3 | 28.0 + TT | 10 - TT |  |
| 87 | 26 | 0 | 1.5 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 88 | 26 | 0 | 1.5 | 5 | 0 |  | 21 |  | 2.0 |  | 3 | 28.0 + TT | 18 - TT |  |
| 89 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 90 | 26 | 0 | 1.5 | 8 | 0 |  | 18 |  | 4 |  | 3 | 28.0 + TT | 14 - TT |  |
| 91 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 92 | 26 | 0 | 3 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 93 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 94 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 95 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 96 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 97 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 98 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 99 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 100 | 26 | 0 | 3 | 7 | 0 |  | 19 |  | 3.5 |  | 3 | 28.0 + TT | 15.5 - TT |  |
| 101 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 102 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 103 | 26 | 0 | 1.5 | 2 | 0 |  | 24 |  | 2.0 |  | 3 | 28.0 + TT | 21 - TT |  |
| 104 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 – TT |  |
| 105 | 26 | 0 | 3 | 7 | 0 |  | 19 |  | 3.5 |  | 3 | 28.0 + TT | 15.5 – TT |  |
| 106 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 107 | 26 | 0 | 1.5 | 2 | 0 |  | 24 |  | 2.0 |  | 3 | 28.0 + TT | 21 - TT |  |
| 108 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 109 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 – TT |  |
| 110 | 26 | 0 | 6.5 | 12.5 | 0 |  | 13.5 |  | 5.0 |  | 3 | 28.0 + TT | 8.5 - TT |  |
| 111 | 26 | 0 | 6.5 | 0 | 0 |  | 19.5 |  | 3.5 |  | 3 | 28.0 + TT | 16 - TT |  |
| 112 | 26 | 0 | 6.5 | 11 | 0 |  | 15 |  | 5.0 |  | 3 | 28.0 + TT | 10 - TT |  |
| 113 | 26 | 0 | 6.5 | 0 | 0 |  | 19.5 |  | 3.5 |  | 3 | 28.0 + TT | 16 - TT |  |
| 114 | 26 | 0 | 6.5 | 8 | 0 |  | 18 |  | 4.0 |  | 3 | 28.0 + TT | 14 - TT |  |
| 115 | 26 | 0 | 3 | 12.5 | 0 |  | 13.5 |  | 5.0 |  | 3 | 28.0 + TT | 8.5 - TT |  |
| 116 | 26 | 0 | 1.5 | 11 | 0 |  | 15 |  | 5.0 |  | 3 | 28.0 + TT | 10 - TT |  |
| 117 | 26 | 0 | 1.5 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 118 | 26 | 0 | 1.5 | 5 | 0 |  | 21 |  | 2.0 |  | 3 | 28.0 + TT | 18 – TT |  |
| 119 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 – TT |  |
| 120 | 26 | 0 | 1.5 | 8 | 0 |  | 18 |  | 4.0 |  | 3 | 28.0 + TT | 14 - TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Void. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-39: UE Power Class 3 test requirements (NS_50) for band n39 (Step 3)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0- TT |  |
| 2 | 23 | 0 | 1 | 2 | 0 |  | 21 |  | 2.0 |  | 2 | 25.0 + TT | 19.0 - TT |  |
| 3 | 23 | 0 | 1 | 0 | 0 |  | 22 |  | 2.0 |  | 2 | 25.0 + TT | 20.0 - TT |  |
| 4 | 23 | 0 | 1 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 5 | 23 | 0 | 1 | 2 | 0 |  | 21 |  | 2.0 |  | 2 | 25.0 + TT | 19.0 - TT |  |
| 6 | 23 | 0 | 1 | 0 | 0 |  | 23 |  | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT |  |
| 7 | 23 | 0 | 1 | 11 | 0 |  | 12 |  | 6.0 |  | 2 | 25.0 + TT | 6.0 - TT |  |
| 8 | 23 | 0 | 1 | 3 | 0 |  | 20 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 9 | 23 | 0 | 1 | 7 | 0 |  | 16 |  | 5.0 |  | 2 | 25.0 + TT | 11.0 - TT |  |
| 10 | 23 | 0 | 1 | 2 | 0 |  | 21 |  | 2.0 |  | 2 | 25.0 + TT | 19.0 - TT |  |
| 11 | 23 | 0 | 1 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 12 | 23 | 0 | 1 | 11 | 0 |  | 12 |  | 6.0 |  | 2 | 25.0 + TT | 6.0 - TT |  |
| 13 | 23 | 0 | 1 | 3 | 0 |  | 20 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 14 | 23 | 0 | 1 | 7 | 0 |  | 16 |  | 5.0 |  | 2 | 25.0 + TT | 11.0 - TT |  |
| 15 | 23 | 0 | 1 | 2 | 0 |  | 21 |  | 2.0 |  | 2 | 25.0 + TT | 19.0 - TT |  |
| 16 | 23 | 0 | 1 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 17 | 23 | 0 | 4.5 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 18 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 19 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5- TT |  |
| 20 | 23 | 0 | 4.5 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 21 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 22 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 23 | 23 | 0 | 4.5 | 11 | 0 |  | 12 |  | 6.0 |  | 2 | 25.0 + TT | 6.0 - TT |  |
| 24 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 25 | 23 | 0 | 4.5 | 7 | 0 |  | 16 |  | 5.0 |  | 2 | 25.0 + TT | 11.0 - TT |  |
| 26 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 27 | 23 | 0 | 4.5 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 28 | 23 | 0 | 4.5 | 11 | 0 |  | 12 |  | 6.0 |  | 2 | 25.0 + TT | 6.0 - TT |  |
| 29 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 30 | 23 | 0 | 3 | 7 | 0 |  | 16 |  | 5.0 |  | 2 | 25.0 + TT | 11.0 - TT |  |
| 31 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 32 | 23 | 0 | 3 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 33 | 23 | 0 | 3 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 – TT |  |
| 34 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 35 | 23 | 0 | 3 | 0 | 0 |  | 20.0 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 36 | 23 | 0 | 3 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 37 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 38 | 23 | 0 | 3 | 12 | 0 |  | 11 |  | 6.0 |  | 2 | 25.0 + TT | 5.0 - TT |  |
| 39 | 23 | 0 | 3 | 4.5 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 40 | 23 | 0 | 3 | 8 | 0 |  | 15 |  | 5.0 |  | 2 | 25.0 + TT | 10.0 – TT |  |
| 41 | 23 | 0 | 3 | 3.5 | 0 |  | 19.5 |  | 3.5 |  | 2 | 25.0 + TT | 16 - TT |  |
| 42 | 23 | 0 | 3 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 43 | 23 | 0 | 3 | 0 | 0 |  | 20.0 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 – TT |  |
| 44 | 23 | 0 | 3 | 12 | 0 |  | 11 |  | 6.0 |  | 2 | 25.0 + TT | 5.0 - TT |  |
| 45 | 23 | 0 | 3 | 4.5 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 46 | 23 | 0 | 3 | 3.5 | 0 |  | 19.5 |  | 3.5 |  | 2 | 25.0 + TT | 16 - TT |  |
| 47 | 23 | 0 | 3 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 48 | 23 | 0 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 49 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 50 | 23 | 0 | 6.5 | 6 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 51 | 23 | 0 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 52 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 53 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 54 | 23 | 0 | 6.5 | 12 | 0 |  | 11 |  | 6.0 |  | 2 | 25.0 + TT | 5.0 - TT |  |
| 55 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 56 | 23 | 0 | 6.5 | 8 | 0 |  | 15 |  | 5.0 |  | 2 | 25.0 + TT | 10.0 – TT |  |
| 57 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 58 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 59 | 23 | 0 | 6.5 | 12 | 0 |  | 11 |  | 6.0 |  | 2 | 25.0 + TT | 5.0 – TT |  |
| 60 | 23 | 0 | 6.5 | 4..5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 – TT |  |
| 61 | 23 | 0 | 6.5 | 8 | 0 |  | 15 |  | 5.0 |  | 2 | 25.0 + TT | 10.0 – TT |  |
| 62 | 23 | 0 | 6.5 | 3.5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 – TT |  |
| 63 | 23 | 0 | 6.5 | 5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Void NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2.4	Configured transmitted power

6.2.4.1	Test purpose

To verify the measured UE configured maximum output power PUMAX,f,c is within the specified bounds.

6.2.4.2	Test applicability

This test case applies to all types of NR Power Class 1 release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

6.2.4.3	Minimum conformance requirements

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc) + ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass }

where

PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PPowerClass is the maximum UE power specified in Table 6.2.1.3-1 without taking into account the tolerance specified in the Table 6.2.1.3-1;

When the IE powerBoostPi2BPSK is set to 1, PEMAX,c is increased by +3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission when PEMAX,c ≥ 20 dBm (The exact evaluation period is no less than one radio frame).

When the IE powerBoostPi2BPSK is set to 1, ΔPPowerClass = -3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission.

ΔPPowerClass =

-	3 dB for a power class 2 UE or 6 dB for a power class 1.5 UE when P-max of 23 dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3 dB for a power class 1.5 UE when P-max of between 23 dBm and 26 dB is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1/2 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3dB when the UE is configured with SUL configurations and the requirements of default power class are applied as specified in sub-clause 6.2C.1.3 on the band where UE indicates power class 2;

-	3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC2 UE with txDiversity-r16 capability or PC1.5 UE further indicates SRS-TxSwitch capability ‘t1r2’ or ‘t1r4’ or ‘t1r1-t1r2’ or ‘t1r1-t1r2-t1r4’;

-	0 dB otherwise;

∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 [4] clause  6.2B.4.2 for EN-DC; ∆TIB,c = 0 dB otherwise; In case the UE supports more than one of band combinations for V2X operating bands for concurrent operation, CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL in this specification and 6.2B.4.2 in TS 38.101-3 [4] for EN-DC, truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL in this specification and 6.2B.4.2 in TS 38.101-3 [4] for EN-DC, for the applicable operating bands.

∆TC,c = 1.5dB when NOTE 3 in Table 6.2.1.3-1 in 38.521-1 applies for a serving cell c, otherwise ∆TC,c = 0 dB ;

MPRc and A-MPRc for serving cell c are specified in clause 6.2.2.3 and clause 6.2.3.3, respectively;

∆MPRc for serving cell c is specified in clause 6.2.2.3.

∆TRxSRS is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ when

a)	UE transmits SRS on the second SRS resource in every configured SRS resource set when the SRS-TxSwitch capability is indicated as 't1r2' or 't1r1-t1r2'

b)	UE transmits SRS on the second, third and fourth SRS resources of the total 4 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the SRS-TxSwitch capability is indicated as 't1r4' or, 't1r4-t2r4' or 't1r1-t1r2-t1r4' or, 't1r1-t1r2-t2r2-t1r4-t2r4'

c)	UE transmits SRS from the second SRS port pair on the second SRS resource in every configured SRS resource set consisting of two SRS ports when the SRS-TxSwitch capability is indicated as ' t2r4' or ' t1r4-t2r4', or 't1r1-t1r2-t2r2-t2r4' or 't1r1-t1r2-t2r2-t1r4-t2r4', or

d)	UE transmits SRS to a DL-only carrier

The value of ∆TRxSRS is 4.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16..

The value of ∆TRxSRS is 7.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is capable of power class 2 in the band and ΔPPowerClass = 0 dB and not indicating txDiversity-r16.

For other SRS transmissions ∆TRxSRS is zero;

P-MPRc is the allowed maximum output power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self defence requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the eNB the available maximum output transmit power. This information can be used by the eNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

TREF and Teval are specified in Table 6.2.4.3-1. For each TREF, the PCMAX,L,c for serving cell c are evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L,f,c over one or more Teval is then applied for the entire TREF

Table 6.2.4.3-1: Evaluation and reference periods for Pcmax

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| Physical channel length | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

The measured configured maximum output power PUMAX,f,c shall be within the following bounds:

PCMAX_L,f,c – MAX{TL,c, T(PCMAX_L,f,c)} ≤ PUMAX,f,c ≤ PCMAX_H,f,c + T(PCMAX_H,f,c).

where the tolerance T(PCMAX,f,c) for applicable values of PCMAX,f,c is specified in Table 6.2.4.3-2. The tolerance TL,c is the absolute value of the lower tolerance for the applicable operating band as specified in Table 6.2.1.3-1.

Table 6.2.4.3-2: PCMAX tolerance

| PCMAX,f,c (dBm) | Tolerance T(PCMAX,f,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.4.

6.2.4.4	Test description

6.2.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range (NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| 33 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: A power class 3 UE operating in TDD mode with PI/2 PBSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.4.4.3.

6.2.4.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step to ensure that the UE reaches the Pumax level of the test point.

3.	Measure the mean power of the UE in the channel bandwidth for each test point in table 6.2.4.5-1 according to the test configuration from table 6.2.4.4.1-1. The period of measurement shall be at least the continuous duration of one active slot and in the uplink symbols. For TDD slots with transient periods are not under test.

6.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.2.4.4.3-0: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2.4.4.3-1: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2.4.4.3-2: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

Table 6.2.4.4.3-3: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 |  |  |

Table 6.2.4.4.3-4: FrequencyInfoUL-SIB: Test point 4

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  |  |

Table 6.2.4.4.3-5: ServingCellConfig

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 0 |  | Test ID 1, 2 |
|  | 1 |  | Test ID 3 |
| } |  |  |  |
| } |  |  |  |

6.2.4.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2.4.5-1.

Table 6.2.4.5-1: PCMAX configured UE output power

|  | Maximum output power |  |
| --- | --- | --- |
|  | Test ID 1,2 | Test ID 3 |
| Measured UE output power test point 1 | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) |
| Measured UE output power test point 2 | 10 dBm ± (6+TT) | 10 dBm ± (6+TT) |
| Measured UE output power test point 3 | 15 dBm ± (5+TT) | 15 dBm ± (5+TT) |
| Measured UE output power test point 4 | Note 3 | Note 4 |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. Note 2: Power class 3 is default power class unless otherwise stated. Note 3: The maximum output power shall be within the range in Table 6.2.4.5-1a. Note 4: The maximum output power shall be within the range in Table 6.2.4.5-1b. |  |  |

Table 6.2.4.5-1a: Measured UE output power test point 4 for Test ID 1,2

| NR band | Tolerance (dB) |
| --- | --- |
| n1 | 20 dBm ±(2.5+TT) |
| n2 | 20 dBm ±(2.5+TT) |
| n3 | 20 dBm ±(2.5+TT) |
| n5 | 20 dBm ±(2.5+TT) |
| n7 | 20 dBm ±(2.5+TT) |
| n8 | 20 dBm ±(2.5+TT) |
| n12 | 20 dBm ±(2.5+TT) |
| n14 | 20 dBm ±(2.5+TT) |
| n20 | 20 dBm ±(2.5+TT) |
| n24 | 20 dBm + 2.5+TT/-3-TT |
| n25 | 20 dBm ±(2.5+TT) |
| n26 | 20 dBm ±(2.51+TT) |
| n28 | 20 dBm ±(2.5+TT) |
| n30 | 20 dBm ±(2.5+TT) |
| n34 | 20 dBm ±(2.5+TT) |
| n38 | 20 dBm ±(2.5+TT) |
| n39 | 20 dBm ±(2.5+TT) |
| n40 | 20 dBm ±(2.5+TT) |
| n41 | 20 dBm ±(2.5+TT) |
| n50 | 20 dBm ±(2.5+TT) |
| n51 | 20 dBm ±(2.5+TT) |
| n54 | 20 dBm ±(2.5+TT) |
| n65 | 20 dBm ±(2.5+TT) |
| n66 | 20 dBm ±(2.5+TT) |
| n70 | 20 dBm ±(2.5+TT) |
| n71 | 20 dBm ±(2.5+TT) |
| n74 | 20 dBm ±(2.5+TT) |
| n77 | 20 dBm + 2.5+TT/-3-TT |
| n78 | 20 dBm + 2.5+TT/-3-TT |
| n79 | 20 dBm + 2.5+TT/-3-TT |
| NOTE 1: Void NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. |  |

Table 6.2.4.5-1b: Measured UE output power test point 4 for Test ID 3

| NR band | Tolerance (dB) |
| --- | --- |
| n40 | 23 dBm +2+TT/-2-TT |
| n41 | 23 dBm +2+TT/-2-TT |
| n77 | 23 dBm +2+TT/-3-TT |
| n78 | 23 dBm +2+TT/-3-TT |
| n79 | 23 dBm +2+TT/-3-TT |
| NOTE 1: Void NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. |  |

Table 6.2.4.5-2: Test Tolerance (Configured transmitted power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

## 6.2D	Transmitter power for UL MIMO

### 6.2D.1	UE maximum output power for UL MIMO

Editor’s Note:

- No test points are defined for 2-layer UL MIMO since there is no CP-OFDM configuration satisfying MPR=0dB requirements in TS 38.101-1. Testing with minimum MPR has been covered in 6.2D.2.

- No test points are defined for Power Class 1.5 non-FWA UEs supporting UFPTx since there is no configuration satisfying MPR=0dB requirements in TS 38.101-1. Testing with minimum MPR has been covered in 6.2D.2.

- TP analysis needs to be updated regarding Note 3 in Table 6.2D.1.4.1-2

6.2D.1.1	Test purpose

To verify that the error of the UE maximum output power for UL MIMO does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2D.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 FWA UEs, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.2D.1.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1.3-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-31 | 23 | +2/-3 |  |  |
| n2 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n3 |  |  |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n5 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n7 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n8 |  |  |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n24 |  |  |  |  |  |  | 23 | +2/-41 |  |  |
| n25 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n28 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n30 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n34 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n38 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n39 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n40 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n41 |  |  | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n66 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n70 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n71 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n77 |  |  | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 |  |  | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 |  |  | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n97 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 2: Power class 3 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.1.3-2: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Number of layers | TPMI index |
| --- | --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | 2 | 0 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. |  |  |  |

For UEs supporting uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power requirements specified in Table 6.2D.1.3-1 shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

Table 6.2D.1.3-3: PUSCH Configuration for uplink full power transmission (ULFPTx)

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 | 2 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0 or 1NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0,1 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [9]. NOTE 3: For PUSCH configured with ULFPTx Mode set to Mode-1, all the transmitter requirement for CP-OFDM based modulation is not needed to be verified if the requirement for UL MIMO has been validated. |  |  |  |  |  |  |

If the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2 apply for at least one antenna connector for the power class as indicated by the ue-PowerClass field in capability signalling with the following exception: for UEs indicating txDiversity-r16, the requirements in clause 6.2G for the power class indicated by the ue-PowerClass.

A UE indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2 for at least one antenna connector when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission on a single antenna port.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.1.

6.2D.1.4	Test description

6.2D.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2D.1.4.1-1 and Table 6.2D.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.1.4.1-1: Test Configuration Table for 2-layer UL MIMO

NOTE:	No test points are defined since there is no CP-OFDM configuration satisfying MPR=0dB requirements in TS 38.101-1.

Table 6.2D.1.4.1-2: Test Configuration Table for uplink full power transmission (ULFPTx) for Power Class 3, Power Class 2 and Power Class 1.5 FWA UEs

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Only Test IDs 1, 2 and 3 apply to Power Class 2 UEs if TxD is indicated. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.1.4.1-1 and Table 6.2D.1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.1.4.3.

6.2D.1.4.2	Test procedure

Sub-test for 2-Layer codebook based UL-MIMO:

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

Sub-test for ULFPTx

4.	If the UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.1.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on the UE supported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

6.2D.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.1.5	Test requirement

The maximum output power, derived in step 3 or step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.1.5-1.

Table 6.2D.1.5-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-3-TT |  |  |
| n2 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n3 |  |  |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n5 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n7 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n8 |  |  |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n24 |  |  |  |  |  |  | 23 | +2+TT/-41-TT |  |  |
| n25 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n28 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n30 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n34 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n38 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n39 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n40 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n41 |  |  | 29 | _+2+TT/-31-TT | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n48 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n66 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n70 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n71 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n77 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n78 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n79 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n97 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.1.5-2 NOTE 3: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.1.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2D.1_1	UE maximum output power for SUL with UL MIMO

6.2D.1_1.1	Test purpose

Same test purpose as in clause 6.2D.1.1.

6.2D.1_1.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support SUL and UL full power transmission (ULFPTx) operating on the SUL bands.

6.2D.1_1.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1_1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1_1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1 ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1_1.3-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n80 |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n84 |  |  | 26 | +2/-31 | 23 | +2/-3 |  |  |
| n95 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n97 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n98 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n99 |  |  |  |  | 23 | +2/-41 |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

Table 6.2D.1_1.3-2: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Number of layers | TPMI index |
| --- | --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | 2 | 0 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. |  |  |  |

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power requirements specified in Table 6.2D.1_1.3-1 shall be met with the PUSCH configurations specified in Table 6.2D.1_1.3-3, based upon UE’s support of uplink full power transmission mode.

Table 6.2D.1_1.3-3: PUSCH Configuration for uplink full power transmission (ULFPTx)

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 | 2 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0 or 1NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0,1 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. NOTE 3: For PUSCH configured with ULFPTxModes set to Mode-1, all the transmitter requirement for CP-OFDM based modulation is not needed to be verified if the requirement for UL MIMO has been validated. |  |  |  |  |  |  |

If the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2 apply for at least one antenna connector for the power class as indicated by the ue-PowerClass field in capability signalling with the following exception: for UEs indicating txDiversity-r16, the requirements in clause 6.2G for the power class indicated by the ue-PowerClass.

A UE indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2 for at least one antenna connector when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission on a single antenna port.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.2D.1.

6.2D.1_1.4	Test description

6.2D.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2D.1_1.4.1-1 and Table 6.2D.1_1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.1_1.4.1-1: Test Configuration Table for 2-layer UL MIMO on SUL

NOTE:	No test points are defined since there is no configuration satisfying MPR=0dB requirements in TS 38.101-1.

Table 6.2D.1_1.4.1-2: Test Configuration Table for uplink full power transmission on SUL(ULFPTx)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for NUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for NUL carrier |  |
| Test SCS as specified in Table 5.5C-1 |  |  | Lowest supported SCS for NUL carrier 15kHz for SUL carrier |  |
| Test Parameters |  |  |  |  |
| Test ID | DL config | UL config | SUL Configuration |  |
|  |  |  | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | N/A | N/A | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure 3.1.1.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2 and G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.1_1.4.1-1 and Table 6.2D.1_1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.1_1.4.3.

6.2D.1_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.1_1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.1_1.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

6.2D.1_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, and Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.1_1.5	Test requirement

The maximum output power, derived in step 3 or step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.1_1.5-1.

Table 6.2D.1_1.5-1: UE Power Class

| NR band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n80 |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n84 |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-3-TT |  |  |
| n95 |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n97 |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n98 |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n99 |  |  |  |  | 23 | +2+TT/-41-TT |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

Table 6.2D.1_1.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2D.2	UE maximum output power reduction for UL MIMO

6.2D.2.1	Test purpose

To verify that the power reduction of UE due to higher order modulations and transmit bandwidth configuration does not exceed the specified maximum power reduction.

6.2D.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

NOTE:	Test execution is not necessary if TS 38.521-1 6.5D.2.4.1 is executed.

6.2D.2.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1.3-1 is specified in Table 6.2.2.3-1 for PC3, Table 6.2D.2.3-1 for 2Tx PC2 when the UE does not indicate ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band and Table 6.2.2.3-2 for 2Tx PC2 when the UE indicates ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band, Table 6.2D.2.3-2 and Table 6.2D.2.3-3 for PC1.5 respectively. For UE power class 1.5, the allowed maximum power reduction (MPR) defined in Table 6.2D.2.3-3 is in accordance with the indicated modifiedMPR-Behavior specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both antenna connectors.

For UE support uplink full power transmission (ULFPTx) for UL MIMO except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16, the allowed MPR for the maximum output power in Table 6.2D.1.3-1 is specified in Table 6.2.2.3-1 for PC3, Table 6.2D.2.3-1 when TxD is indicated and Table 6.2.2.3-2 when TxD is not indicated for PC2 , Table 6.2D.2.3-2 and Table 6.2D.2.3-3 for PC1.5 respectively, and the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode. A UE indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the maximum output power requirement with MPR according to clause 6.2.2.3. When a UE that indicates PC1.5 for a given band is limited to PC2 by the rules in clause 6.2.1, the MPR requirements in Table 6.2.2-2 apply.

The same MPR requirements shall be applicable to UE with 1-layer UL MIMO transmission (either with or without ULPFTx) as with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of .

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2D.4.3 apply.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the corresponding requirements in clause 6.2D.1.3 apply for the power class as indicated by the ue-PowerClass field in capability signalling. A UE indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2 with MPR according to clause 6.2.2.3 for at least one antenna connector when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission on a single antenna port.

Table 6.2D.2.3-1: Maximum power reduction (MPR) for power class 2 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |

Table 6.2D.2.3-2: Maximum power reduction (MPR) for power class 1.5 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Table 6.2D.2.3-3: Maximum power reduction (MPR) for power class 1.5 FWA with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 1.5 | ≤ 0 |
|  | QPSK | ≤ 6.5 | ≤ 2 | ≤ 0 |
|  | 16 QAM | ≤ 6.5 | ≤ 3 | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Inner, outer and edge allocations are as defined in section 6.2.2 except for 	PC1.5 edge allocations which is for LCRB ≤ 4 RBs instead of LCRB ≤ 2 RBs for other power classes. The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.2.

6.2D.2.4	Test description

6.2D.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2D.2.4.1-1, Table 6.2D.2.4.1-1a, Table 6.2D.2.4.1-2, Table 6.2D.2.4.1-2a, Table 6.2D.2.4.1-3 and Table 6.2D.2.4.1-3a. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.2.4.1-1: Test Configuration Table for Power Class 3 UEs supporting 2-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

Table 6.2D.2.4.1-1a: Test Configuration Table for power class 3 UEs supporting ULFPTx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power Reduction (MPR) test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 14 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 24 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 34 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 44 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 19 ~ 32 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Table 6.2D.2.4.1-2: Test Configuration Table for Power Class 2 UEs supporting 2-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

Table 6.2D.2.4.1-2a: Test Configuration Table for power class 2 UEs supporting ULFPTx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 19 ~ 32 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. |  |  |  |  |

Table 6.2D.2.4.1-3: Test Configuration Table for Power Class 1.5 UEs supporting 2-layer codebook based UL MIMO

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

Table 6.2D.2.4.1-3a: Test Configuration Table for power class 1.5 UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.2.4.1-1, Table 6.2D.2.4.1-2 or Table 6.2D.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.2.4.3.

6.2D.2.4.2	Test procedure

Sub-test for 2-Layer codebook based UL-MIMO

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE from both transmit antenna connectors in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbol with transient periods are not under test.

Sub-test for ULFPTx

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.2.4.1-1a, Table 6.2D.2.4.1-2a or Table 6.2D.2.4.1-3a. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2D.2.4.1-1a, Table 6.2D.2.4.1-2a or Table 6.2D.2.4.1-3a, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2D.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1, Table 6.2D.2.5-2, Table 6.2D.2.5-2a, Table 6.2D.2.5-3, or Table 6.2D.2.5-4. The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1a, Table 6.2D.2.5-2b, Table 6.2D2.5-2c, Table 6.2D.2.5-3a, or Table 6.2D.2.5-4a.

Table 6.2D.2.5-1: UE Power Class test requirements (for Band n1, n2, n3, n5, n7, n8, n24, n25, n28, n30, n34, n38, n39, n40, n41, n48, n66, n70, n71, n77, n78, n79, n97) for Power Class 3 UEs supporting 2-layer codebook based UL MIMO

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 (44) | 25.0 + TT | 16.5 - TT | （15.02 - TT） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n25, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. NOTE 4: Applicable for band n24. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-1a: UE Power Class test requirements (for Band n1, n2, n3, n5, n7, n8, n24, n25, n28, n30, n34, n38, n41, n48, n66, n70, n71, n77, n78, n79) for Power Class 3 supporting ULFPTx

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 (44) | 25.0 + TT | 20.0 - TT (19.0 – TT5) | （16.52 - TT） |
| 2 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 4 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 (44) | 25.0 + TT | 17.5 - TT | （16.02 - TT） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 (44) | 25.0 + TT | 20.0 - TT (19.0 – TT5) | （16.52 - TT） |
| 6 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 7 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 8 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 9 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 (44) | 25.0 + TT | 17.0 - TT | （15.52 - TT） |
| 10 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 11 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 12 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 13 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 15.5 - TT | （14.02 - TT） |
| 14 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 15.5 - TT | （14.02 - TT） |
| 15 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （14.02 - TT） |
| 16 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 17 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 18 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 (44) | 25.0 + TT | 13.5 - TT | （12.02 - TT） |
| 19 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 (44) | 25.0 + TT | 16.5 - TT | （15.02 - TT） |
| 20 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 21 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 22 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 23 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 (44) | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 (44) | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 27 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 28 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 29 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 (44) | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 30 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 31 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 32 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | (6.02) | 3 (44) | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n25 and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-3. NOTE 4: Only applicable for band n24. NOTE 5: For band n24 when transmission bandwidths are not confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2: UE Power Class test requirements (for Bands n1, n3, n40, n41, n66, n70, n77, n78, n79) for Power Class 2 UEs supporting 2-layer codebook based UL MIMO without indicating ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 2 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 3 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 4 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 5 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.02 - TT） |
| 6 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 7 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 8 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 9 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 10 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 11 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 12 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 13 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 14 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For n1, n3, and Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2a: UE Power Class test requirements (for Bands n40, n41, n77, n78, n79) for Power Class 2 UEs supporting 2-layer codebook based UL MIMO and indicating ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.02 - TT） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 4 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 5 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 8 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 9 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 12 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| 13 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| 14 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2b: UE Power Class test requirements (for Bands n1, n3, n40, n41, n66, n70, n77, n78, n79) for Power Class 2 UEs supporting ULFPTx and ul-FullPwrMode1-TPMIGroup-r1 and indicating TxD

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.52- TT） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02- TT） |
| 4 | 26 | 0 | 1.0 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.52 - TT） |
| 5 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.02 - TT） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 8 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 9 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.02 - TT） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 12 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.02 - TT） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 15 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 16 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （14.02 - TT） |
| 17 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （14.02 - TT） |
| 18 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （14.02 - TT） |
| 19 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 20 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 21 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 22 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 23 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.02 - TT） |
| 24 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 25 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 26 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 27 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 28 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 29 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 30 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 31 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 32 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n1, n3, and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2c: UE Power Class test requirements (for Bands n1, n3, n40, n41, n66, n70, n77, n78, n79) for Power Class 2 UEs supporting ULFPTx and, (supporting ul-FullPwrMode1-TPMIGroup-r1 without indicating TxD) or (supporting ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.52 - TT） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02- TT） |
| 4 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.02 - TT） |
| 5 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.52 - TT） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 8 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.52 - TT） |
| 9 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.52 - TT） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 12 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02- TT） |
| 15 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.02 - TT） |
| 16 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 17 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 18 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （15.02 - TT） |
| 19 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.02 - TT） |
| 20 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 21 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 22 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 23 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 24 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 25 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 26 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.52 - TT） |
| 27 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 28 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 29 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.02 - TT） |
| 30 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| 31 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| 32 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-3: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 UEs

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.52 - TT） |
| 2 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 3 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 4 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | （20.02 - TT） |
| 5 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.02 - TT） |
| 6 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 7 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 8 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | （20.02 - TT） |
| 9 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 10 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 29 | 0 | 5 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 31.0 + TT | 21.0 - TT | （17.52 - TT） |
| 12 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 13 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| 14 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | （19.02） | 5.0 |  | 3 | 31.0 + TT | 15.5 - TT | （14.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n1, n3, and n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-3a: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | （27.02） | 3.0 |  | 3 | 31.0 + TT | 25.5 – TT | （24.02 - TT） |
| 2 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 | 31.0 + TT | 20.0 – TT | （18.52 - TT） |
| 3 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | (5.02) | 3 | 31.0 + TT | 20.0 – TT | （18.52 - TT） |
| 4 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | （25.52） | 3.0 |  | 3 | 31.0 + TT | 24.0 – TT | （22.52 - TT） |
| 5 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | （27.02） | 3.0 |  | 3 | 31.0 + TT | 25.5 – TT | （24.02 - TT） |
| 6 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.02 - TT） |
| 7 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.02 - TT） |
| 8 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | （25.02） | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | （22.02 - TT） |
| 9 | 29 | 0 | 1.5 | 0 | （1.52） | 27.5 | （26.02） | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | （23.02 - TT） |
| 10 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 11 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 12 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | （21.02 - TT） |
| 13 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 14 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 15 | 29 | 0 | 4 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | （20.52 - TT） |
| 16 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 17 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| 18 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-4: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 FWA UEs

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | （26.03） | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | （23.03 - TT） |
| 2 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 3 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 4 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | （23.53） | 3.0 |  | 3 | 31.0 + TT | 22.0 – TT | （20.53 - TT） |
| 5 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | （25.53） | 3.0 |  | 3 | 31.0 + TT | 24.0 – TT | （22.53 - TT） |
| 6 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 7 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 8 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | （23.53） | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | （20.53 - TT） |
| 9 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 10 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 11 | 29 | 0 | 4.5 | 0 | （1.53） | 24.5 | （23.03） | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | （20.03 - TT） |
| 12 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | （20.03） | 5.0 |  | 3 | 31.0 + TT | 16.5 - TT | （15.03 - TT） |
| 13 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | （20.03） | 5.0 |  | 3 | 31.0 + TT | 16.5 - TT | （15.03 - TT） |
| 14 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | （20.03） | 5.0 |  | 3 | 31.0 + TT | 16.5 - TT | （15.03 - TT） |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-4a: UL MIMO MPR test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 FWA UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | （27.53） | 3.0 |  | 3 | 31.0 + TT | 26.0 – TT | （24.53 - TT） |
| 2 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | （21.53） | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 – TT | （18.53 - TT） |
| 3 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | （21.53） | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 – TT | （18.53 - TT） |
| 4 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | （26.03） | 3.0 |  | 3 | 31.0 + TT | 24.5 – TT | （23.03 - TT） |
| 5 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | （27.53） | 3.0 |  | 3 | 31.0 + TT | 26.0 – TT | （24.53 - TT） |
| 6 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 7 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 8 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | （25.53） | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | （22.53 - TT） |
| 9 | 29 | 0 | 1 | 0 | （1.53） | 28.0 | （26.53） | 3.0 |  | 3 | 31.0 + TT | 25.0 - TT | （23.53 - TT） |
| 10 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 11 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 12 | 29 | 0 | 3 | 0 | （1.53） | 26.0 | （24.53） | 3.0 |  | 3 | 31.0 + TT | 23.0 - TT | （21.53 - TT） |
| 13 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 14 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 – TT | （16.03 - TT） |
| 15 | 29 | 0 | 3.5 | 0 | （1.53） | 25.5 | （24.03） | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | （21.03 - TT） |
| 16 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 17 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | （21.03） | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | （16.03 - TT） |
| 18 | 29 | 0 | 5.5 | 0 | （1.53） | 23.5 | （22.03） | 3.0 | (5.03) | 3 | 31.0 + TT | 20.5 - TT | （17.03 - TT） |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-5: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2D.2_1	UE maximum output power reduction for SUL with UL MIMO

6.2D.2_1.1	Test purpose

Same test purpose as in clause 6.2D.2.1

6.2D.2_1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5D.2.4.1_1 is executed.

6.2D.2_1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The minimum requirements are same as in 6.2D.1.3 with following exceptions:

Instead of Table 6.2D.1.3-1  use Table 6.2D.1_1.3-1

6.2D.2_1.4	Test description

6.2D.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in Table 6.2D.2_1.4.1-1 ~ 6.2D.2_1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.2_1.4-1: Test Configuration Table for Power Class 3

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range and high range for SUL carrier Mid range for Non-SUL carrier. |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  |  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 | Default |  |  |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  |  |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  |  |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2D.2_1.4.1-2: Test Configuration Table for Power Class 2

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range and high range for SUL carrier Mid range for Non-SUL carrier. |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  |  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 | Default |  |  |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  |  |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  |  |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.2_1.4.1-1 ~ Table 6.2D.2_1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.2_1.4.3.

6.2D.2_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.2_1.4.1-1 and Table 6.2D.2_1.4.1-2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE from both transmit antenna connectors in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbol with transient periods are not under test.

6.2D.2_1.4.3	Message contents

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Table 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL.

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.2_1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2_1.5-1~6.2D.2_1.5-2.

Table 6.2D.2_1.5-1: UE Power Class test requirements (for Band n80, n84, n95, n97) for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 3 | 25.0 + TT | 16.5 - TT | （14.02 - TT） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 3 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n80, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2D.2_1.5-1a: UE Power Class test requirements for Band n99 for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 |  | 4 | 25.0 + TT | 16.5 - TT | （15.02 - TT） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 4 | 25.0 + TT | 16.0 - TT | （14.52 - TT） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 5.0 |  | 4 | 25.0 + TT | 15.0 - TT | （13.52 - TT） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 4 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 4 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 4 | 25.0 + TT | 14.5 - TT | （13.02 - TT） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 4 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 4 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 4 | 25.0 + TT | 11.5 - TT | （9.02 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n80, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2_1.5-2: UE Power Class test requirements (for Band n80, n84, n95, n97) for Power Class 2

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.52 - TT） |
| 2 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 3 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 4 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （212） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （162 - TT） |
| 5 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （222） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （172 - TT） |
| 6 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 7 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT | （15.52 - TT） |
| 8 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （212） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （162 - TT） |
| 9 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （202） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （152 - TT） |
| 10 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （202） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （152 - TT） |
| 11 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （202） | 5.0 |  | 3 | 28.0 + TT | 16.5 - TT | （152 - TT） |
| 12 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 13 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| 14 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.52 - TT） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n80 and n84, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2_1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2_1.5-3: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2D.3	UE additional maximum output power reduction for UL MIMO

Editor’s Note:

- Supporting of ULFPTx is only completed for NS_04, NS_03, NS_03U.

6.2D.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3.2	Test applicability

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04 to all types of NR UE release 15 and forward that support UL MIMO.

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04 to all types of NR UE release 16 and forward that support UL full power transmission (ULFPTx).

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, and NS_100 to all types of NR Power Class 2 and 3 UE release 15 and forward.

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U to all types of NR Power Class 2 and 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

The requirements of this test apply in test case 6.5D.3.3 Additional Spurious Emissions for network signalling value NS_04, NS_47, NS_21, NS_56, NS_50 to all types of NR UE release 15 and forward that support UL-MIMO.

The requirements of this test apply in test case 6.5D.3_1.3 Additional Spurious Emissions for network signalling value NS_04 to all types of NR UE release 16 and forward that support UL full power transmission (ULFPTx).

NOTE:	Test execution is not necessary if 6.5D.2.3, 6.5D.2.4.2 and 6.5D.3.3 are executed.

6.2D.3.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in subclause 6.2.3.3 shall apply to the maximum output power specified in Table 6.2D.1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the A-MPR values specified in clause 6.2.3.3 shall apply to the maximum output power specified in Table 6.2D.1.3-1. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2D.4.3 apply.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.3.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.3.

6.2D.3.4	Test description

6.2D.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2D.3.4.1-1 to 6.2D.3.4.1-11. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.2D.3.4.1-1: Test Configuration table for NS_04

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 2 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 3 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 4 | Low |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 5 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 6 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 7 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 9 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 10 | Low |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 11 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 12 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 13 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 14 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 16 | Low |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 17 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 18 | High |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 19 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 20 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 22 | Low |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| 23 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-1a: Test Configuration table for NS_04 with supporting ULFPTx

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 (Note 3, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 3 (Note 3) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 4 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 5 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 6 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 7 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 8 (Note 4, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 9 (Note 4) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 10 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 11 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 12 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 14 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 16 | Low |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 17 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 18 | High |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 22 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 23 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 25 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 26 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 27 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 28 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 29 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 30 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 31 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 32 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 33 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 34 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 35 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 36 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 37 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 38 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 39 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 40 | Low |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 41 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 42 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 43 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 44 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 45 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 46 | Low |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 47 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 48 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 49 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 50 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 51 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 52 | Low |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 53 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 54 | High |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 55 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 56 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 57 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 58 | Low |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| 59 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 60 | High |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Only applicable for 10 MHz and 15 MHz channel bandwidth |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-2: Test Configuration table for NS_35

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_35 |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 2 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Default | N/A | CP-OFDM QPSK | Outer Full |
| 4 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 5 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 6 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 7 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 8 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 9 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 10 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 11 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 12 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |

Table 6.2D.3.4.1-3: Test Configuration table for NS_05

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |  |
| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 1 | 1922.5 | 5 | 15 |  | A3 | CP-OFDM | QPSK |  | Outer_Full |  |  |  |
| 2 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 3 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 42@10 | 18@5 |  | 8@3 |
| 4 | 1925 | 10 | Default |  | A2 |  | QPSK |  | 6@40 | 3@20 |  | 1@10 |
| 5 | 1935 | 10 | Default |  | A4 |  | QPSK |  | Outer_Full |  |  |  |
| 6 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 7 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 60@19 | 28@10 |  | 12@5 |
| 8 | 1927.5 | 15 | Default |  | A2 |  | QPSK |  | 6@56 | 3@28 |  | 1@14 |
| 9 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 10 | 1932.5 | 15 | Default |  | A2 |  | QPSK |  | 6@68 | 3@34 |  | 1@17 |
| 11 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |  |
| 12 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 13 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 78@28 | 37@14 |  | 17@7 |
| 14 | 1930 | 20 | Default |  | A2 |  | QPSK |  | 6@76 | 3@38 |  | 1@19 |
| 15 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |  |
| 16 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |  |
| 17 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 18 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 42@10 | 18@5 |  | 8@3 |
| 19 | 1925 | 10 | Default |  | A2 |  | 16 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 20 | 1935 | 10 | Default |  | A4 |  | 16 QAM |  | Outer_Full |  |  |  |
| 21 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 22 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 60@19 | 28@10 |  | 12@5 |
| 23 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 24 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 25 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@68 | 3@34 |  | 1@17 |
| 26 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM |  | Outer_Full |  |  |  |
| 27 | 1930 | 20 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 28 | 1930 | 20 | Default |  | A7 |  | 16 QAM |  | 78@28 | 37@14 |  | 17@7 |
| 29 | 1930 | 20 | Default |  | A2 |  | 16 QAM |  | 6@76 | 3@38 |  | 1@19 |
| 30 | 1950 | 20 | Default |  | A6 |  | 16 QAM |  | Outer_Full |  |  |  |
| 31 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM |  | Outer_Full |  |  |  |
| 32 | 1925 | 10 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 33 | 1925 | 10 | Default |  | A7 |  | 64 QAM |  | 42@10 | 18@5 |  | 8@3 |
| 34 | 1925 | 10 | Default |  | A2 |  | 64 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 35 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 36 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM |  | 60@19 | 28@10 |  | 12@5 |
| 37 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 38 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 39 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@68 | 3@34 |  | 1@17 |
| 40 | 1930 | 20 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 41 | 1930 | 20 | Default |  | A7 |  | 64 QAM |  | 78@28 | 37@14 |  | 17@7 |
| 42 | 1930 | 20 | Default |  | A2 |  | 64 QAM |  | 6@76 | 3@38 |  | 1@19 |
| 43 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM |  | Outer_Full |  |  |  |
| 44 | 1925 | 10 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 45 | 1925 | 10 | Default |  | A7 |  | 256 QAM |  | 42@10 | 18@5 |  | 8@3 |
| 46 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 47 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM |  | 60@19 | 28@10 |  | 12@5 |
| 48 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 49 | 1930 | 20 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 50 | 1930 | 20 | Default |  | A7 |  | 256 QAM |  | 78@28 | 37@14 |  | 17@7 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-4: Test Configuration table for NS_05U

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |  |
| A-MPR test parameters for NS_05U |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |  |
| 1 | 1922.5 | 5 | 15 |  | A3 | CP-OFDM | QPSK |  | Outer_Full |  |  |  |
| 2 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 3 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 42@10 | 18@5 |  | 8@3 |
| 4 | 1925 | 10 | Default |  | A2 |  | QPSK |  | 6@40 | 3@20 |  | 1@10 |
| 5 | 1935 | 10 | Default |  | A4 |  | QPSK |  | Outer_Full |  |  |  |
| 6 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 7 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 60@19 | 28@10 |  | 12@5 |
| 8 | 1927.5 | 15 | Default |  | A2 |  | QPSK |  | 6@56 | 3@28 |  | 1@14 |
| 9 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 10 | 1932.5 | 15 | Default |  | A2 |  | QPSK |  | 6@68 | 3@34 |  | 1@17 |
| 11 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |  |
| 12 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |
| 13 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 78@28 | 37@14 |  | 17@7 |
| 14 | 1930 | 20 | Default |  | A2 |  | QPSK |  | 6@76 | 3@38 |  | 1@19 |
| 15 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |  |
| 16 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |  |
| 17 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 18 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 42@10 | 18@5 |  | 8@3 |
| 19 | 1925 | 10 | Default |  | A2 |  | 16 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 20 | 1935 | 10 | Default |  | A4 |  | 16 QAM |  | Outer_Full |  |  |  |
| 21 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 22 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 60@19 | 28@10 |  | 12@5 |
| 23 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 24 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 25 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@68 | 3@34 |  | 1@17 |
| 26 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM |  | Outer_Full |  |  |  |
| 27 | 1930 | 20 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |
| 28 | 1930 | 20 | Default |  | A7 |  | 16 QAM |  | 78@28 | 37@14 |  | 17@7 |
| 29 | 1930 | 20 | Default |  | A2 |  | 16 QAM |  | 6@76 | 3@38 |  | 1@19 |
| 30 | 1950 | 20 | Default |  | A6 |  | 16 QAM |  | Outer_Full |  |  |  |
| 31 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM |  | Outer_Full |  |  |  |
| 32 | 1925 | 10 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 33 | 1925 | 10 | Default |  | A7 |  | 64 QAM |  | 42@10 | 18@5 |  | 8@3 |
| 34 | 1925 | 10 | Default |  | A2 |  | 64 QAM |  | 6@40 | 3@20 |  | 1@10 |
| 35 | 1935 | 10 | Default |  | A4 |  | 64 QAM |  | Outer_Full |  |  |  |
| 36 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 37 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM |  | 60@19 | 28@10 |  | 12@5 |
| 38 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@56 | 3@28 |  | 1@14 |
| 39 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 40 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@68 | 3@34 |  | 1@17 |
| 41 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM |  | Outer_Full |  |  |  |
| 42 | 1930 | 20 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |
| 43 | 1930 | 20 | Default |  | A7 |  | 64 QAM |  | 78@28 | 37@14 |  | 17@7 |
| 44 | 1930 | 20 | Default |  | A2 |  | 64 QAM |  | 6@76 | 3@38 |  | 1@19 |
| 45 | 1950 | 20 | Default |  | A6 |  | 64 QAM |  | Outer_Full |  |  |  |
| 46 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM |  | Outer_Full |  |  |  |
| 47 | 1925 | 10 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 48 | 1925 | 10 | Default |  | A7 |  | 256 QAM |  | 42@10 | 18@5 |  | 8@3 |
| 49 | 1935 | 10 | Default |  | A4 |  | 256 QAM |  | Outer_Full |  |  |  |
| 50 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 51 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM |  | 60@19 | 28@10 |  | 12@5 |
| 52 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 53 | 1942.5 | 15 | Default |  | A5 |  | 256 QAM |  | Outer_Full |  |  |  |
| 54 | 1930 | 20 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |
| 55 | 1930 | 20 | Default |  | A7 |  | 256 QAM |  | 78@28 | 37@14 |  | 17@7 |
| 56 | 1950 | 20 | Default |  | A6 |  | 256 QAM |  | Outer_Full |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-5: Test Configuration table for NS_48

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  |  |  |
| 2 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 3 | Default | 30 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 5 | Default | 40 | Default |  |  | QPSK | 16@0 (A2) |  | 8@0 (A2) | 4@0 (A2) |
| 6 | Default | 40 | Default |  |  | QPSK | 95@0 (A3) |  | 48@0 (A3) | 24@0 (A3) |
| 7 | Default | 40 | Default |  |  | QPSK | 152@0 (A4) |  | 76@0 (A4) | 38@0 (A4) |
| 8 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) |  | 96@0 (A2) | 48@0 (A2) |
| 9 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) |  | 2@94 (A3) | 1@47 (A3) |
| 10 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 11 | Default | 50 | Default |  |  | QPSK | 34@0 (A2) |  | 18@0 (A2) | 9@0 (A2) |
| 12 | Default | 50 | Default |  |  | QPSK | 115@0 (A4) |  | 58@0 (A4) | 29@0 (A4) |
| 13 | Default | 50 | Default |  |  | QPSK | 228@0 (A2) |  | 114@0 (A2) | 57@0 (A2) |
| 14 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) |  | 2@112 (A5) | 1@56 (A5) |
| 15 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 16 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 17 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 18 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 19 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |  |
| 20 | Default | 40 | Default |  |  | 256 QAM | 16@0 (A2) |  | 8@0 (A2) | 4@0 (A2) |
| 21 | Default | 40 | Default |  |  | 256 QAM | 95@0 (A3) |  | 48@0 (A3) | 24@0 (A3) |
| 22 | Default | 40 | Default |  |  | 256 QAM | 152@0 (A4) |  | 76@0 (A4) | 38@0 (A4) |
| 23 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A2) |  | 96@0 (A2) | 48@0 (A2) |
| 24 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A3) |  | 2@94 (A3) | 1@47 (A3) |
| 25 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 26 | Default | 50 | Default |  |  | 256 QAM | 34@0 (A2) |  | 18@0 (A2) | 9@0 (A2) |
| 27 | Default | 50 | Default |  |  | 256 QAM | 115@0 (A4) |  | 58@0 (A4) | 29@0 (A4) |
| 28 | Default | 50 | Default |  |  | 256 QAM | 228@0 (A2) |  | 114@0 (A2) | 57@0 (A2) |
| 29 | Default | 50 | Default |  |  | 256 QAM | 5@223 (A5) |  | 2@112 (A5) | 1@56 (A5) |
| 30 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-6: Test Configuration table for NS_49

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  |  |  |
| 2 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 3 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 5 | Default | 30 | Default |  |  | QPSK | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 6 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 7 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 8 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 9 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 10 | Default | 40 | Default |  |  | QPSK | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 11 | Default | 40 | Default |  |  | QPSK | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |
| 12 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 13 | Default | 40 | Default |  |  | QPSK | 159@33 (A2) |  | 79@17 (A2) | 39@9 (A2) |
| 14 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 15 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 16 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 17 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 18 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 19 | Default | 50 | Default |  |  | QPSK | 175@45 (A2) |  | 87@23 (A2) | 43@12 (A2) |
| 20 | Default | 50 | Default |  |  | QPSK | 220@0 (A1) |  | 110@0 (A1) | 55@0 (A1) |
| 21 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 22 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 23 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 24 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |  |
| 25 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 26 | Default | 30 | Default |  |  | 256 QAM | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 27 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 28 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 29 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 30 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |  |
| 31 | Default | 40 | Default |  |  | 256 QAM | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 32 | Default | 40 | Default |  |  | 256 QAM | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |
| 33 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 34 | Default | 40 | Default |  |  | 256 QAM | 159@33 (A2) |  | 79@17 (A2) | 39@9 (A2) |
| 35 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 36 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 37 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 38 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 39 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 40 | Default | 50 | Default |  |  | 256 QAM | 175@45 (A2) |  | 87@23 (A2) | 43@12 (A2) |
| 41 | Default | 50 | Default |  |  | 256 QAM | 220@0 (A1) |  | 110@0 (A1) | 55@0 (A1) |
| 42 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-7: Test Configuration table for NS_03, NS_03U and NS_100

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_03, NS_03U and NS_100 |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation |  | RB allocation (Note 1) |
| 1 | Low | Default | Default | N/A for A-MPR test cases | CP- OFDM | QPSK | Edge_1RB_Left |
| 2 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 3 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 4 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 5 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 6 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 7 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 8 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 9 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 10 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 11 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |
| 12 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-7a: Test Configuration Table for NS_03, NS_03U and NS_100 for UEs supporting ULFPTx

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |
| Test parameters for NS_03U |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Low | Default | Default | N/A | DFT-s OFDM | PI/2 BPSK | Edge_1RB_Left |
| 2 | High | Default | Default |  |  | PI/2 BPSK | Edge_1RB_Right |
| 3 | Default | Default | Default |  |  | PI/2 BPSK | Outer_Full |
| 4 | Low | Default | Default |  |  | QPSK | Edge_1RB_Left |
| 5 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 6 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 7 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 8 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 9 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 10 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 11 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 12 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 13 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 14 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |
| 15 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| 16 | Low | Default | Default |  | CP-s OFDM | QPSK | Edge_1RB_Left |
| 17 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 18 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 19 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 20 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 21 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 22 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 23 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 24 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 25 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 26 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |
| 27 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-8: Test Configuration table for NS_46

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 15MHz, 20MHz, 25 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_46 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc | Ch BW | SCS |  | Uplink Configuration |  |  |  |  |  |
|  | (MHz) | (MHz) | (kHz) |  | Modulation |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  | (Note 2) |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default |  |  | QPSK | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 2 | Default | 25 | Default |  |  | 256QAM | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 3 | Default | 50 | Default |  |  | QPSK | Edge_1RB_Left (A4) |  |  |  |
| 4 | Default | 50 | Default |  |  | 64QAM | Edge_1RB_Left (A4) |  |  |  |
| 5 | Default | 50 | Default | Downlink Configuration | CP-OFDM | QPSK | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 6 | Default | 50 | Default |  |  | 64QAM | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 7 | Default | 50 | Default |  |  | QPSK | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 8 | Default | 50 | Default |  |  | 64QAM | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 9 | Default | 50 | Default |  |  | QPSK | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 10 | Default | 50 | Default |  |  | 256QAM | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 11 | Default | 50 | Default |  |  | QPSK | Outer_Full (A8) |  |  |  |
| 12 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A8) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-9: Test Configuration table for NS_21

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| A-MPR test parameters for NS_21 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink | Uplink Configuration |  |  |  |  |  |
|  |  |  |  | Configuration | Modulation |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  | (Note 2) |  |  | SCS 15 kHz | SCS 30 kHz |  |
| 1 | Low | Default | Default |  |  | QPSK |  | Edge_1RB_Left |  |  |
| 2 | High | Default | Default |  |  | QPSK |  | Edge_1RB_Right |  |  |
| 3 | Default | Default | Default |  |  | QPSK |  | Outer_Full |  |  |
| 4 | Default | 10 MHz | Default |  |  | QPSK |  | 4@0 |  | 2@0 |
| 5 | Default | 10 MHz | Default |  |  | QPSK |  | 4@48 |  | 2@22 |
| 6 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |  |  |
| 7 | High | Default | Default |  |  | 16 QAM |  | Edge_1RB_Right |  |  |
| 8 | Default | Default | Default | N/A | CP-OFDM | 16 QAM |  | Outer_Full |  |  |
| 9 | Default | 10 MHz | Default |  |  | 16 QAM |  | 4@0 |  | 2@0 |
| 10 | Default | 10 MHz | Default |  |  | 16 QAM |  | 4@48 |  | 2@22 |
| 11 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |  |  |
| 12 | High | Default | Default |  |  | 64 QAM |  | Edge_1RB_Right |  |  |
| 13 | Default | Default | Default |  |  | 64 QAM |  | Outer_Full |  |  |
| 14 | Default | 10 MHz | Default |  |  | 64 QAM |  | 4@0 |  | 2@0 |
| 15 | Default | 10 MHz | Default |  |  | 64 QAM |  | 4@48 |  | 2@22 |
| 16 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |  |  |
| 17 | High | Default | Default |  |  | 256 QAM |  | Edge_1RB_Right |  |  |
| 18 | Default | Default | Default |  |  | 256 QAM |  | Outer_Full |  |  |
| 19 | Default | 10 MHz | Default |  |  | 256 QAM |  | 4@0 |  | 2@0 |
| 20 | Default | 10 MHz | Default |  |  | 256 QAM |  | 4@48 |  | 2@22 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-10: Test Configuration table for NS_44

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Defined for each test ID |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Defined for each test ID |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_44 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc | Ch BW | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  | (MHz) | (MHz) | (kHz) |  | Modulation |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  | (Note 2) |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | 2602.5 | 25 | Default |  | CP-OFDM | 64 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 2 | 2602.5 | 25 | Default |  |  | 64 QAM | 39@94 (A3) |  | 19@46 (A3) | 9@22 (A3) |
| 3 | 2602.5 | 25 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 4 | 2602.5 | 25 | Default |  |  | 256 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 5 | 2602.5 | 25 | Default | N/A |  | 256 QAM | 39@94 (A3) |  | 19@46 (A3) | 9@22 (A3) |
| 6 | 2602.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 7 | 2600 | 30 | Default |  |  | 64 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 8 | 2600 | 30 | Default |  |  | 64 QAM | 69@91 (A3) |  | 34@44 (A3) | 17@21 (A3) |
| 9 | 2600 | 30 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 10 | 2600 | 30 | Default |  |  | 256 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 11 | 2600 | 30 | Default |  |  | 256 QAM | 69@91 (A3) |  | 34@44 (A3) | 17@21 (A3) |
| 12 | 2600 | 30 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 13 | 2595 | 40 | Default |  |  | 64 QAM | 16@0 (A1) |  | 8@0 (A1) | 4@0 (A1) |
| 14 | 2595 | 40 | Default |  |  | 64 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 15 | 2595 | 40 | Default |  |  | 64 QAM | 130@0 (A3) |  | 65@0 (A3) | 33@0 (A3) |
| 16 | 2595 | 40 | Default |  |  | 64 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 17 | 2595 | 40 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |  |
| 18 | 2595 | 40 | Default |  |  | 256 QAM | 16@0 (A1) |  | 8@0 (A1) | 4@0 (A1) |
| 19 | 2595 | 40 | Default |  |  | 256 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 20 | 2595 | 40 | Default |  |  | 256 QAM | 130@0 (A3) |  | 65@0 (A3) | 33@0 (A3) |
| 21 | 2595 | 40 | Default |  |  | 256 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 22 | 2595 | 40 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-11: Test Configuration table for NS_27

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies |  |  |  |  |  |  | Refer to uplink carrier centre frequency (Fc) in test parameters. Associated parameters defined in Table 6.2D.3.4.1-9a and 6.2D.3.4.1-9b. |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Refer to test parameters (15, 20, 40 MHz) |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |  |
| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc | Ch BW | SCS | Downlink | Uplink Configuration |  |  |  |  |  |
|  | (MHz) | (MHz) | (kHz) | Configuration | Modulation |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  | (Note 2) |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 3557.52 | 15 | Default |  |  | QPSK |  | Edge_1RB_Left (A3) |  |  |
| 2 | 3557.52 | 15 | Default |  |  | QPSK |  | Outer_Full (A3) |  |  |
| 3 | 3692.49 | 15 | Default |  |  | QPSK |  | Edge_1RB_Right (A3) |  |  |
| 4 | 3692.49 | 15 | Default |  |  | QPSK |  | Outer_Full (A3) |  |  |
| 5 | 3562.5 | 15 | Default |  |  | QPSK |  | Edge_1RB_Left (A4) |  |  |
| 6 | 3562.5 | 15 | Default |  |  | QPSK |  | Outer_Full (2) |  |  |
| 7 | 3687.48 | 15 | Default |  |  | QPSK |  | Edge_1RB_Right (A4) |  |  |
| 8 | 3687.48 | 15 | Default |  |  | QPSK |  | Outer_Full (2) |  |  |
| 9 | 3560.01 | 20 | Default |  |  | QPSK |  | Edge_1RB_Left (A5) |  |  |
| 10 | 3560.01 | 20 | Default |  |  | QPSK |  | Outer_Full (A5) |  |  |
| 11 | 3690 | 20 | Default |  |  | QPSK |  | Edge_1RB_Right (A5) |  |  |
| 12 | 3690 | 20 | Default |  |  | QPSK |  | Outer_Full (A5) |  |  |
| 13 | 3570 | 20 | Default |  |  | QPSK |  | Edge_1RB_Left (A6) |  |  |
| 14 | 3570 | 20 | Default |  |  | QPSK |  | Outer_Full (2) |  |  |
| 15 | 3679.98 | 20 | Default |  |  | QPSK |  | Edge_1RB_Right (A6) |  |  |
| 16 | 3679.98 | 20 | Default |  |  | QPSK |  | Outer_Full (2) |  |  |
| 17 | 3570 | 40 | Default | N/A |  | QPSK |  | Edge_1RB_Left (A7) |  |  |
| 18 | 3570 | 40 | Default |  |  | QPSK |  | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 19 | 3570 | 40 | Default |  |  | QPSK |  | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 20 | 3570 | 40 | Default |  |  | QPSK |  | Edge_1RB_Right (A7) |  |  |
| 21 | 3679.98 | 40 | Default |  |  | QPSK |  | Edge_1RB_Right (A7) |  |  |
| 22 | 3679.98 | 40 | Default |  |  | QPSK |  | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 23 | 3679.98 | 40 | Default |  |  | QPSK |  | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 24 | 3679.98 | 40 | Default |  |  | QPSK |  | Edge_1RB_Left (A7) |  |  |
| 25 | 3600 | 40 | Default |  |  | QPSK |  | Edge_1RB_Left (A8) |  |  |
| 26 | 3600 | 40 | Default |  |  | QPSK |  | Outer_Full (4.5) |  |  |
| 27 | 3649.98 | 40 | Default |  |  | QPSK |  | Edge_1RB_Right (A8) |  |  |
| 28 | 3649.98 | 40 | Default |  |  | QPSK |  | Outer_Full (4.5) |  |  |
| 29 | 3557.52 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A3) |  |  |
| 30 | 3557.52 | 15 | Default |  |  | 16 QAM |  | Outer_Full (A3) |  |  |
| 31 | 3692.49 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A3) |  |  |
| 32 | 3692.49 | 15 | Default |  |  | 16 QAM |  | Outer_Full (A3) |  |  |
| 33 | 3562.5 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A4) |  |  |
| 34 | 3562.5 | 15 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 35 | 3687.48 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A4) |  |  |
| 36 | 3687.48 | 15 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 37 | 3560.01 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A5) |  |  |
| 38 | 3560.01 | 20 | Default |  |  | 16 QAM |  | Outer_Full (A5) |  |  |
| 39 | 3690 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A5) |  |  |
| 40 | 3690 | 20 | Default |  |  | 16 QAM |  | Outer_Full (A5) |  |  |
| 41 | 3570 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A6) |  |  |
| 42 | 3570 | 20 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 43 | 3679.98 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A6) |  |  |
| 44 | 3679.98 | 20 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 45 | 3570 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A7) |  |  |
| 46 | 3570 | 40 | Default |  |  | 16 QAM |  | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 47 | 3570 | 40 | Default |  |  | 16 QAM |  | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 48 | 3570 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A7) |  |  |
| 49 | 3679.98 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A7) |  |  |
| 50 | 3679.98 | 40 | Default |  |  | 16 QAM |  | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 51 | 3679.98 | 40 | Default |  |  | 16 QAM |  | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 52 | 3679.98 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A7) |  |  |
| 53 | 3600 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A8) |  |  |
| 54 | 3600 | 40 | Default |  | CP-OFDM | 16 QAM |  | Outer_Full (4.5) |  |  |
| 55 | 3649.98 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A8) |  |  |
| 56 | 3649.98 | 40 | Default |  |  | 16 QAM |  | Outer_Full (4.5) |  |  |
| 57 | 3557.52 | 15 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A3) |  |  |
| 58 | 3557.52 | 15 | Default |  |  | 64 QAM |  | Outer_Full (A3) |  |  |
| 59 | 3692.49 | 15 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A3) |  |  |
| 60 | 3692.49 | 15 | Default |  |  | 64 QAM |  | Outer_Full (A3) |  |  |
| 61 | 3562.5 | 15 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A4) |  |  |
| 62 | 3562.5 | 15 | Default |  |  | 64 QAM |  | Outer_Full (2) |  |  |
| 63 | 3687.48 | 15 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A4) |  |  |
| 64 | 3687.48 | 15 | Default |  |  | 64 QAM |  | Outer_Full (2) |  |  |
| 65 | 3560.01 | 20 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A5) |  |  |
| 66 | 3560.01 | 20 | Default |  |  | 64 QAM |  | Outer_Full (A5) |  |  |
| 67 | 3690 | 20 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A5) |  |  |
| 68 | 3690 | 20 | Default |  |  | 64 QAM |  | Outer_Full (A5) |  |  |
| 69 | 3570 | 20 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A6) |  |  |
| 70 | 3570 | 20 | Default |  |  | 64 QAM |  | Outer_Full (2) |  |  |
| 71 | 3679.98 | 20 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A6) |  |  |
| 72 | 3679.98 | 20 | Default |  |  | 64 QAM |  | Outer_Full (2) |  |  |
| 73 | 3570 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A7) |  |  |
| 74 | 3570 | 40 | Default |  |  | 64 QAM |  | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 75 | 3570 | 40 | Default |  |  | 64 QAM |  | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 76 | 3570 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A7) |  |  |
| 77 | 3679.98 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A7) |  |  |
| 78 | 3679.98 | 40 | Default |  |  | 64 QAM |  | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 79 | 3679.98 | 40 | Default |  |  | 64 QAM |  | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 80 | 3679.98 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A7) |  |  |
| 81 | 3600 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A8) |  |  |
| 82 | 3600 | 40 | Default |  |  | 64 QAM |  | Outer_Full (4.5) |  |  |
| 83 | 3649.98 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A8) |  |  |
| 84 | 3649.98 | 40 | Default |  |  | 64 QAM |  | Outer_Full (4.5) |  |  |
| 85 | 3557.52 | 15 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A3) |  |  |
| 86 | 3557.52 | 15 | Default |  |  | 256 QAM |  | Outer_Full (A3) |  |  |
| 87 | 3692.49 | 15 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A3) |  |  |
| 88 | 3692.49 | 15 | Default |  |  | 256 QAM |  | Outer_Full (A3) |  |  |
| 89 | 3562.5 | 15 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A4) |  |  |
| 90 | 3562.5 | 15 | Default |  |  | 256 QAM |  | Outer_Full (2) |  |  |
| 91 | 3687.48 | 15 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A4) |  |  |
| 92 | 3687.48 | 15 | Default |  |  | 256 QAM |  | Outer_Full (2) |  |  |
| 93 | 3560.01 | 20 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A5) |  |  |
| 94 | 3560.01 | 20 | Default |  |  | 256 QAM |  | Outer_Full (A5) |  |  |
| 95 | 3690 | 20 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A5) |  |  |
| 96 | 3690 | 20 | Default |  |  | 256 QAM |  | Outer_Full (A5) |  |  |
| 97 | 3570 | 20 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A6) |  |  |
| 98 | 3570 | 20 | Default |  |  | 256 QAM |  | Outer_Full (2) |  |  |
| 99 | 3679.98 | 20 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A6) |  |  |
| 100 | 3679.98 | 20 | Default |  |  | 256 QAM |  | Outer_Full (2) |  |  |
| 101 | 3570 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A7) |  |  |
| 102 | 3570 | 40 | Default |  |  | 256 QAM |  | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 103 | 3570 | 40 | Default |  |  | 256 QAM |  | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 104 | 3570 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A7) |  |  |
| 105 | 3679.98 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A7) |  |  |
| 106 | 3679.98 | 40 | Default |  |  | 256 QAM |  | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 107 | 3679.98 | 40 | Default |  |  | 256 QAM |  | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 108 | 3679.98 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A7) |  |  |
| 109 | 3600 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A8) |  |  |
| 110 | 3600 | 40 | Default |  |  | 256 QAM |  | Outer_Full (4.5) |  |  |
| 111 | 3649.98 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A8) |  |  |
| 112 | 3649.98 | 40 | Default |  |  | 256 QAM |  | Outer_Full (4.5) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-11a: Test frequencies for NS_27 (SCS=15 kHz, ΔFRaster = 15kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 79 | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
|  |  | 3562.5 | 637500 | 3555.39 | 637026 |  |  | 7888 | 637344 | 6 | 0 | 1 (6) | 6 |
|  |  | 3687.48 | 645832 | 3680.37 | 645358 |  |  | 7975 | 645696 | 2 | 2 | 1 (6) | 8 |
|  |  | 3692.49 | 646166 | 3685.38 | 645692 |  |  | 7978 | 645984 | 4 | 2 | 0 (2) | 4 |
| 20 | 106 | 3560.01 | 637334 | 3550.47 | 636698 |  |  | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
|  |  | 3570 | 638000 | 3560.46 | 637364 |  |  | 7891 | 637632 | 4 | 0 | 0 (2) | 2 |
|  |  | 3679.98 | 645332 | 3670.44 | 644696 |  |  | 7968 | 645024 | 4 | 1 | 1 (6) | 7 |
|  |  | 3690 | 646000 | 3680.46 | 645364 |  |  | 7975 | 645696 | 8 | 1 | 1 (6) | 7 |
| 40 | 216 | 3570 | 638000 | 3550.56 | 636704 |  |  | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
|  |  | 3600 | 640000 | 3580.56 | 638704 |  |  | 7905 | 638976 | 8 | 0 | 0 (2) | 2 |
|  |  | 3649.98 | 643332 | 3630.54 | 642036 |  |  | 7940 | 642336 | 0 | 3 | 0 (2) | 5 |
|  |  | 3679.98 | 645332 | 3660.54 | 644036 |  |  | 7961 | 644352 | 4 | 0 | 1 (6) | 6 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-11b: Test frequencies for NS_27 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 38 | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
|  |  | 3562.5 | 637500 | 3555.66 | 637044 |  |  | 7888 | 637344 | 12 | 0 | 2 (2) | 4 |
|  |  | 3687.48 | 645832 | 3680.64 | 645376 |  |  | 7975 | 645696 | 8 | 0 | 3 (3) | 6 |
|  |  | 3692.49 | 646166 | 3685.65 | 645710 |  |  | 7978 | 645984 | 10 | 0 | 1 (1) | 2 |
| 20 | 51 | 3560.01 | 637334 | 3550.83 | 636722 |  |  | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
|  |  | 3570 | 638000 | 3560.82 | 637388 |  |  | 7891 | 637632 | 4 | 0 | 0 (0) | 0 |
|  |  | 3679.98 | 645332 | 3670.8 | 644720 |  |  | 7968 | 645024 | 16 | 0 | 2 (2) | 4 |
|  |  | 3690 | 646000 | 3680.82 | 645388 |  |  | 7975 | 645696 | 20 | 0 | 2 (2) | 4 |
| 40 | 106 | 3570 | 638000 | 3550.92 | 636728 |  |  | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
|  |  | 3600 | 640000 | 3580.92 | 638728 |  |  | 7905 | 638976 | 8 | 0 | 0 (0) | 0 |
|  |  | 3649.98 | 643332 | 3630.9 | 642060 |  |  | 7940 | 642336 | 12 | 0 | 1 (1) | 2 |
|  |  | 3679.98 | 645332 | 3660.9 | 644060 |  |  | 7961 | 644352 | 4 | 0 | 2 (2) | 4 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-12: Test Configuration table for NS_56

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Low | 5 | 15 | N/A for A-MPR testing | 14 | CP-OFDM | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 2 | Low | 5 | 30 |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | N/A |
| 3 | Low | 5 | Default |  | 6 |  | QPSK |  | Outer_Full | Outer_Full | N/A |
| 4 | Low | 5 | Default |  | 4 |  | QPSK |  | 21@4 | 8@3 | N/A |
| 5 | Low | 5 | Default |  | 4 |  | QPSK |  | 1@4 | 1@3 | N/A |
| 6 | Low | 5 | 15 |  | 14 |  | 16 QAM |  | Edge_1RB_Left | N/A | N/A |
| 7 | Low | 5 | 30 |  | 8 |  | 16 QAM |  | N/A | Edge_1RB_Left | N/A |
| 8 | Low | 5 | Default |  | 6 |  | 16 QAM |  | Outer_Full | Outer_Full | N/A |
| 9 | Low | 5 | Default |  | 4 |  | 16 QAM |  | 21@4 | 8@3 | N/A |
| 10 | Low | 5 | Default |  | 4 |  | 16 QAM |  | 1@4 | 1@3 | N/A |
| 11 | Low | 5 | 15 |  | 14 |  | 64 QAM |  | Edge_1RB_Left | N/A | N/A |
| 12 | Low | 5 | 30 |  | 8 |  | 64 QAM |  | N/A | Edge_1RB_Left | N/A |
| 13 | Low | 5 | Default |  | 6 |  | 64 QAM |  | Outer_Full | Outer_Full | N/A |
| 14 | Low | 5 | Default |  | 4 |  | 64 QAM |  | 21@4 | 8@3 | N/A |
| 15 | Low | 5 | Default |  | 4 |  | 64 QAM |  | 1@4 | 1@3 | N/A |
| 16 | Low | 5 | 15 |  | 14 |  | 256 QAM |  | Edge_1RB_Left | N/A | N/A |
| 17 | Low | 5 | 30 |  | 8 |  | 256 QAM |  | N/A | Edge_1RB_Left | N/A |
| 18 | Low | 5 | Default |  | 6 |  | 256 QAM |  | Outer_Full | Outer_Full | N/A |
| 19 | Low | 5 | Default |  | 4 |  | 256 QAM |  | 21@4 | 8@3 | N/A |
| 20 | Low | 5 | Default |  | 4 |  | 256 QAM |  | 1@4 | 1@3 | N/A |
| 21 | Low | 10 | 15 |  | 12 |  | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 22 | Low | 10 | Highest |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 23 | Low | 10 | Default |  | 8 |  | QPSK |  | Outer_Full | Outer_Full | Outer_Full |
| 24 | Low | 10 | Default |  | 6 |  | QPSK |  | 1@3 | 1@2 | 1@1 |
| 25 | Low | 10 | Default |  | 6 |  | QPSK |  | 43@9 | 19@5 | 8@2 |
| 26 | Low | 10 | Default |  | 4 |  | QPSK |  | 1@35 | 1@17 | 1@8 |
| 27 | Low | 10 | Default |  | 2 |  | QPSK |  | 17@35 | 5@19 | 3@8 |
| 28 | Low | 10 | Default |  | 5 |  | QPSK |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 29 | Low | 10 | 15 |  | 5 |  | QPSK |  | 1@40 | N/A | N/A |
| 30 | Low | 10 | 30 |  | 5 |  | QPSK |  | N/A | 1@20 | N/A |
| 31 | Low | 10 | 15 |  | 3 |  | QPSK |  | 8@44 | N/A | N/A |
| 32 | Low | 10 | 15 |  | 12 |  | 16 QAM |  | Edge_1RB_Left | N/A | N/A |
| 33 | Low | 10 | Highest |  | 8 |  | 16 QAM |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 34 | Low | 10 | Default |  | 8 |  | 16 QAM |  | Outer_Full | Outer_Full | Outer_Full |
| 35 | Low | 10 | Default |  | 6 |  | 16 QAM |  | 1@3 | 1@2 | 1@1 |
| 36 | Low | 10 | Default |  | 6 |  | 16 QAM |  | 43@9 | 19@5 | 8@2 |
| 37 | Low | 10 | Default |  | 4 |  | 16 QAM |  | 1@35 | 1@17 | 1@8 |
| 38 | Low | 10 | Default |  | 2 |  | 16 QAM |  | 17@35 | 5@19 | 3@8 |
| 39 | Low | 10 | Default |  | 5 |  | 16 QAM |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 40 | Low | 10 | 15 |  | 5 |  | 16 QAM |  | 1@40 | N/A | N/A |
| 41 | Low | 10 | 30 |  | 5 |  | 16 QAM |  | N/A | 1@20 | N/A |
| 42 | Low | 10 | 15 |  | 3 |  | 16 QAM |  | 8@44 | N/A | N/A |
| 43 | Low | 10 | 15 |  | 12 |  | 64 QAM |  | Edge_1RB_Left | N/A | N/A |
| 44 | Low | 10 | Highest |  | 8 |  | 64 QAM |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 45 | Low | 10 | Default |  | 8 |  | 64 QAM |  | Outer_Full | Outer_Full | Outer_Full |
| 46 | Low | 10 | Default |  | 6 |  | 64 QAM |  | 1@3 | 1@2 | 1@1 |
| 47 | Low | 10 | Default |  | 6 |  | 64 QAM |  | 43@9 | 19@5 | 8@2 |
| 48 | Low | 10 | Default |  | 4 |  | 64 QAM |  | 1@35 | 1@17 | 1@8 |
| 49 | Low | 10 | Default |  | 2 |  | 64 QAM |  | 17@35 | 5@19 | 3@8 |
| 50 | Low | 10 | Default |  | 5 |  | 64 QAM |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 51 | Low | 10 | 15 |  | 5 |  | 64 QAM |  | 1@40 | N/A | N/A |
| 52 | Low | 10 | 30 |  | 5 |  | 64 QAM |  | N/A | 1@20 | N/A |
| 53 | Low | 10 | 15 |  | 3 |  | 64 QAM |  | 8@44 | N/A | N/A |
| 54 | Low | 10 | 15 |  | 12 |  | 256 QAM |  | Edge_1RB_Left | N/A | N/A |
| 55 | Low | 10 | Highest |  | 8 |  | 256 QAM |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 56 | Low | 10 | Default |  | 8 |  | 256 QAM |  | Outer_Full | Outer_Full | Outer_Full |
| 57 | Low | 10 | Default |  | 6 |  | 256 QAM |  | 1@3 | 1@2 | 1@1 |
| 58 | Low | 10 | Default |  | 6 |  | 256 QAM |  | 43@9 | 19@5 | 8@2 |
| 59 | Low | 10 | Default |  | 4 |  | 256 QAM |  | 1@35 | 1@17 | 1@8 |
| 60 | Low | 10 | Default |  | 2 |  | 256 QAM |  | 17@35 | 5@19 | 3@8 |
| 61 | Low | 10 | Default |  | 5 |  | 256 QAM |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 62 | Low | 10 | 15 |  | 5 |  | 256 QAM |  | 1@40 | N/A | N/A |
| 63 | Low | 10 | 30 |  | 5 |  | 256 QAM |  | N/A | 1@20 | N/A |
| 64 | Low | 10 | 15 |  | 3 |  | 256 QAM |  | 8@44 | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-13: Test Configuration table for NS_50

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | (See Freq column for test frequency, and configuration detail specified in Table 6.2.3.4.1-32a, Table 6.2.3.4.1-32a and Table 6.2.3.4.1-32c) |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 25 MHz, 30MHz, 35MHz, 40MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |  |
| A-MPR test parameters for NS_50 (Power Class 3) |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |  |
| 1 | 1897.5 | 25 | Default | N/A | CP-OFDM | QPSK | Outer_Full (A7) |  |  |  |  |
| 2 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |  |  |
| 3 | 1897.5 | 25 | Default |  |  | QPSK | 20@0 (A9) |  | 10@0 (A9) |  | 5@0 (A9) |
| 4 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A7) |  |  |  |  |
| 5 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |  |  |
| 6 | 1900 | 30 | Default |  |  | QPSK | 20@0 (A9) |  | 10@0 (A9) |  | 5@0 (A9) |
| 7 | 1900 | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 8 | 1900 | 35 | Default |  |  | QPSK | 50@10 (A3) |  | 25@5 (A3) |  | 12@3 (A3) |
| 9 | 1900 | 35 | Default |  |  | QPSK | 178@10 (A2) |  | 87@5 (A2) |  | 41@3 (A2) |
| 10 | 1900 | 35 | Default |  |  | QPSK | 111@77 (A6) |  | 53@39 (A6) |  | 24@20 (A6) |
| 11 | 1900 | 35 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |  |
| 12 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 13 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) |  | 30@13 (A3) |  | 15@7 (A3) |
| 14 | 1900 | 40 | Default |  |  | QPSK | 191@25 (A2) |  | 93@13 (A2) |  | 44@7 (A2) |
| 15 | 1900 | 40 | Default |  |  | QPSK | 115@101 (A6) |  | 55@51 (A6) |  | 25@26 (A6) |
| 16 | 1900 | 40 | Default |  |  | QPSK | 39@177 (A5) |  | 17@89 (A5) |  | 6@45 (A5) |
| 17 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |  |  |
| 18 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |  |  |
| 19 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@0 (A9) |  | 10@0 (A9) |  | 5@0 (A9) |
| 20 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |  |  |
| 21 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |  |  |
| 22 | 1900 | 30 | Default |  |  | 256 QAM | 20@0 (A9) |  | 10@0 (A9) |  | 5@0 (A9) |
| 23 | 1900 | 35 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 24 | 1900 | 35 | Default |  |  | 256 QAM | 50@10 (A3) |  | 25@5 (A3) |  | 12@3 (A3) |
| 25 | 1900 | 35 | Default |  |  | 256 QAM | 178@10 (A2) |  | 87@5 (A2) |  | 41@3 (A2) |
| 26 | 1900 | 35 | Default |  |  | 256 QAM | 111@77 (A6) |  | 53@39 (A6) |  | 24@20 (A6) |
| 27 | 1900 | 35 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |  |  |
| 28 | Low | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 29 | Low | 40 | Default |  |  | 256 QAM | 60@25 (A3) |  | 30@13 (A3) |  | 15@7 (A3) |
| 30 | Low | 40 | Default |  |  | 256 QAM | 191@25 (A2) |  | 93@13 (A2) |  | 44@7 (A2) |
| 31 | Low | 40 | Default |  |  | 256 QAM | 115@101 (A6) |  | 55@51 (A6) |  | 25@26 (A6) |
| 32 | Low | 40 | Default |  |  | 256 QAM | 39@177 (A5) |  | 17@89 (A5) |  | 6@45 (A5) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2D.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable Table in clause 6.2D.3.4.1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.3.4.3.

6.2D.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to applicable table in clause 6.2D.3.4.1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE. Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to relevant configuration tables. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

6.2D.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.1 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO, with the following exceptions for each network signalling value.

6.2D.3.4.3.1	Message contents exceptions for network signalling value "NS_04"

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

6.2D.3.4.3.2	Message contents exceptions for network signalling value "NS_35"

1.	Information element additionalSpectrumEmission is set to NS_35. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_35" and NR band n71

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_35) |  |  |

6.2D.3.4.3.3	Message contents exceptions for network signalling value "NS_05"

1.	Information element additionalSpectrumEmission is set to NS_05. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_05) |  |  |

6.2D.3.4.3.4	Message contents exceptions for network signalling value "NS_05U"

1.	Information element additionalSpectrumEmission is set to NS_05U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.4-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_05U) |  |  |

6.2D.3.4.3.5	Message contents exceptions for network signalling value "NS_48"

1.	Information element additionalSpectrumEmission is set to NS_48. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_48"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_48) |  |  |

6.2D.3.4.3.6	Message contents exceptions for network signalling value "NS_49"

1.	Information element additionalSpectrumEmission is set to NS_49. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.6-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_49"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_49) |  |  |

6.2D.3.4.3.7	Message contents exceptions for network signalling value "NS_100"

1.	Information element additionalSpectrumEmission is set to NS_100. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.7-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_100) |  | not for band n65 |
|  | 2 (NS_100) |  | for band n65 |

6.2D.3.4.3.8	Message contents exceptions for network signalling value “NS_03”

1.	Information element additionalSpectrumEmission is set to NS_03. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.8-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n2, n25 and n66

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_03) |  |  |

Table 6.2D.3.4.3.8-2: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n70

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_03) |  |  |

6.2D.3.4.3.9	Message contents exceptions for network signalling value "NS_03U"

1.	Information element additionalSpectrumEmission is set to NS_03U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.9-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03U) |  | for band n2, n25, n66 |

6.2D.3.4.3.10	Message contents exceptions for network signalling value "NS_46"

1.	Information element additionalSpectrumEmission is set to NS_46. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.10-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_46"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_46) |  | for band n7 |

6.2D.3.4.3.11	Message contents exceptions for network signalling value "NS_21"

1.	Information element additionalSpectrumEmission is set to NS_21. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.11-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_21"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_21) |  |  |

6.2D.3.4.3.12	Message contents exceptions for network signalling value "NS_44"

1.	Information element additionalSpectrumEmission is set to NS_44. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.12-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_44"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_44) |  |  |

6.2D.3.4.3.13	Message contents exceptions for network signalling value "NS_27"

1.	Information element additionalSpectrumEmission is set to NS_27. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.13-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_27"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_27) |  |  |

6.2D.3.4.3.14	Message contents exceptions for network signalling value "NS_56"

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.14-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_56) |  |  |

6.2D.3.4.3.15	Message contents exceptions for network signalling value "NS_50"

1.	Information element additionalSpectrumEmission is set to NS_50. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.15-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_50"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_50) |  |  |

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-14. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

Table 6.2D.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

Table 6.2D.3.5-1: UE Power Class 2 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.0-TT |
| 2 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 3 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.0-TT |
| 4 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 5 | 26 | 4 | 0 | 1.52 | 20.5 | 5 | 3 | 28+TT | 15.5-TT |
| 6 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 7 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.0-TT |
| 8 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 9 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.0-TT |
| 10 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 11 | 26 | 4 | 0 | 1.52 | 20.5 | 5 | 3 | 28+TT | 15.5-TT |
| 12 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 13 | 26 | 4.5 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.0-TT |
| 14 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 15 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 16 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 17 | 26 | 4.5 | 0 | 1.52 | 20 | 5 | 3 | 28+TT | 15.0-TT |
| 18 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 19 | 26 | 8 | 10 | 1.52 | 14.5 | 6 | 3 | 28+TT | 8.5-TT |
| 20 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| 21 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| 22 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| 23 | 26 | 8 | 0 | 1.52 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 24 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-1a: UE Power Class 2 test requirements (NS_04) for band n41 with supporting ULFPTx

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 26 | 3.5 | 5.5 | 1.52 | 19 | 5 | 3 | 28+TT | 14.-TT |
| 8 | 26 | 3.5 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 9 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 10 | 26 | 1 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 11 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 12 | 26 | 1 | 0 | 0 | 25 | 3 | 3 | 28+TT | 22.-TT |
| 13 | 26 | 3.5 | 6 | 1.52 | 18.5 | 5 | 3 | 28+TT | 13.5-TT |
| 14 | 26 | 3.5 | 4.5 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 15 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 16 | 26 | 2 | 4.5 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 17 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 18 | 26 | 2 | 0 | 0 | 24 | 3 | 3 | 28+TT | 21.-TT |
| 19 | 26 | 3.5 | 6 | 1.52 | 18.5 | 5 | 3 | 28+TT | 13.5-TT |
| 20 | 26 | 3.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 21 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 22 | 26 | 2.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 23 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 24 | 26 | 2.5 | 0 | 0 | 23.5 | 3 | 3 | 28+TT | 20.5-TT |
| 25 | 26 | 3.5 | 6.5 | 1.52 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 26 | 26 | 3.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 27 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 28 | 26 | 3 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 29 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 30 | 26 | 3 | 0 | 0 | 23 | 3 | 3 | 28+TT | 20.-TT |
| 31 | 26 | 5.5 | 8 | 1.52 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 32 | 26 | 5.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 33 | 26 | 5.5 | 0 | 0 | 20.5 | 5 | 3 | 28+TT | 15.5-TT |
| 34 | 26 | 5.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 35 | 26 | 5.5 | 0 | 1.52 | 19 | 5 | 3 | 28+TT | 14.-TT |
| 36 | 26 | 5.5 | 0 | 0 | 20.5 | 5 | 3 | 28+TT | 15.5-TT |
| 37 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.-TT |
| 38 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 39 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.-TT |
| 40 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 41 | 26 | 4 | 0 | 1.52 | 20.5 | 5 | 3 | 28+TT | 15.5-TT |
| 42 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 43 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.-TT |
| 44 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 45 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.-TT |
| 46 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 47 | 26 | 4 | 0 | 1.52 | 20.5 | 5 | 3 | 28+TT | 15.5-TT |
| 48 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 49 | 26 | 4.5 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.-TT |
| 50 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 51 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 52 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 53 | 26 | 4.5 | 0 | 1.52 | 20 | 5 | 3 | 28+TT | 15.-TT |
| 54 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 55 | 26 | 8 | 10 | 1.52 | 14.5 | 6 | 3 | 28+TT | 8.5-TT |
| 56 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 57 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 58 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 59 | 26 | 8 | 0 | 1.52 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 60 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2: UE Power Class 3 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.0-TT |
| 2 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0-TT |
| 4 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 5 | 23 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 6 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0-TT |
| 7 | 23 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.0-TT |
| 8 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0-TT |
| 10 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 11 | 23 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 12 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0-TT |
| 13 | 23 | 3.5 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.0-TT |
| 14 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 15 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 16 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 17 | 23 | 3.5 | 0 | 1.52 | 18 | 5 | 3 | 25+TT | 13.0-TT |
| 18 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 19 | 23 | 6.5 | 8 | 1.52 | 13.5 | 6 | 3 | 25+TT | 7.5-TT |
| 20 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 21 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 22 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 6.5 | 0 | 1.52 | 15 | 6 | 3 | 25+TT | 9.0-TT |
| 24 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2a: UE Power Class 3 test requirements (NS_04) for band n41 with supporting ULFPTx

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 5.5 | 1.52 | 19 | 5 | 3 | 28+TT | 14-TT |
| 2 | 23 | -3 | 3.5 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 3 | 23 | -3 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 4 | 23 | -3 | 1.2 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 5 | 23 | -3 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 6 | 23 | -3 | 1.2 | 0 | 0 | 24.8 | 3 | 3 | 28+TT | 21.8-TT |
| 7 | 23 | 0 | 0.5 | 3.5 | 1.52 | 18 | 5 | 3 | 25+TT | 13.-TT |
| 8 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 9 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 5 | 3 | 25+TT | 17.5-TT |
| 10 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 11 | 23 | 0 | 0.5 | 0 | 1.52 | 21 | 5 | 3 | 25+TT | 16.-TT |
| 12 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 5 | 3 | 25+TT | 17.5-TT |
| 13 | 23 | 0 | 1 | 4 | 1.52 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 14 | 23 | 0 | 1 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 15 | 23 | 0 | 1 | 0 | 0 | 22 | 5 | 3 | 25+TT | 17.-TT |
| 16 | 23 | 0 | 1 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 17 | 23 | 0 | 1 | 0 | 1.52 | 20.5 | 5 | 3 | 25+TT | 15.5-TT |
| 18 | 23 | 0 | 1 | 0 | 0 | 22 | 5 | 3 | 25+TT | 17.-TT |
| 19 | 23 | 0 | 2 | 4 | 1.52 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 2 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 21 | 23 | 0 | 2 | 0 | 0 | 21 | 5 | 3 | 25+TT | 16.-TT |
| 22 | 23 | 0 | 2 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 23 | 23 | 0 | 2 | 0 | 1.52 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 2 | 0 | 0 | 21 | 5 | 3 | 25+TT | 16.-TT |
| 25 | 23 | 0 | 2.5 | 4.5 | 1.52 | 17 | 5 | 3 | 25+TT | 12.-TT |
| 26 | 23 | 0 | 2.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 27 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 5 | 3 | 25+TT | 15.5-TT |
| 28 | 23 | 0 | 2.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 29 | 23 | 0 | 2.5 | 0 | 1.52 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 30 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 5 | 3 | 25+TT | 15.5-TT |
| 31 | 23 | 0 | 4.5 | 6 | 1.52 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 32 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 33 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 34 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 35 | 23 | 0 | 4.5 | 0 | 1.52 | 17 | 5 | 3 | 25+TT | 12.-TT |
| 36 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 37 | 23 | 0 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.-TT |
| 38 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 39 | 23 | 0 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.-TT |
| 40 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 41 | 23 | 0 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 42 | 23 | 0 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.-TT |
| 43 | 23 | 0 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.-TT |
| 44 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 45 | 23 | 0 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.-TT |
| 46 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 47 | 23 | 0 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 48 | 23 | 0 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.-TT |
| 49 | 23 | 0 | 3.5 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.-TT |
| 50 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 51 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 52 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 53 | 23 | 0 | 3.5 | 0 | 1.52 | 18 | 5 | 3 | 25+TT | 13.-TT |
| 54 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 55 | 23 | 0 | 6.5 | 8 | 1.52 | 13.5 | 6 | 3 | 25+TT | 7.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 57 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 58 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 59 | 23 | 0 | 6.5 | 0 | 1.52 | 15 | 6 | 3 | 25+TT | 9.-TT |
| 60 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-3: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0 - TT |
| 2 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0 - TT |
| 3 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0 - TT |
| 4 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0 - TT |
| 5 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0 - TT |
| 6 | 23 | 3 | 0 | 0 | 20 | 5 | 3 | 25+TT | 15.0 - TT |
| 7 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 8 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 9 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 10 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 11 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 12 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-4: UE Power Class 3 test requirements (NS_05) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 36 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 37 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 38 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 39 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 40 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 41 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 42 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 43 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 44 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 45 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 46 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 47 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 48 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 49 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 50 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-5: UE Power Class 3 test requirements (NS_05U) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 36 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 37 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 39 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 40 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 41 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 42 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 43 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 44 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 45 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 46 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 3 | 25+TT | 7-TT |
| 47 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 48 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 49 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 50 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 51 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 53 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 54 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 3 | 25+TT | 3-TT |
| 55 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-6: UE Power Class 3 test requirements (NS_48) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 2 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 3 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 4 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 6 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 9 | 23 | 0 | 1.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 10 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 11 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 12 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 13 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 14 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 16 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 17 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 18 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 19 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 20 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 21 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 22 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 24 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 25 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 26 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 27 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 29 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 30 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-7: UE Power Class 3 test requirements (NS_49) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 2 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 3 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 4 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 5 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 6 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 9 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 10 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 11 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 12 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 13 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 14 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 16 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 17 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 18 | 23 | 0 | 1.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 19 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 20 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 21 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 22 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 24 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 25 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 26 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 27 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 28 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 29 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 30 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 31 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 32 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 33 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 34 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 35 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 36 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 37 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 38 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 39 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 40 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 41 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 42 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-8: UE Power Class 3 test requirements (NS_100) for band n1

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4, 5 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7, 8 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-9: UE Power Class 3 test requirements (NS_100) for band n2, n3, n25

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4, 5 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7, 8 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 1.5 | 15 | 6 | 3 | 25+TT | 9-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-9a: UE Power Class 3 test requirements (NS_100) for band n2, n3, n25 supporting ULFPTx

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_100 | 23 | 0.5 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 3 | NS_100 | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 4, 5 | NS_100 | 23 | 1 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | NS_100 | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 7, 8 | NS_100 | 23 | 2 | 2.5 | 1.5 | 19 | 5 | 3 | 25+TT | 14-TT |
| 9 | NS_100 | 23 | 2 | 2.5 | 0 | 20.5 | 5 | 3 | 25+TT | 15.5-TT |
| 10, 11 | NS_100 | 23 | 2.5 | 3 | 1.5 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 12 | NS_100 | 23 | 2.5 | 3 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 13, 14 | NS_100 | 23 | 4.5 | 4.5 | 1.5 | 17 | 5 | 3 | 25+TT | 12-TT |
| 15 | NS_100 | 23 | 4.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 16, 17 | NS_100 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 18 | NS_100 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 19, 20 | NS_100 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 21 | NS_100 | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 22, 23 | NS_100 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 24 | NS_100 | 23 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 25, 26 | NS_100 | 23 | 6.5 | 6.5 | 1.5 | 15 | 6 | 3 | 25+TT | 9-TT |
| 27 | NS_100 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-10: UE Power Class 3 test requirements (NS_03/NS_03U) for band n66, n70

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 3 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4,5 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7,8 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 9 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 10,11 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 12 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-10a: UE Power Class 3 test requirements (NS_03/NS_03U) for band n66, n70 supporting ULFPTx

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 4, 5 | NS_03, NS_03U | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 6 | NS_03, NS_03U | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 7, 8 | NS_03, NS_03U | 23 | 2 | 3 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 9 | NS_03, NS_03U | 23 | 2 | 3 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 10, 11 | NS_03, NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 12 | NS_03, NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 13, 14 | NS_03, NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 15 | NS_03, NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 16, 17 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 18 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 19, 20 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 21 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 22, 23 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 24 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 25, 26 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 27 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-11: UE Power Class 3 test requirements (NS_03/NS_03U) for band n2, n25

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | NS_03, NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4,5 | NS_03, NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 6 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7,8 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 1.5 | 17 | 5 | 3 | 25+TT | 12-TT |
| 9 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 10,11 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 1.5 | 14 | 6 | 3 | 25+TT | 8-TT |
| 12 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-11a: UE Power Class 3 test requirements (NS_03/NS_03U) for band n2, n25 supporting ULFPTx

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 1.5 | 20 | 5 | 3 | 25+TT | 14-TT |
|  | NS_03U | 23 | 0.5 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 4, 5 | NS_03; NS_03U | 23 | 1 | 2 | 1.5 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | NS_03; NS_03U | 23 | 1 | 2 | 0 | 21 | 5 | 3 | 25+TT | 16-TT |
| 7, 8 | NS_03; NS_03U | 23 | 2 | 3 | 1.5 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 9 | NS_03; NS_03U | 23 | 2 | 3 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 10, 11 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 1.5 | 18 | 5 | 3 | 25+TT | 13-TT |
| 12 | NS_03; NS_03U | 23 | 2.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 13, 14 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 1.5 | 16 | 5 | 3 | 25+TT | 11-TT |
| 15 | NS_03; NS_03U | 23 | 4.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 16, 17 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 18 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 19, 20 | NS_03; NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 21 | NS_03; NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 22, 23 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 1.5 | 17 | 5 | 3 | 25+TT | 12-TT |
| 24 | NS_03; NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 25, 26 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 1.5 | 14 | 6 | 3 | 25+TT | 8-TT |
| 27 | NS_03; NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-12: UE Power Class 3 test requirements (NS_46) for band n7

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 2 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 3 | 23 | 0 | 3 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 4 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 6 | 23 | 0 | 3.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 7 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 8 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 10 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 11 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| 12 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 3 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-13: UE Power Class 3 test requirements (NS_21) for band n30

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 5 | 23 | 3 | 0 | 0 |  | 20 | 5 | 3 | 25+TT | 15-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 3 | 25+TT | 12-TT |
| 3 | 5 | 23 | 3 | 0 | 0 |  | 20 | 5 | 3 | 25+TT | 15-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 4,5 | 10 | 23 | 3 | 4 | 0 |  | 19 | 5 | 3 | 25+TT | 14-TT |
| 6,7 | 5 | 23 | 3 | 0 | 0 |  | 20 | 5 | 3 | 25+TT | 15-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 3 | 25+TT | 12-TT |
| 8 | 5 | 23 | 3 | 0 | 0 |  | 20 | 5 | 3 | 25+TT | 15-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9,10 | 10 | 23 | 3 | 4 | 0 |  | 19 | 5 | 3 | 25+TT | 14-TT |
| 11,12 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
|  | 10 | 23 | 3.5 | 6 | 0 |  | 17 | 5 | 3 | 25+TT | 12-TT |
| 13 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
|  | 10 | 23 | 3.5 | 5.5 | 0 |  | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 14,15 | 10 | 23 | 3.5 | 4 | 0 |  | 19 | 5 | 3 | 25+TT | 14-TT |
| 16,17 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 6 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 18 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 5.5 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 19,20 | 10 | 23 | 6.5 | 4 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-14: UE Power Class 3 test requirements (NS_44) for band n38

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 2 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 3 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 5 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 6 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 7 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 8 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 10 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 11 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 12 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 13 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 14 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 16 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 17 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 18 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 19 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 20 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 21 | 23 | 0 | 6.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 22 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-15: UE Power Class 3 test requirements (NS_27) for band n48

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 2 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 3 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 5 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 7 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 8 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 9 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 10 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 11 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 12 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 13 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 14 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 15 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 16 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 17 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 18 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 19 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 21 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 22 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 23 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 24 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 25 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 26 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 27 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 28 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 29 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 30 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 31 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 32 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 33 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 34 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 35 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 36 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 37 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 39 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 40 | 23 | 0 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 41 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 42 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 43 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 44 | 23 | 0 | 3 | 2 | 0 | 20 | 5 | 3 | 25+TT | 15-TT |
| 45 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 46 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 47 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 48 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 49 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 50 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 51 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 52 | 23 | 0 | 3 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 53 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 54 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 55 | 23 | 0 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 56 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 57 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 58 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 59 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 60 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 61 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 62 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 63 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 64 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 65 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 66 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 67 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 68 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 69 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 70 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 71 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 72 | 23 | 0 | 3.5 | 2 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 73 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 74 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 75 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 76 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 77 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 78 | 23 | 0 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 79 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 80 | 23 | 0 | 3.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 81 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 82 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 83 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 84 | 23 | 0 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 85 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 86 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 87 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 88 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 89 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 90 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 91 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 92 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 93 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 94 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 95 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 96 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 97 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 98 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 99 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 100 | 23 | 0 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 101 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 102 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 103 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 104 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 105 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 106 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 107 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 108 | 23 | 0 | 6.5 | 11.5 | 0 | 11.5 | 6 | 3 | 25+TT | 5.5-TT |
| 109 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 110 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 111 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 112 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

Table 6.2D.3.5-16: UE Power Class 3 test requirements (NS_56) for band n24

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 2 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 3 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 4 | 23 | 3 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 5 | 23 | 1.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 6 | 23 | 3 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 7 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 8 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 9 | 23 | 3 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14 -TT |
| 10 | 23 | 2 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 11 | 23 | 3.5 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 12 | 23 | 3.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 13 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 14 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 15 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 16 | 23 | 6.5 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 17 | 23 | 6.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 18 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 19 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 20 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 21 | 23 | 3 | 12 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 22 | 23 | 3 | 8 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 23 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 24 | 23 | 1.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 25 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 26 | 23 | 1.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 27 | 23 | 3 | 2 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 28 | 23 | 3 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 29 | 23 | 1.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 30 | 23 | 1.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 31 | 23 | 3 | 3 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 32 | 23 | 3 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 33 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 34 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 35 | 23 | 2 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 36 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 37 | 23 | 2 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 38 | 23 | 3 | 2 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 39 | 23 | 3 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 40 | 23 | 2 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 41 | 23 | 2 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 42 | 23 | 3 | 3 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 43 | 23 | 3.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 44 | 23 | 3.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 45 | 23 | 3.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 46 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 47 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 48 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 49 | 23 | 3.5 | 2 | 0 | 19.5 | 5 | 4 | 25+TT | 14.5-TT |
| 50 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 51 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 52 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 53 | 23 | 3.5 | 3 | 0 | 19.5 | 5 | 4 | 25+TT | 14.5-TT |
| 54 | 23 | 6.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 55 | 23 | 6.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 56 | 23 | 6.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 57 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 58 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 59 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 60 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 61 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 62 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 63 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 64 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-17: UE Power Class 3 test requirements (NS_50) for band n39

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3 | 5 | 0 |  | 18 |  | 5.0 |  | 3 | 25.0 + TT | 13.0 - TT |  |
| 2 | 23 | 0 | 3 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 3 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 5.0 |  | 3 | 25.0 + TT | 15 - TT |  |
| 4 | 23 | 0 | 3 | 0 | 0 |  | 20.0 |  | 5.0 |  | 3 | 25.0 + TT | 15 - TT |  |
| 5 | 23 | 0 | 3 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 6 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 5.0 |  | 3 | 25.0 + TT | 15 - TT |  |
| 7 | 23 | 0 | 3 | 12 | 0 |  | 11 |  | 6.0 |  | 3 | 25.0 + TT | 5.0 - TT |  |
| 8 | 23 | 0 | 3 | 4.5 | 0 |  | 18.5 |  | 4.0 |  | 3 | 25.0 + TT | 14.5 - TT |  |
| 9 | 23 | 0 | 3 | 8 | 0 |  | 15 |  | 5.0 |  | 3 | 25.0 + TT | 10.0 - TT |  |
| 10 | 23 | 0 | 3 | 3.5 | 0 |  | 19.5 |  | 3.5 |  | 3 | 25.0 + TT | 16.0 - TT |  |
| 11 | 23 | 0 | 3 | 5 | 0 |  | 18 |  | 4.0 |  | 3 | 25.0 + TT | 14.0 - TT |  |
| 12 | 23 | 0 | 3 | 0 | 0 |  | 20.0 |  | 5.0 |  | 3 | 25.0 + TT | 15 – TT |  |
| 13 | 23 | 0 | 3 | 12 | 0 |  | 11 |  | 6.0 |  | 3 | 25.0 + TT | 5.0 - TT |  |
| 14 | 23 | 0 | 3 | 4.5 | 0 |  | 18.5 |  | 5.0 |  | 3 | 25.0 + TT | 13.5 - TT |  |
| 15 | 23 | 0 | 3 | 3.5 | 0 |  | 19.5 |  | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT |  |
| 16 | 23 | 0 | 3 | 5 | 0 |  | 18 |  | 5.0 |  | 3 | 25.0 + TT | 13.0 - TT |  |
| 17 | 23 | 0 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 18 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 19 | 23 | 0 | 6.5 | 6 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 20 | 23 | 0 | 6.5 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 21 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 22 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 23 | 23 | 0 | 6.5 | 12 | 0 |  | 11 |  | 6.0 |  | 3 | 25.0 + TT | 5.0 - TT |  |
| 24 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 25 | 23 | 0 | 6.5 | 8 | 0 |  | 15 |  | 5.0 |  | 3 | 25.0 + TT | 10.0 - TT |  |
| 26 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 27 | 23 | 0 | 6.5 | 0 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 28 | 23 | 0 | 6.5 | 12 | 0 |  | 11 |  | 6.0 |  | 3 | 25.0 + TT | 5.0 - TT |  |
| 29 | 23 | 0 | 6.5 | 4..5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 30 | 23 | 0 | 6.5 | 8 | 0 |  | 15 |  | 6.0 |  | 3 | 25.0 + TT | 9.0 - TT |  |
| 31 | 23 | 0 | 6.5 | 3.5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| 32 | 23 | 0 | 6.5 | 5 | 0 |  | 16.5 |  | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.2D.3_1	UE additional maximum output power reduction for SUL with UL MIMO

6.2D.3_1.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1_1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3_1.2	Test applicability

The requirements of this test apply in test case 6.5D.2.4.2_1 UTRA Adjacent channel leakage ratio for network signalling values NS_05U to all types of NR UE release 17 and forward that support SUL and UL-MIMO.

The requirements of this test apply in test case 6.5D.3_2.3 Additional Spurious Emissions for network signalling value NS_05, NS_05U and NS_56 to all types of NR UE release 17 and forward that support SUL and UL-MIMO.

NOTE:	Test execution is not necessary if 6.5D.2.4.2_1 and 6.5D.3_2.3 are executed.

6.2D.3_1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in subclause 6.2.3.3 shall apply to the maximum output power specified in Table 6.2D.1_1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1_1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2D.4_1.3 apply.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.3.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.3.1.

6.2D.3_1.4	Test description

6.2D.3_1.4.1	Test procedure

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in Table 6.2D.3_1.4.1-1 to Table 6.2D.3_1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.3_1.4.1-1: Test Configuration Table for NS_05

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: use Fc as specified in test parameters NUL carrier: Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz, 20 MHz NUL carrier: Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for NUL carrier |  |  |  |
| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink Config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 2 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 3 | 1925 | 10 |  |  | A7 |  | QPSK | 42@10 |
| 4 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 5 | 1935 | 10 |  |  | A4 |  | QPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 12 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 13 | 1930 | 20 |  |  | A7 |  | QPSK | 78@28 |
| 14 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 15 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 17 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 18 | 1925 | 10 |  |  | A7 |  | 16 QAM | 42@10 |
| 19 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 20 | 1935 | 10 |  |  | A4 |  | 16 QAM | Outer_Full |
| 21 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 22 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 23 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 24 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 25 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 26 | 1942.5 | 15 |  |  | A5 |  | 16 QAM | Outer_Full |
| 27 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 28 | 1930 | 20 |  |  | A7 |  | 16 QAM | 78@28 |
| 29 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |
| 30 | 1950 | 20 |  |  | A6 |  | 16 QAM | Outer_Full |
| 31 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  | A7 |  | 64 QAM | 42@10 |
| 34 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |
| 35 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 37 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 38 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 39 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 40 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 41 | 1930 | 20 |  |  | A7 |  | 64 QAM | 78@28 |
| 42 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 43 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 44 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 45 | 1925 | 10 |  |  | A7 |  | 256 QAM | 42@10 |
| 46 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 47 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 48 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 49 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 50 | 1930 | 20 |  |  | A7 |  | 256 QAM | 78@28 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.4.1-2: Test Configuration Table for NS_05U

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: Use Fc as specified in test parameters NUL carrier: Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz, 20 MHz NUL carrier: Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carrier |  |  |  |
| AMPR Test parameters for NS_05U |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 2 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 3 | 1925 | 10 |  |  | A7 |  | QPSK | 42@10 |
| 4 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 5 | 1935 | 10 |  |  | A4 |  | QPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 12 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 13 | 1930 | 20 |  |  | A7 |  | QPSK | 78@28 |
| 14 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 15 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 17 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 18 | 1925 | 10 |  |  | A7 |  | 16 QAM | 42@10 |
| 19 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 20 | 1935 | 10 |  |  | A4 |  | 16 QAM | Outer_Full |
| 21 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 22 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 23 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 24 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 25 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 26 | 1942.5 | 15 |  |  | A5 |  | 16 QAM | Outer_Full |
| 27 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 28 | 1930 | 20 |  |  | A7 |  | 16 QAM | 78@28 |
| 29 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |
| 30 | 1950 | 20 |  |  | A6 |  | 16 QAM | Outer_Full |
| 31 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  | A7 |  | 64 QAM | 42@10 |
| 34 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |
| 35 | 1935 | 10 |  |  | A4 |  | 64 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 37 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 38 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 39 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 40 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 41 | 1942.5 | 15 |  |  | A5 |  | 64 QAM | Outer_Full |
| 42 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 43 | 1930 | 20 |  |  | A7 |  | 64 QAM | 78@28 |
| 44 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 45 | 1950 | 20 |  |  | A6 |  | 64 QAM | Outer_Full |
| 46 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 47 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 48 | 1925 | 10 |  |  | A7 |  | 256 QAM | 42@10 |
| 49 | 1935 | 10 |  |  | A4 |  | 256 QAM | Outer_Full |
| 50 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 51 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 52 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 53 | 1942.5 | 15 |  |  | A5 |  | 256 QAM | Outer_Full |
| 54 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 55 | 1930 | 20 |  |  | A7 |  | 256 QAM | 78@28 |
| 56 | 1950 | 20 |  |  | A6 |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.4.1-3: Test Configuration Table for NS_56

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters for SUL carrier Lowest for non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carriers |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  | SCS 15 kHz |
| 1 | Low | 5 | N/A for A-MPR testing | N/A | 14 | CP-OFDM | QPSK | Edge_1RB_Left |
| 2 | Low | 5 |  |  | 6 |  | QPSK | Outer_Full |
| 3 | Low | 5 |  |  | 4 |  | QPSK | 21@4 |
| 4 | Low | 5 |  |  | 4 |  | QPSK | 1@4 |
| 5 | Low | 5 |  |  | 14 |  | 16 QAM | Edge_1RB_Left |
| 6 | Low | 5 |  |  | 6 |  | 16 QAM | Outer_Full |
| 7 | Low | 5 |  |  | 4 |  | 16 QAM | 21@4 |
| 8 | Low | 5 |  |  | 4 |  | 16 QAM | 1@4 |
| 9 | Low | 5 |  |  | 14 |  | 64 QAM | Edge_1RB_Left |
| 10 | Low | 5 |  |  | 6 |  | 64 QAM | Outer_Full |
| 11 | Low | 5 |  |  | 4 |  | 64 QAM | 21@4 |
| 12 | Low | 5 |  |  | 4 |  | 64 QAM | 1@4 |
| 13 | Low | 5 |  |  | 14 |  | 256 QAM | Edge_1RB_Left |
| 14 | Low | 5 |  |  | 6 |  | 256 QAM | Outer_Full |
| 15 | Low | 5 |  |  | 4 |  | 256 QAM | 21@4 |
| 16 | Low | 5 |  |  | 4 |  | 256 QAM | 1@4 |
| 17 | Low | 10 |  |  | 12 |  | QPSK | Edge_1RB_Left |
| 18 | Low | 10 |  |  | 8 |  | QPSK | Outer_Full |
| 19 | Low | 10 |  |  | 6 |  | QPSK | 1@3 |
| 20 | Low | 10 |  |  | 6 |  | QPSK | 43@9 |
| 21 | Low | 10 |  |  | 4 |  | QPSK | 1@35 |
| 22 | Low | 10 |  |  | 2 |  | QPSK | 17@35 |
| 23 | Low | 10 |  |  | 5 |  | QPSK | Edge_1RB_Right |
| 24 | Low | 10 |  |  | 5 |  | QPSK | 1@40 |
| 25 | Low | 10 |  |  | 3 |  | QPSK | 8@44 |
| 26 | Low | 10 |  |  | 12 |  | 16 QAM | Edge_1RB_Left |
| 27 | Low | 10 |  |  | 8 |  | 16 QAM | Outer_Full |
| 28 | Low | 10 |  |  | 6 |  | 16 QAM | 1@3 |
| 29 | Low | 10 |  |  | 6 |  | 16 QAM | 43@9 |
| 30 | Low | 10 |  |  | 4 |  | 16 QAM | 1@35 |
| 31 | Low | 10 |  |  | 2 |  | 16 QAM | 17@35 |
| 32 | Low | 10 |  |  | 5 |  | 16 QAM | Edge_1RB_Right |
| 33 | Low | 10 |  |  | 5 |  | 16 QAM | 1@40 |
| 34 | Low | 10 |  |  | 3 |  | 16 QAM | 8@44 |
| 35 | Low | 10 |  |  | 12 |  | 64 QAM | Edge_1RB_Left |
| 36 | Low | 10 |  |  | 8 |  | 64 QAM | Outer_Full |
| 37 | Low | 10 |  |  | 6 |  | 64 QAM | 1@3 |
| 38 | Low | 10 |  |  | 6 |  | 64 QAM | 43@9 |
| 39 | Low | 10 |  |  | 4 |  | 64 QAM | 1@35 |
| 40 | Low | 10 |  |  | 2 |  | 64 QAM | 17@35 |
| 41 | Low | 10 |  |  | 5 |  | 64 QAM | Edge_1RB_Right |
| 42 | Low | 10 |  |  | 5 |  | 64 QAM | 1@40 |
| 43 | Low | 10 |  |  | 3 |  | 64 QAM | 8@44 |
| 44 | Low | 10 |  |  | 12 |  | 256 QAM | Edge_1RB_Left |
| 45 | Low | 10 |  |  | 8 |  | 256 QAM | Outer_Full |
| 46 | Low | 10 |  |  | 6 |  | 256 QAM | 1@3 |
| 47 | Low | 10 |  |  | 6 |  | 256 QAM | 43@9 |
| 48 | Low | 10 |  |  | 4 |  | 256 QAM | 1@35 |
| 49 | Low | 10 |  |  | 2 |  | 256 QAM | 17@35 |
| 50 | Low | 10 |  |  | 5 |  | 256 QAM | Edge_1RB_Right |
| 51 | Low | 10 |  |  | 5 |  | 256 QAM | 1@40 |
| 52 | Low | 10 |  |  | 3 |  | 256 QAM | 8@44 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2D.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement channels are set according to the applicable tables 6.2D.3_1.4.1-1 to  6.2D.3_1.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.3_1.4.3.

6.2D.3_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.3_1.4.1-1 to Table 6.2D.3_1.4.1-3. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE. Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

6.2D.3_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.1 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO, with the following exceptions for each network signalling value.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL. All the AdditionalSpectrumEmission in 6.2.3.4.3 are sent in SIB1 as part of supplementaryUplink instead of uplinkConfigCommon.

6.2D.3_1.4.3.1	Message contents exceptions for network signalling value "NS_05"

1.	Information element additionalSpectrumEmission is set to NS_05. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3_1.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_05) |  |  |

6.2D.3_1.4.3.2	Message contents exceptions for network signalling value "NS_05U"

1.	Information element additionalSpectrumEmission is set to NS_05U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3_1.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_05U) |  |  |

6.2D.3_1.4.3.3	Message contents exceptions for network signalling value "NS_56"

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3_1.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_56) |  |  |

6.2D.3_1.5	Test requirement

The measured maximum output power shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable tables from Table 6.2D.3_1.5-1 to Table 6.2D.3_1.5-3. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2C.4. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2D.1_1.3-1 apply.

Table 6.2D.3_1.5-1: UE Power Class 3 test requirements (NS_05) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 3.5 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 2 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 36 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 37 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 38 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 39 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 40 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 41 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 42 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 43 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 44 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 45 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 46 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 47 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 48 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 49 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 50 | 23 | 0 | 6.5 | 6 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.5-2: UE Power Class 3 test requirements (NS_05U) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 2 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 3 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 4 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 5 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 7 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 8 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 10 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 11 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 13 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 15 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 16 | 23 | 0 | 3.0 | 7.5 | 0 | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 17 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 18 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 19 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 20 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 21 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 22 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 23 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 24 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 25 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 26 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 27 | 23 | 0 | 3.0 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 28 | 23 | 0 | 3.0 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 3.0 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 30 | 23 | 0 | 3.0 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 31 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 2 | 25+TT | 9-TT |
| 32 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 33 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 34 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 35 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 37 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 38 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 39 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 40 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 41 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 42 | 23 | 0 | 3.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 43 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 44 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 45 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 46 | 23 | 0 | 6.5 | 10 | 0 | 13 | 6 | 2 | 25+TT | 7-TT |
| 47 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 48 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 49 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 50 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 51 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 53 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 54 | 23 | 0 | 6.5 | 13 | 0 | 10 | 7 | 2 | 25+TT | 3-TT |
| 55 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3_1.5-3: UE Power Class 3 test requirements (NS_56) for band n99

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 2 | 23 | 3 | 8 | 0 | 15 | 5 | 4 | 25+TT | 10-TT |
| 3 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 4 | 23 | 3 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 5 | 23 | 1.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 6 | 23 | 3 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 7 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 8 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 9 | 23 | 3 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 10 | 23 | 2 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 11 | 23 | 3.5 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 12 | 23 | 3.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 13 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 14 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 15 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 16 | 23 | 6.5 | 14 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 17 | 23 | 6.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 18 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 19 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 20 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 21 | 23 | 3 | 12 | 0 | 9 | 7 | 4 | 25+TT | 2-TT |
| 22 | 23 | 3 | 8 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 23 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 24 | 23 | 1.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 25 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 26 | 23 | 1.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 27 | 23 | 3 | 2 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 28 | 23 | 3 | 5 | 0 | 18 | 5 | 4 | 25+TT | 14-TT |
| 29 | 23 | 1.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 14-TT |
| 30 | 23 | 1.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 14-TT |
| 31 | 23 | 3 | 3 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 32 | 23 | 3 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 33 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 34 | 23 | 3 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 35 | 23 | 2 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 36 | 23 | 3 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 37 | 23 | 2 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 38 | 23 | 3 | 2 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 39 | 23 | 3 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 23 | 2 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 41 | 23 | 2 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 42 | 23 | 3 | 3 | 0 | 20 | 5 | 4 | 25+TT | 15-TT |
| 43 | 23 | 3.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 44 | 23 | 3.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 45 | 23 | 3.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 46 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 47 | 23 | 3.5 | 6 | 0 | 17 | 5 | 4 | 25+TT | 12-TT |
| 48 | 23 | 3.5 | 4 | 0 | 19 | 5 | 4 | 25+TT | 14-TT |
| 49 | 23 | 3.5 | 2 | 0 | 19.5 | 5 | 4 | 25+TT | 14.5-TT |
| 50 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 51 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 52 | 23 | 3.5 | 5 | 0 | 18 | 5 | 4 | 25+TT | 13-TT |
| 53 | 23 | 3.5 | 3 | 0 | 19.5 | 5 | 4 | 25+TT | 15.5-TT |
| 54 | 23 | 6.5 | 12 | 0 | 11 | 6 | 4 | 25+TT | 5-TT |
| 55 | 23 | 6.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 56 | 23 | 6.5 | 8 | 0 | 15 | 6 | 4 | 25+TT | 9-TT |
| 57 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 58 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 59 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 60 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 61 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 62 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 63 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| 64 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 4 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n99, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2D.4	Configured transmitted power for UL MIMO

6.2D.4.1	Test purpose

To verify the measured UE configured maximum output power PUMAX,f,c for UL MIMO is within the specified bounds.

6.2D.4.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.2D.4.3	Minimum conformance requirements

For UE supporting UL MIMO, the transmitted power is configured per each UE.

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in subclause 6.2.4 shall apply to UE supporting UL MIMO, where

PPowerClass, ΔPPowerClass and TC,c are specified in subclause 6.2D.1.3;

MPRc is specified in subclause 6.2D.2.3;

A-MPRc is specified in subclause 6.2D.3.3.

The measured configured maximum output power PUMAX,c for serving cell c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL, T LOW(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T HIGH(PCMAX_H,c)

where TLOW(PCMAX_L,c) and THIGH(PCMAX_H,c) are defined as the tolerance and applies to PCMAX_L,c and PCMAX_H,c separately, while TL is the absolute value of the lower tolerance in Table 6.2D.1.3-1 for the applicable operating band.

For UE with two transmit antenna connectors in closed-loop spatial amultiplexing scheme, the tolerance is specified in Table 6.2D.4.3-1. The requirements shall be met with UL MIMO configurations specified in Table 6.2D.1.3-2.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the tolerance is specified in Table 6.2D.4.3-1. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

Table 6.2D.4.3-1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2D.1.3 apply for the power class as indicated by the ue-PowerClass field in capability signalling.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.4.

6.2D.4.4	Test description

6.2D.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2D.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

Table 6.2D.4.4.1-2: Test Configuration Table for uplink full power transmission (ULFPTx)

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| 33 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with PI/2 PBSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.4.4.3.

6.2D.4.4.2	Test procedure

Sub-test for 2-Layer codebook based UL-MIMO

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level of the test point.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under test.

Sub-test for ULFPTx

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.4.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.

6.2D.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO and following exception.

Table 6.2D.4.4.3-1: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 0 |  |  |

Table 6.2D.4.4.3-2: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 14 |  |  |

Table 6.2D.4.4.3-3: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  |  |

Table 6.2D.4.4.3-4: ServingCellConfig: ULFPTx

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 0 |  | ULFPTx Test ID 1, 2 |
|  | 1 |  | ULFPTx Test ID 3 |
| } |  |  |  |
| } |  |  |  |

6.2D.4.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2D.4.5-1.

Table 6.2D.4.5-1: PCMAX configured UE output power

| Configured transmitted power |  |
| --- | --- |
| Measured UE output power test point 1 | 0 dBm ± (7+TT) |
| Measured UE output power test point 2 | 14 dBm ± (6+TT) |
| Measured UE output power test point 3 | 20 dBm + (4+TT) / - (6+TT) |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2D.4.5-2. Note 2: In addition note 2 in Table 6.2D.1.3-1 shall apply to the tolerances. |  |

Table 6.2D.4.5-2: Test Tolerance (Configured transmitted power for UL MIMO)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2D.4_1	Configured transmitted power for SUL with UL MIMO

6.2D.4_1.1	Test purpose

Same test purpose as in clause 6.2D.4.1.

6.2D.4_1.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

This test applies to all types of NR UE release 17 and forward that support SUL and UL full power transmission (ULFPTx) on the SUL bands.

6.2D.4_1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2D.4.3.

6.2D.4_1.4	Test description

6.2D.4_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2D.4_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.4_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for both SUL carrier and Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | CP-OFDM QPSK | Inner Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |

Table 6.2D.4_1.4.1-2: Test Configuration Table for uplink full power transmission (ULFPTx)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for both SUL carrier and Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2 and G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.4_1.4.1-1 or 6.2D.4_1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.4_1.4.3.

6.2D.4_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.4_1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level of the test point.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols.

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.4_1.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.

6.2D.4_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, and Table 4.6.3-182 with the condition 2TX_UL_MIMO and following exception.

Table 6.2D.4_1.4.3-1: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 0 |  |  |

Table 6.2D.4_1.4.3-2: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 14 |  |  |

Table 6.2D.4_1.4.3-3: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 18 |  |  |

6.2D.4_1.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2D.4_1.5-1.

Table 6.2D.4_1.5-1: PCMAX configured UE output power

| Configured transmitted power |  |
| --- | --- |
| Measured UE output power test point 1 | 0 dBm ± (7+TT) |
| Measured UE output power test point 2 | 14 dBm ± (6+TT) |
| Measured UE output power test point 3 | 18 dBm ± (5+TT) |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2D.4_1.5-2. Note 2: In addition note 2 in Table 6.2D.1_1.3-1 shall apply to the tolerances. |  |

Table 6.2D.4_1.5-2: Test Tolerance (Configured transmitted power for UL MIMO)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

## 6.2G	Transmitter power for Tx Diversity

### 6.2G.1	UE maximum output power for Tx Diversity

Editor’s Note: The following aspects are either missing or not yet determined:

- No test points are defined for Power Class 1.5 non-FWA UEs since there is no configuration satisfying MPR=0dB requirements in TS 38.101-1. Testing with minimum MPR has been covered in 6.2G.2.

6.2G.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2G.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 FWA UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.2G.1.3	Minimum conformance requirements

For UE supporting Tx Diversity, the maximum output power as indicated by UE power class in Table 6.2G.1.3-1 is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1 ms).

When a UE indicates PC1.5 for a given band it achieves maximum power by means of TxD in the current version of the spec.

Table 6.2G.1.3-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n2 |  |  |  |  |  |  | 23 | ±23 |
| n3 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n5 |  |  |  |  |  |  | 23 | ±2 |
| n7 |  |  |  |  |  |  | 23 | ±23 |
| n8 |  |  |  |  |  |  | 23 | ±23 |
| n12 |  |  |  |  |  |  | 23 | ±23 |
| n14 |  |  |  |  |  |  | 23 | ±2 |
| n20 |  |  |  |  |  |  | 23 | ±23 |
| n24 |  |  |  |  |  |  | 23 | +2/-33 |
| n25 |  |  |  |  |  |  | 23 | ±23 |
| n26 |  |  |  |  |  |  | 23 | ±23 |
| n28 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  |  |  | 23 | ±2 |
| n34 |  |  | 297 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n38 |  |  |  |  |  |  | 23 | ±2 |
| n39 |  |  | 297 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n40 |  |  | 297 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n41 |  |  | 297 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  |  |  | 23 | ±2 |
| n51 |  |  |  |  |  |  | 23 | ±2 |
| n53 |  |  |  |  |  |  | 23 | ±2 |
| n54 |  |  |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  |  |  | 23 | ±2 |
| n66 |  |  |  |  |  |  | 23 | ±2 |
| n70 |  |  |  |  |  |  | 23 | ±2 |
| n71 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n74 |  |  |  |  |  |  | 23 | ±2 |
| n77 |  |  | 297 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n78 |  |  | 297 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 297 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  |  |  | 23 | ±23 |
| n81 |  |  |  |  |  |  | 23 | ±2 |
| n82 |  |  |  |  |  |  | 23 | ±2 |
| n83 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n84 |  |  |  |  |  |  | 23 | ±2 |
| n86 |  |  |  |  |  |  | 23 | ±2 |
| n95 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n97 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n99 |  |  |  |  |  |  | 23 | +2/-33 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: FFS NOTE 5: FFS NOTE 6: Generally, PC1 UE for Band n14 is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. NOTE 7: Achieved via dual Tx |  |  |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else if the UE does not support a power class with higher maximum output power than PC2; or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [6] is provided and set to the maximum output power of the power class 2 or lower;

-	shall apply all requirements for power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2.4.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.1 and 6.2G.1.

6.2G.1.4	Test description

6.2G.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2G.1.4.1-1 and Table 6.2G.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2G.1.4.1-1: Test Configuration Table for Power Class 2 indicating TxD support

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For P-Max test (Step 4 and Step 5 in Section 6.2G.1.4.2) and maxUplinkDutyCycle test (Step 6, Step 7, Step 8 and Step 9 in Section 6.2G.1.4.2), only “Normal” test environment, “Low range” test frequencies, “Lowest” Test Channel Bandwidth, “Lowest” test SCS and “Inner Full” RB allocation to be used. |  |  |  |

Table 6.2G.1.4.1-2: Test Configuration Table for Power Class 1.5 non-FWA UE

NOTE 1:	No test points are defined for Power Class 1.5 non-FWA UE since there is no configuration satisfying MPR=0dB requirements in TS 38.101-1.

Table 6.2G.1.4.1-3: Test Configuration Table for Power Class 3 indicating TxD support

NOTE 2:	Test points defined for single Tx Power Class 3 in Table Table 6.2.2.3-1 to be reused as test points defined for Power Class 3 indicating TxD support since single Tx Power Class 3 and TxD Power Class 3 share the same MPR requirements.

Table 6.2G.1.4.1-4: Test Configuration Table for Power Class 1.5 FWA UE

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2G.1.4.1-1 and Table 6.2G.1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2G.1.4.3.

6.2G.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2G.1.4.1-1, Table 6.2G.1.4.1-3 and Table 6.2G.1.4.1-4. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

4.	For UEs supporting Power Class 1.5, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2G.1.4.3-2. For P-Max=23, UL RMC shall be set according to Table 6.2G.1.4.1-3. For P-Max=26, UL RMC shall be set according to Table 6.2G.1.4.1-1.

5.	For UEs supporting Power Class 2, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2G.1.4.3-2. UL RMC shall be set according to Table 6.2G.1.4.1-3.

6.	For UEs supporting Power Class 1.5 with UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16, repeat steps 1~3 on the applicable bands with message exception of TDD UL-DL pattern defined in Table 6.2.1.4.3-4, Table 6.2.1.4.3-5, Table 6.2G.1.4.3-3, Table 6.2G.1.4.3-4, Table 6.2G.1.4.3-5, Table 6.2G.1.4.3-6, Table 6.2G.1.4.3-7, Table 6.2G.1.4.3-8, Table 6.2G.1.4.3-9 and Table 6.2G.1.4.3-10 which the UplinkDutyCycle is closest less than or equal to the maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 reported by UE. Before Step 3, wait for at least 10ms for the UE to complete the evaluation period. UL RMC shall be set according to Table 6.2G.1.4.1-4.

7.	For UEs supporting Power Class 1.5 with UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16, repeat steps 1~3 on the applicable bands with message exception of TDD UL-DL pattern defined in Table 6.2.1.4.3-4, Table 6.2.1.4.3-5, Table 6.2G.1.4.3-3, Table 6.2G.1.4.3-4, Table 6.2G.1.4.3-5, Table 6.2G.1.4.3-6, Table 6.2G.1.4.3-7, Table 6.2G.1.4.3-8, Table 6.2G.1.4.3-9 and Table 6.2G.1.4.3-10 which the UplinkDutyCycle is closest larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 and half the UplinkDutyCycle is no larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 reported by UE. Before Step 3, wait for at least 10ms for the UE to complete the evaluation period. UL RMC shall be set according to Table 6.2G.1.4.1-1.

8.	For UEs supporting Power Class 1.5 with UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16, repeat steps 1~3 on the applicable bands with message exception of TDD UL-DL pattern defined in Table 6.2.1.4.3-4, Table 6.2.1.4.3-5, Table 6.2G.1.4.3-3, Table 6.2G.1.4.3-4, Table 6.2G.1.4.3-5, Table 6.2G.1.4.3-6, Table 6.2G.1.4.3-7, Table 6.2G.1.4.3-8, Table 6.2G.1.4.3-9 and Table 6.2G.1.4.3-10 which half the UplinkDutyCycle is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 reported by UE. Before Step 3, wait for at least 10ms for the UE to complete the evaluation period. UL RMC shall be set according to Table 6.2G.1.4.1-3.

9.	For TDD UEs supporting Power Class 2 with UE capability maxUplinkDutyCycle-PC2-FR1, repeat steps 1~3 on the applicable bands with message exception of TDD UL-DL pattern defined in Table 6.2.1.4.3-4 and Table 6.2.1.4.3-5 which the UplinkDutyCycle is closest less than or equal to the maxUplinkDutyCycle-PC2-FR1 reported by UE. Before Step 3, wait for at least 10ms for the UE to complete the evaluation period.

6.2G.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

UplinkDutyCycle in Table 6.2G.1.4.3-3, Table 6.2G.1.4.3-4, Table 6.2G.1.4.3-5, Table 6.2G.1.4.3-6, Table 6.2G.1.4.3-7, Table 6.2G.1.4.3-8, Table 6.2G.1.4.3-9 and Table 6.2G.1.4.3-10 is the percentage of uplink slots transmitted in one radio frame.

Table 6.2G.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2G.1.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 | PC2 UE or PC1.5 UE will fallback to PC3 UE with P-Max=23 | PC2 UE or PC1.5 UE |
|  | 26 | PC1.5 UE will fallback to PC2 UE with P-Max=26 | PC1.5 UE |

Table 6.2G.1.4.3-3: TDD UL-DL pattern for SCS 15 KHz (UplinkDutyCycle=10%)

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | 7DS2U |
| Special Slot Configuration (Note 2) |  |  | 6D+4G+4U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 15 |
|  | dl-UL-TransmissionPeriodicity | ms | 10 |
|  | nrofDownlinkSlots |  | 7 |
|  | nrofDownlinkSymbols |  | 6 |
|  | nrofUplinkSlot |  | 2 |
|  | nrofUplinkSymbols |  | 4 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [8] if mod(i,10) = 0 [7] if mod(i,10) = 1 [6] if mod(i,10) = 2 [5] if mod(i,10) = 3 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,9} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 7 and slot 9 to meet the specific UplinkDutyCycle. |  |  |  |

Table 6.2G.1.4.3-4: TDD UL-DL pattern for SCS 15 KHz (UplinkDutyCycle=20%)

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | 7DS2U |
| Special Slot Configuration (Note 2) |  |  | 6D+4G+4U4 |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 15 |
|  | dl-UL-TransmissionPeriodicity | ms | 10 |
|  | nrofDownlinkSlots |  | 7 |
|  | nrofDownlinkSymbols |  | 6 |
|  | nrofUplinkSlot |  | 2 |
|  | nrofUplinkSymbols |  | 4 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [8] if mod(i,10) = 0 [7] if mod(i,10) = 1 [6] if mod(i,10) = 2 [5] if mod(i,10) = 3 [5] if mod(i,10) = 4 [4] if mod(i,10) = 5 [3] if mod(i,10) = 6 [2] if mod(i,10) = 7 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,9} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 7 to meet the specific UplinkDutyCycle. |  |  |  |

Table 6.2G.1.4.3-5: TDD UL-DL pattern for SCS 15 KHz (UplinkDutyCycle=30%)

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | DDDSUDDSUU |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U |
| UL-DL configuration1 (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 15 |
|  | dl-UL-TransmissionPeriodicity | ms | 5 dual |
|  | nrofDownlinkSlots |  | 3 |
|  | nrofDownlinkSymbols |  | 10 |
|  | nrofUplinkSlot |  | 1 |
|  | nrofUplinkSymbols |  | 2 |
| UL-DL configuration2 (tdd-UL-DL-ConfigurationCommon2) | referenceSubcarrierSpacing | kHz | 30 |
|  | dl-UL-TransmissionPeriodicity | ms | 5 dual |
|  | nrofDownlinkSlots |  | 2 |
|  | nrofDownlinkSymbols |  | 10 |
|  | nrofUplinkSlot |  | 2 |
|  | nrofUplinkSymbols |  | 2 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [4] if mod(i,10) = 0 [3] if mod(i,10) = 1 [6] if mod(i,10) = 2 [5] if mod(i,10) = 3 [4] if mod(i,10) = 5 [3] if mod(i,10) = 6 [2] if mod(i,10) = 7 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,9} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 3 and slot 7 to meet the specific UplinkDutyCycle. |  |  |  |

Table 6.2G.1.4.3-6: TDD UL-DL pattern for SCS 15 KHz (UplinkDutyCycle=40%)

| Parameter |  | Unit | UL-DL patte4rn |
| --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | DDSUU |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 15 |
|  | dl-UL-TransmissionPeriodicity | ms | 5 |
|  | nrofDownlinkSlots |  | 2 |
|  | nrofDownlinkSymbols |  | 10 |
|  | nrofUplinkSlot |  | 2 |
|  | nrofUplinkSymbols |  | 2 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [4] if mod(i,5) = 0 [7] if mod(i,5) = 1 [6] if mod(i,5) = 2 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,9} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 2 and slot 7 to meet the specific UplinkDutyCycle. |  |  |  |

Table6.2G.1.4.3-7: TDD UL-DL pattern for SCS 30 KHz (UplinkDutyCycle=10%)

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | 7DS2U |
| Special Slot Configuration (Note 2) |  |  | 6D+4G+4U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 30 |
|  | dl-UL-TransmissionPeriodicity | ms | 5 |
|  | nrofDownlinkSlots |  | 7 |
|  | nrofDownlinkSymbols |  | 6 |
|  | nrofUplinkSlot |  | 2 |
|  | nrofUplinkSymbols |  | 4 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [8] if mod(i,10) = 0 [7] if mod(i,10) = 1 [6] if mod(i,10) = 2 [5] if mod(i,10) = 3 [5] if mod(i,10) = 4 [4] if mod(i,10) = 5 [3] if mod(i,10) = 6 [2] if mod(i,10) = 7 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,19} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 7, slot 17, slot 18 and slot 19 to meet the specific UplinkDutyCycle. |  |  |  |

Table6.2G.1.4.3-8: TDD UL-DL pattern for SCS 30 KHz (UplinkDutyCycle=20%)

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | 7DS2U |
| Special Slot Configuration (Note 2) |  |  | 6D+4G+4U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 30 |
|  | dl-UL-TransmissionPeriodicity | ms | 5 |
|  | nrofDownlinkSlots |  | 7 |
|  | nrofDownlinkSymbols |  | 6 |
|  | nrofUplinkSlot |  | 2 |
|  | nrofUplinkSymbols |  | 4 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [8] if mod(i,10) = 0 [7] if mod(i,10) = 1 [6] if mod(i,10) = 2 [5] if mod(i,10) = 3 [5] if mod(i,10) = 4 [4] if mod(i,10) = 5 [3] if mod(i,10) = 6 [2] if mod(i,10) = 7 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,19} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 7 and slot 17 to meet the specific UplinkDutyCycle. |  |  |  |

Table 6.2G.1.4.3-9: TDD UL-DL pattern for SCS 30 KHz (UplinkDutyCycle=30%)

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | DDDSU DDSUU |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U4 |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 30 |
|  | dl-UL-TransmissionPeriodicity | ms | 2.5 dual |
|  | nrofDownlinkSlots |  | 3 |
|  | nrofDownlinkSymbols |  | 10 |
|  | nrofUplinkSlot |  | 1 |
|  | nrofUplinkSymbols |  | 2 |
| UL-DL configuration2 (tdd-UL-DL-ConfigurationCommon2) | referenceSubcarrierSpacing | kHz | 30 |
|  | dl-UL-TransmissionPeriodicity | ms | 2.5 dual |
|  | nrofDownlinkSlots |  | 2 |
|  | nrofDownlinkSymbols |  | 10 |
|  | nrofUplinkSlot |  | 2 |
|  | nrofUplinkSymbols |  | 2 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [4] if mod(i,10) = 0 [3] if mod(i,10) = 1 [6] if mod(i,10) = 2 [5] if mod(i,10) = 3 [4] if mod(i,10) = 5 [3] if mod(i,10) = 6 [2] if mod(i,10) = 7 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,19} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 3, slot 7, slot 13 and slot 17 to meet the specific UplinkDutyCycle. |  |  |  |

Table 6.2G.1.4.3-10: TDD UL-DL pattern for SCS 30 KHz (UplinkDutyCycle=40%)

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | DDSUU |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U4 |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 30 |
|  | dl-UL-TransmissionPeriodicity | ms | 2.5 single |
|  | nrofDownlinkSlots |  | 2 |
|  | nrofDownlinkSymbols |  | 10 |
|  | nrofUplinkSlot |  | 2 |
|  | nrofUplinkSymbols |  | 2 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | [4] if mod(i,5) = 0 [7] if mod(i,5) = 1 [6] if mod(i,5) = 2 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,19} Note 4: There shall be no PUSCH or PUCCH or SRS transmitted in slot 2, slot 7, slot 12 and slot 17 to meet the specific UplinkDutyCycle. |  |  |  |

6.2G.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.1.5-1 for Power Class 2, Table 6.2G.1.5-2 for Power Class 1.5 and Table 6.2G.1.5-4 for Power Class 3.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.1.5-4 for Power Class 3 and Table 6.2G.1.5-1 for Power Class 2.

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.1.5-4 for Power Class 3.

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.1.5-2 for Power Class 1.5.

The maximum output power, derived in step 7 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.1.5-1 for Power Class 2.

The maximum output power, derived in step 8 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.1.5-4 for Power Class 3.

The maximum output power, derived in step 9 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.1.5-1 for Power Class 2.

Table 6.2G.1.5-1: Maximum Output Power test requirement for Power Class 2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2+TT/-3-TT |  |  |
| n3 |  |  | 26 | +2+TT/-33-TT |  |  |
| n34 |  |  | 26 | +2+TT/-3-TT |  |  |
| n39 |  |  | 26 | +2+TT/-3-TT |  |  |
| n40 |  |  | 26 | +2+TT/-3-TT |  |  |
| n41 |  |  | 26 | +2+TT/-33-TT |  |  |
| n77 |  |  | 26 | +2+TT/-3-TT |  |  |
| n78 |  |  | 26 | +2+TT/-3-TT |  |  |
| n79 |  |  | 26 | +2+TT/-3-TT |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2G.1.5-3 |  |  |  |  |  |  |

Table 6.2G.1.5-2: Maximum Output Power test requirement for Power Class 1.5

| NR band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n34 | 295 | +2+TT/-3-TT |  |  |  |  |
| n39 | 295 | +2+TT/-3-TT |  |  |  |  |
| n40 | 295 | +2+TT/-3-TT |  |  |  |  |
| n41 | 295 | +2+TT/-33-TT |  |  |  |  |
| n77 | 295 | +2+TT/-3-TT |  |  |  |  |
| n78 | 295 | +2+TT/-3-TT |  |  |  |  |
| n79 | 295 | +2+TT/-3-TT |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2G.1.5-3 NOTE 5: Achieved via dual Tx NOTE 6: No test required since there is no satisfying test points defined. |  |  |  |  |  |  |

Table 6.2G.1.5-3: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2G.1.5-4: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | +2+TT/-3-TT |
| n2 |  |  |  |  | 23 | +2+TT/-33-TT |
| n3 |  |  |  |  | 23 | +2+TT/-33-TT |
| n5 |  |  |  |  | 23 | +2+TT/-3-TT |
| n7 |  |  |  |  | 23 | +2+TT/-33-TT |
| n8 |  |  |  |  | 23 | +2+TT/-33-TT |
| n12 |  |  |  |  | 23 | +2+TT/-33-TT |
| n14 |  |  |  |  | 23 | +2+TT/-3-TT |
| n20 |  |  |  |  | 23 | +2+TT/-33-TT |
| n24 |  |  |  |  | 23 | +2+TT/-33-TT |
| n25 |  |  |  |  | 23 | +2+TT/-33-TT |
| n26 |  |  |  |  | 23 | +2+TT/-33-TT |
| n28 |  |  |  |  | 23 | +2+TT/-3-TT |
| n30 |  |  |  |  | 23 | +2+TT/-3-TT |
| n34 |  |  |  |  | 23 | +2+TT/-3-TT |
| n38 |  |  |  |  | 23 | +2+TT/-3-TT |
| n39 |  |  |  |  | 23 | +2+TT/-3-TT |
| n40 |  |  |  |  | 23 | +2+TT/-3-TT |
| n41 |  |  |  |  | 23 | +2+TT/-33-TT |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | +2+TT/-3-TT |
| n51 |  |  |  |  | 23 | +2+TT/-3-TT |
| n53 |  |  |  |  | 23 | +2+TT/-3-TT |
| n54 |  |  |  |  | 23 | +2+TT/-3-TT |
| n65 |  |  |  |  | 23 | +2+TT/-3-TT |
| n66 |  |  |  |  | 23 | +2+TT/-3-TT |
| n70 |  |  |  |  | 23 | +2+TT/-3-TT |
| n71 |  |  |  |  | 23 | +2+TT/-3-TT |
| n74 |  |  |  |  | 23 | +2+TT/-3-TT |
| n77 |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  | 23 | +2+TT/-3-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2G.1.5-3 |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2G.2	UE maximum output power reduction for Tx Diversity

6.2G.2.1	Test purpose

The number of RB identified in Table 6.2D.2.3-1 and Table 6.2D.2.3-2 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2G.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5G.2.3.1 is executed.

6.2G.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the allowed MPR for the maximum output power is specified in Table 6.2.2-1, Table 6.2D.2.3-1, Table 6.2D.2.3-2 and Table 6.2D.2.3-3 for UE power class 3, 2 and 1.5 respectively. For UE power class 1.5, the allowed maximum power reduction (MPR) defined in Table 6.2D.2.3-3 is in accordance with the indicated modifiedMPR-Behavior specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The maximum output power is defined as the sum of the maximum output power at each UE antenna connector. If a UE that supports PC1.5 has to apply the requirements of PC2 according to the rules in clause 6.2.1, the MPR requirements in Table 6.2.2-2 apply.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2G.4 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2G.2.

6.2G.2.4	Test description

6.2G.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2G.2.4.1-1, Table 6.2G.2.4.1-2, and Table 6.2G.2.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2G.2.4.1-1: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

Table 6.2G.2.4.1-2: Test Configuration Table for power class 1.5 UE and power class 1.5 FWA (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Default |  | DFT-s-OFDM_64_QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 17 | Default |  | DFT-s-OFDM_256_QAM | Inner Full |
| 18 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 19 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 20 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 21 | Default |  | CP-OFDM QPSK | Inner Full |
| 22 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 23 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM QPSK | Outer Full |
| 25 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 26 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 27 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 28 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 29 | Default |  | CP-OFDM 64 QAM | Inner Full |
| 30 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 33 | Default |  | CP-OFDM 256 QAM | Inner Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

Table 6.2G.2.4.1-2a: Test Configuration Table for power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 23 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 54 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 64 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 31 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 32 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 374,5 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 384,5 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 394,5 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

Table 6.2G.2.4.1-3: Test Configuration Table for power class 2 and 3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2.2.4.1-4, 6.2.2.4.1-4a and 6.2.2.4.1-4b. NOTE 2: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2G.2.4.1-1, Table 6.2G.2.4.1-2 and Table 6.2G.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2G.2.4.3.

6.2G.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2G.2.4.1-1, Table 6.2G.2.4.1-2, Table 6.2G.2.4.1-2a and Table 6.2G.2.4.1-3. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	For UEs supporting Power Class 1.5 and Power Class 2, repeat steps 1~3 for Test ID 16 and 22 in Table 6.2G.2.4.1-2a on the applicable bands with message exception of P-Max defined in Table 6.2G.2.4.3-2.

5.	For UEs supporting Power Class 1.5, repeat steps 1~3 for all Test IDs in Table 6.2G.2.4.1-1 on the applicable bands with message exception of P-Max defined in Table 6.2G.2.4.3-3.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2G.2.4.1-1 and Table 6.2G.2.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2G.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

Table 6.2G.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.2G.2.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE or PC1.5 UE |

Table 6.2G.2.4.3-3: P-Max (Step 5)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 26 |  | PC1.5 UE |

6.2G.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.2.5-1 to Table 6.2G.2.5-12.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.2.5-5 to Table 6.2G.2.5-12.

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.2.5-2b.

Table 6.2G.2.5-1: UE Power Class test requirements (for Bands n34, n39, n40, n41, n77, n78, n79) for Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 4 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5- TT2） |
| 5 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 9 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 12 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 15 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 16 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 17 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 18 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 19 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 20 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 21 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5- TT2） |
| 22 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 23 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 24 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 26 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 27 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 28 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 29 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 30 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 31 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 32 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-2: UE Power Class test requirements (for Band n34, n39, n40, n41, n77, n78, n79) for Power Class 1.5 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | (27.0²) | 3.0 |  | 3 | 31.0 + TT | 25.5 - TT | (24.0 - TT²) |
| 2 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 31.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 3 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 31.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 4 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | (25.5²) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 - TT²) |
| 5 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | (27.0²) | 3.0 |  | 3 | 31.0 + TT | 25.5 - TT | (24.0 - TT²) |
| 6 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 7 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 8 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | (25.0²) | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | (22.0 - TT²) |
| 9 | 29 | 0 | 1.5 | 0 | （1.52） | 27.5 | (26.0²) | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | (23.0 - TT²) |
| 10 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 11 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 12 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | (24.0²) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 - TT²) |
| 13 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | (24.0²) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 - TT²) |
| 14 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 15 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 16 | 29 | 0 | 4 | 0 | （1.52） | 25.0 | (23.5²) | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | (20.5 - TT²) |
| 17 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 18 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 19 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 20 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 21 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | (25.5²) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 - TT²) |
| 22 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 23 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 24 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 25 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | (25.0²) | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | (22.0 - TT²) |
| 26 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 27 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 29 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 30 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 31 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 32 | 29 | 0 | 5 | 0 | （1.52） | 24.0 | (22.5²) | 3.0 | （5.02） | 3 | 31.0 + TT | 21.0 - TT | (17.5 - TT²) |
| 33 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| 34 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| 35 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| 36 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-2a: UE Power Class test requirements (for Band n34, n39, n40, n41, n77, n78, n79) for Power Class 1.5 FWA (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | (27.53) | 3.0 |  | 3 | 31.0 + TT | 26.0 - TT | (24.5 – TT3) |
| 2 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | (21.53) | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 - TT | (16.5 – TT3) |
| 3 | 29 | 0 | 6 | 0 | （1.53） | 23.0 | (21.53) | 3.0 | (5.03) | 3 | 31.0 + TT | 20.0 - TT | (16.5 – TT3) |
| 4 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | (26.03) | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | (23.0 – TT3) |
| 5 | 29 | 0 | 0 | 0 | （1.53） | 29.0 | (27.53) | 3.0 |  | 3 | 31.0 + TT | 26.0 - TT | (24.5 – TT3) |
| 6 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 7 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 8 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | (25.53) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 – TT3) |
| 9 | 29 | 0 | 1 | 0 | （1.53） | 28.0 | (26.53) | 3.0 |  | 3 | 31.0 + TT | 25.0 - TT | (23.5 – TT3) |
| 10 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 11 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 12 | 29 | 0 | 3.5 | 0 | （1.53） | 25.5 | (24.03) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 – TT3) |
| 13 | 29 | 0 | 3 | 0 | （1.53） | 26.0 | (24.53) | 3.0 |  | 3 | 31.0 + TT | 23.0 - TT | (21.5 – TT3) |
| 14 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 15 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 16 | 29 | 0 | 3.5 | 0 | （1.53） | 25.5 | (24.03) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 – TT3) |
| 17 | 29 | 0 | 5.5 | 0 | （1.53） | 23.5 | (22.03) | 3.0 |  | 3 | 31.0 + TT | 20.5 - TT | (19.0 – TT3) |
| 18 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 19 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 20 | 29 | 0 | 5.5 | 0 | （1.53） | 23.5 | (22.03) | 3.0 | (5.03) | 3 | 31.0 + TT | 20.5 - TT | (17.0 – TT3) |
| 21 | 29 | 0 | 1.5 | 0 | （1.53） | 27.5 | (26.03) | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | (23.0 – TT3) |
| 22 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 23 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 24 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | (23.53) | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | (20.5 – TT3) |
| 25 | 29 | 0 | 2 | 0 | （1.53） | 27.0 | (25.53) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 – TT3) |
| 26 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 27 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | (23.53) | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | (20.5 – TT3) |
| 29 | 29 | 0 | 4 | 0 | （1.53） | 25.0 | (23.53) | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | (20.5 – TT3) |
| 30 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 31 | 29 | 0 | 6.5 | 0 | （1.53） | 22.5 | (21.03) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 – TT3) |
| 32 | 29 | 0 | 4.5 | 0 | （1.53） | 24.5 | (23.03) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 – TT3) |
| 33 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | (20.03) | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | (14.0 – TT3) |
| 34 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | (20.03) | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | (14.0 – TT3) |
| 35 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | (20.03) | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | (14.0 – TT3) |
| 36 | 29 | 0 | 7.5 | 0 | （1.53） | 21.5 | (20.03) | 5.0 | （6.03） | 3 | 31.0 + TT | 16.5 - TT | (14.0 – TT3) |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-2b: UE Power Class test requirements (for Band n34, n39, n40 n41, n77, n78, n79) for Power Class 1.5 falling back to Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 3 | 0 | 0 | （1.52） | 26.0 | (24.5²) | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | (21.5 - TT²) |
| 2 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 3 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 4 | 29 | 3 | 0.5 | 0 | （1.52） | 25.5 | (24.0²) | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | (21.0 - TT²) |
| 5 | 29 | 3 | 0 | 0 | （1.52） | 26.0 | (24.5²) | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | (21.5 - TT²) |
| 6 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 7 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 8 | 29 | 3 | 1 | 0 | （1.52） | 25.0 | (23.5²) | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | (20.5 - TT²) |
| 9 | 29 | 3 | 1 | 0 | （1.52） | 25.0 | (23.5²) | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | (20.5 - TT²) |
| 10 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 11 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 12 | 29 | 3 | 2 | 0 | （1.52） | 24.0 | (22.5²) | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | (17.5 - TT²) |
| 13 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 14 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 15 | 29 | 3 | 2.5 | 0 | （1.52） | 23.5 | (22.0²) | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | (17.0 - TT²) |
| 16 | 29 | 3 | 4.5 | 0 | （1.52） | 21.5 | (20.0²) | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | (14.0 - TT²) |
| 17 | 29 | 3 | 4.5 | 0 | （1.52） | 21.5 | (20.0²) | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | (14.0 - TT²) |
| 18 | 29 | 3 | 4.5 | 0 | （1.52） | 21.5 | (20.0²) | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | (14.0 - TT²) |
| 19 | 29 | 3 | 1.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 20 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 21 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 22 | 29 | 3 | 3 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 23 | 29 | 3 | 2 | 0 | （1.52） | 24.0 | (22.5²) | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | (17.5 - TT²) |
| 24 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 25 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 26 | 29 | 3 | 3 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 27 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 28 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 29 | 29 | 3 | 3.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 30 | 29 | 3 | 6.5 | 0 | （1.52） | 19.5 | (18.0²) | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | (13.0 - TT²) |
| 31 | 29 | 3 | 6.5 | 0 | （1.52） | 19.5 | (18.0²) | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | (13.0 - TT²) |
| 32 | 29 | 3 | 6.5 | 0 | （1.52） | 19.5 | (18.0²) | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | (13.0 - TT²) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-3: UE Power Class test requirements (for Bands n34, n39, n40, n41, n77, n78, n79) for Power Class 2 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 26 | 0 | 2 | 1.5 | 0 | 1.52 | 22.5 | 21.02 | 5.0 | 5.02 | 3 | 28.0 + TT | 17.5 - TT | 16 - TT2 |
| 15 | 26 | 0 | 2 | 1 | 0 | 1.52 | 23.0 | 21.52 | 3.0 | 5.02 | 3 | 28.0 + TT | 20.0 - TT | 16.5 - TT2 |
| 26 | 26 | 0 | 3.5 | 1.5 | 0 | 1.52 | 21.0 | 19.52 | 5.0 | 5.02 | 3 | 28.0 + TT | 16.0 - TT | 14.5 - TT2 |
| 27 | 26 | 0 | 3.5 | 1 | 0 | 1.52 | 21.5 | 20.02 | 5.0 | 6.02 | 3 | 28.0 + TT | 16.5 - TT | 14.0 - TT2 |
| 34 | 26 | 0 | 2.5 | 1.5 | 0 | 1.52 | 22.0 | 20.52 | 5.0 | 6.02 | 3 | 28.0 + TT | 17.0 - TT | 14.5 - TT2 |
| 35 | 26 | 0 | 2.5 | 1 | 0 | 1.52 | 22.5 | 21.02 | 5.0 | 5.02 | 3 | 28.0 + TT | 17.5 - TT | 16.0 - TT2 |
| 46 | 26 | 0 | 3.5 | 1.5 | 0 | 1.52 | 21.0 | 19.52 | 5.0 | 5.02 | 3 | 28.0 + TT | 16.0 - TT | 14.5 - TT2 |
| 47 | 26 | 0 | 3.5 | 1 | 0 | 1.52 | 21.5 | 20.02 | 5.0 | 6.02 | 3 | 28.0 + TT | 16.5 - TT | 14.0 - TT2 |
| 56 | 26 | 0 | 4.5 | 1.5 | 0 | 1.52 | 20.0 | 18.52 | 6.0 | 5.02 | 3 | 28.0 + TT | 14.0 - TT | 13.5 - TT2 |
| 57 | 26 | 0 | 4.5 | 1 | 0 | 1.52 | 20.5 | 19.02 | 6.0 | 5.02 | 3 | 28.0 + TT | 14.5 - TT | 14.0 - TT2 |
| 66 | 26 | 0 | 8 | 1.5 | 0 | 1.52 | 16.5 | 152 | 5.0 | 5.02 | 3 | 28.0 + TT | 11.5 - TT | 10.0 - TT2 |
| 67 | 26 | 0 | 8 | 1 | 0 | 1.52 | 17.0 | 15.52 | 5.0 | 5.02 | 3 | 28.0 + TT | 12.0 - TT | 10.5 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 8: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-4: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2G.2.5-5: UE Power Class test requirements(for Bands n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n54, n65, n66, n70, n74) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | （1.52） | 25.8 | （24.32） | 3.0 |  | 2 | 28.0 + TT | 22.8 - TT | （21.3 - TT2） |
| 2 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 4 | 23 | -3 | 1.2 | 0 | （1.52） | 24.8 | （23.32） | 3.0 |  | 2 | 28.0 + TT | 21.8 - TT | （20.3 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 2 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 2 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 2 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 2 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 2 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 2 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 2 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 2 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 2 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 2 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 2 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 2 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 2 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 2 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 2 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 2 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | (16.0 - TT2) |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 2 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25, n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-6: UE Power Class test requirements (for Bands n24, n48, n77, n78, n79) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | （1.52） | 25.8 | （24.32） | 3.0 |  | 3 | 28.0 + TT | 22.8 - TT | （21.3 - TT2） |
| 2 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 4 | 23 | -3 | 1.2 | 0 | （1.52） | 24.8 | （23.32） | 3.0 |  | 3 | 28.0 + TT | 21.8 - TT | （20.3 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 5.0 |  | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 |  | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 2 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 2 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 2 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 2 | 25.0 + TT | 17.5 - TT | (16.0 - TT2) |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 2 | 25.0 + TT | 20.0 - TT | （16.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n24, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-7: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 3.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 6 | 23 | 0 | 0.5 | 0 | 22.5 | 5.0 | 2.5 | 25.0 + TT | 17.5 - TT |
| 7 | 23 | 0 | 0.5 | 0 | 22.5 | 5.0 | 2.5 | 25.0 + TT | 17.5 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 5.0 | 2.5 | 25.0 + TT | 17.5 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 3.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 5.0 | 2.5 | 25.0 + TT | 17.0 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 5.0 | 2.5 | 25.0 + TT | 17.0 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 5.0 | 2.5 | 25.0 + TT | 17.0 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 5.0 | 2.5 | 25.0 + TT | 17.0 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 5.0 | 2.5 | 25.0 + TT | 16.0 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 5.0 | 2.5 | 25.0 + TT | 16.0 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 5.0 | 2.5 | 25.0 + TT | 16.0 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 6.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 6.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 6.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 5.0 | 2.5 | 25.0 + TT | 13.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 5.0 | 2.5 | 25.0 + TT | 13.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 5.0 | 2.5 | 25.0 + TT | 13.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 5.0 | 2.5 | 25.0 + TT | 16.5 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 6.0 | 2.5 | 25.0 + TT | 14.0 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 6.0 | 2.5 | 25.0 + TT | 14.0 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 6.0 | 2.5 | 25.0 + TT | 14.0 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 5.0 | 2.5 | 25.0 + TT | 16.0 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 6.0 | 2.5 | 25.0 + TT | 14.0 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 6.0 | 2.5 | 25.0 + TT | 14.0 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 6.0 | 2.5 | 25.0 + TT | 14.0 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 5.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 5.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 5.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 5.0 | 2.5 | 25.0 + TT | 17.5 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 5.0 | 2.5 | 25.0 + TT | 17.5 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 3.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-8: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0.5 | 0 | 22.5 | 5 | 2.5 | 25.0 + TT | 17.5-TT |
| 6 | 23 | 0 | 1 | 0 | 22 | 5 | 2.5 | 25.0 + TT | 17.0-TT |
| 7 | 23 | 0 | 1 | 0 | 22 | 5 | 2.5 | 25.0 + TT | 17.0-TT |
| 8 | 23 | 0 | 1 | 0 | 22 | 5 | 2.5 | 25.0 + TT | 17.0-TT |
| 9 | 23 | 0 | 0.5 | 0 | 22.5 | 5 | 2.5 | 25.0 + TT | 17.5-TT |
| 10 | 23 | 0 | 1.5 | 0 | 21.5 | 5 | 2.5 | 25.0 + TT | 16.5-TT |
| 11 | 23 | 0 | 1.5 | 0 | 21.5 | 5 | 2.5 | 25.0 + TT | 16.5-TT |
| 12 | 23 | 0 | 1.5 | 0 | 21.5 | 5 | 2.5 | 25.0 + TT | 16.5-TT |
| 13 | 23 | 0 | 1.5 | 0 | 21.5 | 5 | 2.5 | 25.0 + TT | 16.5-TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 6 | 2.5 | 25.0 + TT | 14.5-TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 6 | 2.5 | 25.0 + TT | 14.5-TT |
| 16 | 23 | 0 | 2.5 | 0 | 20.5 | 6 | 2.5 | 25.0 + TT | 14.5-TT |
| 17 | 23 | 0 | 3 | 0 | 20 | 6 | 2.5 | 25.0 + TT | 14.0-TT |
| 18 | 23 | 0 | 3 | 0 | 20 | 6 | 2.5 | 25.0 + TT | 14.0-TT |
| 19 | 23 | 0 | 3 | 0 | 20 | 6 | 2.5 | 25.0 + TT | 14.0-TT |
| 20 | 23 | 0 | 5 | 0 | 18 | 5 | 2.5 | 25.0 + TT | 13.0-TT |
| 21 | 23 | 0 | 5 | 0 | 18 | 5 | 2.5 | 25.0 + TT | 13.0-TT |
| 22 | 23 | 0 | 5 | 0 | 18 | 5 | 2.5 | 25.0 + TT | 13.0-TT |
| 23 | 23 | 0 | 2 | 0 | 21 | 5 | 2.5 | 25.0 + TT | 16.0-TT |
| 24 | 23 | 0 | 3.5 | 0 | 19.5 | 5 | 2.5 | 25.0 + TT | 14.5-TT |
| 25 | 23 | 0 | 3.5 | 0 | 19.5 | 5 | 2.5 | 25.0 + TT | 14.5-TT |
| 26 | 23 | 0 | 3.5 | 0 | 19.5 | 5 | 2.5 | 25.0 + TT | 14.5-TT |
| 27 | 23 | 0 | 2.5 | 0 | 20.5 | 6 | 2.5 | 25.0 + TT | 14.5-TT |
| 28 | 23 | 0 | 3.5 | 0 | 19.5 | 5 | 2.5 | 25.0 + TT | 14.5-TT |
| 29 | 23 | 0 | 3.5 | 0 | 19.5 | 5 | 2.5 | 25.0 + TT | 14.5-TT |
| 30 | 23 | 0 | 3.5 | 0 | 19.5 | 5 | 2.5 | 25.0 + TT | 14.5-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 5 | 2.5 | 25.0 + TT | 14.0-TT |
| 32 | 23 | 0 | 4 | 0 | 19 | 5 | 2.5 | 25.0 + TT | 14.0-TT |
| 33 | 23 | 0 | 4 | 0 | 19 | 5 | 2.5 | 25.0 + TT | 14.0-TT |
| 34 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11.0-TT |
| 35 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11.0-TT |
| 36 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11.0-TT |
| 37 | 23 | 0 | 1 | 0 | 22 | 5 | 2.5 | 25.0 + TT | 17.0-TT |
| 38 | 23 | 0 | 1 | 0 | 22 | 5 | 2.5 | 25.0 + TT | 17.0-TT |
| 39 | 23 | 0 | 0.5 | 0 | 22.5 | 5 | 2.5 | 25.0 + TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-9: UE Power Class test requirements (for Bands n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n54, n65, n66, n70, n74) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 23 | 0 | 1.5 | 1.5 | 0 | （1.52） | 20 | （18.52） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 15 | 23 | 0 | 1.5 | 1 | 0 | （1.52） | 20.5 | （192） | 6.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 26 | 23 | 0 | 3 | 1.5 | 0 | （1.52） | 18.5 | （172） | 5.0 | （5.02） | 2 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 27 | 23 | 0 | 3 | 1 | 0 | （1.52） | 19 | （17.52） | 5.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （12.5 - TT2） |
| 34 | 23 | 0 | 2 | 1.5 | 0 | （1.52） | 19.5 | （182） | 5.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 35 | 23 | 0 | 2 | 1 | 0 | （1.52） | 20 | （18.52） | 6.0 | （6.02） | 2 | 25.0 + TT | 14.0 - TT | （12.5 - TT2） |
| 46 | 23 | 0 | 3 | 1.5 | 0 | （1.52） | 18.5 | （172） | 5.0 | （5.02） | 2 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 47 | 23 | 0 | 3 | 1 | 0 | （1.52） | 19 | （17.52） | 5.0 | （5.02） | 2 | 25.0 + TT | 14.0 - TT | （12.5 - TT2） |
| 56 | 23 | 0 | 3.5 | 1.5 | 0 | （1.52） | 18 | （16.52） | 5.0 | （5.02） | 2 | 25.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 57 | 23 | 0 | 3.5 | 1 | 0 | （1.52） | 18.5 | （172） | 5.0 | （5.02） | 2 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 66 | 23 | 0 | 6.5 | 1.5 | 0 | （1.52） | 15 | （13.52） | 6.0 | （6.02） | 2 | 25.0 + TT | 9.0 - TT | （7.5- TT2） |
| 67 | 23 | 0 | 6.5 | 1 | 0 | （1.52） | 15.5 | （142） | 6.0 | （6.02） | 2 | 25.0 + TT | 9.5 - TT | （8.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25, n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-10: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 6.0 | 2.5 | 25.0 + TT | 14.0-TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 6.0 | 2.5 | 25.0 + TT | 14.5-TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 5.0 | 2.5 | 25.0 + TT | 13.5-TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 5.0 | 2.5 | 25.0 + TT | 14.0-TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 5.0 | 2.5 | 25.0 + TT | 14.5-TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 6.0 | 2.5 | 25.0 + TT | 14.0-TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 5.0 | 2.5 | 25.0 + TT | 13.5-TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 5.0 | 2.5 | 25.0 + TT | 14.0-TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 5.0 | 2.5 | 25.0 + TT | 13.0-TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 5.0 | 2.5 | 25.0 + TT | 13.5-TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 6.0 | 2.5 | 25.0 + TT | 9.0-TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 6.0 | 2.5 | 25.0 + TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-11: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 5 | 2.5 | 25.0 + TT | 13.0-TT |
| 24 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 5 | 2.5 | 25.0 + TT | 13.5-TT |
| 43 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 5 | 2.5 | 25.0 + TT | 13.0-TT |
| 44 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 5 | 2.5 | 25.0 + TT | 13.5-TT |
| 53 | 23 | 0 | 4 | 1.5 | 0 | 17.5 | 5 | 2.5 | 25.0 + TT | 12.5-TT |
| 54 | 23 | 0 | 4 | 1 | 0 | 18 | 5 | 2.5 | 25.0 + TT | 13.0-TT |
| 63 | 23 | 0 | 7 | 1.5 | 0 | 14.5 | 6 | 2.5 | 25.0 + TT | 8.5-TT |
| 64 | 23 | 0 | 7 | 1 | 0 | 15 | 6 | 2.5 | 25.0 + TT | 9.0-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. NOTE 3: Applicable for CBW=30MHz when SCS=60kHz. NOTE 4: Applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz when SCS=30kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-12: UE Power Class test requirements (for Bands n24, n48, n77, n78, n79) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 23 | 0 | 1.5 | 1.5 | 0 | （1.52） | 20 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 15 | 23 | 0 | 1.5 | 1 | 0 | （1.52） | 20.5 | （192） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 26 | 23 | 0 | 3 | 1.5 | 0 | （1.52） | 18.5 | （172） | 5.0 | （5.02） | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 27 | 23 | 0 | 3 | 1 | 0 | （1.52） | 19 | （17.52） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （12.5 - TT2） |
| 34 | 23 | 0 | 2 | 1.5 | 0 | （1.52） | 19.5 | （182） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 35 | 23 | 0 | 2 | 1 | 0 | （1.52） | 20 | （18.52） | 6.0 | （6.02） | 3 | 25.0 + TT | 14.0 - TT | （12.5 - TT2） |
| 46 | 23 | 0 | 3 | 1.5 | 0 | （1.52） | 18.5 | （172） | 5.0 | （5.02） | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 47 | 23 | 0 | 3 | 1 | 0 | （1.52） | 19 | （17.52） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （12.5 - TT2） |
| 56 | 23 | 0 | 3.5 | 1.5 | 0 | （1.52） | 18 | （16.52） | 5.0 | （5.02） | 3 | 25.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 57 | 23 | 0 | 3.5 | 1 | 0 | （1.52） | 18.5 | （172） | 5.0 | （5.02） | 3 | 25.0 + TT | 13.5 - TT | （12.0 - TT2） |
| 66 | 23 | 0 | 6.5 | 1.5 | 0 | （1.52） | 15 | （13.52） | 6.0 | （6.02） | 3 | 25.0 + TT | 9.0 - TT | （7.5- TT2） |
| 67 | 23 | 0 | 6.5 | 1 | 0 | （1.52） | 15.5 | （142） | 6.0 | （6.02） | 3 | 25.0 + TT | 9.5 - TT | （8.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n24, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2G.3	UE additional maximum output power reduction for Tx Diversity

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_47 for Power Class 2 and Power Class 1.5 with contiguous allocation is complete.

- Tests for network signalling values NS_55 for Power Class 2 and, NS_55 for Power Class 1.5 are FFS.

- Tests for Power Class 3 are FFS.

6.2G.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2G.2. Outer and inner allocation notation used in clause 6.2G.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2G.3.2	Test applicability

The requirements of this test apply in test case 6.5G.2.2 Additional Spectrum Emission mask for Tx Diversity for network signalling value NS_04 to all types of NR Power Class 1.5, Power Class 2 reporting Tx diversity UE release 15 and forward.

The requirements of this test apply in test case 6.5G.3.3 Additional Spurious Emissions Tx Diversity for network signalling values NS_04, NS_47 and NS_50 to all types of NR Power Class 2 and Power Class 1.5 reporting Tx diversity UE release 15 and forward.

6.2G.3.3	Minimum conformance requirements

For UE supporting Tx diversity, the A-MPR values specified in clause 6.2.3 shall apply to the maximum output power specified in Table 6.2.1-1, and the maximum output power is defined as the sum of the maximum output power at each UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Table 6.2G.3.3-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2G.3.3-2.

Table 6.2G.3.3-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_04 | 6.5G.2.3.3.2, 6.5G.3.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_47 | 6.5G.3.3.3.15 | n41 (Note 3) | 30 | Table 6.2.3.3.18-1 | Table 6.2.3.3.18-2 |
| NS_50 | 6.5G.3.3.3.16 | n39 | 10, 15, 20, 25, 30, 35, 40 |  | Clause 6.2.3.3.19 |
| NS_55 | NOTE 4 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWchannel where the lower channel edge is ≥1930 MHz, 10 MHz BWchannel where the lower channel edge is ≥1950 MHz and 15 MHz BWchannel where the lower channel edge is ≥1955 MHz. NOTE 3: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 4: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. |  |  |  |  |  |

Table 6.2G.3.3-2: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n77 | NS_01 | NS_55 |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.1 and 6.2G.3.

6.2G.3.4	Test description

6.2G.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.3.4.1-2 for NS_04 and Table 6.2.3.4.1-18 for NS_47 PC2 and PC3, and in Table 6.2G.3.4.1-1 for NS_47 PC1.5, and in Table 6.2G.3.4.1-2 for NS_50 PC1.5. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.2G.3.4.1-1: Test Configuration table for NS_47 power class 1.5

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 30 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 30 | Default | N/A for A-MPR testing. |  | PI/2 BPSK | Edge_1RB_Left (A1) |  |  |  |
| 2 | Default | 30 | Default |  |  | PI/2 BPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 3 | Default | 30 | Default |  |  | PI/2 BPSK | Edge_1RB_Right (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | PI/2 BPSK | Outer_Full (A2) |  |  |  |
| 5 | Default | 30 | Default |  |  | PI/2 BPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 6 | Default | 30 | Default |  |  | PI/2 BPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 7 | Default | 30 | Default |  |  | PI/2 BPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 8 | Default | 30 | Default |  |  | PI/2 BPSK | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 9 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 10 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 11 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 12 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 13 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 14 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 15 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 16 | Default | 30 | Default |  |  | QPSK | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 17 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 18 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 19 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |  |
| 20 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 21 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 22 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 23 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 24 | Default | 30 | Default |  |  | 16 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 25 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |
| 26 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 27 | Default | 30 | Default |  |  | 64 QAM | Edge 1RB_Right (A3) |  |  |  |
| 28 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |  |
| 29 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 30 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 31 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 32 | Default | 30 | Default |  |  | 64 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 33 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |  |
| 34 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 35 | Default | 30 | Default |  |  | 256 QAM | Edge 1RB_Right (A3) |  |  |  |
| 36 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 37 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 38 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 39 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 40 | Default | 30 | Default |  |  | 256 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 41 | Default | 30 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 42 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 43 | Default | 30 | Deafult |  |  | QPSK | Edge 1RB_Right (A3) |  |  |  |
| 44 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 45 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 46 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 47 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 48 | Default | 30 | Default |  |  | QPSK | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 49 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 50 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 51 | Default | 30 | Default |  |  | 16 QAM | Edge 1RB_Right (A3) |  |  |  |
| 52 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 53 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 54 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 55 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 56 | Default | 30 | Default |  |  | 16 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 57 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |

| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
| 58 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 59 | Default | 30 | Default |  |  | 64 QAM | Edge 1RB_Right (A3) |  |  |
| 60 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |
| 61 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 62 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 63 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 64 | Default | 30 | Default |  |  | 64 QAM | 80@35 (A5) | 40@18 (A5) | 20@9 (A5) |
| 65 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |
| 66 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 67 | Default | 30 | Default |  |  | 256 QAM | Edge 1RB_Right (A3) |  |  |
| 68 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |
| 69 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 70 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 71 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 72 | Default | 30 | Default |  |  | 256 QAM | 80@35 (A5) | 40@18 (A5) | 20@9 (A5) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |

Table 6.2G.3.4.1-2: Test Configuration table for NS_50 power class 1.5

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-32a, 6.2.3.4.1-32b and 6.2.3.4.1-32c |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 10MHz, 15MHz, 20MHz, 25 MHz, 30MHz, 35MHz, 40MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |  |  |
| A-MPR test parameters for NS_50 (Power Class 1.5) |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |  |
| 1 | Default | 10 | Default | N/A | DFT-s-OFDM | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 2 | Default | 10 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |  |
| 3 | Default | 10 | Default |  |  | QPSK | 40@11 (A4) | 18@6 (A4) |  | / |  |
| 4 | Default | 15 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 5 | Default | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |  |
| 6 | Default | 15 | Default |  |  | QPSK | 60@19 (A4) | 27@10 (A4) |  | 15@5 (A4) |  |
| 7 | Default | 15 | Default |  |  | QPSK | 16@1 (A10) | 9@1 (A10) |  | 5@1 (A10) |  |
| 8 | Default | 20 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 9 | Default | 20 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |  |
| 10 | Default | 20 | Default |  |  | QPSK | 80@26 (A4) | 36@13 (A4) |  | 18@7 (A4) |  |
| 11 | Default | 20 | Default |  |  | QPSK | 21@1 (A10) | 10@1 (A10) |  | 6@1 (A10) |  |
| 12 | Default | 25 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |  |
| 13 | Default | 25 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 14 | Default | 25 | Default |  |  | QPSK | 75@48 (A4) | 36@25 (A4) |  | 18@13 (A4) |  |
| 15 | Default | 25 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |  |
| 16 | Default | 25 | Default |  |  | QPSK | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |  |
| 17 | Default | 25 | Default |  |  | QPSK | 38@11 (A10) | 33@6 (A10) |  | 30@3 (A10) |  |
| 18 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |  |
| 19 | Default | 30 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 20 | Default | 30 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |  |
| 21 | Default | 30 | Default |  |  | QPSK | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |  |
| 22 | Default | 30 | Default |  |  | QPSK | 90@63 (A4) | 45@32 (A4) |  | 24@18 (A4) |  |
| 23 | Default | 30 | Default |  |  | QPSK | 38@11 (A10) | 33@6 (A10) |  | 30@3 (A10) |  |
| 24 | Default | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 25 | Default | 35 | Default |  |  | QPSK | 50@10 (A3) | 30@5 (A3) |  | 12@3 (A3) |  |
| 26 | Default | 35 | Default |  |  | QPSK | 178@10 (A8) | 87@5 (A8) |  | 40@3 (A8) |  |
| 27 | Default | 35 | Default |  |  | QPSK | 111@77 (A4) | 52@39 (A4) |  | 24@20 (A4) |  |
| 28 | Default | 35 | Default |  |  | QPSK | 16@171 (A9) | 6@86 (A9) |  | 1@43 (A9) |  |
| 29 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 30 | Default | 40 | Default |  |  | QPSK | 180@25 (A8) | 90@13 (A8) |  | 40@7 (A8) |  |
| 31 | Default | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |  |
| 32 | Default | 40 | Default |  |  | QPSK | 90@25 (A7) | 45@13 (A7) |  | 20@7 (A7) |  |
| 33 | Default | 40 | Default |  |  | QPSK | 108@101 (A4) | 54@51 (A4) |  | 25@26 (A4) |  |
| 34 | Default | 40 | Default |  |  | QPSK | 36@177 (A9) | 16@89 (A9) |  | 6@45 (A9) |  |
| 35 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 36 | Default | 10 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |  |
| 37 | Default | 10 | Default |  |  | 256 QAM | 40@11 (A4) | 18@6 (A4) |  | / |  |
| 38 | Default | 15 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 39 | Default | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |  |
| 40 | Default | 15 | Default |  |  | 256 QAM | 60@19 (A4) | 27@10 (A4) |  | 15@5 (A4) |  |
| 41 | Default | 15 | Default |  |  | 256 QAM | 16@1 (A10) | 9@1 (A10 |  | 5@1 (A10) |  |
| 42 | Default | 20 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 43 | Default | 20 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |  |
| 44 | Default | 20 | Default |  |  | 256 QAM | 80@26 (A4) | 36@13 (A4) |  | 18@7 (A4) |  |
| 45 | Default | 20 | Default |  |  | 256 QAM | 21@1 (A10) | 10@1 (A10) |  | 6@1 (A10) |  |
| 46 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |  |
| 47 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 48 | Default | 25 | Default |  |  | 256 QAM | 75@48 (A4) | 36@25 (A4) |  | 18@13 (A4) |  |
| 49 | Default | 25 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |  |
| 50 | Default | 25 | Default |  |  | 256 QAM | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |  |
| 51 | Default | 25 | Default |  |  | 256 QAM | 38@11 (A10) | 33@6 (A10) |  | 30@3 (A10) |  |
| 52 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |  |
| 53 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 54 | Default | 30 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |  |
| 55 | Default | 30 | Default |  |  | 256 QAM | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |  |
| 56 | Default | 30 | Default |  |  | 256 QAM | 90@63 (A4) | 45@32 (A4) |  | 24@18 (A4) |  |
| 57 | Default | 30 | Default |  |  | 256 QAM | 38@11 (A10) | 33@6 (A10) |  | 30@3 (A10) |  |
| 58 | Default | 35 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 59 | Default | 35 | Default |  |  | 256 QAM | 50@10 (A3) | 30@5 (A3) |  | 12@3 (A3) |  |
| 60 | Default | 35 | Default |  |  | 256 QAM | 178@10 (A8) | 87@5 (A8) |  | 40@3 (A8) |  |
| 61 | Default | 35 | Default |  |  | 256 QAM | 111@77 (A4) | 52@39 (A4) |  | 24@20 (A4) |  |
| 62 | Default | 35 | Default |  |  | 256 QAM | 16@171 (A9) | 6@86 (A9) |  | 1@43 (A9) |  |
| 63 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 64 | Default | 40 | Default |  |  | 256 QAM | 180@25 (A8) | 90@13 (A8) |  | 40@7 (A8) |  |
| 65 | Default | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |  |
| 66 | Default | 40 | Default |  |  | 256 QAM | 90@25 (A7) | 45@13 (A7) |  | 20@7 (A7) |  |
| 67 | Default | 40 | Default |  |  | 256 QAM | 108@101 (A4) | 54@51 (A4) |  | 25@26 (A4) |  |
| 68 | Default | 40 | Default |  |  | 256 QAM | 36@177 (A9) | 16@89 (A9) |  | 6@45 (A9) |  |
| 69 | Default | 10 | Default | N/A | CP-OFDM | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 70 | Default | 10 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |  |
| 71 | Default | 10 | Default |  |  | QPSK | 41@11 (A4) |  | 18@6 (A4) |  | / |
| 72 | Default | 15 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 73 | Default | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |  |
| 74 | Default | 15 | Default |  |  | QPSK | 60@19 (A4) |  | 28@10 (A4) |  | 13@5 (A4) |
| 75 | Default | 15 | Default |  |  | QPSK | 16@1(A10) |  | 9@1(A10) |  | 5@1(A10) |
| 76 | Default | 20 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 77 | Default | 20 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |  |
| 78 | Default | 20 | Default |  |  | QPSK | 80@26 (A4) |  | 38@13 (A4) |  | 17@7 (A4) |
| 79 | Default | 20 | Default |  |  | QPSK | 21@1 (A10) |  | 10@1 (A10) |  | 6@1 (A10) |
| 80 | Default | 25 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |  |
| 81 | Default | 25 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 82 | Default | 25 | Default |  |  | QPSK | 74@47 (A4) |  | 37@24 (A4) |  | 19@13 (A4) |
| 83 | Default | 25 | Default |  |  | QPSK | 20@11 (A6) |  | 10@6 (A6) |  | 5@3 (A6) |
| 84 | Default | 25 | Default |  |  | QPSK | 27@0 (A4) |  | 13@0 (A4) |  | 6@0 (A4) |
| 85 | Default | 25 | Default |  |  | QPSK | 38@11(A10) |  | 33@6(A10) |  | 30@3(A10) |
| 86 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |  |
| 87 | Default | 30 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 88 | Default | 30 | Default |  |  | QPSK | 20@11 (A6) |  | 10@6 (A6) |  | 5@3 (A6) |
| 89 | Default | 30 | Default |  |  | QPSK | 27@0 (A4) |  | 13@0 (A4) |  | 6@0 (A4) |
| 90 | Default | 30 | Default |  |  | QPSK | 88@61 (A4) |  | 44@31 (A4) |  | 22@16 (A4) |
| 91 | Default | 30 | Default |  |  | QPSK | 38@11(A10) |  | 33@6(A10) |  | 30@3(A10) |
| 92 | Default | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 93 | Default | 35 | Default |  |  | QPSK | 50@10 (A3) |  | 30@5 (A3) |  | 12@3 (A3) |
| 94 | Default | 35 | Default |  |  | QPSK | 178@10 (A8) |  | 87@5 (A8) |  | 41@3 (A8) |
| 95 | Default | 35 | Default |  |  | QPSK | 111@77(A4) |  | 53@39(A4) |  | 24@20(A4) |
| 96 | Default | 35 | Default |  |  | QPSK | 17@171(A9) |  | 6@86(A9) |  | 1@43(A9) |
| 97 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 98 | Default | 40 | Default |  |  | QPSK | 191@25 (A8) |  | 93@13 (A8) |  | 44@7 (A8) |
| 99 | Default | 40 | Default |  |  | QPSK | 60@25 (A3) |  | 30@13 (A3) |  | 15@7 (A3) |
| 100 | Default | 40 | Default |  |  | QPSK | 91@25 (A7) |  | 46@13 (A7) |  | 23@7 (A7) |
| 101 | Default | 40 | Default |  |  | QPSK | 115@101 (A4) |  | 55@51 (A4) |  | 25@26 (A4) |
| 102 | Default | 40 | Default |  |  | QPSK | 39@177 (A9) |  | 17@89 (A9) |  | 6@45 (A9) |
| 103 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 104 | Default | 10 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |  |
| 105 | Default | 10 | Default |  |  | 256 QAM | 41@11 (A4) |  | 18@6 (A4) |  | / |
| 106 | Default | 15 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 107 | Default | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |  |
| 108 | Default | 15 | Default |  |  | 256 QAM | 60@19 (A4) |  | 28@10 (A4) |  | 13@5 (A4) |
| 109 | Default | 15 | Default |  |  | 256 QAM | 16@1(A10) |  | 9@1(A10) |  | 5@1(A10) |
| 110 | Default | 20 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 111 | Default | 20 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |  |
| 112 | Default | 20 | Default |  |  | 256 QAM | 80@26 (A4) |  | 38@13 (A4) |  | 17@7 (A4) |
| 113 | Default | 20 | Default |  |  | 256 QAM | 21@1 (A10) |  | 10@1 (A10) |  | 6@1 (A10) |
| 114 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |  |
| 115 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 116 | Default | 25 | Default |  |  | 256 QAM | 74@47 (A4) |  | 37@24 (A4) |  | 19@13 (A4) |
| 117 | Default | 25 | Default |  |  | 256 QAM | 20@11 (A6) |  | 10@6 (A6) |  | 5@3 (A6) |
| 118 | Default | 25 | Default |  |  | 256 QAM | 27@0 (A4) |  | 13@0 (A4) |  | 6@0 (A4) |
| 119 | Default | 25 | Default |  |  | 256 QAM | 38@11(A10) |  | 33@6(A10) |  | 30@3(A10) |
| 120 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |  |
| 121 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 123 | Default | 30 | Default |  |  | 256 QAM | 20@11 (A6) |  | 10@6 (A6) |  | 5@3 (A6) |
| 124 | Default | 30 | Default |  |  | 256 QAM | 27@0 (A4) |  | 13@0 (A4) |  | 6@0 (A4) |
| 125 | Default | 30 | Default |  |  | 256 QAM | 88@61 (A4) |  | 44@31 (A4) |  | 22@16 (A4) |
| 126 | Default | 30 | Default |  |  | 256 QAM | 38@11(A10) |  | 33@6(A10) |  | 30@3(A10) |
| 127 | Default | 35 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 128 | Default | 35 | Default |  |  | 256 QAM | 50@10 (A3) |  | 30@5 (A3) |  | 12@3 (A3) |
| 129 | Default | 35 | Default |  |  | 256 QAM | 178@10 (A8) |  | 87@5 (A8) |  | 41@3 (A8) |
| 130 | Default | 35 | Default |  |  | 256 QAM | 111@77(A4) |  | 53@39(A4) |  | 24@20(A4) |
| 131 | Default | 35 | Default |  |  | 256 QAM | 17@171(A9) |  | 6@86(A9) |  | 1@43(A9) |
| 132 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 133 | Default | 40 | Default |  |  | 256 QAM | 191@25 (A8) |  | 93@13 (A8) |  | 44@7 (A8) |
| 134 | Default | 40 | Default |  |  | 256 QAM | 60@25 (A3) |  | 30@13 (A3) |  | 15@7 (A3) |
| 135 | Default | 40 | Default |  |  | 256 QAM | 91@25 (A7) |  | 46@13 (A7) |  | 23@7 (A7) |
| 136 | Default | 40 | Default |  |  | 256 QAM | 115@101 (A4) |  | 55@51 (A4) |  | 25@26 (A4) |
| 137 | Default | 40 | Default |  |  | 256 QAM | 39@177 (A9) |  | 17@89 (A9) |  | 6@45 (A9) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2G.3.4.1-1 to Table 6.2G.3.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2G.3.4.3.

6.2G.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2G.3.4.1-1 to Table 6.2G.3.4.1-2. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD, only slots consisting of only UL symbols are under test.

4.	For network signalling value “NS_04” and UEs supporting Power Class 2 and Power Class 1.5, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2G.3.4.3.1-2.

5.	For network signalling value “NS_04” and UEs supporting Power Class 1.5, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2G.3.4.3.1-3.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2G.3.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2G.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.1, with the following exceptions for each network signalling value.

6.2G.3.4.3.1	Message contents exceptions for network signalling value "NS_04"

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2G.3.4.3.1-1: AdditionalSpectrumEmission Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

Table 6.2G.3.4.3.1-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE or PC1.5 UE |

Table 6.2G.3.4.3.1-3: P-Max (Step 5)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 26 |  | PC1.5 UE |

6.2G.3.4.3.2	Message contents exceptions for network signalling value "NS_47"

1.	Information element additionalSpectrumEmission is set to NS_47. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2G.3.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_47"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_47) |  |  |

6.2G.3.4.3.3	Message contents exceptions for network signalled value "NS_50"

1.	Information element additionalSpectrumEmission is set to NS_50. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2G.3.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_50"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_50) |  |  |

6.2G.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2G.3.5-1. The allowed A-MPR values specified in table 6.2G.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2G.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2G.1.3-1 apply.

Table 6.2G.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2G.3.5-1: UE Power Class 2 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 26 | 3.5 | 5.5 | 1.5 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 8 | 26 | 3.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 9 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 10 | 26 | 0.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 11 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 12 | 26 | 0.5 | 0 | 0 | 25.5 | 2.0 | 3 | 28+TT | 22.5-TT |
| 13 | 26 | 3.5 | 6 | 1.5 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 14 | 26 | 3.5 | 4.5 | 0 | 21 | 2.0 | 3 | 28+TT | 18.5-TT |
| 15 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 16 | 26 | 1 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 17 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 18 | 26 | 1 | 0 | 0 | 25 | 2.0 | 3 | 28+TT | 22-TT |
| 19 | 26 | 3.5 | 6 | 1.5 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 20 | 26 | 3.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 21 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 22 | 26 | 2 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 23 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 24 | 26 | 2 | 0 | 0 | 24 | 2.0 | 3 | 28+TT | 21-TT |
| 25 | 26 | 3.5 | 6.5 | 1.5 | 18 | 4.0 | 3 | 28+TT | 14-TT |
| 26 | 26 | 3.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 27 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 28 | 26 | 2.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 29 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 30 | 26 | 2.5 | 0 | 0 | 23.5 | 2.0 | 3 | 28+TT | 20.5-TT |
| 31 | 26 | 4.5 | 8 | 1.5 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 32 | 26 | 4.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 33 | 26 | 4.5 | 0 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 34 | 26 | 4.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 35 | 26 | 4.5 | 0 | 1.5 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 36 | 26 | 4.5 | 0 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 37 | 26 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 38 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 39 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 40 | 26 | 3 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 41 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 42 | 26 | 3 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 43 | 26 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 44 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 45 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 46 | 26 | 3 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 47 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 48 | 26 | 3 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 49 | 26 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 50 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 51 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 52 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 53 | 26 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 54 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 55 | 26 | 8 | 10 | 1.5 | 14.5 | 6.0 | 3 | 28+TT | 8.5-TT |
| 56 | 26 | 8 | 7.5 | 0 | 18 | 5.0 | 3 | 28+TT | 13-TT |
| 57 | 26 | 8 | 0 | 0 | 18 | 5.0 | 3 | 28+TT | 13-TT |
| 58 | 26 | 8 | 7.5 | 0 | 18 | 5.0 | 3 | 28+TT | 13-TT |
| 59 | 26 | 8 | 0 | 1.5 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 60 | 26 | 8 | 0 | 0 | 18 | 5.0 | 3 | 28+TT | 13-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2G.3.5-2: UE Power Class 1.5 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 29 | 6 | 7 | 1.5 | 20.5 | 2.5 | 3 | 31+TT | 17.5-TT |
| 8 | 29 | 6 | 5 | 0 | 23 | 2.0 | 3 | 31+TT | 20-TT |
| 9 | 29 | 6 | 0 | 0 | 23 | 2.0 | 3 | 31+TT | 20-TT |
| 10 | 29 | 2 | 5 | 0 | 24 | 2.0 | 3 | 31+TT | 21-TT |
| 11 | 29 | 6 | 0 | 1.5 | 21.5 | 2.0 | 3 | 31+TT | 18.5-TT |
| 12 | 29 | 2 | 0 | 0 | 27 | 2.0 | 3 | 31+TT | 24-TT |
| 13 | 29 | 6.5 | 7.5 | 1.5 | 20 | 2.5 | 3 | 31+TT | 17-TT |
| 14 | 29 | 6.5 | 6 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 15 | 29 | 6.5 | 0 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 16 | 29 | 2.5 | 6 | 0 | 23 | 2.0 | 3 | 31+TT | 20-TT |
| 17 | 29 | 6.5 | 0 | 1.5 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 18 | 29 | 2.5 | 0 | 0 | 26.5 | 2.0 | 3 | 31+TT | 23.5-TT |
| 19 | 29 | 6.5 | 7.5 | 1.5 | 20 | 2.0 | 3 | 31+TT | 17-TT |
| 20 | 29 | 6.5 | 6.5 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 21 | 29 | 6.5 | 0 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 22 | 29 | 3.5 | 6.5 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 23 | 29 | 6.5 | 0 | 1.5 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 24 | 29 | 3.5 | 0 | 0 | 25.5 | 2.0 | 3 | 31+TT | 22.5-TT |
| 25 | 29 | 6.5 | 8 | 1.5 | 19.5 | 3.5 | 3 | 31+TT | 16-TT |
| 26 | 29 | 6.5 | 6.5 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 27 | 29 | 6.5 | 0 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 28 | 29 | 4 | 6.5 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 29 | 29 | 6.5 | 0 | 1.5 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 30 | 29 | 4 | 0 | 0 | 25 | 2.0 | 3 | 31+TT | 22-TT |
| 31 | 29 | 6.5 | 9.5 | 1.5 | 18 | 4.0 | 3 | 31+TT | 14-TT |
| 32 | 29 | 6.5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 33 | 29 | 6.5 | 0 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 34 | 29 | 6.5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 35 | 29 | 6.5 | 0 | 1.5 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 36 | 29 | 6.5 | 0 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 37 | 29 | 6.5 | 9 | 1.5 | 18.5 | 4.0 | 3 | 31+TT | 14.5-TT |
| 38 | 29 | 6.5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 39 | 29 | 6.5 | 0 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 40 | 29 | 4.5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 41 | 29 | 6.5 | 0 | 1.5 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 42 | 29 | 4.5 | 0 | 0 | 24.5 | 2.0 | 3 | 31+TT | 21.5-TT |
| 43 | 29 | 6.5 | 9 | 1.5 | 18.5 | 4.0 | 3 | 31+TT | 14.5-TT |
| 44 | 29 | 6.5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 45 | 29 | 6.5 | 0 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 46 | 29 | 4.5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 47 | 29 | 6.5 | 0 | 1.5 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 48 | 29 | 4.5 | 0 | 0 | 24.5 | 2.0 | 3 | 31+TT | 21.5-TT |
| 49 | 29 | 6.5 | 9 | 1.5 | 18.5 | 4.0 | 3 | 31+TT | 14.5-TT |
| 50 | 29 | 6.5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 51 | 29 | 6.5 | 0 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 52 | 29 | 5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 53 | 29 | 6.5 | 0 | 1.5 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 54 | 29 | 5 | 0 | 0 | 24 | 2.0 | 3 | 31+TT | 21-TT |
| 55 | 29 | 8.5 | 11.5 | 1.5 | 16 | 5.0 | 3 | 31+TT | 11-TT |
| 56 | 29 | 8.5 | 9 | 0 | 20 | 2.5 | 3 | 31+TT | 17-TT |
| 57 | 29 | 8.5 | 0 | 0 | 20.5 | 2.5 | 3 | 31+TT | 17.5-TT |
| 58 | 29 | 8.5 | 9 | 0 | 20 | 2.5 | 3 | 31+TT | 17-TT |
| 59 | 29 | 8.5 | 0 | 1.5 | 19 | 3.5 | 3 | 31+TT | 15.5-TT |
| 60 | 29 | 8.5 | 0 | 0 | 20.5 | 2.5 | 3 | 31+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2G.3.5-3: UE Power Class 1.5 and Power Class 2 test requirements (NS_04) for band n41 (Step 4)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 293 (264) | 63 (34) | 0.5 | 3.5 | 1.5 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 8 | 293 (264) | 63 (34) | 0.5 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 9 | 293 (264) | 63 (34) | 0.5 | 0 | 0 | 22.5 | 2.0 | 3 | 25+TT | 19.5-TT |
| 10 | 293 (264) | 63 (34) | 0.5 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 11 | 293 (264) | 63 (34) | 0.5 | 0 | 1.5 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 12 | 293 (264) | 63 (34) | 0.5 | 0 | 0 | 22.5 | 2.0 | 3 | 25+TT | 19.5-TT |
| 13 | 293 (264) | 63 (34) | 1 | 4 | 1.5 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 14 | 293 (264) | 63 (34) | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 15 | 293 (264) | 63 (34) | 1 | 0 | 0 | 22 | 2.0 | 3 | 25+TT | 19-TT |
| 16 | 293 (264) | 63 (34) | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 17 | 293 (264) | 63 (34) | 1 | 0 | 1.5 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 18 | 293 (264) | 63 (34) | 1 | 0 | 0 | 22 | 2.0 | 3 | 25+TT | 19-TT |
| 19 | 293 (264) | 63 (34) | 2 | 4 | 1.5 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 20 | 293 (264) | 63 (34) | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 21 | 293 (264) | 63 (34) | 2 | 0 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 22 | 293 (264) | 63 (34) | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 23 | 293 (264) | 63 (34) | 2 | 0 | 1.5 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 24 | 293 (264) | 63 (34) | 2 | 0 | 0 | 21 | 2.0 | 3 | 25+TT | 18-TT |
| 25 | 293 (264) | 63 (34) | 2.5 | 4.5 | 1.5 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 26 | 293 (264) | 63 (34) | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 27 | 293 (264) | 63 (34) | 2.5 | 0 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 28 | 293 (264) | 63 (34) | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 29 | 293 (264) | 63 (34) | 2.5 | 0 | 1.5 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 30 | 293 (264) | 63 (34) | 2.5 | 0 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 31 | 293 (264) | 63 (34) | 4.5 | 6 | 1.5 | 15.5 | 5.0 | 3 | 25+TT | 10.5-TT |
| 32 | 293 (264) | 63 (34) | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 33 | 293 (264) | 63 (34) | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 34 | 293 (264) | 63 (34) | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 35 | 293 (264) | 63 (34) | 4.5 | 0 | 1.5 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 36 | 293 (264) | 63 (34) | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 37 | 293 (264) | 63 (34) | 3 | 5.5 | 1.5 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 38 | 293 (264) | 63 (34) | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 39 | 293 (264) | 63 (34) | 3 | 0 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 40 | 293 (264) | 63 (34) | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 41 | 293 (264) | 63 (34) | 3 | 0 | 1.5 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 42 | 293 (264) | 63 (34) | 3 | 0 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 43 | 293 (264) | 63 (34) | 3 | 5.5 | 1.5 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 44 | 293 (264) | 63 (34) | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 45 | 293 (264) | 63 (34) | 3 | 0 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 46 | 293 (264) | 63 (34) | 3 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 47 | 293 (264) | 63 (34) | 3 | 0 | 1.5 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 48 | 293 (264) | 63 (34) | 3 | 0 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 49 | 293 (264) | 63 (34) | 3.5 | 5.5 | 1.5 | 16 | 5.0 | 3 | 25+TT | 11-TT |
| 50 | 293 (264) | 63 (34) | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 51 | 293 (264) | 63 (34) | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 52 | 293 (264) | 63 (34) | 3.5 | 5.5 | 0 | 17.5 | 5.0 | 3 | 25+TT | 12.5-TT |
| 53 | 293 (264) | 63 (34) | 3.5 | 0 | 1.5 | 18 | 4.0 | 3 | 25+TT | 14-TT |
| 54 | 293 (264) | 63 (34) | 3.5 | 0 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 55 | 293 (264) | 63 (34) | 6.5 | 8 | 1.5 | 13.5 | 5.0 | 3 | 25+TT | 8.5-TT |
| 56 | 293 (264) | 63 (34) | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 57 | 293 (264) | 63 (34) | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 58 | 293 (264) | 63 (34) | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| 59 | 293 (264) | 63 (34) | 6.5 | 0 | 1.5 | 15 | 5.0 | 3 | 25+TT | 10-TT |
| 60 | 293 (264) | 63 (34) | 6.5 | 0 | 0 | 16.5 | 5.0 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. NOTE 3: Power Class 1.5 UE NOTE 4: Power Class 2 UE |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.3.5-4: UE Power Class 1.5 test requirements (NS_04) for band n41 (Step 5)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 29 | 3 | 3.5 | 5.5 | 1.5 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 8 | 29 | 3 | 3.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 9 | 29 | 3 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 10 | 29 | 3 | 0.5 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 11 | 29 | 3 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 12 | 29 | 3 | 0.5 | 0 | 0 | 25.5 | 2.0 | 3 | 28+TT | 22.5-TT |
| 13 | 29 | 3 | 3.5 | 6 | 1.5 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 14 | 29 | 3 | 3.5 | 4.5 | 0 | 21 | 2.0 | 3 | 28+TT | 18.5-TT |
| 15 | 29 | 3 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 16 | 29 | 3 | 1 | 4.5 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 17 | 29 | 3 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 18 | 29 | 3 | 1 | 0 | 0 | 25 | 2.0 | 3 | 28+TT | 22-TT |
| 19 | 29 | 3 | 3.5 | 6 | 1.5 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 20 | 29 | 3 | 3.5 | 5 | 0 | 19.5 | 2.0 | 3 | 28+TT | 18-TT |
| 21 | 29 | 3 | 3.5 | 0 | 0 | 21 | 2.0 | 3 | 28+TT | 19.5-TT |
| 22 | 29 | 3 | 2 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 23 | 29 | 3 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 24 | 29 | 3 | 2 | 0 | 0 | 24 | 2.0 | 3 | 28+TT | 21-TT |
| 25 | 29 | 3 | 3.5 | 6.5 | 1.5 | 18 | 4.0 | 3 | 28+TT | 14-TT |
| 26 | 29 | 3 | 3.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 27 | 29 | 3 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 28 | 29 | 3 | 2.5 | 5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 29 | 29 | 3 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 30 | 29 | 3 | 2.5 | 0 | 0 | 23.5 | 2.0 | 3 | 28+TT | 20.5-TT |
| 31 | 29 | 3 | 4.5 | 8 | 1.5 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 32 | 29 | 3 | 4.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 33 | 29 | 3 | 4.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 34 | 29 | 3 | 4.5 | 0 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 35 | 29 | 3 | 4.5 | 0 | 1.5 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 36 | 29 | 3 | 4.5 | 0 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 37 | 29 | 3 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 38 | 29 | 3 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 39 | 29 | 3 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 40 | 29 | 3 | 3 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 41 | 29 | 3 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 42 | 29 | 3 | 3 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 43 | 29 | 3 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 44 | 29 | 3 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 45 | 29 | 3 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 46 | 29 | 3 | 3 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 47 | 29 | 3 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 48 | 29 | 3 | 3 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 49 | 29 | 3 | 3.5 | 7.5 | 1.5 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 50 | 29 | 3 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 51 | 29 | 3 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 52 | 29 | 3 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 53 | 29 | 3 | 3.5 | 0 | 1.5 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 54 | 29 | 3 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 55 | 29 | 3 | 6.5 | 10 | 1.5 | 14.5 | 5.0 | 3 | 28+TT | 9.5-TT |
| 56 | 29 | 3 | 6.5 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 57 | 29 | 3 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 58 | 29 | 3 | 6.5 | 7.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 59 | 29 | 3 | 6.5 | 0 | 1.5 | 18 | 4.0 | 3 | 28+TT | 14-TT |
| 60 | 29 | 3 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.3.5-5: UE Power Class 2 test requirements for NS_47 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 2 | 26 | 0 | 0.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 3 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 4 | 26 | 0 | 0.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 5 | 26 | 0 | 0.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 6 | 26 | 0 | 0.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 7 | 26 | 0 | 0.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 8 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 9 | 26 | 0 | 0.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 10 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 11 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 12 | 26 | 0 | 1.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 13 | 26 | 0 | 1.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 14 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 15 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 16 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 17 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 18 | 26 | 0 | 2.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 19 | 26 | 0 | 2.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 20 | 26 | 0 | 2.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 21 | 26 | 0 | 2.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 22 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 23 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 24 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 25 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 26 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 27 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 28 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 29 | 26 | 0 | 4.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 30 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 31 | 26 | 0 | 4.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 32 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 33 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 34 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 35 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 36 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 37 | 26 | 0 | 1.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 38 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 39 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 40 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 41 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 42 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 43 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 44 | 26 | 0 | 2.0 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 45 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 46 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 47 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 48 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 49 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 50 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 51 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 52 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 53 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 54 | 26 | 0 | 3.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 55 | 26 | 0 | 3.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 56 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 57 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 58 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 59 | 26 | 0 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 61 | 26 | 0 | 6.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 62 | 26 | 0 | 6.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 63 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.3.5-6: UE Power Class 1.5 test requirements for NS_47 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 6 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 2 | 29 | 0 | 0.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 3 | 29 | 0 | 6 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 4 | 29 | 0 | 2 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 5 | 29 | 0 | 2 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 6 | 29 | 0 | 2 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 7 | 29 | 0 | 2 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 8 | 29 | 0 | 2 | 3 | 0 | 26 | 3 | 3 | 31+TT | 23.0-TT |
| 9 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 10 | 29 | 0 | 0.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 11 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 12 | 29 | 0 | 2.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 13 | 29 | 0 | 2.5 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 14 | 29 | 0 | 2.5 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 15 | 29 | 0 | 2.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 16 | 29 | 0 | 2.5 | 3 | 0 | 26 | 3 | 3 | 31+TT | 23.0-TT |
| 17 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 18 | 29 | 0 | 1.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 19 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 20 | 29 | 0 | 3.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 21 | 29 | 0 | 3.5 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 22 | 29 | 0 | 3.5 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 23 | 29 | 0 | 3.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 24 | 29 | 0 | 3.5 | 3 | 0 | 25.5 | 3 | 3 | 31+TT | 22.5-TT |
| 25 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 26 | 29 | 0 | 3.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 27 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 28 | 29 | 0 | 4 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 29 | 29 | 0 | 4 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 30 | 29 | 0 | 4 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 31 | 29 | 0 | 4 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 32 | 29 | 0 | 4 | 0 | 0 | 25 | 3 | 3 | 31+TT | 22.0-TT |
| 33 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 34 | 29 | 0 | 6.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 35 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 36 | 29 | 0 | 6.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 37 | 29 | 0 | 6.5 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 38 | 29 | 0 | 6.5 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 39 | 29 | 0 | 6.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 40 | 29 | 0 | 6.5 | 0 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 41 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 42 | 29 | 0 | 2 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 43 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 44 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 45 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 46 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 47 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 48 | 29 | 0 | 4.5 | 4 | 0 | 24.5 | 3 | 3 | 31+TT | 21.5-TT |
| 49 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 50 | 29 | 0 | 2.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 51 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 52 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 53 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 54 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 55 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 56 | 29 | 0 | 4.5 | 4 | 0 | 24.5 | 3 | 3 | 31+TT | 21.5-TT |
| 57 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 58 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 59 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 29 | 0 | 5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 61 | 29 | 0 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 62 | 29 | 0 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 63 | 29 | 0 | 5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 64 | 29 | 0 | 5 | 0 | 0 | 24 | 3 | 3 | 31+TT | 21.0-TT |
| 65 | 29 | 0 | 8.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 66 | 29 | 0 | 8.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 67 | 29 | 0 | 8.5 | 8 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 68 | 29 | 0 | 8.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 69 | 29 | 0 | 8.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 70 | 29 | 0 | 8.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 71 | 29 | 0 | 8.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 72 | 29 | 0 | 8.5 | 0 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.3.5-7: UE Power Class 1.5 test requirements for NS_47 (contiguous allocation, large FWA form factor with 20 dB or above antenna isolation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 6 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 2 | 29 | 0 | 0 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 3 | 29 | 0 | 6 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 4 | 29 | 0 | 1.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 5 | 29 | 0 | 1.5 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 6 | 29 | 0 | 1.5 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 7 | 29 | 0 | 1.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 8 | 29 | 0 | 1.5 | 3 | 0 | 26 | 3 | 3 | 31+TT | 23.0-TT |
| 9 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 10 | 29 | 0 | 0 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 11 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 12 | 29 | 0 | 2 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 13 | 29 | 0 | 2 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 14 | 29 | 0 | 2 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 15 | 29 | 0 | 2 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 16 | 29 | 0 | 2 | 3 | 0 | 26 | 3 | 3 | 31+TT | 23.0-TT |
| 17 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 18 | 29 | 0 | 1 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 19 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 20 | 29 | 0 | 3 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 21 | 29 | 0 | 3 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 22 | 29 | 0 | 3 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 23 | 29 | 0 | 3 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 24 | 29 | 0 | 3 | 3 | 0 | 26 | 3 | 3 | 31+TT | 23.0-TT |
| 25 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 26 | 29 | 0 | 3 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 27 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 28 | 29 | 0 | 3.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 29 | 29 | 0 | 3.5 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 30 | 29 | 0 | 3.5 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 31 | 29 | 0 | 3.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 32 | 29 | 0 | 3.5 | 0 | 0 | 25.5 | 3 | 3 | 31+TT | 22.5-TT |
| 33 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 34 | 29 | 0 | 5.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 35 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 36 | 29 | 0 | 5.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 37 | 29 | 0 | 5.5 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 38 | 29 | 0 | 5.5 | 8.5 | 0 | 20.5 | 6 | 3 | 31+TT | 14.5-TT |
| 39 | 29 | 0 | 5.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13.0-TT |
| 40 | 29 | 0 | 5.5 | 0 | 0 | 23.5 | 3 | 3 | 31+TT | 20.5-TT |
| 41 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 42 | 29 | 0 | 1.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 43 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 44 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 45 | 29 | 0 | 4 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 46 | 29 | 0 | 4 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 47 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 48 | 29 | 0 | 4 | 4 | 0 | 25 | 3 | 3 | 31+TT | 22.0-TT |
| 49 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 50 | 29 | 0 | 2 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 51 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 52 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 53 | 29 | 0 | 4 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 54 | 29 | 0 | 4 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 55 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 56 | 29 | 0 | 4 | 4 | 0 | 25 | 3 | 3 | 31+TT | 22.0-TT |
| 57 | 29 | 0 | 6.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 58 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 59 | 29 | 0 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 61 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 62 | 29 | 0 | 4.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 63 | 29 | 0 | 4.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 64 | 29 | 0 | 4.5 | 0 | 0 | 24.5 | 3 | 3 | 31+TT | 21.5-TT |
| 65 | 29 | 0 | 7.5 | 13 | 0 | 16 | 5 | 3 | 31+TT | 11.0-TT |
| 66 | 29 | 0 | 7.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 67 | 29 | 0 | 7.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16.0-TT |
| 68 | 29 | 0 | 7.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 69 | 29 | 0 | 7.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 70 | 29 | 0 | 7.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 71 | 29 | 0 | 7.5 | 12.5 | 0 | 16.5 | 5 | 3 | 31+TT | 11.5-TT |
| 72 | 29 | 0 | 7.5 | 0 | 0 | 21.5 | 5 | 3 | 31+TT | 16.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.3.5-8: UE Power Class 1.5 test requirements (NS_50) for band n39 (Step 3)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 6.5 | 6.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 2 | 29 | 0 | [2.5] | 5.0 | 0 | 24 |  | 3.0 |  | 3 | 31.0 + TT | 21 |  |
| 3 | 29 | 0 | 0.5 | 5.0 | 0 | 24 |  | 3.0 |  | 3 | 31.0 + TT | 21 |  |
| 4 | 29 | 0 | 6.5 | 6.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 5 | 29 | 0 | [2.5] | 6.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 6 | 29 | 0 | 0.5 | 5.0 | 0 | 24 |  | 3.0 |  | 3 | 31.0 + TT | 21 |  |
| 7 | 29 | 0 | 0.5 | 3.0 | 0 | 26 |  | 3.0 |  | 3 | 31.0 + TT | 23 |  |
| 8 | 29 | 0 | 6.5 | 6.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 9 | 29 | 0 | [2.5] | 6.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 10 | 29 | 0 | 0.5 | 5.0 | 0 | 24 |  | 3.0 |  | 3 | 31.0 + TT | 21 |  |
| 11 | 29 | 0 | 0.5 | 3.0 | 0 | 26 |  | 3.0 |  | 3 | 31.0 + TT | 23 |  |
| 12 | 29 | 0 | [2.5] | 8.5 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 13 | 29 | 0 | 6.5 | 6.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 14 | 29 | 0 | 0.5 | 5.0 | 0 | 24 |  | 3.0 |  | 3 | 31.0 + TT | 21 |  |
| 15 | 29 | 0 | 0.5 | 3.5 | 0 | 25.5 |  | 3.0 |  | 3 | 31.0 + TT | 22.5 |  |
| 16 | 29 | 0 | 0.5 | 5.0 | 0 | 24 |  | 3.0 |  | 3 | 31.0 + TT | 21 |  |
| 17 | 29 | 0 | 0.5 | 3.0 | 0 | 26 |  | 3.0 |  | 3 | 31.0 + TT | 23 |  |
| 18 | 29 | 0 | [2.5] | 8.5 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 19 | 29 | 0 | 6.5 | 6.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 20 | 29 | 0 | 0.5 | 3.5 | 0 | 25.5 |  | 3.0 |  | 3 | 31.0 + TT | 22.5 |  |
| 21 | 29 | 0 | 0.5 | 5.0 | 0 | 24 |  | 3.0 |  | 3 | 31.0 + TT | 21 |  |
| 22 | 29 | 0 | 0.5 | 5.0 | 0 | 24 |  | 3.0 |  | 3 | 31.0 + TT | 21 |  |
| 23 | 29 | 0 | 0.5 | 3.0 | 0 | 26 |  | 3.0 |  | 3 | 31.0 + TT | 23 |  |
| 24 | 29 | 0 | [2.5] | 14.5 | 0 | 14.5 |  | 6.0 |  | 3 | 31.0 + TT | 8.5 |  |
| 25 | 29 | 0 | 0.5 | 6.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 26 | 29 | 0 | 0.5 | 12.5 | 0 | 16.5 |  | 5.0 |  | 3 | 31.0 + TT | 11.5 |  |
| 27 | 29 | 0 | 0.5 | 5.0 | 0 | 24 |  | 3.0 |  | 3 | 31.0 + TT | 21 |  |
| 28 | 29 | 0 | 0.5 | 10.5 | 0 | 18.5 |  | 5.0 |  | 3 | 31.0 + TT | 13.5 |  |
| 29 | 29 | 0 | [2.5] | 14.5 | 0 | 14.5 |  | 6.0 |  | 3 | 31.0 + TT | 8.5 |  |
| 30 | 29 | 0 | 0.5 | 12.5 | 0 | 16.5 |  | 5.0 |  | 3 | 31.0 + TT | 11.5 |  |
| 31 | 29 | 0 | 0.5 | 6.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 32 | 29 | 0 | 0.5 | 5.5 | 0 | 23.5 |  | 5.0 |  | 3 | 31.0 + TT | 18.5 |  |
| 33 | 29 | 0 | 0.5 | 5.0 | 0 | 24 |  | 3.0 |  | 3 | 31.0 + TT | 21 |  |
| 34 | 29 | 0 | 0.5 | 10.5 | 0 | 18.5 |  | 5.0 |  | 3 | 31.0 + TT | 13.5 |  |
| 35 | 29 | 0 | 6.5 | 6.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 36 | 29 | 0 | 6.5 | 5.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 37 | 29 | 0 | [6.5] | 5.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 38 | 29 | 0 | 6.5 | 6.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 39 | 29 | 0 | 6.5 | 6.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 40 | 29 | 0 | [6.5] | 5.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 41 | 29 | 0 | [6.5] | 0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 42 | 29 | 0 | 6.5 | 6.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 43 | 29 | 0 | 6.5 | 6.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 44 | 29 | 0 | [6.5] | 5.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 45 | 29 | 0 | [6.5] | 0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 46 | 29 | 0 | 6.5 | 8.5 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 47 | 29 | 0 | 6.5 | 6.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 48 | 29 | 0 | [6.5] | 5.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 49 | 29 | 0 | [6.5] | 0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 50 | 29 | 0 | [6.5] | 5.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 51 | 29 | 0 | [6.5] | 0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 52 | 29 | 0 | 6.5 | 8.5 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 53 | 29 | 0 | 6.5 | 6.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 54 | 29 | 0 | [6.5] | 0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 55 | 29 | 0 | [6.5] | 5.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 56 | 29 | 0 | [6.5] | 5.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 57 | 29 | 0 | [6.5] | 0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 58 | 29 | 0 | 6.5 | 14.5 | 0 | 14.5 |  | 6.0 |  | 3 | 31.0 + TT | 8.5 |  |
| 59 | 29 | 0 | [6.5] | 6.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 60 | 29 | 0 | [6.5] | 12.5 | 0 | 16.5 |  | 5.0 |  | 3 | 31.0 + TT | 11.5 |  |
| 61 | 29 | 0 | [6.5] | 5.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 62 | 29 | 0 | [6.5] | 10.5 | 0 | 18.5 |  | 5.0 |  | 3 | 31.0 + TT | 13.5 |  |
| 63 | 29 | 0 | 6.5 | 14.5 | 0 | 14.5 |  | 6.0 |  | 3 | 31.0 + TT | 8.5 |  |
| 64 | 29 | 0 | [6.5] | 12.5 | 0 | 16.5 |  | 5.0 |  | 3 | 31.0 + TT | 11.5 |  |
| 65 | 29 | 0 | [6.5] | 6.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 66 | 29 | 0 | [6.5] | 5.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 67 | 29 | 0 | [6.5] | 5.0 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 68 | 29 | 0 | [6.5] | 10.5 | 0 | 18.5 |  | 5.0 |  | 3 | 31.0 + TT | 13.5 |  |
| 69 | 29 | 0 | 6.5 | 6.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 70 | 29 | 0 | [4.5] | 7.0 | 0 | 22 |  | 5.0 |  | 3 | 31.0 + TT | 17 |  |
| 71 | 29 | 0 | 2 | 7.0 | 0 | 22 |  | 5.0 |  | 3 | 31.0 + TT | 17 |  |
| 72 | 29 | 0 | 6.5 | 6.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 73 | 29 | 0 | [4.5] | 8.5 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 74 | 29 | 0 | 2 | 7.0 | 0 | 22 |  | 5.0 |  | 3 | 31.0 + TT | 17 |  |
| 75 | 29 | 0 | 2 | 4.5 | 0 | 24.5 |  | 3.0 |  | 3 | 31.0 + TT | 21.5 |  |
| 76 | 29 | 0 | 6.5 | 6.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 77 | 29 | 0 | [4.5] | 8.5 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 78 | 29 | 0 | 2 | 7.0 | 0 | 22 |  | 5.0 |  | 3 | 31.0 + TT | 17 |  |
| 79 | 29 | 0 | 2 | 4.5 | 0 | 24.5 |  | 3.0 |  | 3 | 31.0 + TT | 21.5 |  |
| 80 | 29 | 0 | [4.5] | 10.0 | 0 | 19 |  | 5.0 |  | 3 | 31.0 + TT | 14 |  |
| 81 | 29 | 0 | 6.5 | 6.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 82 | 29 | 0 | 2 | 7.0 | 0 | 22 |  | 5.0 |  | 3 | 31.0 + TT | 17 |  |
| 83 | 29 | 0 | 2 | 5.0 | 0 | 24 |  | 3.0 |  | 3 | 31.0 + TT | 21 |  |
| 84 | 29 | 0 | 2 | 7.0 | 0 | 22 |  | 5.0 |  | 3 | 31.0 + TT | 17 |  |
| 85 | 29 | 0 | 2 | 4.5 | 0 | 24.5 |  | 3.0 |  | 3 | 31.0 + TT | 21.5 |  |
| 86 | 29 | 0 | [4.5] | 10.0 | 0 | 19 |  | 5.0 |  | 3 | 31.0 + TT | 14 |  |
| 87 | 29 | 0 | 6.5 | 6.5 | 0 | 22.5 |  | 5.0 |  | 3 | 31.0 + TT | 17.5 |  |
| 88 | 29 | 0 | 2 | 5.0 | 0 | 24 |  | 3.0 |  | 3 | 31.0 + TT | 21 |  |
| 89 | 29 | 0 | 2 | 7.0 | 0 | 22 |  | 5.0 |  | 3 | 31.0 + TT | 17 |  |
| 90 | 29 | 0 | 2 | 7.0 | 0 | 22 |  | 5.0 |  | 3 | 31.0 + TT | 17 |  |
| 91 | 29 | 0 | 2 | 4.5 | 0 | 24.5 |  | 3.0 |  | 3 | 31.0 + TT | 21.5 |  |
| 92 | 29 | 0 | [4.5] | 15.5 | 0 | 13.5 |  | 6.0 |  | 3 | 31.0 + TT | 7.5 |  |
| 93 | 29 | 0 | 2 | 8.5 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 94 | 29 | 0 | 2 | 13.5 | 0 | 15.5 |  | 6.0 |  | 3 | 31.0 + TT | 9.5 |  |
| 95 | 29 | 0 | 2 | 7.0 | 0 | 22 |  | 5.0 |  | 3 | 31.0 + TT | 17 |  |
| 96 | 29 | 0 | 2 | 10.5 | 0 | 18.5 |  | 5.0 |  | 3 | 31.0 + TT | 13.5 |  |
| 97 | 29 | 0 | [4.5] | 15.5 | 0 | 13.5 |  | 6.0 |  | 3 | 31.0 + TT | 7.5 |  |
| 98 | 29 | 0 | 2 | 13.5 | 0 | 15.5 |  | 6.0 |  | 3 | 31.0 + TT | 9.5 |  |
| 99 | 29 | 0 | 2 | 8.5 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 100 | 29 | 0 | 2 | 7.0 | 0 | 22 |  | 5.0 |  | 3 | 31.0 + TT | 17 |  |
| 101 | 29 | 0 | 2 | 7.0 | 0 | 22 |  | 5.0 |  | 3 | 31.0 + TT | 17 |  |
| 102 | 29 | 0 | 2 | 10.5 | 0 | 18.5 |  | 5.0 |  | 3 | 31.0 + TT | 13.5 |  |
| 103 | 29 | 0 | 8.5 | 0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 104 | 29 | 0 | 8.5 | 7.0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 105 | 29 | 0 | [8.5] | 7.0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 106 | 29 | 0 | 8.5 | 0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 107 | 29 | 0 | 8.5 | 8.5 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 108 | 29 | 0 | [8.5] | 7.0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 109 | 29 | 0 | [8.5] | 0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 110 | 29 | 0 | 8.5 | 0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 111 | 29 | 0 | 8.5 | 8.5 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 112 | 29 | 0 | [8.5] | 7.0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 113 | 29 | 0 | [8.5] | 0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 114 | 29 | 0 | 8.5 | 10.0 | 0 | 19 |  | 5.0 |  | 3 | 31.0 + TT | 14 |  |
| 115 | 29 | 0 | 8.5 | 0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 116 | 29 | 0 | [8.5] | 7.0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 117 | 29 | 0 | [8.5] | 0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 118 | 29 | 0 | [8.5] | 7.0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 119 | 29 | 0 | [8.5] | 0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 120 | 29 | 0 | 8.5 | 10.0 | 0 | 19 |  | 5.0 |  | 3 | 31.0 + TT | 14 |  |
| 121 | 29 | 0 | 8.5 | 0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 122 | 29 | 0 | [8.5] | 0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 123 | 29 | 0 | [8.5] | 7.0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 124 | 29 | 0 | [8.5] | 7.0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 125 | 29 | 0 | [8.5] | 0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 126 | 29 | 0 | 8.5 | 15.5 | 0 | 13.5 |  | 6.0 |  | 3 | 31.0 + TT | 7.5 |  |
| 127 | 29 | 0 | [8.5] | 8.5 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 128 | 29 | 0 | [8.5] | 13.5 | 0 | 15.5 |  | 6.0 |  | 3 | 31.0 + TT | 9.5 |  |
| 129 | 29 | 0 | [8.5] | 7.0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 130 | 29 | 0 | [8.5] | 10.5 | 0 | 18.5 |  | 5.0 |  | 3 | 31.0 + TT | 13.5 |  |
| 131 | 29 | 0 | 8.5 | 15.5 | 0 | 13.5 |  | 6.0 |  | 3 | 31.0 + TT | 7.5 |  |
| 132 | 29 | 0 | [8.5] | 13.5 | 0 | 15.5 |  | 6.0 |  | 3 | 31.0 + TT | 9.5 |  |
| 133 | 29 | 0 | [8.5] | 8.5 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 134 | 29 | 0 | [8.5] | 7.0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 135 | 29 | 0 | [8.5] | 7.0 | 0 | 20.5 |  | 6.0 |  | 3 | 31.0 + TT | 14.5 |  |
| 136 | 29 | 0 | [8.5] | 10.5 | 0 | 18.5 |  | 5.0 |  | 3 | 31.0 + TT | 13.5 |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |  |  |  |  |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

### 6.2G.4	Configured transmitted power for Tx Diversity

6.2G.4.1	Test purpose

To verify the measured UE configured maximum output power PUMAX,f,c for Tx diversity is within the specified bounds.

6.2G.4.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2, and Power Class 3 UE release 15 and forward that support Tx diversity.

6.2G.4.3	Minimum conformance requirements

For UE supporting Tx diversity, the transmitted power is configured per each UE.

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in subclause 6.2.4.3 shall apply to UE supporting Tx diversity, where

-	PPowerClass, ΔPPowerClass and TC,c are specified in subclause 6.2.4.3 unless otherwise stated;

-	MPRc is specified in subclause 6.2G.2.3;

The measured configured maximum output power PUMAX,c for serving cell c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL, T LOW(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T HIGH(PCMAX_H,c)

where TLOW(PCMAX_L,c) and THIGH(PCMAX_H,c) are defined as the tolerance and applies to PCMAX_L,c and PCMAX_H,c separately, while TL is the absolute value of the lower tolerance in Table 6.2.1.3-1 for the applicable operating band.

For UE supporting Tx diversity, the tolerance is specified in Table 6.2G.4.3-1.

Table 6.2G.4.3-1: PCMAX,c tolerance for Tx Diversity

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2G.4.

6.2G.4.4	Test description

6.2G.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2G.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2G.4.4.1-1: Test Configuration Table for Tx Diversity

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range (NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| 33 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with PI/2 PBSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n34, n40, n41, n77, n78 and n79. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2G.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2G.4.4.3.

6.2G.4.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2G.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level of the test point.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under test.

6.2G.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.2G.4.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2G.4.4.3-2: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 0 |  |  |

Table 6.2G.4.4.3-3: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 14 |  |  |

Table 6.2G.4.4.3-4: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 18 |  |  |

Table 6.2G.4.4.3-5: FrequencyInfoUL-SIB: Test point 4

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  |  |

Table 6.2G.4.4.3-6: ServingCellConfig

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 0 |  | Test ID 1, 2 |
|  | 1 |  | Test ID 3 |
| } |  |  |  |
| } |  |  |  |

6.2G.4.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2G.4.5-1.

Table 6.2G.4.5-1: PCMAX configured UE output power

|  | Maximum output power |  |
| --- | --- | --- |
|  | Test ID 1,2 | Test ID 3 |
| Measured UE output power test point 1 | 0 dBm ± (7+TT) | 0 dBm ± (7+TT) |
| Measured UE output power test point 2 | 14 dBm ± (6+TT) | 14 dBm ± (6+TT) |
| Measured UE output power test point 3 | 18 dBm ± (5+TT) | 18 dBm ± (5+TT) |
| Measured UE output power test point 4 | Note 3 | Note 4 |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2G.4.5-4. Note 2: Power class 3 is default power class unless otherwise stated. Note 3: The maximum output power shall be within the range in Table 6.2G.4.5-2. Note 4: The maximum output power shall be within the range in Table 6.2G.4.5-3. |  |  |

Table 6.2G.4.5-2: Measured UE output power test point 4 for Test ID 1,2

| NR band | Tolerance (dB) |
| --- | --- |
| n1 | 20 dBm + 4+TT/-6-TT |
| n2 | 20 dBm + 4+TT/-6-TT |
| n3 | 20 dBm + 4+TT/-6-TT |
| n5 | 20 dBm + 4+TT/-6-TT |
| n7 | 20 dBm + 4+TT/-6-TT |
| n8 | 20 dBm + 4+TT/-6-TT |
| n12 | 20 dBm + 4+TT/-6-TT |
| n14 | 20 dBm + 4+TT/-6-TT |
| n20 | 20 dBm + 4+TT/-6-TT |
| n24 | 20 dBm + 4+TT/-6-TT |
| n25 | 20 dBm + 4+TT/-6-TT |
| n26 | 20 dBm + 4+TT/-6-TT |
| n28 | 20 dBm + 4+TT/-6-TT |
| n30 | 20 dBm + 4+TT/-6-TT |
| n34 | 20 dBm + 4+TT/-6-TT |
| n38 | 20 dBm + 4+TT/-6-TT |
| n39 | 20 dBm + 4+TT/-6-TT |
| n40 | 20 dBm + 4+TT/-6-TT |
| n41 | 20 dBm + 4+TT/-6-TT |
| n50 | 20 dBm + 4+TT/-6-TT |
| n51 | 20 dBm + 4+TT/-6-TT |
| n54 | 20 dBm + 4+TT/-6-TT |
| n65 | 20 dBm + 4+TT/-6-TT |
| n66 | 20 dBm + 4+TT/-6-TT |
| n70 | 20 dBm + 4+TT/-6-TT |
| n71 | 20 dBm + 4+TT/-6-TT |
| n74 | 20 dBm + 4+TT/-6-TT |
| n77 | 20 dBm + 4+TT/-6-TT |
| n78 | 20 dBm + 4+TT/-6-TT |
| n79 | 20 dBm + 4+TT/-6-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2D.4.5-4. |  |

Table 6.2G.4.5-3: Measured UE output power test point 4 for Test ID 3

| NR band | Tolerance (dB) |
| --- | --- |
| n34 | 23 dBm + 2+TT/-3-TT |
| n39 | 23 dBm + 2+TT/-3-TT |
| n40 | 23 dBm + 2+TT/-3-TT |
| n41 | 23 dBm + 2+TT/-3-TT |
| n77 | 23 dBm + 2+TT/-3-TT |
| n78 | 23 dBm + 2+TT/-3-TT |
| n79 | 23 dBm + 2+TT/-3-TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2G.4.5-4. |  |

Table 6.2G.4.5-4: Test Tolerance (configured transmitted power for Tx Diversity)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

## 6.3G	Output power dynamics for Tx Diversity

### 6.3G.1	Minimum output power for Tx Diversity

6.3G.1.1	Test purpose

Same test purpose as in 6.3.1.1.

6.3G.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the minimum output power is defined as the sum of the mean power at each transmit connector in one sub-frame (1 ms). The minimum output power shall not exceed the values specified in Table 6.3.1.3-1.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.1.

6.3G.1.4	Test description

Same test description as specified in clause 6.3.1.4 with following exceptions:

Step 3 of Test procedure as in 6.3.1.4.2 is replaced by:

3.	Measure the sum of mean power at each antenna connector in the associated measurement channel bandwidth specified in Table 6.3G.1.5-1 for the specific channel bandwidth under test. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

6.3G.1.5	Test requirement

The minimum output power, derived in step 3 shall not exceed the values specified in Table 6.3G.1.5-1.

| Table 6.3G.1.5-1: Minimum output powerChannel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -40+TT | 4.515 |
| 10 | -40+TT | 9.375 |
| 15 | -40+TT | 14.235 |
| 20 | -40+TT | 19.095 |
| 25 | -39+TT | 23.955 |
| 30 | -38.2+TT | 28.815 |
| 35 | -37.6+TT | 33.855 |
| 40 | -37+TT | 38.895 |
| 45 | -36.5+TT | 43.575 |
| 50 | -36+TT | 48.615 |
| 60 | -35.2+TT | 58.35 |
| 70 | -34.6+TT | 68.07 |
| 80 | -34+TT | 78.15 |
| 90 | -33.5+TT | 88.23 |
| 100 | -33+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3G.1.5-2 |  |  |

Table 6.3G.1.5-2: Test Tolerance (Minimum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.3 dB |
| 40MHz < BW ≤ 100MHz | 1.3 dB | 1.3 dB |

### 6.3G.2	Transmit OFF power for Tx Diversity

6.3G.2.1	Test purpose

Same test purpose as in 6.3.2.1.

6.3G.2.2	Test applicability

The requirements of this test apply in test cases 6.3G.3 Transmit ON/OFF time mask for Tx Diversity to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward support Tx diversity.

6.3G.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the transmit OFF power is defined as the mean power at each transmit antenna connector in a duration of at least one sub-frame (1 ms) excluding any transient periods.

The transmit OFF power at each transmit antenna connector shall not exceed the values specified in Table 6.3.2.3-1.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.2.

6.3G.2.4	Test description

This test is covered by clause 6.3G.3 Transmit ON/OFF time mask.

6.3G.2.5	Test requirement

For each transmit antenna connector, the requirement for the Transmit OFF power shall not exceed the values specified in Table 6.3G.2.5-1.

| Table 6.3G.2.5-1: Transmit OFF powerChannel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -50+TT | 4.515 |
| 10 | -50+TT | 9.375 |
| 15 | -50+TT | 14.235 |
| 20 | -50+TT | 19.095 |
| 25 | -50+TT | 23.955 |
| 30 | -50+TT | 28.815 |
| 35 | -50+TT | 33.855 |
| 40 | -50+TT | 38.895 |
| 45 | -50+TT | 43.575 |
| 50 | -50+TT | 48.615 |
| 60 | -50+TT | 58.35 |
| 70 | -50+TT | 68.07 |
| 80 | -50+TT | 78.15 |
| 90 | -50+TT | 88.23 |
| 100 | -50+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3G.2.5-2 |  |  |

Table 6.3G.2.5-2: Test Tolerance (Transmit OFF power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

### 6.3G.3	Transmit ON/OFF time mask for Tx Diversity

#### 6.3G.3.1	General ON/OFF time mask for Tx Diversity

6.3G.3.1.1	Test purpose

Same test purpose as in 6.3.3.2.1.

6.3G.3.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.3.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the general ON/OFF time mask requirements in clause 6.3.3.2.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.3.

6.3G.3.1.4	Test description

Same test description as in clause 6.3.3.2.4 with following exceptions:

-	The ON power is measured as sum of both transmit antenna connectors.

-	The OFF power is measured at each transmit antenna connector.

6.3G.3.1.5	Test requirement

The measured ON power and OFF power shall not exceed the values specified in Table 6.3G.3.1.5-1.

Table 6.3G.3.1.5-1: General ON/OFF time mask

|  |  |  |  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power |  |  |  | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth |  | 4.515 | 9.375 |  | 14.235 | 19.095 | 23.955 | 28.815 | 33.855 | 38.895 | 43.575 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Transmit ON power |  |  |  | Same as Test ID 9 of Tables 6.2G.2.5-5 to 6.2G.2.5-8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: TT of OFF power for each frequency and channel bandwidth is specified in Table 6.3G.3.1.5-2 NOTE 2: TT of ON power for each frequency and channel bandwidth is specified in the referred ON power test requirement tables. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.3.1.5-2: Test Tolerance for OFF power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

Table 6.3G.3.1.5-3: Void

#### 6.3G.3.2	PRACH time mask for Tx Diversity

6.3G.3.2.1	Test purpose

Same test purpose as in 6.3.3.4.1.

6.3G.3.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.3.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the PRACH time mask requirements in clause 6.3.3.4.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.3.

6.3G.3.2.4	Test description

Same test description as in clause 6.3.3.4.4 with following exceptions:

-	The ON power is measured as sum of both transmit antenna connectors.

-	The OFF power is measured at each transmit antenna connector.

6.3G.3.2.5	Test requirement

The measured ON power and OFF power shall not exceed the values specified in Table 6.3G.3.2.5-1.

Table 6.3G.3.2.5-1: PRACH time mask

|  |  |  |  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power |  |  |  | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth |  | 4.515 | 9.375 |  | 14.235 | 19.095 | 23.955 | 28.815 | 33.855 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Expected PRACH Transmission ON Measured Power for PRACH Format 0 and PRACH Format A3 for SCS 30kHz |  | -1 dBm | -1 dBm |  | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm |
| Expected PRACH Transmission ON Measured Power for PRACH Format A3 for SCS 15kHz and SCS 60kHz |  | -2 dBm | -2 dBm |  | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm |
| ON Power Tolerance |  |  |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: TT of OFF power for each frequency and channel bandwidth is specified in Table 6.3G.3.2.5-2 NOTE 2: TT of ON power for each frequency and channel bandwidth is specified in Table 6.3.3.4.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.3.2.5-2: Test Tolerance (Transmit OFF power and PRACH time mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

#### 6.3G.3.3	SRS time mask for Tx Diversity

6.3G.3.3.1	Test purpose

Same test purpose as in 6.3.3.6.1.

6.3G.3.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.3.3.3	Minimum conformance requirements

For UE supporting Tx diversity, the SRS time mask requirements in clause 6.3.3.6.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.3.

6.3G.3.3.4	Test description

Same test description as in clause 6.3.3.6.4 with following exceptions:

-	The ON power is measured as sum of both transmit antenna connectors.

-	The OFF power is measured at each transmit antenna connector.

6.3G.3.3.5	Test requirement

The measured ON power and OFF power shall not exceed the values specified in Table 6.3G.3.3.5-1.

Table 6.3G.3.3.5-1: SRS time mask

|  |  |  |  |  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power |  |  |  |  | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth |  |  | 4.515 | 9.375 |  | 14.235 | 19.095 | 23.955 | 28.815 | 33.855 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Transmit ON power |  |  |  |  | Same as Table 6.2G.1.5-4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NOTE 1: TT of OFF power for each frequency and channel bandwidth is specified in Table 6.3G.3.3.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.3.3.5-2: Test Tolerance (Transmit OFF power and SRS time mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

### 6.3G.4	Power control for Tx Diversity

#### 6.3G.4.1	Absolute power tolerance for Tx Diversity

6.3G.4.1.1	Test purpose

Same test purpose as in 6.3.4.2.1.

6.3G.4.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.4.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the power control tolerance applies to the sum of output power at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.4

6.3G.4.1.4	Test description

Same test description as in clause 6.3.4.2.4 with following exceptions:

-	The UE’s transmit power is measured as sum of both transmit antenna connectors.

6.3G.4.1.5	Test requirement

The requirement for the power measured in step (2) of the test procedure is not to exceed the values specified in Table 6.3G.4.1.5-1 and 6.3G.4.1.5-2.

Table 6.3G.4.1.5-1: Absolute power tolerance: test point 1

|  |  |  |  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Expected Measured power |  | SCS15 | -17.6 | -14.4 | -12.6 | -11.3 | -10.4 | -9.6 | -8.9 | -8.3 | -7.8 | -7.3 | N/A | N/A | N/A | N/A | N/A |
|  |  | SCS30 | -18.2 | -14.8 | -12.8 | -11.5 | -10.5 | -9.7 | -9 | -8.3 | -7.8 | -7.4 | -6.5 | -5.8 | -5.2 | -4.7 | -4.2 |
|  |  | SCS60 |  | -15.2 | -13 | -11.8 | -10.7 | -9.8 | -9.1 | -8.5 | -7.9 | -7.5 | -6.6 | -5.9 | -5.3 | -4.8 | -4.3 |
| Power tolerance |  |  |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3G.1.5. Note 2: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3G.4.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.4.1.5-2: Absolute power tolerance: test point 2

|  |  |  |  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Expected Measured power |  | SCS15 | -3.6 | 0.4 | 1.4 | 2.7 | 3.6 | 4.4 | 5.1 | 5.7 | 6.2 | 6.7 | N/A | N/A | N/A | N/A | N/A |
|  |  | SCS30 | -4.2 | -0.8 | 1.2 | 2.5 | 3.5 | 4.3 | 5 | 5.7 | 6.2 | 6.6 | 7.5 | 8.2 | 8.8 | 9.3 | 9.8 |
|  |  | SCS60 | N/A | -1.2 | 1 | 2.2 | 3.3 | 4.2 | 4.9 | 5.5 | 6.1 | 6.5 | 7.4 | 8.1 | 8.7 | 9.2 | 9.7 |
| Power tolerance |  |  |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Note 1: The higher power limit shall not exceed the maximum output power requirements defined in sub-clause 6.2G.1.5. Note 2: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3G.4.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.4.1.5-3: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.4 dB | 1.4 dB |
| 40MHz < BW ≤ 100MHz | 1.4 dB | 1.4 dB | 1.4 dB |

#### 6.3G.4.2	Relative power tolerance for Tx Diversity

6.3G.4.2.1	Test purpose

Same test purpose as in 6.3.4.3.1.

6.3G.4.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.4.2.3	Minimum conformance requirement

For UE supporting Tx diversity, the relative power tolerance applies to the sum of output power at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.4.

6.3G.4.2.4	Test description

Same test description as in clause 6.3.4.3.4 with the output power is measured as the sum of both antenna connectors.

6.3G.4.2.5	Test requirement

Each UE power step measured in the test procedure 6.3G.4.2.4 should satisfy the test requirements specified in Table 6.3G.4.2.5-1 through 6.3G.4.2.5-7.

To account for RF Power amplifier mode changes 2 exceptions are allowed for each of ramping up and ramping down test patterns. For these exceptions the power tolerance limit is a maximum of ± (6.0 + TT) dB. If there is an exception in the power step caused by the RB change for all test patterns (A, B, C) then fail the UE.

Table 6.3G.4.2.5-1: Test Requirements Relative Power Tolerance for Transmission, channel BW 5MHz, ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 15 RBs | TPC=+1dB | 12.76 | 10dB ≤ ΔP < 15dB | 12.76 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 15 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 10 RBs | TPC=+1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 10 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Sub-frames Pattern B the position of RB uplink allocation change is after 20 active uplink Sub-frames Pattern C the position of RB uplink allocation change is after 30 active uplink Sub-frames Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-2: Test Requirements Relative Power Tolerance for Transmission, channel BW 5MHz, ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Sub-frames before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 |  | Sub-frames before RB change | Fixed = 15 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 15 RBs to 1 RB | TPC=-1dB | 12.76 | 10dB ≤ ΔP < 15dB | 12.76 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Sub-frames before RB change | Fixed = 10 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 10 RBs to 1 RB | TPC=-1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Sub-frames Pattern B the position of RB uplink allocation change is after 20 active uplink Sub-frames Pattern C the position of RB uplink allocation change is after 30 active uplink Sub-frames Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-3: Test Requirements Relative Power Tolerance for Transmission, channel BW 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 35MHz, 40MHz, 50MHz ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 2 | RB change | 1RB to 20 RBs | TPC=+1dB | 14.01 | 10dB ≤ ΔP < 15dB | 14.01 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 20 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 1RB to 50 RBs | TPC=+1dB | 17.99 | 15dB ≤ ΔP | 17.99 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 50 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 2 | RB change | 1RB to 24 RBs | TPC=+1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80+/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 24 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 10 RBs | TPC=+1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 10 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-4: Test Requirements Relative Power Tolerance for Transmission, channel BW 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 35MHz, 40MHz, 50MHz ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1RBs | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 20 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 2 | RB change | 20 RBs to 1 RB | TPC=-1dB | 14.01 | 10dB ≤ ΔP < 15dB | 14.01 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 50 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 50 RBs to 1 RB | TPC=-1dB | 17.99 | 15dB ≤ ΔP | 17.99 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  |  |  |  |  |  |  |
|  |  | Subframes before RB change | Fixed = 24 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 2 | RB change | 24 RBs to 1 RB | TPC=-1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | Fixed = 10 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 2 | RB change | 10 RBs to 1 RB | TPC=-1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-5: Test Requirements Relative Power Tolerance for Transmission, channel BW 60MHz, 70MHz, 80MHz, 90MHz, 100MHz ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 | 2 | RB change | 1RB to 24 RBs | TPC=+1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 24 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 1RB to 81 RBs | TPC=+1dB | 20.08 | 15dB < ΔP | 20.08 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 81 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 75 RBs | TPC=+1dB | 19.75 | 15dB < ΔP | 19.75 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 75 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-6: Test Requirements Relative Power Tolerance for Transmission, channel BW 60MHz, 70MHz, 80MHz, 90MHz, 100MHz ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 24 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 | 2 | RB change | 24 RBs to 1 RB | TPC=-1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 81 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 81 RBs to 1 RB | TPC=-1dB | 20.08 | 15dB < ΔP | 20.08 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | Fixed = 75 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 75 RBs to 1 RB | TPC=-1dB | 19.75 | 15dB < ΔP | 19.75 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-7: Test Requirements Relative Power Tolerance for Transmission, alternating sub-test

| BW | Test SCS [kHz] | Sub-test ID | Uplink RB allocation | TPC command | Expected power step size (Up or Down) | Power step size range (Up or Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 15 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
| 5 |  | 3 | Alternating 1 and 15 | TPC=0dB | 11.76 | 10dB ≤ ΔP < 15dB | 11.76 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 30 | 2 | Alternating 1 and 10 | TPC=0dB | 10.00 | 10dB ≤ ΔP < 15dB | 10.00 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  |  | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  | 15 | 3 | Alternating 1 and 20 | TPC=0dB | 13.01 | 10dB ≤ ΔP < 15dB | 13.01 +/- (4 + TT) |
|  |  | 4 | Alternating 1 and 50 | TPC=0dB | 16.99 | 15dB ≤ ΔP | 16.99 +/- (5 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
| 10,15,20, 25,30,40,50 | 30 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 24 | TPC=0dB | 13.80 | 10dB ≤ ΔP < 15dB | 13.80 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  | 60 | 2 | Alternating 1 and 10 | TPC=0dB | 10.00 | 10dB ≤ ΔP < 15dB | 10.00 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 30 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 81 | TPC=0dB | 19.08 | 15dB < ΔP | 19.08 +/- (5 + TT) |
| 60, 70,80,90,100 |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 60 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 75 | TPC=0dB | 18.75 | 15dB < ΔP | 18.75 +/- (5 + TT) |
| Note 1: The starting resource block shall be RB# 0. Note 2: TT=0.7dB Note 3: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

#### 6.3G.4.3	Aggregate power tolerance for Tx Diversity

6.3G.4.3.1	Test purpose

Same test purpose as in 6.3.4.4.1.

6.3G.4.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.4.3.3	Minimum conformance requirements

For UE supporting Tx diversity, the absolute power tolerance applies to the sum of output power at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.4.

6.3G.4.3.4	Test description

Same test description as in clause 6.3.4.4.4 with the output power is measured as the sum of both antenna connectors.

6.3G.4.3.5	Test requirement

The requirement for the power measurements made in step 1.3 and 2.3 of the test procedure shall not exceed the values specified in Table 6.3G.4.3.5-1. The power measurement period shall be 1 sub-frame(1ms).

Table 6.3G.4.3.5-1: Power control tolerance

| TPC command | UL channel | Test requirement measured power |
| --- | --- | --- |
| 0 dB | PUCCH | Given 5power measurements in the pattern, the 2nd, and later measurements shall be within ± (2.5 + TT) dB of the 1st measurement. |
| 0 dB | PUSCH | Given 5 power measurements in the pattern, the 2nd, and later measurements shall be within ± (3.5 + TT) dB of the 1st measurement. |
| Note 1: For SCS 30kHz 1 sub-frame corresponds to 2 slots and for SCS 60kHz 1 sub-frame corresponds to 4 slots, so 2 TPC commands will be sent for a single measurement period. Note 2: TT=0.7dB. |  |  |

## 6.5D	Output RF spectrum emissions for UL MIMO

### 6.5D.1	Occupied bandwidth for UL MIMO

6.5D.1.1	Test purpose

To verify that the UE occupied bandwidth for all transmission bandwidth configurations supported by the UE supporting UL MIMO are less than their specific limits when UE is configured using UL MIMO transmission.

6.5D.1.2	Test applicability

This test applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

6.5D.1.3	Minimum conformance requirements

For UE supporting UL MIMO, the requirements for occupied bandwidth apply to the sum of the powers from both UE transmit antenna connectors. The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the occupied bandwidth at each transmitter antenna shall be less than the channel bandwidth specified in table 6.5.1.3-1. The requirements shall be met with UL MIMO configurations described in sub-clause 6.2D.1.3.

If UE is configured for transmission on single-antenna port by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in subclause 6.5.1.3 apply

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5D.1.

6.5D.1.4	Test description

6.5D.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5D.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range by default, exceptions listed in Table 6.5D.1.4.1-2 |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | All |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest SCS |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for occupied bandwidth test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Outer_full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

Table 6.5D.1.4.1-2: Test frequency exceptions for Occupied Bandwidth

| 5G NR Band | Test Frequency |
| --- | --- |
| n77 | Low Range, Mid Range, High Range |
| n78 | Low Range, Mid Range, High Range |
| n79 | Low Range, Mid Range, High Range |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5D.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.1.4.3

6.5D.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the power spectrum distribution as the sum of the powers from both UE transmit antenna connectors within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency. The characteristics of the filter shall be approximately Gaussian (typical spectrum analyser filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	Calculate the total power within the range of all frequencies measured in step 3 and save this value as “Total power”.

5.	Sum up the power upward from the lower boundary of the measured frequency range in step 3 and seek the limit frequency point by which this sum becomes 0.5% of “Total power” and save this point as “Lower Frequency”.

6.	Sum up the power downward from the upper boundary of the measured frequency range in step 3 and seek the limit frequency point by which this sum becomes 0.5% of “Total power” and save this point as “Upper Frequency”.

7.	Calculate the difference “Upper Frequency” – “Lower Frequency” = “Occupied Bandwidth” between the two limit frequencies obtained in step 5 and step 6.

6.5D.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO

6.5D.1.5	Test requirement

The measured Occupied Bandwidth in step 7 shall not exceed values in Table 6.5D.1.5-1.

Table 6.5D.1.5-1: Occupied channel bandwidth

|  | NR channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Occupied channel bandwidth (MHz) | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |

### 6.5D.1_1	Void

### 6.5D.1_2	Occupied bandwidth for SUL with UL MIMO

6.5D.1_2.1	Test purpose

Same as test purpose in clause 6.5D.1.1.

6.5D.1_2.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

6.5D.1_2.3	Minimum conformance requirements

For UE supporting UL MIMO, the requirements for occupied bandwidth apply to the sum of the powers from both UE transmit antenna connectors. The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the occupied bandwidth shall be less than the channel bandwidth specified in table 6.5.1.3-1. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1_1.3.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook-based transmission, the requirements in clause 6.5.1 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5D.1.

6.5D.1_2.4	Test description

6.5D.1_2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5D.1_2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.1_2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid Range for SUL carrier Mid Range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | All for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
| Test ID | N/A | N/A |  | Modulation | RB allocation |
| 1 |  |  |  | CP-OFDM QPSK | Outer_full |
| Note 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. Note 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 6.5D.1_2.4.1-1

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3D.4.3.4.3.

6.5D.1_2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.1_2.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the power spectrum distribution as the sum of the powers from both UE transmit antenna connectors within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency. The characteristics of the filter shall be approximately Gaussian (typical spectrum analyser filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	Calculate the total power within the range of all frequencies measured in step 3 and save this value as “Total power”.

5.	Sum up the power upward from the lower boundary of the measured frequency range in step 3 and seek the limit frequency point by which this sum becomes 0.5% of “Total power” and save this point as “Lower Frequency”.

6.	Sum up the power downward from the upper boundary of the measured frequency range in step 3 and seek the limit frequency point by which this sum becomes 0.5% of “Total power” and save this point as “Upper Frequency”.

7.	Calculate the difference “Upper Frequency” – “Lower Frequency” = “Occupied Bandwidth” between the two limit frequencies obtained in step 5 and step 6.

6.5D.1_2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, and Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.5D.1_2.5	Test requirement

The measured Occupied Bandwidth in step 7 shall not exceed values in Table 6.5D.1_2.5-1.

Table 6.5D.1_2.5-1: Occupied channel bandwidth

|  | NR channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 45 MHz | 50 MHz |
| Occupied channel bandwidth (MHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 |

### 6.5D.2	Out of band emission for UL MIMO

#### 6.5D.2.1	General

For UE supporting UL MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters are specified as the sum of the emissions from both UE transmit antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclause 6.5.2 apply. The requirements shall be met with UL MIMO configurations described in sub-clause 6.2D.1.3

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook-based transmission, the requirements in subclause 6.5.2 apply.

#### 6.5D.2.2	Spectrum emission mask for UL MIMO

6.5D.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth.

6.5D.2.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that supports 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.5D.2.2.3	Minimum conformance requirements

The sum of power of any UE emission shall not exceed the levels specified in Table 6.5.2.2.3-1 for the specified channel bandwidth from both transmit antenna connectors.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the requirements in clause 6.5.2.2 shall apply. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.5.2.2 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5D.2 and 6.5.2.2

6.5D.2.2.4	Test description

6.5D.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in tables 6.5D.2.2.4.1-1 to 6.5D.2.2.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5D.2.2.4.1-1: Test Configuration Table for power class 3, power class 2 and power class 1.5

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Spectrum Emission Mask test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 2 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Default |  |  |  | CP-OFDM QPSK | Outer_Full |
| 4 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 5 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 6 | Default |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 7 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 8 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 9 | Default |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| 10 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | CP-OFDM 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.5D.2.2.4.1-2: Test Configuration Table for power class 3 with supporting ULFPTx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power Reduction (MPR) test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 14 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 24 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 34 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 44 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 18 | Default |  | CP-OFDM QPSK | Outer Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 21 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 16 ~ 27 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Table 6.5D.2.2.4.1-3: Test Configuration Table for power class 2 with supporting ULFPTx

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power Reduction (MPR) test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 18 | Default |  | CP-OFDM QPSK | Outer Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 21 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 16 ~ 27 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. |  |  |  |  |

Table 6.5D.2.2.4.1-4: Test Configuration Table for power class 1.5 UEs supporting ULFPTx except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 18 | Default |  | CP-OFDM QPSK | Outer Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 21 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Test ID 16 ~ 27 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to tables 6.5D.2.2.4.1-1 to 6.5D.2.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.2.2.4.3

6.5D.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.2.2.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2D.2.5-1 or 6.2D.2.5-2 as appropriate. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of power of the transmitted signal from both antenna connectors with a measurement filter of bandwidths according to Table 6.5D.2.2.5-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

5.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.5D.2.2.4.1-2 through Table 6.5D.2.2.4.1-4 as appropriate. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.5D.2.2.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5D.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.5D.2.2.5	Test requirement

The measured sum of the UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in clause 6.2D.2.5 as appropriate, and the sum of power of any UE emission measured from both antennas in step 4 and step 5 shall fulfill requirements in Table 6.5D.2.2.5-1.

Table 6.5D.2.2.5-1: NR General spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 + TT | -13 + TT |  | 1 % of channel BW |
| ± 0-1 |  |  | -24 + TT | 30 kHz |
| ± 1-5 | -10 + TT | -10 + TT |  | 1 MHz |
| ± 5-6 | -13 + TT |  |  |  |
| ± 6-10 | -25 + TT |  |  |  |
| ± 5-BWChannel |  | -13 + TT |  |  |
| ± BWChannel-(BWChannel+5) |  | -25 + TT |  |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5D.2.2.5-2. |  |  |  |  |

Table 6.5D.2.2.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5D.2.2_1	Spectrum emission mask for SUL with UL MIMO

6.5D.2.2_1.1	Test purpose

Same as test purpose in clause 6.5D.2.2.1.

6.5D.2.2_1.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

This test applies to all types of NR UE release 17 and forward that support SUL and UL full power transmission (ULFPTx) on the SUL bands.

6.5D.2.2_1.3	Minimum conformance requirements

Same as minimum conformance requirements in clause 6.5D.2.2.3.

6.5D.2.2_1.4	Test description

6.5D.2.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing and are shown in tables 6.5D.2.2_1.4.1-1 to 6.5D.2.2_1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5D.2.2_1.4.1-1: Test Configuration Table for power class 3 and power class 2

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range for SUL carrier Mid-range for Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration | SUL Configuration |  |  |
|  |  | N/A | N/A | Modulation |  | RB allocation |
| 1 | Low |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 2 | High |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 3 | Default |  |  | CP-OFDM QPSK |  | Outer_Full |
| 4 | Low |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 5 | High |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 6 | Default |  |  | CP-OFDM 16 QAM |  | Outer_Full |
| 7 | Low |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 8 | High |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  | CP-OFDM 64 QAM |  | Outer_Full |
| 10 | Low |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 11 | High |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  | CP-OFDM 256 QAM |  | Outer_Full |
| Note 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. Note 2: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.5D.2.2_1.4.1-2: Test Configuration Table for UE supporting ULFPTx

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range for SUL carrier Mid-range for Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration | SUL Configuration |  |  |
|  |  | N/A | N/A | Modulation |  | RB allocation |
| 15 | Low |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 25 | High |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 35 | Default |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer_Full |
| 45 | Low |  |  | DFT-s-OFDM QPSK |  | Edger_1RB_Left |
| 5 | High |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  | DFT-s-OFDM QPSK |  | Outer_Full |
| 7 | Low |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  | DFT-s-OFDM 16 QAM |  | Outer_Full |
| 10 | Low |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  | DFT-s-OFDM 64 QAM |  | Outer_Full |
| 13 | Low |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 14 | High |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 15 | Default |  |  | DFT-s-OFDM 256 QAM |  | Outer_Full |
| 16 | Low |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 17 | High |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 18 | Default |  |  | CP-OFDM QPSK |  | Outer_Full |
| 19 | Low |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 | High |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 21 | Default |  |  | CP-OFDM 16 QAM |  | Outer_Full |
| 22 | Low |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 23 | High |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 24 | Default |  |  | CP-OFDM 64 QAM |  | Outer_Full |
| 25 | Low |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 26 | High |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 27 | Default |  |  | CP-OFDM 256 QAM |  | Outer_Full |
| Note 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. Note 2: The specific configuration of each RF allocation is defined in Table 6.1-1. Note 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. Note 4: Test ID 16 ~ 27 with CP-OFDM modulation are not needed if PDCCH DCI format 0_1 indicates ULFPTx_Mode1. Note 5: For UE indicating PC2, or UE indicating PC3 and operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 6.5D.2.2_1.4.1-1 and 6.5D.2.2_1.4.1-2

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3D.4.3.4.3.

6.5D.2.2_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.2.2_1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2D.1_1.5-1. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of power of the transmitted signal from both antenna connectors with a measurement filter of bandwidths according to Table 6.5D.2.2_1.5-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

5.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.5D.2.2_1.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.5D.2.2_1.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5D.2.2_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, and Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.5D.2.2_1.5	Test requirement

The measured sum of the UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables [6.2D.2.5-1, 6.2D.2.5-2, 6.2D.2.5-2a and 6.2D.2.5-2b] as appropriate, and the sum of power of any UE emission measured from both antennas in step 4 and step 5 shall fulfil requirements in Table 6.5D.2.2.5-1.

Table 6.5D.2.2_1.5-1: NR General spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 + TT | -13 + TT |  | 1 % of channel BW |
| ± 0-1 |  |  | -24 + TT | 30 kHz |
| ± 1-5 | -10 + TT | -10 + TT |  | 1 MHz |
| ± 5-6 | -13 + TT |  |  |  |
| ± 6-10 | -25 + TT |  |  |  |
| ± 5-BWChannel |  | -13 + TT |  |  |
| ± BWChannel-(BWChannel+5) |  | -25 + TT |  |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5D.2.2.5-2. |  |  |  |  |

Table 6.5D.2.2_1.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5D.2.3	Additional spectrum emission mask for UL MIMO

Editor’s note:

-Test coverage for the NS_XXs other than NS_03, NS_03U, NS_04 and NS_35 is FFS

- Supporting of ULFPTx is only completed for NS_04, NS_03, NS_03U in A-MPR

6.5D.2.3.1	Test purpose

To verify that the sum of power of any UE emission from both transmit antennas shall not exceed specified lever for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.5D.2.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3UE release 15 and forward that supports 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3UE release 16 and forward that supports UL full power transmission (ULFPTx).

6.5D.2.3.3	Minimum conformance requirements

The sum of power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3-1 for NS_35, Table 6.5.2.3.3.2-1 for NS_04, and Table 6.5.2.3.3.3 for NS_03 and NS_03U for the specified channel bandwidth from both transmit antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5D.2 and 6.5.2.3

6.5D.2.3.4	Test description

6.5D.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in clause 6.2D.3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.2.3.4.1-1: Void

Table 6.5D.2.3.4.1-2: Void

Table 6.5D.2.3.4.1-3: Void

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to applicable test configuration table in clause 6.2D.3.4.1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.2.3.4.3.

6.5D.2.3.4.2	Test procedure

Same test procedure as defined in clause 6.5D.2.2.4.2 with the following exceptions:

-	Instead of Table 6.2D.2.5-1, test requirements in clause 6.2D.3.5 are applied in step 3;

-	Instead of Table 6.5D.2.2.5-1, test requirements in clause 6.5D.2.3.5 are applied in step 4;

-	Instead of Table 6.5D.2.2.4.1-2 and Table 6.5D.2.2.4.1-3, test configurations for ULFPTx in clause 6.2D.3 are applied in step 5;

6.5D.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO and exceptions listed in clause 6.2D.3.4.3.

6.5D.2.3.5	Test requirement

The measured sum of the UE mean power in the channel bandwidth, derived in step 3 and step 5 shall fulfill the requirements as specified in clause 6.2D.3.5 for NS_35, NS_04, NS_03 and NS_03U as appropriate, and the sum of power of any UE emission measured from both antennas in step 4 and step 5 shall fulfill requirements in current subclause.

Table 6.5D.2.3.5-1: Additional requirements for “NS_35”

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| 0-0.1 | -15 + TT | -18 + TT | -20 + TT | -21 + TT | 30 kHz |
| 0.1-6 | -13 + TT | -13 + TT | -13 + TT | -13 + TT | 100 kHz |
| 6-10 | -251 + TT | -13 + TT | -13 + TT | -13 + TT | 100 kHz |
| 10-15 |  | -251 + TT | -13 + TT | -13 + TT | 100 kHz |
| 15-20 |  |  | -251 + TT | -13 + TT | 100 kHz |
| 20-25 |  |  |  | -25 + TT | 1 MHz |
| NOTE 1: The measurement bandwidth shall be 1 MHz; NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.5D.2.3.5-2 |  |  |  |  |  |

Table 6.5D.2.3.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

Table 6.5D.2.3.5-3: Additional requirements for "NS_04"

|  | Spectrum emission limit (dBm) / measurement bandwidth for each channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Measurement bandwidth |
| ± 0 - 1 | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT |  |  |  |  |  |  | 2 % channel bandwidth |
|  |  |  |  |  |  | -10 + TT |  |  |  |  |  | 1 MHz |
| ± 1 - 5 | -10 + TT |  |  |  |  |  |  |  |  |  |  | 1 MHz |
| ± 5 - X | -13 + TT |  |  |  |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: X is defined in Table 6.5D.2.3.5-3a for CP-OFDM NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.5D.2.3.5-2. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5D.2.3.5-3a: n41 maximum transmission bandwidths (MHz) for CP-OFDM

| SCS (kHz) | Channel bandwidths (MHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| 15 | 9.36 | 14.22 | 19.08 | 28.80 | 38.88 | 48.6 | N/A | N/A | N/A | N/A | N/A |
| 30 | 8.64 | 13.68 | 18.36 | 28.08 | 38.16 | 47.88 | 58.32 | 68.04 | 78.12 | 88.02 | 98.28 |
| 60 | 7.92 | 12.96 | 17.28 | 27.36 | 36.72 | 46.8 | 56.88 | 66.96 | 77.04 | 87.12 | 97.20 |

Table 6.5D.2.3.5-4: Additional requirements for "NS_03 and NS_03U"

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 35, 40, 45 |  |
| 0-1 | -13 + TT | -13 + TT | 1 % of channel BW |
| 1-6 | -13 + TT | -13 + TT | 1 MHz |
| 6-10 | -25 + TT |  |  |
| 1-BWChannel |  |  |  |
| BWChannel-(BWChannel+5) |  | -25 + TT |  |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5D.2.3.5-2. |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5D.2.4	Adjacent channel leakage ratio for UL MIMO

Adjacent channel leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

##### 6.5D.2.4.1	NR ACLR for UL MIMO

6.5D.2.4.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5D.2.4.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that supports 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.5D.2.4.1.3	Minimum conformance requirements

For UE supporting UL MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters is defined as the sum of the emissions from both UEtransmit antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclause 6.5.2.4.2.3 apply. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1.3.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the requirements in clause 6.5.2.4.2.3 shall apply. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.5.2.4.2.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] clauses and 6.5D.2 and 6.5.2.4.1.

6.5D.2.4.1.4	Test description

6.5D.2.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in the test configuration tables in clause 6.2D.2. 4.1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5D.2.4.1.4.1-1: Void

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to the test configuration tables in clause 6.2D.2.4.1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.2.4.1.4.3 (If UE supports ULFPTx. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.)

6.5D.2.4.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the test configuration tables in clause 6.2D.2. 4.1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at from both antenna connectors in the channel bandwidth of the radio access mode according to the test configuration, as measured in step 3 of 6.2D.2.4.2,which shall meet the requirements described in clause 6.2D.2.5 as appropriate.

4.	Measure the sum of the rectangular filtered mean power for the assigned NR channel at each antenna connector of UE using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the sum of rectangular filtered mean power of the first NR adjacent channel at each antenna connector of UE on both lower and upper side of the assigned NR channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NR ACLR, respectively.

7.	If UE supports ULFPTx, repeat test steps 1~6 with UL RMC according to Table 6.2D.2.4.1-1a through Table 6.2D.2.4.1-3a. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.

6.5D.2.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.5D.2.4.1.5	Test requirement

The measured UE mean total power derived in step 3, shall fulfil requirements in Clause 6.2D.2.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 6 for each antenna connector, shall be higher than the limits in Table 6.5D.2.4.1.5-2.

Table 6.5D.2.4.1.5-1: Void

Table 6.5D.2.4.1.5-2: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR |  | 31 - TT dB | 31 - TT dB | 30 - TT dB |
| NOTE 1: TT = 0.8 dB |  |  |  |  |

##### 6.5D.2.4.1_1	NR ACLR for SUL with UL MIMO

6.5D.2.4.1_1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage Power Ratio (ACLR).

6.5D.2.4.1_1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

6.5D.2.4.1_1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

For UE supporting UL MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters is defined as the sum of the emissions from both UEtransmit antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclause 6.5.2.4.2.3 apply. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1_1.3.

The normative reference for this requirement is TS 38.101-1 [2] clauses and 6.5D.2 and 6.5.2.4.1.

6.5D.2.4.1_1.4	Test description

6.5D.2.4.1_1.4.1	Initial conditions

Same initial conditions as in 6.2D.2_1.4.1.

6.5D.2.4.1_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the test configuration tables in clause 6.2D.2_1. 4.1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at from both antenna connectors in the channel bandwidth of the radio access mode according to the test configuration, as measured in step 3 of 6.2D.2_1.4.2, which shall meet the requirements described in clauses 6.2D.2_1.5 as appropriate.

4.	Measure the sum of the rectangular filtered mean power for the assigned NR channel at each antenna connector of UE using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the sum of rectangular filtered mean power of the first NR adjacent channel at each antenna connector of UE on both lower and upper side of the assigned NR channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NR ACLR, respectively.

6.5D.2.4.1_1.4.3	Message contents

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Table 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL.

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.5D.2.4.1_1.5	Test requirement

The measured UE mean total power derived in step 3, shall fulfil requirements in clause 6.2D.2_1.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 6 for each antenna connector, shall be higher than the limits in Table 6.5D.2.4.1_1.5-1.

Table 6.5D.2.4.1_1.5-1: NR ACLR requirement

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NR ACLR |  | 31 - TT dB | 30 - TT dB |
| NOTE 1: TT = 0.8 dB |  |  |  |

##### 6.5D.2.4.2	UTRA ACLR for UL MIMO

6.5D.2.4.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5D.2.4.2.2	Test applicability

This test case applies for NS_03U, NS_05U, and NS_100 to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that supports 2-layer codebook based UL MIMO.

This test case applies for NS_03U to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that supports UL full power transmission (ULFPTx).

6.5D.2.4.2.3	Minimum conformance requirements

For UE supporting UL MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters is defined as the sum of the emissions from both UE transmit antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclause 6.5.2.4.2.3 apply. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1.3.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the requirements in clause 6.5.2.4.2.3 shall apply. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.5.2.4.2.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.5D.2 and 6.5.2.4.2.

6.5D.2.4.2.4	Test description

6.5D.2.4.2.4.1	Initial conditions

Same as in subclause 6.2D.3.4.1 with the following exception;

-	Only network signalling values NS_03U, NS_05U, NS_100 with the corresponding band defined in Table 6.2.3.3.1-1 need to perform UTRA ACLR for MIMO testing.

-	Message contents in step 6 are defined in clause 6.5D.2.4.2.4.3.

Table 6.5D.2.4.2.4.1-1: Void

Table 6.5D.2.4.2.4.1-2: Void

6.5D.2.4.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the applicable test configuration table in clause 6.2D.3.4.1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in clause 6.2D.3.5 as appropriate. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of the rectangular filtered mean power for the assigned NR channel at each antenna connector of UE using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the sum of the RRC filtered mean power of the first and the second UTRA adjacent channel at each antenna connector of UE on both lower and upper side of the assigned NR channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.	Calculate the ratio of the power between the values measured in step 4 over step 5 for UTRAACLR1, UTRAACLR2 for both lower an upper side of the assigned NR channel, respectively.

7.	If UE supports ULFPTx, repeat test steps 1~6 with UL RMC according to the applicable test configuration table supporting ULFPTx in clause 6.2D.3.4.1. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.

6.5D.2.4.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO and with the exception specified in Table 6.5D.2.4.2.4.3 - 1

Table 6.5D.2.4.2.4.3-1: AdditionalSpectrumEmission: UTRA ACLR test requirement for "NS_XX"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission from SIB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 | NS_100 for band n1, n2, n3, n5, n25, n66, n80, n84 NOTE1 |  |
|  | 3 | NS_03U for n2, n25, n66 NS_05U for n1, n84 |  |
|  | 4 | NS_05U for n65 |  |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed |  |  |  |

6.5D.2.4.2.5	Test requirement

UTRA ACLR requirement is applicable when signalled by the network with network signalling value indicated by the field additionalSpectrumEmission.

The measured UE mean total power in the channel bandwidth at all the antenna connectors, derived in step 3, shall fulfil requirements in Clause 6.2D.3.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured UTRA ACLR, derived in step 6 for each antenna connector, shall be higher than the limits in Table 6.5D.2.4.2.5-1.

Table 6.5D.2.4.2.5-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB - TT |
| UTRAACLR2 | 36 dB - TT |
| NOTE 1: TT = 0.8 dB |  |

##### 6.5D.2.4.2_1	UTRA ACLR for SUL with UL MIMO

6.5D.2.4.2_1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5D.2.4.2_1.2	Test applicability

This test case applies for NS_05U to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

6.5D.2.4.2_1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

For UE supporting UL MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters is defined as the sum of the emissions from both UE transmit antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclause 6.5.2.4.2.3 apply. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1_1.3.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.5D.2 and 6.5.2.4.2.

6.5D.2.4.2_1.4	Test description

6.5D.2.4.2_1.4.1	Initial conditions

S Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5D.2.4.2_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5D.2.4.2_1.4.1-1: Test Configuration Table for NS_05U

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: Use Fc as specified in test parameters NUL carrier: Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz, 20 MHz NUL carrier: Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carrier |  |  |  |
| AMPR Test parameters for NS_05U |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink config | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 2 | 1925 | 10 |  |  | A1 |  | QPSK | Outer_Full |
| 3 | 1925 | 10 |  |  | A7 |  | QPSK | 42@10 |
| 4 | 1925 | 10 |  |  | A2 |  | QPSK | 6@40 |
| 5 | 1935 | 10 |  |  | A4 |  | QPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  | A7 |  | QPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  | A2 |  | QPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  | A1 |  | QPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  | A2 |  | QPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  | A5 |  | QPSK | Outer_Full |
| 12 | 1930 | 20 |  |  | A1 |  | QPSK | Outer_Full |
| 13 | 1930 | 20 |  |  | A7 |  | QPSK | 78@28 |
| 14 | 1930 | 20 |  |  | A2 |  | QPSK | 6@76 |
| 15 | 1950 | 20 |  |  | A6 |  | QPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  | A3 |  | 16 QAM | Outer_Full |
| 17 | 1925 | 10 |  |  | A1 |  | 16 QAM | Outer_Full |
| 18 | 1925 | 10 |  |  | A7 |  | 16 QAM | 42@10 |
| 19 | 1925 | 10 |  |  | A2 |  | 16 QAM | 6@40 |
| 20 | 1935 | 10 |  |  | A4 |  | 16 QAM | Outer_Full |
| 21 | 1927.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 22 | 1927.5 | 15 |  |  | A7 |  | 16 QAM | 60@19 |
| 23 | 1927.5 | 15 |  |  | A2 |  | 16 QAM | 6@56 |
| 24 | 1932.5 | 15 |  |  | A1 |  | 16 QAM | Outer_Full |
| 25 | 1932.5 | 15 |  |  | A2 |  | 16 QAM | 6@68 |
| 26 | 1942.5 | 15 |  |  | A5 |  | 16 QAM | Outer_Full |
| 27 | 1930 | 20 |  |  | A1 |  | 16 QAM | Outer_Full |
| 28 | 1930 | 20 |  |  | A7 |  | 16 QAM | 78@28 |
| 29 | 1930 | 20 |  |  | A2 |  | 16 QAM | 6@76 |
| 30 | 1950 | 20 |  |  | A6 |  | 16 QAM | Outer_Full |
| 31 | 1922.5 | 5 |  |  | A3 |  | 64 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  | A1 |  | 64 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  | A7 |  | 64 QAM | 42@10 |
| 34 | 1925 | 10 |  |  | A2 |  | 64 QAM | 6@40 |
| 35 | 1935 | 10 |  |  | A4 |  | 64 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 37 | 1927.5 | 15 |  |  | A7 |  | 64 QAM | 60@19 |
| 38 | 1927.5 | 15 |  |  | A2 |  | 64 QAM | 6@56 |
| 39 | 1932.5 | 15 |  |  | A1 |  | 64 QAM | Outer_Full |
| 40 | 1932.5 | 15 |  |  | A2 |  | 64 QAM | 6@68 |
| 41 | 1942.5 | 15 |  |  | A5 |  | 64 QAM | Outer_Full |
| 42 | 1930 | 20 |  |  | A1 |  | 64 QAM | Outer_Full |
| 43 | 1930 | 20 |  |  | A7 |  | 64 QAM | 78@28 |
| 44 | 1930 | 20 |  |  | A2 |  | 64 QAM | 6@76 |
| 45 | 1950 | 20 |  |  | A6 |  | 64 QAM | Outer_Full |
| 46 | 1922.5 | 5 |  |  | A3 |  | 256 QAM | Outer_Full |
| 47 | 1925 | 10 |  |  | A1 |  | 256 QAM | Outer_Full |
| 48 | 1925 | 10 |  |  | A7 |  | 256 QAM | 42@10 |
| 49 | 1935 | 10 |  |  | A4 |  | 256 QAM | Outer_Full |
| 50 | 1927.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 51 | 1927.5 | 15 |  |  | A7 |  | 256 QAM | 60@19 |
| 52 | 1932.5 | 15 |  |  | A1 |  | 256 QAM | Outer_Full |
| 53 | 1942.5 | 15 |  |  | A5 |  | 256 QAM | Outer_Full |
| 54 | 1930 | 20 |  |  | A1 |  | 256 QAM | Outer_Full |
| 55 | 1930 | 20 |  |  | A7 |  | 256 QAM | 78@28 |
| 56 | 1950 | 20 |  |  | A6 |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement channels are set according to Table 6.5D.2.4.2_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.2.4.2_1.4.3

6.5D.2.4.2_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the test configuration tables in clause 6.2D.3_1. 4.1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in clause 6.2D.3_1.5 as appropriate. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of the rectangular filtered mean power for the assigned NR channel at each antenna connector of UE using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the sum of the RRC filtered mean power of the first and the second UTRA adjacent channel at each antenna connector of UE on both lower and upper side of the assigned NR channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.	Calculate the ratio of the power between the values measured in step 4 over step 5 for UTRAACLR1, UTRAACLR2 for both lower an upper side of the assigned NR channel, respectively.

6.5D.2.4.2_1.4.3	Message contents

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Table 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL.

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO and with the exception specified in Table 6.5D.2.4.2_1.4.3-1.

Table 6.5D.2.4.2_1.4.3-1: AdditionalSpectrumEmission: UTRA ACLR test requirement for NS_05U

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission from SIB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 3 | NS_05U for band n84 |  |

6.5D.2.4.2_1.5	Test requirement

UTRA ACLR requirement is applicable when signalled by the network with network signalling value indicated by the field additionalSpectrumEmission.

The measured UE mean total power in the channel bandwidth at all the antenna connectors, derived in step 3, shall fulfil requirements in clause 6.2D.3_1.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured UTRA ACLR, derived in step 6, shall be higher than the limits in Table 6.5D.2.4.2_1.5-1.

Table 6.5D.2.4.2_1.5-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB - TT |
| UTRAACLR2 | 36 dB - TT |
| NOTE 1: TT = 0.8 dB |  |

### 6.5D.2_1	Void

### 6.5D.3	Spurious emissions for UL MIMO

For UE supporting UL MIMO, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products are specified at each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements specified in subclause 6.5.3 apply to each transmit antenna connector. The requirements shall be met with the UL MIMO configurations described in sub-clause 6.2D.1.3.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.5.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5D.3

#### 6.5D.3.1	General spurious emissions for UL MIMO

6.5D.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5D.3.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 that support 2-layer codebook based UL MIMO.

6.5D.3.1.3	Minimum conformance requirements

The general spurious emission requirement specified in clause 6.5.3.1.3 applies to the sum of the emissions from both UE antenna connectors..

The normative reference for this requirement is TS 38.101-1 [2] subclauses 6.5D.3 and 6.5.3.1.

6.5D.3.1.4	Test description

6.5D.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5D.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1. |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | OuterFull |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex [A, Figure A.3.1.2.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.3.1.4.3.

6.5D.3.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.3.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of the transmitted power at each antenna connector with a measurement filter of bandwidths according to table 6.5D.3.1.5-1. The centre frequency of the filter shall be stepped in contiguous steps according to table 6.5D.3.1.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5D.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.5D.3.1.5	Test requirement

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in Table 6.5D.3.1.5-1.

The spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.1.3-1from the edge of the channel bandwidth.

Table 6.5D.3.1.5-1: General spurious emissions test requirements

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than 2.69 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz |  |  |  |

#### 6.5D.3.2	Spurious emissions for UE co-existence for UL MIMO

6.5D.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

6.5D.3.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 that support 2-layer codebook based UL MIMO.

6.5D.3.2.3	Minimum conformance requirements

The requirements for NR bands for coexistence with protected bands specified in subclause 6.5.3.2.3 apply to the sum of the emissions from both UE transmit antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] subclauses 6.5.3.2 and 6.5D.3.

6.5D.3.2.4	Test description

6.5D.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5D.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1. |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Outer_Full |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause 4.4.3..

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5D.3.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.3.2.4.3.

6.5D.3.2.4.2	Test procedure

1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of the transmitted power at each UE antenna connector with a measurement filter of bandwidths according to 6.5.3.2.3. The centre frequency of the filter shall be stepped in contiguous steps according to 6.5.3.2.3. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5D.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.5D.3.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements and are not repeated in this section.

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in  clause 6.5.3.2.3 according to the following rule:

The release of the requirements for the UE depends on vendor declaration, and shall not be less than the UE release.

If there is no vendor declaration, the default release of requirements is same as UE release (if UE support a band from a later release compare to UE release, the release of requirements is the earliest release where requirements for this band are defined).

#### 6.5D.3.3	Additional spurious emissions for UL MIMO

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- There are NS-XXs other than NS_04, NS_21 and NS_47 that are FFS:

6.5D.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5D.3.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 that support 2-layer codebook based UL MIMO.

6.5D.3.3.3	Minimum conformance requirements

The additional spurious emission requirements specified in 6.5.3.3.3 apply to the sum of the emissions from both UE antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.5D.3 and 6.5.3.3

6.5D.3.3.4	Test description

6.5D.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in Table 6.5D.3.3.4.1-1 through Table 6.5D.3.3.4.1-4. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.3.3.4.1-1: Test Configuration Table (network signalling value "NS_04")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| Additional spurious emissions test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation |  | RB allocation (NOTE 1) |
|  |  |  |  |  | (NOTE 2) |  |  |
| 1 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 2 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 3 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 4 | Low |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 5 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 6 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 7 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 9 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 10 | Low |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 11 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 12 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 13 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 14 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 16 | Low |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 17 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 18 | High |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 19 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 20 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 22 | Low |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| 23 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |  |

Table 6.5D.3.3.4.1-2: Test Configuration table for NS_47 power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-19 and 6.2.3.4.1-20 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 30 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest |  |  |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 43 | Default | 30 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 44 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 45 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 46 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 47 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 48 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 49 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 50 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 51 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 52 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |  |
| 53 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 54 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 55 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 56 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 57 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |
| 58 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 59 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |  |
| 60 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |  |
| 61 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 62 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 63 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 64 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |  |
| 65 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 66 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 67 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 68 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 69 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 70 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Void |  |  |  |  |  |  |  |  |  |  |

Table 6.5D.3.3.4.1-3: Test Configuration table for NS_47 power class 3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-19 and 6.2.3.4.1-20 |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 30 MHz |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation |  | RB allocation (Note 1) |
| 1 | Default | 30 | Default |  | CP-OFDM | QPSK | Outer_Full (A2) |
| 2 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |
| 3 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |
| 4 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2.2.4.1-4. NOTE 2: Void. |  |  |  |  |  |  |  |

Table 6.5D.3.3.4.1-4: Test Configuration table for NS_47 power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-19 and 6.2.3.4.1-20 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 30 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest |  |  |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 36 | Default | 30 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 37 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 38 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 39 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 40 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 41 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 42 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 43 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 44 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 45 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |  |
| 46 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 47 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 48 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 49 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 50 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |
| 51 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 52 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |  |
| 53 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |  |
| 54 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 55 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 56 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 57 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |  |
| 58 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 59 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 60 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 61 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 62 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 63 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void |  |  |  |  |  |  |  |  |  |  |

Table 6.5D.3.3.4.1-5: Test Configuration table for NS_21

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_21 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |
|  |  |  |  | N/A for A-MPR test cases |  |  |  |  |
| 26 | Low | Default | Default |  | CP-s OFDM | QPSK | Edge_1RB_Left |  |
| 27 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |  |
| 28 | Default | Default | Default |  |  | QPSK | Outer_Full |  |
| 29 | Default | 10 MHz | Default |  |  | QPSK | 4@0 | 2@0 |
| 30 | Default | 10 MHz | Default |  |  | QPSK | 4@48 | 2@22 |
| 31 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |  |
| 32 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |  |
| 33 | Default | Default | Default |  |  | 16 QAM | Outer_Full |  |
| 34 | Default | 10 MHz | Default |  |  | 16 QAM | 4@0 | 2@0 |
| 35 | Default | 10 MHz | Default |  |  | 16 QAM | 4@48 | 2@22 |
| 36 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |  |
| 37 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |  |
| 38 | Default | Default | Default |  |  | 64 QAM | Outer_Full |  |
| 39 | Default | 10 MHz | Default |  |  | 64 QAM | 4@0 | 2@0 |
| 40 | Default | 10 MHz | Default |  |  | 64 QAM | 4@48 | 2@22 |
| 41 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |  |
| 42 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |  |
| 43 | Default | Default | Default |  |  | 256 QAM | Outer_Full |  |
| 44 | Default | 10 MHz | Default |  |  | 256 QAM | 4@0 | 2@0 |
| 45 | Default | 10 MHz | Default |  |  | 256 QAM | 4@48 | 2@22 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A Figure A.3.1.2.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5D.3.3.4.1-1 for NS_04 and to Table 6.5D.3.3.4.1-2 to 6.5D.3.3.4.1-4 for NS_47.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.3.3.4.3.

6.5D.3.3.4.2	Test procedure

1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.3.3.4.1-1 as appropriate for NS_04. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of the mean power at each UE antenna connector in the channel bandwidth of the radio access mode, which shall meet the requirements described in Clauses from 6.2D.2.5, or 6.2D.3.5 as appropriate for NS_04 and NS_47. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of the transmitted power  at each UE antenna connector with a measurement filter of bandwidths according to Tables 6.5.3.3.5.1-1 as appropriate for NS_04, Table 6.5.3.3.3.151 as appropriate for NS_47, and Table 6.5.3.3.3.12-1 as appropriate for NS_21. The centre frequency of the filter shall be stepped in contiguous steps according to the same table the measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5D.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO and same exceptions listed in clause 6.5.3.3.4.3

6.5D.3.3.5	Test requirement

The measured power derived in step 4 shall meet the requirements for the specified NR band for an additional spurious emission requirement with protected bands as indicated in clause 6.5.3.3.5 for different NS values.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

### 6.5D.3_1	Spurious emissions for UL MIMO (Rel-16 onward)

#### 6.5D.3_1.1	General spurious emissions for UL MIMO (Rel-16 onward)

6.5D.3_1.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5D.3_1.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.5D.3_1.1.3	Minimum conformance requirements

For UE supporting UL MIMO, the requirements for spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products is defined as the sum of the emissions from both UE transmit antenna connectors.

The general spurious emission requirements specified in clause 6.5.3.1.3 apply. For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements shall be met with the UL MIMO configurations described in clause 6.2D.1. For UEs supporting ULFPTx for UL MIMO, the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.5.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] subclauses 6.5D.3 and 6.5.3.1.

6.5D.3_1.1.4	Test description

6.5D.3_1.1.4.1	Initial conditions

Same initial conditions as in clause 6.5D.3.1.4.1.

6.5D.3_1.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.3.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of transmitted power at each antenna connector with a measurement filter of bandwidths according to table 6.5D.3_1.1.5-1. The centre frequency of the filter shall be stepped in contiguous steps according to table 6.5D.3_1.1.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

4.	If UE supports ULFPTx Mode-2 or Mode-full power, repeat test steps 1~3 with the exception that the PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.

6.5D.3_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.5D.3_1.1.5	Test requirement

The measured average power of spurious emission, derived in step 3 or step 4, shall not exceed the described value in Table 6.5D.3_1.1.5-1.

The spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.1.3-1from the edge of the channel bandwidth.

Table 6.5D.3_1.1.5-1: General spurious emissions test requirements

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than 2.69 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz |  |  |  |

#### 6.5D.3_1.2	Spurious emission for UE co-existence for UL MIMO (Rel-16 onward)

6.5D.3_1.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

6.5D.3_1.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.5D.3_1.2.3	Minimum conformance requirements

For UE supporting UL MIMO, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products is defined as the sum of the emissions from both UE transmit antenna connectors.

The spurious emission for UE co-existence requirements specified in clause 6.5.3.2.3 apply. For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements shall be met with the UL MIMO configurations described in clause 6.2D.1. For UEs supporting ULFPTx for UL MIMO, the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

The normative reference for this requirement is TS 38.101-1 [2] subclauses 6.5.3.2 and 6.5D.3.

6.5D.3_1.2.4	Test description

6.5D.3_1.2.4.1	Initial conditions

Same initial conditions as in clause 6.5D.3.2.4.1.

6.5D.3_1.2.4.2	Test procedure

1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of transmitted power at each UE antenna connector with a measurement filter of bandwidths according to table 6.5.3.2.3-1. The centre frequency of the filter shall be stepped in contiguous steps according to table 6.5.3.2.3-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

4.	If UE supports ULFPTx Mode-2 or Mode-full power, repeat test steps 1~3 with the exception that the PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.

6.5D.3_1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.5D.3_1.2.5	Test requirement

The measured average power of spurious emission, derived in step 3 or step 4 from both UE antenna connectors, shall not exceed the described value in Table 6.5.3.2.3-1.

#### 6.5D.3_1.3	Additional spurious emissions for UL MIMO (Rel-16 onward)

6.5D.3_1.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5D.3_1.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.5D.3_1.3.3 Minimum conformance requirements

For UE supporting UL MIMO, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products is defined as the sum of the emissions from both UE transmit antenna connectors.

The additional spurious emission requirements specified in clause 6.5.3.3.3 apply. For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements shall be met with the UL MIMO configurations described in clause 6.2D.1. For UEs supporting ULFPTx for UL MIMO, the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.5D.3 and 6.5.3.3

6.5D.3_1.3.4	Test description

6.5D.3_1.3.4.1	Initial conditions

Same initial conditions as in clause 6.5D.3.3.4.1 together with tables 6.5D.3_1.3.4-1 to 6.5D.3_1.3.4-3.

Table 6.5D.3_1.3.4.1-1: Test Configuration table for NS_05

Same test configuration as listed in Table 6.2.3.4.1-4 shall be used with the following exceptions:

-	Test SCS shall be: [Lowest].

-	Only Test IDs 66 to 115 shall be tested.

Table 6.5D.3_1.3.4.1-2: Test Configuration table for NS_48

Same test configuration as listed in Table 6.2.3.4.1-19 shall be used with the following exceptions:

-	Test Channel Bandwidths shall be: 25, 30, 40, 45 and 50 MHz

-	Test SCS shall be: Lowest.

-	Only Test IDs 41 to 80 shall be tested.

Table 6.5D.3_1.3.4.1-3: Test Configuration table for NS_49

Same test configuration as listed in Table 6.2.3.4.1-29 shall be used with the following exceptions:

-	Test Channel Bandwidths shall be: 25, 30, 40, 45 and 50 MHz

-	Test SCS shall be: Lowest.

-	Only Test IDs 53 to 104 shall be tested.

Table 6.5D.3_1.3.4.1-4: Test Configuration table for NS_46

Same test configuration as listed in Table 6.2.3.4.1-25 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

-	Only Test IDs 11 to 22 shall be tested.

Table 6.5D.3_1.3.4.1-5: Test Configuration table for NS_21

Same test configuration as listed in Table 6.2.3.4.1-27 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

-	Only Test IDs 26 to 45 shall be tested.

Table 6.5D.3_1.3.4.1-6: Test Configuration table for NS_44

Same test configuration as listed in Table 6.2.3.4.1-26 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

-	Only Test IDs 23 to 44 shall be tested.

Table 6.5D.3_1.3.4.1-7: Test Configuration table for NS_27

Same test configuration as listed in Table 6.2.3.4.1-13 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

-	Only Test IDs 141 to 252 shall be tested.

Table 6.5D.3_1.3.4.1-8: Test Configuration table for NS_50

Same test configuration as listed in Table 6.2.3.4.1-33 shall be used with the following exceptions:

-	Test SCS shall be: Lowest.

-	Only Test IDs 33 to 64 shall be tested.

6.5D.3_1.3.4.2	Test procedure

1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to tables in clause 6.5D.3_1.3.4.1 as appropriate. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of the mean power at each UE antenna connector in the channel bandwidth of the radio access mode according to the test configurations in clause 6.5D.3_1.3.4.1, which shall meet the requirements described in clauses 6.2D.3.5 as appropriate for each network signalling. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of transmitted power at each UE antenna connector with a measurement filter of bandwidths according to clauses 6.5.3.3.3.1 to 6.5.3.3.3.25 as appropriate. The centre frequency of the filter shall be stepped in contiguous steps according to the same table the measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

5.	If UE supports ULFPTx Mode-2 or Mode-full power, repeat test steps 1~4 with the exception that the PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.

6.5D.3_1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO and same exceptions listed in clause 6.5.3.3.4.3.

6.5D.3_1.3.5	Test requirement

The measured power from both UE antenna connector derived in step 4 or step 5 shall meet the requirements for the specified NR band for an additional spectrum emission requirement with protected bands as indicated in clause 6.5.3.3.5.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

### 6.5D.3_2	Spurious emissions for SUL with UL MIMO

#### 6.5D.3_2.1	General spurious emissions for SUL with UL MIMO

6.5D.3_2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5D.3_2.1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

6.5D.3_2.1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

For UE supporting UL MIMO, the requirements for spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products is defined as the sum of the emissions from both UE transmit antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] subclauses 6.5D.3 and 6.5.3.1.

6.5D.3_2.1.4	Test description

6.5D.3_2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5D.3_2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.3_2.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | CP-OFDM QPSK | OuterFull |
| 2 |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: Void |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex [A, Figure A.3.1.2.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement channels are set according to Table 6.5D.3_2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.3_2.1.4.3.

6.5D.3_2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.3_2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of transmitted power at each antenna connector with a measurement filter of bandwidths according to Table 6.5D.3_2.1.5-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5D.3_2.1.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5D.3_2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Tables 4.6.1-28 with condition SUL AND (RF OR RRM), 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL.

6.5D.3_2.1.5	Test requirement

The measured average power of spurious emission on the SUL carrier shall not exceed the described value in Table 6.5D.3_2.1.5-1.

The spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5D.3_2.1.5-1: General spurious emissions test requirements

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than 2.69 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz |  |  |  |

#### 6.5D.3_2.2	Spurious emissions for UE co-existence for SUL with UL MIMO

6.5D.3_2.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

6.5D.3_2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5D.3_2.2.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

For UE supporting UL MIMO, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products is defined as the sum of the emissions from both UE transmit antenna connectors.

The spurious emission for UE co-existence requirements specified in clause 6.5.3.2.3 apply. For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements shall be met with the UL MIMO configurations described in clause 6.2D.1_1.

The normative reference for this requirement is TS 38.101-1 [2] subclauses 6.5D.3 and 6.5.3.2.

6.5D.3_2.2.4	Test description

6.5D.3_2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5D.3_2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.3_2.2.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 1) |
| 1 |  |  |  | CP-OFDM QPSK | OuterFull |
| 2 |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement channels are set according to Table 6.5D.3_2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.3_2.2.4.3.

6.5D.3_2.2.4.2	Test procedure

1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.3_2.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of transmitted power at each UE antenna connector with a measurement filter of bandwidths according to Table 6.5.3.2.3-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5.3.2.3-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5D.3_2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Tables 4.6.1-28 with condition SUL AND (RF OR RRM), 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL.

6.5D.3_2.2.5	Test requirement

The measured average power of spurious emission, derived in step 3 from both UE antenna connectors, shall not exceed the described value in Table 6.5.3.2.3-1.

#### 6.5D.3_2.3	Additional spurious emissions for SUL with UL MIMO

6.5D.3_2.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5D.3_2.3.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

6.5D.3_2.3.3	Minimum conformance requirements

For UE supporting UL MIMO, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products is defined as the sum of the emissions from both UE transmit antenna connectors.

The additional spurious emission requirements specified in clause 6.5.3.3.3 apply to the UE operating on SUL bands. For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements shall be met with the UL MIMO configurations described in clause 6.2D.1_1.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.5D.3 and 6.5.3.3

6.5D.3_2.3.4	Test description

6.5D.3_2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in Table 6.5D.3_2.3.4.1-1 through Table 6.5D.3_2.3.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.3_2.3.4-1: Test Configuration Table (network signalling value “NS_05”)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Use centre frequency (Fc) as specified in test parameters for SUL carrier Mid range for Non-SUL carrier |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz for SUL carrier Lowest for Non-SUL carrier |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |  |  |
| Additional Spurious for SUL test parameters for NS_05 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | SUL ChBw (MHz) | Downlink Config. | Uplink Config |  | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  |  | Modulation |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 |  |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 2 | 1925 | 10 |  |  |  | A1 |  | QPSK | Outer_Full |
| 3 | 1925 | 10 |  |  |  | A7 |  | QPSK | 42@10 |
| 4 | 1925 | 10 |  |  |  | A2 |  | QPSK | 6@40 |
| 5 | 1935 | 10 |  |  |  | A4 |  | QPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  |  | A7 |  | QPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  |  | A2 |  | QPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  |  | A2 |  | QPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  |  | A5 |  | QPSK | Outer_Full |
| 12 | 1930 | 20 |  |  |  | A1 |  | QPSK | Outer_Full |
| 13 | 1930 | 20 |  |  |  | A7 |  | QPSK | 78@28 |
| 14 | 1930 | 20 |  |  |  | A2 |  | QPSK | 6@76 |
| 15 | 1950 | 20 |  |  |  | A6 |  | QPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  |  | A3 |  | 16 QAM | Outer_Full |
| 17 | 1925 | 10 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 18 | 1925 | 10 |  |  |  | A7 |  | 16 QAM | 42@10 |
| 19 | 1925 | 10 |  |  |  | A2 |  | 16 QAM | 6@40 |
| 20 | 1935 | 10 |  |  |  | A4 |  | 16 QAM | Outer_Full |
| 21 | 1927.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 22 | 1927.5 | 15 |  |  |  | A7 |  | 16 QAM | 60@19 |
| 23 | 1927.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@56 |
| 24 | 1932.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 25 | 1932.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@68 |
| 26 | 1942.5 | 15 |  |  |  | A5 |  | 16 QAM | Outer_Full |
| 27 | 1930 | 20 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 28 | 1930 | 20 |  |  |  | A7 |  | 16 QAM | 78@28 |
| 29 | 1930 | 20 |  |  |  | A2 |  | 16 QAM | 6@76 |
| 30 | 1950 | 20 |  |  |  | A6 |  | 16 QAM | Outer_Full |
| 31 | 1922.5 | 5 |  |  |  | A3 |  | 64 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  |  | A7 |  | 64 QAM | 42@10 |
| 34 | 1925 | 10 |  |  |  | A2 |  | 64 QAM | 6@40 |
| 35 | 1927.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  |  | A7 |  | 64 QAM | 60@19 |
| 37 | 1927.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@56 |
| 38 | 1932.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 39 | 1932.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@68 |
| 40 | 1930 | 20 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 41 | 1930 | 20 |  |  |  | A7 |  | 64 QAM | 78@28 |
| 42 | 1930 | 20 |  |  |  | A2 |  | 64 QAM | 6@76 |
| 43 | 1922.5 | 5 |  |  |  | A3 |  | 256 QAM | Outer_Full |
| 44 | 1925 | 10 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 45 | 1925 | 10 |  |  |  | A7 |  | 256 QAM | 42@10 |
| 46 | 1927.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 47 | 1927.5 | 15 |  |  |  | A7 |  | 256 QAM | 60@19 |
| 48 | 1932.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 49 | 1930 | 20 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 50 | 1930 | 20 |  |  |  | A7 |  | 256 QAM | 78@28 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.5D.3_2.3.4-2: Test Configuration Table (network signalling value “NS_05U”)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Use centre frequency (Fc) as specified in test parameters for SUL carrier Mid range for Non-SUL carrier |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz for SUL carrier Lowest for Non-SUL carrier |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |  |  |
| Additional Spurious for SUL test parameters for NS_05 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | SUL ChBw (MHz) | Downlink Config. | Uplink Config |  | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  |  | Modulation |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 |  |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 2 | 1925 | 10 |  |  |  | A1 |  | QPSK | Outer_Full |
| 3 | 1925 | 10 |  |  |  | A7 |  | QPSK | 42@10 |
| 4 | 1925 | 10 |  |  |  | A2 |  | QPSK | 6@40 |
| 5 | 1935 | 10 |  |  |  | A4 |  | QPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  |  | A7 |  | QPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  |  | A2 |  | QPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  |  | A2 |  | QPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  |  | A5 |  | QPSK | Outer_Full |
| 12 | 1930 | 20 |  |  |  | A1 |  | QPSK | Outer_Full |
| 13 | 1930 | 20 |  |  |  | A7 |  | QPSK | 78@28 |
| 14 | 1930 | 20 |  |  |  | A2 |  | QPSK | 6@76 |
| 15 | 1950 | 20 |  |  |  | A6 |  | QPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  |  | A3 |  | 16 QAM | Outer_Full |
| 17 | 1925 | 10 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 18 | 1925 | 10 |  |  |  | A7 |  | 16 QAM | 42@10 |
| 19 | 1925 | 10 |  |  |  | A2 |  | 16 QAM | 6@40 |
| 20 | 1935 | 10 |  |  |  | A4 |  | 16 QAM | Outer_Full |
| 21 | 1927.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 22 | 1927.5 | 15 |  |  |  | A7 |  | 16 QAM | 60@19 |
| 23 | 1927.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@56 |
| 24 | 1932.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 25 | 1932.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@68 |
| 26 | 1942.5 | 15 |  |  |  | A5 |  | 16 QAM | Outer_Full |
| 27 | 1930 | 20 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 28 | 1930 | 20 |  |  |  | A7 |  | 16 QAM | 78@28 |
| 29 | 1930 | 20 |  |  |  | A2 |  | 16 QAM | 6@76 |
| 30 | 1950 | 20 |  |  |  | A6 |  | 16 QAM | Outer_Full |
| 31 | 1922.5 | 5 |  |  |  | A3 |  | 64 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  |  | A7 |  | 64 QAM | 42@10 |
| 34 | 1925 | 10 |  |  |  | A2 |  | 64 QAM | 6@40 |
| 35 | 1935 | 10 |  |  |  | A4 |  | 64 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 37 | 1927.5 | 15 |  |  |  | A7 |  | 64 QAM | 60@19 |
| 38 | 1927.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@56 |
| 39 | 1932.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 40 | 1932.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@68 |
| 41 | 1942.5 | 15 |  |  |  | A5 |  | 64 QAM | Outer_Full |
| 42 | 1930 | 20 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 43 | 1930 | 20 |  |  |  | A7 |  | 64 QAM | 78@28 |
| 44 | 1930 | 20 |  |  |  | A2 |  | 64 QAM | 6@76 |
| 45 | 1950 | 20 |  |  |  | A6 |  | 64 QAM | Outer_Full |
| 46 | 1922.5 | 5 |  |  |  | A3 |  | 256 QAM | Outer_Full |
| 47 | 1925 | 10 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 48 | 1925 | 10 |  |  |  | A7 |  | 256 QAM | 42@10 |
| 49 | 1935 | 10 |  |  |  | A4 |  | 256 QAM | Outer_Full |
| 50 | 1927.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 51 | 1927.5 | 15 |  |  |  | A7 |  | 256 QAM | 60@19 |
| 52 | 1932.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 53 | 1942.5 | 15 |  |  |  | A5 |  | 256 QAM | Outer_Full |
| 54 | 1930 | 20 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 55 | 1930 | 20 |  |  |  | A7 |  | 256 QAM | 78@28 |
| 56 | 1950 | 20 |  |  |  | A6 |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

Table 6.5D.3_2.3.4-3: Test Configuration Table (network signalled value “NS_56”)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters for SUL carrier Lowest for non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | 15 kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Configuration | Uplink Configuration | A-MPR | Uplink Configuration |  |  |
|  |  |  |  |  |  | Modulation |  | RB allocation (Note 1) |
|  |  |  |  |  |  |  |  | SCS 15 kHz |
| 66 | Low | 5 | N/A for A-MPR testing | N/A | 14 | CP-OFDM | QPSK | Edge_1RB_Left |
| 67 | Low | 5 |  |  | 6 |  | QPSK | Outer_Full |
| 68 | Low | 5 |  |  | 4 |  | QPSK | 21@4 |
| 69 | Low | 5 |  |  | 4 |  | QPSK | 1@4 |
| 70 | Low | 5 |  |  | 14 |  | 16 QAM | Edge_1RB_Left |
| 71 | Low | 5 |  |  | 6 |  | 16 QAM | Outer_Full |
| 72 | Low | 5 |  |  | 4 |  | 16 QAM | 21@4 |
| 73 | Low | 5 |  |  | 4 |  | 16 QAM | 1@4 |
| 74 | Low | 5 |  |  | 14 |  | 64 QAM | Edge_1RB_Left |
| 75 | Low | 5 |  |  | 6 |  | 64 QAM | Outer_Full |
| 76 | Low | 5 |  |  | 4 |  | 64 QAM | 21@4 |
| 77 | Low | 5 |  |  | 4 |  | 64 QAM | 1@4 |
| 78 | Low | 5 |  |  | 14 |  | 256 QAM | Edge_1RB_Left |
| 79 | Low | 5 |  |  | 6 |  | 256 QAM | Outer_Full |
| 80 | Low | 5 |  |  | 4 |  | 256 QAM | 21@4 |
| 81 | Low | 5 |  |  | 4 |  | 256 QAM | 1@4 |
| 82 | Low | 10 |  |  | 12 |  | QPSK | Edge_1RB_Left |
| 83 | Low | 10 |  |  | 8 |  | QPSK | Outer_Full |
| 84 | Low | 10 |  |  | 6 |  | QPSK | 1@3 |
| 85 | Low | 10 |  |  | 6 |  | QPSK | 43@9 |
| 86 | Low | 10 |  |  | 4 |  | QPSK | 1@35 |
| 87 | Low | 10 |  |  | 2 |  | QPSK | 17@35 |
| 88 | Low | 10 |  |  | 5 |  | QPSK | Edge_1RB_Right |
| 89 | Low | 10 |  |  | 5 |  | QPSK | 1@40 |
| 90 | Low | 10 |  |  | 3 |  | QPSK | 8@44 |
| 91 | Low | 10 |  |  | 12 |  | 16 QAM | Edge_1RB_Left |
| 92 | Low | 10 |  |  | 8 |  | 16 QAM | Outer_Full |
| 93 | Low | 10 |  |  | 6 |  | 16 QAM | 1@3 |
| 94 | Low | 10 |  |  | 6 |  | 16 QAM | 43@9 |
| 95 | Low | 10 |  |  | 4 |  | 16 QAM | 1@35 |
| 96 | Low | 10 |  |  | 2 |  | 16 QAM | 17@35 |
| 97 | Low | 10 |  |  | 5 |  | 16 QAM | Edge_1RB_Right |
| 98 | Low | 10 |  |  | 5 |  | 16 QAM | 1@40 |
| 99 | Low | 10 |  |  | 3 |  | 16 QAM | 8@44 |
| 100 | Low | 10 |  |  | 12 |  | 64 QAM | Edge_1RB_Left |
| 101 | Low | 10 |  |  | 8 |  | 64 QAM | Outer_Full |
| 102 | Low | 10 |  |  | 6 |  | 64 QAM | 1@3 |
| 103 | Low | 10 |  |  | 6 |  | 64 QAM | 43@9 |
| 104 | Low | 10 |  |  | 4 |  | 64 QAM | 1@35 |
| 105 | Low | 10 |  |  | 2 |  | 64 QAM | 17@35 |
| 106 | Low | 10 |  |  | 5 |  | 64 QAM | Edge_1RB_Right |
| 107 | Low | 10 |  |  | 5 |  | 64 QAM | 1@40 |
| 108 | Low | 10 |  |  | 3 |  | 64 QAM | 8@44 |
| 109 | Low | 10 |  |  | 12 |  | 256 QAM | Edge_1RB_Left |
| 110 | Low | 10 |  |  | 8 |  | 256 QAM | Outer_Full |
| 111 | Low | 10 |  |  | 6 |  | 256 QAM | 1@3 |
| 112 | Low | 10 |  |  | 6 |  | 256 QAM | 43@9 |
| 113 | Low | 10 |  |  | 4 |  | 256 QAM | 1@35 |
| 114 | Low | 10 |  |  | 2 |  | 256 QAM | 17@35 |
| 115 | Low | 10 |  |  | 5 |  | 256 QAM | Edge_1RB_Right |
| 116 | Low | 10 |  |  | 5 |  | 256 QAM | 1@40 |
| 117 | Low | 10 |  |  | 3 |  | 256 QAM | 8@44 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A Figure A.3.1.2.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement channels are set according to Table 6.5D.3_2.3.4.1-1 to Table 6.5D.3_2.3.4.1-3 as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.3_2.3.4.3.

6.5D.3_2.3.4.2	Test procedure

1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.3_2.3.4.1-1 to Table 6.5D.3_2.3.4.1-3 as appropriate. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of the mean power at each UE antenna connector in the channel bandwidth of the radio access mode, which shall meet the requirements described in clauses from 6.2D.2.5, or 6.2D.3.5 as appropriate. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of transmitted power at each UE antenna connector with a measurement filter of bandwidths according to clauses 6.5.3.3.3.1 to 6.5.3.3.3.25 as appropriate. The centre frequency of the filter shall be stepped in contiguous steps according to the same table the measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5D.3_2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO and same exceptions listed in clause 6.5.3.3.4.3

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, together with exceptions as specified in clause 6.5.3.3.4.3 as appropriate for different NS values.

6.5D.3_2.3.5	Test requirement

For SUL operation, the measured power at each UE antenna connector shall meet the requirements for the specified NR band for an additional spurious emission requirement with protected bands as indicated in clause 6.5.3.3.5 for different NS values.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

### 6.5D.4	Transmit intermodulation for UL MIMO

6.5D.4.1	Test purpose

To verify that the UE transmit intermodulation does not exceed the described value in the test requirement.

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

6.5D.4.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

6.5D.4.3	Minimum conformance requirements

For UE supporting UL MIMO, the transmit intermodulation requirements are specified at each transmit antenna connector and the wanted signal is defined as the sum of output power at each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements specified in subclause 6.5.4 apply to each transmit antenna connector. The requirements shall be met with the UL MIMO configurations described in sub-clause 6.2D.1.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.5.4 apply.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.5D.4 and 6.5.4.

6.5D.4.4	Test description

6.5D.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in table 6.5D.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for transmit intermodulation test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.3.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5D.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.4.4.3.

6.5D.4.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously uplink power control "up" commands to the UE until the UE transmits at its PUMAX level.

3.	Measure the rectangular filtered mean power at each antenna connector of the UE. For TDD, only slots consisting of only UL symbols are under test.

4.	Set the interference signal frequency below the UL carrier frequency using the first offset in table 6.5D.4.5-1.

5.	Set the interference CW signal level according to table 6.5D.4.5-1.

6.	Search the intermodulation product signals below and above the UL carrier frequency at each UE antenna connector, then measure the rectangular filtered mean power of transmitting intermodulation for both signals, and calculate the ratios for each UE antenna connector with the power measured in step 3.

7.	Set the interference signal frequency above the UL carrier frequency using the first offset in table 6.5D.4.5-1.

8.	Search the intermodulation product signals below and above the UL carrier frequency at each UE antenna connector, then measure the rectangular filtered mean power of transmitting intermodulation for both signals, and calculate the ratios for each UE antenna with the power measured in step 3.

9.	Repeat the measurement using the second offset in table 6.5D.4.5-1.

10.	Repeat step 3) until 9) for each of transmit antenna of the UE.

6.5D.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.5D.4.5	Test requirement

The ratio derived in step 6 and 8, shall not exceed the described value in table 6.5D.4.5-1.

Table 6.5D.4.5-1: Transmit Intermodulation

| Wanted signal channel bandwidth | BWChannel |  |
| --- | --- | --- |
| Interference signal frequency offset from channel centre | BWChannel | 2*BWChannel |
| Interference CW signal level | -40dBc |  |
| Intermodulation product | < -29dBc | < -35dBc |
| Measurement bandwidth | The maximum transmission bandwidth configuration among the different SCSs for the channel BW as defined in Table 6.5.2.4.1.5-1 |  |
| Measurement offset from channel centre | BWChannel and 2*BWChannel | 2*BWChannel and 4*BWChannel |

### 6.5D.4_1	Transmit intermodulation for SUL with UL MIMO

6.5D.4_1.1	Test purpose

To verify that the UE transmit intermodulation does not exceed the described value in the test requirement.

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

6.5D.4_1.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

6.5D.4_1.3	Minimum conformance requirements

For UE supporting UL MIMO, the transmit intermodulation requirements are specified at each transmit antenna connector and the wanted signal is defined as the sum of output power at each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements specified in subclause 6.5.4 apply to each transmit antenna connector. The requirements shall be met with the UL MIMO configurations described in sub-clause 6.2D.1.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.5.4 apply.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.5D.4 and 6.5.4.

6.5D.4_1.4	Test description

6.5D.4_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing and are shown in table 6.5D.4_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.4_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for transmit intermodulation test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.5 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5D.4_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.4_1.4.3.

6.5D.4_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5D.4.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously uplink power control "up" commands to the UE until the UE transmits at its PUMAX level.

3.	Measure the rectangular filtered mean power at each antenna connector of the UE. For TDD, only slots consisting of only UL symbols are under test.

4.	Set the interference signal frequency below the UL carrier frequency using the first offset in table 6.5D.4_1.5-1.

5.	Set the interference CW signal level according to table 6.5D.4_1.5-1.

6.	Search the intermodulation product signals below and above the UL carrier frequency at each UE antenna connector, then measure the rectangular filtered mean power of transmitting intermodulation for both signals and calculate the ratios for each UE antenna connector with the power measured in step 3.

7.	Set the interference signal frequency above the UL carrier frequency using the first offset in table 6.5D.4_1.5-1.

8.	Search the intermodulation product signals below and above the UL carrier frequency at each UE antenna connector, then measure the rectangular filtered mean power of transmitting intermodulation for both signals and calculate the ratios for each UE antenna with the power measured in step 3.

9.	Repeat the measurement using the second offset in table 6.5D.4_1.5-1.

10.	Repeat step 3) until 9) for each of transmit antenna of the UE.

6.5D.4_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, and Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.5D.4_1.5	Test requirement

The ratio derived in step 6 and 8, shall not exceed the described value in table 6.5D.4_1.5-1.

Table 6.5D.4_1.5-1: Transmit Intermodulation

| Wanted signal channel bandwidth | BWChannel |  |
| --- | --- | --- |
| Interference signal frequency offset from channel centre | BWChannel | 2*BWChannel |
| Interference CW signal level | -40dBc |  |
| Intermodulation product | < -29dBc | < -35dBc |
| Measurement bandwidth | The maximum transmission bandwidth configuration among the different SCSs for the channel BW as defined in Table 6.5.2.4.1.5-1 |  |
| Measurement offset from channel centre | BWChannel and 2*BWChannel | 2*BWChannel and 4*BWChannel |

## 6.5G	Output RF spectrum emissions for Tx Diversity

### 6.5G.1	Occupied bandwidth for Tx Diversity

6.5G.1.1	Test purpose

To verify that the UE occupied bandwidth for Tx Diversity for all transmission bandwidth configurations supported by the UE are less than their specific limits.

6.5G.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for occupied bandwidth apply to the transmitted spectrum as measured as the sum of the power from all UE transmit antenna connectors. The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.1.

6.5G.1.4	Test description

Same test description as specified in clause 6.5.1.4 with following exceptions:

Step 3 of Test procedure as in 6.5.1.4.2 is replaced by:

3.	Measure the power spectrum distribution as the sum of the power from all UE transmit antenna connectors within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency. The characteristics of the filter shall be approximately Gaussian (typical spectrum analyser filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is at least 1ms over consecutive active uplink slots.

6.5G.1.5	Test requirement

The measured Occupied Bandwidth shall not exceed values in Table 6.5.1.5-1.

### 6.5G.2	Out of band emission for Tx Diversity

#### 6.5G.2.1	Spectrum emission mask for Tx Diversity

6.5G.2.1.1	Test purpose

Same test purpose as in 6.5.2.2.1.

6.5G.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.2.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters apply to the sum of the emissions from all UE transmit antenna connectors.

If UE indicates Tx diversity capability, Adjacent Channel Leakage power Ratio (ACLR) is defined as the ratio of sum of the filtered mean power at each antenna connector centred on the assigned channel frequency to sum of the filtered mean power at each antenna connector centred on an adjacent channel frequency.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.2.1

6.5G.2.1.4	Test description

6.5G.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5G.2.1.4.1-1, 6.5G.2.1.4.1-2, 6.5G.2.1.4.1-2a and Table 6.5G.2.1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5G.2.1.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Spectrum Emission Mask test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |
| 23 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |
| 33 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 44 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |
| 54 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |
| 64 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM QPSK | Edger_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 9 | Default |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 13 | Low |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 16 | Low |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  |  |  | DFT-s-OFDM 256 QAM | Outer_Full |
| 19 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 20 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 21 | Default |  |  |  | CP-OFDM QPSK | Outer_Full |
| 22 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 23 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 24 | Default |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 25 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 26 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 27 | Default |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| 28 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 29 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 30 | Default |  |  |  | CP-OFDM 256 QAM | Outer_Full |
| 315,6 | Low |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 325,6 | High |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 335,6 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: For Power Class 3 testing, UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n77, n78 and n79. NOTE 6: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. NOTE 7: It is essential that all test points in this table also exist in table 6.2G.2.4.1-2a. |  |  |  |  |  |  |

Table 6.5G.2.1.4.1-2: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 18 | Default |  | CP-OFDM QPSK | Outer Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 21 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: It is essential that all test points in this table also exist in table 6.2G.2.4.1-1. |  |  |  |  |

Table 6.5G.2.1.4.1-3: Test Configuration Table for power class 1.5 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Spectrum Emission Mask test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 18 | Default |  | CP-OFDM QPSK | Outer Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 21 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 28 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 29 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 30 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. NOTE 4: It is essential that all test points in this table also exist in table 6.2G.2.4.1-2. |  |  |  |  |

Table 6.5G.2.1.4.1-4: Test Configuration Table for power class 2&3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2G.2.4.1-4. NOTE 2: It is essential that all test points in this table also exist in table 6.2G.2.4.1-3. NOTE 3: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5G.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5G.2.1.4.3.

6.5G.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5G.2.1.4.1-1, Table 6.5G.2.1.4.1-2, Table 6.5G.2.1.4.1-3 and Table 6.5G.2.1.4.1-4. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE from both transmit antenna connectors in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in clause 6.2G.2.5. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of the mean power of the transmitted signal from both transmit antenna connectors with a measurement filter of bandwidths according to table 6.5G.2.1.5-1 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5G.2.1.4.1-1, table 6.5G.2.1.4.1-2, and table 6.5G.2.1.4.1-2a, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5G.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.5G.2.1.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.5G.2.1.4.3-2: DMRS-UplinkConfig (Test ID 31 – 33 in Table 6.5G.2.1.4.1-1)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.5G.2.1.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in clause 6.2G.2.5 as appropriate, and the power of any UE emission shall fulfil requirements in Table 6.5G.2.1.5-1.

Table 6.5G.2.1.5-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 + TT | -13 + TT |  | 1 % of channel BW |
| ± 0-1 |  |  | -24 + TT | 30 kHz |
| ± 1-5 | -10 + TT | -10 + TT |  | 1 MHz |
| ± 5-6 | -13 + TT |  |  |  |
| ± 6-10 | -25 + TT |  |  |  |
| ± 5-BWChannel |  | -13 + TT |  |  |
| ± BWChannel-(BWChannel+5) |  | -25 + TT |  |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5G.2.1.5-2. |  |  |  |  |

Table 6.5G.2.1.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5G.2.2	Additional spectrum emission mask for Tx Diversity

Editor’s Note: The following aspects are either missing or not yet determined:

- Tests for network signalling values other than NS_04 are incomplete due to lack of test requirement in 6.2G.3.

6.5G.2.2.1	Test purpose

The same test purpose as in 6.5.2.3.1.

6.5G.2.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.2.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters apply to the sum of the emissions from all UE transmit antenna connectors.

If UE indicates IE Tx diversity capability, Adjacent Channel Leakage power Ratio (ACLR) is defined as the ratio of sum of the filtered mean power at each antenna connector centred on the assigned channel frequency to sum of the filtered mean power at each antenna connector centred on an adjacent channel frequency.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.2.

6.5G.2.2.4	Test description

Same test description as specified in clause 6.5.2.3.4 with following exceptions:

Step 3 and 4 of Test procedure as in 6.5.2.3.4.2 is replaced by:

3.	Measure the mean power of the UE as the sum of the power from all UE transmit antenna connectors in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in applicable table of clause 6.2G.3.5. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the power of the transmitted signal as the sum of the power from all UE transmit antenna connectors with a measurement filter of bandwidths according to applicable test configuration tables in subclause 6.5G.2.2.5 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the applicable test requirement table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.5G.2.2.5	Test requirement

Table 6.5G.2.2.5-1: Test Tolerance (Additional spectrum emission mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

6.5G.2.2.5.1	Test requirements (network signalling value "NS_35")

When "NS_35" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in table FFS as appropriate for a NR UE.

and

-	the power of any UE emission shall fulfil requirements in table 6.5G.2.2.5.1-1, as applicable.

Table 6.5G.2.2.5.1-1: Additional test requirements "NS_35"

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| 0-0.1 | -15.0 + TT | -18.0 + TT | -20.0 + TT | -21.0 + TT | 30 kHz |
| 0.1-6 | -13.0 + TT- | -13.0 + TT | -13.0 + TT | -13.0 + TT | 100 kHz |
| 6-10 | -251 + TT | -13.0 + TT | -13.0 + TT | -13.0 + TT | 100 kHz |
| 10-15 |  | -251 + TT | -13.0 + TT | -13.0 + TT | 100 kHz |
| 15-20 |  |  | -251 + TT | -13.0 + TT | 100 kHz |
| 20-25 |  |  |  | -25 + TT | 1 MHz |
| NOTE 1: The measurement bandwidth shall be 1 MHz. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.5G.2.2.5-1. |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5G.2.2.5.2	Test requirements (network signalling value "NS_04")

When "NS_04" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2G.3.5-1 for UE power class 2, Table 6.2G.3.5-2 for UE power class 1.5 or Table FFS for UE power class 3.

and

-	the power of any UE emission shall fulfil requirements in table 6.5G.2.2.5.2-1.

Table 6.5G.2.2.5.2-1: Additional test requirements for "NS_04"

| ΔfOOB MHz |  | Spectrum emission limit (dBm) / measurement bandwidth for each channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 15 | 20 | 30 | 40 | 50 |  | 60 | 70 | 80 | 90 | 100 |  |
| ± 0 - 1 | -10+TT | -10+TT | -10+TT | -10+TT | -10+TT |  |  |  |  |  |  |  | 2 % channel bandwidth |
|  |  |  |  |  |  |  | -10 |  |  |  |  |  | 1 MHz |
| ± 1 - 5 |  | -10 + TT |  |  |  |  |  |  |  |  |  |  | 1 MHz |
| ± 5 - X |  | -13 + TT |  |  |  |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: X is defined in Table 6.5.2.3.3.2-1 for CP-OFDM and 6.5.2.3.3.2-2 for DFT-S-OFDM. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.5G.2.2.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.5G.2.2.5.3	Test requirements (network signalling value "NS_03", "NS_03U" and "NS_21")

When "NS_03" or "NS_03U" or “NS_21” is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in table FFS as appropriate for a NR UE.

and

-	the power of any UE emission shall fulfil requirements in table 6.5G.2.2.5.3-1, as applicable.

Table 6.5G.2.2.5.3-1: Additional requirements for "NS_03", "NS_03U" and "NS_21"

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |
| 0-1 | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | 1 % of channel BW |
| 1-6 | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| 6-10 | -25 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| 10-15 |  | -25 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| 15-20 |  |  | -25 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| 20-25 |  |  |  | -25 + TT | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| 25-30 |  |  |  |  | -25 + TT | -13 + TT | -13 + TT | 1 MHz |
| 30-35 |  |  |  |  |  | -25 + TT | -13 + TT | 1 MHz |
| 35-40 |  |  |  |  |  |  | -13 + TT | 1 MHz |
| 40-45 |  |  |  |  |  |  | -25 + TT | 1 MHz |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5G.2.2.5-1. |  |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5G.2.2.5.4	Test requirements (network signalling value "NS_06")

When "NS_06" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in table FFS as appropriate for a NR UE

and

-	the power of any UE emission shall fulfil requirements in table 6.5G.2.2.5.4-1, as applicable.

Table 6.5G.2.2.5.4-1: Additional requirements for "NS_06"

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | Measurement bandwidth |
| ± 0 – 0.1 | -15 + TT | -18 + TT | -20 + TT | 30 kHz |
| ± 0.1 – 1 | -13 + TT | -13 + TT | -13 + TT | 100 kHz |
| ± 1 – 6 | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| ± 6 – 10 | -25 + TT | -13 + TT | -13 + TT | 1 MHz |
| ± 10 – 15 |  | -25 + TT | -13 + TT | 1 MHz |
| ± 15 – 20 |  |  | -25 + TT | 1 MHz |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5G.2.2.5-1. |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5G.2.3	Adjacent channel leakage ratio for Tx Diversity

If UE indicates IE Tx diversity capability, Adjacent Channel Leakage power Ratio (ACLR) is defined as the ratio of sum of the filtered mean power at each antenna connector centred on the assigned channel frequency to sum of the filtered mean power at each antenna connector centred on an adjacent channel frequency.

##### 6.5G.2.3.1	NR ACLR for Tx Diversity

6.5G.2.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5G.2.3.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.2.3.1.3	Minimum conformance requirements

NR adjacent channel leakage power ratio (NRACLR) is the ratio of sum of the filtered mean power at each antenna connector centred on the assigned NR channel frequency to sum of the filtered mean power at each antenna connector centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5G.2.3.1.3-1.

If the measured adjacent channel power is greater than –50dBm then the NRACLR shall be higher than the value specified in Table 6.5G.2.3.1.3-2.

Table 6.5G.2.3.1.3-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |

Table 6.5G.2.3.1.3-2: NR ACLR requirement

|  | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NR ACLR | 31 dB | 31 dB | 30 dB |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.2.3.1.

6.5G.2.3.1.4	Test description

6.5G.2.3.1.4.1	Initial conditions

Same initial conditions as 6.2G.2.4.1.

6.5G.2.3.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the test configuration tables in clause 6.2.2.4.1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode according to the test configuration, as measured in step 3 of 6.2G.2.4.2, which shall meet the requirements described in clause 6.2G.2.5 as appropriate.

4.	Measure the sum of the rectangular filtered mean power at each antenna connector for the assigned NR channel using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the sum of the rectangular filtered mean power at each antenna connector of the first NR adjacent channel on both lower and upper side of the assigned NR channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NR ACLR, respectively.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration tables in clause 6.2.2.4.1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5G.2.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

Table 6.5G.2.3.1.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

6.5G.2.3.1.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in clause 6.2G.2.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 6, shall be higher than the limits in Table 6.5G.2.3.1.5-2 with TT applying for the sum of power at each of UE antenna connector.

Table 6.5G.2.3.1.5-1: NR ACLR measurement bandwidth

|  | NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR ACLR measurement bandwidth (MHz) |  | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 33.855 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |

Table 6.5G.2.3.1.5-2: NR ACLR requirement for Tx Diversity

|  | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NR ACLR | 31 - TT dB | 31 - TT dB | 30 - TT dB |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5G.2.3.1.5-3. NOTE 2: TT for the sum of power at each of UE antenna connector. |  |  |  |

Table 6.5G.2.3.1.5-3: Test Tolerance (NR ACLR)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 0.8 dB | 0.8 dB | 0.8 dB |

##### 6.5G.2.3.2	UTRA ACLR for Tx Diversity

6.5G.2.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5G.2.3.2.2	Test applicability

This test case applies for network signalling values NS_03U, NS_05U, NS_43U, and NS_100 to all types of NR Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.2.3.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters apply to the sum of the emissions from all UE transmit antenna connectors.

If UE indicates IE txDiversity-r16, Adjacent Channel Leakage power Ratio (ACLR) is defined as the ratio of sum of the filtered mean power at each antenna connector centred on the assigned channel frequency to sum of the filtered mean power at each antenna connector centred on an adjacent channel frequency.

The requirements specified in clause 6.5.2 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.2.

6.5G.2.3.2.4	Test description

Same test description as specified in clause 6.5.2.4.2.4 with following exceptions:

Step 3, 4 and 5 of Test procedure as in 6.5.1.4.2 is replaced by:

3.	Measure the mean power of the UE as the sum of the power from all UE transmit antenna connectors in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in 6.2.3.5 as appropriate. The period of the measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD slots with transient periods are not under test.

4.	Measure the rectangular filtered mean power as the sum of the power from all UE transmit antenna connectors for the assigned NR channel using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the RRC filtered mean power as the sum of the power from all UE transmit antenna connectors of the first and the second UTRA adjacent channel on both lower and upper side of the NR channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.5G.2.3.2.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements described in clause 6.2G.3.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured UTRA ACLR, derived in step 6, shall be higher than the limits in table 6.5G.2.3.2.5-1.

Table 6.5G.2.3.2.5-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB -TT |
| UTRAACLR2 | 36 dB - TT |
| NOTE 1: TT = 0.8 dB |  |

### 6.5G.3	Spurious emission for Tx Diversity

#### 6.5G.3.1	General spurious emissions for Tx Diversity

6.5G.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5G.3.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.3.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products apply to the sum of the emissions from all UE transmit antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5G.3.

6.5G.3.1.4	Test description

Same test description as specified in clause 6.5.3.1.4 with following exceptions:

Step 3 of Test procedure as in 6.5.3.1.4.2 is replaced by:

3.	Measure the sum power of the transmitted signal at all antenna connectors with a measurement filter of bandwidths according to Table 6.5G.3.1.5-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5G.3.1.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5G.3.1.5	Test requirement

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in Table 6.5G.3.1.5-1.

Table 6.5G.3.1.5-1: General spurious emissions for Tx Diversity test requirements

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz | 4 |
|  | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 2.55 GHz and less than or equal to 5.2 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in sub-clause 5.2B of 38.101-3 [4] when NS_04 is signalled. NOTE 4: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [4] when NS_04 is signalled. |  |  |  |

#### 6.5G.3.2	Spurious emissions for UE co-existence for Tx Diversity

6.5G.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

6.5G.3.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.3.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products apply to the sum of the emissions from all UE transmit antenna connectors.

The requirements specified in clause 6.5.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5G.3.

6.5G.3.2.4	Test description

Same test description as specified in clause 6.5.3.2.4 with following exceptions:

Step 3 of Test procedure as in 6.5.3.2.4.2 is replaced by:

3.	Measure the sum power of the transmitted signal at all antenna connectors with a measurement filter of bandwidths according to table 6.5.3.2.3-1 to 6.5.3.2.3-3. The centre frequency of the filter shall be stepped in contiguous steps according to table 6.5.3.2.3-1 to 6.5.3.2.3-3. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5G.3.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements and are not repeated in this section.

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Tables 6.5.3.1.3-1 from the edge of the channel bandwidth. The spurious emission limits in Tables 6.5.3.2.3-1 to 6.5.3.2.3-3 apply for all transmitter band configurations (NRB) and channel bandwidths.

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in Table 6.5.3.2.3-1 to 6.5.3.2.3-3 for difference releases.

The requirements for the UE are release specific and can be found in Tables 6.5.3.2.3-1 to 6.5.3.2.3-3. If the UE support a band, which is not defined in the table corresponding UE’s release, the requirements for this band are taken from the table of earliest release where requirements for this band are defined. This has been described in following Table 6.5G.3.2.5-1.

Table 6.5G.3.2.5-1: UE Requirements according to UE NR release and supported E-UTRA and NR band

| UE Requirements per release |  |  |  |
| --- | --- | --- | --- |
| NR Band | Rel-15 | Rel-16 | R17 |
| n1 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n2 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n3 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n5 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n7 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n8 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n12 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n14 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n18 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n20 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n24 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 |
| n25 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n26 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n28 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n30 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n34 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n38 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n39 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n40 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n41 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n48 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n50 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n51 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n53 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n65 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n66 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n70 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n71 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n74 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n77, n78 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| n79 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 |
| NOTE 1: The frequency range applicable with network signalling values of NS_04, NS_17, NS_18, NS_05, NS_43, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_45 and NS_56 are covered in subclause 6.5.3.3 Additional Spurious Emissions NOTE 2: The restriction on the maximum uplink transmission to 54 RB in Notes 21 and 22 of Tables 6.5G.3.2.3-1 to 6.5G.3.2.3-3 is intended for conformance testing and may be applied to network operation to facilitate coexistence when the aggressor and victim bands are deployed in the same geographical area. The applicable spurious emission requirement of -15.5 dBm/5MHz is a least restrictive technical condition for FDD/TDD coexistence and may have to be revised in the future. |  |  |  |

#### 6.5G.3.3	Additional spurious emissions for Tx Diversity

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values other than NS_04 are incomplete due to lack of test requirement in 6.2G.3.

6.5G.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5G.3.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.3.3.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products apply to the sum of the emissions from all UE transmit antenna connectors.

The requirements specified in clause 6.5.3.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5G.3.

6.5G.3.3.4	Test description

For Power Class 2 and Power Class 3 UE, same test description as specified in clause 6.5.3.3.4 with following exceptions:

Steps 3 and 4 of Test procedure as in 6.5.3.3.4.2 is replaced by:

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode, which shall meet the requirements described in Clauses  6.2G.3.5. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum power of the transmitted signal at all antenna connectors with a measurement filter of bandwidths according to Tables 6.5.3.3.3.1 to 6.5.3.3.3.27 as appropriate. The centre frequency of the filter shall be stepped in contiguous steps according to the same table the measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

For Power Class 1.5 UE, same test configuration as listed in Table 6.2G.3.4.1-2 shall be used for NS_50 (Power Class 1.5).

6.5G.3.3.5	Test requirement

This clause specifies the requirements for the specified NR band for an additional spectrum emission requirement with protected bands as indicated from Table 6.5.3.3.5.1 to Table 6.5.3.3.5.27 for different NS_values.

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements specified as 6.2G.3.5 for different NS_values.

The measured power derived in step 4 shall meet the requirements for the specified NR band for an additional spurious emission requirement with protected bands as indicated in clause 6.5.3.3.5 for different NS values.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

### 6.5G.4	Transmit intermodulation for Tx Diversity

6.5G.4.1	Test purpose

To verify that the UE transmit intermodulation for Tx Diversity does not exceed the described value in the test requirement.

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

6.5G.4.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.4.3	Minimum conformance requirements

For UE supporting Tx diversity, the transmit intermodulation requirements are specified at each transmit antenna connector and the wanted signal is defined as the sum of output power from all UE transmit antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.4.

6.5G.4.4	Test description

Same test description as specified in clause 6.5.4.4 with following exceptions:

Test procedure as in 6.5.4.4.2 is replaced by:

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE until the UE transmits at its PUMAX level.

3.	Measure the rectangular filtered mean power at each antenna connector of the UE. For TDD, only slots consisting of only UL symbols are under test for the wanted signal and for the intermodulation product.

4.	Set the interference signal frequency below the UL carrier frequency using the first offset in table 6.5.4.5-1.

5.	Set the interference CW signal level according to table 6.5.4.5-1.

6.	Search the intermodulation product signals below and above the UL carrier frequency at each UE antenna connector, then measure the rectangular filtered mean power at each antenna connector of transmitting intermodulation for both signals, and calculate the ratios with the power measured in step 3.

7.	Set the interference signal frequency above the UL carrier frequency using the first offset in table 6.5.4.5-1.

8.	Search the intermodulation product signals below and above the UL carrier frequency at each UE antenna connector, then measure the rectangular filtered mean power at each antenna connector of transmitting intermodulation for both signals, and calculate the ratios with the power measured in step 3.

9.	Repeat the measurement using the second offset in table 6.5.4.5-1.

10.	Repeat step 3) until 9) for each of transmit antenna of the UE.

6.5G.4.5	Test requirement

The ratio derived in step 6 and 8, shall not exceed the described value in table 6.5.4.5-1.