Title:	Adding test point selection for new CBW 35MHz for NS_50 A-MPR and A-SE in FR1

Document for:	Discussion and approval

# Background

The purpose of this paper is to provide an analysis of the parameters for the A-MPR test configuration table with a selection of test points that minimize testing and still achieve good test coverage for NS_50. The intention with NS_50 is to protect DL of E-UTRA band 3/NR band n3 as indicated in Table 1 below.

Table 1: Additional requirements for “NS_50” (Table 6.5.3.3.3.16-1 from [4])

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 1805 | - | 1855 | -40 | 1 | 1 |
| Frequency range | 1855 | - | 1880 | -15.5 | 5 | 1, 2, 3 |
| NOTE 1: This requirement is applicable for carriers with aggregated channel bandwidths confined in 1885-1920 MHz for ≤ 30MHz channel BWs and confined in 1880-1920 MHz for 40MHz channel BW. NOTE 2: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 and Table 6.5A.3.1.0-1 from the edge of the channel bandwidth. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

# Discussion

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for test point selection for A-MPR in FR1.

The selection of test points below is partly based on approved test point analyzes for SEM [2, 3] for FR1. For NS_50 the operating band n39 (1880-1920 MHz) has specific A-MPR requirements, see Table 2 and Table 3.

Table 2: A-MPR regions for NS_50 (Power Class 3) (Table 6.2.3.19-1 from [4])

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 25 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 6.48 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 30 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 8.64 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 35 MHz | ≤ 1.62 | > 0 | A1 |
|  | > 1.62, ≤ 9.36 | ≤ 9.0 | A3 |
|  | > 1.62, ≤ 13.68 | > 9.0 | A2 |
|  | > 13.68, ≤ 30.6 | > max (29.7 –RBstart *12*SCS, 0) | A6 |
|  | > 30.6 | > 0 | A5 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32, ≤ 10.44 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8 | A2 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A6 |
|  | > 31.68 | > 0 | A5 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.19-2. |  |  |  |

Table 3: A-MPR regions for NS_50 (Power Class 2) (Table 6.2.3.19-3 from [4])

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10 MHz | ≤ 1.44 | < 1.44 | A5 |
|  | ≤ 1.8 | ≥ 2.7+2* RBstart*12*SCS | A4 |
|  | >1.8 | ≥ 8.1- RBstart*12*SCS | A4 |
| 15 MHz | ≤ 2.88 | < 2.7 | A5 |
|  | ≤ 3.24 | ≥ 2.7+2* RBstart*12*SCS | A3 |
|  | >3.24 | ≥ 12.42- RBstart*12*SCS | A4 |
| 20 MHz | ≤ 4.32 | < 3.6 | A5 |
|  | ≤ 4.5 | ≥ 3.6+2* RBstart*12*SCS | A3 |
|  | >4.5 | ≥ 17.1- RBstart*12*SCS | A4 |
| 25 MHz | ≤ LCRB*12*SCS – 5 | > 5 | A2 |
|  | ≤ 6.48 | ≤ 1.44 | A5 |
|  | > 8.28 | > max (21.6 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | >1.8, ≤6.48 | > 1.44, ≤ 3.6 | A6 |
|  | > LCRB *12*SCS – 5, ≤ 1.8 | > 1.44 | A4 |
| 30 MHz | ≤ LCRB*12*SCS – 5 | >5 | A2 |
|  | ≤ 7.56 | ≤ 1.44 | A5 |
|  | >1.8, ≤7.56 | > 1.44, ≤ 3.6 | A6 |
|  | ≤ 1.8 | >1.44, <RBstart*12*SCS+5 | A4 |
|  | > 10.8 | > max (26.64 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
| 35 MHz | ≤ 1.62 | > 0 | A1 |
|  | > 1.62, ≤ 9.36 | ≤ 9.0 | A3 |
|  | > 1.62, ≤ 13.68 | > 9.0 | A8 |
|  | > 13.68, ≤ 30.6 | > max (29.7 –RBstart *12*SCS, 0) | A4 |
|  | > 30.6 | > 0 | A9 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32 | > RBstart*12*SCS + 11.88 | A8 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8, <= RBstart*12*SCS + 11.88 | A7 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A4 |
|  | > 31.68 | > 0 | A9 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.19-4. |  |  |  |

Table 3A: A-MPR regions for NS_50 (Power Class 1.5) (Table 6.2.3.19-5 from [4])

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10 MHz | ≤ 1.44 | < 1.44 | A5 |
|  | ≤ 1.8 | ≥ 2.7+2* RBstart*12*SCS | A4 |
|  | >1.8 | ≥ 8.1- RBstart*12*SCS | A4 |
| 15 MHz | ≤ 2.88 | < 2.7 | A5 |
|  | ≤ 3.24 | ≥ 2.7+2* RBstart*12*SCS | A3 |
|  | >3.24 | ≥ 12.42- RBstart*12*SCS | A4 |
|  | < 12.42- LCRB*12*SCS | ≥ 2.7 < 2.7+2* RBstart*12*SCS ≥ 2* RBstart*12*SCS-1.08 | A10 |
| 20 MHz | ≤ 4.32 | < 3.6 | A5 |
|  | ≤ 4.5 | ≥ 3.6+2* RBstart*12*SCS | A3 |
|  | >4.5 | ≥ 17.1- RBstart*12*SCS | A4 |
|  | < 17.1- LCRB*12*SCS | ≥ 3.6 < 3.6+2* RBstart*12*SCS ≥ 2* RBstart*12*SCS-3.6 | A10 |
| 25 MHz | ≤ LCRB*12*SCS – 5 | > 5 | A2 |
|  | ≤ 6.48 | ≤ 1.44 | A5 |
|  | > 8.28 | > max (21.6 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | >1.8, ≤6.48 | > 1.44, ≤ 3.6 | A6 |
|  | > LCRB *12*SCS – 5, ≤ 1.8 | > 1.44 | A4 |
|  | > 1.8, ≤ 6.48 > LCRB *12*SCS -5 | > 3.6 | A10 |
| 30 MHz | ≤ LCRB*12*SCS – 5 | >5 | A2 |
|  | ≤ 7.56 | ≤ 1.44 | A5 |
|  | >1.8, ≤7.56 | > 1.44, ≤ 3.6 | A6 |
|  | ≤ 1.8 | >1.44, <RBstart*12*SCS+5 | A4 |
|  | > 10.8 | > max (26.64 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | > 1.8, ≤ 7.56 > LCRB *12*SCS -5 | > 3.6 | A10 |
| 35 MHz | ≤ 1.62 | > 0 | A1 |
|  | > 1.62, ≤ 9.36 | ≤ 9.0 | A3 |
|  | > 1.62, ≤ 13.68 | > 9.0 | A8 |
|  | > 13.68, ≤ 30.6 | > max (29.7 –RBstart *12*SCS, 0) | A4 |
|  | > 30.6 | > 0 | A9 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32 | > RBstart*12*SCS + 11.88 | A8 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8, <= RBstart*12*SCS + 11.88 | A7 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A4 |
|  | > 31.68 | > 0 | A9 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.19-4. |  |  |  |

The RB allocation for testing of these A-MPR regions depends on both channel bandwidth and SCS.

## Test Environment

Based on the baseline agreed in [2], the test environment for NR A-MPR measurement for FR1 can be defined as in LTE.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR NS_50.

## Test Frequencies

There is a NOTE 1 in Table 1 indicating the test frequency limitation: This requirement is applicable for carriers with aggregated channel bandwidths confined in 1885-1920 MHz for ≤ 30MHz channel BWs and confined in 1880-1920 MHz for 40MHz channel BW.

Within the above frequency range restriction, the lowest carrier centre frequency for each channel bandwidth is selected since this frequency is closest to the frequency range 1805-1880MHz to be protected.

PROPOSAL 2: Select the lowest carrier centre frequency for aggregated channel bandwidth confined in 1885-1920 MHz for <=30MHz channel BWs, and confined in 1880-1920 MHz for 40MHz channel BW for each A-MPR requirement.

## Test Channel Bandwidth

The minimum requirements for NS_50 (Power Class 3) apply for channel bandwidths 25MHz, 30MHz, 35MHz and 40MHz.

The minimum requirements for NS_50 (Power Class 2) apply for channel bandwidths 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 35MHz and 40MHz.

The minimum requirements for NS_50 (Power Class 1.5) apply for channel bandwidths 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 35MHz and 40MHz.

PROPOSAL 3: If supported, select 25MHz, 30MHz, 35MHz and 40MHz channel bandwidth for NS_50 (Power Class 3), and select 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 35MHz and 40MHz channel bandwidth for NS_50 (Power Class 2), and select 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 35MHz and 40MHz channel bandwidth for NS_50 (Power Class 1.5).

## Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM [2], i.e. Lowest and Highest supported subcarrier spacing.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing.

## UL Modulation

The selection of UL modulation can be based on the agreed proposals for SEM [2], i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms). But in order to reduce the testing workload, down selection of UL modulation can be considered. The highest modulation type need to be tested considering the high PAPR feature. But with the highest modulation, MPR might be higher than A-MPR, and in that case A-MPR is not verified properly, therefore the lowest modulation also needs to be tested. For DFT-s-OFDM QPSK is selected instead of BPSK since BPSK is optionally supported.

PROPOSAL 5: Test QPSK and 256QAM for DFT-s-OFDM and CP-OFDM.

## Uplink RB allocation

The UL RB allocation can be based on the agreed proposals for SEM [2], i.e. Edge_1RB_left, Edge_1RB_Right and Outer_Full. These common uplink configurations are listed in [5].

A-MPR for NS_50 has some Region specific RB allocations in Table 2 and Table 3, where the ranges for the RB allocation is specified in terms of RBend × 12 × SCS and LCRB × 12 × SCS. Table 4 and Table 5 shows the corresponding RB ranges for 15/30/60 kHz SCS and proposed allocation expressed as LCRB@RBstart.

Table 4: UL RB allocation for NS_50 (Power Class 3)

| Ch BW (MHz) | SCS (kHz) | RBstart | LCRB | Prop. Alloc (DFT-s-OFDM) | Prop. Alloc (CP-OFDM) | A-MPR |
| --- | --- | --- | --- | --- | --- | --- |
| 25 | 15 | <=LCRB-28 | >=28 | Outer_Full | Outer_Full | A7 |
|  | 30 | <=LCRB-14 | >=14 | Outer_Full | Outer_Full | A7 |
|  | 60 | <=LCRB-7 | >=7 | Outer_Full | Outer_Full | A7 |
|  | 15 | <=36 | <=8 | Edge_1RB_left | Edge_1RB_left | A8 |
|  | 30 | <=18 | <=4 | Edge_1RB_left | Edge_1RB_left | A8 |
|  | 60 | <=9 | <=2 | Edge_1RB_left | Edge_1RB_left | A8 |
|  | 15 | <=36 | <=20 | 20@0 | 20@0 | A9 |
|  | 30 | <=18 | <=10 | 10@0 | 10@0 | A9 |
|  | 60 | <=9 | <=5 | 5@0 | 5@0 | A9 |
| 30 | 15 | <=LCRB-28 | >=28 | Outer_Full | Outer_Full | A7 |
|  | 30 | <=LCRB-14 | >=14 | Outer_Full | Outer_Full | A7 |
|  | 60 | <=LCRB-7 | >=7 | Outer_Full | Outer_Full | A7 |
|  | 15 | <=48 | <=8 | Edge_1RB_left | Edge_1RB_left | A8 |
|  | 30 | <=24 | <=4 | Edge_1RB_left | Edge_1RB_left | A8 |
|  | 60 | <=12 | <=2 | Edge_1RB_left | Edge_1RB_left | A8 |
|  | 15 | <=48 | <=20 | 20@0 | 20@0 | A9 |
|  | 30 | <=24 | <=10 | 10@0 | 10@0 | A9 |
|  | 60 | <=12 | <=5 | 5@0 | 5@0 | A9 |
| 35 | 15 | <=9 | >0 | Outer_Full | Outer_Full | A1 |
|  | 30 | <=4.5 | >0 | Outer_Full | Outer_Full | A1 |
|  | 60 | <=2.25 | >0 | Outer_Full | Outer_Full | A1 |
|  | 15 | (9, 52] | <=50 | 50@10 | 50@10 | A3 |
|  | 30 | (4.5, 26] | <=25 | 25@5 | 25@5 | A3 |
|  | 60 | (2.25, 13] | <=12.5 | 12@3 | 12@3 | A3 |
|  | 15 | (9, 76] | >50 | 178@10 | 178@10 | A2 |
|  | 30 | (4.5, 38] | >25 | 87@5 | 87@5 | A2 |
|  | 60 | (2.25, 19] | >12.5 | 40@3 | 41@3 | A2 |
|  | 15 | (76, 170] | >165-RBstart | 111@77 | 111@77 | A6 |
|  | 30 | (38, 85] | >82.5-RBstart | 52@39 | 53@39 | A6 |
|  | 60 | (19, 42.5] | >41.25-RBstart | 24@20 | 24@20 | A6 |
|  | 15 | >170 | >0 | Edge 1RB right | Edge 1RB right | A5 |
|  | 30 | >85 | >0 | Edge 1RB right | Edge 1RB right | A5 |
|  | 60 | >42.5 | >0 | Edge 1RB right | Edge 1RB right | A5 |
| 40 | 15 | <=24 | >0 | Outer_Full | Outer_Full | A1 |
|  | 30 | <=12 | >0 | Outer_Full | Outer_Full | A1 |
|  | 60 | <=6 | >0 | Outer_Full | Outer_Full | A1 |
|  | 15 | (24，58] | <=60 | 60@25 | 60@25 | A3 |
|  | 30 | (12，29] | <=30 | 30@13 | 30@13 | A3 |
|  | 60 | (6，14] | <=15 | 15@7 | 15@7 | A3 |
|  | 15 | (24，100] | >60 | 180@25 | 191@25 | A2 |
|  | 30 | (12，50] | >30 | 90@13 | 93@13 | A2 |
|  | 60 | (6，25] | >15 | 40@7 | 44@7 | A2 |
|  | 15 | (100，176] | >75 | 108@101 | 115@101 | A6 |
|  | 30 | (50，88] | >37 | 54@51 | 55@51 | A6 |
|  | 60 | (25，44] | >18 | 25@26 | 25@26 | A6 |
|  | 15 | >176 | >0 | 36@177 | 39@177 | A5 |
|  | 30 | >=88 | >0 | 16@89 | 17@89 | A5 |
|  | 60 | >=44 | >0 | 6@45 | 6@45 | A5 |

Table 5: UL RB allocation for NS_50 (Power Class 2)

| Ch BW (MHz) | SCS (kHz) | RBstart | LCRB | Prop. Alloc (DFT-s-OFDM) | Prop. Alloc (CP-OFDM) | A-MPR |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 15 | <=8 | <8 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 30 | <=4 | <4 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 60 | <=2 | <2 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 15 | <=10 | >=35 | Outer_Full | Outer_Full | A4 |
|  | 30 | <=5 | >=17.5 | Outer_Full | Outer_Full | A4 |
|  | 60 | <=2.5 | >=8.75 | Outer_Full | Outer_Full | A4 |
|  | 15 | >10 | >=34 | 40@11 | 41@11 | A4 |
|  | 30 | >5 | >=16.5 | 18@6 | 18@6 | A4 |
|  | 60 | >2.5 | >=8.75 | / | / | A4 |
| 15 | 15 | <=16 | <15 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 30 | <=8 | <7.5 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 60 | <=4 | <3.75 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 15 | <=18 | >=15+ 2*Rbstart | Outer_Full | Outer_Full | A3 |
|  | 30 | <=9 | >=7.5+ 2*Rbstart | Outer_Full | Outer_Full | A3 |
|  | 60 | <=4.5 | >=3.75+ 2*Rbstart | Outer_Full | Outer_Full | A3 |
|  | 15 | >18 | >=50 | 60@19 | 60@19 | A4 |
|  | 30 | >9 | >=24.5 | 27@10 | 28@10 | A4 |
|  | 60 | >4.5 | >=12.25 | 15@5 | 13@5 | A4 |
| 20 | 15 | <=24 | <20 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 30 | <=12 | <10 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 60 | <=6 | <5 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 15 | <=25 | >=20 | Outer_Full | Outer_Full | A3 |
|  | 30 | <=12.5 | >=10 | Outer_Full | Outer_Full | A3 |
|  | 60 | <=6.25 | >=5 | Outer_Full | Outer_Full | A3 |
|  | 15 | >25 | >=69 | 80@26 | 80@26 | A4 |
|  | 30 | >12.5 | >=34.5 | 36@13 | 38@13 | A4 |
|  | 60 | >6.25 | >=16.75 | 18@7 | 17@7 | A4 |
| 25 | 15 | <=LCRB-28 | >28 | Outer_Full | Outer_Full | A2 |
|  | 30 | <=LCRB-14 | >14 | Outer_Full | Outer_Full | A2 |
|  | 60 | <=LCRB-7 | >7 | Outer_Full | Outer_Full | A2 |
|  | 15 | <=36 | <=8 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 30 | <=18 | <=4 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 60 | <=9 | <=2 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 15 | >46 | (72，75.78) | 75@48 | 74@47 | A4 |
|  | 30 | >23 | (35，38.89) | 36@25 | 37@24 | A4 |
|  | 60 | >11.5 | (17，19.95) | 18@13 | 19@13 | A4 |
|  | 15 | [11, 36] | (8, 20] | 20@11 | 20@11 | A6 |
|  | 30 | [6, 18] | (4, 10] | 10@6 | 10@6 | A6 |
|  | 60 | [3, 9] | (2, 5] | 5@3 | 5@3 | A6 |
|  | 15 | (LCRB–27.78, 10] | >8 | 27@0 | 27@0 | A4 |
|  | 30 | (LCRB–13.89, 5] | >4 | 12@0 | 13@0 | A4 |
|  | 60 | (LCRB–6.94, 2.5] | >2 | 6@0 | 6@0 | A4 |
| 30 | 15 | <=LCRB-28 | >28 | Outer_Full | Outer_Full | A2 |
|  | 30 | <=LCRB-14 | >14 | Outer_Full | Outer_Full | A2 |
|  | 60 | <=LCRB-7 | >7 | Outer_Full | Outer_Full | A2 |
|  | 15 | <=42 | <=8 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 30 | <=21 | <=4 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 60 | <=10.5 | <=2 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 15 | (10, 42] | (8, 20] | 20@11 | 20@11 | A6 |
|  | 30 | (5, 21] | (4, 10] | 10@6 | 10@6 | A6 |
|  | 60 | (2.5, 10] | (2, 5] | 5@3 | 5@3 | A6 |
|  | 15 | <=10 | (8, Rbstart+28) | 27@0 | 27@0 | A4 |
|  | 30 | <=5 | (4, Rbstart+14) | 12@0 | 13@0 | A4 |
|  | 60 | <=2.5 | (2, Rbstart+7) | 6@0 | 6@0 | A4 |
|  | 15 | >60 | (85, 90.78) | 90@63 | 88@61 | A4 |
|  | 30 | >30 | (42, 45.89) | 45@32 | 44@31 | A4 |
|  | 60 | >15 | (19, 24.95) | 24@18 | 22@16 | A4 |
| 35 | 15 | <=9 | >0 | Outer_Full | Outer_Full | A1 |
|  | 30 | <=4.5 | >0 | Outer_Full | Outer_Full | A1 |
|  | 60 | <=2.25 | >0 | Outer_Full | Outer_Full | A1 |
|  | 15 | (9, 52] | <=50 | 50@10 | 50@10 | A3 |
|  | 30 | (4.5, 26] | <=25 | 25@5 | 25@5 | A3 |
|  | 60 | (2.25, 13] | <=12.5 | 12@3 | 12@3 | A3 |
|  | 15 | (9, 76] | >50 | 178@10 | 178@10 | A8 |
|  | 30 | (4.5, 38] | >25 | 87@5 | 87@5 | A8 |
|  | 60 | (2.25, 19] | >12.5 | 40@3 | 41@3 | A8 |
|  | 15 | (76, 170] | >165-RBstart | 111@77 | 111@77 | A4 |
|  | 30 | (38, 85] | >82.5-RBstart | 52@39 | 53@39 | A4 |
|  | 60 | (19, 42.5] | >41.25-RBstart | 24@20 | 24@20 | A4 |
|  | 15 | >170 | >0 | Edge 1RB right | Edge 1RB right | A9 |
|  | 30 | >85 | >0 | Edge 1RB right | Edge 1RB right | A9 |
|  | 60 | >42.5 | >0 | Edge 1RB right | Edge 1RB right | A9 |
| 40 | 15 | <=24 | >0 | Outer_Full | Outer_Full | A1 |
|  | 30 | <=12 | >0 | Outer_Full | Outer_Full | A1 |
|  | 60 | <=6 | >0 | Outer_Full | Outer_Full | A1 |
|  | 15 | >24 | >Rbstart+66 | 180@25 | 191@25 | A8 |
|  | 30 | >12 | >Rbstart+33 | 90@13 | 93@13 | A8 |
|  | 60 | >6 | >Rbstart+16.5 | 40@7 | 44@7 | A8 |
|  | 15 | (24, 72] | <=60 | 60@25 | 60@25 | A3 |
|  | 30 | (12, 36] | <=30 | 30@13 | 30@13 | A3 |
|  | 60 | (6, 18] | <=15 | 15@7 | 15@7 | A3 |
|  | 15 | (24, 100] | (60,Rbstart+66] | 90@25 | 91@25 | A7 |
|  | 30 | (12, 50] | (30,Rbstart+33] | 45@13 | 46@13 | A7 |
|  | 60 | (6, 25] | (15,Rbstart+16.5] | 20@7 | 23@7 | A7 |
|  | 15 | (100, 176] | >176- Rbstart | 108@101 | 115@101 | A4 |
|  | 30 | (50, 88] | >88- Rbstart | 54@51 | 55@51 | A4 |
|  | 60 | (25, 44] | >44- Rbstart | 25@26 | 25@26 | A4 |
|  | 15 | >176 | >0 | 36@177 | 39@177 | A9 |
|  | 30 | >88 | >0 | 16@89 | 17@89 | A9 |
|  | 60 | >44 | >0 | 6@45 | 6@45 | A9 |

Table 5A: UL RB allocation for NS_50 (Power Class 1.5)

| Ch BW (MHz) | SCS (kHz) | RBstart | LCRB | Prop. Alloc (DFT-s-OFDM) | Prop. Alloc (CP-OFDM) | A-MPR |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 15 | <=8 | <8 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 30 | <=4 | <4 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 60 | <=2 | <2 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 15 | <=10 | >=35 | Outer_Full | Outer_Full | A4 |
|  | 30 | <=5 | >=17.5 | Outer_Full | Outer_Full | A4 |
|  | 60 | <=2.5 | >=8.75 | Outer_Full | Outer_Full | A4 |
|  | 15 | >10 | >=34 | 40@11 | 41@11 | A4 |
|  | 30 | >5 | >=16.5 | 18@6 | 18@6 | A4 |
|  | 60 | >2.5 | >=8.75 | / | / | A4 |
| 15 | 15 | <=16 | <15 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 30 | <=8 | <7.5 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 60 | <=4 | <3.75 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 15 | <=18 | >=15+ 2*Rbstart | Outer_Full | Outer_Full | A3 |
|  | 30 | <=9 | >=7.5+ 2*Rbstart | Outer_Full | Outer_Full | A3 |
|  | 60 | <=4.5 | >=3.75+ 2*Rbstart | Outer_Full | Outer_Full | A3 |
|  | 15 | >18 | >=50 | 60@19 | 60@19 | A4 |
|  | 30 | >9 | >=24.5 | 27@10 | 28@10 | A4 |
|  | 60 | >4.5 | >=12.25 | 15@5 | 13@5 | A4 |
|  | 15 | <69- LCRB | >=15 <15+2* RBstart >=2* RBstart-6 | 16@1 | 16@1 | A10 |
|  | 30 | <34.5- LCRB | >=7.5 <7.5+2* RBstart >=2* RBstart-3 | 9@1 | 9@1 | A10 |
|  | 60 | <17.25- LCRB | >=3.75 <3.75+2* RBstart >=2* RBstart-1.5 | 5@1 | 5@1 | A10 |
| 20 | 15 | <=24 | <20 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 30 | <=12 | <10 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 60 | <=6 | <5 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 15 | <=25 | >=20 | Outer_Full | Outer_Full | A3 |
|  | 30 | <=12.5 | >=10 | Outer_Full | Outer_Full | A3 |
|  | 60 | <=6.25 | >=5 | Outer_Full | Outer_Full | A3 |
|  | 15 | >25 | >=69 | 80@26 | 80@26 | A4 |
|  | 30 | >12.5 | >=34.5 | 36@13 | 38@13 | A4 |
|  | 60 | >6.25 | >=16.75 | 18@7 | 17@7 | A4 |
|  | 15 | <95- LCRB | >=20 <20+2* RBstart >=2* RBstart-20 | 21@1 | 21@1 | A10 |
|  | 30 | <47.5- LCRB | >=10 <10+2* RBstart >=2* RBstart-10 | 10@1 | 11@1 | A10 |
|  | 60 | <23.75- LCRB | >=5 <5+2* RBstart >=2* RBstart-5 | 6@1 | 6@1 | A10 |
| 25 | 15 | <=LCRB-28 | >28 | Outer_Full | Outer_Full | A2 |
|  | 30 | <=LCRB-14 | >14 | Outer_Full | Outer_Full | A2 |
|  | 60 | <=LCRB-7 | >7 | Outer_Full | Outer_Full | A2 |
|  | 15 | <=36 | <=8 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 30 | <=18 | <=4 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 60 | <=9 | <=2 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 15 | >46 | (72，75.78) | 75@48 | 74@47 | A4 |
|  | 30 | >23 | (35，38.89) | 36@25 | 37@24 | A4 |
|  | 60 | >11.5 | (17，19.95) | 18@13 | 19@13 | A4 |
|  | 15 | [11, 36] | (8, 20] | 20@11 | 20@11 | A6 |
|  | 30 | [6, 18] | (4, 10] | 10@6 | 10@6 | A6 |
|  | 60 | [3, 9] | (2, 5] | 5@3 | 5@3 | A6 |
|  | 15 | (LCRB–27.78, 10] | >8 | 27@0 | 27@0 | A4 |
|  | 30 | (LCRB–13.89, 5] | >4 | 12@0 | 13@0 | A4 |
|  | 60 | (LCRB–6.94, 2.5] | >2 | 6@0 | 6@0 | A4 |
|  | 15 | (10, 36] | >20 | 38@11 | 38@11 | A10 |
|  | 30 | (5, 18] | >10 | 33@6 | 33@6 | A10 |
|  | 60 | (2.5, 9] | >5 | 30@3 | 30@3 | A10 |
| 30 | 15 | <=LCRB-28 | >28 | Outer_Full | Outer_Full | A2 |
|  | 30 | <=LCRB-14 | >14 | Outer_Full | Outer_Full | A2 |
|  | 60 | <=LCRB-7 | >7 | Outer_Full | Outer_Full | A2 |
|  | 15 | <=42 | <=8 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 30 | <=21 | <=4 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 60 | <=10.5 | <=2 | Edge_1RB_left | Edge_1RB_left | A5 |
|  | 15 | (10, 42] | (8, 20] | 20@11 | 20@11 | A6 |
|  | 30 | (5, 21] | (4, 10] | 10@6 | 10@6 | A6 |
|  | 60 | (2.5, 10] | (2, 5] | 5@3 | 5@3 | A6 |
|  | 15 | <=10 | (8, Rbstart+28) | 27@0 | 27@0 | A4 |
|  | 30 | <=5 | (4, Rbstart+14) | 12@0 | 13@0 | A4 |
|  | 60 | <=2.5 | (2, Rbstart+7) | 6@0 | 6@0 | A4 |
|  | 15 | >60 | (85, 90.78) | 90@63 | 88@61 | A4 |
|  | 30 | >30 | (42, 45.89) | 45@32 | 44@31 | A4 |
|  | 60 | >15 | (19, 24.95) | 24@18 | 22@16 | A4 |
|  | 15 | (10, 42] | >20 | 38@11 | 38@11 | A10 |
|  | 30 | (5, 21] | >10 | 33@6 | 33@6 | A10 |
|  | 60 | (2.5, 10] | >5 | 30@3 | 30@3 | A10 |
| 35 | 15 | <=9 | >0 | Outer_Full | Outer_Full | A1 |
|  | 30 | <=4.5 | >0 | Outer_Full | Outer_Full | A1 |
|  | 60 | <=2.25 | >0 | Outer_Full | Outer_Full | A1 |
|  | 15 | (9, 52] | <=50 | 50@10 | 50@10 | A3 |
|  | 30 | (4.5, 26] | <=25 | 30@5 | 30@5 | A3 |
|  | 60 | (2.25, 13] | <=12.5 | 12@3 | 12@3 | A3 |
|  | 15 | (9, 76] | >50 | 178@10 | 178@10 | A8 |
|  | 30 | (4.5, 38] | >25 | 87@5 | 87@5 | A8 |
|  | 60 | (2.25, 19] | >12.5 | 40@3 | 41@3 | A8 |
|  | 15 | (76, 170] | >165-RBstart | 111@77 | 111@77 | A4 |
|  | 30 | (38, 85] | >82.5-RBstart | 52@39 | 53@39 | A4 |
|  | 60 | (19, 42.5] | >41.25-RBstart | 24@20 | 24@20 | A4 |
|  | 15 | >170 | >0 | 16@171 | 17@171 | A9 |
|  | 30 | >86 | >0 | 6@86 | 6@86 | A9 |
|  | 60 | >43 | >0 | 1@43 | 1@43 | A9 |
| 40 | 15 | <=24 | >0 | Outer_Full | Outer_Full | A1 |
|  | 30 | <=12 | >0 | Outer_Full | Outer_Full | A1 |
|  | 60 | <=6 | >0 | Outer_Full | Outer_Full | A1 |
|  | 15 | >24 | >Rbstart+66 | 180@25 | 191@25 | A8 |
|  | 30 | >12 | >Rbstart+33 | 90@13 | 93@13 | A8 |
|  | 60 | >6 | >Rbstart+16.5 | 40@7 | 44@7 | A8 |
|  | 15 | (24, 72] | <=60 | 60@25 | 60@25 | A3 |
|  | 30 | (12, 36] | <=30 | 30@13 | 30@13 | A3 |
|  | 60 | (6, 18] | <=15 | 15@7 | 15@7 | A3 |
|  | 15 | (24, 100] | (60,Rbstart+66] | 90@25 | 91@25 | A7 |
|  | 30 | (12, 50] | (30,Rbstart+33] | 45@13 | 46@13 | A7 |
|  | 60 | (6, 25] | (15,Rbstart+16.5] | 20@7 | 23@7 | A7 |
|  | 15 | (100, 176] | >176- Rbstart | 108@101 | 115@101 | A4 |
|  | 30 | (50, 88] | >88- Rbstart | 54@51 | 55@51 | A4 |
|  | 60 | (25, 44] | >44- Rbstart | 25@26 | 25@26 | A4 |
|  | 15 | >176 | >0 | 36@177 | 39@177 | A9 |
|  | 30 | >88 | >0 | 16@89 | 17@89 | A9 |
|  | 60 | >44 | >0 | 6@45 | 6@45 | A9 |

The proposals for LCRB@RBstart in Table 4, Table 5 and Table 5A are all based on allocating as many RBs as possible located to the lower part of the channel bandwidth, i.e. select lowest RBstart.

PROPOSAL 6: Select as many RBs as possible close to the lower band edge.

2.7.	Number of Test Points

Based on the proposals the maximum number of test points for A-MPR tests for NS_50 is:

Table 5: Numbers of Test Points for A-MPR NS_50 (Power Class 3)

| Network Signalling label | Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number of Steps (freq. & RB alloc.) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| NS_50 | 1 | 2 (L/H) | 4 (25, 30, 35, 40) | 4 | 5 | 160 |

Table 6: Numbers of Test Points for A-MPR NS_50 (Power Class 2)

| Network Signalling label | Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number of Steps (freq. & RB alloc.) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| NS_50 | 1 | 2 (L/H) | 7 (10, 15, 20, 25, 30, 35, 40) | 4 | 6 | 336 |

Table 6A: Numbers of Test Points for A-MPR NS_50 (Power Class 1.5)

| Network Signalling label | Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number of Steps (freq. & RB alloc.) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| NS_50 | 1 | 2 (L/H) | 7 (10, 15, 20, 25, 30, 35, 40) | 4 | 6 | 336 |

# Handling of Additional spurious emission

Given the test point analysis of A-MPR for NS_50 has already taken Additional spurious emission requirement into consideration, the test point of A-SE could reuse that of A-MPR as much as possible.

PROPOSAL 7: A-SE for NS_50 reuses the test points selected for A-MPR for NS_50.

The maximum number of test points for A-SE tests for NS_50:

Table 7: Numbers of Test Points for A-SE NS_50 (Power Class 3)

| Network Signalling label | Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number of Steps (freq. & RB alloc.) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| NS_50 | 1 | 2 (L/H) | 4 (25, 30, 35, 40) | 4 | 5 | 160 |

Table 8: Numbers of Test Points for A-SE NS_50 (Power Class 2)

| Network Signalling label | Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number of Steps (freq. & RB alloc.) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| NS_50 | 1 | 2 (L/H) | 7 (10, 15, 20, 25, 30, 35, 40) | 4 | 6 | 336 |

Table 9: Numbers of Test Points for A-SE NS_50 (Power Class 1.5)

| Network Signalling label | Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number of Steps (freq. & RB alloc.) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| NS_50 | 1 | 2 (L/H) | 7 (10, 15, 20, 25, 30, 35, 40) | 4 | 6 | 336 |

# Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_50 A-MPR test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR NS_50.

PROPOSAL 2: Select the lowest carrier centre frequency for aggregated channel bandwidth confined in 1885-1920 MHz for <=30MHz channel BWs, and confined in 1880-1920 MHz for 40MHz channel BW for each A-MPR requirement.

PROPOSAL 3: If supported, select 25MHz, 30MHz, 35MHz and 40MHz channel bandwidth for NS_50 (Power Class 3), and select 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 35MHz and 40MHz channel bandwidth for NS_50 (Power Class 2), and select 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 35MHz and 40MHz channel bandwidth for NS_50 (Power Class 1.5).

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing.

PROPOSAL 5: Test QPSK and 256QAM for DFT-s-OFDM and CP-OFDM.

PROPOSAL 6: Select as many RBs as possible close to the lower band edge.

PROPOSAL 7: A-SE for NS_50 reuses the test points selected for A-MPR for NS_50.

# References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- 3GPP TS 38.101-1, V18.7.0 (2024-09)

- 3GPP TS 38.521-1, V18.4.0 (2024-09), Table 6.1-1