3GPP TSG-RAN5 Meeting #105	R5-247355

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4790 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New testcase 7.1.3.5.11 SN gap report / Transmit operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no testcase for SN gap reporting / transmit operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding testcase for SN gap reporting / transmit operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functionality of SN gap reporting / transmit operation is not verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.11 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0535 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------------- Start of first change ---------------------------------------------------------

##### 7.1.3.5.11	SN gap report / Transmit operation

7.1.3.5.11.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { PDCP SDU has been discarded and at least one buffered PDCP SDU has COUNT value larger than the COUNT value of discarded PDCP SDU and the discarded PDCP SDU have not been submitted by RLC to lower layers to be transmitted }

then { UE submits the PDCP SN gap report for lower layers to be sent for receiving PDCP entity }

}

7.1.3.5.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clause 5.16.1, 6.1.2 and 6.2.3.5. Unless otherwise stated these are Rel-18 requirements.

[TS 38.323, clause 5.16.1]

For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when:

-	the PDCP SDU(s) are discarded as specified in clause 5.3; and

-	there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and

-	the discarded PDCP SDU(s) have not been submitted by RLC to lower layers.

If a PDCP SN gap report is triggered, the transmitting PDCP entity shall:

-	compile a PDCP SN gap report as indicated below by:

-	setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s);

-	if more than one PDCP SDUs are discarded:

-	allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU up to and including the last discarded PDCP SDU, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

-	setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded;

-	setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded;

-	submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface.

NOTE:	It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

[TS 38.323, clause 6.1.2]

The PDCP Control PDU is used to convey one of followings in addition to the PDU header:

…

-	a PDCP SN gap report.

[TS 38.323, clause 6.2.3.5]

#### 6.2.3.5	Control PDU for PDCP SN gap report

Figure 6.2.3.5-1 shows the format of the PDCP control PDU carrying the PDCP SN gap report. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.3.5-1: PDCP Control PDU format for PDCP SN gap report

7.1.3.5.11.3	Test description

7.1.3.5.11.3.1	Pre-test conditions

Pre-test conditions as in clause 7.1.3.0.

Table 7.1.3.5.11.3.1-1: Logical Channel Configuration Settings

| Parameter | DRB1 |
| --- | --- |
| discardTimer | 20ms |
| sn-GapReport-r18 | true |

7.1.3.5.11.3.2	Test procedure sequence

Table 7.1.3.5.11.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 2 | For the duration of discardTimer, the SS sends as many PDCP DATA PDUs to UE as possible. (Note 1) | <-- | PDCP DATA PDUs | - | - |
| 3 | After step 2 the SS continues responding to scheduling requests and continues allocating uplink grants. | - | - | - | - |
| 4 | Check: Does the UE send PDCP CONTROL PDU to SS containing SN gap report. | --> | PDCP CONTROL PDU | 1 | P |
| Note 1: Small PDCP SDU size will be used. |  |  |  |  |  |

7.1.3.5.11.3.3	Specific message contents

None.

--------------------------------------------------------- End of first change ---------------------------------------------------------