3GPP TSG-RAN5 Meeting #105	R5-247422

# Orlando , US, 18th Nov 2024 – 22nd Nov 2024

Agenda item:	5.3.28.9

Source: 	Apple

Title: 	Discussion on FR2 beam correspondence during initial access

Document for:	Discussion and Endorsement

# Introduction

There have been several discussions in RAN5 meetings in response to the RAN4 LS [1] for the UE Conformance testing for FR2 beam correspondence during initial access and RRC_Inactive.

The goal is to consider the beam lock function during initial access for beam correspondence test.

This paper provides Apple’s view on the beam lock functionality for beam correspondence test during initial access and RRC_Inactive.

# Discussion

## On UE Beam Lock Function (UBF) during Initial Access

The intention of this topic is to further discuss and promote the use of Beam Lock function for UE beam lock activation and deactivation during initial access.

Following factor should be considered to allow the usage of UE beam lock function for beam correspondence test during initial access and RRC_Inactive:

- As specified in TS 38.509 section 5.4 for UE Beamlock test Function (UBF) [2]:

  - The intention of UE beamlock test function is to only perform activation and deactivation of UE antenna beam pattern.

  - The objective of UE to activate beam lock is to lock antenna beam pattern once it has formed a beam towards the base station (SS).

  - The objective of UE to deactivate beam lock is to command the UE to re-tracking the beam towards the SS.

- In RAN5#103, the necessity of beam lock function and potential testability issues were discussed like unstable power measurements and sub-optimal beam selection. It was concluded in the contribution that UBF is needed in RRC_Connected mode to address potential testability issues and similar method could also be considered during initial access [3]. As defined in TS 38.508-1 section 4.6.3, NR serving cell SIB is configured with single SSB transmission for RF conformance testing in RRC_Connected mode [4].

- As specified in TS 38.521-2 Section 6.2.1.2.4.2 for UE maximum output power (MOP) - Spherical coverage [5]:

  - UE is allowed to select best Tx beam using its beam correspondence before SS sends UBF command and starts measuring the UE EIRP at each grid point.

  - SS initiates the requests to activate UE beamlock function before performing measurement and initiates the requests to deactivate after completing the measurements.

- The table 1 is depicting the impact of the beam lock function on the UE beamforming at the same non-LOS AOA in RRC_CONNECTED state. UE is beamforming using different beam types and able to activate beam lock for rough and fine beams. UBF can lock both rough and fine UE beams.

Table 1: UE Beamforming with UBF

| Beam Type | Fine | Rough | Rough | Fine | Fine | Rough | Rough |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UBF | Activate | Activate | Activate | Activate | Activate | Activate | Activate |

- In the TR 38.891 section 6.4[7], it is clearly captured, “RAN4 concludes that dedicated beam lock function for RRC_IDLE state is needed, and the details of new beam lock function design can be left to RAN5.” This shows using beam lock function was the RAN4 conclusion at the time.

Observation 1: UE beamlock test function only perform activation/deactivation as defined in TS 38.509 section 5.4 [2] and UE is allowed to handle UL power control as specified in TS 38.521-2 section 6.2.1.2 test procedure [5] for UE maximum output power- Spherical coverage[6].

Observation 2: Similar to the UE MOP - spherical coverage test, beam correspondence test is also required to verify the spatial coverage of the UE in expected directions.

Observation 3: UE is allowed to find the best beam after rotation for each grid point using beam correspondence, as specified in TS 38.521-2 section 6.2.1.2 [5] test procedure for UE maximum output power- Spherical coverage.

Observation 4: NR serving cell SIB is configured with single SSB transmission for RF conformance testing in RRC_Connected mode.

Observation 5: UE beamlock function is able to lock rough and fine UE beams as depicted in table 1.

Observation 6: RAN4 concluded that dedicated UE beam lock function is needed for RRC_IDLE state.

Based on the above findings and observations, we re-iterate the proposals from [6]:

Proposal 1: To consider the UE beam lock function (UBF) for beam correspondence test.

Proposal 2:  Agree that the beam lock function need not control the UE’s power setting, and the power ramping can be left to be handled on the UE side.

# Conclusion

The following observation and proposal were made in this contribution.

Observation 1: The intention of UE beamlock test function is to only perform activation and deactivation of UE antenna beam pattern as specified in TS 38.509 section 5.4 [2].

Observation 2: Similar to the UE MOP - spherical coverage test, beam correspondence test is also required to verify the spatial coverage of the UE in expected directions.

Observation 3: UE is allowed to find the best beam after rotation for each grid point using beam correspondence, as specified in TS 38.521-2 section 6.2.1.2 [5] test procedure for UE maximum output power- Spherical coverage.

Observation 4: NR serving cell SIB is configured with single SSB transmission for RF conformance testing in RRC_Connected mode.

Observation 5: UE beamlock function is able to lock rough and fine UE beams as depicted in table 1.

Observation 6: RAN4 concluded that dedicated UE beam lock function is needed for RRC_Idle state.

Proposal 1: To allow the UE beam lock function (UBF) to lock UE Tx beams while performing PRACH measurements to verify that the spatial coverage of the UE in expected directions is acceptable.

Proposal 2:  Agree that the beam lock function need not control the UE’s power setting, and the power ramping can be left to be handled on the UE side.

# References

- R4-2317764, “LS on testability issues of beam correspondence requirements”, RAN4#108-bis, October 9 – October 13, 2023

- TS 38.509, “Special conformance testing functions for User Equipment (UE)”, V17.5.0 (2024-03).

- R5-243823,“Discussion on FR2 beam correspondence for initial access”, RAN4#103, May 20 – May 24, 2024

- TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment. V18.4.0 (2024-10)

- TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, V18.2.0 (2024-03)

- R5-244945, “Discussion on FR2 beam correspondence during initial access”, RAN4#104, Aug 19 – Aug 23, 2024

- TR 38.891, “User Equipment (UE) Further enhancements of NR RF requirements for frequency range 2 (FR2) for Rel-18”, V18.1.0 (2024-03)