3GPP TSG-RAN5 Meeting #105	R5-246725r1

# Orlando, US, Nov.18th – 22nd 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3369 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to default configuration of 2-step RACH related message contents for RRM TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_2step_RACH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Default configurations of message contents in 38.508-1 cl.4.6.3 are used in 2-step RACH test cases, which don’t comply with 2-step RACH RMCs given in 38.133/38.533 Annex A. Hence RRM-specific message contents needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Default configurations of message contents for 2-step RACH RRM test cases are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conforamnt UE will fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency: R4-2418634 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Updated based comments received: - msgB-ResponseWindow-r16 is set to sl10; - msgA-CB-PreamblesPerSSB-PerSharedRO-r16 is set to 16; - msgA-TransformPrecoder-r16 = Not present - Table number is corrected. |  |  |  |  |  |  |  |  |

<Start of Change 1>

–	RACH-ConfigCommon

Table 7.3.1-13: RACH-ConfigCommon

| Derivation Path: TS 38.508-1 Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| oneFourth | n48 |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer | sf48 |  |  |
| rsrp-ThresholdSSB | 51 |  |  |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

–	RACH-ConfigCommonTwoStepRA

Table 7.3.1-13A: RACH-ConfigCommonTwoStepRA

| Derivation Path: Table 4.6.3-128A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommonTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| rach-ConfigGenericTwoStepRA-r16 SEQUENCE { |  |  |  |
| msgA-PRACH-ConfigurationIndex-r16 | Not present | Same value in |  |
| msgA-RO-FDM-r16 | Not present | RACH- |  |
| msgA-RO-FrequencyStart-r16 | Not present | ConfigGeneric will |  |
| msgA-ZeroCorrelationZoneConfig-r16 | Not present | be used |  |
| msgA-PreamblePowerRampingStep-r16 | Not present |  |  |
| msgA-PreambleReceivedTargetPower-r16 | Not present |  |  |
| msgB-ResponseWindow-r16 | sl10 |  |  |
| preambleTransMax-r16 | Not present |  |  |
| } |  |  |  |
| msgA-TotalNumberOfRA-Preambles-r16 | Not present | Same value in RACH- |  |
| msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB-r16 | Not present | ConfigCommon will be used |  |
| msgA-CB-PreamblesPerSSB-PerSharedRO-r16 | 16 |  |  |
| msgA-RSRP-Threshold-r16 | 0 | To ensure that UE always chooses MsgA transmission in 2-step RACH TCs |  |
| msgA-RSRP-ThresholdSSB-r16 | To be specified in test |  |  |
| ra-ContentionResolutionTimer-r16 | Not present |  |  |
| } |  |  |  |

–	RACH-ConfigGeneric

Table 7.3.1-14: RACH-ConfigGeneric

| Derivation Path: TS 38.508-1 Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 102 |  | FR1 |
|  | 190 |  | FR2 |
| msg1-FDM | one |  |  |
| msg1-FrequencyStart | 0 |  |  |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -120 |  |  |
| preambleTransMax | n6 |  |  |
|  | n200 |  | PRACH.4 FR1 or PRACH.4 FR2 |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
|  | sl1 |  | PRACH.4 FR1 or |
|  | sl40 |  | PRACH.4 FR2 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PRACH.4 FR1 | When PRACH reference configuration PRACH.4 FR1 is used in test case |
| PRACH.4 FR2 | When PRACH reference configuration PRACH.4 FR2 is used in test case |

–	MsgA-PUSCH-Config

Table 7.3.1-14A: MsgA-PUSCH-Config

| Derivation Path: Table 4.6.3-81B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-PUSCH-Config-r16 ::= SEQUENCE { |  |  |  |
| msgA-PUSCH-ResourceGroupA-r16 SEQUENCE { |  |  |  |
| msgA-PUSCH-TimeDomainOffset-r16 | 1 |  |  |
| nrofPRBs-PerMsgA-PO-r16 | 2 |  |  |
| msgA-IntraSlotFrequencyHopping-r16 | Not present |  |  |
| msgA-HoppingBits-r16 | Not present |  |  |
| msgA-DMRS-Config-r16 SEQUENCE { |  |  |  |
| msgA-DMRS-AdditionalPosition-r16 | pos1 |  |  |
| msgA-PUSCH-NrofPorts-r16 | 1 |  |  |
| } |  |  |  |
| nrofDMRS-Sequences-r16 | 1 |  |  |
| msgA-Alpha-r16 | alpha1 |  |  |
| interlaceIndexFirstPO-MsgA-PUSCH-r16 | 1 |  |  |
| nrofInterlacesPerMsgA-PO-r16 | 1 |  |  |
| } |  |  |  |
| msgA-TransformPrecoder-r16 | Not present |  |  |
| msgA-DeltaPreamble-r16 | 1 | Actual value = 2dB |  |
| } |  |  |  |

<End of Change 1>