3GPP TSG-RAN5 Meeting #105	R5-246942r1

# Orlando, United States, Nov 18-22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0897 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.3.2.1.9 2Rx PMI reporting for eType II Codebook |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| rr |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 38.101-4 introduced PMI reporting requirements for Rel-18 eType II Codebook. This CR is to introduce the new test case for PMI reporting for Rel-18 eType II Codebook for CJT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the new test case for PMI reporting for Rel-18 eType II Codebook for CJT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel-18 eType II Codebook for CJT can’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.9(new), F.1.1.3, F.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st revision: - Correct the title number mistakes. |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3.2.1.9	2Rx FDD FR1 Multiple PMI with 8 ports Enhanced Type II Codebook for CJT for both SA and NSA

6.3.2.1.9.1	Test purpose

The purpose of this test is to test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.1.9.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting eType-II codebook with refinement for multi-TRP CJT.

This test also applies to all types of EUTRA UE release 18 and forward supporting EN-DC and eType-II codebook with refinement for multi-TRP CJT.

6.3.2.1.9.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.1.9.3-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.9.3-2.

Table 6.3.2.1.9.3-1: Test parameters

| Parameter |  |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #1 |  |
|  |  |  | CORESETPoolIndex |  |  | 0 |  |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |  |
|  |  |  | Density |  |  | 3 |  |
|  |  |  | CSI-RS periodicity |  | Slots | 20 |  |
|  |  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 |  |
|  |  |  | QCL info |  |  | TCI state #0 |  |
| Duplex mode |  |  |  |  |  | FDD |  |
| Bandwidth |  |  |  |  | MHz | 10 |  |
| Subcarrier spacing |  |  |  |  | kHz | 15 |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |
| Propagation channel |  |  |  |  |  | TDLA30-10 |  |
| Antenna configuration per TRxP |  |  |  |  |  | High XP 8 x 2 (N1,N2) = (4,1) |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 4) |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |
|  | k0 |  |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |
|  | Length (L) |  |  |  |  | 12 |  |
|  | PRB bundling type |  |  |  |  | Static |  |
|  | PRB bundling size |  |  |  |  | 2 |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |
|  | RBG size |  |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000,1001 |  |
|  | TCI state |  |  |  |  | TCI State #1 |  |
|  | DMRS Type |  |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |
| TCI State #1 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  |
|  |  |  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  |
|  |  |  |  | QCL Type |  | N/A |  |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |
| Number of HARQ Processes |  |  |  |  |  | 4 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | 2 |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |
|  |  | Density (ρ) |  |  |  | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,(4) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (9) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #5 | Resource #6 |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |
|  |  | Number of CSI-RS ports (X) |  |  |  | 8 | 8 |
|  |  | CDM Type |  |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  |  | Density (ρ) |  |  |  | 1 | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 8, (4,6) | Row 8, (4,6) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (5) | (9) |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 0 |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |
| CQI-table |  |  |  |  |  | Table 1 |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |
| csi-ReportMode |  |  |  |  |  | Mode2 |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  |  |  | Not Configured |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 5 |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration |  | CodebookType |  |  |  | TypeII-CJT-r18 |  |
|  |  | CodebookMode |  |  |  | Mode 2 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) |  |
|  |  | CodebookSubsetRestriction |  |  |  | 0x FFFF |  |
|  |  | paramCombination-CJT-L-r18 |  |  |  | 7 ({4, 4}) |  |
|  |  | paramCombination-CJT-r18 |  |  |  | 4 (pν=1/4 and 1/8, β=1/2) |  |
|  |  | RI Restriction |  |  |  | 00000010 |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 14 |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |
| Measurement channel |  |  |  |  |  | R.PDSCH.1-6.3 |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs, where both TRxPs will send the same PDSCH data Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. For each TRP, use random type I SP PMI precoding independently. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-7), this reported PMI cannot be applied at the gNB downlink before slot#(n+7). Note 4: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |  |  |  |  |

Table 6.3.2.1.9.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.8 |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.3.2.1.9.

6.3.2.1.9.4	Test description

6.3.2.1.9.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in section A.3.1.7 for TE diagram and section A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 6.1.2-1 and Table 6.3.2.1.9.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On for SA or (EN-DC, DC bearer MCG and SCG, Connected without Release On) for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 6.3.2.1.9.4.3.

6.3.2.1.9.4.2	Test procedure

1.	Set the parameters of bandwidth, the propagation condition, antenna configuration and measurement channel according to Table 6.3.2.1.9.3-1 as appropriate.

2.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC with precoding matrix according to PMI report from the UE. The SS sends downlink MAC padding bits on the DL RMC. SS schedules the UL transmission with an UL RMC for CP-OFDM QPSK with 5 RBs allocated according to A.2.2.6 of TS 38.521-1 [21] to carry the PUSCH CSI feedback via PDCCH DCI format 0_1 with aperiodic CSI request triggered. No transport block is sent in parallel to the CQI feedback. Establish and  according to Annex G.3.2.

3.	Set SNR to. The SS shall transmit PDSCH with randomly selected precoding matrix from codebook (Table 5.2.2.2.1-6 in TS 38.214 [12]) every slot regardless of PMI reports from the UE. Note that each precoding matrix shall be selected in equal probabilities. The SS sends downlink MAC padding bits on the DL RMC. SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0_1 with aperiodic CSI request triggered. Measure according to Annex G.3.3.

4.	Calculate. If the ratio  which is specified in Table 6.3.2.1.9.5-1, then the test is pass. Otherwise, the test is failed.

6.3.2.1.9.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 and 5.4.

6.3.2.1.9.4.3.1	Message exceptions for SA

Table 6.3.2.1.9.4.3.1-1: CSI-MeasConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.5-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n entries | n=7 |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-Resource[5] | NZP-CSI-RS-Resource for CSI Acquisition | entry 5 CSI-RS resource 5 |  |
| NZP-CSI-RS-Resource[6] | NZP-CSI-RS-Resource for CSI Acquisition | entry 6 CSI-RS resource 6 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | n entries | n=2 |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1, Table 5.4.2.0-12 in TS 38.508-1 [6] |  |
| NZP-CSI-RS-ResourceSet[2] | NZP-CSI-RS-ResourceSet for CSI Acquisition | entry 2, Table 6.3.2.1.9.4.3.1-2 |  |
| } |  |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { |  | 1 entry |  |
| CSI-ReportConfig[1] | CSI-ReportConfig | Table 6.3.2.1.9.4.3.1-9 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-2: NZP-CSI-RS-ResourceSet for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | 1 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | 4 | entry 1 CSI-RS resource 5 |  |
| NZP-CSI-RS-ResourceId[2] | 5 | entry 2 CSI-RS resource 6 |  |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | 0 |  |  |
| trs-Info | Not present |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-3: NZP-CSI-RS-Resource for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 4 | CSI-RS resource 5 |  |
|  | 5 | CSI-RS resource 6 |  |
| resourceMapping | CSI-RS-ResourceMapping for CSI Acquistion | Table 6.3.2.1.9.4.3.1-4 |  |
| powerControlOffset | 0 |  |  |
| periodicityAndOffset | Not present |  |  |
| qcl-InfoPeriodicCSI-RS | Not present |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-4: CSI-RS-ResourceMapping for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 001100 | k0 = 4, k1 = 6, row8 |  |
| } |  |  |  |
| nrofPorts | p8 |  |  |
| firstOFDMSymbolInTimeDomain | 5 | l0 = 5 for CSI-RS resource 5 |  |
|  | 9 | l0 = 9 for CSI-RS resource 6 |  |
| cdm-Type | cdm4-FD2-TD2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-4A: CSI-RS-ResourceMapping for TRS

Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-9

Table 6.3.2.1.9.4.3.1-5: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
| ZP-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { |  | 1 entry |  |
| ZP-CSI-RS-Resource[1] | ZP-CSI-RS-Resource | Table 6.3.2.1.9.4.3.1-6 |  |
| } |  |  |  |
| p-ZP-CSI-RS-ResourceSet | p-ZP-CSI-RS-ResourceSet | Table 5.4.2.5-5A in TS 38.508-1 [6] |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-6: ZP-CSI-RS-Resource

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-204 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| zp-CSI-RS-ResourceId | 0 |  |  |
| resourceMapping | CSI-RS-ResourceMapping for ZP CSI-RS | Table 6.3.2.1.9.4.3.1-7 |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for ZP CSI-RS | Table 6.3.2.1.9.4.3.1-8 |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-7: CSI-RS-ResourceMapping for ZP CSI-RS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 | k0 = 4, row5 |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 |  |  |
| firstOFDMSymbolInTimeDomain2 | Not present |  |  |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-8: CSI-ResourcePeriodicityAndOffset for ZP CSI-RS

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots5 | 1 |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-9: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.5-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| resourcesForChannelMeasurement | 1 | CSI-ResourceConfigId for CSI Acquisition |  |
| csi-IM-ResourcesForInterference | 2 | CSI-ResourceConfigId for CSI-IM |  |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList SEQUENCE { | 2 entries |  |  |
| reportSlotOffsetList[1] | 5 | entry 1 |  |
| reportSlotOffsetList[2] | 5 | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| pmi-FormatIndicator | Not present |  |  |
| } |  |  |  |
| csi-ReportMode-r17 | mode2 |  |  |
| codebookConfig-r18 | CodebookConfig-r18 | Table 6.3.2.1.9.4.3.1-10 |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-10: CodebookConfig-r18

| Derivation Path: TS 38.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig-r18 ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type2 CHOICE { |  |  |  |
| typeII-CJT-r18 SEQUENCE { |  |  |  |
| n1-n2-codebookSubsetRestrictionList-r18 | N1-N2-CBSR-List-r18 | Table 6.3.2.1.9.4.3.1-11 |  |
| paramCombination-CJT-r18 | 4 |  |  |
| paramCombination-CJT-L-r18 SEQUENCE { |  | 1 entry |  |
|  | 7 |  |  |
| } |  |  |  |
| restrictedCMR-Selection-r18 | enable |  |  |
| valueOfO3-r18 | Not present |  |  |
| numberOfPMI-SubbandsPerCQI-Subband-r18 | 1 |  |  |
| typeII-RI-Restriction-r18 | 0010 |  |  |
| codebookMode-r18 | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-11: N1-N2-CBSR-List-r18

| Derivation Path: TS 38.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| N1-N2-CBSR-List-r18 ::= CHOICE { |  |  |  |
| four-one-r18 CHOICE { |  |  |  |
| cbsr-list-r18 SEQUENCE { |  | 2 entries |  |
|  | 0x FFFF | entry 1 |  |
|  | 0x FFFF | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-12: Physical layer parameters for DCI format 1_1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.5-16 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PDSCH-to-HARQ_feedback timing indicator | K1 = 2 | “010” |  |
| Antenna port(s) | DMRS port 0 and DMRS port 1 as per Table 7.3.1.2.2-1 in TS 38.212 (dmrs-Type = DMRS type 1 and maxLength = len1 as per Table 4.6.3-50 in 38.508-1) | “0010” |  |

Table 6.3.2.1.9.4.3.1-13: Physical layer parameters for DCI format 0_1

| Derivation Path: TS 38.508-1 [6], Table 4.3.6.1.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| CSI request | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |

6.3.2.1.9.4.3.2	Message exceptions for NSA

Same as in clause 6.3.2.1.9.4.3.1.

6.3.2.1.9.5	Test requirement

Table 6.3.2.1.9.5-1: Test requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.79 |

<Unchanged Text Skipped>

F.1.1.3	Measurement of Channel State Information reporting

This clause defines the maximum test system uncertainty for channel state information reporting requirements. The maximum test system uncertainty allowed for the measurement uncertainty contributors are defined in Table F.1.1.3-1.

Table F.1.1.3-1: Maximum measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Channel BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWconfig | dB | Same as in table F.1.1.2-1 |  |
| Signal to noise ratio uncertainty | dB | Same as in table F.1.1.2-1 |  |
| Signal to noise ratio variation | dB | Same as in table F.1.1.2-1 |  |
| Fading profile power uncertainty for 1Tx | dB | Same as in table F.1.1.2-1 |  |
| Fading profile power uncertainty for 2Tx | dB | Same as in table F.1.1.2-1 |  |
| Fading profile power uncertainty for 4Tx | dB | Same as in table F.1.1.2-1 |  |

The maximum test system uncertainty for test cases defined in section 6 is defined in Table F.1.1.3-2.

Table F.1.1.3-2: Maximum test system uncertainty for FR1 channel state information reporting test cases

| Subclause |  | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 6.2.1.1.1.1 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap |  | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |
| 6.2.1.1.1.2 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for eRedCap |  | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |
| 6.2.1.1.2.1 1Rx FDD FR1 periodic wideband CQI reporting under fading conditions for RedCap |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |
| 6.2.1.2.1.1 1Rx TDD FR1 periodic CQI reporting under AWGN conditions for RedCap |  | Same as 6.2.2.2.1.1 |  | Same as 6.2.2.2.1.1 |  |
| 6.2.1.2.2.1 1Rx TDD FR1 periodic wideband CQI reporting under fading conditions for RedCap |  | Same as 6.2.1.1.2.1 |  | Same as 6.2.1.1.2.1 |  |
| 6.2.2.1.1.1 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA |  | +/- 0.3 dB |  | Overall system uncertainty for AWGN conditions comprises: Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |
| 6.2.2.1.1.2 2Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA |  | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |
| 6.2.2.1.1.3 2Rx FDD FR1 periodic CQI reporting with Table 4 under AWGN conditions for both SA and NSA |  | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |
| 6.2.2.1.1.4 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap |  | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |
| <<Unchanged Text Skipped>> |  |  |  |  |  |
| 6.3.2.1.4 2Rx FDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |
| 6.3.2.1.5 2Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |
| 6.3.2.1.7 2Rx FDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |
| 6.3.2.1.9 2Rx FDD FR1 Multiple PMI with 8 ports Enhanced Type II Codebook for CJT for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |
| 6.3.2.1.6 2Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA |  | Same as 6.2.2.1.2.1 |  | Same as 6.2.2.1.2.1 |  |
| 6.2.3.2.1.2 4Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA |  | Same as 6.2.2.1.1.1 |  | Same as 6.2.2.1.1.1 |  |
| <<Unchanged Text Skipped>> |  |  |  |  |  |

<Unchanged Text Skipped>

F.1.3.3	Measurement of Channel State Information reporting

The derivation of the test requirements for the test cases in section 6 is defined in Table F.1.3.3-1.

Table F.1.3.3-1: Derivation of Test Requirements (FR1 channel state information reporting tests)

| Test | Minimum Requirement in TS 38.101-4 | Test Tolerance (TT) | Test Requirement in TS 38.521-4 |
| --- | --- | --- | --- |
| 6.2.1.1.1.1 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.1.1.1.2 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for eRedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.1.1.2.1 1Rx FDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.1.2.1.1 1Rx TDD FR1 periodic CQI reporting under AWGN conditions for RedCap | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.1.2.2.1 1Rx TDD FR1 periodic wideband CQI reporting under fading conditions for RedCap | SNRs as specified  20% 1.05 BLER 0.02 | SNR 0 dB  0% 0.01 BLER 0 | SNR unchanged  unchanged  1.04 BLER limit unchanged |
| 6.2.2.1.1.1 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| 6.2.2.1.1.2 2Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | SNRs as specified Limits as in the Test Procedure | No test tolerances applied | SNR unchanged |
| <<Unchanged Text Skipped>> |  |  |  |
| 6.3.2.1.6 2Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | SNRs as specified  2.2 | SNR 0 dB  0.01 | SNR unchanged  2.19 |
| 6.3.2.1.7 Single PMI with 8 ports TypeI-SinglePanel Codebook for Single-DCI based transmission scheme | SNRs as specified  1.6 | SNR 0 dB  0.01 | SNR unchanged  1.59 |
| 6.3.2.1.9 2Rx FDD FR1 Multiple PMI with 8 ports Enhanced Type II Codebook for CJT for both SA and NSA | SNRs as specified  1.8 | SNR 0 dB  0.01 | SNR unchanged  1.79 |
| 6.3.2.2.1 2Rx TDD FR1 Single PMI with 4Tx TypeI - SinglePanel codebook for both SA and NSA | SNRs as specified  1.30 | SNR 0 dB  0.01 | SNR unchanged  1.29 |
| 6.3.2.2.2 2Rx TDD FR1 Single PMI with 8Tx TypeI - SinglePanel codebook for both SA and NSA | SNRs as specified  1.50 | SNR 0 dB  0.01 | SNR unchanged  1.49 |
| <<Unchanged Text Skipped>> |  |  |  |

<End of Changes>