3GPP TSG-RAN5 Meeting #105			R5-247141r1

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

Agenda item:	5.5.1.13

Source: 	Qualcomm Inc

Title: 	Simulation results to derive min test time for nb-ntn npdcch and npdsch demod test cases

Document for:	Discussion and Endorsement

## 1.	Introduction

TS 36.521-4 [4] introduced NTN-TDL channel models and legacy TDL models for NPDSCH and NPDCCH Demodulation test cases with satellite access based on the requirements defined in [5]. Some of the NPDSCH simulation results were provided in [1] and the average of company A and company B simulation results were used to come up with the minimum test time for those NPDSCH test cases in [7].

This paper provides the simulation results for remaining NPDSCH test case and all the NPDCCH test cases.

## 2.	Discussion

Pasting below relevant section of G.3.5 of 36.521-4 [4] which provides the procedure to come up with the minimum test time for NPDSCH and NPDCCH.

### G.3.5.1	Minimum Test Time procedure for PDSCH scenarios with 30% or 70% Throughput requirement

1.	Run the PDSCH simulations for an extended period of time at the SNR of interest for multiple seeds.

2.	The minimum test time is determined by the time it took the throughput curve to settle within +/- 2% of the target throughput value across all seeds.

3. 	If the throughput does not settle within the +/- 2% throughput limit within a reasonable amount of time for any seed, expand the threshold (for example, +/- 3.5%) of the target throughput value.

4.	To account for the additional 1.5% throughput variation, determine from the Throughput vs SNR curve, what is the SNR delta corresponding to this 1.5 % throughput increase.

5.	That SNR value is the added SNR uncertainty due to finite test time.

### G.3.5.2	Minimum Test Time procedure for PDSCH and PDCCH scenarios with 1% BLER requirement

1.	Run the PDSCH simulations for an extended period of time at the SNR of interest for multiple seeds.

2.	The minimum test time is determined by the time it took the residual BLER curve to converge within +/- 10% of the target 1% residual BLER (i.e. within 0.9% and 1.1% BLER) across all seeds.

3. 	If the residual BLER does not converge within the +/- 10% of the 1% target limit within a reasonable amount of time for any seed, increase SNR in steps of 0.1 dB and rerun the simulations within the same target BLER limit

4.	The additional delta SNR required to meet the residual BLER convergence limit is the added SNR uncertainty due to finite test time.

5.	This SNR uncertainty due to finite test time would be one of the MU term in the overall MU calculation.

NPDSCH Simulations for below mentioned NB-NTN NPDSCH demod test cases in [4] were run.

Table 8.3.1.1.3.3-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | R.NB.7 FDD | 1 | EPA5 | 1 | 70% | 9.4 | NB2 |

NPDCCH Simulations for below mentioned NB-NTN NPDCCH demod test cases in [4] were run.

Table 8.3.1.2.1.3-1: Minimum performance NPDCCH

| Test number | Deployment mode | Repetition number (R) | Carrier Type | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |  |
| 1 | Stand-alone | 128 | Anchor | R.NB.3 FDD | EPA5 | 1 | 1 | -4.9 | NB1, NB2 |
| 2 (Note 1) | Stand-alone | 1024 | Non-anchor | R.NB.3 FDD | ETU1 | 1 | 1 | -11.4 | NB1, NB2 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. |  |  |  |  |  |  |  |  |  |

Figure1: NPDSCH Throughput Simulations for Test1 of TC 8.3.1.1.3.3-2 (EPA5) at 70% target throughput SNR, 30 seeds

As seen in the simulation results, for +/- 5% convergence criteria, the minimum test needed is 75 seconds. As per our simulation for throughput vs SNR run at the 70% SNR, 0.5 dB SNR change results in 5% change in throughput.

Hence we propose to add 0.5 dB SNR uncertainty due to finite test time.

Proposal 1: Use 0.5 dB SNR uncertainty due to finite test time for TC 8.3.1.1.3 test1

For the NPDCCH scenarios, pasting below the current minimum test time for test1 and test2.

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) | MNSF (Min No Sub Frames, mandatory) |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | HD-FDD |
| 8.3.1.2.1: Test 1 | R.NB.3 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 36166 | 15430827 | 15431000 |
| 8.3.1.2.1: Test 2 | R.NB.3 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 36166 | 55550976 | 55551000 |

For Test1, the MNSF translates to 4.29 hours of test time and 15.43 hours for test2. Such large test time is impractical.

Observation 1:  Current minimum test time for NPDCCH runs into several hours which is impractical.

NPDCCH simulation results <pending>

Summarizing the convergence time in table 2-1 for the NPDSCH scenarios

Table 2-1: Convergence time

| Scenario | REP | Rmax | G | Rmax*G | Spec SNR | Convergence time for +/- 5% threshold | Scaling factor for QC sim to acount for RTT | Updated QC convergence time for +/- 5% threshold | QC MNAS (# of TB) | Min Gap between NPDCCH with RTT | Actual GAP between NPDCCH with RTT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.3.1.1.3_Test1_EPA5_GSO config_non-anchor-NB2 | 4 | 8 | 1.5 | 12 | 9.4 | 75 | 8 | 600 | 2083 | 283 | 288 |

## 3.	Conclusion

Proposal 1: Use 0.5 dB SNR uncertainty due to finite test time for TC 8.3.1.1.3 test1

Observation 1:  Current minimum test time for NPDCCH runs into several hours which is impractical.

Proposal 2:  Use the convergence time as specified in table 2-1 for the NPDSCH scenarios.

## 4.		References

[1]	R5-245291, Simulation results to derive min test time for nb-ntn demod test cases, Qualcoomm

[2]	R5-191250, Discussion paper on statistical tput calculation for demod test cases, Qualcomm

[3]	R5-194687, Discussion on minimum test time for PDSCH and PDCCH with more simulation results, Qualcomm

[4]	TS 36.521-4 v18.3.0

[5]	TS 36.102 v18.7.0

[6]	R5-242427, Discussion on minimum test time simulation methods for NB-IoT NTN, Mediatek

[7]	R5-245292, min test time update for nb-ntn demod test cases, Qualcomm, Mediatek India Technology Pvt.