3GPP TSG-RAN5 Meeting #105	R5-247200r1

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3391 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 4.9.41 Test procedure for 5G ProSe Layer-2 U2N Relay initial access / Relay UE side |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 4.9.41 Test procedure for 5G ProSe Layer-2 U2N Relay initial access / Relay UE side is incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding AS security and RRCReconfiguration procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.41 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Deleted the condition NOT IMS_PDU in Table 4.9.41.3-6. |  |  |  |  |  |  |  |  |

<Start of Changed Section>

4.9.41	Test procedure for 5G ProSe Layer-2 U2N Relay initial access / Relay UE side

4.9.41.1	Scope

The purpose of this procedure is to test relay UE’s behaviour during 5G ProSe Layer-2 U2N Relay Initial Access procedure.

4.9.41.2	Procedure description

4.9.41.2.1	Initial conditions

The UE is in state 3N-A.

4.9.41.2.2	Procedure sequence

Table 4.9.41.2.2-1: Procedure for 5G ProSe Layer-2 U2N Relay initial access / Relay UE side

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Procedure for The UE initiated 5G ProSe U2N Relay Discovery, procedure specified in subclause 4.9.43 is performed. |  |  |  |  |
| - | EXCEPTION: Steps 2a1 and 2b1 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 2a1 | IF Cast type = Unicast AND UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.39 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. | - | - | - | - |
| 2b1 | ELSE IF Cast type = Unicast AND NR-SS-UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.40 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. | - | - | - | - |
| 3 | The NR-SS-UE1 transmits an RRCSetupRequest message via Relay UE. | <-- | PC5 Relay RLC Channel: RRCSetupRequest | - | - |
| 4 | The UE transmits a SidelinkUEInformationNR message to SS-NW. | --> | NR RRC: SidelinkUEInformationNR | - | - |
| 5 | The SS-NW transmits an RRCReconfiguration message. | <-- | NR RRC: RRCReconfiguration | - | - |
| 6 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - |  |
| 7 | The UE relays the RRCSetupRequest message received at step 3. | --> | Uu Relay RLC Channel: RRCSetupRequest | - | - |
| 8 | The SS-NW transmits an RRCSetup message to L2 U2N Remote UE via Relay UE. | <-- | Uu Relay RLC Channel: RRCSetup | - | - |
| 9 | The UE relays the RRCSetup message received at step 8 to NR-SS-UE1. | --> | PC5 Relay RLC Channel: RRCSetup | - | - |
| 10 | The NR-SS-UE1 transmits an RRCSetupComplete message and a REGISTRATION REQUEST message via Relay UE. | <-- | PC5 Relay RLC Channel: RRCSetupComplete | - | - |
| 11 | The UE relays the RRCSetupComplete message received at step 10 and a REGISTRATION REQUEST message to SS-NW. | --> | Uu Relay RLC Channel: RRCSetupComplete | - | - |
| 12 | The NR-SS-UE1 transmits an RemoteUEInformationSidelink message with sl-PagingInfo-RemoteUE set to release. | <-- | PC5 RRC: RemoteUEInformationSidelink | - | - |
| 13 | The SS-NW transmits an SecurityModeCommand message to L2 U2N Remote UE via Relay UE. | <-- | Uu Relay RLC Channel: SecurityModeCommand | - | - |
| 14 | The UE relays the SecurityModeCommand message to NR-SS-UE1. | --> | PC5 Relay RLC Channel: SecurityModeCommand | - | - |
| 15 | The NR-SS-UE1 transmits an SecurityModeComplete message via Relay UE. | <-- | PC5 Relay RLC Channel: SecurityModeComplete | - | - |
| 16 | The UE relays the SecurityModeComplete message to SS-NW. | --> | Uu Relay RLC Channel: SecurityModeComplete | - | - |
| 17 | The SS-NW transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message to L2 U2N Remote UE via Relay UE. | <-- | Uu Relay RLC Channel: DLInformationTransfer |  |  |
| 18 | The UE relays the DLInformationTransfer message to NR-SS-UE1. | --> | PC5 Relay RLC Channel: DLInformationTransfer |  |  |
| 19 | The NR-SS-UE1 transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message via Relay UE. | <-- | PC5 Relay RLC Channel: ULInformationTransfer |  |  |
| 20 | The UE relays the ULInformationTransfer message to SS-NW. | --> | Uu Relay RLC Channel: ULInformationTransfer |  |  |
| 21 | The SS-NW transmits an RRCReconfiguration message. | <-- | NR RRC: RRCReconfiguration | - | - |
| 22 | The UE transmits an RRCReconfigurationComplete message to SS-NW. | --> | NR RRC: RRCReconfigurationComplete | - |  |
| 23 | The SS-NW transmits an RRCReconfiguration message to L2 U2N Remote UE via Relay UE. | <-- | NR RRC: RRCReconfiguration | - | - |
| 24 | The UE relays the RRCReconfiguration message to NR-SS-UE1. | --> | PC5 Relay RLC Channel: RRCReconfiguration | - | - |
| 25 | The NR-SS-UE1 transmits an RRCReconfigurationComplete message via Relay UE. | <-- | PC5 Relay RLC Channel: RRCReconfigurationComplete | - | - |
| 26 | The UE relays the RRCReconfigurationComplete message to SS-NW. | --> | Uu Relay RLC Channel: RRCReconfigurationComplete | - | - |

4.9.41.3	Specific message contents

Table 4.9.41.3-1: RRCSetupRequest (Steps 3 and 7, Table 4.9.41.2.2-1)

Derivation Path: Table 4.6.1-23 with condition L2RemoteUE

Table 4.9.41.3-2: SidelinkUEInformationNR (Step 4, Table 4.9.41.2.2-1)

Derivation Path: Table 4.6.1-28A with condition L2RelayUE

Table 4.9.41.3-3: RRCReconfiguration (Step 5, Table 4.9.41.2.2-1)

Derivation path: Table 4.6.1-13 with condition L2RelayUE and RELAY

Table 4.9.41.3-4: RemoteUEInformationSidelink (Step 12, Table 4.9.41.2.2-1)

Derivation Path: Table 4.6.1A-2A with condition Paging_Release

Table 4.9.41.3-5: RRCReconfiguration (Step 21, Table 4.9.41.2.2-1)

| Derivation Path: Table 4.6.1-13 and condition L2RelayUE and RELAY |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition RELAY, SRB2 and DRB1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sl-L2RelayUE-Config-r17 CHOICE { |  |  |  |  |
| setup |  | SL-L2RelayUE-Config with condition SRB2 and DRB1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.41.3-6: RRCReconfiguration (Step 23, Table 4.9.41.2.2-1)

| Derivation path: Table 4.6.1-13 with condition NR and L2RemoteUE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB2 and DRB1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with condition SRB2_DRB1 |  |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} | DedicatedNAS-Message |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Changed Section>