3GPP TSG-RAN5 Meeting #105	R5-247468r1

# Orlando, USA, November 18 - 22 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3639 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR SA FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used test case 11.5.4.3 including Test Tolerance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR SA FR1 SSB based L1-RSRP measurement on SCC when DRX is not used 11.5.4.3 was incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR SA FR1 SSB based L1-RSRP measurement on SCC when DRX is not used 11.5.4.3 is corrected: - Test Tolerance applied - Message content added - Applicability updated - Editor’s notes removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will remain incomplete and WP cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR 38.903 CR 0929 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: test applicability corrected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/// Start of changes

#### 11.5.4.3	NR SA FR1 SSB based L1-RSRP measurement on SCC when DRX is not used

11.5.4.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement on SCC under CCA when DRX is not used.

11.5.4.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting standalone NR-U and supporting RRM measurements with a shared spectrum NR carrier.

11.5.4.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 11.5.4.0.

The normative reference for this requirement is TS 38.133 [6] clause A.11.5.4.3.

11.5.4.3.4	Test description

There are two cells in the test, the FR1 PCell (Cell 1) and FR1 SCell (Cell 2). Both Cell 1 and Cell 2 operate on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model. The  test consists of two consecutive time periods, with durations T1 and T2, respectively.

11.5.4.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.5.4.3.4.1-1.

Table 11.5.4.3.4.1-1: Supported test configurations

| Test ID | Description |
| --- | --- |
| 11.5.4.3-1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 11.5.4.3.4.1-2.

Table 11.5.4.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | NA |  |  |

1.	The general test parameter settings are set up according to Table 11.5.4.3.4.1-3.

2.	Message contents are defined in clause 11.5.4.3.4.3.

Table 11.5.4.3.4.1-3: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| Active PCell | 1 |  | Cell 1 |
| Active SCell | 1 |  | Cell 2 |
| RF Channel Number | 1 |  | 1: Cell 1 2: Cell 2 |
| DL CCA model | 1 |  | As specifieed in A.3.20.2.1 |
| UL CCA model | 1 |  | As specified in A.3.20.2.2 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.1.1 CCA |
| BWchannel | 1 | MHz | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 CCA |
| SSB configuration | 1 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration | 1 |  | DBT.1 |
| TRS Configuration | 1 |  | TRS.1.2 TDD |
| DRX configuration | 1 |  | Off |
| reportConfigType | 1 |  | periodic |
| reportQuantity | 1 |  | ssb-Index-RSRP |
| Number of reported RS | 1 |  | 2 |
| L1-RSRP reporting period | 1 | slot | 80 |
| T1 | 1 | s | 5 |
| T2 | 1 | s | 1 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1 |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. |  |  |  |

11.5.4.3.4.2	Test procedure

Same test procedure as in subclause 11.5.4.1.4.2 with tables 11.5.4.1.4.1-2 and 11.5.4.1.5-1 replaced by tables 11.5.4.3.4.1-2 and 11.5.4.3.5-1.

11.5.4.3.4.3	Message contents

Same message content as in subclause11.5.4.1.4.3.

11.5.4.3.5	Test requirements

Table 11.5.4.3.5-1 defines the primary level settings including test tolerances for SSB based L1-RSRP measurement on SCC in non-DRX.

Table 11.5.4.3.5-1: SSB specific test parameters for NR SA FR1 SSB based L1-RSRP measurement on SCC when DRX is not used

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| DL CCA Probability PCCA_DL Note 4,6 | 1 |  | 0.9375 | 0.9375 | 0.9375 | 0.9375 |
| DL CCA Probability PCCA_DL Note 4.7 | 1 |  | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 |
| UL CCA probability PCCA_UL | 1 |  | 1.0 | 1.0 | 1.0 | 1.0 |
| Note2 | 1 | dBm/15kHz | -94.65s |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -91.65 |  |  |  |
|  | 1 | dB | 0 | 0 | -Infinity | 3.5 |
| SSB RSRP Note3 | 1 | dBm/SSB SCS | -91.65 | -91.65 | -Infinity | -88.15 |
| Io Note3 | 1 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.49 |
|  | 1 | dB | 0 | 0 | -Infinity | 3.5 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. Note 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement and relative accuracy requirement in TS 38.133 [6] in Table 11.5.4.3.5-2 and 11.5.4.3.5-3.

Table 11.5.4.3.5-2: L1-RSRP absolute accuracy requirements for
the reported values for all test configurations

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 58 |
| Highest reported value (SSB#1) | - | 78 |

Table 11.5.4.3.5-3: L1-RSRP relative accuracy requirements for
the reported values for all test configurations

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | - | 0 |
| Highest DIFF RSRP reported (SSB#0) | - | 3 |

The UE shall send L1-RSRP report of both SSB0 and SSB1 in Cell 2.

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

/// End of changes