3GPP TSG-RAN5 Meeting #105	R5-247607

# Orlando, United States, 18th - 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1542 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections and clarifications for use of conditions in test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - At RAN5#104 R5-244439 has been presented and discussed proposing changes to remove inconsistencies in the use of conditions for IMS default message contents (AP#103.02). But there have been no CRs in concrete for 34.229-1/5 yet. - For some specific contents conditions have been missing or conditions have been invalid. - Editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Implementation of proposals 2.1.1, 2.1.2, 2.1.3 of R5-244439. - Conditions added or corrected: - When condition A2 is specified for A.1.1, condition A17 needs to be specified too: test case 9.2 REGISTER (Step 5), test case 11.2 REGISTER (Step 3, 5), test case 19.4.5 REGISTER (Step 16), test case 19.4.6 REGISTER (Step 1), test case 19.4.7 REGISTER (Step 2A) - Test case 12.26: A5 (re-INVITE) added to re-INVITE - Test case 12.26, 12.27, 12.27a, 17.1, 17.2, 22.7, G.17.1, G.17.2: A23 (200 OK for re-INVITE) added to 200 OK for INVITE; A5 (ACK for re-INVITE) added to ACK. - Test case 15.18: A3 (SS sends NOTIFY to indicate that UE is now subscribed to the conference event package) added to NOTIFY at step 7 as otherwise Subscription-State and Content-Type are not specified - Test case 15.21a, 15.21c, 19.3.1: A3 (Response sent reliably) added to 180 Ringing - Test case 15.21c, 17.1, G.17.1: A11 (creating video) added to INVITE - Test case 17.1, 17.2, G.17.1, G.17.2: A15 (183 sent for re-INVITE) added to 183 Session Progress - Test case 17.2, G.17.2: A8 (creating video) added to INVITE - Test case 18.2, 18.2a: A5 “Any response sent by the UE within a dialog” removed from 200 OK as SIP MESSAGE does not create a dialog. - Test case 19.1.6: Condition A3 (GIBA) added to REGISTER (Step 15), A2 corrected to A3 (Response for an emergency registration) for 200 OK for REGISTER (Step 16) - Test case 19.4.6: A1 (Initial unprotected REGISTER) added to REGISTER (Step 13) - Test case 21.4: A12 added to 200 OK at step 7 - Test case 21.4, 21.5: Specific message content added for INFO for MO - Test case 21.6: A2 added to INFO for MO (Step 10) - Test case 21.13, 21.14: A1 added to 486 Busy Here - Test case 21.15, 21.16: A1 added to 600 Busy Everywhere - Test case 21.17, 21.18: A1 added to 603 Decline - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10.4, 8.11.4, 8.12.4, 8.16.4, 9.2.4, 11.2.4, 12.13b.4, 12.13c.4, 12.26.4, 12.27.4, 12.27a.4, 15.4a.4, 15.8.4, 15.18.4, 15.21a.4, 15.21c.4, 15.25.4, 15.30.4, 16.2.4, 17.1.4, 17.2.4, 18.2.4, 18.2a.4, 19.1.6.4, 19.4.5.4, 19.4.6.4, 19.5.6.4, 19.5.7.4, 21.1.4, 21.2.4, 21.4.4, 21.5.4, 21.6.4, 21.13.4, 21.14.4, 21.15.4, 21.16.4, 21.17.4, 21.18.4, 22.1.4, 22.2.4, 22.3.4, 22.4.4, 22.5.4, 22.6.4, 22.7.4, 22.8.4, G.17.1.4, G.17.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-246634. |  |  |  |  |  |  |  |  |

<Start of modification>

### 8.10.4	Method of test

Initial conditions

UE contains ISIM and USIM applications or only USIM application on UICC. UE is not registered to IMS services, but has an active PDP context and has discovered the SS as P-CSCF by executing the generic test procedure in Annex C.2a up to step 3.

SS is configured with the IMSI, the home domain name, public and private user identities and the currently assigned IP address. SS is listening to SIP default port 5060 for both UDP and TCP protocols.

Test procedure

1)	The UE initiates IMS registration indicating support of GIBA. SS waits for the UE to send an initial REGISTER request.

2)	The SS responds to the REGISTER request with a 200 OK response,

3)	The SS waits for the UE to send a SUBSCRIBE request.

4)	The SS responds to the SUBSCRIBE request with a 200 OK response.

5)	The SS sends a valid NOTIFY request for the subscribed registration event package.

6)	The SS waits for the UE to respond to the NOTIFY with a 200 OK response.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | REGISTER | The UE sends initial registration for IMS services indicating support for GIBA procedure by not including an Authorization header field. |
| 2 |  |  | 200 OK | The SS responds with 200 OK. |
| 3 |  |  | SUBSCRIBE | The UE subscribes to its registration event package. |
| 4 |  |  | 200 OK | The SS responds with 200 OK. |
| 5 |  |  | NOTIFY | The SS sends initial NOTIFY for registration event package, containing full registration state information for the registered public user identity in the XML body |
| 6 |  |  | 200 OK | The UE responds with 200 OK. |

NOTE:	The default message contents in annex A are used.

Specific Message Contents

REGISTER (Step 1)

Use the default message “REGISTER” in annex A.1.1 with condition A3 "REGISTER for the case UE supports GIBA".

200 OK for REGISTER (Step 2)

Use the default message “200 OK for REGISTER” in annex A.1.3.

SUBSCRIBE (Step 3)

Use the default message “SUBSCRIBE for reg-event package” in annex A.1.4 with condition A2 “GIBA”.

200 OK for SUBSCRIBE (Step 4)

Use the default message “200 OK for SUBSCRIBE” in annex A.1.5 with condition A2 “GIBA”.

NOTIFY (Step 5)

Use the default message “NOTIFY for reg-event package” in annex A.1.6 with condition A2 “GIBA”.

200 OK for NOTIFY (Step 6)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

<End of modification>

<Start of modification>

### 8.11.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE is not registered to IMS services, but has an active PDP context and has discovered the SS as P-CSCF by executing the generic test procedure in Annex C.2 up to step 3. The UE has no knowledge about the IMS network capabilities.

SS is configured with the IMSI, the home domain name, public and private user identities and the currently assigned IP address. SS is listening to SIP default port 5060 for both UDP and TCP protocols.

Test procedure

1)	IMS registration is initiated on the UE. SS waits for the UE to send an initial REGISTER request.

2)	The SS responds to the REGISTER request with a 420 Bad Extension response,

3)	The UE initiates IMS registration indicating support of GIBA. SS waits for the UE to send an initial REGISTER request.

4)	The SS responds to the REGISTER request with valid 200 OK response,

5)	The SS waits for the UE to send a SUBSCRIBE request.

6)	The SS responds to the SUBSCRIBE request with a valid 200 OK response.

7)	The SS sends a NOTIFY request for the subscribed registration event package.

8)	The SS waits for the UE to respond to the NOTIFY with a 200 OK response.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | REGISTER | UE sends initial registration for IMS services. |
| 2 |  |  | 420 Bad Extension | The SS responds with a failure, since the option tag sec-agree in the Proxy-Require header field is not supported. |
| 3 |  |  | REGISTER | The UE sends initial registration for IMS services indicating support for GIBA procedure by not including an Authorization header field. |
| 4 |  |  | 200 OK | The SS responds with 200 OK. |
| 5 |  |  | SUBSCRIBE | The UE subscribes to its registration event package. |
| 6 |  |  | 200 OK | The SS responds with 200 OK. |
| 7 |  |  | NOTIFY | The SS sends initial NOTIFY for registration event package, containing full registration state information for the registered public user identity in the XML body |
| 8 |  |  | 200 OK | The UE responds with 200 OK. |

NOTE:	The default message contents in annex A are used.

Specific Message Contents

REGISTER (Step 1)

Use the default message “REGISTER” in annex A.1.1 with condition A1 "Initial unprotected REGISTER".

420 Bad Extension (Step 2)

Use the default message “420 Bad Extension for REGISTER” in annex A.1.8

REGISTER (Step 3)

Use the default message “REGISTER” in annex A.1.1 with condition A3 "REGISTER for the case UE supports GIBA".

200 OK for REGISTER (Step 4)

Use the default message “200 OK for REGISTER” in annex A.1.3

SUBSCRIBE (Step 5)

Use the default message “SUBSCRIBE for reg-event package” in annex A.1.4 with condition A2 “GIBA”.

200 OK for SUBSCRIBE (Step 6)

Use the default message “200 OK for SUBSCRIBE” in annex A.1.5 with condition A2 “GIBA”

NOTIFY (Step 7)

Use the default message “NOTIFY for reg-event package” in annex A.1.6 with condition A2 “GIBA”

200 OK for NOTIFY (Step 8)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1

<End of modification>

<Start of modification>

### 8.12.4	Method of test

Initial conditions

UE contains ISIM and USIM applications or only USIM application on UICC. UE is not registered to IMS services. Execute the generic test procedure in annex C.2a up to step 3.

SS is configured with the IMSI, the home domain name, public and private user identities and the currently assigned IP address. SS is listening to SIP default port 5060 for both UDP and TCP protocols.

Test procedure

1-6) The same procedure as in subclause 8.10.4 are used with the exception that the SS sets the expiration time to 120 seconds in Step 4.

7)	Before half of the time has expired from the initial registration SS receives re-register message request with the From, To, Via, Contact, Expires, Supported, and P-Access-Network-Info header fields.

8)	SS responds to the REGISTER request with valid 200 OK response with the list of URIs contained in the P-Associated-URI header value, the new expiration time (1200 seconds) of the registration for this public user identity.

9)	SS waits for the REGISTER request and verifies it is received at least 600 seconds before the expected expiration time.

10)	SS responds to the REGISTER request with valid 200 OK response with the list of URIs contained in the P-Associated-URI header value, the new expiration time (1800 seconds) of the registration for this public user identity.

11)	SS waits for the REGISTER request and verifies it is received at least 600 seconds before the expected expiration time.

12)	SS responds to the REGISTER request with valid 200 OK response. SS shall populate the headers of the 200 OK response according to the 200 response for REGISTER common message definition.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-6 |  |  | Messages in Initial Registration Test case (subclause 8.10.4) | The same messages as in subclause 8.10.4 are used with the exception that in Step 4, the SS responds with 200 OK indicating 120 seconds expiration time. |
| 7 |  |  | REGISTER | The SS receives REGISTER from the UE 60 seconds before the expiration time set in the initial registration request. |
| 8 |  |  | 200 OK | The SS responds with 200 OK indicating 1200 seconds expiration time. |
| 9 |  |  | REGISTER | The SS receives REGISTER from the UE 600 seconds before the expiration time set in step 8. |
| 10 |  |  | 200 OK | The SS responds with 200 OK indicating 1800 seconds expiration time. |
| 11 |  |  | REGISTER | The SS receives REGISTER from the UE 600 seconds before the expiration time set in step 10 |
| 12 |  |  | 200 OK | The SS responds with 200 OK indicating the default expiration time. |

Specific Message Contents

Messages in Step 1-6

Messages in Step 1-6 are the same as those specified in subclause 8.10.4 with the following exception for the 200 OK for REGISTER in Step 4:

Use the default message “200 OK for REGISTER” in annex A.1.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| expires | 120 |

REGISTER (Step 7)

Use the default message “REGISTER” in annex A.1.1 with condition A3 “REGISTER for the case UE supports GIBA”.

200 OK for REGISTER (Step 8)

Use the default message “200 OK for REGISTER” in annex A.1.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| expires | 1200 |

REGISTER (Step 9)

Use the default message “REGISTER” in annex A.1.1 with condition A3 “REGISTER for the case UE supports GIBA”.

200 OK for REGISTER (Step 10)

Use the default message “200 OK for REGISTER” in annex A.1.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| expires | 1800 |

REGISTER (Step 11)

Use the default message “REGISTER” in annex A.1.1 with condition A3 “REGISTER for the case UE supports GIBA”.

200 OK for REGISTER (Step 12)

Use the default message “200 OK for REGISTER” in annex A.1.3.

<End of modification>

<Start of modification>

### 8.16.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE is not registered to IMS services, but has performed the generic test procedure in Annex C.2 up to step 3.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure

1-8) The same procedures as in steps 4-11 of C.2 are used with the exception that the SS sets the expiration time to 120 seconds in Step 4.

9)	Before half of the time has expired from the initial registration SS receives re-register message request with the From, To, Via, Contact, Authorization, Expires, Security-Client, Security-verify, Supported, and P-Access-Network-Info header fields.

10)	SS responds to the re-register message request with a 423 (Interval Too Brief) response.

11)	SS waits for the UE to send another REGISTER request populating the Expires header or the expires parameter in the Contact header with an expiration timer of at least the value received in the Min-Expires header of the 423 (Interval Too Brief) response.

12)	The SS responds to the REGISTER request with a valid 200 OK response indicating the default expiration timeout.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-8 |  |  | Messages 3-11 of C.2 | The same messages as in C.2 (steps 3-11) are used with the exception that in Step 4 (resp Step 7 in C.2), the SS responds with 200 OK indicating 120 seconds expiration time. |
| 9 |  |  | REGISTER | The SS receives REGISTER from the UE 60 seconds before the expiration time set in the initial registration request. |
| 10 |  |  | 423 Interval Too Brief | The SS responds with a 423 (Interval Too Brief) too brief response to the REGISTER request with T value in Min-Expires header. |
| 11 |  |  | REGISTER | UE sends a new REGISTER request with expires parameter value set to Tmod (equal or greater to T value in Min-Expires header of 423 (Interval Too Brief)). |
| 12 |  |  | 200 OK | The SS responds with 200 OK indicating the default expiration time. |

Specific Message Contents

Messages in Step 1-8

Messages in Step 1-8 are the same as those specified in steps 3-11 of C.2 with the following exception for the 200 OK for REGISTER in Step 4 (resp Step 7 in C.2):

Use the default message “200 OK for REGISTER” in annex A.1.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| expires | 120 |

REGISTER (Step 9)

Use the default message “REGISTER” in annex A.1.1 with conditions A2 "Subsequent REGISTER sent over security associations" and A17 "UE initiated IMS re-registration or de-registration" and with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Security-Client |  |
| spi-c | new SPI number of the inbound SA at the protected client port, shall be different than in step 3 |
| spi-s | new SPI number of the inbound SA at the protected server port, shall be different than in step 3 |
| port-c | new protected client port, shall be different than in step 3 |
| port-s | Same value as in the previous REGISTER |

423 Interval Too Brief for REGISTER (Step 10)

Use the default message “423 Interval Too Brief for REGISTER” in annex A.1.7 with the following exception:

| Header/param | Value/remark |
| --- | --- |
| Min-Expires |  |
| delta-seconds | 800000 (referred to as T in the test procedure and test requirement) |

REGISTER (Step 11)

Use the default message “REGISTER” in annex A.1.1 with conditions A2 "Subsequent REGISTER sent over security associations" and A17 "UE initiated IMS re-registration or de-registration" with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| expires | 800000 (referred to as Tmod in the expected sequence) (if present, see Rule 1) |
| Expires | (if present, see Rule 1) |
| delta-seconds | 800000 (referred to as Tmod in the expected sequence) |
| CSeq |  |
| value | must be incremented from the previous REGISTER |

Rule 1:	The REGISTER request must contain either an Expires header or an expires parameter in the Contact header. If both are present the value of Expires header is not important.

200 OK (Step 12)

Use the default message “200 OK for REGISTER” in annex A.1.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| expires | 800000 |

<End of modification>

<Start of modification>

### 9.2.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE is not registered to IMS services, but has discovered the SS as P-CSCF by executing the generic test procedure in Annex C.2 up to step 3.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17]. SS is listening to SIP default port 5060 for both UDP and TCP protocols.

Test procedure

1)	IMS registration is initiated on the UE. SS waits for the UE to send an initial REGISTER request, in accordance to 3GPP TS 24.229 [10], clause 5.1.1.2

2)	SS responds to the initial REGISTER request with an invalid 401 Unauthorized response, headers populated as follows:

a)	To, From, Via, CSeq, Call-ID and Content-Length headers according to RFC 3261 [15] clauses 8.2.6.2 and 20.14; and

b)	WWW-Authentication header with AKAv1-MD5 authentication challenge according to 3GPP TS `24.229 [10], clause 5.4.1.2.1 and RFC 3310 [17] clause 3; except that the SQN value in AUTN should be out of range

c)	Security-Server header according to 3GPP TS 24.229 [10], clause 5.2.2 and RFC 3329 [21] clause 2.

3)	SS waits for the UE to send a second Registration message indicating that the received 401 Unauthorized message was invalid

4)	SS sends a valid 401 Unauthorized message to the UE

5)	SS waits for the UE to send a Registration request using the temporary set of security associations to protect the message. The Registration request shall contain the valid answer to the authentication challenge in 401 Unauthorized sent in the previous step

6)	Continue test execution with the Generic test procedure in Annex C.2, step 7, sent over the same temporary set of security associations that the UE used for sending the REGISTER request

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | REGISTER | UE sends initial registration for IMS services. |
| 2 |  |  | 401 Unauthorized | The SS responds with an invalid AKAv1-MD5 authentication challenge with SQN out of range. |
| 3 |  |  | REGISTER | REGISTER request: - contains AUTS directive - UE populates a new Security-Client header set to specify the security mechanism it supports, the IPsec layer algorithms it supports and the parameters needed for the new security association setup. |
| 4 |  |  | 401 Unauthorized | This is a valid 401 (Unauthorized) message. |
| 5 |  |  | REGISTER | Message is sent using the temporary set of security associations to protect the message. Contains the valid answer to the authentication challenge sent in the 401 (Unauthorized) message. |
| 6 |  |  | Continue with Annex C.2 step 7 | Execute the Generic test procedure Annex C.2 steps 7-11 in order to get the UE in a stable registered state. |

Specific message contents

REGISTER (Step 1)

Use the default message “REGISTER” in annex A.1.1 with condition A1.

401 UNAUTHORIZED (Step 2)

Use the default message “401 Unauthorized for REGISTER” in annex A.1.2 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| WWW-Authenticate |  |
| nonce | Base 64 encoding of RAND and AUTN, generated with SQN out of range with the AMF information field set to AMFRESYNCH value to trigger SQN re-synchronisation procedure in test ISIM/USIM, see TS 34.108 clause 8.1.2.2. |

REGISTER (Step 3)

Use the default message “REGISTER” in annex A.1.1 with condition A1 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| CSeq |  |
| value | The value sent in the previous REGISTER message + 1 (incremented) |
| Call-ID |  |
| callid | The same value as in REGISTER in Step 1 |
| Authorization |  |
| nonce | Same value as the opaque value in the previous 401 UNAUTHORIZED message |
| opaque | Same value as the opaque value in the previous 401 UNAUTHORIZED message |
| response | parameter may exist, but value not to be checked |
| auth-param | auts= “auts-value”, auts-value not to be checked |
| nonce-count | value or presence of the parameter not to be checked |
| Security-Client |  |
| spi-c | new SPI number of the inbound SA at the protected client port, must be different from the value used in step 1 |
| spi-s | new SPI number of the inbound SA at the protected server port, must be different from the value used in step 1 |
| port-c | new protected client port needed for the setup of new pairs of security associations, must be different from the value used in step 1 |

REGISTER (Step 5)

Use the default message “REGISTER” in annex A.1.1 with conditions A2 and A17.

<End of modification>

<Start of modification>

### 11.2.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has activated a PDP context, discovered P-CSCF and registered to IMS services by executing the generic test procedure in Annex C.2 up to the last step.. The expiration time for the registration must be at least 600 seconds. Security associations have been set up between UE and the SS.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure

1)	SS sends UE a NOTIFY request for the subscribed registration event package, indicating the shortened expiration time as 60 seconds. Request is sent over the existing security associations between SS and UE.

2)	SS waits for the UE to respond the NOTIFY with 200 OK response.

3)	SS waits for the UE send a REGISTER request 30 seconds before the expected new expiration time.

4)	SS responds to the REGISTER request with a valid 401 Unauthorized response, headers populated according to the 401 response common message definition.

5)	SS waits for the UE to set up a new set of security associations and send another REGISTER request, over those security associations.

6)	The SS responds with 200 OK over the new security association

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | NOTIFY | The SS sends a NOTIFY for registration event package, containing partial registration state information, indicating shortened expiration time (60 seconds) for the registered public user identity in the XML body. |
| 2 |  |  | 200 OK | The UE responds the NOTIFY with 200 OK. |
| 3 |  |  | REGISTER | UE re-registers the user 30 seconds before the expected expiration. |
| 4 |  |  | 401 Unauthorized | The SS responds with a valid AKAv1-MD5 authentication challenge and security mechanisms supported by the network. |
| 5 |  |  | REGISTER | UE completes the security negotiation procedures, sets up a new temporary set of SAs and uses those for sending another REGISTER with AKAv1-MD5 credentials. |
| 6 | <- |  | 200 OK | The UE responds with 200 OK. |

Specific Message Contents

NOTIFY (Step 1)

Use the default message “NOTIFY for reg-event package” in annex A.1.6 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| CSeq |  |  |
| value |  | 2 |
| Message-body |  |  |
|  | NOT A1 | <?xml version=”1.0” encoding="UTF-8"?> <reginfo xmlns=”urn:ietf:params:xml:ns:reginfo” version=”1” state=”partial”> <registration aor=” PublicUserIdentity1 (NOTE 1)” id=”a100” state=”active”> <contact id=”980” state=”active” event=”shortened” expires="60"> <uri>same value as in Contact header of REGISTER request</uri> </contact> </registration> </reginfo> |
|  | A1 | <?xml version=”1.0” encoding="UTF-8"?> <reginfo xmlns=”urn:ietf:params:xml:ns:reginfo” xmlns:gr="urn:ietf:params:xml:ns:gruuinfo" version=”1” state=”partial”> <registration aor=” PublicUserIdentity1 (NOTE 1)” id=”a100” state=”active”> <contact id=”980” state=”active” event=”shortened” expires="60"> callid="Call-Id of most recent REGISTER" cseq="CSeq value of most recent REGISTER"> <uri>same value as in Contact header of REGISTER request</uri> <unknown-param name="+sip.instance"> "Instance ID of the UE;" </unknown-param> <gr:pub-gruu uri="public GRUU associated to this aor"/> <gr:temp-gruu uri="temporary GRUU associated to this aor" first-cseq="CSeq of the REGISTER request that caused the temporary GRUU to assigned for the UE"/> </contact> </registration> </reginfo> |

| Condition | Explanation |
| --- | --- |
| A1 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |

NOTE 1:	The public user ids and the associated TEL URI are as returned to the UE in the P-Associated-URI header of the 200 (OK) response to the REGISTER request;
PublicUserId1 is the default public user id i.e. the first one contained in P-Associated-URI

200 OK for NOTIFY (Step 2)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1

REGISTER (Step 3)

Use the default message “REGISTER” in annex A.1.1 conditions A2 and A17 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Security-Client |  |
| spi-c | new SPI number of the inbound SA at the protected client port |
| spi-s | new SPI number of the inbound SA at the protected server port |
| port-c | new protected client port needed for the setup of new pairs of security associations |
| port-s | Same value as in the previous REGISTER |

401 Unauthorized for REGISTER (Step 4)

Use the default message “401 Unauthorized for REGISTER” in annex A.1.2 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Security-Server |  |
| spi-c | new SPI number of the inbound SA at the protected client port |
| spi-s | new SPI number of the inbound SA at the protected server port |
| port-c | new protected client port needed for the setup of new pairs of security associations |
| port-s | Same value as in the previous Security-Server headers |
| WWW-Authenticate |  |
| nonce | Base 64 encoding of a new RAND and AUTN |

REGISTER (Step 5)

Use the default message “REGISTER” in annex A.1.1 with conditions A2 and A17.

<End of modification>

<Start of modification>

### 12.13b.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipp5ed into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-26)	UE executes the procedures described in TS 36.508 [94] table 4.5A.7.3-1 steps 1 to26.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-6 |  |  | Steps 1-6 defined in annex C.11 | MTSI MT speech call. Referred from 36.508 [94] table 4.5A.7.3-1 for a UE with E-UTRA support. |
| 7 |  |  | Step 9 defined in annex C.11 | The UE responds to INVITE with 180 Ringing |
| 8-9 |  |  | Step 12-13 defined in annex C.11 | SS responds to INVITE with a 200 OK final response and SS acknowledges the receipt of 200 OK for INVITE. |
| 10-11 |  |  | Step 14-15 defined in annex C.11 | The SS sends BYE to release the call and UE sends 200 OK for the BYE request and ends the call. |

NOTE:	The default messages contents in annex A are used with condition “IMS security” or “GIBA” when applicable

Specific Message Content

INVITE (Step 1)

Use the default message “INVITE for MT Call” in annex A.2.9 with the following exceptions:

| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 97 98 99 100 - b=AS:37 - b=RS:0 - b=RR:2000 Attributes for media: - a=rtpmap:97 AMR-WB/16000/1 - a=fmtp:97 mode-change-capability=2; max-red=220 - a=rtpmap: 98 telephone-event/16000 - a=fmtp: 98 0-15 - a=rtpmap:99 AMR/8000/1 - a=fmtp:99 mode-change-capability=2; max-red=220 - a=rtpmap: 100 telephone-event/8000 - a=fmtp: 100 0-15 - a=ptime:20 - a=maxptime:240 |
| --- | --- |

183 Session Progress (Step 4)

Use the default message "183 Session Progress" in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Status-Line |  |
| Reason-Phrase | Not checked |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(user-name) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 3] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR-WB/16000 [Note 2] - a=fmtp:(format) [Note 2, 3] Note 1: At least one "c=" field shall be present. Note 2: The values for fmt, payload type and format are not checked Note 3: Parameters for the AMR codec are not checked |

180 Ringing (Step 7)

Use the default message “180 Ringing for INVITE” in annex A.2.6.

BYE (Step 10)

Use the default message “BYE” in annex A.2.8.

200 OK for BYE (Step 11)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

<End of modification>

<Start of modification>

### 12.13c.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipp5ed into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-26)	UE executes the procedures described in TS 36.508 [94] table 4.5A.7.3-1 steps 1 to26.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-6 |  |  | Steps 1-6 defined in annex C.11 | MTSI MT speech call. Referred from 36.508 [94] table 4.5A.7.3-1 for a UE with E-UTRA support. |
| 7 |  |  | Step 9 defined in annex C.11 | The UE responds to INVITE with 180 Ringing |
| 8-9 |  |  | Step 12-13 defined in annex C.11 | SS responds to INVITE with a 200 OK final response and SS acknowledges the receipt of 200 OK for INVITE. |
| 10-11 |  |  | Step 14-15 defined in annex C.11 | The SS sends BYE to release the call and UE sends 200 OK for the BYE request and ends the call. |

NOTE:	The default messages contents in annex A are used with condition “IMS security” or “GIBA” when applicable

Specific Message Content

INVITE (Step 1)

Use the default message “INVITE for MT Call” in annex A.2.9 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 97 98 99 100 - b=AS:37 - b=RS:0 - b=RR:2000 Attributes for media: - a=rtpmap:97 AMR-WB/16000/1 - a=fmtp:97 mode-change-capability=2; max-red=220 - a=rtpmap: 98 telephone-event/16000 - a=fmtp: 98 0-15 - a=rtpmap:99 AMR/8000/1 - a=fmtp:99 mode-change-capability=2; max-red=220 - a=rtpmap: 100 telephone-event/8000 - a=fmtp: 100 0-15 - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv |

183 Session Progress (Step 4)

Use the default message "183 Session Progress" in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Status-Line |  |
| Reason-Phrase | Not checked |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(user-name) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 3] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR-WB/16000 [Note 2] - a=fmtp:(format) [Note 2, 3] Note 1: At least one "c=" field shall be present. Note 2: The values for fmt, payload type and format are not checked Note 3: Parameters for the AMR codec are not checked |

180 Ringing (Step 7)

Use the default message “180 Ringing for INVITE” in annex A.2.6.

BYE (Step 10)

Use the default message “BYE” in annex A.2.8.

200 OK for BYE (Step 11)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

<End of modification>

<Start of modification>

### 12.26.4	Method of test

Initial conditions

UE contains ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

0-13)	The UE executes the procedure described in TS 36.508 [94] table 4.5A.19.3-1 steps 1 to 14.

14)		SS sends a re-INVITE request to the UE.

15)		SS expects and receives 200 OK for re-INVITE from the UE.

15A)		SS acknowledges the receipt of 200 OK for re-INVITE.

16-19) SS executes the procedure C.33.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-13 |  |  | Steps defined in annex C.44 | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.19.3-1 for a UE with E-UTRA support. |
| 14 |  |  | INVITE | The SS sends re-INVITE with second SDP offer to switch to EVS AMR-WB IO mode. |
| 14A |  |  | 100 Trying | (Optional) The UE responds with a 100 Trying provisional response. |
| 15 |  |  | 200 OK | The UE responds to the re-INVITE with a 200 OK final response. |
| 15A |  |  | ACK | The SS acknowledges the receipt of 200 OK for re-INVITE. |
| 16-19 |  |  | Steps defined in annex C.33 | The SS releases the call. |

Specific Message Contents

INVITE (Step 14)

Use the default message “INVITE for MT Call” in annex A.2.9 with condition A5 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | SDP body copied from the previous 200 OK (C.44 step 6 or 8), and modified as follows: - “o=” line identical to previous SDP sent by SS except that sess-version is incremented. - “a=fmtp” line identical to previous SDP sent by SS except that “evs-mode-switch=1” is added. |

200 OK (Step 15)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1 with condition A23 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length | header shall be present if UE uses TCP to send this message and if there is a message body |
| value | length of message-body |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(user-name) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 4] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) EVS/16000 [Note 2] - a=fmtp:(format) evs-mode-switch=1; [Note 2, 3] Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The values for fmt, payload type and format are not checked. Note 3: The evs-mode-switch is checked, but no other codec parameters. Note 4: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. |

ACK (Step 15A)

Use the default message “ACK” in annex A.2.7 with condition A5.

<End of modification>

<Start of modification>

### 12.27.4	Method of test

Initial conditions

UE contains either SIM application (GIBA), ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-14)	The UE executes the procedure described in TS 36.508 [94] table 4.5A.6.3-1 steps 1 to 14.

15)		SS sends a re-INVITE request to the UE.

16)		Optional: SS waits for the UE to respond to the INVITE request with a 100 Trying response.

17)		SS waits for the UE to respond to the INVITE request with valid 200 OK response..

18)		SS sends an ACK to acknowledge receipt of the 200 OK for INVITE..

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-13 |  |  | Steps defined in annex C.21 | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.6.3-1 for a UE with E-UTRA support. |
| 14 |  |  | INVITE | SS sends re-INVITE with an SDP offer to switch to AMR-NB. |
| 15 |  |  | 100 Trying | (Optional) The UE responds with a 100 Trying provisional response |
| 16 |  |  | 200 OK | The UE responds re-INVITE with 200 OK final response. |
| 17 |  |  | ACK | The SS acknowledges the receipt of 200 OK for re-INVITE. |

Specific Message Contents

INVITE (Step 14)

Use the default message “INVITE for MT Call” in annex A.2.9 with condition A5 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | SDP body copied from the previous 200 OK (C.21 step 6 or 8), and modified as follows: - “o=” line identical to previous SDP sent by SS except that sess-version is incremented. Media description: - m=audio (transport port) RTP/AVP 99 100 - b=AS:37 - b=RS:300 - b=RR:900 Attributes for media: - a=rtpmap:99 AMR/8000/1 - a=fmtp:99 mode-set=7; octet-align=1 ; max-red=0 - a=rtpmap: 100 telephone-event/8000 - a=fmtp: 100 0-15 - a=ptime:20 - a=maxptime:20 - Attribute for preconditions are removed. |

200 OK (Step 16)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1 with condition A23 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length | header shall be present if UE uses TCP to send this message and if there is a message body |
| value | length of message-body |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(user-name) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 3] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR/8000 [Note 2] - a=fmtp:(format) Note 1: At least one "c=" field shall be present. Note 2: The values for fmt, payload type and format are not checked. Note 3: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. |

ACK (Step 17)

Use the default message “ACK” in annex A.2.7 with condition A5.

<End of modification>

<Start of modification>

### 12.27a.4	Method of test

Initial conditions

UE contains either SIM application (GIBA), ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-14)	The UE executes the procedure described in TS 36.508 [94] table 4.5A.19.3-1 steps 1 to 14.

15)		SS sends a re-INVITE request to the UE.

16)		Optional: SS waits for the UE to respond to the INVITE request with a 100 Trying response.

17)		SS waits for the UE to respond to the INVITE request with valid 200 OK response..

18)		SS sends an ACK to acknowledge receipt of the 200 OK for INVITE..

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-13 |  |  | Steps defined in annex C.44 | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.19.3-1 for a UE with E-UTRA support. |
| 14 |  |  | INVITE | SS sends re-INVITE with an SDP offer to switch to AMR-NB. |
| 15 |  |  | 100 Trying | (Optional) The UE responds with a 100 Trying provisional response |
| 16 |  |  | 200 OK | The UE responds re-INVITE with 200 OK final response. |
| 17 |  |  | ACK | The SS acknowledges the receipt of 200 OK for re-INVITE. |

Specific Message Contents

INVITE (Step 14)

Use the default message “INVITE for MT Call” in annex A.2.9 with condition A5 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | SDP body copied from the previous 200 OK (C.44 step 6 or 8), and modified as follows: - “o=” line identical to previous SDP sent by SS except that sess-version is incremented. Media description: - m=audio (transport port) RTP/AVP 99 100 - b=AS:37 - b=RS:300 - b=RR:900 Attributes for media: - a=rtpmap:99 AMR/8000/1 - a=fmtp:99 mode-set=7; octet-align=1 ; max-red=0 - a=rtpmap: 100 telephone-event/8000 - a=fmtp: 100 0-15 - a=ptime:20 - a=maxptime:20 - Attribute for preconditions are removed. |

200 OK (Step 16)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1 with condition A23 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length | header shall be present if UE uses TCP to send this message and if there is a message body |
| value | length of message-body |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(user-name) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 3] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR/8000 [Note 2] - a=fmtp:(format) Note 1: At least one "c=" field shall be present. Note 2: The values for fmt, payload type and format are not checked. Note 3: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. |

ACK (Step 17)

Use the default message “ACK” in annex A.2.7 with condition A5.

<End of modification>

<Start of modification>

### 15.4a.4	Method of test

Same as clause 12.13 with the following exceptions.

Initial conditions

Same as clause 12.13 with the following addition:

The UE is configured to use Terminating Identification Restriction in temporary mode. The corresponding default is currently set to “presentation not restricted”.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-11 |  |  | Steps 1-11 defined in annex C.11 | MTSI MT speech call. Referred from 36.508 [94] table 4.5A.7.3-1 for a UE with E-UTRA support. |
| 12 |  |  |  | Make UE accept the MTSI MT speech offer with Terminating Identification Restriction |
| 13-16 |  |  | Steps 12-15 defined in annex C.11 | MTSI MT speech call. Referred from 36.508 [94] table 4.5A.7.3-1 for a UE with E-UTRA support. |

Specific Message Contents

Steps 1-11 and 13-16 as specified in annex C.11 with the following exceptions:

183 Session Progress (Step 4)

Use the default message "183 Session Progress" in annex A.2.3 with the following exceptions:

| Header/param | Value/remark | Reference |
| --- | --- | --- |
| Privacy | id | RFC 3323 [135] RFC 3325 [89] |

180 Ringing (Step 9)

Use the default message “180 Ringing for INVITE” in annex A.2.6 with the following exceptions:

| Header/param | Value/remark | Reference |
| --- | --- | --- |
| Privacy | id | RFC 3323 [135] RFC 3325 [89] |

200 Ok (Step 13)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark | Reference |
| --- | --- | --- |
| Privacy | id | RFC 3323 [135] RFC 3325 [89] |

<End of modification>

<Start of modification>

### 15.8.4	Method of test

Initial conditions

UE contains either SIM application (GIBA), ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-9B)	UE executes the procedures described in TS 36.508 [94] table 4.5A.6.3-1 steps 1 to 14 but only steps 1 to 11 of Annex C.21 in the parallel behaviour to steps 13-14 of table 4.5A.6.3-1.

10)	SS responds to the INVITE with a valid 181 Call Is Being Forwarded response.

11)	SS (now starting to simulate the UE to which call was forwarded) sends another 183 Session Progress response to the INVITE request. As this response contains an SDP answer it is sent reliably.

12)	SS waits for the UE to send a PRACK request, containing an SDP offer in which the UE tells to have reserved the local resources.

13)	SS responds to the PRACK request with valid 200 OK response. The response contains an SDP answer which tells that SS has reserved its local resources as well.

13A)	UE may send an UPDATE request.

13B)	If UE sent and UPDATE request, SS responds with 200 OK.

14)	SS responds to the INVITE request with 180 Ringing response.

14A)	As the 180 Ringing response was sent reliably, UE sends a PRACK request.

14B)	SS responds to PRACK with 200 OK.

15)	SS responds to the INVITE request with valid 200 OK response.

16)	SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for INVITE.

17)	Call is released on the UE. SS waits the UE to send a BYE request.

18)	SS responds to the BYE request with valid 200 OK response.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-9B |  |  | Steps 1-11 as defined in Annex C.21 | The same messages as in steps 2 - 11 of Annex C.21 are used. Referred from 36.508 [94] table 4.5A.6.3-1 for a UE with E-UTRA support. |
| 10 |  |  | 181 Call is being forwarded | SS sends 181 response to indicate that call forwarding has been started as the user did not answer to the phone |
| 11 |  |  | 183 Session Progress | SS (simulating the phone to which the call was forwarded) responds with 183 Session Progress containing an SDP answer indicating support for AMR-WB codec and state of the local preconditions. UE will consider this response as forked one since it has different To tag this time compared to step 8. |
| 12-13B |  |  | Steps 5-8 as defined in Annex C.21 | The same messages as specified in steps 5 - 8 of Annex C.21 are used with To-tag and Contact Address as in the 183 response of step 11 |
| 14 |  |  | 180 Ringing | The SS sends 180 Ringing response to the UE |
| 14A |  |  | PRACK | UE acknowledges the receipt of 180 response by sending PRACK. |
| 14B |  |  | 200 OK | The SS responds PRACK with 200 OK. |
| 15 |  |  | 200 OK | The SS responds INVITE with 200 OK to indicate that the virtual remote UE had answered the call |
| 16 |  |  | ACK | The UE acknowledges the receipt of 200 OK for INVITE |
| 17 |  |  | BYE | The UE releases the call with BYE |
| 18 |  |  | 200 OK | The SS sends 200 OK for BYE |

NOTE:	The default messages contents in annex A are used with condition “IMS security“ or “GIBA” when applicable

Specific Message Contents

181 Call is being forwarded for INVITE (Step 10)

Use the default message “181 Call is being forwarded” in annex A.2.14

183 Session Progress for INVITE (Step 11)

Use the default message “183 Session Progress for INVITE” in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| To |  |
| tag | different tag must be used than the one used in steps 3-9 as this response is now from another UE and belongs to another dialog instance. Note that this new tag must be used within the rest of the steps (10-17) in this test case instead of the tag used within steps 3-9. |
| Contact |  |
| addr-spec | different URI must be used than the one used in step 3 as this is supposed now to represent another UE to which the call is being forwarded.. Note that this new Contact must be used within the rest of the steps (13-14) in this test case. |
| Require |  |
| option-tag | precondition |
| Message-body | Same contents as specified in step 4 annex C.21. except for o-line: o=- 22222222 22222222 IN (addrtype) (unicast-address for new remote UE). |

PRACK or UPDATE (Step 12 or 13A)

The UE shall include an SDP body as described in C.21, Step 5 , but with the following exceptions:

-	Contents of o= line is not checked

180 Ringing for INVITE (Step 14)

Use the default message “180 Ringing for INVITE” in annex A.2.6 applying condition A3 (Response sent reliably) and with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | Same value as in the 183 response of step 11 |
| History-Info |  |
| hi-targeted-to-uri | Same value as in the 181 response of step 10 |
| hi-index | Same value as in the 181 response of step 10 |

PRACK (Step 14A)

Use the default message “PRACK” in annex A.2.4.

200 OK for PRACK (Step 14B)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1.

200 OK for INVITE (Step 15)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | Same value as in the 183 response of step 11 |
| History-Info |  |
| hi-targeted-to-uri | Same value as in the 181 response of step 10 |
| hi-index | Same value as in the 181 response of step 10 |

ACK (Step 16)

Use the default message “ACK” in annex A.2.7.

BYE (Step 17)

Use the default message “BYE” in annex A.2.8.

200 OK for BYE (Step 18)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

<End of modification>

<Start of modification>

### 15.18.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has activated a PDP context, discovered P-CSCF, registered to IMS services by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step and thereafter created a conference by executing the generic test procedure in Annex C.10 up to its last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration and conference.

Test procedure

1)	UE invites a user to the conference created. SS waits the UE to send to the invited user a REFER request, which refers to the conference created.

2)	SS responds to the REFER request with a valid 202 Accepted response.

3)	SS sends an initial NOTIFY to tell that the invited user is trying to join the conference.

4)	UE responds to the NOTIFY request with valid 200 OK response.

5)	SS sends the final NOTIFY to tell that the invited user has successfully joined the conference.

6)	UE responds to the NOTIFY request with a valid 200 OK response.

7)	Optional: If UE subscribed the conference event package during the generic test procedure of Annex C.10, SS sends a NOTIFY for the conference event package to the UE to notify that the user joined the conference.

8) If SS sent a NOTIFY, SS waits the UE to respond the NOTIFY with 200 OK.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | REFER | UE sends REFER to SS referring to the conference |
| 2 |  |  | 202 Accepted | The SS responds with a 202 final response |
| 3 |  |  | NOTIFY | The SS sends initial NOTIFY for the implicit subscription created by the REFER request |
| 4 |  |  | 200 OK | The UE responds the NOTIFY with 200 OK |
| 5 |  |  | NOTIFY | The SS sends a NOTIFY related to REFER request to confirm that the invited user was able to join the conference |
| 6 |  |  | 200 OK | The UE responds the NOTIFY with 200 OK |
| 7 |  |  | NOTIFY | Optional: If the UE has subscribed the conference event package, the SS sends a NOTIFY for conference event package to inform that the invited user was able to join the conference |
| 8 |  |  | 200 OK | Optional: The UE responds the NOTIFY with 200 OK |

Specific Message Contents

REFER (Step 1)

Use the default message “MO REFER” in annex A.2.10 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Request-URI | SIP URI of the user invited to the conference |
| Refer-To |  |
| addr-spec | sip:final@conf-factory. appended with px_IMS_HomeDomainName |
| To |  |
| addr-spec | SIP URI of the user invited to the conference |
| tag | no tag given |
| Call-ID |  |
| callid | value different to that received in INVITE message used to create the conference |
| CSeq |  |
| value | must be present, value not checked |

202 Accepted for REFER (Step 2)

Use the default message “202 Accepted” in annex A.3.3.

NOTIFY (Step 3)

Use the default message “MT NOTIFY for refer package” in annex A.2.11 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | SIP/2.0 100 Trying |

200 OK for NOTIFY (Step 4)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

NOTIFY (Step 5)

Use the default message “MT NOTIFY for refer package” in annex A.2.11 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Subscription-State |  |
| substate-value | terminated |
| expires | omitted from the request |
| reason | noresource |
| Message-body | SIP/2.0 200 OK |

200 OK for NOTIFY (Step 6)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

NOTIFY (Step 7)

Use the default message “NOTIFY for conference event package” in annex A.5.3 with condition A3 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | <?xml version="1.0" encoding="UTF-8"?> <conference-info xmlns="urn:ietf:params:xml:ns:conference-info"> entity="sip:final@conf-factory. appended with px_IMS_HomeDomainName" state="partial" version="1" <users> <user entity=" SIP URI of the invited user"> <endpoint entity=" Contact URI of the invited user"> <status>connected</status> <joining-method>dialed-in</joining-method> <media id="1"> <type>audio</type> <label>11223</label> <src-id>random SSRC value</src-id> <status>sendrecv</status> </media> </endpoint> </users> </conference-info> |

200 OK for NOTIFY (Step 8)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

<End of modification>

<Start of modification>

### 15.21a.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF, registered to IMS services and set up the MO call, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step and thereafter executing the generic test procedure in TS 36.508 [94] table 4.5A.6.3-1 steps 1 to 14 for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1).

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration and MO call.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-4)	Call hold is initiated on the UE. The same steps defined in Annex C.8 are used to put the call into hold.

5-17)	A new session is created by using the steps defined in Annex C.21.

17A)	The UE is triggered to start a multiparty call. This causes the UE to first put the second call on hold as described in Steps 17B-17E, and then to initiate the following steps 19-46D.

17B-17E)	The UE puts the second call on hold by executing the steps described in Annex C.8

19-30)	UE initiates the conference creation process by executing steps 2-13 of the generic test procedure in Annex C.10.

31-38)	UE invites one of the user who have session with the UE to the conference by performing the same procedure as in Annex C.19.

39-46D)	UE invites another user who have session with the UE to the conference by performing the same procedure as in Annex C.19.

UE shall send two BYE requests to terminate the two initial calls it put on hold. SS responds to the BYE requests with a valid 200 OK response each.

47)	SS sends a BYE request to the UE in order to release the active session if BYE request has not already been received.

48)	UE responds to the BYE request with valid 200 OK response.

49)	SS notifies the UE that its subscription to conf event is terminated.

50)	UE responds with 200 OK.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-4 |  |  | Messages in Annex C.8 | The same messages as in Annex C.8 Steps 1-4 are used to put the first call on hold. |
| 5-17 |  |  | Steps defined in Annex C.21 | The same messages as in Annex C.21 are used to start a second call. |
| 17A |  |  |  | Make UE start a Multiparty Call |
| 17B-17E |  |  | Messages in Annex C.8 | The same messages as in Annex C.8 Steps 1-4 are used to put the second call on hold |
| 18 |  |  |  | Void |
| 19-30 |  |  | Steps 2-13 defined in Annex C.10 | The same messages as in Annex C.10 are used. |
| 31-38 |  |  | Steps defined in Annex C.19 | The same messages as in Annex C.19 steps 1-8 are used. |
| 39-46 |  |  | Steps defined in Annex C.19 | The same messages as in Annex C.19 steps 1-8 are used. |
| 46A |  |  | BYE | UE shall send a BYE to terminate the first call |
| 46B |  |  | 200 OK | The SS responds the received BYE with 200 OK |
| 46C |  |  | BYE | UE shall send a BYE to terminate the second call. |
| 46D |  |  | 200 OK | The SS responds the received BYE with 200 OK |
| 47 |  |  | BYE | The SS releases the active session with BYE |
| 48 |  |  | 200 OK | The UE sends 200 OK for BYE |
| 49 |  |  | NOTIFY | If the UE had subscribed to the conference event package, the SS notifies the UE that its subscription to conference event package is terminated |
| 50 |  |  | 200 OK | The UE sends 200 OK for NOTIFY (if sent by SS) |

NOTE 1:	Steps 27-30 (i.e., steps 10-13 of C.10) are optional. Therefore, UE can start with steps 31-46 right away after Step 26. If Steps 27-30 are executed, they can happen in parallel to Steps 31-46.

NOTE 2:	The two executions of Annex C.19, i.e., steps 31-38 and steps 39-46, can run in parallel.

NOTE 3:	Step 46A can happen any time after step 35. The SS sends the corresponding 200 OK message right after having received the BYE message.

NOTE 4:	Step 46C can happen any time after step 43. The SS sends the corresponding 200 OK message right after having received the BYE message.

Specific Message Contents

INVITE(Step 6)

Use the default message “INVITE” in annex A.2.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | px_IMS_CalleeUri2 px_IMS_CalleeUri2 is used to invite another user to the session. px_IMS_CalleeUri2 may be either SIP or Tel URI. It may contain a dialstring and phone-context parameter, when calling to dialstring. When calling to dialstring SIP URI must also contain user=phone or user=dialstring parameter. The dialstring, if used, may be global, home local number or geo-local number. For home local numbers the value of phone-context parameter must equal the home domain name i.e. px_IMS_HomeDomainName. For geo-local numbers the home domain name must be prefixed by string “geo-local.” or access technology specific prefix, if the UE supports that option. Note: The way how the UE determines whether numbers in a non-international format are geo-local, home-local or relating to another network, is UE implementation specific. For instance the UE might have a UI setting. |
| To |  |
| addr-spec | px_IMS_CalleeUri2 |

183 Session Progress for INVITE (Step 8)

Use the default message “183 Session Progress for INVITE” in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | px_IMS_CalleeContactUri2 |
| Message-body | o=- 1111111112 1111111111 IN (addrtype) (unicast-address for SS) |

180 Ringing for INVITE (Step 13)

Use the default message “180 Ringing for INVITE” in annex A.2.6 with condition A3 (Response sent reliably) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | px_IMS_CalleeContactUri2 |

200 OK for INVITE (Step 16)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | px_IMS_CalleeContactUri2 |

183 Session Progress for INVITE (Step 22)

Use the default message “183 Session Progress for INVITE” in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | o=- 1111111113 1111111111 IN (addrtype) (unicast-address for SS) |

BYE (Step 47)

Use the default message “BYE” in annex A.2.8.

200 OK for BYE (Step 48)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

NOTIFY (Step 49)

Use the default message “NOTIFY for conference event package” in annex A.5.3 with condition A4.

200 OK for NOTIFY (Step 50)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

<End of modification>

<Start of modification>

### 15.21c.4	Method of test

Initial conditions

UE contains either SIM application (GIBA), ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step and thereafter executing the generic test procedure in TS 36.508 [94] table 4.5A.8.3-1, steps 1 to 15.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration and MO Video call.

Test procedure

1-4)	Call hold is initiated on the UE. The same steps defined in Annex C.8 are used to put the call into hold.

5-17)	A new session is created by using the steps defined in Annex C.25.

17A)	The UE is triggered to start a multiparty call. This causes the UE to first put the second call on hold as described in Steps 17B-17E, and then to initiate the following steps 19-46D.

17B-17E)	The UE puts the second call on hold by executing the steps described in Annex C.8

19-30)	UE initiates the conference creation process by executing steps 2-13 of the generic test procedure in Annex C.38.

31-38)	UE invites one of the user who have session with the UE to the conference by performing the same procedure as in Annex C.37.

39-46D)	UE invites another user who has a session with the UE to the conference by performing the same procedure as in Annex C.37.

UE shall send two BYE requests to terminate the two initial calls it put on hold.

SS responds to the BYE requests with a valid 200 OK response each.

47)	SS sends a BYE request to the UE in order to release the active session if BYE request has not already been received.

48)	UE responds to the BYE request with valid 200 OK response.

49)	SS notifies the UE that its subscription to conf event is terminated.

50)	UE responds with 200 OK.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-4 |  |  | Messages in Annex C.8 | The same messages as in Annex C.8 Steps 1-4 are used. |
| 5-17 |  |  | Steps defined in Annex C.25 | The same messages as in Annex C.25 are used. |
| 17A |  |  |  | Make UE start a Multiparty Call |
| 17B-17E |  |  | Messages in Annex C.8 | The same messages as in Annex C.8 Steps 1-4 are used to put the second call on hold |
| 18 |  |  |  | Void |
| 19-30 |  |  | Steps 2-13 defined in Annex C.38 | The same messages as in Annex C.38 are used. |
| 31-38 |  |  | Steps defined in Annex C.37 | The same messages as in Annex C.37 steps 1-8 are used. |
| 39-46 |  |  | Steps defined in Annex C.37 | The same messages as in Annex C.37 steps 1-8 are used. |
| 46A |  |  | BYE | UE shall send a BYE to terminate the first call. |
| 46B |  |  | 200 OK | The SS responds the received BYE with 200 OK |
| 46C |  |  | BYE | UE shall send a BYE to terminate the second call. |
| 46D |  |  | 200 OK | The SS responds the received BYE with 200 OK |
| 47 |  |  | BYE | The SS releases the active session with BYE |
| 48 |  |  | 200 OK | The UE sends 200 OK for BYE |
| 49 |  |  | NOTIFY | If the UE had subscribed to the conference event package, the SS notifies the UE that its subscription to conference event package is terminated |
| 50 |  |  | 200 OK | The UE sends 200 OK for NOTIFY (if sent by SS) |

NOTE 1:	Steps 27-30 (i.e., steps 10-13 of C.10) are optional. Therefore, UE can start with steps 31-46 right away after Step 26. If Steps 27-30 are executed, they can happen in parallel to Steps 31-46.

NOTE 2:	The two executions of Annex C.19, i.e., steps 31-38 and steps 39-46, can run in parallel.

NOTE 3:	Step 46A can happen any time after step 35. The SS sends the corresponding 200 OK message right after having received the BYE message.

NOTE 4:	Step 46C can happen any time after step 43. The SS sends the corresponding 200 OK message right after having received the BYE message.

Specific Message Contents

INVITE (Step 6)

Use the default message “INVITE” in annex A.2.1 with condition A11 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | px_IMS_CalleeUri2 px_IMS_CalleeUri2 is used to invite another user to the session. px_IMS_CalleeUri2 may be either SIP or Tel URI. It may contain a dialstring and phone-context parameter, when calling to dialstring. When calling to dialstring SIP URI must also contain user=phone or user=dialstring parameter. The dialstring, if used, may be global, home local number or geo-local number. For home local numbers the value of phone-context parameter must equal the home domain name i.e. px_IMS_HomeDomainName. For geo-local numbers the home domain name must be prefixed by string “geo-local.” or access technology specific prefix, if the UE supports that option. Note: The way how the UE determines whether numbers in a non-international format are geo-local, home-local or relating to another network, is UE implementation specific. For instance the UE might have a UI setting. |
| To |  |
| addr-spec | px_IMS_CalleeUri2 |

183 Session Progress for INVITE (Step 8)

Use the default message “183 Session Progress for INVITE” in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | px_IMS_CalleeContactUri2 |
| Message-body | o=- 1111111112 1111111111 IN (addrtype) (unicast-address for SS) |

180 Ringing for INVITE (Step 13)

Use the default message “180 Ringing for INVITE” in annex A.2.6 with condition A3 (Response sent reliably) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | px_IMS_CalleeContactUri2 |

200 OK for INVITE (Step 16)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | px_IMS_CalleeContactUri2 |

183 Session Progress for INVITE (Step 22)

Use the default message “183 Session Progress for INVITE” in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | o=- 1111111113 1111111111 IN (addrtype) (unicast-address for SS) |

BYE (Step 47)

Use the default message “BYE” in annex A.2.8.

200 OK for BYE (Step 48)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

NOTIFY (Step 49)

Use the default message “NOTIFY for conference event package” in annex A.5.3 with condition A4.

200 OK for NOTIFY (Step 50)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

<End of modification>

<Start of modification>

### 15.25.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF, registered to IMS services and set up an MO call, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step and thereafter executing the generic test procedure in TS 36.508 [94] table 4.5A.6.3-1 steps 1 to 14 for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1).

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration and MO call.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-4)	UE is in an active call with the SS (simulating the transferee). Consultative Call Transfer is initiated at the UE. UE puts the ongoing call on hold.

5-16)	UE sets up an MO call with the transfer target (also simulated by the SS).

17-20)	UE puts the call with the transfer target UE on hold.

21)	SS waits for UE to send a REFER request to the transferee within the existing dialog between the UE and the transferee.

22)	SS responds to the REFER request with a valid 200 OK response.

23)	 SS sends UE an initial NOTIFY to indicate that the implicit refer subscription is pending.

24)	SS waits for UE to respond to NOTIFY with valid 200 OK response.

25-28)	 Call between UE and the transferee UE is put on hold by SS.

29)	SS releases call between UE and the transfer target by sending a BYE request.

30)	SS waits for UE to respond to the BYE request with valid 200 OK response.

31)	SS sends UE the final NOTIFY to indicate that the call transfer was successfully completed.

32)	SS waits for UE to respond to NOTIFY with valid 200 OK response.

33)	UE may send a BYE request to release the call with the transferee UE.

34)	If UE has sent a BYE request in Step 33, SS responds to this request with valid 200 OK response.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
|  |  |  | Make the UE put the call on hold (Note 1) |  |
| 1-4 |  |  | Steps defined in Annex C.8 to hold the call | UE holds the call with the transferee. |
| 5-16 |  |  | Steps 2-13 defined in Annex C.21 | In order to establish a call with the transfer target, the same messages as in Annex C.21 are used. |
| 16A |  |  | Make the UE confirm the Consultative Call Transfer (Note 2) |  |
| 17-20 |  |  | Steps defined in Annex C.8 to hold the call | UE holds call with transfer target. |
| 21 |  |  | REFER | The UE sends REFER to SS, simulating the transferee, referring to the transfer target |
| 22 |  |  | 200 OK | The SS responds to REFER with 200 OK |
| 23 |  |  | NOTIFY | The SS, simulating the transferee, sends initial NOTIFY for the implicit subscription created by the REFER request |
| 24 |  |  | 200 OK | The UE responds to NOTIFY with 200 OK |
| 25-28 |  |  | Steps defined in Annex C.9 to hold the call | The SS, simulating the transferee, puts the UE on hold, setting the direction attribute to inactive. |
| 29 |  |  | BYE | The SS, simulating the transfer target, releases the call between UE and the transfer target with BYE |
| 30 |  |  | 200 OK | The UE responds to BYE with 200 OK |
| 31 |  |  | NOTIFY | The SS, simulating the transferee, sends a NOTIFY to confirm that the call transfer has been completed |
| 32 |  |  | 200 OK | The UE responds to NOTIFY with 200 OK |
| 33 |  |  | BYE | Optional: UE may send BYE request to release call with the transferee |
| 34 |  |  | 200 OK | If the UE has sent BYE in step 33 then SS sends 200 OK for BYE |
| NOTE 1: This can be done with AT command AT+CHLD=2 if supported by the UE or with an appropriate MMI command otherwise. NOTE 2: This can be done with AT command AT+CHLD=4 if supported by the UE or by with appropriate MMI command otherwise. |  |  |  |  |

Specific Message Contents

INVITE(Step 5 resp Step 2 of C.21)

Use the default message “INVITE” in annex A.2.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | px_IMS_CalleeUri2 px_IMS_CalleeUri2 may be either SIP or Tel URI. It may contain a dialstring and phone-context parameter, when calling to dialstring. When calling to dialstring SIP URI must also contain user=phone or user=dialstring parameter. The dialstring, if used, may be global, home local number or geo-local number. For home local numbers the value of phone-context parameter must equal the home domain name i.e. px_IMS_HomeDomainName. For geo-local numbers the home domain name must be prefixed by string “geo-local.” or access technology specific prefix, if the UE supports that option. Note: The way how the UE determines whether numbers in a non-international format are geo-local, home-local or relating to another network, is UE implementation specific. For instance the UE might have a UI setting. |
| To |  |
| addr-spec | px_IMS_CalleeUri2 |

183 Session Progress for INVITE (Step 7 resp Step 4 of C.21)

Use the default message “183 Session Progress for INVITE” in annex A.2.3 with the following exceptions:

| Header/param |  | Value/remark |  |
| --- | --- | --- | --- |
| Contact |  |  |  |
| addr-spec |  | px_IMS_CalleeContactUri2 |  |
| Message-body |  | o=- 1111111112 1111111111 IN (addrtype) (unicast-address for SS) |  |

180 Ringing for INVITE (Step 12 resp Step 9 of C.21)

Use the default message “180 Ringing for INVITE” in annex A.2.6 with condition A3 (Response sent reliably) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | px_IMS_CalleeContactUri2 |

200 OK for INVITE (Step 15 resp Step 12 of C.21)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | px_IMS_CalleeContactUri2 |

Messages in Steps 17-20

Messages in Steps 17-20 are the same as those specified in Annex C.8 with the following exceptions:

INVITE or UPDATE (Step 17)

Use the default message "INVITE" in annex A.2.1 with condition A5 and the following exceptions or the default message "UPDATE" with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | px_IMS_CalleeContactUri2 |

REFER (Step 21)

Use the default message “MO REFER” in annex A.2.10 on the first dialog created in the preamble with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Refer-To |  |
| value | <public address of transfer target?Replaces=(dialog id of the dialog between the UE and the transfer target)&Require=replaces> |
| Referred-By |  |
| value | same value as addr-spec field in From header in the first INVITE during initial call setup, if header present |
| Privacy |  |
| value | user (shall be included if privacy was required during original communication dialog and Referred-By header field is included) |

NOTIFY (Step 23)

Use the default message “MT NOTIFY for refer package” in annex A.2.11 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | SIP/2.0 100 Trying |

200 OK for NOTIFY (Step 24)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1

Messages in Steps 25-28

Messages in Steps 25-28 are the same as those specified in Annex C.9 with the following exceptions:

-	each media line shall carry direction attribute “a=inactive”.

NOTIFY (Step 31)

Use the default message “MT NOTIFY for refer package” in annex A.2.11 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Subscription-State |  |
| substate-value | Terminated |
| expires | omitted from the request |
| reason | Noresource |
| Message-body | SIP/2.0 200 OK |

200 OK for NOTIFY (Step 32)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1

<End of modification>

<Start of modification>

### 15.30.4	Method of test

Initial conditions

UE contains either SIM application (GIBA), ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt an user initiated USSI with USSD string “*#60#” | The SS may use AT command “CUSD” to trigger the user initiated USSI |
| 2 |  |  | INVITE | UE sends INVITE with the SDP offer. |
| 3 |  |  | 100 Trying | SS sends a 100 Trying provisional response. |
| 4 |  |  | 200 OK | SS sends a 200 OK. |
| 5 |  |  | ACK | UE acknowledges. |
| 6 |  |  | BYE | SS sends BYE to release the session. |
| 7 |  |  | 200 OK | UE sends a 200 OK. |

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE” in annex A.2.1 with the following exception.

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | Request-URI set to a SIP URI with user part including the percent-encoded USSD string as used at step 1 and a "phone-context" parameter set to the home network domain name used in REGISTER request, a host part set to the home netwok domain name used in REGISTER request and a "user" URI parameter set to value "dialstring" Note: In USSD string *#60#, * may or may not be percent-encoded. |
| To |  |
| addr-spec | same as Request URI |
| tag | not present |
| Recv-Info |  |
| Info-package-type | g.3gpp.ussd |
| Accept | application/vnd.3gpp.ussd+xml, application/sdp, multipart/mixed (additional medias can be added in any order) |
| Content-Type |  |
| media-type | multipart/mixed;boundary=any value |
| Message-body | The following SDP types and values. --boundary value (as provided in SIP hdr Content-Type) Content-Type: application/sdp Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=(media) 0 [Note2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) --boundary value (as provided in SIP hdr Content-Type) Content-Type: application/vnd.3gpp.ussd+xml <?xml version="1.0" encoding="UTF-8"?> <ussd-data> <language>(language)</language> [Note 3] <ussd-string>( USSD string as used at step 1)</ussd-string> </ussd-data> --boundary value (as provided in SIP hdr Content-Type) Note 1: At least one "c=" field shall be present. Note 2: media is the type of media like audio. Note 3: language is the type of USSD language coded as defined in IETF RFC 5646 [153] |

200 OK for INVITE (Step 4)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exception.

| Header/param | Value/remark |
| --- | --- |
| Recv-Info |  |
| Info-package-type | g.3gpp.ussd |
| Accept | application/vnd.3gpp.ussd+xml, application/sdp, multipart/mixed MIME types |
| Content-Type |  |
| media-type | application/sdp |
| Message-body | SDP body of the 200 response copied from the received INVITE and modified as follows: - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - IP address on "c=" line changed to indicate to which IP address and port the UE should start sending the media; - "a=" lines are all removed. |

BYE (Step 6)

Use the default message “BYE” in annex A.2.8 with the following exception.

| Header/param | Value/remark |
| --- | --- |
| Content-Type |  |
| media-type | application/vnd.3gpp.ussd+xml |
| Message-body | <?xml version="1.0" encoding="UTF-8"?> <ussd-data> <language>en</language> <ussd-string>148*7#</ussd-string> </ussd-data> |

<End of modification>

<Start of modification>

### 16.2.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has activated a PDP context, discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure

- SS sends an INVITE request to the UE.

- Void.

3)	SS may receive 100 Trying from the UE.

3A)	SS may receive 183 Session Progress from the UE.
SS triggers the activation of a dedicated bearer.

3B)	SS may send PRACK to the UE to acknowledge the 183 Session Progress.

3C)	SS may receive 200 OK for PRACK from the UE.

4)	SS may receive 180 Ringing from the UE.

5)	SS may send PRACK to the UE to acknowledge the 180 Ringing.

6)	SS may receive 200 OK for PRACK from the UE.

6A)	The UE accepts the session invite.
If 180 Ringing is not received from the UE after 5s from step 1, the MMI command shall be started to trigger the UE to accept the call.

7)	SS expects and receives 200 OK for INVITE from the UE.

8)	SS send an ACK to acknowledge receipt of the 200 OK for INVITE

9)	SS sends BYE to the UE.

10)	SS expects and receives 200 Ok for BYE from the UE

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | SS sends INVITE with the first SDP offer. |
| 2 |  |  |  | Void |
| 3 |  |  | 100 Trying | (Optional) The UE responds with a 100 Trying provisional response. |
| 3A |  |  | 183 Session Progress | (Optional) The UE sends 183 response reliably with the SDP answer to the offer in INVITE |
| 3B |  |  | PRACK | (Optional) SS acknowledges if a 183 Session Progress is received. |
| 3C |  |  | 200 OK | (Optional) The UE responds if a PRACK is sent. |
| 4 |  |  | 180 Ringing | (Optional) The UE responds to INVITE with 180 Ringing. |
| 5 |  |  | PRACK | (Optional) SS shall send PRACK if the 180 response contains 100rel option-tag in the Require header. |
| 6 |  |  | 200 OK | (Optional) The UE acknowledges the PRACK with 200 OK. |
| 6A |  |  |  | Make UE accept the speech AMR offer. |
| 7 |  |  | 200 OK | The UE responds INVITE with 200 OK. |
| 8 |  |  | ACK | The SS acknowledges the receipt of 200 OK for INVITE. |
| 9 |  |  | BYE | The SS releases the call with BYE. |
| 10 |  |  | 200 OK | The UE sends 200 OK for BYE. |

NOTE 1:	The default messages contents in annex A are used with condition “IMS security” or “GIBA” when applicable

NOTE 2:	Steps 4, 5, and 6 can happen in parallel to steps 3B and 3C

Specific Message Contents

INVITE (Step 1)

Use the default message "INVITE for MT Call" in annex A.2.9 with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 99 100 - b=AS:37 - b=RS:0 - b=RR:2000 Attributes for media: - a=rtpmap:99 AMR/8000/1 - a=fmtp:99 mode-set=0,2,4,7; mode-change-capability=2; max-red=220 - a=rtpmap: 100 telephone-event/8000 - a=fmtp: 100 0-15 - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv |

100 Trying for INVITE (Step 3)

Use the default message “100 Trying for INVITE” in annex A.2.2.

183 Session Progress (Step 3A)

Use the default message "183 Session Progress" in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Status-Line |  |
| Reason-Phrase | Not checked |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR/8000 [Note 2] - a=fmtp:(format) Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. |

180 Ringing (Step 4)

Use the default message “180 Ringing for INVITE” in annex A.2.6 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length | header shall be present if UE uses TCP to send this message and if there is a message body |
| value | length of message-body |
| Message-body | optional if 183 Session Progress is not used not present if 183 Session Progress is used (step 3A) Contents if present: The following SDP types and values shall be present. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR/8000 [Note 2] - a=fmtp:(format) mode-set=0,2,4,7; Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. |

200 OK for INVITE (Step 7)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length | header shall be present if UE uses TCP to send this message and if there is a message body |
| value | length of message-body |
| Message-body | not present if 183 Session Progress is used (step 3A) or 180 Ringing (step 4) contained SDP. present if 183 Session Progress is not used (step 3A) and 180 Ringing (step 4) did not contain SDP. Contents if present: The same requirements for SDP types and values as specified in step 4. |

<End of modification>

<Start of modification>

### 17.1.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF, registered to IMS services and set up the MO call, by executing annex C.21.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration and MO call.

Test procedure

1)	Adding video to the voice call is initiated on the UE.

2-10)	UE executes the procedure described in TS 36.508 [94] table 4.5A.11.3-1, steps 2 to 7. In detail the following steps are done in IMS.

2)	UE to sends a re-INVITE request to the SS.

3)	SS responds to the re-INVITE request with a 100 Trying response.

4)	SS responds to the re-INVITE request with a 183 Session Progress response.

5)	SS waits for the UE to send a PRACK request possibly containing the second SDP offer for update of precondition state.

6)	SS responds to the PRACK request with a valid 200 OK response.

7)	SS waits for the UE to optionally send an UPDATE request containing the final SDP offer. UE will not send the UPDATE request if the PRACK in step 4 already contained the final offer with preconditions met.

8)	SS responds to the UPDATE request (if UE sent one) with a valid 200 OK response.

9)	SS responds to the re-INVITE request with a valid 200 OK response.

10)	SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for INVITE.

10A)	The UE is triggered to remove the video stream from the multimedia call

11)	SS waits the UE to send a re-INVITE request with a SDP offer indicating the removal of the video stream.

12)	SS responds to the re-INVITE request with a 100 Trying response.

12A)	SS deactivates the EPS bearer corresponding to the video stream and releases the associated radio resources by applying the procedure described in TS 36.508 [94] clause 4.5A.15

13)	SS responds to the re-INVITE request with a valid 200 OK response.

14)	SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for re-INVITE.

15-19)	MO Call release according to procedure C.32.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make UE add video to the voice call. |  |
| 2 |  |  | INVITE | UE sends re-INVITE with an SDP offer containing media lines for both voice and video |
| 3 |  |  | 100 Trying | The SS responds with a 100 Trying provisional response |
| 4 |  |  | 183 Session in Progress | SS responds with an SDP answer indicating that SS has not reserved its resources for video. |
| 5 |  |  | PRACK | UE acknowledges the receipt of 183 response with PRACK and optionally offers second SDP to indicate the changed precondition status. |
| 6 |  |  | 200 OK | The SS responds PRACK with 200 OK and answers the second SDP (if any) with mirroring its contents. |
| 7 |  |  | UPDATE | UE sends an UPDATE after having reserved the resources for video if meeting the preconditions was not already indicated in step 2 or 5. |
| 8 |  |  | 200 OK | The SS responds UPDATE, if received, with 200 OK and indicates having reserved the resources |
| 9 |  |  | 200 OK | The SS responds re-INVITE with 200 OK |
| 10 |  |  | ACK | The UE acknowledges the receipt of 200 OK for re-INVITE |
| 10A |  |  | Make UE release video from the media call |  |
| 11 |  |  | INVITE | UE sends re-INVITE with a SDP offer indicating that the video component is removed from the call |
| 12 |  |  | 100 Trying | The SS responds with a 100 Trying provisional response |
| 12A |  |  | SS deactivates the EPS bearer for video |  |
| 13 |  |  | 200 OK | The SS responds re-INVITE with 200 OK |
| 14 |  |  | ACK | The UE acknowledges the receipt of 200 OK for re-INVITE |
| 15-19 |  |  | Steps defined in annex C.32 | MO Call release |

NOTE:	The default messages contents in annex A are used with condition “IMS security“ or “GIBA” when applicable

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE for MO Call” in annex A.2.1 with condition A5 (re-INVITE within a dialog), A11 (INVITE for creating a video call) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 4] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000 [Note 3] - a=fmtp: (format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Media description: - m=video (transport port) RTP/AVPF (fmt) or RTP/AVP (fmt) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=tcap:1 RTP/AVPF [Note 2] - a=pcfg:1 t=1 [Note 2] - a=rtpmap: (payload type) H264/90000 - a=fmtp: (format) profile-level-id= (att-field) Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The tcap/pcfg attributes are present if RTP/AVP is present on the m line. Note 3: The AMR channel number shall be “/1” or omitted. Note 4: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. |

183 Session Progress (Step 4)

Use the default message "183 Session Progress" in annex A.2.3 with condition A15 (183 sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:30 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 1] - b=AS: (bandwidth-value) [Note 1] - b=RS: (bandwidth-value) [Note 1] - b=RR: (bandwidth-value) [Note 1] Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000/1 [Note 1] - a=fmtp: (format) mode-change-capability=2; max-red=220 [Note 1] - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Media description: - m=video (transport port) RTP/AVPF (fmt) [Note 1] - b=AS: (bandwidth-value) [Note 1] - b=RS: (bandwidth-value) [Note 1] - b=RR: (bandwidth-value) [Note 1] Attributes for media: - a=acfg:1 t=1 [Note 2] - a=rtpmap: (payload type) H264/90000 [Note 1] - a=fmtp: (format) (format specific parameters) [Note 1] Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv - a=conf:qos remote sendrecv Note 1: The value for fmt, bandwidth, payload type, format and format specific parameters copied from step 2. Note 2: Present if tcap/pcfg attributes were included in step 2. |

PRACK (Step 5)

Use the default message “PRACK” in annex A.2.4 with the exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported option-tag | precondition [Note 4] |
| Message-body | Header optional Contents if present: The following SDP types and values shall be present. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 2] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000 [Note 3] - a=fmtp: (format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Media description: - m=video (transport port) RTP/AVPF (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) H264/90000 - a=fmtp: (format) profile-level-id= (att-field) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. Note 3: The AMR channel number shall be “/1” or omitted. Note 4: Shall be present if message-body present. |

200 OK for PRACK (Step 6)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition (shall be present if SDP message-body present) |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length | Contents if header Content-Type is present: |
| Value | length of message-body |
| Message-body | Header present if Prack (step 5) contained SDP. Contents if present: SDP body of the 200 response copied from the received PRACK and modified as follows: - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - IP address on "c=" line and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media; Attributes for preconditions (video): - a=curr:qos remote sendrecv |

UPDATE (Step 7)

Use the default message “UPDATE” in annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | Same contents as specified in step 5. |
| Message-body | Same contents as specified in step 5. |

200 OK for UPDATE (Step 8)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition (shall be present if SDP message-body present) |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length | Contents if header Content-Type is present: |
| Value | length of message-body |
| Message-body | SDP body of the 200 response copied from the received UPDATE and modified as follows: - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - IP address on "c=" line and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media; Attributes for preconditions (video): - a=curr:qos remote sendrecv |

200 OK for INVITE (Step 9)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A23 (Response sent for re-INVITE within an established dialog).

ACK (Step 10)

Use the default message “ACK” in annex A.2.7 with condition A5.

INVITE (Step 11)

Use the default message “INVITE for MO Call” in annex A.2.1 with condition A5 (re-INVITE within a dialog) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 2] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR:(bandwidth-value) Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000 [Note 3] - a=fmtp: (format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Media description: - m=video 0 RTP/AVPF (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] Attributes for media: - a=rtpmap: (payload type) - a=fmtp: (format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. Note 3: The AMR channel number shall be “/1” or omitted. |

200 OK for INVITE (Step 13)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A23 (Response sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| Value | length of message-body |
| Message-body | SDP body of the 200 response copied from the received INVITE and modified as follows: - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - IP address on "c=" line and, for audio, transport port on "m=" line changed to indicate to which IP address and port the UE should start sending the media; |

ACK (Step 14)

Use the default message “ACK” in annex A.2.7 with condition A5.

<End of modification>

<Start of modification>

### 17.2.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF, registered to IMS services and established an MT MTSI speech call, by executing the generic test procedure in Annex C.11 steps 1 to 13.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-9) UE executes the procedures described in TS 36.508 [94] table 4.5A.12.3-1, steps 1 to 12. In detail the following steps are done in IMS:

1) SS sends a re-INVITE request to the UE.

2) Void

2a) SS may receive 100 Trying from the UE.

3) SS receives 183 Session Progress from the UE.

4) SS sends PRACK to the UE to acknowledge the 183 Session Progress.

5) SS receives 200 OK for PRACK from the UE.

6) SS sends UPDATE to the UE, with SDP indicating that precondition is met on the server side.

7) SS receives 200 OK for UPDATE from the UE.

7A) The UE accepts the session invite.

8) SS expects and receives 200 OK for re-INVITE from the UE.

9) SS sends ACK to the UE.

10) SS sends a re-INVITE to the UE with a SDP offer indicating that the video component is removed from the call.

11) SS expects and receives 200 OK for re-INVITE from the UE.

11A) SS deactivates the EPS bearer corresponding to the video stream and releases the associated radio resources by applying the procedure described in TS 36.508 [94] clause 4.5A.15

12) SS sends ACK to the UE.

13-16) MT Call release according to procedure C.33

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | SS sends re-INVITE with second SDP offer to add video. |
| 2 |  |  |  | Void. |
| 2a |  |  | 100 Trying | (Optional) The UE responds with a 100 Trying provisional response. |
| 3 |  |  | 183 Session Progress | The UE responds to re-INVITE by sending 183 response reliably with the SDP answer |
| 4 |  |  | PRACK | SS acknowledges the receipt of 183 response from the UE |
| 5 |  |  | 200 OK | The UE acknowledges the PRACK with 200 OK. |
| 6 |  |  | UPDATE | SS sends an UPDATE with SDP offer indicating SS reserved resources. |
| 7 |  |  | 200 OK | The UE acknowledges the UPDATE with 200 OK and includes SDP answer to acknowledge its current precondition status. |
| 7A |  |  |  | Make UE accept the speech and video offer (Note 1). |
| 8 |  |  | 200 OK | The UE responds to the re-INVITE with a 200 OK final response. |
| 9 |  |  | ACK | The SS acknowledges the receipt of 200 OK for the re-INVITE. |
| 10 |  |  | INVITE | SS sends a re-INVITE with a SDP offer indicating that the video component is removed from the call |
| 10A |  |  | 100 Trying | (Optional) The UE responds with a 100 Trying provisional response. |
| 11 |  |  | 200 OK | The UE responds to the re-INVITE with a 200 OK final response. |
| 11A |  |  | SS deactivates the EPS bearer for video |  |
| 12 |  |  | ACK | The SS acknowledges the receipt of 200 OK for the re-INVITE. |
| 13 |  |  |  | Void |
| 14-16 |  |  | Steps defined in annex C.33 | MT Call release |
| NOTE 1: This can be done with AT command AT+CCMMD if supported by the UE or with an appropriate MMI command otherwise.. |

NOTE:	The default messages contents in annex A are used with condition “IMS security“or “GIBA” when applicable

Specific Message Content

INVITE (Step 1)

Use the default message “INVITE for MT Call” in annex A.2.9 with condition A5 (re-INVITE within a dialog), condition A8 (INVITE for creating a video stream) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:352 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 97 - b=AS: 37 - b=RS: 0 - b=RR: 2000 Attributes for media: - a=rtpmap: 97 AMR-WB/16000/1 - a=fmtp: 97 mode-change-capability=2; max-red=220 - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv Media description: - m=video (transport port) RTP/AVPF 101 - b=AS: 315 - b=RS: 0 - b=RR: 2500 Attributes for media: - a=rtpmap: 101 H264/90000 - a=fmtp: 101 packetization-mode=0;profile-level-id=42e00c; \ sprop-parameter-sets=J0LgDJWgUH6Af1A=,KM46gA== - a=rtcp-fb:* trr-int 5000 - a=rtcp-fb:* nack - a=rtcp-fb:* nack pli - a=rtcp-fb:* ccm fir - a=rtcp-fb:* ccm tmmbr Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv |

183 Session Progress (step 3)

Use the default message "183 Session Progress" in annex A.2.3 with condition A15 (183 sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Status-Line |  |
| Reason-Phrase | Not checked |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 3] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR-WB/16000 [Note 2] - a=fmtp:(format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Media description: - m=video (transport port) RTP/AVPF (fmt) - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) H264/90000 - a=fmtp: (format) packetization-mode=0;profile-level-id= (att-field); \ Attributes for preconditions: - a=curr:qos local none or a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv - a=conf:qos remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. Note 3: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. |

PRACK (step 4)

Use the default message "PRACK" in annex A.2.4. No content body is included in this PRACK message.

200 OK (step 5)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1.

UPDATE (step 6)

Use the default message "UPDATE" in annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:352 Time description: -t=0 0 Media description: - m=audio (transport port) RTP/AVP 97 - b=AS: 37 - b=RS: 0 - b=RR: 2000 Attributes for media: - a=rtpmap:97 AMR-WB/16000/1 - a=fmtp:97 mode-change-capability=2; max-red=220 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Media description: - m=video (transport port) RTP/AVPF 101 - b=AS: 315 - b=RS: 0 - b=RR: 2500 Attributes for media: - a=rtpmap: 101 H264/90000 - a=fmtp: 101 packetization-mode=0;profile-level-id=42e00c; \ sprop-parameter-sets=J0LgDJWgUH6Af1A=,KM46gA== - a=rtcp-fb:* trr-int 5000 - a=rtcp-fb:* nack - a=rtcp-fb:* nack pli - a=rtcp-fb:* ccm fir - a=rtcp-fb:* ccm tmmbr Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none or curr:qos remote sendrecv [Note 1] - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: Use the value (none/sendrecv) received from 183 Session Progress and attribute a=curr:qos local. |

200 OK for UPDATE (step 7)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length | header shall be present if UE uses TCP to send this message and if there is a message body |
| value | length of message-body |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 3] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR-WB/16000 [Note 2] - a=fmtp:(format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Media description: - m=video (transport port) RTP/AVPF (fmt) - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) H264/90000 - a=fmtp: (format) packetization-mode=0;profile-level-id= (att-field); \ Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. Note 3: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one |

200 OK for INVITE (Step 8)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1 with condition A23 (Response sent for re-INVITE within an established dialog).

ACK (Step 9)

Use the default message “ACK” in annex A.2.7 with condition A5.

ACK (step 9)

Use the default message "ACK" in annex A.2.7.

INVITE (step 10)

Use the default message “INVITE for MT Call” in annex A.2.9 with condition A5 (re-INVITE within a dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: v=0 "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one s=- c=IN (addrtype) (connection-address for SS) b=AS:37 Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP 97 b=AS: 37 b=RS: 0 b=RR: 2000 Attributes for media: a=rtpmap:97 AMR-WB/16000/1 a=fmtp:97 mode-change-capability=2; max-red=220 a=ptime:20 a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv Media description: - m=video 0 RTP/AVPF 101 - b=AS: 315 - b=RS: 0 - b=RR: 2500 Attributes for media: - a=rtpmap: 101 H264/90000 - a=fmtp: 101 packetization-mode=0;profile-level-id=42e00c; \ sprop-parameter-sets=J0LgDJWgUH6Af1A=,KM46gA== Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv |

100 Trying (step 10A)

Use the default message "100 Trying for INVITE" in annex A.2.2.

200 OK for INVITE (step 11)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1 with condition A23 (Response sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length | header shall be present if UE uses TCP to send this message and if there is a message body |
| value | length of message-body |
| Message-body | SDP body not checked. |

ACK (step 12)

Use the default message "ACK" in annex A.2.7 with condition A5.

BYE (step 14)

Use the default message "BYE" in annex A.2.8.

200 OK (step 15)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1.

<End of modification>

<Start of modification>

### 18.2.4	Method of test

Initial conditions

UE contains either SIM application (GIBA), ISIM and USIM applications or only USIM application on UICC. UE has activated a PDP context, discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure

1)	SS sends a Short Message included in the message-body of MESSAGE.

2)	UE responds with a 200 OK.

3)	When the payload is extracted, the UE responds with a delivery report included in the message-body of MESSAGE.

4)	SS responds with a 202 ACCEPTED.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | MESSAGE | The SS sends a Short Message. |
| 2 |  |  | 200 OK | The UE responds with 200 OK. |
| 3 |  |  | MESSAGE | The UE responds with a delivery report. |
| 4 |  |  | 202 ACCEPTED | The SS sends an accepted response. |

Specific Message Contents

MESSAGE (Step 1)

Use the default message “MESSAGE for MT SMS” in annex A.7.1.

200 OK (Step 2)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

MESSAGE (Step 3)

Use the default message “MESSAGE for delivery report” in annex A.7.2.

202 ACCEPTED (Step 4)

Use the default message “202 ACCEPTED” in annex A.3.3

<End of modification>

<Start of modification>

### 18.2a.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has activated a PDP context, discovered P-CSCF, and registered to IMS services.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration.

Test procedure

1)	SS sends a first segment of a concatenated SMS in the message-body of SIP_MESSAGE.

2)	UE responds with a 200 OK.

3)	When the payload is extracted, the UE responds with a delivery report included in the message-body of MESSAGE.

4)	SS responds with a 202 ACCEPTED.

5)	SS sends a second segment of a concatenated SMS in the message-body of SIP_MESSAGE.

6)	The UE responds with a 200 OK.

7)	When the payload is extracted, the UE responds with a delivery report included in the message-body of MESSAGE.

8)	SS responds with a 202 ACCEPTED.

9)	SS sends a fnal segment of a concatenated SMS in the message-body of SIP_MESSAGE.

10)	The UE responds with a 200 OK.

11)	When the payload is extracted, the UE responds with a delivery report included in the message-body of MESSAGE.

12)	SS responds with a 202 ACCEPTED.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | SIP MESSAGE request | SS sends a first segment of a concatenated SMS in the message-body of SIP MESSAGE |
| 2 |  |  | 200 OK | UE responds with 200 OK |
| 3 |  |  | SIP MESSAGE request | UE responds with a delivery report included in the message-body of MESSAGE |
| 4 |  |  | 202 Accepted | SS responds with 202 Accepted |
| 5 |  |  | SIP MESSAGE request | SS sends a second segment of a concatenated SMS in the message-body of SIP MESSAGE |
| 6 |  |  | 200 OK | UE responds with 200 OK |
| 7 |  |  | SIP MESSAGE request | UE responds with a delivery report included in the message-body of MESSAGE |
| 8 |  |  | 202 Accepted | SS responds with 202 Accepted |
| 9 |  |  | SIP MESSAGE request | SS sends a final segment of a concatenated SMS in the message-body of SIP MESSAGE |
| 10 |  |  | 200 OK | UE responds with 200 OK |
| 11 |  |  | SIP MESSAGE request | UE responds with a delivery report included in the message-body of MESSAGE |
| 12 |  |  | 202 Accepted | SS responds with 202 Accepted |

Specific Message Contents

SIP MESSAGE request (Step 1)

Use the default message “MESSAGE for MT SMS” in annex A.7.1 with the following exceptions.

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Message-body |  | - TP-RP=’0’B (TP Reply Path parameter is not set in this SMS SUBMIT/DELIVER) - TP-MMS=’0’B (More messages are waiting for the MS in this SC) - TP-UDHI=’1’B (The beginning of the TP UD field contains a Header in addition to the short message.) - TP-PID=’00000000’B - TP-UD - Length of User Data Header (UDHL)=5 - Information Element Identifier (IEI)=0x00 (Concatenated short messages, 8-bit reference number) - Length of Information Element (IEIDL)=3 - Concatenated short message reference number=any allowed value - Maximum number of short messages in the concatenated short message=3 - Sequence number of the current short message=1 |  | TS 24.011 [92] TS 23.040 [93] |

200 OK (Step 2)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

SIP MESSAGE (Step 3)

Use the default message “MESSAGE for delivery report” in annex A.7.2.

202 ACCEPTED (Step 4)

Use the default message “202 ACCEPTED” in annex A.3.3

SIP MESSAGE request (Step 5)

Use the default message “MESSAGE for MT SMS” in annex A.7.1 with the following exceptions.

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Message-body |  | - TP-RP=’0’B (TP Reply Path parameter is not set in this SMS SUBMIT/DELIVER) - TP-MMS=’0’B (More messages are waiting for the MS in this SC) - TP-UDHI=’1’B (The beginning of the TP UD field contains a Header in addition to the short message.) - TP-PID=’00000000’B - TP-UD - Length of User Data Header (UDHL)=5 - Information Element Identifier (IEI)=0x00 (Concatenated short messages, 8-bit reference number) - Length of Information Element (IEIDL)=3 - Concatenated short message reference number=The same value sent in the step1 - Maximum number of short messages in the concatenated short message=3 - Sequence number of the current short message=2 |  | TS 24.011 [92] TS 23.040 [93] |

200 OK (Step 6)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

SIP MESSAGE (Step 7)

Use the default message “MESSAGE for delivery report” in annex A.7.2.

202 ACCEPTED (Step 8)

Use the default message “202 ACCEPTED” in annex A.3.3

SIP MESSAGE request (Step 9)

Use the default message “MESSAGE for MT SMS” in annex A.7.1 with the following exceptions.

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Message-body |  | - TP-RP=’0’B (TP Reply Path parameter is not set in this SMS SUBMIT/DELIVER) - TP-MMS=’1’B (No more messages are waiting for the MS in this SC) - TP-UDHI=’1’B (The beginning of the TP UD field contains a Header in addition to the short message.) - TP-PID=’00000000’B - TP-UD - Length of User Data Header (UDHL)=5 - Information Element Identifier (IEI)=0x00 (Concatenated short messages, 8-bit reference number) - Length of Information Element (IEIDL)=3 - Concatenated short message reference number=The same value sent in the step5 - Maximum number of short messages in the concatenated short message=3 - Sequence number of the current short message=3 |  | TS 24.011 [92] TS 23.040 [93] |

200 OK (Step 10)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

SIP MESSAGE (Step 11)

Use the default message “MESSAGE for delivery report” in annex A.7.2.

202 ACCEPTED (Step 12)

Use the default message “202 ACCEPTED” in annex A.3.3

<End of modification>

<Start of modification>

#### 19.1.6.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. In the E-UTRA attach SS has indicated to the UE that the NW is VPLMN and the cell supports E-UTRA emergency bearers. UE is registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure

1)-12) UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 12 and parallel behaviour steps 1 for EPS emergency bearer context activation,

13) SS waits for the UE to send an initial REGISTER request containing “sos” SIP URI parameter in the Contact header field.

14) The SS responds to the REGISTER request with a 420 Bad Extension response,

15) The UE initiates IMS registration indicating support of GIBA. SS waits for the UE to send an initial REGISTER request.

16) The SS responds to the REGISTER request with valid 200 OK response,

17) The SS waits for the UE to send an INVITE request.

18)-20A) UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 13 to 15 and parallel behaviour Steps 2-5 defined in annex C.22 of TS 34.229-1 for IMS Emergency call for EPS is established,

21)-24A) The UE releases the call.

25)-26) The SS deactivates EPS emergency bearer context.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-12 |  |  | Steps defined in TS 36.508 [94] table 4.5A.4.3-1 | EPS Bearer Activation procedure and IP address allocation according TS 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 13 |  |  | REGISTER | UE sends initial registration for IMS services. |
| 14 |  |  | 420 Bad Extension | The SS responds with a failure. |
| 15 |  |  | REGISTER | The UE sends initial registration for IMS services indicating support for GIBA procedure by not including an Authorization header field. |
| 16 |  |  | 200 OK | The SS responds with 200 OK. |
| 17 |  |  | INVITE | UE sends INVITE request. |
| 18-20A |  |  | Steps 2 to 5 defined in annex C.22 | IMS emergency call setup with PSAP |
| 21-24A |  |  | Steps defined in annex C.32 | The UE releases the call |
| 25-26 |  |  | EPS emergency bearer context deactivation by the SS. | EPS Bearer Deactivation procedure according TS 36.508 [94] subclause 4.5A.15, applied to the identity of the Default EPS Bearer of the emergency PDN. |

Specific Message Contents

REGISTER (Step 13)

Use the default message “REGISTER” in annex A.1.1 with condition A1 "Initial unprotected REGISTER" and A7 “Initial unprotected or subsequent REGISTER for emergency registration”

420 Bad Extension for REGISTER (Step 14)

Use the default message “420 Bad Extension for REGISTER” in annex A.1.8 with condition A1 “IMS emergency registration failure for an anonymous emergency call”.

REGISTER (Step 15)

Use the default message “REGISTER” in annex A.1.1 with condition A3 "GIBA" and A7 “Initial unprotected or subsequent REGISTER for emergency registration”.

200 OK for REGISTER (Step 16)

Use the default message “200 OK for REGISTER” in annex A.1.3 with condition A3 "Response for an emergency registration"

INVITE (Step 17)

Use the default message “INVITE” in annex A.2.1 with condition A19 “INVITE for creating an emergency session within an emergency registration using GIBA”.

<End of modification>

<Start of modification>

#### 19.4.5.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE is  registered to IMS services, by executing the generic test procedure in Annex C.2 and  it is attached to the HPLMN E-UTRA service as provided by SS. In the attach SS has indicated that the cell supports E-UTRA emergency bearers.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols.

Test environment shall be set up to send in response to Emergency REGISTER message a 401 Unauthorized such that UE will not be able to establish temporary set of security associations.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	IMS emergency call is initiated on the UE.

2-13)	UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 12 (parallel behaviour steps 1) for EPS emergency bearer context activation, IMS emergency speech call establishment with PSAP

14)	UE sends initial REGISTER message.

15)	The SS responds to the initial REGISTER request with a valid 401 Unauthorized response.

16)	The SS waits for the UE to set up a temporary set of security associations and to send another REGISTER request, over those security associations.

17)	The SS responds REGISTER message with 403 Forbidden and ignores any further REGISTER message reception.

18-22)	UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 13 to 15 (parallel behaviour steps 6-10) IMS emergency speech call establishment with PSAP.

23-23F)	The Call is released on the UE using annex C.32a procedure which includes the Emergency Bearer context deactivation

24-25)	Void

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | User initiates an emergency call |  |
| 2-13 |  |  | EPS emergency bearer context activation by the UE. | Referred from 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. Steps 2-10 of the parallel behaviour in Table 4.5A.4.3-2 is replaced by below steps 14-22. |
| 14 |  |  | REGISTER | The UE sends initial IMS emergency registration |
| 15 |  |  | 401 Unauthorized |  |
| 16 |  |  | REGISTER | The UE completes the security negotiation procedures, sets up a temporary set of SAs and uses those for sending another REGISTER with AKAv1-MD5 credentials. |
|  |  |  |  | Note: From this point onward the SS shall ignore any Registration message sent by the UE. |
| 17 |  |  | 403 Forbidden The following messages are exchanged on non protected port. | The SS sends this message to get the UE in a stable state. |
| 18-22 |  |  | Steps defined in annex C.22 | IMS emergency call setup with PSAP. Referred from 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 23-23F |  |  | Steps defined in annex C.32a | The UE releases the call |
| 24-25 |  |  | Void. |  |

Specific Message Contents

REGISTER (Step 14)

Use the default message “REGISTER” in annex A.1.1 with condition A1.

REGISTER (Step 16)

Use the default message “REGISTER” in annex A.1.1 with conditions A2 and A17.

INVITE (Step 18 resp step 1 of Annex C.22)

Use the default message “INVITE for MO call setup” in annex A.2.1. with the following conditions:

-	A6 “INVITE for creating an emergency session in case of no registration”

180 Ringing for INVITE (Step 20 resp step 3 of Annex C.22)

Use the default message “180 Ringing for INVITE” in annex A.2.6 with the following conditions:

-	A4 “180 sent by the SS when setting up an emergency call or a non-UE detectable emergency call”

-	A7 “Response sent by SS for emergency call without emergency registration”

200 OK for INVITE (Step 20 resp step 4 of Annex C.22)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following conditions:

- A7 “Response sent by SS for INVITE for emergency call without emergency registration”

BYE (Step 23A)

Use the default message “BYE” in annex A.2.8 with the following conditions:

-	A6 “BYE for emergency call with no registration”.

<End of modification>

<Start of modification>

#### 19.4.6.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. In the E-UTRA attach SS has indicated to the UE that the NW is VPLMN and the cell supports E-UTRA emergency bearers. UE is registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure

1)-12) UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 12 and parallel behaviour steps 1 for EPS emergency bearer context activation,

13) SS waits for the UE to send an initial REGISTER request containing “sos” SIP URI parameter in the Contact header field.

14) The SS responds to the REGISTER request with a 403 Forbidden response,

15) The SS waits for the UE to send an INVITE request.

16)-18A) UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 13 to 15 and parallel behaviour Steps 2-5 defined in annex C.22 of TS 34.229-1 for IMS Emergency call for EPS is established,

19)-23B) The UE releases the call as defined in annex C.32a which includes the Emergency Bearer context deactivation.

24)-25) Void.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-12 |  |  | Steps defined in TS 36.508 [94] table 4.5A.4.3-1 | EPS Bearer Activation procedure and IP address allocation according TS 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 13 |  |  | REGISTER | UE sends initial registration for IMS services. |
| 14 |  |  | 403 Forbidden The following messages are exchanged on non protected port. | The SS responds with a failure. |
| 15 |  |  | INVITE | UE sends INVITE request without emergency registration. |
| 16-18A |  |  | Steps 2 to 5 defined in annex C.22 | IMS emergency call setup with PSAP |
| 19-23B |  |  | Steps defined in annex C.32a | The UE releases the call |
| 24-25 |  |  | Void. |  |

Specific Message Contents

REGISTER (Step 13)

Use the default message “REGISTER” in annex A.1.1 with condition A1 "Initial unprotected REGISTER" and A7 “Initial unprotected or subsequent REGISTER for emergency registration”

403 Forbidden for REGISTER (Step 14)

Use the default message “403 FORBIDDEN” in annex A.3.2 with condition A1 “IMS emergency registration for an  anonymous emergency call”.

INVITE (Step 15)

Use the default message “INVITE” in annex A.2.1 with condition A6 “INVITE for creating an emergency session in case of no registration”.

BYE (Step 20)

Use the default message “BYE” in annex A.2.8 with condition A6 “BYE for emergency call with no registration”.

<End of modification>

<Start of modification>

#### 19.5.6.4	Method of test

Initial conditions

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

UE contains either ISIM and USIM applications or only USIM application on UICC. In the E-UTRA attach SS has indicated to the UE that the cell supports E-UTRA emergency bearers. UE is registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step. The UE has performed EPS emergency bearer context activation, IMS emergency registration and the subsequent IMS emergency call, s described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 15. When performing the steps of Annex C.20 the SS sets the expiration time to 120 seconds in Step 4. Thereafter the UE has initiated an emergency call by executing the generic test procedure in Annex C.22 up to the last step.

Test procedure

1)	When half of the initial emergency registration time has expired and while emergency call is still going on SS receives REGISTER request from the UE.

2)	SS responds to the REGISTER request with a valid 401 Unauthorized response, headers populated according to the 401 response common message definition.

3)	SS waits for the UE to set up a new set of security associations and send another REGISTER request, over those security associations.

4)	 The SS responds with 200 OK over the new security association, setting the new expiration time as 1200 seconds

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | REGISTER | UE re-registers to the emergency services 60 seconds before the expected expiration. |
| 2 |  |  | 401 Unauthorized | The SS responds with a valid AKAv1-MD5 authentication challenge and security mechanisms supported by the network. |
| 3 |  |  | REGISTER | UE completes the security negotiation procedures, sets up a new temporary set of SAs and uses those for sending another REGISTER with AKAv1-MD5 credentials. |
| 4 |  |  | 200 OK | The SS responds with 200 OK. |

Specific Message Contents

REGISTER (Step 1)

Use the default message “REGISTER” in annex A.1.1. with conditions A2 "Subsequent REGISTER sent over security associations” and A17 "UE initiated IMS re-registration or de-registration" and the following exceptions applying:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | SIP URI with IP address or FQDN and protected server port of UE. The SIP URI shall contain the sos URI parameter. |
| Security-Client |  |
| spi-c | new SPI number of the inbound SA at the protected client port |
| spi-s | new SPI number of the inbound SA at the protected server port |
| port-c | new protected client port needed for the setup of new pairs of security associations |
| port-s | Same value as in the previous REGISTER |

401 Unauthorized for REGISTER (Step 2)

Use the default message “401 Unauthorized for REGISTER” in annex A.1.2 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Security-Server |  |
| spi-c | new SPI number of the inbound SA at the protected client port |
| spi-s | new SPI number of the inbound SA at the protected server port |
| port-c | new protected client port needed for the setup of new pairs of security associations |
| port-s | Same value as in the previous Security-Server headers |
| WWW-Authenticate |  |
| nonce | Base 64 encoding of a new RAND and AUTN |

REGISTER (Step 3)

Use the default message “REGISTER” in annex A.1.1 like in step 1 above. The only difference is that the response value within Authorization header shall have been recalculated based on the nonce received from SS within 401 response.

200 OK for REGISTER (Step 4)

Use the default message “200 OK for REGISTER” in annex A.1.3 with condition A3 “Response for an emergency registration” and the expires parameter of Contact header set to 1200.

<End of modification>

<Start of modification>

#### 19.5.7.4	Method of test

Initial conditions

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

UE contains either ISIM and USIM applications or only USIM application on UICC. In the E-UTRA attach SS has indicated to the UE that the cell supports E-UTRA emergency bearers. UE is registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

Test procedure

Expected sequence, procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Emergency call is initiated on the UE as described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 15 for EPS emergency bearer context activation and subsequent IMS emergency registration. The UE registers to IMS emergency services, by executing the generic test procedure in Annex C.20 up to the last step with the exception that the SS sets the expiration time to 30 seconds in Step 1 of Expected Sequence.

2)	After completing the IMS emergency registration UE starts the process of initiating an emergency call, by executing the generic test procedure in steps 1-3 of Annex C.22.

2A-2D)	The UE re-registers for IMS emergency services.

3)	The SS accepts the call.

4)	The UE acknowledges call acceptance.

5)	The UE is made to release the call.

6)	The SS answers the call release.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Steps defined in annex C.20 | EPS emergency bearer context activation and subsequent IMS emergency registration by the UE. SS sets the expiration time of emergency registration to 30 seconds. |
| 2 |  |  | Steps 1-3 defined in annex C.22. | IMS emergency call setup with PSAP using preconditions |
| 2A |  |  | REGISTER | UE re-registers to the emergency services in a time span between half of the expiration time and the full expiration time. Note: in this test case, the re-registration time is set to an untypically short value of 30 seconds. As there are no requirements on the duration of a re-registration procedure it is only checked that the re-registration procedure starts between half of the expiration time and the full expiration time. |
| 2B |  |  | 401 Unauthorized | The SS responds with a valid AKAv1-MD5 authentication challenge and security mechanisms supported by the network. |
| 2C |  |  | REGISTER | UE completes the security negotiation procedures, sets up a new temporary set of SAs and uses those for sending another REGISTER with AKAv1-MD5 credentials. |
| 2D |  |  | 200 OK | The SS responds with 200 OK. |
| 3 |  |  | 200 OK | Response for INVITE sent in step 2 Note: 200 OK will be sent using previous socket connection before using old SA |
| 4 |  |  | ACK | Response from UE to confirm the dialog |
| 5 |  |  | BYE | The UE releases the call with BYE |
| 6 |  |  | 200 OK | The SS sends 200 OK for BYE |

Specific Message Contents

REGISTER (Step 2A)

Use the default message “REGISTER” in annex A.1.1. with condition A2 "Subsequent REGISTER sent over security associations” and A17 "UE initiated IMS re-registration or de-registration" and the following exceptions applying:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | SIP URI with IP address or FQDN and protected server port of UE. The SIP URI shall contain the sos URI parameter. |
| Security-Client |  |
| spi-c | new SPI number of the inbound SA at the protected client port |
| spi-s | new SPI number of the inbound SA at the protected server port |
| port-c | new protected client port needed for the setup of new pairs of security associations |
| port-s | Same value as in the previous REGISTER |

401 Unauthorized for REGISTER (Step 2B)

Use the default message “401 Unauthorized for REGISTER” in annex A.1.2 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Security-Server |  |
| spi-c | new SPI number of the inbound SA at the protected client port |
| spi-s | new SPI number of the inbound SA at the protected server port |
| port-c | new protected client port needed for the setup of new pairs of security associations |
| port-s | Same value as in the previous Security-Server headers |
| WWW-Authenticate |  |
| nonce | Base 64 encoding of a new RAND and AUTN |

REGISTER (Step 2C)

Use the default message “REGISTER” in annex A.1.1 like in step 1 above. The only difference is that the response value within Authorization header shall have been recalculated based on the nonce received from SS within 401 response.

200 OK for REGISTER (Step 2D)

Use the default message “200 OK for REGISTER” in annex A.1.3 with condition A3 “Response for an emergency registration” and the expires parameter of Contact header set to 1200.

<End of modification>

<Start of modification>

### 21.1.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application with eCall subscription on UICC. The UE has discovered P-CSCF, and registered to IMS services.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall over IMS initiated at UE

2)	Emergency registration according to C.20 is executed

3)	SS waits for UE to send an INVITE request.

4)	SS sends 200 OK with ACK.

5)	Void

6)-9)	MT Call release according to steps 1-4 of procedure C.33.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | UE is triggered to initiate a manual eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1 for a UE with E-UTRA support. |
| 6-8 |  |  | Steps 1-3 defined in C.47 | eCall setup |
| 9-12 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

Specific Message Contents

Step 6 as specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A20.

Step 7 as specified in annex C.47 (Step 2), which is referring to A.3.1 default message content of 200 OK with conditions A12 and A13.

BYE (Step 9)

Use the default message “BYE” in annex A.2.8.

200 OK for BYE (Step 10)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

<End of modification>

<Start of modification>

### 21.2.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription. The UE has discovered P-CSCF, and registered to IMS services.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall over IMS initiated at UE

2)	Emergency registration according to C.20 is executed

3)	SS waits for UE to send an INVITE request.

4)	SS sends 200 OK with ACK.

5)	Void

6)-9)	MT Call release according to steps 1-4 of procedure C.33.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | UE is triggered to initiate an automatic eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1 for a UE with E-UTRA support. |
| 6-8 |  |  | Steps 1-3 defined in C.47 | eCall setup |
| 9-12 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

Specific Message Contents

Step 6 as specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A21.

Step 7 as specified in annex C.47 (Step 2), which is referring to A.3.1 default message content of 200 OK with conditions A12 and A13.

BYE (Step 9)

Use the default message “BYE” in annex A.2.8.

200 OK for BYE (Step 10)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

<End of modification>

<Start of modification>

### 21.4.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription. UE has discovered P-CSCF, and registered to IMS services.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall initiated at UE.

2)	Emergency registration according to C.20 is executed

3)	SS waits for UE to send an INVITE request.

4)	SS sends with 200 OK.

5)	SS waits for ACK.

6)	SS sends INFO request to transfer MSD.

7)	SS expects and receives 200 OK.

8)	SS expects and receives INFO request from the UE with MSD contents.

9)	SS responds with 200 OK for INFO request from UE.

10)	SS sends BYE to the UE.

11)	SS expects and receives 200 OK for BYE from the UE.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt manual eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1 for a UE with E-UTRA support. |
| 6-12 |  |  | Steps 1-7 defined in C.47 | eCall setup |
| 13 |  |  | BYE | The SS sends BYE to release the call. |
| 14 |  |  | 200 OK | The UE sends 200 OK for the BYE request and ends the call. |

Specific Message Contents:

Step 6 as specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A20.

Step 7 as specified in annex C.47 (Step 2), which is referring to A.3.1 default message content of 200 OK with conditions A12 and A13.

Step 11 as specified in annex C.47 (Step 6), which is referring to A.2.19 default message content of INFO for MO with conditions A1 and A3

<End of modification>

<Start of modification>

### 21.5.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription. UE has discovered P-CSCF, and registered to IMS services.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall over IMS initiated at UE

2-5)	UE executes the procedures described in TS 36.508 [94] table 4.5A.26.3-1 steps 2 to 15 and parallel behaviour steps 1-4 for EPS emergency bearer context activation, and subsequent IMS emergency registration,

6-12)	eCall is established successfully and MSD transferred successfully using INFO procedure as defined in C.47

13)	SS sends BYE to the UE

14)	SS expects and receives 200 OK for BYE from the UE

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE to initiate automatic eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | EPS emergency bearer context activation and subsequent IMS emergency registration by the UE. Referred from 36.508 [94] table 4.5A.26.3-1 for a UE with E-UTRA support. |
| 6-12 |  |  | Steps 1-7 defined in C.47 | eCall Setup and MSD transfer using INFO message |
| 13 |  |  | BYE | The SS sends BYE to release the call. |
| 14 |  |  | 200 OK | The UE sends 200 OK for the BYE request and ends the call. |

Specific Message Contents:

Step 6 as specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A21.

Step 7 as specified in annex C.47 (Step 2), which is referring to A.3.1 default message content of 200 OK with condition A12 and A13.

Step 11 as specified in annex C.47 (Step 6), which is referring to A.2.19 default message content of INFO for MO with conditions A2 and A3

<End of modification>

<Start of modification>

### 21.6.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription. UE has discovered P-CSCF, and registered to IMS services.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1) 	Automatic eCall over IMS initiated at UE

2-5)	UE executes the procedures described in TS 36.508 [94] table 4.5A.26.3-1 steps 2 to 15 and parallel behaviour steps 1-4 for EPS emergency bearer context activation, and subsequent IMS emergency registration,

6-7)	eCall is established successfully

8-11)	Execute MSD transfer procedure as defined in Annex C.47 from steps 4-7

12)	SS sends BYE to the UE

13)	SS expects and receives 200 OK for BYE from the UE

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt Automatic eCall |  |
| 2-5 |  |  | Step 1-4 defined in C.20 | EPS emergency bearer context activation and subsequent IMS emergency registration by the UE. Referred from 36.508 [94] table 4.5A.26.3-1 for a UE with E-UTRA support. |
| 6-7A |  |  | Steps 1-3 defined in C.47 | eCall over IMS initiated automatically. Referred from 36.508 [94] table 4.5A.26.3-1 for a UE with E-UTRA support. |
| 8-11 |  |  | Steps 4-7 defined in C.47 | eCall over IMS initiated automatically. Referred from 36.508 [94] table 4.5A.26 for a UE with E-UTRA support. |
| 12 |  |  | BYE | The SS sends BYE to release the call. |
| 13 |  |  | 200 OK | The UE sends 200 OK for the BYE request and ends the call. |

NOTE:	The default messages contents in annex A are used with condition "IMS security" or "GIBA" when applicable

Specific Message Contents:

Step 6 as specified in Annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A21.

Step 7 as specified in Annex C.47 (Step 2), which is referring to A.3.1 default message content of 200 OK with condition A12 and A13.

Step 8 as specified in Annex C.47 (Step 3) except datatype attribute with the value ‘eCall.invalidMSD’ in message body.

Step 10 as specified in Annex C.47 (Step 6), which is referring to A.2.19 default message content of MO INFO with conditions A2 and A4.

<End of modification>

<Start of modification>

### 21.13.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall Only subscription. UE is switched on not registered to IMS services.

The SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

NOTE:	Setting px_RATComb_Tested = EUTRA_Only is not allowed.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall over IMS initiated at UE

1A-1H)	IMS registration according to C.2 is executed.

2-5)	IMS Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request.

7)	SS sends 486 Busy here.

8-9)	UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates CS domain emergency call with MM registration if necessary. CS eCall is established and maintained for 5 seconds and then the call is cleared by SS.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate a manual eCall | Step 2 in generic procedure 36.508 [94] table 4.5A.27.3-1. |
| 2-5 | - |  | Steps 1-4 defined in Annex C.20 | Steps 19 to 25 in generic procedure 36.508 [94] table 4.5A.27.3-1. UE establishes the emergency PDN and performs IMS emergency registration. |
| 6 | - |  | Step 1 defined in C.47 | eCall setup is initiated. |
| 7 | <- |  | 486 Busy here | The SS sends 486 Busy here |
|  | - |  | EXCEPTION: The UE performs a domain selection for the emergency call and within 2 seconds of step 7 the UE may transmit EXTENDED SERVICE REQUEST |  |
| 8a1 |  |  | EXTENDED SERVICE REQUEST | If CS Fallback is performed, the UE sends service request with Service type set to mobile originating CS fallback emergency call as defined in 24.301 clause 9.9.3.27 |
| 8a2 | - |  | SS releases the RRC connection | SS waits for two seconds before sending RRC connection release. UE state is changed from RRC_CONNECTED to RRC_IDLE, and UE is redirected to UTRAN/GERAN (if supported) |
| - | - |  | EXCEPTION: Either step 9a1 or 9b1 is performed, depending on the value of px_RATComb_Tested |  |
| 9a1 | - |  | IF px_RATComb_Tested = EUTRA_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration | NOTE: RAU procedure can take place in parallel to normal CS call. |
| 9a2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9a3 | - |  | eCall is maintained for 5 seconds |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex C.46 of TS 34.229-1 can take place |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 can take place | The optional de-registration happens in parallel with the CS call release procedures. |
| 9a4 | - |  | eCall is cleared by SS |  |
| 9b1 | - |  | IF px_RATComb_Tested = EUTRA_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain |  |
| 9b2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9b3 | - |  | eCall is maintained for 5 seconds |  |
| 9b4 | - |  | eCall is cleared by SS |  |
| 9b5 | - |  | UE performs MM/GMM registration |  |

Specific Message Contents

Step 6 as specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A20

Step 7 486 Busy Here message as in Annex A.2.21 with condition A1

RRCConnectionRelease (step 8a2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| _utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| geran | ARFCN of cell 24 |  | GERAN |
| } |  |  |  |
| } |  |  |  |

ROUTING AREA UPDATE ACCEPT (Step 9b5)

Use the default message with the following specific contents

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

<End of modification>

<Start of modification>

### 21.14.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall Only subscription. The UE is switched on not registered to IMS services.

The SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

NOTE:	Setting px_RATComb_Tested = EUTRA_Only is not allowed.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall over IMS is initiated at the UE

1A-1H)	IMS registration according to C.2 is executed.

2-5)	Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request

7)	SS sends 486 Busy here.

8-9)	UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates CS domain emergency call with MM registration if necessary. CS eCall is established and maintained for 5 seconds and then the call is cleared by SS.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate an automatic eCall | Step 2 in generic procedure 36.508 [94] table 4.5A.27.3-1. |
| 1A-1H | - |  | Steps 4-11 in Annex C.2 | Steps 3 to 18 in generic procedure in 36.508 [94] table 4.5A.27.3-1. UE attaches to the NW, establishes the default PDN, and performs IMS registration. |
| 2-5 | - |  | Step 1-4 defined in C.20 | Steps 19 to 25 in generic procedure 36.508 [94] table 4.5A.27.3-1. UE establishes the emergency PDN and performs IMS emergency registration. |
| 6 | - |  | Step1 defined in C.47 | UE sends INVITE along with initial SDP offer and MSD |
| 7 | <- |  | 486 Busy here | The SS sends 486 Busy here |
| - | - |  | EXCEPTION: The UE performs a domain selection for the emergency call and the UE may transmit EXTENDED SERVICE REQUEST |  |
| 8a1 | -> |  | EXTENDED SERVICE REQUEST | If CS Fallback is performed, the UE sends service request with Service type set to mobile originating CS fallback emergency call as defined in 24.301 clause 9.9.3.27 |
| 8a2 | <- |  | SS releases the RRC connection | SS waits for two seconds before sending RRC connection release. UE state is changed from RRC_CONNECTED to RRC_IDLE, and UE is redirected to UTRAN/GERAN (if supported) |
| - | - |  | EXCEPTION: Either step 9a1 or 9b1 is performed, depending on the value of px_RATComb_Tested |  |
| 9a1 | - |  | IF px_RATComb_Tested = EUTRA_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration | NOTE: RAU procedure can take place in parallel to normal CS call. |
| 9a2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9a3 | - |  | eCall is maintained for 5 seconds |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex C.46 of TS 34.229-1 can take place |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 can take place | The optional de-registration happens in parallel with the CS call release procedures. |
| 9a4 | - |  | eCall is cleared by SS |  |
| 9b1 | - |  | IF px_RATComb_Tested = EUTRA_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain |  |
| 9b2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9b3 | - |  | eCall is maintained for 5 seconds |  |
| 9b4 | - |  | eCall is cleared by SS |  |
| 9b5 | - |  | UE performs MM/GMM registration |  |

Specific Message Contents

Step 6 as specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A21

Step 7 486 Busy Here message as in Annex A.2.21 with condition A1.

RRCConnectionRelease (step 8a2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| _utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| geran | ARFCN of cell 24 |  | GERAN |
| } |  |  |  |
| } |  |  |  |

ROUTING AREA UPDATE ACCEPT (Step 9b5)

Use the default message with the following specific contents

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

<End of modification>

<Start of modification>

### 21.15.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall Only subscription. The UE is switched on not registered to IMS services.

The SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall over IMS is initiated at the UE

1A-1H)	IMS registration according to C.2 is executed.

2-5)	Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request

7)	SS sends 600 Busy everywhere.

8-9)	UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates CS domain emergency call with MM registration if necessary. CS eCall is established and maintained for 5 seconds and then the call is cleared by SS.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate a manual eCall |  |
| 1A-1H | - |  | Steps 4-11 in Annex C.2 | Steps 3 to 18 in generic procedure in 36.508 [94] table 4.5A.27.3-1. UE attaches to the NW, establishes the default PDN, and performs IMS registration. |
| 2-5 | - |  | Step 1-4 defined in C.20 | Steps 19 to 25 in generic procedure 36.508 [94] table 4.5A.27.3-1. UE establishes the emergency PDN and performs IMS emergency registration. |
| 6 | - |  | Step1 defined in C.47 | UE sends INVITE along with initial SDP offer and MSD |
| 7 | <- |  | 600 Busy everywhere | The SS sends 600 Busy everywhere |
| - | - |  | EXCEPTION: The UE performs a domain selection for the emergency call and the UE may transmit EXTENDED SERVICE REQUEST |  |
| 8a1 | -> |  | EXTENDED SERVICE REQUEST | If CS Fallback is performed, the UE sends service request with Service type set to mobile originating CS fallback emergency call as defined in 24.301 clause 9.9.3.27 |
| 8a2 | <- |  | SS releases the RRC connection | SS waits for two seconds before sending RRC connection release. UE state is changed from RRC_CONNECTED to RRC_IDLE, and UE is redirected to UTRAN/GERAN (if supported) |
| - | - |  | EXCEPTION: Either step 9a1 or 9b1 is performed, depending on the value of px_RATComb_Tested |  |
| 9a1 | - |  | IF px_RATComb_Tested = EUTRA_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration | NOTE: RAU procedure can take place in parallel to normal CS call. |
| 9a2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9a3 | - |  | eCall is maintained for 5 seconds |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex C.46 of TS 34.229-1 can take place |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 can take place | The optional de-registration happens in parallel with the CS call release procedures. |
| 9a4 | - |  | eCall is cleared by SS |  |
| 9b1 | - |  | IF px_RATComb_Tested = EUTRA_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain |  |
| 9b2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9b3 | - |  | eCall is maintained for 5 seconds |  |
| 9b4 | - |  | eCall is cleared by SS |  |
| 9b5 | - |  | UE performs MM/GMM registration |  |

Specific Message Contents

Step 6 as specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A20

Step 7 600 Busy Everywhere message as in Annex A.2.22 with condition A1.

RRCConnectionRelease (step 8a2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| _utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| geran | ARFCN of cell 24 |  | GERAN |
| } |  |  |  |
| } |  |  |  |

ROUTING AREA UPDATE ACCEPT (Step 9b5)

Use the default message with the following specific contents

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

<End of modification>

<Start of modification>

### 21.16.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall Only subscription. UE is switched on not registered to IMS services.

The SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall over IMS is initiated at the UE

1A-1H)	IMS registration according to C.2 is executed.

2-5)	Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request

7)	SS sends 600 Busy everywhere.

8-9)	UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates CS domain emergency call with MM registration if necessary. CS eCall is established and maintained for 5 seconds and then the call is cleared by SS.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate an automatic eCall | Step 2 in generic procedure 36.508 [94] table 4.5A.27.3-1. |
| 1A-1H | - |  | Steps 4-11 in Annex C.2 | Steps 3 to 18 in generic procedure in 36.508 [94] table 4.5A.27.3-1. UE attaches to the NW, establishes the default PDN, and performs IMS registration. |
| 2-5 | - |  | Step 1-4 defined in C.20 | Steps 19 to 25 in generic procedure 36.508 [94] table 4.5A.27.3-1. UE establishes the emergency PDN and performs IMS emergency registration. |
| 6 | - |  | Step 1 defined in C.47 | UE sends INVITE along with initial SDP offer and MSD |
| 7 | <- |  | 600 Busy everywhere | The SS sends 600 Busy everywhere |
| - | - |  | EXCEPTION: The UE performs a domain selection for the emergency call and the UE may transmit EXTENDED SERVICE REQUEST |  |
| 8a1 | -> |  | EXTENDED SERVICE REQUEST | If CS Fallback is performed, the UE sends service request with Service type set to mobile originating CS fallback emergency call as defined in 24.301 clause 9.9.3.27 |
| 8a2 | <- |  | SS releases the RRC connection | SS waits for two seconds before sending RRC connection release. UE state is changed from RRC_CONNECTED to RRC_IDLE, and UE is redirected to UTRAN/GERAN (if supported) |
| - | - |  | EXCEPTION: Either step 9a1 or 9b1 is performed, depending on the value of px_RATComb_Tested |  |
| 9a1 | - |  | IF px_RATComb_Tested = EUTRA_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration | NOTE: RAU procedure can take place in parallel to normal CS call. |
| 9a2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9a3 | - |  | eCall is maintained for 5 seconds |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex C.46 of TS 34.229-1 can take place |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 can take place | The optional de-registration happens in parallel with the CS call release procedures. |
| 9a4 | - |  | eCall is cleared by SS |  |
| 9b1 | - |  | IF px_RATComb_Tested = EUTRA_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain |  |
| 9b2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9b3 | - |  | eCall is maintained for 5 seconds |  |
| 9b4 | - |  | eCall is cleared by SS |  |
| 9b5 | - |  | UE performs MM/GMM registration |  |

Specific Message Contents

Step 6 as specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A21.

Step 7 600 Busy Everywhere message as in Annex A.2.22 with condition A1.

RRCConnectionRelease (step 8a2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| _utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| geran | ARFCN of cell 24 |  | GERAN |
| } |  |  |  |
| } |  |  |  |

ROUTING AREA UPDATE ACCEPT (Step 9b5)

Use the default message with the following specific contents

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

<End of modification>

<Start of modification>

### 21.17.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall Only subscription.

The UE is switched on not registered to IMS services.

The SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall over IMS is initiated at the UE

1A-1H)	IMS registration according to C.2 is executed.

2-5)	IMS Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request

7)	SS sends 603 Decline.

8-9)	UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates CS domain emergency call with MM registration if necessary.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate a manual eCall | Step 2 in generic procedure 36.508 [94] table 4.5A.27.3-1. |
| 1A-1H | - |  | Steps 4-11 in Annex C.2 | Steps 3 to 18 in generic procedure 36.508 [94] table 4.5A.27.3-1. UE attaches to the NW, establishes the default PDN, and performs IMS registration. |
| 2-5 | - |  | Step 1-4 defined in C.20 | Steps 19 to 25 in generic procedure 36.508 [94] table 4.5A.27.3-1. UE establishes the emergency PDN and performs IMS emergency registration. |
| 6 | - |  | Step 1 defined in C.47 | UE sends INVITE along with initial SDP offer and MSD |
| 7 | <- |  | 603 Decline | The SS sends 603 Decline |
| - | - |  | EXCEPTION: The UE performs a domain selection for the emergency call and the UE may transmit EXTENDED SERVICE REQUEST |  |
| 8a1 | -> |  | EXTENDED SERVICE REQUEST | If CS Fallback is performed, the UE sends service request with Service type set to mobile originating CS fallback emergency call as defined in 24.301 clause 9.9.3.27 |
| 8a2 | <- |  | SS releases the RRC connection | SS waits for two seconds before sending RRC connection release. UE state is changed from RRC_CONNECTED to RRC_IDLE, and UE is redirected to UTRAN/GERAN (if supported) |
| - | - |  | EXCEPTION: Either step 9a1 or 9b1 is performed, depending on the value of px_RATComb_Tested |  |
| 9a1 | - |  | IF px_RATComb_Tested = EUTRA_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration | NOTE: RAU procedure can take place in parallel to normal CS call. |
| 9a2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9a3 | - |  | eCall is maintained for 5 seconds |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex C.46 of TS 34.229-1 can take place |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 can take place | The optional de-registration happens in parallel with the CS call release procedures. |
| 9a4 | - |  | eCall is cleared by SS |  |
| 9b1 | - |  | IF px_RATComb_Tested = EUTRA_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain |  |
| 9b2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9b3 | - |  | eCall is maintained for 5 seconds |  |
| 9b4 | - |  | eCall is cleared by SS |  |
| 9b5 | - |  | UE performs MM/GMM registration |  |

Specific Message Contents

Step 6 as specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A21

Step 7 603 Decline message as in Annex A.2.23 with condition A1.

RRCConnectionRelease (step 8a2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| _utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| geran | ARFCN of cell 24 |  | GERAN |
| } |  |  |  |
| } |  |  |  |

ROUTING AREA UPDATE ACCEPT (Step 9b5)

Use the default message with the following specific contents

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

<End of modification>

<Start of modification>

### 21.18.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall Only subscription.

The UE is switched on not registered to IMS services.

The SS is configured:

-	with 2 cells: as in TS 36.508 [94]

-	E-UTRAN cell A

-	if px_RATComb_Tested = EUTRA_UTRA, cell 5

-	if px_RATComb_Tested = EUTRA_GERAN, GERAN cell 24

-	Cell A power level is set as “serving cell” and cell 24/cell 5 power level is set as “suitable cell”

NOTE: Setting px_RATComb_Tested = EUTRA_Only is not allowed.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall over IMS is initiated at the UE

1A-1H)	IMS registration according to C.2 is executed.

2-5)	Emergency registration according to C.20 is executed.

6)	SS waits for UE to send an INVITE request

7)	SS sends 603 Decline.

8-9)	UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates CS domain emergency call with MM registration if necessary. CS eCall is established and maintained for 5 seconds and then the call is cleared by SS.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate an automatic eCall | Step 2 in generic procedure 36.508 [94] table 4.5A.27.3-1. |
| 1A-1H | - |  | Steps 4-11 in Annex C.2 | Steps 3 to 18 in generic procedure in 36.508 [94] table 4.5A.27.3-1. UE attaches to the NW, establishes the default PDN, and performs IMS registration. |
| 2-5 | - |  | Step 1-4 defined in C.20 | Steps 19 to 25 in generic procedure 36.508 [94] table 4.5A.27.3-1. UE establishes the emergency PDN and performs IMS emergency registration. |
| 6 | - |  | Step 1 defined in C.47 | UE sends INVITE along with initial SDP offer and MSD |
| 7 | <- |  | 603 Decline | The SS sends 603 Decline |
| - | - |  | EXCEPTION: The UE performs a domain selection for the emergency call and the UE may transmit EXTENDED SERVICE REQUEST |  |
| 8a1 | -> |  | EXTENDED SERVICE REQUEST | If CS Fallback is performed, the UE sends service request with Service type set to mobile originating CS fallback emergency call as defined in 24.301 clause 9.9.3.27 |
| 8a2 | <- |  | SS releases the RRC connection | SS waits for two seconds before sending RRC connection release. UE state is changed from RRC_CONNECTED to RRC_IDLE, and UE is redirected to UTRAN/GERAN (if supported) |
| - | - |  | EXCEPTION: Either step 9a1 or 9b1 is performed, depending on the value of px_RATComb_Tested |  |
| 9a1 | - |  | IF px_RATComb_Tested = EUTRA_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration | NOTE: RAU procedure can take place in parallel to normal CS call. |
| 9a2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9a3 | - |  | eCall is maintained for 5 seconds |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex C.46 of TS 34.229-1 can take place |  |
| - | - |  | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex C.30 of TS 34.229-1 can take place | The optional de-registration happens in parallel with the CS call release procedures. |
| 9a4 | - |  | eCall is cleared by SS |  |
| 9b1 | - |  | IF px_RATComb_Tested = EUTRA_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain |  |
| 9b2 | - |  | SS configures cell A as a “non-suitable cell” |  |
| 9b3 | - |  | eCall is maintained for 5 seconds |  |
| 9b4 | - |  | eCall is cleared by SS |  |
| 9b5 | - |  | UE performs MM/GMM registration |  |

Specific Message Contents

Step 6 as specified in annex C.47 (Step 1), which is referring to A.2.1 default message content of INVITE with condition A21

Step 7 603 Decline message as in Annex A.2.23 with condition A1.

RRCConnectionRelease (step 8a2)

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo ::= CHOICE { |  |  |  |
| utra-FDD | Downlink UARFCN of cell 5 |  | UTRA-FDD |
| _utra-TDD | Downlink UARFCN of cell 5 |  | UTRA-TDD |
| geran | ARFCN of cell 24 |  | GERAN |
| } |  |  |  |
| } |  |  |  |

ROUTING AREA UPDATE ACCEPT (Step 9b5)

Use the default message with the following specific contents

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

<End of modification>

<Start of modification>

### 22.1.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2  up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt an IMS speech call | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.6.3-1 for a UE with E-UTRA support. |
| 2 |  |  | INVITE | UE sends INVITE as described in C.21, step 2, with either the Session-Expires value set to 1800 or no Session-Expires header. |
| 3 |  |  |  |  |
| - |  |  | EXCEPTION: Steps 4a0 to 4a7 describe behaviour that depends on UE capability: the "lower case letter" identifies a step sequence that takes place if the UE included Session-Expires in step 2 |  |
| 4a0 |  |  | 100 Trying | SS sends 100 Trying response. |
| 4a1 |  |  | 422 Session Interval Too Small | SS sends 422 Session Interval Too Small response with Min-SE value of 1860. |
| 4a2 |  |  | ACK | UE sends ACK. |
| 4a3 |  |  | INVITE | UE sends INVITE with Min-SE value and Session-Expires value being 1860. |
| 4a4 |  |  | 100 Trying | SS sends 100 Trying response. |
| 4a5 |  |  | 422 Session Interval Too Small | SS sends 422 Session Interval Too Small response with Min-SE value of 1920. |
| 4a6 |  |  | ACK | UE sends ACK. |
| 4a7 |  |  | INVITE | UE sends INVITE with Min-SE value and Session-Expires value being 1920. |
| 5-13 |  |  | Steps 3-11 defined in annex C.21 |  |
| 14 |  |  | 200 OK | SS sends 200 OK for INVITE with negotiated Session-Expires value set to 1920 and refresher value set to uac. |
| 15 |  |  | ACK | UE sends ACK. |
| 16 |  |  | UPDATE | 960 seconds after step 15, UE sends an UPDATE request to refresh the session. |
| 17 |  |  | 200 OK | SS sends 200 OK for UPDATE. |
| 18 |  |  | UPDATE | 960 seconds after step 17, UE sends an UPDATE request to refresh the session. |
| 19 |  |  | 200 OK | SS sends 200 OK for UPDATE. |
| 20-23 |  |  | Steps defined in annex C.33 | SS releases the call. |

Specific Message Contents

INVITE (Step 2)

Use the default Message "INVITE" in Annex C.21 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Session-Expires | (if present) |
| delta-seconds | 1800 |
| refresher | uac (if present) |

422 Session Interval Too Small (Step 4a1)

Use the default Message "422 Session Interval Too Small" in Annex A.2.24 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Min-SE |  |
| delta-seconds | 1860 |

INVITE (Step 4a3)

Use the default Message "INVITE" in Annex C.21 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Session-Expires |  |
| delta-seconds | 1860 |
| Min-SE |  |
| delta-seconds | 1860 |

422 Session Interval Too Small (Step 4a5)

Use the default Message 422 Session Interval Too Small in Annex A.2.24 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Min-SE |  |
| delta-seconds | 1920 |

INVITE (Step 4a7)

Use the default Message "INVITE" in Annex A.2.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Call-ID |  |
| Session-Expires |  |
| delta-seconds | 1920 |
| Min-SE |  |
| delta-seconds | 1920 |

183 Session Progress (Step 6)

Use the default Message "183 Session Progress for INVITE" in Annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Allow | INVITE, UPDATE, PRACK, ACK, OPTIONS, CANCEL, BYE |

200 OK (Step 14)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Allow | INVITE, UPDATE, PRACK, ACK, OPTIONS, CANCEL, BYE |
| Require | timer |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1920 |
| refresher | uac |
| Min-SE |  |
| delta-seconds | 1920 |

UPDATE (Steps 16 and 18)

Use the default Message "UPDATE" in Annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1920 |
| refresher | uac |
| Min-SE |  |
| delta-seconds | 1920 |
| Content-Type | any value if present |

200 OK (Steps 17 and 19)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1920 |
| refresher | uac |
| Min-SE |  |
| delta-seconds | 1920 |

<End of modification>

<Start of modification>

### 22.2.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2  up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt an IMS speech call | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.6.3-1 for a UE with E-UTRA support. |
| 2 |  |  | INVITE | UE sends INVITE as described in C.21, step 2. |
| 3-11 |  |  | Steps 3-11 defined in annex C.21 |  |
| 12 |  |  | 200 OK | SS sends 200 OK for INVITE with Session-Expires value set to 1800 and refresher value set to uas. |
| 13 |  |  | ACK | UE sends ACK. |
| 14 |  |  | UPDATE | 900 seconds after step 12, SS sends an UPDATE request to refresh the session. |
| 15 |  |  | 200 OK | UE sends 200 OK for UPDATE. |
| 16-19 |  |  | Steps 2-5 defined in annex C.32 | 1800 seconds after step 15, UE releases the call due to session expiring. |

Specific Message Contents

INVITE (Step 2)

Use the default Message "INVITE" in Annex C.21 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Session-Expires | (if present) |
| delta-seconds | 1800 |
| refresher | not present |

200 OK (Step 12)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require | timer |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uas |

UPDATE (Step 14)

Use the default Message "UPDATE" in Annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uac |
| Content-Type | not present |

200 OK (Step 15)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require | timer |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uac |

<End of modification>

<Start of modification>

### 22.3.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt an IMS speech call | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.6.3-1 for a UE with E-UTRA support. |
| 2 |  |  | INVITE | UE sends INVITE as described in C.21, step 2, indicating support for Session Timer, with either the Session-Expires value set to 1800 or no Session-Expires header |
| 3-11 |  |  | Steps 3-11 defined in annex C.21 |  |
| 12 |  |  | 200 OK | SS sends 200 OK for INVITE, without timer tag in Supported and Require headers and without Session-Expires header |
| 13 |  |  | ACK | UE sends ACK. |
| 14 |  |  | UPDATE | 900 seconds after step 13, UE sends an UPDATE request to refresh the session. |
| 15 |  |  | 200 OK | SS sends 200 OK for UPDATE, without timer tag in Supported and Require headers and without Session-Expires header |
| 16 |  |  | UPDATE | 900 seconds after step 15, UE sends an UPDATE request to refresh the session. |
| 17 |  |  | 200 OK | SS sends 200 OK for UPDATE, without timer tag in Supported and Require headers and without Session-Expires header |
| 18-21 |  |  | Steps defined in annex C.33 | SS releases the call. |

Specific Message Contents

INVITE (Step 2)

Use the default Message "INVITE" in Annex C.21 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Session-Expires | (if present) |
| delta-seconds | 1800 |
| refresher | uac (if present) |

183 Session Progress (Step 4)

Use the default Message "183 Session Progress for INVITE" in Annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Allow | INVITE, UPDATE, PRACK, ACK, OPTIONS, CANCEL, BYE |

200 OK (Step 12)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Allow | INVITE, UPDATE, PRACK, ACK, OPTIONS, CANCEL, BYE |

UPDATE (Steps 14 and 16)

Use the default Message "UPDATE" in Annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uac |
| Content-Type | any value if present |

<End of modification>

<Start of modification>

### 22.4.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt an IMS speech call using a Session-Expires | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.6.3-1 for a UE with E-UTRA support. |
| 2 |  |  | INVITE | UE sends INVITE as described in C.21, step 2, indicating support for Session Timer, with either the Session-Expires value set to 1800 or no Session-Expires header |
| 3-11 |  |  | Steps 3-11 defined in annex C.21 |  |
| 12 |  |  | 200 OK | SS sends 200 OK for INVITE, with timer tag in Supported header but without Session-Expires header |
| 13 |  |  | ACK | UE sends ACK. |
| 14-17 |  |  | Steps defined in annex C.33 | 1860 seconds after step 13, SS releases the call. |

Specific Message Contents

INVITE (Step 2)

Use the default Message "INVITE" in Annex C.21 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Session-Expires | (if present) |
| delta-seconds | 1800 |
| refresher | uac (if present) |

200 OK (Step 12)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | timer |

<End of modification>

<Start of modification>

### 22.5.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipp5ed into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-26)	UE executes the procedures described in TS 36.508 [94] table 4.5A.7.3-1 steps 1 to 26.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | SS sends INVITE as described in C.11, step 1. |
| 2-11 |  |  | Steps 3-11A defined in annex C.11 |  |
| 12 |  |  | 200 OK | UE sends 200 OK for INVITE with Session-Expires value set to 1800 and refresher value set to uac. |
| 13 |  |  | ACK | SS sends ACK. |
| 14 |  |  | UPDATE | 900 seconds after step 12, SS sends an UPDATE request to refresh the session. |
| 15 |  |  | 200 OK | UE sends 200 OK for UPDATE. |
| 16-19 |  |  | Steps 2-5 defined in annex C.32 | 1800 seconds after step 15, UE releases the call due to session expiry. |

Specific Message Contents

INVITE (Step 1)

Use the default message “INVITE for MT Call” in annex A.2.9

200 OK (Step 12)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require | timer |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uac |

UPDATE (Step 14)

Use the default Message "UPDATE" in Annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uac |
| Content-Type | not present |

200 OK (Steps 15)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uac |

<End of modification>

<Start of modification>

### 22.6.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-26)	UE executes the procedures described in TS 36.508 [94] table 4.5A.7.3-1 steps 1 to 26.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | SS sends INVITE as described in C.11, step 2, with timer tag set in Supported header and Session-Expires value set to 1800. |
| 2-11 |  |  | Steps 3-11A defined in annex C.11 |  |
| 12 |  |  | 200 OK | UE sends 200 OK for INVITE with Session-Expires value set to 1800 and refresher value set to uac. |
| 13 |  |  | ACK | SS sends ACK. |
| 14 |  |  | UPDATE | 900 seconds after step 17, SS sends an UPDATE request to refresh the session. |
| 15 |  |  | 200 OK | UE sends 200 OK for UPDATE. |
| 16-19 |  |  | Steps defined in annex C.33 | SS releases the call. |

Specific Message Contents

INVITE (Step 1)

Use the default message “INVITE for MT Call” in annex A.2.9 with following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Session-Expires |  |
| delta-seconds | 1800 |

200 OK (Step 12)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require | timer |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uac |

UPDATE (Step 14)

Use the default Message "UPDATE" in Annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1800 |
| Refresher | uac |
| Content-Type | not present |

200 OK (Steps 15)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uac |

<End of modification>

<Start of modification>

### 22.7.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipp5ed into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-26)	UE executes the procedures described in TS 36.508 [94] table 4.5A.7.3-1 steps 1 to 26.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | SS sends INVITE as described in C.11, step 1, with Session-Expires value set to 1800 and refresher set to uas. |
| 2-11 |  |  | Steps 3-11A defined in annex C.11 |  |
| 12 |  |  | 200 OK | UE sends 200 OK for INVITE with Session-Expires value set to 1800 and refresher value set to uas. |
| 13 |  |  | ACK | SS sends ACK. |
| 14 |  |  | INVITE | 900 seconds after step 13, UE sends an INVITE request to refresh the session. |
| 15 |  |  | 200 OK | SS sends 200 OK for INVITE. |
| 16 |  |  | ACK | UE sends ACK. |
| 17 |  |  | INVITE | 900 seconds after step 16, UE sends an INVITE request to refresh the session. |
| 18 |  |  | 200 OK | SS sends 200 OK for INVITE. |
| 19 |  |  | ACK | UE sends ACK. |
| 20-23 |  |  | Steps defined in annex C.33 | SS releases the call. |

Specific Message Contents

INVITE (Step 1)

Use the default message “INVITE for MT Call” in annex A.2.9 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Allow | INVITE, ACK, OPTIONS, CANCEL, BYE |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uas |

200 OK (Step 12)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uas |

INVITE (Step 14 and 17)

Use the default Message "INVITE" in Annex A.2.1 with condition A5 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uac |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message-body |
| Message-body | Same SDP message as the last SDP message sent by the UE in the 200 OK at step 7 (step 8 of C.11); the session version in the origin shall not be incremented |

200 OK (Steps 15 and 18)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with condition A23 and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require | timer |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uac |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| value | length of message-body |
| Message-body | Same SDP message as the last SDP message sent by the SS in the UPDATE at step 6 (step 7 of C.11); the session version in the origin shall not be incremented |

ACK (Steps 16 and 19)

Use the default message “ACK” in annex A.2.7 with condition A5.

<End of modification>

<Start of modification>

### 22.8.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-26)	UE executes the procedures described in TS 36.508 [94] table 4.5A.7.3-1 steps 1 to 26.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | SS sends INVITE as described in C.11, step 1, without support for Session-Timer. |
| 2-11 |  |  | Steps 3-11A defined in annex C.11 |  |
| 12 |  |  | 200 OK | UE sends 200 OK for INVITE with Session-Expires value set to 1800 and refresher value set to uas. |
| 13 |  |  | ACK | SS sends ACK. |
| 14 |  |  | UPDATE | 900 seconds after step 13, UE sends an UPDATE request to refresh the session. |
| 15 |  |  | 200 OK | SS sends 200 OK for UPDATE. |
| 16-19 |  |  | Steps 1-4 defined in annex C.33 | SS ends the call. |

Specific Message Contents

INVITE (Step 1)

Use the default Message "INVITE" in Annex C.11 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Allow | INVITE, UPDATE, PRACK, ACK, OPTIONS, CANCEL, BYE |

200 OK (Step 12)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uas |

UPDATE (Step 14)

Use the default Message "UPDATE" in Annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | timer |
| Session-Expires |  |
| delta-seconds | 1800 |
| refresher | uac |
| Content-Type | any value if present |

200 OK (Steps 15)

Use the default Message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1.

<End of modification>

<Start of modification>

### G.17.1.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has activated the IP-CAN to the Evolved Packet Core (EPC) via Wireless Local Access Network (WLAN). The UE has registered to IMS according the procedures C.2c and set up an MO call over WLAN by executing annex C.21a.

Test procedure

1)	Adding video to the voice call is initiated on the UE.

2)	UE sends a re-INVITE request to the SS.

3)	SS responds to the re-INVITE request with a 100 Trying response.

4)	SS responds to the re-INVITE request with valid 200 OK response.

5)	UE acknowledges receipt of 200 OK for re-INVITE.

6)	Removing video from the media call is initiated on the UE.

7)	UE sends a re-INVITE request to the SS.

8)	SS responds to the re-INVITE request with a 100 Trying response.

9)	SS responds to the re-INVITE request with valid 200 OK response.

10)	UE acknowledges receipt of 200 OK for re-INVITE.

11-12)	MO Call release.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE add IMS video to voice call |  |
| 2 |  |  | INVITE | UE sends re-INVITE with an SDP offer containing media lines for both voice and video. |
| 3 |  |  | 100 Trying | SS sends a 100 Trying provisional response. |
| 4 |  |  | 200 OK | SS sends a 200 OK for INVITE. |
| 5 |  |  | ACK | UE acknowledges the receipt of 200 OK for re-INVITE. |
| 6 |  |  | Make UE release video from media call |  |
| 7 |  |  | INVITE | UE sends re-INVITE with an SDP offer indicating that the video component is removed from the call |
| 8 |  |  | 100 Trying | SS sends a 100 Trying provisional response. |
| 9 |  |  | 200 OK | The SS responds re-INVITE with 200 OK |
| 10 |  |  | ACK | The UE acknowledges the receipt of 200 OK for re-INVITE |
| 11 |  |  | BYE | UE releases the call |
| 12 |  |  | 200 OK | SS sends 200 OK for BYE |

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE for MO Call” in annex A.2.1 with condition A5 (re-INVITE within a dialog), A11 (INVITE for creating a video call) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 4] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000 [Note 2] - a=fmtp: (format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Media description: - m=video (transport port) RTP/AVPF (fmt) or RTP/AVP (fmt) [Note 3] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=tcap:1 RTP/AVPF [Note 3] - a=pcfg:1 t=1 [Note 3] - a=rtpmap: (payload type) H264/90000 - a=fmtp: (format) profile-level-id= (att-field) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. Note 3: The tcap/pcfg attributes are present if RTP/AVP is present on the m line. Note 4: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. |

200 OK (Step 4)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with with condition A23 (Response sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - “o=” line identical to previous SDP sent by SS except that sess-version is incremented by one - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:30 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 1, 2] - b=AS: (bandwidth-value) [Note 1] - b=RS: (bandwidth-value) [Note 1] - b=RR: (bandwidth-value) [Note 1] Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000/1 [Note 1] - a=fmtp: (format) mode-change-capability=2; max-red=220 [Note 1] - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Media description: - m=video (transport port) RTP/AVPF (fmt) [Note 1] - b=AS: (bandwidth-value) [Note 1] - b=RS: (bandwidth-value) [Note 1] - b=RR: (bandwidth-value) [Note 1] Attributes for media: - a=acfg:1 t=1 [Note 3] - a=rtpmap: (payload type) H264/90000 [Note 1] - a=fmtp: (format) (format specific parameters) [Note 1] Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: The value for fmt, bandwidth, payload type, format and format specific parameters copied from step 2. Note 2: Transport port is the port number of the SS (see RFC 3264 clause 6). Note 3: Present if tcap/pcfg attributes were included in step 2. |

ACK (Step 5)

Use the default message “ACK” in annex A.2.7 with condition A5.

INVITE (Step 7)

Use the default message “INVITE for MO Call” in annex A.2.1 with condition A5 (re-INVITE within a dialog) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 2] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000 [Note 3] - a=fmtp: (format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Media description: - m=video 0 RTP/AVPF (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] Attributes for media: - a=rtpmap: (payload type) - a=fmtp: (format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. Note 3: The AMR channel number shall be “/1” or omitted. |

200 OK (Step 9)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with with condition A23 (Response sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| Value | length of message-body |
| Message-body | SDP body of the 200 response copied from the received INVITE and modified as follows: - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - IP address on "c=" line and, for audio, transport port on "m=" line changed to indicate to which IP address and port the UE should start sending the media; |

ACK (Step 10)

Use the default message “ACK” in annex A.2.7 with condition A5.

<End of modification>

<Start of modification>

### G.17.2.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has activated the IP-CAN to the Evolved Packet Core (EPC) via Wireless Local Access Network (WLAN). The UE has registered to IMS according the procedures C.2c and SS set up an MT call over WLAN by executing annex C.11a.

Test procedure

1)	SS sends a re-INVITE request to the UE in order to add video to the ongoing call.

2)	UE optionally responds to the re-INVITE request with a 100 Trying response.

2A)	SS may receive 183 Session Progress from the UE.

2B)	SS send PRACK to the UE to acknowledge the 183 Session Progress.

2C)	SS receive 200 OK for PRACK from the UE.

2D)	The UE accepts the session modification.

3)	SS expects and receives 200 OK for re-INVITE from the UE.

4)	SS acknowledges receipt of 200 OK for re-INVITE.

5)	SS sends a re-INVITE request to the UE in order to remove video from the ongoing call.

6)	UE optionally responds to the re-INVITE request with a 100 Trying response.

7)	UE responds to the re-INVITE request with valid 200 OK response.

8)	SS acknowledges receipt of 200 OK for re-INVITE.

9-10) MT Call release.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | SS sends re-INVITE with an SDP offer containing media lines for both voice and video. |
| 2 |  |  | 100 Trying | (Optional) UE sends a 100 Trying provisional response. |
| 2A |  |  | 183 Session Progress | (Optional) The UE sends 183 response reliably with the SDP answer to the offer in INVITE |
| 2B |  |  | PRACK | (Optional) SS acknowledges the receipt of 183 response from the UE. |
| 2C |  |  | 200 OK | (Optional) The UE responds to PRACK with 200 OK |
| 2D |  |  |  | Make UE accept the speech and video offer. |
| 3 |  |  | 200 OK | The UE responds to re-INVITE with 200 OK and includes SDP answer if SDP answer was not included with 183 Session Progress in step 2A. |
| 4 |  |  | ACK | SS acknowledges the receipt of 200 OK for re-INVITE. |
| 5 |  |  | INVITE | SS sends re-INVITE with an SDP offer indicating that the video component is removed from the call |
| 6 |  |  | 100 Trying | (Optional) UE sends a 100 Trying provisional response. |
| 7 |  |  | 200 OK | The UE responds re-INVITE with 200 OK |
| 8 |  |  | ACK | The SS acknowledges the receipt of 200 OK for re-INVITE |
| 9 |  |  | BYE | SS send BYE |
| 10 |  |  | 200 OK | UE sends 200 OK for BYE |

Specific Message Content

INVITE (Step 1)

Use the default message “INVITE for MO Call” in annex A.2.9 with condition A5 (re-INVITE within a dialog), condition A8 (INVITE for creating a video stream) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - s=- - c= IN (addrtype) (connection-address for SS) - b=AS:352 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 97 - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS:37 - b=RS:0 - b=RR:2000 Attributes for media: - a=rtpmap: 97 AMR-WB/16000/1 - a=fmtp: 97 mode-change-capability=2; max-red=220 - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv Media description: - m=video (transport port) RTP/AVPF 101 - b=AS:315 - b=RS:0 - b=RR:2500 Attributes for media: - a=rtpmap:1018 H264/90000 - a=fmtp: 101 packetization-mode=0;profile-level-id=42e00c; \ sprop-parameter-sets=J0LgDJWgUH6Af1A=,KM46gA== - a=rtcp-fb:* trr-int 5000 - a=rtcp-fb:* nack - a=rtcp-fb:* nack pli - a=rtcp-fb:* ccm fir - a=rtcp-fb:* ccm tmmbr Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv |

183 Session Progress (Step 2A)

Use the default message "183 Session Progress" in annex A.2.3 with condition A15 (183 sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 3] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000 [Note 2] - a=fmtp:(format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Media description: - m=video (transport port) RTP/AVPF (fmt) - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) H264/90000 - a=fmtp: (payload type) packetization-mode=0;profile-level-id= (att-field); \ Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. Note 3: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one |

200 OK (Step 3)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A23 (Response sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Require |  |
| option-tag | precondition |
| Message-body | Present if there has been no SDP answer at step 2A. Contents if present: Same as specified in step 2A. |

ACK (Step 4)

Use the default message “ACK” in annex A.2.7 with condition A5.

INVITE (Step 5)

Use the default message “INVITE for MO Call” in annex A.2.9 with condition A5 (re-INVITE within a dialog) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 97 - b=AS: 37 - b=RS: 0 - b=RR: 2000 Attributes for media: - a=rtpmap:97 AMR-WB/16000/1 - a=fmtp:97 mode-change-capability=2; max-red=220 - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv Media description: - m=video 0 RTP/AVPF 101 - b=AS: 315 - b=RS: 0 - b=RR: 2500 Attributes for media: - a=rtpmap: 101 H264/90000 - a=fmtp: 101 packetization-mode=0;profile-level-id=42e00c; \ sprop-parameter-sets=J0LgDJWgUH6Af1A=,KM46gA== Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv |

200 OK (Step 7)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A23 (Response sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length | header shall be present if UE uses TCP to send this message and if there is a message body |
| value | length of message-body |
| Message-body | SDP body not checked. |

ACK (Step 8)

Use the default message “ACK” in annex A.2.7 with condition A5.

<End of modification>