3GPP TSG-RAN5 Meeting #105	R5-247622

# Orlando, United States, Nov 18-22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4769 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 7.1.1.14.1.1 and 7.1.1.14.1.2 Cell DTX operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The reverse test in TP1 may fail a conformant UE, so it shall be redesigned. And there are some editorial mistakes in message content tables. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct TP1 to test that UE monitor PDCCH during cell DTX active period. - Correct the editorial mistakes in message content tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.14.1.1, 7.1.1.14.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-246934 |  |  |  |  |  |  |  |  |

<Start of Changes>

7.1.1.14.1	Cell DTX/DRX adaptation

7.1.1.14.1.1	Cell DTX operation / Correct handling of DL assignment / (De)activation by RRC configuration

7.1.1.14.1.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { cell DTX is configured and activated by RRC configuration and a new DL transmission is indicated on the PDCCH during the cell DTX Active Period }

then { UE monitors the PDCCH during the cell DTX Active Period PDCCH occasions }

}

(2)

with { UE in RRC_CONNECTED state with cell DTX is configured and activated }

ensure that {

when { cell DTX is deactivated by RRC configuration and a new DL transmission is indicated on the PDCCH outside the cell DTX Active Period }

then { UE starts monitors the PDCCH outside the cell DTX Active Period PDCCH occasions }

}

7.1.1.14.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.34.1 and clause 5.34.2. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.34.1]

Each Serving Cell may be configured by RRC with a periodic cell DTX pattern (i.e., Active and Non-Active Periods). The cell DTX operation controls UE's monitoring activity of PDCCH and configured downlink assignments in RRC_CONNECTED. For all activated Serving Cells with cell DTX configured and activated, the MAC entity may monitor PDCCH and configured downlink assignments using the cell DTX operation specified in clause 5.34.2.

Each Serving Cell may be configured by RRC with a periodic cell DRX pattern (i.e., Active and Non-Active Periods). The cell DRX operation controls Scheduling Request and configured uplink grant transmission activity in RRC_CONNECTED. For all activated Serving Cells with cell DRX configured and activated, the MAC entity may transmit configured uplink grant transmissions and Scheduling Request using the cell DRX operation specified in clause 5.34.3.

RRC controls cell DTX and cell DRX operation by configuring the following parameters in cellDTX-DRX-Config per Serving Cell:

-	cellDTX-DRX-ConfigType: defines whether only cell DTX is configured, only cell DRX is configured, or both are configured;

-	cellDTX-DRX-onDurationTimer: the active duration at the beginning of a cell DTX/DRX cycle;

-	cellDTX-DRX-StartOffset: defines the subframe where the cell DTX/DRX cycle starts;

-	cellDTX-DRX-SlotOffset: the delay before starting the cellDTX-DRX-onDurationTimer;

-	cellDTX-DRX-Cycle: the cell DTX/DRX cycle period.

-	ActivationStatus: the initial activation status of cell DTX and cell DRX operation.

[TS 38.321, clause 5.34.2]

Cell DTX is configured if cellDTX-DRX-ConfigType is set to dtx or dtxdrx. Cell DTX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DTX indication from lower layers indicating activation or deactivation of cell DTX operation, as specified in TS 38.213 [6];

-	configuring cellDTX-DRX-Config by upper layers: if cell DTX is configured and cellDTX-DRX-ActivationStatus is set to activated, cell DTX operation is activated upon cell DTX configuration; if cell DTX is configured and cellDTX-DRX-ActivationStatus is set to deactivated, cell DTX operation is deactivated upon cell DTX configuration; if cellDTX-DRX-Config is released, cell DTX operation is deactivated and all the corresponding configurations are released.

When cell DTX is configured and activated for a Serving Cell, the cell DTX Active Period includes the time while:

-	cellDTX-DRX-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DTX, the MAC entity shall:

1>	if cell DTX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (cellDTX-DRX-Cycle) = (cellDTX-DRX-StartOffset):

3>	start cellDTX-DRX-onDurationTimer for this serving cell after cellDTX-DRX-SlotOffset from the beginning of the subframe.

1>	if cell DTX operation is deactivated for this Serving Cell; or

1>	if the Serving Cell is in the cell DTX Active Period:

2>	monitor PDCCH on this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if any drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL (as described in clause 5.7) is running on any Serving Cell in the DRX group of this Serving Cell; or

1>	if ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

1>	if a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5); or

1>	if a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a):

2>	monitor PDCCH on the Serving Cells in the DRX group of this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if ra-ResponseWindow (as described in clause 5.1.4) is running and this Serving Cell is the SpCell:

2>	monitor PDCCH on this Serving Cell (as described in clause 5.1.4).

For each Serving Cell configured with cell DTX, the MAC entity need not:

1>	if cell DTX operation is activated and the Serving Cell is not in the cell DTX Active Period:

2>	monitor PDCCH for the MAC entity's RNTIs listed in clauses 5.7 and 5.7b, irrespective of the requirements of clauses 5.7 and 5.7b, unless stated otherwise in this clause;

2>	instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment;

2>	consider the NDI bit for the HARQ process corresponding to the PDSCH duration of a configured downlink assignment to have been toggled for the configured downlink assignment.

7.1.1.14.1.1.3	Test description

7.1.1.14.1.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.14.1.1.3.2	Test procedure sequence

Table 7.1.1.14.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to configure and activate Cell DTX pattern. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | In the first PDCCH occasion when the cellDTX-DRX-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 4 within 10s? | --> | HARQ ACK | 1 | P |
| 5 | SS transmits an RRCReconfiguration message to deactivate Cell DTX. | <-- | NR RRC: RRCReconfiguration |  |  |
| 6 | The UE transmits RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete |  |  |
| 7 | In the first PDCCH occasion after the cellDTX-DRX-onDurationTimer expired, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 9? | --> | HARQ ACK | 2 | P |

7.1.1.14.1.1.3.3	Specific message contents

Table 7.1.1.14.1.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.14.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | As defined in Table 7.1.1.14.1.1.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.1.3.3-2: CellGroupConfig (Table 7.1.1.14.1.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| drx-onDurationTimer |  | ms40 |  |  |
| drx-InactivityTimer |  | ms10 |  |  |
| drx-HARQ-RTT-TimerDL |  | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  |  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-HARQ-RTT-TimerUL |  | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  |  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-RetransmissionTimerDL |  | sl8 |  |  |
| drx-RetransmissionTimerUL |  | sl8 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |  |
| ms640 |  | 7 |  |  |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| drx-SlotOffset |  | ms0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| servingCellConfig |  | ServingCellConfig | Table 7.1.1.14.1.1.3.3-3 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.1.3.3-3: ServingCellConfig (Table 7.1.1.14.1.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ServingCellConfig SEQUENCE { |  |  |  |  |
| cellDTX-DRX-Config-r18 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cellDTX-DRX-onDurationTimer-r18 CHOICE { |  |  |  |  |
| milliSeconds |  | ms5 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-CycleStartOffset-r18 CHOICE { |  |  |  |  |
| ms640 |  | 0 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-SlotOffset-r18 |  | 0 |  |  |
| cellDTX-DRX-ConfigType-r18 |  | dtx |  |  |
| cellDTX-DRX-ActivationStatus-r18 |  | activated |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.1.3.3-4: RRCReconfiguration (step 6, Table 7.1.1.14.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | As defined in Table 7.1.1.14.1.1.3.3-5 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.1.3.3-5: CellGroupConfig (Table 7.1.1.14.1.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| servingCellConfig |  | ServingCellConfig | Table 7.1.1.14.1.1.3.3-6 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.1.3.3-6: ServingCellConfig (Table 7.1.1.14.1.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ServingCellConfig SEQUENCE { |  |  |  |  |
| cellDTX-DRX-Config-r18 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cellDTX-DRX-onDurationTimer-r18 CHOICE { |  |  |  |  |
| milliSeconds |  | ms5 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-CycleStartOffset-r18 CHOICE { |  |  |  |  |
| ms640 |  | 0 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-SlotOffset-r18 |  | 0 |  |  |
| cellDTX-DRX-ConfigType-r18 |  | dtx |  |  |
| cellDTX-DRX-ActivationStatus-r18 |  | deactivated |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

7.1.1.14.1.2	Cell DTX operation / Correct handling of DL assignment / (De)activation by DCI format 2_9

7.1.1.14.1.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and cell DTX is configured and deactivated by RRC configuration}

ensure that {

when { UE receives DCI format 2_9 activating cell DTX and a new DL transmission is indicated on the PDCCH during the cell DTX Active Period }

then { UE monitors the PDCCH during the cell DTX Active Period PDCCH occasions }

}

(2)

with { UE in RRC_CONNECTED state with cell DTX is configured and activated }

ensure that {

when { UE receives DCI format 2_9 deactivating cell DTX and a new DL transmission is indicated on the PDCCH outside the cell DTX Active Period }

then { UE starts monitors the PDCCH outside the cell DTX Active Period PDCCH occasions }

}

7.1.1.14.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212, clause 7.3.1.3.10, TS 38.213, clause 11.5, TS 38.321, clause 5.34.1 and clause 5.34.2. Unless otherwise stated these are Rel-18 requirements.

[TS 38.212, clause 7.3.1.3.10]

DCI format 2_9 is used for activating or de-activating the cell DTX and/or DRX configuration of one or multiple serving cells for one or more UEs, and/or for providing NES-mode indication of the primary cell for one or more UEs.

The following information is transmitted by means of the DCI format 2_9 with CRC scrambled by cellDTRX-RNTI:

-	block number 1, block number 2,…, block number N

where the starting position of a block associated with a serving cell is determined by the parameter positionInDCI-cellDTRX provided by higher layers for the UE.

If the UE is configured to monitor DCI 2_9 with CRC scrambled by cellDTRX-RNTI, one or more blocks are configured for the UE by higher layers, with the following fields defined for each block:

-	Cell DTX/DRX indication - number of bits determined by the following:

-	If higher layer parameter cellDTXDRX-L1activation is configured

-	2 bits as defined in Clause 11.5 of [5, TS38.213] if cellDTXDRXconfigType is configured to dtxdrx for the associated serving cell of the block, with the MSB corresponding to cell DTX configuration and the LSB corresponding to cell DRX configuration;

-	1 bit as defined in Clause 11.5 of [5, TS38.213] if cellDTXDRXconfigType is configured to either dtx or drx for the associated serving cell of the block;

-	0 bit otherwise.

-	NES-mode indication – 1 bit indicating NES-specific CHO execution condition as defined in Clause 11.5 of [5, TS38.213], if the higher layer parameter nesEvent is configured and the associated serving cell of the block is primary cell; 0 bit otherwise.

The size of DCI format 2_9 is indicated by the higher layer parameter sizeDCI-2-9.

[TS 38.213, clause 11.5]

A UE configured for operation on a serving cell according to one or both of a cell DTX operation and a cell DRX operation by cellDTXDRX-Config for the serving cell [11, TS 38.321], can be additionally provided by dci-Format2-9 a Type3-PDCCH CSS set to monitor PDCCH for detection of DCI format 2_9 as described in clause 10.1 during Active Time [11, TS 38.321], and a location in DCI format 2_9 by positionInDCI-cellDTRX of a cell DTX/DRX indication field for the serving cell and/or a NES-mode indication field for the PCell

-	if the UE is configured with both cell DTX operation and cell DRX operation for the serving cell and if cellDTXDRX-L1activation is provided, the cell DTX/DRX indication field includes two bits where the first bit indicates the cell DTX operation and the second bit indicates the cell DRX operation

-	if the UE is configured with only one of the cell DTX operation and cell DRX operation for the serving cell and if cellDTXDRX-L1activation is provided, the cell DTX/DRX indication field includes one bit indicating one of the cell DTX operation and cell DRX operation, respectively, for the serving cell

-	a '0' value for a bit of the cell DTX/DRX indication field indicates deactivation of cell DTX or of cell DRX

-	a '1' value for a bit of the cell DTX/DRX indication field indicates activation of cell DTX or of cell DRX

-	if the serving cell is configured with a SUL carrier, the cell DTX/DRX indication field indication for activation or deactivation of cell DRX applies to both the UL carrier and the SUL carrier

-	if nesEvent is configured, the NES-mode indication field includes one bit indicating NES-specific CHO execution condition, as described in [12, TS 38.331]

-	a '0' value for the NES-mode indication field indicates NES-specific CHO execution condition is disabled

-	a '1' value for the NES-mode indication field, indicates NES-specific CHO execution condition is enabled

A UE does not expect to monitor PDCCH for detection of DCI format 2_9 on more than one serving cells of one cell group.

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in activation or deactivation of a current cell DTX operation or cell DRX operation for a second serving cell, the UE operates on the second serving cell according to the indicated cell DTX operation or cell DRX operation starting from a slot on the active DL BWP or on the active UL BWP of the second serving cell, respectively, that is not before the beginning of the slot m+d on the active DL BWP of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1.

Table 11.5-1: Minimum time gap value d

| SCS (kHz) | Number of slots |
| --- | --- |
| 15 | 3 |
| 30 | 6 |
| 60 | 12 |
| 120 | 24 |
| 480 | 96 |
| 960 | 192 |

[TS 38.321, clause 5.34.1]

Each Serving Cell may be configured by RRC with a periodic cell DTX pattern (i.e., Active and Non-Active Periods). The cell DTX operation controls UE's monitoring activity of PDCCH and configured downlink assignments in RRC_CONNECTED. For all activated Serving Cells with cell DTX configured and activated, the MAC entity may monitor PDCCH and configured downlink assignments using the cell DTX operation specified in clause 5.34.2.

Each Serving Cell may be configured by RRC with a periodic cell DRX pattern (i.e., Active and Non-Active Periods). The cell DRX operation controls Scheduling Request and configured uplink grant transmission activity in RRC_CONNECTED. For all activated Serving Cells with cell DRX configured and activated, the MAC entity may transmit configured uplink grant transmissions and Scheduling Request using the cell DRX operation specified in clause 5.34.3.

RRC controls cell DTX and cell DRX operation by configuring the following parameters in cellDTX-DRX-Config per Serving Cell:

-	cellDTX-DRX-ConfigType: defines whether only cell DTX is configured, only cell DRX is configured, or both are configured;

-	cellDTX-DRX-onDurationTimer: the active duration at the beginning of a cell DTX/DRX cycle;

-	cellDTX-DRX-StartOffset: defines the subframe where the cell DTX/DRX cycle starts;

-	cellDTX-DRX-SlotOffset: the delay before starting the cellDTX-DRX-onDurationTimer;

-	cellDTX-DRX-Cycle: the cell DTX/DRX cycle period.

-	ActivationStatus: the initial activation status of cell DTX and cell DRX operation.

[TS 38.321, clause 5.34.2]

Cell DTX is configured if cellDTX-DRX-ConfigType is set to dtx or dtxdrx. Cell DTX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DTX indication from lower layers indicating activation or deactivation of cell DTX operation, as specified in TS 38.213 [6];

-	configuring cellDTX-DRX-Config by upper layers: if cell DTX is configured and cellDTX-DRX-ActivationStatus is set to activated, cell DTX operation is activated upon cell DTX configuration; if cell DTX is configured and cellDTX-DRX-ActivationStatus is set to deactivated, cell DTX operation is deactivated upon cell DTX configuration; if cellDTX-DRX-Config is released, cell DTX operation is deactivated and all the corresponding configurations are released.

When cell DTX is configured and activated for a Serving Cell, the cell DTX Active Period includes the time while:

-	cellDTX-DRX-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DTX, the MAC entity shall:

1>	if cell DTX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (cellDTX-DRX-Cycle) = (cellDTX-DRX-StartOffset):

3>	start cellDTX-DRX-onDurationTimer for this serving cell after cellDTX-DRX-SlotOffset from the beginning of the subframe.

1>	if cell DTX operation is deactivated for this Serving Cell; or

1>	if the Serving Cell is in the cell DTX Active Period:

2>	monitor PDCCH on this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if any drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL (as described in clause 5.7) is running on any Serving Cell in the DRX group of this Serving Cell; or

1>	if ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

1>	if a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5); or

1>	if a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a):

2>	monitor PDCCH on the Serving Cells in the DRX group of this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if ra-ResponseWindow (as described in clause 5.1.4) is running and this Serving Cell is the SpCell:

2>	monitor PDCCH on this Serving Cell (as described in clause 5.1.4).

For each Serving Cell configured with cell DTX, the MAC entity need not:

1>	if cell DTX operation is activated and the Serving Cell is not in the cell DTX Active Period:

2>	monitor PDCCH for the MAC entity's RNTIs listed in clauses 5.7 and 5.7b, irrespective of the requirements of clauses 5.7 and 5.7b, unless stated otherwise in this clause;

2>	instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment;

2>	consider the NDI bit for the HARQ process corresponding to the PDSCH duration of a configured downlink assignment to have been toggled for the configured downlink assignment.

7.1.1.14.1.2.3	Test description

7.1.1.14.1.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.14.1.2.3.2	Test procedure sequence

Table 7.1.1.14.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to configure Cell DTX pattern. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits DCI format 2_9 indicating to activate cell DTX. (Note 1) | <-- | PDCCH (DCI 2-9) | - | - |
| 4 | Wait 5ms to ensure DCI format 2_9 indication take effects. | - | - | - | - |
| 5 | In the first PDCCH occasion when the cellDTX-DRX-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5 within 10s? | --> | HARQ ACK | 1 | P |
| 7 | The SS transmits DCI format 2_9 before the start of next cellDTX-DRX-onDurationTimer and the DCI format 2_9 indicates to deactivate cell DTX. (Note 2) | <-- | PDCCH (DCI 2-9) | - | - |
| 8 | Wait 5ms to ensure DCI format 2_9 indication take effects. | - | - | - | - |
| 9 | In the first PDCCH occasion after the cellDTX-DRX-onDurationTimer expired, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 10 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 9? | --> | HARQ ACK | 2 | P |
| Note 1: The cell DTX/DRX indication field is value '1' in the DCI 2-9. Note 2: The cell DTX/DRX indication field is value '0' in the DCI 2-9. |  |  |  |  |  |

7.1.1.14.1.2.3.3	Specific message contents

Table 7.1.1.14.1.2.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.14.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.2.3.3-2: CellGroupConfig (Table 7.1.1.14.1.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| drx-onDurationTimer |  | ms40 |  |  |
| drx-InactivityTimer |  | ms10 |  |  |
| drx-HARQ-RTT-TimerDL |  | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  |  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-HARQ-RTT-TimerUL |  | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  |  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-RetransmissionTimerDL |  | sl8 |  |  |
| drx-RetransmissionTimerUL |  | sl8 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |  |
| ms640 |  | 7 |  |  |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| drx-SlotOffset |  | ms0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| physicalCellGroupConfig |  | PhysicalCellGroupConfig | Table 7.1.1.14.1.2.3.3-3 |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| servingCellConfig |  | ServingCellConfig | Table 7.1.1.14.1.2.3.3-4 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.2.3.3-3: PhysicalCellGroupConfig (Table 7.1.1.14.1.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig::= SEQUENCE { |  |  |  |  |
| cellDTRX-DCI-config-r18 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cellDTRX-RNTI-r18 |  | RNTI-Value |  |  |
| sizeDCI-2-9-r18 |  | 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.2.3.3-4: ServingCellConfig (Table 7.1.1.14.1.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ServingCellConfig SEQUENCE { |  |  |  |  |
| initialDownlinkBWP ::= SEQUENCE { |  |  |  |  |
| pdcch-Config CHOICE { |  |  |  |  |
| setup |  | PDCCH-Config | Table 7.1.1.14.1.2.3.3-5 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| cellDTX-DRX-Config-r18 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cellDTX-DRX-onDurationTimer-r18 CHOICE { |  |  |  |  |
| milliSeconds |  | ms5 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-CycleStartOffset-r18 CHOICE { |  |  |  |  |
| ms640 |  | 0 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-SlotOffset-r18 |  | 0 |  |  |
| cellDTX-DRX-ConfigType-r18 |  | dtx |  |  |
| cellDTX-DRX-ActivationStatus-r18 |  | deactivated |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| positionInDCI-cellDTRX-r18 |  | 0 |  |  |
| cellDTX-DRX-L1activation-r18 |  | enabled |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.2.3.3-5: PDCCH-Config (Table 7.1.1.14.1.2.3.3-4)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 with condition DCI_2_9

<End of Changes>