3GPP TSG-RAN WG5 Meeting #105	R5-247737

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3047 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition to ATG Absolute power tolerance TC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TC Absolute power tolerance for ATG is missing. . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new TC of Absolute power tolerance for ATG UE as per WP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3J.4(new), 6.3J.4.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.3J.4	Power control for ATG

6.3J.4.1	Absolute power tolerance for ATG

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- MU/TT are FFS for ATG UEs with more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors.

- MU/TT already defined are based on preliminary analysis of single TE vendor. Values will be revisited once more detailed analysis, including analysis from other TE vendors, is available.

6.3J.4.1.1	Test purpose

To verify the ability of the ATG UE transmitter to set its initial output power to a specific value at the start of a contiguous transmission or non-contiguous transmission with a long transmission gap, i.e. transmission gap is larger than 20ms.

6.3J.4.1.2	Test applicability

This test case applies to all types of NR ATG UE release 18 and forward.

6.3J.4.1.3	Minimum conformance requirements

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame (1 ms) at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20 ms. The tolerance includes the channel estimation error.

The minimum requirement specified in Table 6.3J.4.1-1 apply in the power range bounded by the minimum output power as specified in clause 6.3J.1 and the maximum output power as specified in clause 6.2J.1.

Table 6.3J.4.1-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3J.4.

6.3J.4.1.4	Test description

6.3J.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3J.4.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation (NOTE 1) |
| 1 | N/A for Absolute power tolerance test case |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3J.4.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3J.4.1.4.3. Note that PDCCH DCI format 0_1 sent after resetting uplink power with RRC Connection Reconfiguration, should have TPC command 0dB.

6.3J.4.1.4.2	Test procedure

Same test procedure as clause 6.3.4.2.4.2 with following exceptions.

The power of UE PUSCH first transmissions should be measured as the sum power at each transmit antenna connector/ transceiver array boundary (TAB) connector.

6.3J.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.3J.4.1.4.3-1: UplinkPowerControlCommon: Test point 1

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-119 PUSCH-ConfigCommon |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -114 | Test point 1 to verify a ATG UE relative low initial power transmission |  |
| } |  |  |  |

Table 6.3J.4.1.4.3-2: UplinkPowerControlCommon: Test point 2

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-119 PUSCH-ConfigCommon |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -100 | Test point 2 to verify a ATG UE relative high initial power transmission |  |
| } |  |  |  |

Table 6.3J.4.1.4.3-3: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 18 |  | SCS_15kHz |
|  | 21 |  | SCS_30kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for SS/PBCH block |
| SCS_30kHz | SCS=30kHz for SS/PBCH block |

6.3J.4.1.5	Test requirement

The requirement for the power measured in step (2) of the test procedure is not to exceed the values specified in Table 6.3J.4.1.5-1 and 6.3J.4.1.5-2.

Table 6.3J.4.1.5-1: Absolute power tolerance: test point 1

|  |  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Expected Measured power | SCS15 | -17.6 | -14.4 | -12.6 | -11.3 | -10.4 | -9.6 | -8.9 | -8.3 | -7.8 | -7.3 | N/A | N/A | N/A | N/A | N/A |
|  | SCS30 | -18.2 | -14.8 | -12.8 | -11.5 | -10.5 | -9.7 | -9 | -8.3 | -7.8 | -7.4 | -6.5 | -5.8 | -5.2 | -4.7 | -4.2 |
|  | SCS60 | N/A | -15.2 | -13 | -11.8 | -10.7 | -9.8 | -9.1 | -8.5 | -7.9 | -7.5 | -6.6 | -5.9 | -5.3 | -4.8 | -4.3 |
| Power tolerance |  |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3J.2.3 Note 2: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3J.4.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3J.4.1.5-2: Absolute power tolerance: test point 2

|  |  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Expected Measured power | SCS15 | -3.6 | -0.4 | 1.4 | 2.7 | 3.6 | 4.4 | 5.1 | 5.7 | 6.2 | 6.7 | N/A | N/A | N/A | N/A | N/A |
|  | SCS30 | -4.2 | -0.8 | 1.2 | 2.5 | 3.5 | 4.3 | 5 | 5.7 | 6.2 | 6.6 | 7.5 | 8.2 | 8.8 | 9.3 | 9.8 |
|  | SCS60 | N/A | -1.2 | 1 | 2.2 | 3.3 | 4.2 | 4.9 | 5.5 | 6.1 | 6.5 | 7.4 | 8.1 | 8.7 | 9.2 | 9.7 |
| Power tolerance |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The higher power limit shall not exceed the maximum output power requirements defined in sub-clause 6.2J.1.3 Note 2: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3J.4.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.3J.4.1.5-3: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | [1.0] dB | [1.4] dB | [1.4] dB |
| 40MHz < BW ≤ 100MHz | [1.4] dB | [1.4] dB | [1.4] dB |

<< END OF CHANGES >>