3GPP TSG-RAN WG5 Meeting #105	R5-247755

Orlando, USA,  18 – 22 Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3121 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding receiver requirements for band n109 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing receiver requirements for NR band n109 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added receiver requirements for band n109 in clauses - 7.3 Reference sensitivity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Receiver requirements for band n109 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version of R5-247260. |  |  |  |  |  |  |  |  |

## 7.3	Reference sensitivity

### 7.3.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

In later clauses of Clause 7 where the value of REFSENS is used as a reference to set the corresponding requirement:

In all bands, the UE shall be verified against those requirements by applying the REFSENS value in Table 7.3.2.3-1a, Table 7.3.2.3-1b and Table 7.3.2.3-1c or Table 7.3.2.3-1d with 2 Rx antenna ports tested;

For bands where the UE is required to be equipped with 4 Rx antenna ports, the UE shall additionally be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3.2.3-2 with 4 Rx antenna ports tested.

### 7.3.2	Reference sensitivity power level

7.3.2.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that don’t support RedCap.

7.3.2.3	Minimum conformance requirements

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2.3-1a, 7.3.2.3-1b, Table 7.3.2.3-1c, Table 7.3.2.3-1d and Table 7.3.2.3-2.

Table 7.3.2.3-1a: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

|  | Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band |  | SCS (kHz) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n1 |  | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 |  | -90.6 | -90.1 | -89.6 |
|  |  | 30 |  |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 |  | -90.7 | -90.2 | -89.7 |
|  |  | 60 |  |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 |  | -90.9 | -90.3 | -89.7 |
| n2 |  | 15 |  | -98 | -94.8 | -93 | -91.8 | -90.7 | -84.1 |  | -81.5 |  |  |
|  |  | 30 |  |  | -95.1 | -93.1 | -92 | -90.8 | -84.2 |  | -81.6 |  |  |
|  |  | 60 |  |  | -95.5 | -93.4 | -92.2 | -90.9 | -84.3 |  | -81.7 |  |  |
| n3 |  | 15 |  | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -86.2 | -82.3 | -81.3 | -79.7 |
|  |  | 30 |  |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -86.3 | -82.4 | -81.4 | -79.8 |
|  |  | 60 |  |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -86.4 | -82.6 | -81.5 | -79.9 |
| n5 |  | 15 |  | -98.0 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
|  |  | 30 |  |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |  |  |  |
| n71 |  | 15 |  | -98.0 | -94.8 | -93.0 | -91.8 | -90.7 | -89.9 |  | -88.6 |  | -81.5 |
|  |  | 30 |  |  | -95.1 | -93.1 | -92.0 | -90.8 | -90.0 |  | -88.7 |  | -81.5 |
|  |  | 60 |  |  | -95.5 | -93.4 | -92.2 | -91.0 | -90.1 |  | -88.9 |  | -81.5 |
| n8 |  | 15 |  | -97.0 | -93.8 | -91.4 | -85.8 |  |  | -78.4 |  |  |  |
|  |  | 30 |  |  | -94.1 | -91.7 | -87.2 |  |  | -78.5 |  |  |  |
| n12 |  | 15 |  | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n13 |  | 15 |  | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 |  |  |  |  |  |  |  |  |
| n14 |  | 15 |  | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 |  |  |  |  |  |  |  |  |
| n20 |  | 15 |  | -97.0 | -93.8 | -91.0 | -89.8 |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 | -91.1 | -90.0 |  |  |  |  |  |  |
| n24 |  | 15 |  | -100.0 | -96.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -97.1 |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -97.5 |  |  |  |  |  |  |  |  |
| n25 |  | 15 |  | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 | -81.7 | -79.5 | -77.6 |  |
|  |  | 30 |  |  | -93.6 | -91.6 | -90.5 | -89.4 | -82.3 | -81.8 | -79.6 | -77.7 |  |
|  |  | 60 |  |  | -94.0 | -91.9 | -90.7 | -89.6 | -82.4 | -81.9 | -79.7 | -77.8 |  |
| n26 |  | 15 | -99.7 | -97.56 | -94.56 | -92.76 | -87.6 |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.86 | -92.76 | -87.7 |  |  |  |  |  |  |
| n28 |  | 15 | -100.2 | -98.5 | -95.5 | -93.5 | -90.8 |  | -78.5 |  |  |  |  |
|  |  | 30 |  |  | -95.6 | -93.6 | -91.0 |  | -78.6 |  |  |  |  |
| n30 |  | 15 |  | -99.0 | -95.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -96.1 |  |  |  |  |  |  |  |  |
| n31 |  | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |
| n65 |  | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  | -89.2 |
|  |  | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  | -89.3 |
|  |  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  | -89.4 |
| n66 |  | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 | -92.2 | -91.4 |  | -90.1 | -89.6 |  |
|  |  | 30 |  |  | -96.6 | -94.6 | -93.5 | -92.3 | -91.5 |  | -90.2 | -89.7 |  |
|  |  | 60 |  |  | -97.0 | -94.9 | -93.7 | -92.5 | -91.6 |  | -90.4 | -89.8 |  |
|  |  | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |  |  |  |  |  |
| n70 |  | 30 |  |  | -97.1 | -95.1 | -94.0 | -92.8 |  |  |  |  |  |
|  |  | 60 |  |  | -97.5 | -95.4 | -94.2 | -93.0 |  |  |  |  |  |
| n71 |  | 15 |  | -97.2 | -94.0 | -91.6 | -86.0 | -84.18 -74.89 | -82.58 -67.19 | -80.78 -64.09 |  |  |  |
|  |  | 30 |  |  | -94.3 | -91.9 | -87.4 | -84.28 -74.99 | -82.68 -67.29 | -80.88 -64.19 |  |  |  |
| n72 |  | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |
| n74 |  | 15 |  | -99.53 | -96.33 | -94.53 | -89.33 |  |  |  |  |  |  |
|  |  | 30 |  |  | -96.63 | -94.63 | -89.53 |  |  |  |  |  |  |
|  |  | 60 |  |  | -97.03 | -94.93 | -89.63 |  |  |  |  |  |  |
| n85 |  | 15 | -99.2 |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |
| n100 |  | 15 | -102.2 | -100 |  |  |  |  |  |  |  |  |  |
| n106 |  | 15 | -99.2 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. NOTE 8: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 9: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.3-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating band | SCS (kHz) |  | Channel bandwidth (MHz) |  | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 |  | 5, 10, 15 |  | -100 + 10log10(NRB/25) | TDD |
|  | 30 |  | 10, 15 |  | -97.1 + 10log10(NRB/24) |  |
|  | 60 |  | 10, 15 |  | -97.5 + 10log10(NRB/11) |  |
| n381 | 15 |  | 5, 10, 15, 20, 25, 30, 40 |  | -100 + 10log10(NRB/25) | TDD |
|  | 30 |  | 10, 15, 20, 25, 30, 40 |  | -97.1 + 10log10(NRB/24) |  |
|  | 60 |  | 10, 15, 20, 25, 30, 40 |  | -97.5 + 10log10(NRB/11) |  |
| n39 | 15 |  | 5, 10, 15, 20, 25, 30, 40 |  | -100 + 10log10(NRB/25) | TDD |
|  | 30 |  | 10, 15, 20, 25, 30, 40 |  | -97.1 + 10log10(NRB/24) |  |
|  | 60 |  | 10, 15, 20, 25, 30, 40 |  | -97.5 + 10log10(NRB/11) |  |
| n40 | 15 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  | -100 + 10log10(NRB/25) | TDD |
|  | 30 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | -97.1 + 10log10(NRB/24) |  |
|  | 60 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | -97.5 + 10log10(NRB/11) |  |
| n411 | 15 |  | 10, 15, 20, 30, 40, 50 |  | -94.8 + 10log10(NRB/52) | TDD |
|  | 30 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | -95.1 + 10log10(NRB/24) |  |
|  | 60 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | -95.5 + 10log10(NRB/11) |  |
| n481 | 15 |  | 5, 10, 15, 20, 30, 40, 505 |  | -99 + 10log10(NRB/25) | TDD |
|  | 30 |  | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 |  | -96.1 + 10log10(NRB/24) |  |
|  | 60 |  | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 |  | -96.5 + 10log10(NRB/11) |  |
| n50 | 15 |  | 5, 10, 15, 20, 30, 40, 50 |  | -100 + 10log10(NRB/25) | TDD |
|  | 30 |  | 10, 15, 20, 30, 40, 50, 60, 80 |  | -97.1 + 10log10(NRB/24) |  |
|  | 60 |  | 10, 15, 20, 30, 40, 50, 60, 80 |  | -97.5 + 10log10(NRB/11) |  |
| n51 | 15 |  | 5 |  | -100 | TDD |
| n53 | 15 |  | 5, 10 |  | -100 + 10log10(NRB/25) | TDD |
|  | 30 |  | 10 |  | -97.1 |  |
|  | 60 |  | 10 |  | -97.5 |  |
| n54 | 15 |  | 5 |  | -100 | TDD |
| n757 | 15 |  | 5,10,15,20 |  | -100 + 10log10(NRB/25) | SDL |
|  | 30 |  | 10,15,20 |  | -97.1 + 10log10(NRB/24) |  |
|  | 60 |  | 10,15,20 |  | -97.5 + 10log10(NRB/11) |  |
| n767 | 15 |  | 5 |  | -100 | SDL |
| n771,4 | 15 |  | 10, 15, 20, 40, 50 |  | -95.3 + 10log10(NRB/52) | TDD |
|  | 30 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | -95.6 + 10log10(NRB/24) |  |
|  | 60 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | -96.0 + 10log10(NRB/11) |  |
| n781 | 15 |  | 10, 15, 20, 25, 30, 40, 50 |  | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 |  | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  | -96.1 + 10log10(NRB/24) |  |
|  | 60 |  | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  | -96.5 + 10log10(NRB/11) |  |
| n791 | 15 |  | 10,20,40, 50 |  | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 |  | 10, 20, 40, 50, 60, 80, 100 |  | -96.1 + 10log10(NRB/24) |  |
|  | 60 |  | 10, 20, 40, 50, 60, 80, 100 |  | -96.5 + 10log10(NRB/11) |  |
| n91 | 15 |  | 5 |  | -100 | FDD |
| n92 | 15 |  | 5, 10, 15, 20 |  | -100 + 10log10(NRB/25) | FDD |
|  | 30 |  | 10, 15, 20 |  | -97.1 + 10log10(NRB/24) |  |
| n93 | 15 |  | 5 |  | -100 | FDD |
| n94 | 15 |  | 5, 10, 15, 20 |  | -100 + 10log10(NRB/25) | FDD |
|  | 30 |  | 10, 15, 20 |  | -97.1 + 10log10(NRB/24) |  |
| n101 | 15 |  | 5, 10 |  | -100 + 10log10(NRB/25) | TDD |
|  | 30 |  | 10 |  | -97.1 + 10log10(NRB/24) |  |
| n10911 | 15 | 5,10,15,20,25,30,40,50 |  | -100 + 10log10(NRB/25) |  | FDD |
|  | 30 | 10,15,20,25,30,40,50 |  | -97.1 + 10log10(NRB/24) |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: Void. NOTE 10: FFS NOTE 11: Applies for DL channels for which channels edges are > 15 MHz away from 2xFUL at 15 kHz SCS and > 30 MHz away from 2xFUL at 30 kHz SCS. In case of UL second harmonic direct hit, the value is modified to -71.9 dBm for all channel bandwidths. |  |  |  |  |  |  |

For power class 2 UEs, certain degradation of the reference sensitivity in Table 7.3.2.3-1a is allowed. The maximum amount of degradation is specified in Table 7.3.2.3-1c, and in Table 7.3.2.3-1d for a UE that indicates txDiversity-r16 [26].

Table 7.3.2.3-1c: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n3 | 0.5 | 0.5 | 0.5 | 0.5 | 0.6 | 0.8 | 1.1 | 1.5 | 2.3 | 2.8 |
| n8 | 0.5 | 0.7 | 0.8 | 2.3 | 2.8 | 3.2 | 3.1 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.3-1d Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n3 | 1.4 | 1.5 | 1.5 | 1.5 | 1.6 | 1.7 | 2.8 | 5 | 5.5 | 6.0 |
| n8 | 1.3 | 1.4 | 2.1 | 5.8 | 6.1 | 6.5 | 7.0 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 |  |  |  |  |  |  |  |  |  |  |

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2.3-1a and in Table 7.3.2.3-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3.2.3-2 for the applicable operating bands. For operating band frequency range ≤ 1 GHz, the 4Rx operation is primarily for FWA form factor, and when 4Rx operation is supported by handheld UE, ∆RIB,4R as indicated in Table 7.3.2-2 NOTE 2 is applied.

Table 7.3.2.3-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n8, n28, n71 | -2.71 |
| n5, n8, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79 | -2.2 |
| NOTE 1: When 4Rx operation is supported by FWA form factor. NOTE 2: When 4Rx operation is supported by handheld UE. |  |

The reference sensitivity (REFSENS) requirement specified in Table 7.3.2.3-1a, Table 7.3.2.3-1b, Table 7.3.2.3-1c, Table 7.3.2.3-1d and Table 7.3.2.3-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3-3.

Table 7.3.2.3-3: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n1 | 15 |  | 25 | 501 | 751 | 1001 | 1281 | 1281 |  | 1281 | 1281 | 1281 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 361 | 501 | 641 | 641 |  | 641 | 641 | 641 |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 18 | 24 | 301 | 301 |  | 301 | 301 | 301 |  |  |  |  |  |  |
| n2 | 15 |  | 25 | 501 | 501 | 501 | 501 | 481 |  | 401 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 24 | 241 | 241 | 241 | 241 |  | 201 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 101 | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |  |
| n3 | 15 |  | 25 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 | 501 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 241 | 241 | 241 | 241 | 241 | 241 | 241 | 241 |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 | 101 |  |  |  |  |  |  |
| n5 | 15 |  | 25 | 251 | 251 | 251 | Note 5 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 | Note 5 |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 |  | 25 | 501 | 751 | 751 | 721 | 641 |  | 451 |  | 451 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 361 | 361 | 361 | 321 |  | 201 |  | 201 |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 18 | 181 | 181 | 161 |  | 101 |  | 101 |  |  |  |  |  |  |
| n8 | 15 |  | 25 | 251 | 251 | 251 |  |  | Note 5 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 |  |  | Note 5 |  |  |  |  |  |  |  |  |  |
| n12 | 15 |  | 201 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n13 | 15 |  | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 |  | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 |  | 25 | 201 | 202 | 202 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 102 | 102 |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 |  | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 |  | 25 | 501 | 501 | 501 | 501 | 481 | 401 | 401 | Note 5 |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 241 | 241 | 241 | 241 | 201 | 201 | Note 5 |  |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 101 | 101 | 101 | 101 | 101 | 101 | Note 5 |  |  |  |  |  |  |  |
| n26 | 15 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 121 | 121 | 121 |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 15 | 15 | 25 | 251 | 251 | 251 |  | 251 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 |  | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n31 | 15 | 58 | 58 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| n34 | 15 |  | 25 | 50 | 75 |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 |  | 25 | 50 | 75 | 100 | 128 | 160 |  | 216 |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 | 64 | 75 |  | 100 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 18 | 24 | 30 | 36 |  | 50 |  |  |  |  |  |  |  |  |
| n39 | 15 |  | 25 | 50 | 75 | 100 | 128 | 160 |  | 216 |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 | 64 | 75 |  | 100 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 18 | 24 | 30 | 36 |  | 50 |  |  |  |  |  |  |  |  |
| n40 | 15 |  | 25 | 50 | 75 | 100 | 128 | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 | 64 | 75 |  | 100 |  | 128 | 162 |  | 216 |  |  |  |
|  | 60 |  |  | 10 | 18 | 24 | 30 | 36 |  | 50 |  | 64 | 75 |  | 100 |  |  |  |
| n41 | 15 |  |  | 50 | 75 | 100 |  | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 |  | 75 |  | 100 |  | 128 | 162 | 180 | 216f | 243 | 270 |  |
|  | 60 |  |  | 10 | 18 | 24 |  | 36 |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n48 | 15 |  | 25 | 50 | 75 | 100 |  | 160 |  | 216 |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 |  | 75 |  | 100 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 18 | 24 |  | 36 |  | 50 |  |  |  |  |  |  |  |  |
| n50 | 15 |  | 25 | 50 | 75 | 100 |  | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 |  | 75 |  | 100 |  | 128 | 162 |  | Note 3 |  |  |  |
|  | 60 |  |  | 10 | 18 | 24 |  | 36 |  | 50 |  | 64 | 75 |  | Note 3 |  |  |  |
| n51 | 15 |  | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
| n53 | 15 |  | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n54 | 15 |  | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
| n65 | 15 |  | 25 | 501 | 751 | 1001 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 361 | 501 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 18 | 24 |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 |  | 25 | 501 | 751 | 1001 | 1281 | 160 |  | 216 |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 361 | 501 | 641 | 751 |  | 1001 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 18 | 24 | 301 | 361 |  | 501 |  |  |  |  |  |  |  |  |
| n70 | 15 |  | 25 | 501 | 751 | Note 3 | Note 3 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | 361 | Note 3 | Note 3 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 101 | 18 | Note 3 | Note 3 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 |  | 25 | 251 | 201 | 201 | 201,6 | 201,6 | 201,6 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 121 | 101 | 101 | 101,6 | 101,6 | 101,6 |  |  |  |  |  |  |  |  |  |
| n72 | 15 | 58 | 58 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| n74 | 15 |  | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 51 | 51 | 51 |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  |  | 50 | 75 | 100 |  |  |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 |  |  |  | 100 |  | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  |  | 10 | 18 | 24 |  |  |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n78 | 15 |  |  | 50 | 75 | 100 | 128 | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 | 36 | 50 | 64 | 75 |  | 100 |  | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  |  | 10 | 18 | 24 | 30 | 36 |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n79 | 15 |  |  | 50 |  | 100 |  |  |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 |  | 50 |  |  |  | 100 |  | 128 | 162 |  | 216 |  | 270 |  |
|  | 60 |  |  | 10 |  | 24 |  |  |  | 50 |  | 64 | 75 |  | 100 |  | 135 |  |
| n85 | 15 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n91 | 15 |  | 254 | 201,4 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n92 | 15 |  | 25 | 201 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n93 | 15 |  | 254 | 251,4 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n94 | 15 |  | 25 | 251 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 121 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n100 | 15 | 15 | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n101 | 15 |  | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n106 | 15 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| n109 | 15 |  | 25 | Note 7 | Note 7 | Note 7 | Note 7 | Note 7 |  | Note 7 |  | Note 7 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24 | Note 7 | Note 7 | Note 7 | Note 7 |  | Note 7 |  | Note 7 |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). NOTE 2: For band n20; for 15kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =16; for 30kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =6 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =8; for 60kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =3 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =4. NOTE 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest duplex distance shall be used. NOTE 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. NOTE 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.does not apply. Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. For UEs not supporting this uplink channel bandwidth, the UL configuration of the 20MHz UL channel bandwidth and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply. Note 7: For this DL channel bandwidth, the UL configuration of 5MHz for 15kHz SCS and 10MHz for 30kHz shall be used. Note 8: In the case of 3 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 9 and in the case of 5 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 10. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Unless given by Table 7.3.2.3-4, the minimum requirements specified in Tables 7.3.2.3-1a, Tables 7.3.2.3-1b, Tables 7.3.2.3-1c, Tables 7.3.2.3-1d shall be verified with the network signalling value NS_01 (Table 6.2.3.3-1) configured.

Table 7.3.2.3-4: Network signalling value for reference sensitivity

| Operating band | Network Signalling value |
| --- | --- |
| n2 | NS_03 |
| n12 | NS_06 |
| n13 | NS_06 |
| n14 | NS_06 |
| n24 | NS_56 |
| n25 | NS_03 |
| n30 | NS_21 |
| n48 | NS_27 |
| n53 | NS_45 |
| n54 | NS_62 |
| n66 | NS_03 |
| n70 | NS_03 |
| n71 | NS_35 |

For the UE which supports CA, SUL or DC band combination, the minimum requirement for reference sensitivity in Table 7.3.2.3-1a and Table 7.3.2.3-1b shall be increased by the amount given in ΔRIB,c defined in subclause 7.3.3 for the applicable operating bands.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3.2.

7.3.2.4	Test description

7.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3.2.4.1-1, Table 7.3.2.4.1-2, and Table 7.3.2.4.1-3 The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range, Mid range, High range (NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest Lowest UL / Lowest DL, Lowest UL / Highest DL (NOTE 3) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: Additional test points selected according to asymmetric channel bandwidths specified in clause 5.3.6. DL channel bandwidth shall be selected first. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. NOTE 5: In a band where UE supports 4Rx, the test needs to be repeated with only 2Rx antennas connected and the other antennas terminated. |  |  |  |  |

Table 7.3.2.4.1-2: Downlink Configuration of each RB allocation

| Channel Bandwidth | SCS(kHz) | LCRBmax | Outer RB allocation / Normal RB allocation |
| --- | --- | --- | --- |
| 3MHz | 15 | 15 | 15@0 |
| 5MHz | 15 | 25 | 25@0 |
|  | 30 | 11 | 11@0 |
|  | 60 | N/A | N/A |
| 10MHz | 15 | 52 | 52@0 |
|  | 30 | 24 | 24@0 |
|  | 60 | 11 | 11@0 |
| 15MHz | 15 | 79 | 79@0 |
|  | 30 | 38 | 38@0 |
|  | 60 | 18 | 18@0 |
| 20MHz | 15 | 106 | 106@0 |
|  | 30 | 51 | 51@0 |
|  | 60 | 24 | 24@0 |
| 25MHz | 15 | 133 | 133@0 |
|  | 30 | 65 | 65@0 |
|  | 60 | 31 | 31@0 |
| 30MHz | 15 | 160 | 160@0 |
|  | 30 | 78 | 78@0 |
|  | 60 | 38 | 38@0 |
| 35MHz | 15 | 188 | 188@0 |
|  | 30 | 92 | 92@0 |
|  | 60 | 44 | 38@0 |
| 40MHz | 15 | 216 | 216@0 |
|  | 30 | 106 | 106@0 |
|  | 60 | 51 | 51@0 |
| 45MHz | 15 | 242 | 242@0 |
|  | 30 | 119 | 119@0 |
|  | 60 | 58 | 58@0 |
| 50MHz | 15 | 270 | 270@0 |
|  | 30 | 133 | 133@0 |
|  | 60 | 65 | 65@0 |
| 60MHz | 15 | N/A | N/A |
|  | 30 | 162 | 162@0 |
|  | 60 | 79 | 79@0 |
| 70MHz | 15 | N/A | N/A |
|  | 30 | 189 | 189@0 |
|  | 60 | 93 | 93@0 |
| 80MHz | 15 | N/A | N/A |
|  | 30 | 217 | 217@0 |
|  | 60 | 107 | 107@0 |
| 90MHz | 15 | N/A | N/A |
|  | 30 | 245 | 245@0 |
|  | 60 | 121 | 121@0 |
| 100MHz | 15 | N/A | N/A |
|  | 30 | 273 | 273@0 |
|  | 60 | 135 | 135@0 |
| NOTE 1: Test Channel Bandwidths are checked separately for each NR band, the applicable channel bandwidths are specified in Table 5.3.5-1. |  |  |  |

Table 7.3.2.4.1-3: Uplink configuration for reference sensitivity, LCRB @ RBstart format

| Operating Band | SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 |  | 25@0 | 50@21 | 75@41 | 100@61 | 128@51 | 128@321 |  | 128@881 | 128@1141 | 128@1421 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 36@21 | 50@11 | 64@11 | 64@141 |  | 64@421 | 64@551 | 64@691 |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 18@0 | 24@0 | 30@11 | 30@81 |  | 30@211 | 30@281 | 30@351 |  |  |  |  |  |  |
| n2 | 15 |  | 25@0 | 50@21 | 50@291 | 50@561 | 50@831 | 48@1121 |  | 40@1761 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 10@11 | 24@0 | 24@141 | 24@271 | 24@411 | 24@541 |  | 20@861 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 10@81 | 10@141 | 10@211 | 10@281 |  | 10@411 |  |  |  |  |  |  |  |  |
| n3 | 15 |  | 25@0 | 50@21 | 50@291 | 50@561 | 50@831 | 50@1101 | 50@1381 | 50@1661 | 50@1921 | 50@2201 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 24@141 | 24@271 | 24@411 | 24@541 | 24@681 | 24@821 | 24@951 | 24@1091 |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 10@81 | 10@141 | 10@211 | 10@281 | 10@341 | 10@411 | 10@481 | 10@551 |  |  |  |  |  |  |
| n5 | 15 |  | 25@0 | 25@271 | 25@541 | 25@811 | Note 5 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 | 10@281 | 10@411 | Note 5 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 |  | 25@0 | 50@21 | 75@41 | 75@311 | 72@611 | 64@961 |  | 45@1711 |  | 45@2251 |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 36@21 | 36@151 | 36@291 | 32@461 |  | 20@861 |  | 20@1131 |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 18@0 | 18@61 | 18@131 | 16@221 |  | 10@411 |  | 10@551 |  |  |  |  |  |  |
| n8 | 15 |  | 25@0 | 25@271 | 25@541 | 25@811 |  |  | Note 5 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 | 10@281 | 10@411 |  |  | Note 5 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 |  | 20@51 | 20@321 | 20@591 |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 | 10@281 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n13 | 15 |  | 20@01 | 20@01 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 |  | 20@01 | 20@01 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 |  | 25@0 | 20@01 | 20@112 | 20@162 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@01 | 10@62 | 10@82 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 |  | 25@0 | 50@0 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 |  | 25@0 | 50@21 | 50@291 | 50@561 | 50@831 | 48@1121 | 40@1481 | 40@1761 | Note 5 |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 24@141 | 24@271 | 24@411 | 24@541 | 20@721 | 20@861 | Note 5 |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 10@81 | 10@141 | 10@211 | 10@281 | 10@341 | 10@411 | Note 5 |  |  |  |  |  |  |  |
| n26 | 15 | 15@0 | 25@0 | 25@271 | 25@541 | 25@811 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 12@121 | 12@261 | 12@391 |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 15 | 15@0 | 25@0 | 25@271 | 25@541 | 25@811 |  | 25@1351 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 | 10@281 | 10@411 |  | 10@681 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 |  | 20@51 | 20@321 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n31 | 15 | 5@9 | 5@10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| n34 | 15 |  | 25@0 | 50@0 | 75@0 |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@0 | 18@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 |  | 25@0 | 50@0 | 75@0 | 100@0 | 128@0 | 160@0 |  | 216@0 |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 | 64@0 | 75@0 |  | 100@0 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 | 30@0 | 36@0 |  | 50@0 |  |  |  |  |  |  |  |  |
| n39 | 15 |  | 25@0 | 50@0 | 75@0 | 100@0 | 128@0 | 160@0 |  | 216@0 |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 | 64@0 | 75@0 |  | 100@0 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 | 30@0 | 36@0 |  | 50@0 |  |  |  |  |  |  |  |  |
| n40 | 15 |  | 25@0 | 50@0 | 75@0 | 100@0 | 128@0 | 160@0 |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 | 64@0 | 75@0 |  | 100@0 |  | 128@0 | 162@0 |  | 216@0 |  |  |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 | 30@0 | 36@0 |  | 50@0 |  | 64@0 | 75@0 |  | 100@0 |  |  |  |
| n41 | 15 |  |  | 50@0 | 75@0 | 100@0 |  | 160@0 |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 |  | 75@0 |  | 100@0 |  | 128@0 | 162@0 | 180@0 | 216@0 | 243@0 | 270@0 |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 |  | 36@0 |  | 50@0 |  | 64@0 | 75@0 | 90@0 | 100@0 | 120@0 | 135@0 |  |
| n48 | 15 |  | 25@0 | 50@0 | 75@0 | 100@0 |  | 160@0 |  | 216@0 |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 |  | 75@0 |  | 100@0 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 |  | 36@0 |  | 50@0 |  |  |  |  |  |  |  |  |
| n50 | 15 |  | 25@0 | 50@0 | 75@0 | 100@0 |  |  |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 |  |  |  | 100@0 |  | 128@0 | 162@0 |  | NOTE 3 |  |  |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 |  |  |  | 50@0 |  | 64@0 | 75@0 |  | NOTE 3 |  |  |  |
| n51 | 15 |  | 25@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n53 | 15 |  | 25@0 | 50@0 |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n54 | 15 |  | 25@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 |  | 25@0 | 50@21 | 75@41 | 100@61 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 36@21 | 50@11 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 18@0 | 24@0 |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 |  | 25@0 | 50@21 | 75@41 | 100@61 | 128@51 | 160@0 |  | 216@0 |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 36@21 | 50@11 | 64@11 | 75@31 |  | 100@61 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 18@0 | 24@0 | 30@11 | 36@21 |  | 50@11 |  |  |  |  |  |  |  |  |
| n70 | 15 |  | 25@0 | 50@21 | 75@41 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 24@0 | 36@21 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10@11 | 18@0 | NOTE 3 | NOTE 3 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 |  | 25@0 | 25@01 | 20@01 | 20@01 | 20@01,6 | 20@01,6 | 20@01,6 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 12@01 | 10@01 | 10@01 | 10@01,6 | 10@01,6 | 10@01,6 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n72 | 15 | 5@9 | 5@10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| n74 | 15 |  | 25@0 | 25@271 | 25@541 | 25@811 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 | 10@281 | 10@411 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 5@61 | 5@131 | 5@191 |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  |  | 50@0 | 75@0 | 100@0 | 128@0 | 160@0 |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 | 64@0 | 75@0 |  | 100@0 |  | 128@0 | 162@0 | 180@0 | 216@0 | 243@0 | 270@0 |  |
|  | 60 |  | - | 10@0 | 18@0 | 24@0 | 30@0 | 36@0 |  | 50@0 |  | 64@0 | 75@0 | 90@0 | 100@0 | 120@0 | 135@0 |  |
| n78 | 15 |  |  | 50@0 | 75@0 | 100@0 | 128@0 | 160@0 |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 | 36@0 | 50@0 | 64@0 | 75@0 |  | 100@0 |  | 128@0 | 162@0 | 180@0 | 216@0 | 243@0 | 270@0 |  |
|  | 60 |  |  | 10@0 | 18@0 | 24@0 | 30@0 | 36@0 |  | 50@0 |  | 64@0 | 75@0 | 90@0 | 100@0 | 120@0 | 135@0 |  |
| n79 | 15 |  |  | 50@0 |  | 100@0 |  |  |  | 216@0 |  | 270@0 |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 |  | 50@0 |  |  |  | 100@0 |  | 128@0 | 162@0 |  | 216@0 |  | 270@0 |  |
|  | 60 |  |  | 10@0 |  | 24@0 |  |  |  | 50@0 |  | 64@0 | 75@0 |  | 100@0 |  | 135@0 |  |
| n85 | 15 | 15@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n91 | 15 |  | 25@04 | 20@321, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n92 | 15 |  | 25@0 | 20@321 | 20@591 | 20@861 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 10@141 | 10@281 | 10@411 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n93 | 15 |  | 25@04 | 25@271, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n94 | 15 |  | 25@0 | 25@271 | 20@591 | 20@861 |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 12@121 | 10@281 | 10@411 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n100 | 15 | 15@0 | 25@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n101 | 15 |  | 25@0 | 50@0 |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  | 24@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n106 | 15 | 15@0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
| n109 | 15 |  | 25@0 | 25@0 | 25@0 | 25@0 | 25@0 | 25@0 |  | 25@0 |  | 25@0 |  |  |  |  |  |  |
|  | 30 |  |  | 24@0 | 24@0 | 24@0 | 24@0 | 24@0 |  | 24@0 |  | 24@0 |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). NOTE 2: For band n20; for 15kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =16; for 30kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =6 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =8; for 60kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =3 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =4. NOTE 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest duplex distance shall be used. NOTE 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. NOTE 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3. does not apply. Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. For UEs not supporting this uplink channel bandwidth, the UL configuration of the 20MHz UL channel bandwidth and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Table 7.3.2.4.1-1, Table 7.3.2.4.1-2, and Table 7.3.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.3.2.4.3.

7.3.2.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3.2.5-1 if 2Rx antennas connected or Table 7.3.2.5-2 if 4Rx antennas connected.  Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2

7.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 ensuring Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents are according to TS 38.508-1[5] subclause 4.6 with the following exceptions for each network signalling value.

7.3.2.4.3.1	Message contents exceptions (network signalling value "NS_01")

Message contents according to TS 38.508-1 [5] subclause 4.6 can be used without exceptions.

7.3.2.4.3.2	Message contents exceptions (network signalling value "NS_03")

1. Information element additionalSpectrumEmission is set to NS_03. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n2, n25 and n66

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_03) |  |  |

Table 7.3.2.4.3.2-2: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n70

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_03) |  |  |

7.3.2.4.3.3	Message contents exceptions (network signalling value "NS_06")

1. Information element additionalSpectrumEmission is set to NS_06. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_06" and NR band n12, n13 and n14

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_06) |  |  |

7.3.2.4.3.4	Message contents exceptions (network signalling value "NS_35")

1. Information element additionalSpectrumEmission is set to NS_35. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.4-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_35" and NR band n71

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_35) |  |  |

7.3.2.4.3.5	Message contents exceptions (network signalling value "NS_27")

1. Information element additionalSpectrumEmission is set to NS_27. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_27" and NR band n48

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_27) |  |  |

7.3.2.4.3.6	Message contents exceptions (network signalling value "NS_21")

1. Information element additionalSpectrumEmission is set to NS_21. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.6-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_21" and NR band n30

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_21) |  |  |

7.3.2.4.3.7	Message contents exceptions (network signalling value "NS_45")

1. Information element additionalSpectrumEmission is set to NS_45. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.7-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_45" and NR band n53

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_45) |  |  |

7.3.2.4.3.8	Message contents exceptions (network signalling value "NS_56")

1. Information element additionalSpectrumEmission is set to NS_56. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 7.3.2.4.3.8-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56" and NR band n24

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_56) |  |  |

7.3.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Tables 7.3.2.5-1a and Tables 7.3.2.5-1b for 2 Rx antenna port, Tables 7.3.2.5-2 a and Tables 7.3.2.5-2b for 4 Rx antenna port, Table 7.3.2.5-2c and Table 7.3.2.5-2d for PC2 UE on FDD bands, and parameters specified Table 7.3.2.4.1-1, Table 7.3.2.4.1-2 and Table 7.3.2.4.1-3.

Table 7.3.2.5-1a: Two antenna port Reference sensitivity QPSK PREFSENS for FDD bands for PC3

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS (kHz) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n1 | 15 |  | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT | -91.9 +TT |  | -90.6 +TT | -90.1 +TT | -89.6 +TT | FDD |
|  | 30 |  |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT | -92.0 +TT |  | -90.7 +TT | -90.2 +TT | -89.7 +TT |  |
|  | 60 |  |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT | -92.1 +TT |  | -90.9 +TT | -90.3 +TT | -89.7 +TT |  |
| n2 | 15 |  | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | -90.7 +TT | -84.1 +TT |  | -81.5 +TT |  |  | FDD |
|  | 30 |  |  | -95.1 +TT | -93.1 +TT | -92.0 +TT | -90.8 +TT | -84.2 +TT |  | -81.6 +TT |  |  |  |
|  | 60 |  |  | -95.5 +TT | -93.4 +TT | -92.2 +TT | -90.9 +TT | -84.3 +TT |  | -81.7 +TT |  |  |  |
| n3 | 15 |  | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.8 +TT | -89.7 +TT | -88.9 +TT | -86.2+TT | -82.3 +TT | -81.3+TT | -79.7 +TT | FDD |
|  | 30 |  |  | -94.1 +TT | -92.1 +TT | -91.0 +TT | -89.8 +TT | -89.0 +TT | -86.3+TT | -82.4 +TT | -81.4+TT | -79.8 +TT |  |
|  | 60 |  |  | -94.5 +TT | -92.4 +TT | -91.2 +TT | -90.0 +TT | -89.1 +TT | -86.4+TT | -82.6 +TT | -81.5+TT | -79.9 +TT |  |
| n5 | 15 |  | -98.0 +TT | -94.8 +TT | -93.0 +TT | -86.8 +TT | -84.8 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -95.1 +TT | -93.1 +TT | -88.6 +TT | -84.9 +TT |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 |  | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | -90.7 +TT | -89.9 +TT |  | -88.6 +TT |  | -81.5 +TT | FDD |
|  | 30 |  |  | -95.1 +TT | -93.1 +TT | -92.0 +TT | -90.8 +TT | -90.0 +TT |  | -88.7 +TT |  | -81.5 +TT |  |
|  | 60 |  |  | -95.5 +TT | -93.4 +TT | -92.2 +TT | -91.0 +TT | -90.1 +TT |  | -88.9 +TT |  | -81.5 +TT |  |
| n8 | 15 |  | -97.0 +TT | -93.8 +TT | -91.4 +TT | -85.8 +TT |  |  | -78.4+TT |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -91.7 +TT | -87.2 +TT |  |  | -78.5+TT |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 |  | -97.0 +TT | -93.8 +TT | -84.0 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -84.1 +TT |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n13 | 15 |  | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 |  | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 |  | -97.0 +TT | -93.8 +TT | -91.0 +TT | -89.8 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.1 +TT | -91.1 +TT | -90.0 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 |  | -100.0 +TT | -96.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 +TT |  |  |  |  |  |  |  |  |  |
| n25 | 15 |  | -96.5 +TT | -93.3 +TT | -91.5 +TT | -90.3 +TT | -89.3 +TT | -82.2 +TT | -81.7 +TT | -79.5 +TT | -77.6+TT |  | FDD |
|  | 30 |  |  | -93.6 +TT | -91.6 +TT | -90.5 +TT | -89.4 +TT | -82.3 +TT | -81.8 +TT | -79.6 +TT | -77.7 +TT |  |  |
|  | 60 |  |  | -94.0 +TT | -91.9 +TT | -90.7 +TT | -89.6 +TT | -82.4 +TT | -81.9 +TT | -79.7 +TT | -77.8 +TT |  |  |
| n26 | 15 | -99.7 +TT | -97.5 +TT | -94.5 +TT | -92.7 +TT | -87.6 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -94.8 +TT | -92.7 +TT | -87.7 +TT |  |  |  |  |  |  |  |
| n28 | 15 | -100.2 +TT | -98.5 +TT | -95.5 +TT | -93.5 +TT | -90.8 +TT |  | -78.5 +TT |  |  |  |  | FDD |
|  | 30 |  |  | -95.6 +TT | -93.6 +TT | -91.0 +TT |  | -78.6 +TT |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 |  | -99.0 +TT | -95.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n31 | 15 | -95.7 + TT | -93.5 + TT |  |  |  |  |  |  |  |  |  | FDD |
| n65 | 15 |  | -99.5+TT | -96.3+TT | -94.5+TT | -93.3+TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.6+TT | -94.6+TT | -93.5+TT |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.0+TT | -94.9+TT | -93.7+TT |  |  |  |  |  |  |  |
| n66 | 15 |  | -99.5 +TT | -96.3 +TT | -94.5 +TT | -93.3 +TT | -92.2 +TT | -91.4 +TT |  | -90.1 +TT |  |  | FDD |
|  | 30 |  |  | -96.6 +TT | -94.6 +TT | -93.5 +TT | -92.3 +TT | -91.5 +TT |  | -90.2 +TT |  |  |  |
|  | 60 |  |  | -97.0 +TT | -94.9 +TT | -93.7 +TT | -92.5 +TT | -91.6 +TT |  | -90.4 +TT |  |  |  |
| n70 | 15 |  | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT |  |  |  |  |  |  |
| n71 | 15 |  | -97.2 +TT | -94.0 +TT | -91.6 +TT | -86.0 +TT | -84.17 +TT -74.88+TT | -82.57 +TT -67.18+TT | -80.77 +TT -64.08 +TT |  |  |  | FDD |
|  | 30 |  |  | -94.3 +TT | -91.9 +TT | -87.4 +TT | -84.27 +TT -74.98 +TT | -82.67 +TT -67.28 +TT | -80.87 +TT -64.18 +TT |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n72 | 15 | -95.7 + TT | -93.5 + TT |  |  |  |  |  |  |  |  |  | FDD |
| n74 | 15 |  | -99.53 +TT | -96.33 +TT | -94.53 +TT | -93.33 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.63 +TT | -94.63 +TT | -93.53 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.03 +TT | -94.93 +TT | -93.73 +TT |  |  |  |  |  |  |  |
| n85 | 15 | -99.2 +TT |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
| n100 | 15 | -102.2 +TT | -100.0 +TT |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n106 | 15 | -99.2 +TT |  |  |  |  |  |  |  |  |  |  | FDD |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: 3 indicates that the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. NOTE 7: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 8: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands for PC3, PC2, PC1.5

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS (kHz) | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11)+TT |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)+TT |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)+TT |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11)+TT |  |
| n411 | 15 | 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/52)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11)+TT |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11)+TT |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11)+TT |  |
| n51 | 15 | 5 | -100+TT | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10 | -97.1+TT |  |
|  | 60 | 10 | -97.5+TT |  |
| n54 | 15 | 5 | -100+TT | TDD |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/52)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11)+TT |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52) +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11)+TT |  |
| n791 | 15 | 10, 20, 40, 50 | -95.8 + 10log10(NRB/52)+TT | TDD |
|  | 30 | 10, 20, 40, 50, 60, 80, 100 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 20, 40, 50, 60, 80, 100 | -96.5 + 10log10(NRB/11) +TT |  |
| n91 | 15 | 5 | -100+TT | FDD |
| n92 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | FDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
| n93 | 15 | 5 | -100+TT | FDD |
| n94 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | FDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10 | -97.1 + 10log10(NRB/24)+TT |  |
| n10911 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) +TT | FDD |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) +TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. NOTE 10: FFS. NOTE 11: Applies for DL channels for which channels edges are > 15 MHz away from 2xFUL at 15 kHz SCS and > 30 MHz away from 2xFUL at 30 kHz SCS. In case of UL second harmonic direct hit, the value is modified to -71.9 dBm for all channel bandwidths. |  |  |  |  |

Table 7.3.2.5-2a: Four antenna port Reference sensitivity QPSK PREFSENS FDD bands for PC3

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 3 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 | -102.7 +TT |  | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT | -94.6 +TT |  | -93.3 +TT | -92.8 +TT | -92.3 +TT | FDD |
|  | 30 |  |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT | -94.7 +TT |  | -93.4 +TT | -92.9 +TT | -92.4 +TT |  |
|  | 60 |  |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT | -94.8 +TT |  | -93.6 +TT | -93 +TT | -92.4 +TT |  |
| n2 | 15 | -100.7 +TT |  | -97.5 +TT | -95.7 +TT | -94.5 +TT | -93.4 +TT | -86.8 +TT |  | -84.2 +TT |  |  | FDD |
|  | 30 |  |  | -97.8 +TT | -95.8 +TT | -94.7 +TT | -93.5 +TT | -86.9 +TT |  | -83.3 +TT |  |  |  |
|  | 60 |  |  | -98.2 +TT | -96.1 +TT | -94.9 +TT | -93.6 +TT | -87.0 +TT |  | -84.4 +TT |  |  |  |
| n3 | 15 | -99.7 +TT |  | -96.5 +TT | -94.7 +TT | -93.5 +TT | -92.4 +TT | -91.6 +TT | -88.9+TT | -90.3 +TT | -84.0+TT | -82.4 +TT | FDD |
|  | 30 |  |  | -96.8 +TT | -94.8 +TT | -93.7 +TT | -92.5 +TT | -91.7 +TT | -90.0+TT | -90.4 +TT | -84.1+TT | -82.5 +TT |  |
|  | 60 |  |  | -97.2 +TT | -95.1 +TT | -93.9 +TT | -92.7 +TT | -91.8 +TT | -90.1+TT | -90.6 +TT | -84.2+TT | -82.6 +TT |  |
| n56 | 15 |  | -100.4 +TT | -97.2 +TT | -95.4 +TT | -89.2 +TT | -87.2 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.5 +TT | -95.5 +TT | -91 +TT | -87.3 +TT |  |  |  |  |  |  |
| n71 | 15 | -100.7 +TT |  | -97.5 +TT | -95.7 +TT | -94.5 +TT | -93.4 +TT | -92.6 +TT |  | -91.3 +TT |  | -84.2 +TT | FDD |
|  | 30 |  |  | -97.8 +TT | -95.8 +TT | -94.7 +TT | -93.5 +TT | -92.7 +TT |  | -91.4 +TT |  | -84.2 +TT |  |
|  | 60 |  |  | -98.2 +TT | -97.1 +TT | -94.9 +TT | -93.7 +TT | -92.8 +TT |  | -91.6 +TT |  | -84.2 +TT |  |
| n85 | 15 | -99.7 +TT |  | -96.5 +TT | -94.1 +TT | -88.5 +TT |  |  | -81.1 +TT |  |  |  | FDD |
|  | 30 |  |  | -96.8 +TT | -94.4 +TT | -89.9 +TT |  |  | -81.2 +TT |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n86 | 15 |  | -99.4 +TT | -96.2 +TT | -93.8 +TT | -88.2 +TT |  |  | -80.8 +TT |  |  |  | FDD |
|  | 30 |  |  | -96.5 +TT | -94.1 +TT | -89.6 +TT |  |  | -80.9 +TT |  |  |  |  |
| n206 | 15 |  | -99.4 +TT | -96.2 +TT | -93.4 +TT | -92.2 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -96.5 +TT | -93.5 +TT | -92.4 +TT |  |  |  |  |  |  |  |
| n266 | 15 | -102.1 +TT | -99.9 +TT | -96.97 +TT | -95.17 +TT | -90 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -97.2 +TT | -95.1 +TT | -90.1 +TT |  |  |  |  |  |  |  |
| n286 | 15 | -102.6 +TT | -100.9 +TT | -97.9+ TT | -95.9 +TT | -93.2 +TT |  | -80.9 +TT |  |  |  |  | FDD |
|  | 30 |  |  | -98 +TT | -96 +TT | -93.4 +TT |  | -81 +TT |  |  |  |  |  |
| n25 | 15 | -99.2 +TT |  | -96.0 +TT | -94.2 +TT | -93.0 +TT | -92.0 +TT | -84.9 +TT | -84.4 +TT | -82.2 +TT | -80.3+TT |  | FDD |
|  | 30 |  |  | -96.3 +TT | -94.3 +TT | -93.2 +TT | -92.1 +TT | -85.0 +TT | -84.5 +TT | -82.3 +TT | -80.4 +TT |  |  |
|  | 60 |  |  | -96.7 +TT | -94.6 +TT | -93.4 +TT | -92.3 +TT | -85.1 +TT | -84.6 +TT | -82.4 +TT | -80.5 +TT |  |  |
| n30 | 15 | -101.7 +TT |  | -98.5 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | -98.8 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | -102.2 +TT |  | -99.0 +TT | -97.2 +TT | -96.0 +TT | -94.9 +TT | -94.1 +TT |  | -92.8 +TT |  |  | FDD |
|  | 30 |  |  | -99.3 +TT | -97.3 +TT | -96.2 +TT | -95.0 +TT | -94.2 +TT |  | -92.9 +TT |  |  |  |
|  | 60 |  |  | -99.7 +TT | -97.6 +TT | -96.4 +TT | -95.2 +TT | -94.3 +TT |  | -93.1 +TT |  |  |  |
| n70 | 15 | -102.7 +TT |  | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT |  |  |  |  |  | FDD |
|  | 30 |  |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT |  |  |  |  |  |  |
|  | 60 |  |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT |  |  |  |  |  |  |
| n716 | 15 |  | -99.6 +TT | -96.4 +TT | -94 +TT | -88.4 +TT | -86.68 +TT -77.39 +TT | -85.08 +TT -69.69 +TT | -83.28 +TT -66.59 +TT |  |  |  | FDD |
|  | 30 |  | -96.7 +TT | -94.3 +TT | -89.8 +TT | -86.7 +TT | -85.18 +TT -77.49 +TT | -83.38 +TT -69.79 +TT | -96.78 +TT -66.69 +TT |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for above listed operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2 The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 3: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. NOTE 5: When 4Rx operation is supported by FWA form factor. NOTE 6: When 4Rx operation is supported by handheld UE. NOTE 7: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 8: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 9: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-2b: Four antenna port Reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands for PC3, PC2, PC1.5

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS (kHz) | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24)+-2.7 TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) -2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n411 | 15 | 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/52)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11)-2.7 +TT |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11)-2.2+TT |  |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/52)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11)-2.2 +TT |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11)-2.2 +TT |  |
| n791 | 15 | 10, 20, 40, 50 | -95.8 + 10log10(NRB/52)-2.2 +TT | TDD |
|  | 30 | 10, 20, 40, 50, 60, 80, 100 | -96.1 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 20, 40, 50, 60, 80, 100 | -96.5 + 10log10(NRB/11)-2.2 +TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |

Table 7.3.2.5-2c: Reference Sensitivity for PC2 UE on FDD bands for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | - | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 |
| n3 | REFSENS_n3 +0.5 | REFSENS_n3+ 0.5 | REFSENS_n3+ 0.5 | REFSENS_n3+ 0.5 | REFSENS_n3+ 0.6 | REFSENS_n3+ 0.8 | REFSENS_n3+ 1.1 | REFSENS_n3+ 1.5 | REFSENS_n3+ 2.3 | REFSENS_n3+ 2.8 |
| n8 | REFSENS_n8 +0.5 | REFSENS_n8 +0.7 | REFSENS_n8 +0.8 | REFSENS_n8 +2.3 | - | - | REFSENS_n8 +3.1 | - | - | - |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: REFSENS_n1 refers to the two antenna port and four antenna port Reference Sensitivity of n1 in Table 7.3.2.5-1a and Table 7.3.2.5-2a. NOTE 3: REFSENS_n3 refers to the two antenna port and four antenna port Reference Sensitivity of n3 in Table 7.3.2.5-1a and Table 7.3.2.5-2a. NOTE 4: REFSENS_n8 refers to the two antenna port and four antenna port Reference Sensitivity of n8 in Table 7.3.2.5-1a and Table 7.3.2.5-2a. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-2d: Reference Sensitivity for PC2 UE on FDD bands for UE supporting Tx Diversity operation bands

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 | - | REFSENS_n1 | REFSENS_n1 | REFSENS_n1 |
| n3 | REFSENS_n3+ 1.4 | REFSENS_n3+ 1.5 | REFSENS_n3+ 1.5 | REFSENS_n3+ 1.5 | REFSENS_n3+ 1.6 | REFSENS_n3+ 1.7 | REFSENS_n3+ 2.8 | REFSENS_n3+ 5 | REFSENS_n3+ 5.5 | REFSENS_n3+ 6.0 |
| n8 | REFSENS_n8 +1.3 | REFSENS_n8 +1.4 | REFSENS_n8 +2.1 | REFSENS_n8 +5.8 | - | - | REFSENS_n8 +7.0 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: REFSENS_n1 refers to the two antenna port and four antenna port Reference Sensitivity of n1 in Table 7.3.2.5-1a and Table 7.3.2.5-2a. NOTE 3: REFSENS_n3 refers to the two antenna port and four antenna port Reference Sensitivity of n3 in Table 7.3.2.5-1a and Table 7.3.2.5-2a. NOTE 4: REFSENS_n8 refers to the two antenna port and four antenna port Reference Sensitivity of n8 in Table 7.3.2.5-1a and Table 7.3.2.5-2a. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-3: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

For the UE which supports CA, SUL or DC band combination, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 a and Table 7.3.2.5-1b shall be increased by the amount given in ΔRIB,c defined in subclause 7.3.3 for the applicable operating bands.

### 7.3.2_1	Reference sensitivity power level for XR

7.3.2_1.1	Test purpose

The test purpose is to verify the ability of the XR UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3.2_1.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that indicate support for supportOf2RxXR-r18.

7.3.2_1.3	Minimum conformance requirements

For a XR UE equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2.3-1a and Table 7.3.2.3-1b for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a XR UE equipped with 2 Rx antenna ports shall be adjusted by the value specified in Table [TBD] met with uplink transmission bandwidth less than or equal to that specified in Table [TBD] and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For two Rx antenna port XR UE(s) indicating UE capability supportOf2RxXR-r18, reference sensitivity for two Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRXR,2R in Table 7.3.2_1-1 for the applicable operating bands.

Table 7.3.2_1-1: Two antenna port XR UE reference sensitivity allowance ΔRXR,2R

| Operating band | ΔRXR,2R (dB) |
| --- | --- |
| n7, n38, n41, n48, n77, n78, n79 | -1.0 |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.2-1a, Table 7.3.2-1b along with the modification specified in Table 7.3.2_1-1 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3.2.

7.3.2_1.4	Test description

7.3.2_1.4.1	Initial conditions

FFS

7.3.2_1.4.2	Test procedure

FFS

7.3.2_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 ensuring Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

7.3.2_1.5	Test requirement

FFS

### 7.3.3	ΔRIB,c

For a UE supporting CA, SUL or DC band combination, the minimum requirement for reference sensitivity in Table 7.3.2.3-1 shall be increased by the amount given by ΔRIB,c defined in subclause 7.3A.0.3, 7.3C.0.3, 7.3B in this specification and 7.3A, 7.3B in TS 38.101-3 [4] for the applicable operating bands.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in subclause 7.3A.0.3, 7.3C.0.3, 7.3B in this specification and 7.3A, 7.3B in TS 38.101-3 [4], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in subclause 7.3A.0.3, 7.3C.0.3, 7.3B in this specification and 7.3A, 7.3B in TS 38.101-3 [4] for the applicable operating bands.